Source:	Xiaomi

Title:	Updates to the video decoder interface capabilities and AR UE device capabilities

## Document for:	Agreement

## Agenda Item:	9.5

# 1	Introduction

At 3GPP SA4#121, the section 6.8 Visual functions and capabilities was added to the MeCAR Permanent Document based on the discussion around the TDoc S4-221424 [1].

The present contribution proposes additional updates to the MeCAR Permanent Document to improve is 6.8 section and address the received comments. The proposed changes essentially make concurrent video decoding instances defined as profiles that any MeCAR device may reference.

# 2	Proposal

## 6.8	Visual functions and capabilities

### 6.8.1	Video Decoding

AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [6] bitstreams, with the chroma format being 4:2:0; and the bit depth being 8 bit.

HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[7] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, with the chroma format being 4:2:0; and the bit depth being 8 bit.

### 6.8.2	Video Encoding

AVC-FullHD-Enc: the capability to encode H.264 (AVC) Progressive High Profile Level 4.2 [6] bitstreams, with the chroma format being 4:2:0; and the bit depth being 8 bit.

HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 6.1

### Added text

### 6.8.3	Video decoding interface

AVC-HEVC-2-Dec: the capability to support two concurrent video decoding instances from any of the following profiles that are AVC-FullHD-Dec  and HEVC-FullHD-Dec.

### End of Added text

And the corresponding change in 4.3 Augmented Reality User Equipment (AR UE).

### From

#### 4.3.2.2	Video decoding interface

The AR UE shall support two video decoding instances as defined in clause 4.3.2.1.

### To

#### 4.3.2.2	Video decoding interface

The AR UE shall support the video decoding interface AVC-HEVC-2-Dec defined in clause 4.3.2.1.

# 3	Conclusion

We recommend adopting the changes presented in clause 2 into the next revision of the MeCAR Permanent Document.

# References

- 3GPP TSG SA WG4#121 meeting, Tdoc S4-221424, Xiaomi, 14th – 18th November 2022