Source:	Orange

Title:	Proposed updates on JBM performance

## Document for:	Agreement

## Agenda Item:	7.7

- Introduction

Based on the methodology proposed in [1] we suggest changes to delay tests in TS 26.131 and 26.132 for the eUET work item [2].

- New delay/loss profile for JBM performance evaluation

A methodology for VoLTE realistic profile construction is detailed in [1] (with three sub-profiles: Profiles 1, 2 , 3) and is used to analyze JBM behaviour.

We propose to follow the methodology to construct a JBM test profile based on observed impairment in live VoLTE operation.

Here, we define three sub-profiles:

- Profile 0: packet sequence with no degradation other than a minimal jitter

- Profile 1: derived from Profile 0, including packet losses (2.98% overall, 3.3% on active speech)

- Profile 2: derived from Profile 0, including only delay variation, inversion and duplication (i.e. no packet loss)

Profiles 0 and 1 for validation purposes. Profile 0 is the same as condition 0 in current delay tests in TS 26.132, to get the quality reference (higher quality bound).  Profile 1 has packet losses located at most sensitive segments, to get a lower quality bound.

In the following we refer to Profile 2 as ‘dly_profile_new’. This profile excludes packet losses to avoid mixing JBM and PLC, to focus only on JBM performance.

Notes:

- In [1] we also referred to Profile 3 which combines all impairments, it is not included in the present contribution, however it could be provided if needed.

- Profiles in 26.132 used for delay tests are in two formats: text and TCN files. However, this format does not support duplications. Instead we attach here the profiles (0, 1, 2) as  PCAP files. Since this corresponds to pre-recorded packets with impairments, this should be repeated for each codec: AMR 12.2 in NB, AMR-WB 12.65 in WB and EVS 24.4 in SWB.

- For the sake of providing only example, we focus here on EVS (DTX on). The underlying test sequence is the concatenation of 4 double sentences of 8s to get 32s of speech as in delay tests in TS 26.132, however we did not include the 5 repeats, so the profile length is only 32 sec and not 160 sec.

We highlight that the attached profiles are only provided as examples. We do not exclude some updates, in particular taking into account potential limitations of test equipment.

- Proposed updates to TS 26.131

Annex A contains draft changes to the NB delay requirements; this NB example can be extended to other bandwidths.

To avoid adding extra test time and based on suggestions in previous meetings we propose to replace one of the delay/loss profiles by the new profile described in Section 2. To be specific we propose to replace the current profile (denoted dly_profile_40msDRX_10pct_BLER_e2e) in test condition 2.

- Proposed updated to TS 26.132

Annex B contains draft changes to the NB test methods; this NB example can be extended to other bandwidths. Furthermore, this annex also contains placeholders to changes in the profile description (Annex E) and attachments to TS 26.132.

- Test results

We already reported test results in [1]. Two test cases were reported (with MOS-LQO histograms):

- Simulation based on the reference EVS decoder (with rtpdump support)

- Measurements using an example DUT with the PCAP player feature of Rhode&Schwarz CMW500 equipment (with an RTP stream pre-recorded with packet impairments according to the given profiles).

At the time of writing, experiments to compare these results with measurements conducted with HEAD acoustics LabCore are not completed. A revision of the present Tdoc may be provided during SA4#122 if results are fully available and if the revision can be handled.

## References

- S4-221445, On profiles for JBM behaviour evaluation, Source: Orange

- SP-220610, New WID on Enhancements to UE Testing, Source: Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon

Annex A: Changes to TS 26.131

Change 1 (NB)

## 5.12	Delay

### 5.12.0	UE delay definition

#### 5.12.0.1	Acoustic interfaces

For UMTS circuit-switched operation and MTSI-based speech with LTE, NR or WLAN access, the UE delays in the send and receive directions are defined for acoustic interfaces as:

-	The UE delay in the send (uplink) direction is the delay between the first acoustic event at the MRP to the last bit of the corresponding speech frame at the UE antenna

-	The UE delay in the receive (downlink) direction is the delay between the first bit of a speech frame at the UE antenna and the first acoustic event at the DRP corresponding to that speech frame

NOTE: In order to harmonize UMTS and LTE delay definitions, the reference points for UMTS UE delay have been changed in Rel-12. Prior to Rel-12, the UE reference points for UMTS were implicitly defined by the compensation factors declared by system simulator vendors, i.e. half of the air interface delay was attributed to the UE, and the last acoustic event at DRP was used for the receive measurement instead of the first.

Considering 10ms for half of the transmission time in each direction of a UMTS call, a speech frame size of 20ms, a codec look-ahead of 5ms, and a difference between the first and the last acoustic event of 20 ms, the previous reference points took into account a UE implementation independent delay of 2x10ms + 25 ms + 20 ms = 65 ms. The same UE delay remains in the new definition above, which attributes the full air interface delay to the UE but uses the first acoustic event at the DRP, instead of the last (2x20ms + 25 ms + 0 ms = 65 ms). Hence, the UMTS requirements with these new reference points remain the same.

#### 5.12.0.2	Electrical interfaces

The UE delays in the send and receive directions are defined for electrical interfaces as:

-	The UE delay in the send (uplink) direction is the delay between the first electrical event at the input of the electrical interface (analogue or wired digital connection) or first bit of a speech frame at the UE antenna of the wireless digital interface (wireless digital connection) to the last bit of the corresponding speech frame at the UE antenna

-	The UE delay in the receive (downlink) direction is the delay between the first bit of a speech frame at the UE antenna and the first electrical event at the output of the electrical interface (analogue or wired digital connection) or first bit of a speech frame at the UE antenna of the wireless digital interface (wireless digital connection) corresponding to that speech frame.

### 5.12.1	Handset UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8bis, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE, NR or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8bis, and 80ms for test condition 2 of Table 8bis.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8bis: UE delay and speech quality requirements for LTE, NR and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_new | TS + TR ≤ TBDms | TS + TR ≤ TBDms | MOS-LQOTEST ≥ MOS-LQOREF - TBD |
|  | NOTE 1: The delay profile for test condition 1 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profile is injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. The delay profile in test condition 1 is static delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. |  |  |  |
|  | NOTE 1bis: The delay profile for test condition 2 is based on observed impairments in live operation. Delay profile is injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

### 5.12.2	Headset UE

#### 5.12.2.1	Wired headset

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving gdirections (TS + TR) shall in any case be ≤ 220ms and should be ≤ 185ms.

NOTE: A delay ≤ 185 ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190ms.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8, while meeting the speech quality targets defined.

NOTE: The UE delay requirements for MTSI-based speech-only with LTE, NR or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8ter, and 80ms for test condition 2 of Table 8ter.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8ter: UE delay and speech quality requirements for LTE, NR and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150ms | TS + TR ≤ 190ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150ms | TS + TR ≤ 190ms | MOS-LQOTEST ≥ MOS-LQOREF - 0.3 |
| 2 | dly_profile_new | TS + TR ≤ TBDms | TS + TR ≤ TBDms | MOS-LQOTEST ≥ MOS-LQOREF - TBD |
|  | NOTE 1: The delay profile for test condition 1 istheoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. The delay profile in test condition 1 isstatic delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. |  |  |  |
|  | NOTE 1bis: The delay profile for test condition 2 is based on observed impairments in live operation. Delay profile is injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

#### 5.12.2.2	Wireless headset

For further study.

### 5.12.3	Electrical interface UE

It is in general desirable to minimize UE delays to ensure low enough end-to-end delays and hence a good conversational experience, guidance is found in ITU-T Recommendation G.114.

The delay budget BD to be considered in the performance requirements and objectives depends on the type of electrical interface UE and is given by Table 8quater1.

Table 8quater1: Delay budget for different types of electrical interface UE

| Electrical interface type | BD [ms] |
| --- | --- |
| Analogue | 0 |
| Wireless digital | 20 |
| Wired digital | 20 |
| NOTE: The delay for analogue to digital conversion (and vice versa) in the wired digital connection is considered to be part of the budget allowance for vendor specific implementation in the UE delay requirements. |  |

For UMTS circuit-switched AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 220+BD ms and should be ≤ 185+BD ms.

NOTE1: 	A delay ≤ 185+BD ms might not be achievable in some cases due to UE implementation trade-offs between delay and other parameters such as speech quality enhancement, performance of noise reduction or UE power consumption optimization, and UE implementation issues such as rebuffering between components.

For MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) should be ≤ 150+BD ms. If this performance objective cannot be met, the sum of the UE delays in sending and receiving directions (TS + TR) shall in any case be ≤ 190+BD ms.

For MTSI-based speech-only with LTE, NR or WLAN access in conditions with simulated packet arrival time variations and packet loss and AMR speech codec operation, the sum of the UE delays in sending and receiving directions (TS + TR) shall be less than or equal to the delay requirements in Table 8quater2, while meeting the speech quality targets defined.

NOTE2: 	The UE delay requirements for MTSI-based speech-only with LTE, NR or WLAN access are derived from:

-	A speech frame buffering and codec look-ahead of 25ms.

-	An air interface transmission time of 1ms on receive and 1ms on the send direction.

-	A budget allowance for a jitter buffer depth of 40ms for error and jitter free conditions and test conditions 0 and 1 of Table 8quater, and 80ms for test condition 2 of Table 8quater.

-	A budget allowance for vendor specific implementation of 83ms corresponding to the performance objective and 123ms corresponding to the required maximum UE send and receive delay.

Table 8quater2: UE delay and speech quality requirements for LTE, NR and WLAN access

| Test Condition | Delay and Loss Profile (Note 1) | Performance Objectives for Maximum Delay | Requirements for Maximum Delay | Speech Quality Requirements (Note 2) |
| --- | --- | --- | --- | --- |
| 0 | Error and jitter free condition | TS + TR ≤ 150+BD ms | TS + TR ≤ 190+BD ms | No requirement, reference score MOS-LQOREF |
| 1 | dly_profile_20msDRX_10pct_BLER_e2e | TS + TR ≤ 150+BD ms | TS + TR ≤ 190+BD ms | MOS-LQOTEST ≥ MOS-LQOREF -0.3 |
| 2 | dly_profile_new | TS + TR ≤ TBD+BD ms | TS + TR ≤ TBD+BD ms | MOS-LQOTEST ≥ MOS-LQOREF -TBD |
|  | NOTE 1: The delay profiles for test condition 1 is theoretically constructed to simulate a semi-persistent scheduling transmission scheme with DRX enabled and target BLER in sending and receiving directions of 10%, with +/- 3ms of EPC jitter. Delay profiles are injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. The delay profile in test condition 1 isstatic delay variation conditions and do not expose the UE to packet delay variations in the full range of the packet delay budget as defined for QCI1 in 3GPP TS 23.203 [18]. |  |  |  |
|  | NOTE 1bis: The delay profile for test condition 2 is based on observed impairments in live operation. Delay profile is injected at the IP layer of the test system. Delay profile is attached electronically to document 3GPP TS 26.132 [1]. |  |  |  |
|  | NOTE 2: The purpose of this test is to provide a relative comparison of the objective speech quality between the reference and test conditions. This test is not to be construed as a method to evaluate the absolute objective speech quality of the device. |  |  |  |

Compliance shall be checked by the relevant tests described in 3GPP TS 26.132.

Annex B: Changes to TS 26.132

Change 1 (NB)

### 7.10.4	Delay and speech quality in conditions with packet arrival time variations and packet loss (handset, headset, electrical interface UE)

#### 7.10.4.1	Delay in sending direction

The UE delay in the sending direction, TS, shall be measured in jitter and error free conditions according to clause 7.10.0.

#### 7.10.4.2	Delay in receiving direction

For this test it shall be ensured that the call is originated from the UE.

NOTE 1: 	Differences have been observed between UE originated calls and UE-terminated calls. For better consistency, calls from the UE are used.

The test signal consists of 3 repeats of the Composite Source Signal (CSS) according to ITU-T Recommendation P.501 [22] followed by a speech signal of 160s. During the first two CSS signals the terminal can adapt its jitter buffer. The third CSS is used for measuring the delay in constant-delay condition, and the speech signal is used for delay and quality measurement in the packet impairment condition.

Constant delay Tc corresponding to the minimum delay of the profile (i.e. the compensation value for the profile) shall be added at the beginning of the different delay/loss profiles, to avoid unecessary delay jumps between the two measurement phases and realistic conditions for the second measurement test phase.

In receiving direction, the delay between the electrical access point of the test equipment and reference point (RP), TTEAP-RP(t) = TR-jitter(t) + TTER, is measured in two successive phases:

1)	First the delay in constant-delay condition TTEAP-RP-constant is measured as described in steps 1 to 4, clause 7.10.2 /7.10.2a/7.10.2b, using the third CSS signal. The constant delay Tc is subtracted from TTEAP-RP-constant to obtain TR-constant.

2)	Then the delay with packet impairment TR-jitter(t) is measured continuously for a speech signal during the inclusion of packet delay and loss profiles in the receiving direction RTP voice stream.

The reference point is defined as follows:

-	for handset and headset UE, the reference point is the DRP.

-	for electrical interface UE, the reference point is the input of the electrical reference interface.

Packet impairments shall be applied between the reference client and system simulator eNodeB. Separate calls shall be established for each packet impairment condition.

Editor’s note: implementation details on how to use packet sequences to be discussed

The start of the delay profiles must be synchronized with the start of the downlink speech material reproduction (compensated by the delay between reproduction and the point of impairment insertion, i.e. the delay of the reference client) in order to ensure a repeatable application of impairments to the test speech signal. Tests shall be performed with DTX enabled in the reference client.

NOTE 2: RTP packet impairments representing packet delay variations and loss in LTE transmission scenarios are specified in Annex E. These LTE jitter/loss profiles are reused also for tests with WLAN and NR access. Care must be taken that the system simulator uses a dedicated bearer with no buffering/scheduling of packets for transmission.

For the CSS signal repeated 3 times, the pseudo random noise (pn)-part of the CSS has to be longer than the maximum expected delay. It is recommended to use a pn sequence of 32 k samples (with 48 kHz sampling rate). The test signal level is -16 dBm0 measured at the digital reference point or the equivalent analogue point.

For the speech signal, 8 English test sentences according to ITU-T P.501 Annex C.2.3, normalized to an active speech level of -16dBm0, are used (2 male, 2 female speakers). The sequences are concatenated in such a way that all sentences are centered within a 4.0s time window, which results in an overall duration of 32.0s. The sequences are repeated 5 times, resulting in a test file 160.0s long. The first 2 sentences are used for convergence of the UE jitter buffer manager and are discarded from the analysis. Equivalent implementations of the concatenation by repeating the test sentences in sequence may be used.

For the delay calculation with the speech signal, a cross-correlation with a rectangular window length of 4s, centered at each sentence of the stimulus file, is used. The process is repeated for each sample. For each cross correlation, the maximum of the envelope is obtained producing one delay value per sentence.

The UE delay in the receive direction, TR-jitter(t), is obtained by subtracting the delay introduced by the test equipment and the simulated transport network packet delay introduced by the delay and loss profile (as specified for the respective profile in Annex E) from the first electrical event at the electrical access point of the test equipment to the first bit of the corresponding speech frame at the system simulator antenna, TTER, from the measured TTEAP-DRP(t).

The difference DT between maximum receiving delay obtained with at least 5 individual calls (see clause 7.10.2) and the delay TR-constant measured for the CSS signal in constant delay condition is calculated. The quantity "Call-to-Call Variability Adjustment" (CCVA) = max(0,DT) shall be added to the obtained delay for the speech signal TR-jitter(t).

For stationary packet delay variation test conditions (test condition 1 and 2), the first 2 sentences are used for convergence of the jitter buffer management and are discarded from the analysis. The CCVA-adjusted UE delay (TR-CCVA(t) = TR-jitter(t) + CCVA) in the receiving direction shall be reported as the maximum value excluding the two largest values of the remaining sequence of the 38 sentence delay values, i.e. the 95-percentile value of TR-CCVA(t). The TR-CCVA values for all 40 sentences shall be reported in the test report.

NOTE 3: The synchronization of the speech frame processing in the UE to the bits of the speech frames at the UE antenna may lead to a variability of up to 20 ms of the measured UE receive delay between different calls. This synchronization is attributed to the UE receiving delay according to the definition of the UE delay reference points The effect of this possible call-to-call variation is taken into account with the CCVA = max(0,DT) value.

#### 7.10.4.3	Speech quality loss in conditions with packet arrival time variations and packet loss

For the evaluation of speech quality loss in conditions with packet arrival time variations and packet loss, the test signal described in clause 7.10.4.2 shall be used. The first 2 sentences are used for convergence of the UE jitter buffer manager and are discarded from the analysis. Two 48 kHz recordings are used to produce the speech quality loss metric:

-	A recording obtained in jitter and error free conditions with the test signal described in clause 7.10.4.2 (reference condition)

-	A recording obtained during the application of packet arrival time variations and packet loss as described in clause 7.10.4.2 (test condition)

The speech quality of the signal is estimated using the measurement algorithm described in ITU-T Recommendation P.863 [44] in super-wideband mode. For narrowband speech, the method according to Appendix III of P.863 [44] shall be used. Level pre-alignment to -26 dBov of recordings shall be used – see P.863.1 clause 10.2 [45].

NOTE: 	For the analysis of acoustical measurements, ITU-T P.863 [44] assumes diffuse-field equalized recordings. For this reason, signals at DRP are diffuse-field corrected for testing handset and headset UE. For electrical interface UE, only the level pre-alignment is applied.

A score shall be computed for each 8s speech sentence pair and averaged to produce a mean MOS-LQO value for the reference and test conditions.

MOS-LQOREF

MOS-LQOTEST

NOTE: This evaluation of the speech quality requirement is only applicable to test conditions with a stationary statistic of the packet delay variation. Evaluation of the speech quality for a test condition with non-stationary packet delay variations is for further study.

The synchronization between stimuli and degraded condition shall be done by the test system before applying the P.863 algorithm on each sentence pair.

Change 2 (Annex E)

Annex E (normative):
Packet delay and loss profiles for UE delay testing of MTSI-based speech with LTE access

# E.1	General

This Annex provides packet delay and loss profiles to be inserted on the IP packets in the test system when testing the UE delay and speech quality for MTSI-based speech with LTE access in jitter and error conditions. The profiles are based on the assumption that one IP packet corresponds to 20ms of the speech sequence. In order to preserve the synchronization of the elements in the profile and the timing of the speech sequence (e.g. the RTP time stamp) when the speech codec does not produce one packet every 20ms, e.g. during speech codec DTX operation, the profiles needs to be sub-sampled in accordance with the rate of production of the packets containg the speech frames.

The variation in the packet arrival time and the loss rate of the speech packets experienced by the receiving UE in an LTE connection will vary depending on several parameters and operating conditions of the LTE network. Standardized characteristics with respect to Packet Delay Budget (PDB) and Packet Error Loss Rate (PELR) for different QoS Class Identifiers (QCI) for an LTE access network are defined in 3GPP TS 23.203 [42]:

-	The PDB of a QCI defines an upper bound for the time that a packet may be delayed between the UE and the PCEF. For a certain QCI the value of the PDB is the same in uplink and downlink. The purpose of the PDB is to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). The PDB shall be interpreted as a maximum delay with a confidence level of 98 percent.

-	The PELR of a QCI defines an upper bound for the rate of SDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in E-UTRAN) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in E-UTRAN). Thus, the PELR defines an upper bound for a rate of non congestion related packet losses. The purpose of the PELR is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in E-UTRAN). For a certain QCI the value of the PELR is the same in uplink and downlink.

The stipulated PDB and PELR as defined in 3GPP TS 23.203 for QCI1 intended for Conversational Voice services results in that on each link 99 percent of the packets will be successfully delivered over an LTE air interface with a maximum delay of less than 80ms with a confidence level of 98 percent.

# E.2	Simulation model for generating packed delay and loss profiles

This clause describes a simulation model that generates packet arrival time variations and packet loss experienced by the receiving UE for MTSI-based speech with an end-to-end LTE access.

The model is derived solely for the purpose of testing the UE delay for MTSI-based speech with LTE access. As discussed in Section E.1, several LTE network parameters have a significant impact on the packet delay variations experienced by the UE. This model includes the effect of the DRX cycle, the BLER on the MAC/PHY layer, the HARQ re-transmission and RMC scheduler [46]. In addition, jitter in the EPC and the effect of the mis-alignment of the DRX-cycle between the uplink and downlink eNBs are included.

The model is described in Table E.1 and operates on the following input parameters

-	Uplink and downlink block error rate (BLER), respectively.

-	Maximum number of HARQ re-transmissions on uplink and dowblink, respectively.

-	DRX cycle length.

-	Time differnce between the uplink and the downlink eNB DRX cycle.

-	Maximum and minimum network delay between the uplink and the downlink eNB.

The random number generator used in the model produces pseudorandom values drawn from the standard uniform distribution on the open interval (0, 1).

The model generates packet delay and loss profiles for two different test system configurations of the UE delay:

-	When the system simulator is transparent in the downlink at the MAC/PHY layer and does not operate in DRX. This approach requires the variations of the delay due to downlink HARQ re-transmissions and mapping to the DRX cycle to be simulated. In this case, the model simulates the delay and jitter profiles for the packets from the antenna of the sending UE to the antenna receiving UE (end-to-end simulation). This is the testing condition used in 3GPP TS 26.132.

-	When the system simulator is configured for error insertion in the downlink at the MAC/PHY layer and DRX operation, and the system simulator implements the HARQ re-transmissions and the mapping of the packets to the DRX cycle time at the downlink. In this case, the model simulates the delay and loss profiles from the sending UE up to the receiving eNB. This testing condition is currently not used in 3GPP TS 26.132.

It should be noted that the model does not fully utilize the PDB for QCI1 as defined in 3GPP TS 23.203 and does not include temporary variations of the packet arrival time variation and the loss rate that may be experienced during e.g. hand-over or congestion. Hence, the packet delay and loss profiles generated by the model do not fully exercise the conditions that the jitter buffer management of the UE may be exposed to in LTE systems and the profiles generated by the model are only intended for the testing of the UE delay in stationary operating conditions.

Table E.1: Simulation model for generating packet delay and loss profiles for MTSI-based speech with LTE access

function [UE1_UE2_dly,UE1_eNB2_dly,plr,comp_e2e,comp] = ... VoLTEDelayProfile_vPHY(BLER_tx, BLER_rx, max_tx, max_rx, drx_cycle_length, mis_eNB1_eNB2, max_net_delay, min_net_delay, nFrames, seed) % BLER_tx : The block error rate in uplink. % BLER_rx : The block error rate in downlink. % max_tx : The maximum number of transmission attempts in uplink. % max_rx : The maximum number of transmission attempts in downlink. % drx_cycle_length : The length of the DRX cycle % mis_eNB1_eNB2 : Scheduling time mis-alignment between eNB1 and eNB2 % max_net_delay : The maximum network delay between eNB1 to eNB2 % min_net_delay : The minimum network delay between eNB1 to eNB2 % nFrames : The number of frames for the simulation % seed : Random number generator seed rng(seed); UE1_UE2_time = zeros(nFrames, 1); UE1_eNB2_time = zeros(nFrames,1); eNB1_eNB2_dly = round(min_net_delay + (max_net_delay-min_net_delay).*rand(nFrames,1)); ack1 = zeros(nFrames,1); ack2 = zeros(nFrames,1); wall_clock = 20; frame = 1; frame_size = 20; simulationTime = nFrames*frame_size; % Calculate the delay from UE1 speech encoder delivery to eNB2. If % transmission to eNB1 is not successful after max_tx attempts, dly = 0 (packet loss) while (wall_clock<=simulationTime) % Set the scheduling time if drx_cycle_length == 0 UE1_scheduling_time=wall_clock; else UE1_scheduling_time=ceil(wall_clock/drx_cycle_length)*drx_cycle_length; end % Add the tx effect for the scheduling time n=0; eNB1_receive_delay = 0; while n < max_tx if (rand(1) < BLER_tx) eNB1_receive_delay = eNB1_receive_delay+8; n=n+1; ack=0; else ack=1; n=max_tx; end end while (wall_clock<=UE1_scheduling_time) UE1_eNB2_time(frame)=ack*(UE1_scheduling_time+eNB1_receive_delay+eNB1_eNB2_dly(frame)); wall_clock=wall_clock+frame_size; ack1(frame)=ack; frame=frame+1; end; end % Translate arrival time to packet delay for UL simulation wall_clock = frame_size*(1:nFrames)'; UE1_eNB2_dly = max(-1, UE1_eNB2_time-wall_clock); % Sort for monotonic arrival time to DL for simulation [UE1_eNB2_time,monotonic_index]=sort(UE1_eNB2_time); % Calculate the delay from eNB2 to UE2 (only for those packets that % successfully arrived at the eNB2!). If transmission to UE2 is not % successful after max_tx attempts, dly = 0; (packet loss) frame = 1; UE2_scheduling_time=mis_eNB1_eNB2; while frame<=nFrames % Add the rx effect for the scheduling time n=0; eNB2_transmit_delay = 0; while n < max_rx if (rand(1) < BLER_rx) eNB2_transmit_delay = eNB2_transmit_delay+8; n=n+1; ack=0; else ack=1; n=max_rx; end end while ((frame<=nFrames)&&(UE1_eNB2_time(frame)<UE2_scheduling_time)) if (UE1_eNB2_time(frame)==-1) UE1_UE2_time(frame)=-1; else UE1_UE2_time(frame)=ack*(UE2_scheduling_time+eNB2_transmit_delay); end ack2(frame)=ack; frame=frame+1; end % Update the scheduling time UE2_scheduling_time=UE2_scheduling_time+drx_cycle_length; end % Re-order for orignal transmit order UE1_UE2_time(monotonic_index) = UE1_UE2_time; % Translate arrival time to packet delay wall_clock = frame_size*(1:nFrames)'; UE1_UE2_dly = max(-1, UE1_UE2_time-wall_clock); % Set compensation values if drx_cycle_length==0 comp_e2e=min_net_delay; comp=min_net_delay; else comp_e2e=min(UE1_UE2_dly(UE1_UE2_dly>0)); comp=min(UE1_eNB2_dly(UE1_eNB2_dly>0)); end % Calculates the overall packet loss from UE1 to UE2 pl=0; for frame=1:nFrames if UE1_UE2_dly(frame)==-1; pl=pl+1; end end plr=pl/nFrames;

# E.2a	Methodology to construct a profile based on impairments observed in live operation

Editor’s Note: the description is not available, see S4-221445 for a preliminary description

# E.3	Packet delay and loss profiles for simulated stationary operating conditions with DRX 20 ms and DRX 40 ms

Three delay and loss profiles simulating the packet delay variations and packet loss that a UE may experience in stationary operating conditions with 20ms, 40ms DRX and 40ms DRX with higher BLER, respectively, are generated from the model with the parameter settings as described in Table E.2. The delay profiles are attached as text files and as binary trace files in the form of TCN network emulator.

Table E.2: Parameter settings for packet delay profile generation.

| Operating condition | Parameter setting | System simulator operation | Delay and Loss profile |
| --- | --- | --- | --- |
| Stationary DRX 20 ms | BLER_tx : 0.1 BLER_rx : 0.1 max_tx : 3 max_rx : 3 drx_cycle_length : 20 mis_eNB1_eNB2 : 10 max_net_delay : 33 min_net_delay : 27 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_20msDRX_10pct_BLER_e2e |
|  |  | System simulator generating HARQ re-transmissions and DRX operation on the downlink | dly_profile_20msDRX_10pct_BLER_ue1_to_eNB2 (Not used in 3GPP TS 26.131) |
| Stationary DRX 40 ms | BLER_tx : 0.1 BLER_rx : 0.1 max_tx : 3 max_rx : 3 drx_cycle_length : 40 mis_eNB1_eNB2 : 30 max_net_delay : 33 min_net_delay : 27 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_40msDRX_10pct_BLER_e2e (Not used in 3GPP TS 26.131) |
|  |  | System simulator generating HARQ re-transmissions and DRX operation on the downlink | dly_profile_40msDRX_10pct_BLER_ue1_to_eNB2 (Not used in 3GPP TS 26.131) |
| Stationary DRX 40 ms (22% BLER) | BLER_tx : 0.22 BLER_rx : 0.22 max_tx : 3 max_rx : 3 drx_cycle_length : 40 mis_eNB1_eNB2 : 30 max_net_delay : 36 min_net_delay : 24 nFrames : 8000 seed : 0 | Transparent system simulator downlink | dly_profile_40msDRX_22pct_BLER_e2e |

Since the model simulates the packet transmission end-to-end, the delay profiles include a packet delay component that according to the definition of the UE delay in 3GPP TS 26.131 [1] is not attributed to the UE delay. This delay shall be included in the test equipment delay when deriving the UE delay from the delay measurments based on the cross-correlation of the injected and measured signals. Values for the compensation attributed to the packet delay profiles are shown in Table E.3.

Table E.3: Packet delay profiles compensation values.

| Delay and Loss profile | Compensation value |
| --- | --- |
| dly_profile_20msDRX_10pct_BLER_e2e | 30 ms |
| dly_profile_new | TBD |
| dly_profile_40msDRX_22pct_BLER_e2e | 30 ms |

Change 3 (Attachment)

Include in delay_and_loss_profiles.zip the new profile attached to the present Tdoc