3GPP TSG-SA4 Meeting #122	S4-230354

Athens, Greece, February 20 - 24, 2023    			(Rev 1 S4-230053)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.204 | CR | 0021 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Undefined Behaviour caused by out-of-bounds pointer arithmetic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 34004 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A common pattern is used in the code to obtain offset pointers on arrays, which is valid and legal as long as it points to an element in the array or one past the last one. Our UBSAN-enabled tests caught a few cases when the offset is negative, causing an UB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR corrects the UB by obtaining the offset and passing it around alongside the original array to avoid out-of-bounds pointer arithmetic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An Undefined Behaviour in the code, UBSAN failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | c-code/enc_acelp.c c-code/enc_gain.c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start change 1 ***

--- enc_acelp.c	2009-11-25 18:54:04.000000000 -0800

+++ enc_acelp.c	2022-07-22 20:43:45.000000000 -0700

@@ -1145,6 +1145,7 @@

Float32 val;

Float32 s, cor;

Float32 *p0, *p1, *p2, *p3, *psign;

+   Word32 p0_offset;

Float32 *h, *h_inv, *ptr_h1, *ptr_h2, *ptr_hf;

switch (nbbits)

@@ -1369,6 +1370,7 @@

pos = 256 - 1;

ptr_hf = h + 1;

+   p0_offset = -16;

for(k = 0; k < 16; k++)

{

@@ -1376,7 +1378,7 @@

p3 = &rrixiy[2][pos];

p2 = &rrixiy[1][pos];

p1 = &rrixiy[0][pos];

-      p0 = &rrixiy[3][pos - 16];

+      p0 = &rrixiy[3][pos];

cor = 0.0F;

ptr_h1 = h;

@@ -1399,7 +1401,7 @@

cor += (*ptr_h1) * (*ptr_h2);

ptr_h1++;

ptr_h2++;

-         *p0 = cor;

+         *(p0 + p0_offset) = cor;

p3 -= (16 + 1);

p2 -= (16 + 1);

--- enc_gain.c	2018-01-19 15:20:47.000000000 -0800

+++ enc_gain.c	2022-07-22 20:43:45.000000000 -0700

@@ -448,8 +448,8 @@

*    void

*/

static void E_GAIN_norm_corr(Float32 exc[], Float32 xn[], Float32 h[],

-                             Word32 t_min, Word32 t_max, Float32 corr_norm[])

-{

+                             Word32 t_min, Word32 t_max, Float32 corr_norm[],

+                             Word32 corr_offset) {

Float32 excf[L_SUBFR];  /* filtered past excitation */

Float32 alp, ps, norm;

Word32 t, j, k;

@@ -482,7 +482,7 @@

/* Normalize correlation = correlation * (1/sqrt(energy)) */

-      corr_norm[t] = ps * norm;

+      corr_norm[corr_offset + t] = ps * norm;

/* update the filtered excitation excf[] for the next iteration */

@@ -566,7 +566,7 @@

{

Float32 corr_v[15 + 2 * L_INTERPOL1 + 1];

Float32 cor_max, max, temp;

-   Float32 *corr;

+   Word32 corr_v_offset;

Word32 i, fraction, step;

Word32 t0, t_min, t_max;

@@ -575,21 +575,20 @@

t_min = t0_min - L_INTERPOL1;

t_max = t0_max + L_INTERPOL1;

-   /* allocate memory to normalized correlation vector */

-   corr = &corr_v[-t_min];      /* corr[t_min..t_max] */

+   corr_v_offset = -t_min;

/* Compute normalized correlation between target and filtered excitation */

-   E_GAIN_norm_corr(exc, xn, h, t_min, t_max, corr);

+   E_GAIN_norm_corr(exc, xn, h, t_min, t_max, corr_v, corr_v_offset);

/*  find integer pitch */

-   max = corr[t0_min];

+   max = corr_v[corr_v_offset + t0_min];

t0  = t0_min;

for(i = t0_min + 1; i <= t0_max; i++)

{

-      if( corr[i] > max)

+      if( corr_v[corr_v_offset + i] > max)

{

-         max = corr[i];

+         max = corr_v[corr_v_offset + i];

t0 = i;

}

}

@@ -621,11 +620,12 @@

fraction = 0;

}

-   cor_max = E_GAIN_norm_corr_interpolate(&corr[t0], fraction);

+   cor_max =

+       E_GAIN_norm_corr_interpolate(&corr_v[corr_v_offset + t0], fraction);

for (i = (fraction + step); i <= 3; i += step)

{

-      temp = E_GAIN_norm_corr_interpolate(&corr[t0], i);

+      temp = E_GAIN_norm_corr_interpolate(&corr_v[corr_v_offset + t0], i);

if (temp > cor_max)

{

*** End change 1 ***