Source:	Nokia Corporation

Title:	Proposals for IVAS Codec selection test plan – MASA, 5.1, Objects

Document for:	Discussion and Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Recent inputs, e.g., [1], discuss specifics of IVAS Codec selection testing.

The source has recently experimented with P.800 Supplement [2] DCR testing of MASA format inputs and BS.1534 [3] MUSHRA testing of 5.1 multi-channel and Object format inputs. In this input document, we make proposals for the corresponding IVAS Codec selection test experiments.

# 2. Overview

In IVAS Codec selection, there are planned two P.800 Supplement tests with MASA input format highlighted in the table below: experiments P800-8 and P800-9.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P800-1 | Stereo | Clean speech | Headphones | ≤ 48 | ≤ 3% | Y | No |
| P800-2 | Stereo | Speech+Background | Headphones | ≤ 48 | ≤ 3% | Y | No |
| P800-3 | Stereo | Mixed & Music | Headphones | ≤ 48 | ≤ 3% | N | No |
| P800-4 | FOA | Clean speech | Headphones | ≤ 96 | ≤ 3% | Y |  |
| P800-5 | FOA | Speech+Background | Headphones | ≤ 96 | ≤ 3% | Y |  |
| P800-6 | 1 Object | Clean speech | Headphones | ≤ 24 | ≤ 3% | Y |  |
| P800-7 | 2 Objects | Clean speech | Headphones | ≤ 48 | ≤ 3% | Y |  |
| P800-8 | MASA | Clean speech | Headphones | ≤ 96 | ≤ 3% | Y |  |
| P800-9 | MASA | Speech+Background | Headphones | ≤ 96 | ≤ 3% | Y |  |

In IVAS Codec selection, there are planned two MUSHRA tests with 5.1 input format highlighted in the table below: experiments BS1534-2a and BS1534-2b using Generic Audio.

In IVAS Codec selection, there are furthermore planned two MUSHRA tests with Objects highlighted in the table below: BS1534-6a and BS1534-6b using Generic Audio.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS1534-1a | Stereo | Generic Audio | Headphones |  | ≤ x% | N | No |
| BS1534-1b | Stereo | Generic Audio | Headphones |  | ≤ x% | N | No |
| BS1534-2a | 5.1 | Generic Audio | 5.1 |  | ≤ x% | N | No |
| BS1534-2b | 5.1 | Generic Audio | 5.1 |  | ≤ x% | N | No |
| BS1534-3a | 7.1.4 | Generic Audio | 7.1 + 4 |  | ≤ x% | N | No |
| BS1534-3b | 7.1.4 | Generic Audio | 7.1 + 4 |  | ≤ x% | N | No |
| BS1534-4a | FOA | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-4b | FOA | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-5a | HOA3 | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-5b | HOA3 | Generic Audio | 7.1 + 4 |  | ≤ x% | N | No |
| BS1534-6a | Objects | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-6b | Objects | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-7a | MASA | Generic Audio | Headphones |  | ≤ x% | N |  |
| BS1534-7b | MASA | Generic Audio | Headphones |  | ≤ x% | N |  |

The present input contribution makes first proposals for these experiments.

# 3. Discussion

Some observations and considerations are made relating, e.g., to size of each experiment. In particular, the P.SUPPL800 DCR experiments can have 36 test conditions and the BS.1534 MUSHRA experiments can have 8 test conditions:

- In P800-8/9 experiments, there is not sufficient room to test all conditions.

  - It is proposed to focus on stereo-MASA in IVAS Codec selection.

  - In P800-8, it is proposed to test CuT conditions only up to 80 kbps.

  - In P800-8 and P800-9, the DTX and FER conditions are selected to give a broad coverage of the bitrates.

  - In P800-8/9, towards the lower bitrates, codec references based on multi-EVS coding of Ambisonics inputs are preferred over codec references based on unquantized metadata as the latter can be considered to define an upper limit of quality at the higher bitrates (where EVS becomes transparent for clean speech).

  - It is proposed to limit number of MNRUs to 4 and consider limiting number of ESDRUs to 3.

- In BS1534-2a/b experiments, as also documented in [3], only 2 CuT conditions can be “fully tested” against the requirements. We note that 3 CuT conditions could be considered such that at least one of the CuT conditions does not have both NWT and BT codec references tested in the experiment.

  - BS1534-6a/b experiments allow for 3 CuT conditions to be tested. We propose to take advantage of it.

- In BS1534 experiments, as also documented in [4], only 2 references/anchors can generally be fit: Hidden reference + 1 low-pass anchor condition.

  - Recommendation BS.1534 [3] specifies using at least two hidden anchors: the standard anchor is a low-pass filtered version of the original signal with a cut-off frequency of 3.5 kHz, and the mid quality anchor has a cut-off frequency of 7 kHz.

- It could be useful to consider all experiments for channel-based input (BS1534-2a/b and BS1534-3a/b) together. For example, one pair of experiments could focus on covering a larger range of bitrates, whereas the other could focus on covering clean channel and FER performance. In this proposal, two alternative options are therefore proposed.

- There is ongoing discussion on appropriate error rate for IVAS FER tests. We have used 3% across the experiments, however 5% could also be considered.

# 4. Proposals for specific experiments

## 4.1 P800-8 (MASA – Clean speech)

It is proposed to focus on stereo-MASA inputs in IVAS Codec selection tests.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 7 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48, 64, 80 kbps with DTX off at 0% FER |
|  | 5 | IVAS candidate operated at 13.2, 16.4, 24.4, 48, 64 kbps with DTX off at 3% FER |
| Codec references |  |  |
| Codec references | 6 | Multi-mono EVS operated at 3*7.2, 4*7.2, 4*8, 4*9.6, 4*16.4, 4*24.4 kbps with DTX off at 0% FER |
|  | 5 | Multi-mono EVS with unquantized metadata operated at 2*7.2, 2*8, 2*9.6, 2*16.4, 2*24.4 kbps with DTX off at 0% FER |
|  | 5 | Multi-mono EVS operated at 3*7.2, 4*7.2, 4*8, 4*16.4, 4*24.4 kbps with DTX off at 3% FER |
| Other references |  |  |
| Direct | 1 | Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 4 | Q = 32, 27, 22, 17 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.85, 0.65, 0.45 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to [tbd] |
| Binaural rendering | 1 | IVAS MASA C Reference Software binaural rendering [S4-230221] |
| Audio sampling frequency / bandwidth | 3 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | [Spatial scenes tbd] |
| Number of talkers | 6 | Sentence pairs uttered by different talkers (one each of 3 male and 3 female talkers) |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [P Suppl. 29] |
| Languages | 2 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

## 4.2 P800-9 (MASA – Speech + Background)

It is proposed to focus on stereo-MASA inputs in IVAS Codec selection tests.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 7 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48, 64, 80 kbps with DTX off at 0% FER |
|  | 6 | IVAS candidate operated at 13.2, 16.4, 24.4, 32, 48, 64 kbps with DTX on at 0% FER |
| Codec references |  |  |
| Codec references | 6 | Multi-mono EVS operated at 3*7.2, 4*7.2, 4*8, 4*9.6, 4*16.4, 4*24.4 kbps with DTX off at 0% FER |
|  | 3 | Multi-mono EVS with unquantized metadata operated at 2*7.2, 2*9.6, 2*16.4 kbps with DTX on at 0% FER |
|  | 6 | Multi-mono EVS operated at 3*7.2, 4*7.2, 4*8, 4*9.6, 4*16.4, 4*24.4 kbps with DTX on at 0% FER |
| Other references |  |  |
| Direct | 1 | Nominal input level |
| P.50 MNRU (applied to MASA transport streams) | 4 | Q = 32, 27, 22, 17 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 3 | α = 0.85, 0.65, 0.45 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to [tbd] |
| Binaural rendering | 1 | IVAS MASA C Reference Software binaural rendering [S4-230221] |
| Audio sampling frequency / bandwidth | 3 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | [Spatial scenes with associated background noises tbd] |
| Number of talkers | 6 | Sentence pairs uttered by different talkers (one each of 3 male and 3 female talkers) |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to [P Suppl. 29] |
| Languages | 2 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

## 4.3 BS1534-2a (5.1 – Generic Audio, 5.1 Setup)

According to present proposal, BS1534-2a focuses on lower bit rates (up to 96 kbps) and BS1534-2b focuses on higher bit rates (starting at 128 kbps). Alternatively, one experiment focuses on clean channel while the other focuses on FER performance.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated at 64, 96 kbps at 0% FER |
| Codec references |  |  |
| Codec references | 3 | Multi-mono EVS operated at 5*13.2, 5*16.4, 5*24.4 kbps with the LFE channel processed using EVS operated at 9.6 kbps NB (for all Codec references) at 0% FER |
| Other references |  |  |
| Reference / Direct | 1 | Direct 5.1 signal, nominal level |
| Hidden Reference | 1 | Direct 5.1 signal, nominal level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct 5.1 signal, nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Loudspeaker Rendering | 1 | 5.1 channels direct playback |
| Audio sampling frequency / bandwidth | 2 | 48 kHz / maximum available audio bandwidth (SWB, FB) |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12-16] | Experienced Listeners |
| Randomizations | [12-16] | Individual per listener |
| Rating Scale | 1 | Continuous MUSHRA scale: 0-100 [ITU-R BS.1534] |
| Listening System | 1 | 5.1 high-quality loudspeaker setup |
| Listening Environment | 1 | No room noise |

## 4.4 BS1534-2b (5.1 – Generic Audio, 5.1 Setup)

According to present proposal, BS1534-2a focuses on lower bit rates (up to 96 kbps) and BS1534-2b focuses on higher bit rates (starting at 128 kbps). Alternatively, one experiment focuses on clean channel while the other focuses on FER performance.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | Option 1: IVAS candidate operated at 128, 160 kbps at 0% FER Option 2: IVAS candidate operated at 64, 96 kbps at 3% FER |
| Codec references |  |  |
| Codec references | 3 | Option 1: Multi-mono EVS operated at 4*24.4, 5*32, 5*48 kbps with the LFE channel processed using EVS operated at 9.6 kbps NB (for all Codec references) at 0% FER Option 2: Multi-mono EVS operated at 5*13.2, 5*16.4, 5*24.4 kbps with the LFE channel processed using EVS operated at 9.6 kbps NB (for all Codec references) at 3% FER |
| Other references |  |  |
| Reference | 1 | Direct 5.1 signal, nominal input level |
| Hidden Reference | 1 | Direct 5.1 signal, nominal input level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct 5.1 signal, nominal level |
| Common Conditions |  | (see BS1534-2a for full list) |
| Audio sampling frequency / bandwidth | 1 or 2 | Option 1: 48 kHz / FB Option 2: 48 kHz / maximum available audio bandwidth (SWB, FB) |

## 4.5 BS1534-6a (Objects – Generic Audio, Binaural)

It is proposed to test with 3 Objects in BS1534-6a at mid bitrates and with 4 Objects in BS1534-6b at mid/high bitrates.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 3 | IVAS candidate operated at 48, 64, 96 kbps |
| Codec references |  |  |
| Codec references | 3 | Multi-mono EVS operated at 3*16.4, 3*24.4, 3*32 kbps |
| Other references |  |  |
| Reference / Direct | 1 | Direct Object signal(s), nominal input level |
| Hidden Reference | 1 | Direct Object signal(s), nominal input level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct Object signal(s), nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Binaural rendering | 1 | Objects to binaural (external) rendering according to [tbd] |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12-16] | Experienced Listeners |
| Randomizations | [12-16] | Individual per listener |
| Rating Scale | 1 | Continuous MUSHRA scale: 0-100 [ITU-R BS.1534] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

## 4.6 BS1534-6b (Objects – Generic Audio, Binaural)

It is proposed to test with 3 Objects in BS1534-6a at mid bitrates and with 4 Objects in BS1534-6b at mid/high bitrates. Alternatively, one experiment can focus on clean channel performance while the other focuses on DTX or FER performance.

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 3 or 1 | Option 1: IVAS candidate operated at 96, 160, 256 kbps with DTX off at 0% FER Option 2 and Option 3: IVAS candidate operated at 96 kbps with DTX off at 0% FER |
|  | 0 or 1 | Option 2: IVAS candidate operated at 96 kbps with DTX on at 0% FER Option 3: IVAS candidate operated at 96 kbps with DTX off at 3% FER |
| Codec references |  |  |
| Codec references | 3 or 1 | Option 1: Multi-mono EVS operated at 4*24.4, 4*48, 4*64 kbps with DTX off at 0% FER Option 2 and Option 3: Multi-mono EVS operated at 4*24.4 kbps with DTX off at 0% FER |
|  | 0 or 1 | Option 2: Multi-mono EVS operated at 4*24.4 kbps with DTX on at 0% FER Option 3: Multi-mono EVS operated at 4*24.4 kbps with DTX off at 3% FER |
| Other references |  |  |
| Reference / Direct | 1 | Direct Object signal(s), nominal input level |
| Hidden Reference | 1 | Direct Object signal(s), nominal input level |
| LP7 anchor | 1 | 7kHz lowpass filtered direct Object signal(s), nominal level |
| Common Conditions |  | (see BS1534-6a for full list) |

# 5. Summary

We have presented some Test plan specifics and options for discussion in clause 4 of this input. The proposals cover two P.SUPPL800 DCR experiments and four BS.1534 MUSHRA experiments. These proposals can be, e.g., considered as input for IVAS Codec selection Test plan editing sessions or considered for agreement in Test plan as such.

# References

[1]	S4aA230049: “Suggested sample test plans for IVAS Codec selection testing with SBA content”, Dolby Laboratories Inc.

[2]	ITU-T P.800 – Use cases, P Suppl. 29 (01/2023)

[3]	ITU-R BS.1534-3 (10/2015) – Method for the subjective assessment of intermediate quality level of audio systems

[4]	SA4-230453: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4