3GPP TSG-SA4 Meeting #125	S4-231168

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.532 | CR | 0005 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [TEI18, ADAE, EVEX] UE Application instructing DDCC for immediate data report delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, ADAE, EVEX |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During discussion with SA6 (in LS Reply: S4-231109), following was identified: “The UE Application can send a report to the Direct Data Collection Client using the reportUeData method defined by TS 26.532 at reference point R7 (the UE client API). At present, this method does not specify a means to instruct the Direct Data Collection Client to override its configuration and send a UE data report immediately. However, SA4 believes that this API method could straightforwardly be enhanced to specify a parameter (e.g. expedite) that instructs the Direct Data Collection Client to prioritise immediate delivery of a UE data report to the Data Collection AF. Furthermore, SA4 believes that the DataReport container type specified in TS 26.532 could be enhanced to include an expedited flag that instructs the Data Collection AF to prioritise the processing of the UE data report and exposure of the resulting event to downstream subscribers, such as the Application Service Provider's Event Consumer AF. To this purpose, SA4 welcomes Release 18 co-ordination Change Requests to TS 26.531 and TS 26.532 from interested Individual Members at its forthcoming meetings, appropriately tagged with SA6 Work Item codes.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes enhance the data reporting procedure (clause 4.4.4) and DataReport type (clause 7.3.2.3) to enable UE application to instruct the Direct Data Collection Client to prioritise immediate delivery of a UE data report to the Data Collection AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA6 provided requirements will not be specified in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4, 7.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.4.4	Data reporting procedure

The UE Application reports data to the Direct Data Collection Client by invoking the reportUeData method on the Direct Data Collection Client at reference point R7.

As a consequence, the Direct Data Collection Client may report the UE data provided by invoking the procedure specified in clause 4.3.3. Depending on the Data Reporting Configuration, the Direct Data Collection Client may instead store the UE data and forward it later. If UE Application has instructed to prioritise immediate delivery of a UE data report using expediteReq, the Direct Data Collection Client forwards the UE data by invoking the procedure specified in clause 4.3.3.

Second CHANGE

#### 7.3.2.3	DataReport type

Table 7.3.2.3-1: Definition of DataReport type

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| externalApplicationId | ApplicationID | 1 | External application identifier. |
| expediteReq | boolean | 0..1 | If true, a report is triggered immediately to Data Collection AF. |
| serviceExperienceRecords | array(ServiceExperienceRecord) | 0..1 (see NOTE) | See clause A.2. |
| locationRecords | array(LocationRecord) |  | See clause A.3. |
| communicationRecords | array(CommunicationRecord) |  | See clause A.4. |
| performanceDataRecords | array(PerformanceDataRecord) |  | See clause A.5. |
| applicationSpecificRecords | array(ApplicationSpecificRecord) |  | See clause A.6. |
| tripPlanRecords | array(TripPlanRecord) |  | See clause A.7. |
| mediaStreaming‌Access‌Records | array(MediaStreaming‌AccessRecord) |  | See TS 26.512 [13] clause 17.2. |
| NOTE: Exactly one of these properties must be present in a DataReport. |  |  |  |

END OF CHANGES