3GPP TSG-SA4 Meeting #125	S4-231209

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0046 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_Pro_Ph2] Service URL Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 26.804 and TS 26.501 provides details on Service URL Handling. Specifications for the 3GPP Service Handler and URL including the necessary functions on UE and device to support automatic launch of 5G System services in the context of 5G Media Streaming based on the conclusions in clause 6.13 of TR26.804. The work item objectives state 9) Specifications for the 3GPP Service Handler and URL including the necessary functions on UE and device to support automatic launch of 5G System services in the context of 5G Media Streaming based on the conclusions in clause 6.13 of TR 26.804. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify a concrete URL format for 3GPP services with individual host names registered in the 3GPP-managed domain launch.3gppservices.org as part of 3GPP specifications, and ensure that this can be used in the context of 3GPP-based services, namely: - Specify a suitable website redirection mechanism in case a suitable 3GPP Service Handler is not already installed on the device to handle the URL. The URL itself needs to be sufficiently unambiguous to resolve to the service entry point URL and may embed the service entry point URL as well. - Specify 3GPP Service URL instantiations with parameters suitable for launching at least the Media Session Handler for 5G Media Streaming, MBMS Client (including Receive-Only Mode services) and MBS Client. - Provide the ability for a 5GMS-Aware Application to create a Service URL in order to bootstrap 5G Media Streaming. - Investigate and study the application of 3GPP services and URL handling beyond 5G Media Streaming. - Specify an external service identifier assigned by the 5GMS Application Provider that can be used to launch 5G Media Streaming sessions on different 5GMS Systems via a common 3GPP Service URL. - Modify the Provisioning Session resource to include one (or more) of these identifiers. - Modify the Service Access Information API to accept this instead of a Provisioning Session identifier. - Specify a default 5GMS AF host name to be used by the Media Session Handler when the 3GPP Service URL does not specify one. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1, 11.2.2, X(New), Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Service URL Handling is eventually a candidate to be added to a common specification in 26.51x Session Handling This is really a starting point, details need to be progressed. Until the next meeting, an implementation in the 5G-MAG Reference Tools is planned. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# =====  CHANGE =====

#### 7.2.3.1	ProvisioningSession resource

The data model for the ProvisioningSession resource is specified in Table 7.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 7.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RO U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. | All types. |
| externalApplicationId | ApplicationId | 1..1 | C: RW R: RO U: RO | The external application identifier (see TS 29.571 [12]), nominated by the 5GMS Application Provider, to which this Provisioning Session pertains. | All types. |
| serverCertificateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Server Certificate identifiers currently associated with this Provisioning Session. | downlink |
| contentPreparation‌TemplateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Content Preparation Template identifiers currently associated with this Provisioning Session. | downlink, uplink |
| metricsReporting‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |
| policyTemplateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Policy Template identifiers currently associated with this Provisioning Session. | downlink, uplink |
| edgeResources‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Edge Resources Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |
| eventDataProcessing‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Event Data Processing Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |

# =====  CHANGE =====

### 11.2.2	Resource structure

The Service Access Information API is accessible through the following URL base path:

{apiRoot}/3gpp-m5/{apiVersion}/service-access-information/{external_service_id}

The operations and the corresponding HTTP methods in Table 11.2.2-1 are supported. In each case, the sub-resource path specified in the second column shall be appended to the URL base path.

Table 11.2.2-1: Operations supported by the Service Access Information API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information | {provisioningSessionId} | GET | Used to acquire the Service Access Information resource for the specified Provisioning Session. The {provisioningSessionId} uniquely identifies the Service Access Information Resource and is allocated by the 5GMS AF during creation of a Provisioning Session. |

# =====  CHANGE (NEW) =====

# X	Service URL: Definition and URL Handling

## X.1	Introduction

An 3GPP Service URL identifies a service that is made available over a 3GPP system and uses the Media Session Handler as defined in TS26.501. The intent of 3GPP Service URL handling is to launch UE functions based on the execution of a URL. This enables 5G Media services to be announced within a third-party application, a general web page, a messaging service or shared via social messages using a 3GPP Service URL for 5GMS. When a service is launched using a 3GPP Service URL for 5G Media Delivery, a Meida Client function for media session handling is expected to be launched implicitly alongside, for example, a primary media stream handling function.

For details on the architecture refer to TS 26.501, clause 4.10. Baseline procedures are provided in clause 9 of TS 26.501.

## X.2	URL Structure

The 3GPP Service URL format used to initiate 5G Media Streaming sessions takes the following form:

http[s]://ms.launch.3gppservices.org/service-id/{external_service_id}?{query_parameters}

The formal ABNF, following the syntax of RFC 8820 (HTTP URLs) [X] and RFC 3986 (URLs) [11]:

3gpp-service-URI = "http:" "//" authority path-abempty 
	*( "&" mid-label "=" mid-value ) 
	[ "&label=" resourceURI ]
authority        = <authority, see [RFC3986], Section 3.2>
path-abempty     = <path-abempty, see [RFC3986], Section 3.3>
mid-label        = ALPHA *( ALPHA / DIGIT )
mid-value        = 1*uchar
        =  unreserved / pct-encoded / ";" / "?" / ":" / 
	"@" / "=" / "+" / "$" / "," / "/" 
unreserved       = <unreserved, see [RFC3986], Section 2.3>
pct-encoded      = <pct-encoded, see [RFC3986], Section 2.1>
resourceURI      = <URI, see [RFC3986], Section 3>

The parameters of the 3GPP Service URL are defined in Table X.2-1.

Table X.2-1: 3GPP Service URL parameters

| Path element | Cardinality | Description |
| --- | --- | --- |
| external_service_id | 1 | An External Service Identifier that resolves to a Provisioning Session in the 5GMS System (see NOTE). |
| Query parameter | Cardinality | Description |
| af-host-address | 0..* | The Fully Qualified Domain Name and optional port number of a 5GMS AF endpoint to be used by the Media Session Handler at reference point M5 with the format hostname[:port]. More than one occurrence of this parameter may be present in the Service URL to indicate alternative host endpoint addresses. Any of these may be used by the Media Session Handler at reference point M5. Supplied by the invoking 5GMS-Aware Application when the 5GMS AF is deployed in an External DN. The endpoint address(es) may, for example, have been passed to the 5GMS-Aware Application via reference point M8. If omitted, the Media Session Handler assumes the default 5GMS AF host endpoint address ms.af.3gppservices.org:443 is to be used at reference point M5. |
| media-entry-point | 0..1 | A Media Entry Point reference expressed as a fully qualified URL at reference point M4. If supplied, used by the Media Session Handler to launch the Media Stream Handler (Media Player or Media Streamer) after successfully initiating media session handling. |
| content-type | 0..* | A MIME content type string conforming to section 5 of RFC 2045 [103] identifying a type of Media Entry Point that is acceptable to the Media Stream Handler (Media Player or Media Streamer). More than one occurrence of this parameter may be present in the Service URL to indicate that more than one type of Media Entry Point is acceptable. Used by the Media Session Handler to eliminate unacceptable Media Entry Points from those listed in the Service Access Information. It is an error to supply this parameter if an explicit Media Entry Point is specified using media-entry-point. |
| NOTE: Addition to table 7.2.3.1-1 of TS 26.512 [16]. Several different External Application Identifiers may be associated with a Provisioning Session, but a given External Application Identifier is associated with at most one Provisioning Session in each 5G System, ensuring unique resolution. |  |  |

The external_service_id path element and af-host query parameter correspond to the baseline Service Announcement for 5G Media Streaming in this specification. Together, they enable a full set of Service Access Information to be retrieved by the Media Session Handler from the 5GMS AF using the M5 Service Access Information API.

The media-entry-point parameter is used to support the procedure where the Media Session Handler launches media playback in the Media Stream Handler (Media Player or Media Streamer) after successfully retrieving a full set of Service Access Information via reference point M5 (if needed) and after successfully initiating media session handling.

The remaining parameters are used for client-side filtering of Media Entry Point information provided in the Service Access Information and selection of one Media Entry Point by the Media Session Handler. (They are mutually exclusive with the media-entry-point parameter.) In this case, media playback by the Media Stream Handler (Media Player or Media Streamer) is launched by the Media Session Handler with its chosen Media Entry Point.

If the 5GMS-Aware Application wants to launch media streaming itself (rather than have the Media Session Handler launch media streaming on its behalf), the media-entry-point query parameter and all client-side filtering parameters are omitted from the Service URL. In this case, the Media Session Handler only initiates media session handling for the 5GMS Provisioning Session identified by the External Application Identifier.

NOTE 1: 	The list of parameters may be extended to allow a 5GMS-Aware Application to pass a complete set of Service Access Information (previously acquired by it via reference point M8) to the Media Session Handler, thereby avoiding the need for it to invoke the M5 Service Access Information API on the 5GMS AF. However, since not all parameters available in M5 Service Access Information are provided back to the 5GMS Application Provider by the M1 Provisioning APIs, a fully comprehensive solution may not be possible.

NOTE 2: 	The Provisioning Session ID is different for every 5GMS System, and it is subject to change in a given 5GMS System if 5GMS services are reprovisioned. Both facets of the Provisioning Session ID are therefore inconvenient for a 5GMS-Aware Application that is intended to work on different 5GMS Systems. Using a stable, cross-system identifier to retrieve Service Access Information is preferred.

The Media Session Handler performs decomposition of the URI into the prefix, mid-part, and suffix.

## X.3	Procedures

### X.3.1	Assumptions

In order to address the cases in clause 5.13.4, it is proposed to:

-	Register a unique host name in the 3GPP-managed domain 3gppservices.org for each 3GPP service as part of 3GPP specifications such that service instances can be referenced under a controlled URL, e.g. http://ms.launch.3gppservices.org and https://ms.launch.3gppservices.org for 5G Media Streaming services.

-	Specify a website/redirection mechanism in case a suitable 3GPP Service Handler is not already installed on the UE to handle the URL.

- 	The URL itself needs to be constructed in such a way that it can be resolved to the media service, for example by embedding the media entry point URL.

-	Specify an application/service that realises the UE-based 3GPP Service Handler which declares an intent filter so that the application acts as the default handler for the 3GPPP Services domain name registered above. The intent filter includes the following declarations:

-	The DEFAULT category to allow the application to respond to implicit intents.

-	The BROWSABLE category in order for the intent filter to be invoked when clicking a link in a browser.

-	<data> tags including the android:scheme attribute for both HTTP and HTTPS.

-	A <data> tag including the android:host attribute with the registered 3GPP Services domain.

-	Verify ownership details see here.

An example intent filter is provided below:

<intent-filter android:autoVerify="true"> <action android:name="android.intent.action.VIEW" /> <category android:name="android.intent.category.DEFAULT" /> <category android:name="android.intent.category.BROWSABLE" /> <data android:scheme="http" /> <data android:scheme="https" /> <data android:host=" launch.3gppservices.org " /> </intent-filter>

### X.3.2	General Behaviour

Based on this setup, the following set of actions and calls happen:

1.	A 3GPP Service Handler background service is pre-installed on the UE by the vendor or installed once by the user. The manifest includes an intent filter that declares an interest in the 3GPP Services domain.

2. 	The Media Service Provider has a service with an entry point URL.

3.	The Media Service Provider negotiates all service parameters with the 5G System (provisioning).

4.	The Media Service Provider creates a bootstrapping URL (much like a landing page URL).

-	http://<service type identifier>.launch.3gppservices.org/<service parameters>

-	https://<service type identifier>.launch.3gppservices.org/<service parameters>

Example service type identifiers could include:

-	ms with a service parameter that identifies or resolves to a 5GMS Provisioning Session to indicate the necessity to initiate media session handling for 5G Media Streaming.

-	mbms with MBMS parameters for Receive-Only Mode operation to indicate the necessity to launch the MBMS Client in order to initiate reception of a MBMS User Service.

-	mbs with an MBS Service identifier to indicate the necessity to launch the MBS Client in order to initiate reception of a 5G Multicast–Broadcast User Service.

-	ims to indicate use of the IP Multimedia Subsystem.

-	etc.

Service parameters are formatted as path elements and/or query parameters and could include:

-	A service identifier that is preferably globally unique so that the same value can be used in different 5G Systems.

-	The service entry point URL of an application service to be launched by the UE-based 3GPP Service Handler.

5.	The Media Service Provider provides:

-	Only the 3GPP Service URL to a Portal Service Provider, search engine, etc., if the service requires 3GPP service launch. This may embed the service entry point URL.

-	Both the service entry point URL and the 3GPP Service URL, if the 3GPP service is only an enhancement to launch the third-party service.

6.	The Portal Service Provider provides these URLs to the Portal Application running on the UE, e.g. as part of a script, for display in a web page etc.

When the service is selected by the user, the Portal Application obtains or generates a 3GPP Service URL and submits a request for it.

7.	If a 3GPP Service Handler for the requested service type identifier is already installed on the UE, it is invoked in the background by the mobile Operating System via its registered intent filter to handle the URL and:

-	Uses the service parameters to establish the 3GPP service in the background (if appropriate or available) and potentially connects to the network.

-	Extracts the media service entry point URL from the 3GPP Service URL  and uses it to the Media Service Application.

-	Establish Inter-Process Communication with other UE functions, as required.

-	The Media Session Handler (Media Player or Media Streamer) may be launched by the 3GPP Service Handler, if separate from the Media Service Application.

8.	If a 3GPP Service Handler for the requested service type identifier is not yet installed on the UE:

- 	A network service is called and runs the 3GPP Service URL resolution. In the simplest case, this resolution, included in the MNO's network, redirects to the media service or

- 	An HTTP 404 client error response is returned by the UE Operating System and the Portal Application either terminates the service or uses the alternate over-the-top entry point.

### X.3.2	Launch of 5G Media Streaming service by 5GMS-Aware Application

With reference to clause X.3.1, this clause a format for a 3GPP Service URL that can be used by a 5GMS-Aware Application (playing the dual role of Portal Application and Media Service Application) to launch a 5G Media Streaming Session using an external service identifier proposed in clause 5.13.6.2.3. The proposal is aligned with clause B.1 of TS 26.501 [15] which describes a DNS-based solution to MNO-specific Service Access Information acquisition.

In this proposal, the 5GMS Media Session Handler plays the role of 3GPP Service Handler. A procedure for handling the 5GMS Service URL in the Media Session Handler is proposed in clause 5.13.6.2.4.

The Media Session Handler plays the role of 3GPP Service Handler for 5GMS Service URLs by registering itself as the URL handler for the domain name ms.launch.3gppservices.org (e.g., by declaring an intent filter in an Android application manifest). Hence, the Media Session Handler is launched when a 5GMS-Aware Application requests a 5GMS Service URL (e.g., by means of an Android intent filter).

Based on the information supplied in the 5GMS Service URL, the Media Session Handler forms the M5 request URL for Service Access Informaiton as follows:

http[s]://{apiRoot}/3gpp-m5/service-access-information/{external_service_id}

NOTE:	This implies a change to clause 11.2.2 in TS 26.512 [16] which currently specifies the use of the Provisioning Session ID to retrieve Service Access Information as the final path element of the request URL.

1.	If the Media Session Handler was invoked with one or more af-host-address query parameters, the Media Session Handler chooses one to substitute into {apiRoot} in the above request URL.

a.	This corresponds to collaboration scenarios where the 5GMS AF is deployed in the External DN.

b.	The party operating the 5GMS AF is responsible for ensuring that the hostname(s) resolve to the correct IP address(es) in the External DN.

c.	If the port number is omitted from any af-host-address, port 443 is assumed.

d.	If more than one af-host-address query parameter is supplied in the Service URL, the Media Session Handler may use an alternative host endpoint address at reference point M5 if the one it is using fails to respond after some implementation-specific number of retries.

2.	If the af-host-address query parameter is omitted, the default host name ms.af.3gppservices.org and port number 443 is used instead.

a.	This corresponds to collaboration scenarios where the 5GMS AF is deployed in the Trusted DN.

b.	The 5G System operator is responsible for supporting resolution of this well-known host name to the correct IP address(es) in the Trusted DN, e.g., by managing appropriate DNS records.

c.	The 5G System operator is responsible for ensuring that a resilient service is available at this host endpoint address. If the hostname resolves to multiple IP addresses, the Media Session Handler may use a different one at reference point M5 if the one it is using fails to respond after some implementation-specific number of retires.

d.	It is recognised that correct resolution of the hostname may be hampered if the end user configures an alternative DNS resolution service. Unless the Media Session Handler can override this and use the 5G System DNS resolution service, this is considered a failure case for 5G Media Streaming session initiation that is reportable to the end user.

3.	If the Media Session Handler was invoked with the optional media-entry-point query parameter, the Media Session Handler issues a request for this URL (thereby chaining the Media Stream Handler), but only after it has successfully retrieved the Service Access Information from the 5GMS AF.

Depending on the success (or otherwise) of retrieving the Service Access Information (and optionally chaining the Media Stream Handler with the media entry point), the Media Session Handler returns an appropriate HTTP status code to the invoking 5GMS-Aware Application.

## X.4	Examples

To be completed.

# =====  CHANGE (NEW) =====

Annex X (normative):
Registration of 3GPP Service URLs

# X.1	General

This Annex provides the 3GPP Service URL registration information for different systems.

# X.2	Android association of 3GPP Service URLs