Source:	HEAD acoustics GmbH

Title:	Global Analysis Laboratory report – IVAS Selection Phase

Document for:	Approval

Agenda Item:	7.5

# Introduction

This report presents the collected results of the listening tests for the IVAS Selection Phase and the statistical analysis with respect to the Terms of Reference (ToR) tests. As defined in IVAS-8a [1], these tasks were done by HEAD acoustics as the Global Analysis Laboratory (GAL).

The results are intended to be used in the selection procedure according to IVAS-5 [2].

# Listening Laboratories

The experiments were carried out by four listening laboratories (LL) as listed in Table 1, which also shows amount and test types conducted by each LL. The LL identifiers are used in the following analyses for reference and are identical to the ones defined in IVAS-7a [3].

Table 1: List of listening labs

| LL identifier | Listening Lab Company | P800 tests | BS1534 tests (headphone) | BS1534 tests (loudspeaker) |
| --- | --- | --- | --- | --- |
| a | FORCE Technology | 8 | 3 | 5 |
| b | HEAD acoustics GmbH / IKS (RWTH Aachen) | 3 | 6 | 5 |
| c | Macquarie University | 2 | 0 | 0 |
| d | Mesaqin.com | 5 | 9 | 0 |

# Analysis methods

For both test designs (P800 and BS1534), the ToR requirements in IVAS-8a [1] are defined in the same way. All codec-under-test (CuT) conditions have to perform either better than (BT) or not worse than (NWT) certain reference conditions. Due to the uncertainty of the auditory data, it is necessary to consider not only the averaged per-condition results, but also the variance/standard deviation and the number of votes. For this reason, Student's t-test for independent groups is evaluated on each result of the test and the corresponding reference condition. The t-statistic (referred as "T-Stat" in the following) is calculated without the assumption that the variances of test and reference are equal (Welch's t-test).

T-Stat is compared to t(M), the single-sided 95% confidence level of the inverse t-distribution, which depends on the degree of freedom M, i.e., the number of votes per condition. The check result R is then determined according to equation 1. Note that R might also show that the CuT is worse than (WT) the reference.

R=BT                     T-Stat >t(M)NWT-tM≤T-Stat ≤t(M)WT                T-Stat <-t(M)	(1)

Based on Section 3.3.6.2 of IVAS-8a, it is indicated which and how many requirements have to be met for each condition in each experiment. The tabular data shown in the following sections utilizes status flags for this purpose:

-	FAIL: requirement was not met (cell in tabular data marked in red)

-	PASS: requirement was met (cell in tabular data not marked)

-	EXCEED: requirement was met and is significantly better (cell in tabular data marked in blue).

Table 2 illustrates how the status is determined for each combination of requirement and check result.

Table 2: Status based on check result

| Requirement | Check result | Status |
| --- | --- | --- |
| NWT | WT | FAIL |
| NWT | NWT | PASS |
| NWT | BT | EXCEED |
| BT | WT | FAIL |
| BT | NWT | FAIL |
| BT | BT | PASS |

In some cases, a ToR requirement is defined as the logical disjunction (OR) of two separate checks, which lead to two status values. The resulting status with respect to the requirement is determined as defined in Table 3.

Table 3: Status disjunction

| Status #1 | Status #2 | Result |
| --- | --- | --- |
| EXCEED | EXCEED | EXCEED |
| EXCEED | PASS | PASS |
| EXCEED | FAIL | PASS |
| PASS | PASS | PASS |
| PASS | FAIL | PASS |
| FAIL | FAIL | FAIL |

# Results for P.800 DCR experiments

## Overview

These listening tests were conducted with naïve listeners by the four listening laboratories. Each experiment was done at two different listening laboratories with different languages, an overview on the experiments and their allocation can be found in Tables 3 and 5 in IVAS-8a [1].

In the following, the results of the statistical analyses are provided for the 18 experiments and the comparison with the ToR criteria is done. The confidence level t(M) for the statistical analyses is 1.653 due to the 180 votes per condition for these experiments.

For each experiment, the statistical results for the different languages are presented separately and the relation between the two results is illustrated by scatter plots of the averaged condition ratings.

The full results for each experiment are available as an electronic attachment to this document.

P800-[X]_[N][L].xlsx

With:

X: P800 experiment number as per IVAS-8a (e.g., X=3  P800-3).

N: Index of presentation order (1 or 2) used, see electronic attachments of IVAS-8a.

L: Lab identifier.

Example: P800-3_2d.xlsx (Lab d used presentation order 2 for P800-3)

Each Excel document includes three sheets:

-	Raw: The individual votes of each subject per listening sample.

-	Condition+Category: Raw results aggregated for each category and condition (30 votes).

-	Condition: Raw results aggregated for each condition (180 votes).

Standard deviations and confidence intervals for aggregated results were calculated according to Recommendation ITU-T P.1401 [4].

## P800-1 (Stereo, Clean speech)

The complete statistical evaluation of the requirement ToR tests for experiment P800-1 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 4: Statistical overview on the results of P800-1

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c25 | 1 | 13.2 | off |  | NWT | 3.93 | 1.08 | c10 | 2x8 | 3.45 | 1.06 | 4.27 | BT | EXCEED |
|  |  | 2 | 13.2 | off |  | BT | 3.93 | 1.08 | c09 | 2x7.2 | 3.31 | 1.15 | 5.33 | BT | PASS |
|  | c26 | 1 | 16.4 | off |  | NWT | 4.14 | 1.07 | c11 | 2x9.6 | 3.83 | 1.02 | 2.87 | BT | EXCEED |
|  |  | 2 | 16.4 | off |  | BT | 4.14 | 1.07 | c10 | 2x8 | 3.45 | 1.06 | 6.18 | BT | PASS |
|  | c27 | 1 | 24.4 | off |  | NWT | 4.41 | 0.9 | c12 | 2x13.2 | 4.38 | 0.76 | 0.32 | NWT | PASS |
|  |  | 2 | 24.4 | off |  | BT | 4.41 | 0.9 | c11 | 2x9.6 | 3.83 | 1.02 | 5.69 | BT | PASS |
|  | c28 | 1 | 32 | off |  | NWT | 4.61 | 0.66 | c13 | 2x16.4 | 4.57 | 0.72 | 0.61 | NWT | PASS |
|  |  | 2 | 32 | off |  | BT | 4.61 | 0.66 | c12 | 2x13.2 | 4.38 | 0.76 | 3.13 | BT | PASS |
|  | c29 | 1 | 48 | off |  | NWT | 4.72 | 0.68 | c15 | 2x32 | 4.67 | 0.62 | 0.73 | NWT | PASS |
|  |  | 2 | 48 | off |  | BT | 4.72 | 0.68 | c14 | 2x24.4 | 4.72 | 0.56 | 0.08 | NWT | FAIL |
|  | c30 | 1 | 13.2 | off | 5% | NWT | 3.33 | 1.22 | c17 | 2x8 | 2.88 | 1.14 | 3.58 | BT | EXCEED |
|  |  | 2 | 13.2 | off | 5% | BT | 3.33 | 1.22 | c16 | 2x7.2 | 2.83 | 1.17 | 3.97 | BT | PASS |
|  | c31 | 1 | 16.4 | off | 5% | NWT | 3.52 | 1.11 | c18 | 2x9.6 | 3.11 | 1.21 | 3.36 | BT | EXCEED |
|  |  | 2 | 16.4 | off | 5% | BT | 3.52 | 1.11 | c17 | 2x8 | 2.88 | 1.14 | 5.4 | BT | PASS |
|  | c32 | 1 | 24.4 | off | 5% | NWT | 3.74 | 0.99 | c19 | 2x13.2 | 3.62 | 1.12 | 1.09 | NWT | PASS |
|  |  | 2 | 24.4 | off | 5% | BT | 3.74 | 0.99 | c18 | 2x9.6 | 3.11 | 1.21 | 5.38 | BT | PASS |
|  | c33 | 1 | 32 | off | 5% | NWT | 3.9 | 1.09 | c20 | 2x16.4 | 3.64 | 1.07 | 2.3 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 3.9 | 1.09 | c19 | 2x13.2 | 3.62 | 1.12 | 2.43 | BT | PASS |
|  | c34 | 1 | 48 | off | 5% | NWT | 4.15 | 0.86 | c22 | 2x32 | 3.83 | 1.05 | 3.19 | BT | EXCEED |
|  |  | 2 | 48 | off | 5% | BT | 4.15 | 0.86 | c21 | 2x24.4 | 4.09 | 1.04 | 0.56 | NWT | FAIL |
|  | c35 | 1 | 24.4 | on |  | NWT | 4.4 | 0.84 | c23 | 2x13.2 | 4.23 | 0.85 | 1.87 | BT | EXCEED |
|  | c36 | 1 | 13.2 | on | 5% | NWT | 3.27 | 1.19 | c24 | 2x8 | 2.77 | 1.13 | 4.14 | BT | EXCEED |
| d | c25 | 1 | 13.2 | off |  | NWT | 3.5 | 0.99 | c10 | 2x8 | 2.07 | 0.84 | 14.75 | BT | EXCEED |
|  |  | 2 | 13.2 | off |  | BT | 3.5 | 0.99 | c09 | 2x7.2 | 2.1 | 0.92 | 13.9 | BT | PASS |
|  | c26 | 1 | 16.4 | off |  | NWT | 3.82 | 0.88 | c11 | 2x9.6 | 3.22 | 1.07 | 5.78 | BT | EXCEED |
|  |  | 2 | 16.4 | off |  | BT | 3.82 | 0.88 | c10 | 2x8 | 2.07 | 0.84 | 19.32 | BT | PASS |
|  | c27 | 1 | 24.4 | off |  | NWT | 4.21 | 0.86 | c12 | 2x13.2 | 3.82 | 0.97 | 4.09 | BT | EXCEED |
|  |  | 2 | 24.4 | off |  | BT | 4.21 | 0.86 | c11 | 2x9.6 | 3.22 | 1.07 | 9.67 | BT | PASS |
|  | c28 | 1 | 32 | off |  | NWT | 4.31 | 0.69 | c13 | 2x16.4 | 4.11 | 0.8 | 2.54 | BT | EXCEED |
|  |  | 2 | 32 | off |  | BT | 4.31 | 0.69 | c12 | 2x13.2 | 3.82 | 0.97 | 5.52 | BT | PASS |
|  | c29 | 1 | 48 | off |  | NWT | 4.56 | 0.63 | c15 | 2x32 | 4.32 | 0.77 | 3.24 | BT | EXCEED |
|  |  | 2 | 48 | off |  | BT | 4.56 | 0.63 | c14 | 2x24.4 | 4.38 | 0.68 | 2.51 | BT | PASS |
|  | c30 | 1 | 13.2 | off | 5% | NWT | 2.92 | 1.11 | c17 | 2x8 | 1.76 | 0.72 | 11.74 | BT | EXCEED |
|  |  | 2 | 13.2 | off | 5% | BT | 2.92 | 1.11 | c16 | 2x7.2 | 1.69 | 0.69 | 12.55 | BT | PASS |
|  | c31 | 1 | 16.4 | off | 5% | NWT | 3.23 | 1 | c18 | 2x9.6 | 2.64 | 1.06 | 5.48 | BT | EXCEED |
|  |  | 2 | 16.4 | off | 5% | BT | 3.23 | 1 | c17 | 2x8 | 1.76 | 0.72 | 16.05 | BT | PASS |
|  | c32 | 1 | 24.4 | off | 5% | NWT | 3.39 | 1.01 | c19 | 2x13.2 | 3.12 | 0.97 | 2.55 | BT | EXCEED |
|  |  | 2 | 24.4 | off | 5% | BT | 3.39 | 1.01 | c18 | 2x9.6 | 2.64 | 1.06 | 6.89 | BT | PASS |
|  | c33 | 1 | 32 | off | 5% | NWT | 3.6 | 0.96 | c20 | 2x16.4 | 3.29 | 1 | 2.97 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 3.6 | 0.96 | c19 | 2x13.2 | 3.12 | 0.97 | 4.69 | BT | PASS |
|  | c34 | 1 | 48 | off | 5% | NWT | 3.76 | 1.03 | c22 | 2x32 | 3.61 | 0.9 | 1.52 | NWT | PASS |
|  |  | 2 | 48 | off | 5% | BT | 3.76 | 1.03 | c21 | 2x24.4 | 3.76 | 0.87 | 0.05 | NWT | FAIL |
|  | c35 | 1 | 24.4 | on |  | NWT | 4.14 | 0.81 | c23 | 2x13.2 | 3.86 | 0.95 | 2.99 | BT | EXCEED |
|  | c36 | 1 | 13.2 | on | 5% | NWT | 2.97 | 1.03 | c24 | 2x8 | 1.82 | 0.74 | 12.22 | BT | EXCEED |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 5: Summary of the results of P800-1

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c25 | 13.2 | off |  | NWT c10 OR BT c09 | PASS |
|  | c26 | 16.4 | off |  | NWT c11 OR BT c10 | PASS |
|  | c27 | 24.4 | off |  | NWT c12 OR BT c11 | PASS |
|  | c28 | 32 | off |  | NWT c13 OR BT c12 | PASS |
|  | c29 | 48 | off |  | NWT c15 OR BT c14 | PASS |
|  | c30 | 13.2 | off | 5% | NWT c17 OR BT c16 | PASS |
|  | c31 | 16.4 | off | 5% | NWT c18 OR BT c17 | PASS |
|  | c32 | 24.4 | off | 5% | NWT c19 OR BT c18 | PASS |
|  | c33 | 32 | off | 5% | NWT c20 OR BT c19 | PASS |
|  | c34 | 48 | off | 5% | NWT c22 OR BT c21 | PASS |
|  | c35 | 24.4 | on |  | NWT c23 | EXCEED |
|  | c36 | 13.2 | on | 5% | NWT c24 | EXCEED |
| d | c25 | 13.2 | off |  | NWT c10 OR BT c09 | PASS |
|  | c26 | 16.4 | off |  | NWT c11 OR BT c10 | PASS |
|  | c27 | 24.4 | off |  | NWT c12 OR BT c11 | PASS |
|  | c28 | 32 | off |  | NWT c13 OR BT c12 | PASS |
|  | c29 | 48 | off |  | NWT c15 OR BT c14 | PASS |
|  | c30 | 13.2 | off | 5% | NWT c17 OR BT c16 | PASS |
|  | c31 | 16.4 | off | 5% | NWT c18 OR BT c17 | PASS |
|  | c32 | 24.4 | off | 5% | NWT c19 OR BT c18 | PASS |
|  | c33 | 32 | off | 5% | NWT c20 OR BT c19 | PASS |
|  | c34 | 48 | off | 5% | NWT c22 OR BT c21 | PASS |
|  | c35 | 24.4 | on |  | NWT c23 | EXCEED |
|  | c36 | 13.2 | on | 5% | NWT c24 | EXCEED |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 1: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-1

## P800-2 (Stereo, Speech+Background)

The complete statistical evaluation of the requirement ToR tests for experiment P800-2 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 6: Statistical overview on the results of P800-2

|  |  | Type | CuT |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |
| b | c26 | 1 | 13.2 | off | NWT | 3.28 | 0.95 | c12 | 2x8 | 2.45 | 1.12 | 7.62 | BT | EXCEED |
|  |  | 2 | 13.2 | off | BT | 3.28 | 0.95 | c11 | 2x7.2 | 2.57 | 1.11 | 6.52 | BT | PASS |
|  | c27 | 1 | 16.4 | off | NWT | 3.54 | 0.95 | c13 | 2x9.6 | 3.43 | 0.98 | 1.15 | NWT | PASS |
|  |  | 2 | 16.4 | off | BT | 3.54 | 0.95 | c12 | 2x8 | 2.45 | 1.12 | 10.01 | BT | PASS |
|  | c28 | 1 | 24.4 | off | NWT | 4.04 | 0.85 | c14 | 2x13.2 | 3.81 | 0.89 | 2.54 | BT | EXCEED |
|  |  | 2 | 24.4 | off | BT | 4.04 | 0.85 | c13 | 2x9.6 | 3.43 | 0.98 | 6.39 | BT | PASS |
|  | c29 | 1 | 32 | off | NWT | 4.21 | 0.75 | c15 | 2x16.4 | 4.19 | 0.77 | 0.21 | NWT | PASS |
|  |  | 2 | 32 | off | BT | 4.21 | 0.75 | c14 | 2x13.2 | 3.81 | 0.89 | 4.62 | BT | PASS |
|  | c30 | 1 | 48 | off | NWT | 4.71 | 0.5 | c17 | 2x32 | 4.41 | 0.73 | 4.54 | BT | EXCEED |
|  |  | 2 | 48 | off | BT | 4.71 | 0.5 | c16 | 2x24.4 | 4.54 | 0.6 | 2.95 | BT | PASS |
|  | c31 | 1 | 64 | off | NWT | 4.72 | 0.48 | c18 | 2x48 | 4.82 | 0.47 | -1.89 | WT | FAIL |
|  |  | 2 | 64 | off | BT | 4.72 | 0.48 | c17 | 2x32 | 4.41 | 0.73 | 4.76 | BT | PASS |
|  | c32 | 1 | 13.2 | on | NWT | 3.33 | 1.02 | c20 | 2x8 | 2.48 | 1.06 | 7.78 | BT | EXCEED |
|  |  | 2 | 13.2 | on | BT | 3.33 | 1.02 | c19 | 2x7.2 | 2.39 | 1.02 | 8.72 | BT | PASS |
|  | c33 | 1 | 16.4 | on | NWT | 3.72 | 0.88 | c21 | 2x9.6 | 3.34 | 0.99 | 3.84 | BT | EXCEED |
|  |  | 2 | 16.4 | on | BT | 3.72 | 0.88 | c20 | 2x8 | 2.48 | 1.06 | 12.07 | BT | PASS |
|  | c34 | 1 | 24.4 | on | NWT | 4 | 0.72 | c22 | 2x13.2 | 3.66 | 1 | 3.75 | BT | EXCEED |
|  |  | 2 | 24.4 | on | BT | 4 | 0.72 | c21 | 2x9.6 | 3.34 | 0.99 | 7.27 | BT | PASS |
|  | c35 | 1 | 32 | on | NWT | 4.16 | 0.78 | c23 | 2x16.4 | 4.1 | 0.83 | 0.66 | NWT | PASS |
|  |  | 2 | 32 | on | BT | 4.16 | 0.78 | c22 | 2x13.2 | 3.66 | 1 | 5.31 | BT | PASS |
|  | c36 | 1 | 48 | on | NWT | 4.52 | 0.59 | c25 | 2x32 | 4.28 | 0.73 | 3.47 | BT | EXCEED |
|  |  | 2 | 48 | on | BT | 4.52 | 0.59 | c24 | 2x24.4 | 4.44 | 0.67 | 1.24 | NWT | FAIL |
| d | c26 | 1 | 13.2 | off | NWT | 3.93 | 1.02 | c12 | 2x8 | 2.55 | 1.1 | 12.38 | BT | EXCEED |
|  |  | 2 | 13.2 | off | BT | 3.93 | 1.02 | c11 | 2x7.2 | 2.71 | 1.16 | 10.63 | BT | PASS |
|  | c27 | 1 | 16.4 | off | NWT | 4.21 | 0.88 | c13 | 2x9.6 | 3.67 | 1.14 | 5.06 | BT | EXCEED |
|  |  | 2 | 16.4 | off | BT | 4.21 | 0.88 | c12 | 2x8 | 2.55 | 1.1 | 15.84 | BT | PASS |
|  | c28 | 1 | 24.4 | off | NWT | 4.43 | 0.78 | c14 | 2x13.2 | 4.12 | 0.97 | 3.41 | BT | EXCEED |
|  |  | 2 | 24.4 | off | BT | 4.43 | 0.78 | c13 | 2x9.6 | 3.67 | 1.14 | 7.46 | BT | PASS |
|  | c29 | 1 | 32 | off | NWT | 4.59 | 0.67 | c15 | 2x16.4 | 4.41 | 0.78 | 2.4 | BT | EXCEED |
|  |  | 2 | 32 | off | BT | 4.59 | 0.67 | c14 | 2x13.2 | 4.12 | 0.97 | 5.44 | BT | PASS |
|  | c30 | 1 | 48 | off | NWT | 4.73 | 0.54 | c17 | 2x32 | 4.56 | 0.7 | 2.69 | BT | EXCEED |
|  |  | 2 | 48 | off | BT | 4.73 | 0.54 | c16 | 2x24.4 | 4.63 | 0.62 | 1.65 | NWT | FAIL |
|  | c31 | 1 | 64 | off | NWT | 4.71 | 0.52 | c18 | 2x48 | 4.77 | 0.52 | -1.11 | NWT | PASS |
|  |  | 2 | 64 | off | BT | 4.71 | 0.52 | c17 | 2x32 | 4.56 | 0.7 | 2.37 | BT | PASS |
|  | c32 | 1 | 13.2 | on | NWT | 3.91 | 1.02 | c20 | 2x8 | 2.67 | 1.13 | 10.94 | BT | EXCEED |
|  |  | 2 | 13.2 | on | BT | 3.91 | 1.02 | c19 | 2x7.2 | 2.62 | 1.11 | 11.51 | BT | PASS |
|  | c33 | 1 | 16.4 | on | NWT | 4.27 | 0.84 | c21 | 2x9.6 | 3.68 | 1.18 | 5.45 | BT | EXCEED |
|  |  | 2 | 16.4 | on | BT | 4.27 | 0.84 | c20 | 2x8 | 2.67 | 1.13 | 15.23 | BT | PASS |
|  | c34 | 1 | 24.4 | on | NWT | 4.4 | 0.74 | c22 | 2x13.2 | 4.24 | 0.88 | 1.82 | BT | EXCEED |
|  |  | 2 | 24.4 | on | BT | 4.4 | 0.74 | c21 | 2x9.6 | 3.68 | 1.18 | 6.92 | BT | PASS |
|  | c35 | 1 | 32 | on | NWT | 4.54 | 0.71 | c23 | 2x16.4 | 4.37 | 0.8 | 2.16 | BT | EXCEED |
|  |  | 2 | 32 | on | BT | 4.54 | 0.71 | c22 | 2x13.2 | 4.24 | 0.88 | 3.49 | BT | PASS |
|  | c36 | 1 | 48 | on | NWT | 4.62 | 0.59 | c25 | 2x32 | 4.51 | 0.77 | 1.54 | NWT | PASS |
|  |  | 2 | 48 | on | BT | 4.62 | 0.59 | c24 | 2x24.4 | 4.57 | 0.68 | 0.74 | NWT | FAIL |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 7: Summary of the results of P800-2

| Lab | Cond. | Bitrate | DTX | ToR | Status |
| --- | --- | --- | --- | --- | --- |
| b | c26 | 13.2 | off | NWT c12 OR BT c11 | PASS |
|  | c27 | 16.4 | off | NWT c13 OR BT c12 | PASS |
|  | c28 | 24.4 | off | NWT c14 OR BT c13 | PASS |
|  | c29 | 32 | off | NWT c15 OR BT c14 | PASS |
|  | c30 | 48 | off | NWT c17 OR BT c16 | PASS |
|  | c31 | 64 | off | NWT c18 OR BT c17 | PASS |
|  | c32 | 13.2 | on | NWT c20 OR BT c19 | PASS |
|  | c33 | 16.4 | on | NWT c21 OR BT c20 | PASS |
|  | c34 | 24.4 | on | NWT c22 OR BT c21 | PASS |
|  | c35 | 32 | on | NWT c23 OR BT c22 | PASS |
|  | c36 | 48 | on | NWT c25 OR BT c24 | PASS |
| d | c26 | 13.2 | off | NWT c12 OR BT c11 | PASS |
|  | c27 | 16.4 | off | NWT c13 OR BT c12 | PASS |
|  | c28 | 24.4 | off | NWT c14 OR BT c13 | PASS |
|  | c29 | 32 | off | NWT c15 OR BT c14 | PASS |
|  | c30 | 48 | off | NWT c17 OR BT c16 | PASS |
|  | c31 | 64 | off | NWT c18 OR BT c17 | PASS |
|  | c32 | 13.2 | on | NWT c20 OR BT c19 | PASS |
|  | c33 | 16.4 | on | NWT c21 OR BT c20 | PASS |
|  | c34 | 24.4 | on | NWT c22 OR BT c21 | PASS |
|  | c35 | 32 | on | NWT c23 OR BT c22 | PASS |
|  | c36 | 48 | on | NWT c25 OR BT c24 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 2: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-2

## P800-3 (Stereo, Mixed & Music)

The complete statistical evaluation of the requirement ToR tests for experiment P800-3 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 8: Statistical overview on the results of P800-3

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | Bitrate | DTX | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 13.2 | off |  | NWT | 3.26 | 1.16 | c10 | 2x8 |  | 1.95 | 1 | 11.43 | BT | EXCEED |
|  |  | 2 | 13.2 | off |  | BT | 3.26 | 1.16 | c09 | 2x7.2 |  | 1.89 | 1.02 | 11.83 | BT | PASS |
|  | c25 | 1 | 16.4 | off |  | NWT | 3.79 | 0.93 | c11 | 2x9.6 |  | 2.96 | 1.25 | 7.18 | BT | EXCEED |
|  |  | 2 | 16.4 | off |  | BT | 3.79 | 0.93 | c10 | 2x8 |  | 1.95 | 1 | 18 | BT | PASS |
|  | c26 | 1 | 24.4 | off |  | NWT | 4.13 | 0.88 | c12 | 2x13.2 |  | 3.41 | 1.17 | 6.62 | BT | EXCEED |
|  |  | 2 | 24.4 | off |  | BT | 4.13 | 0.88 | c11 | 2x9.6 |  | 2.96 | 1.25 | 10.32 | BT | PASS |
|  | c27 | 1 | 32 | off |  | NWT | 4.24 | 0.76 | c13 | 2x16.4 |  | 3.69 | 1.09 | 5.54 | BT | EXCEED |
|  |  | 2 | 32 | off |  | BT | 4.24 | 0.76 | c12 | 2x13.2 |  | 3.41 | 1.17 | 8.06 | BT | PASS |
|  | c28 | 1 | 48 | off |  | NWT | 4.44 | 0.71 | c15 | 2x32 |  | 4.14 | 0.92 | 3.42 | BT | EXCEED |
|  |  | 2 | 48 | off |  | BT | 4.44 | 0.71 | c14 | 2x24.4 |  | 4.12 | 0.87 | 3.86 | BT | PASS |
|  | c29 | 1 | 64 | off |  | NWT | 4.47 | 0.65 | c16 | 2x48 |  | 4.55 | 0.64 | -1.22 | NWT | PASS |
|  |  | 2 | 64 | off |  | BT | 4.47 | 0.65 | c15 | 2x32 |  | 4.14 | 0.92 | 3.87 | BT | PASS |
|  | c30 | 1 | 13.2 | off | 5% | NWT | 2.74 | 1.07 | c18 | 2x8 |  | 1.66 | 0.94 | 10.28 | BT | EXCEED |
|  |  | 2 | 13.2 | off | 5% | BT | 2.74 | 1.07 | c17 | 2x7.2 |  | 1.6 | 0.84 | 11.32 | BT | PASS |
|  | c31 | 1 | 16.4 | off | 5% | NWT | 3.26 | 1.14 | c19 | 2x9.6 |  | 2.71 | 1.18 | 4.5 | BT | EXCEED |
|  |  | 2 | 16.4 | off | 5% | BT | 3.26 | 1.14 | c18 | 2x8 |  | 1.66 | 0.94 | 14.6 | BT | PASS |
|  | c32 | 1 | 24.4 | off | 5% | NWT | 3.42 | 1.13 | c20 | 2x13.2 |  | 2.78 | 1.16 | 5.34 | BT | EXCEED |
|  |  | 2 | 24.4 | off | 5% | BT | 3.42 | 1.13 | c19 | 2x9.6 |  | 2.71 | 1.18 | 5.85 | BT | PASS |
|  | c33 | 1 | 32 | off | 5% | NWT | 3.44 | 1.11 | c21 | 2x16.4 |  | 2.98 | 1.1 | 3.96 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 3.44 | 1.11 | c20 | 2x13.2 |  | 2.78 | 1.16 | 5.56 | BT | PASS |
|  | c34 | 1 | 48 | off | 5% | NWT | 3.91 | 0.94 | c23 | 2x32 |  | 3.41 | 1.12 | 4.6 | BT | EXCEED |
|  |  | 2 | 48 | off | 5% | BT | 3.91 | 0.94 | c22 | 2x24.4 |  | 3.33 | 1.12 | 5.3 | BT | PASS |
|  | c35 | 1 | 24.4 | on |  | NWT | 4.17 | 0.87 | c12 | 2x13.2 | off | 3.41 | 1.17 | 7 | BT | EXCEED |
|  |  | 2 | 24.4 | on |  | BT | 4.17 | 0.87 | c11 | 2x9.6 | off | 2.96 | 1.25 | 10.71 | BT | PASS |
|  | c36 | 1 | 13.2 | on | 5% | NWT | 2.76 | 1.16 | c18 | 2x8 | off | 1.66 | 0.94 | 9.98 | BT | EXCEED |
|  |  | 2 | 13.2 | on | 5% | BT | 2.76 | 1.16 | c17 | 2x7.2 | off | 1.6 | 0.84 | 10.92 | BT | PASS |
| d | c24 | 1 | 13.2 | off |  | NWT | 3.46 | 1.05 | c10 | 2x8 |  | 2.18 | 0.98 | 11.97 | BT | EXCEED |
|  |  | 2 | 13.2 | off |  | BT | 3.46 | 1.05 | c09 | 2x7.2 |  | 2.04 | 0.95 | 13.44 | BT | PASS |
|  | c25 | 1 | 16.4 | off |  | NWT | 4.06 | 0.85 | c11 | 2x9.6 |  | 3.29 | 1.2 | 6.97 | BT | EXCEED |
|  |  | 2 | 16.4 | off |  | BT | 4.06 | 0.85 | c10 | 2x8 |  | 2.18 | 0.98 | 19.43 | BT | PASS |
|  | c26 | 1 | 24.4 | off |  | NWT | 4.29 | 0.72 | c12 | 2x13.2 |  | 3.64 | 1.04 | 6.89 | BT | EXCEED |
|  |  | 2 | 24.4 | off |  | BT | 4.29 | 0.72 | c11 | 2x9.6 |  | 3.29 | 1.2 | 9.51 | BT | PASS |
|  | c27 | 1 | 32 | off |  | NWT | 4.42 | 0.67 | c13 | 2x16.4 |  | 3.97 | 0.89 | 5.41 | BT | EXCEED |
|  |  | 2 | 32 | off |  | BT | 4.42 | 0.67 | c12 | 2x13.2 |  | 3.64 | 1.04 | 8.5 | BT | PASS |
|  | c28 | 1 | 48 | off |  | NWT | 4.58 | 0.56 | c15 | 2x32 |  | 4.33 | 0.79 | 3.46 | BT | EXCEED |
|  |  | 2 | 48 | off |  | BT | 4.58 | 0.56 | c14 | 2x24.4 |  | 4.43 | 0.67 | 2.39 | BT | PASS |
|  | c29 | 1 | 64 | off |  | NWT | 4.63 | 0.57 | c16 | 2x48 |  | 4.61 | 0.53 | 0.46 | NWT | PASS |
|  |  | 2 | 64 | off |  | BT | 4.63 | 0.57 | c15 | 2x32 |  | 4.33 | 0.79 | 4.13 | BT | PASS |
|  | c30 | 1 | 13.2 | off | 5% | NWT | 3.12 | 1.17 | c18 | 2x8 |  | 1.88 | 0.99 | 10.85 | BT | EXCEED |
|  |  | 2 | 13.2 | off | 5% | BT | 3.12 | 1.17 | c17 | 2x7.2 |  | 1.83 | 1 | 11.29 | BT | PASS |
|  | c31 | 1 | 16.4 | off | 5% | NWT | 3.48 | 1.06 | c19 | 2x9.6 |  | 2.79 | 1.25 | 5.64 | BT | EXCEED |
|  |  | 2 | 16.4 | off | 5% | BT | 3.48 | 1.06 | c18 | 2x8 |  | 1.88 | 0.99 | 14.7 | BT | PASS |
|  | c32 | 1 | 24.4 | off | 5% | NWT | 3.9 | 0.92 | c20 | 2x13.2 |  | 3.21 | 1.16 | 6.3 | BT | EXCEED |
|  |  | 2 | 24.4 | off | 5% | BT | 3.9 | 0.92 | c19 | 2x9.6 |  | 2.79 | 1.25 | 9.64 | BT | PASS |
|  | c33 | 1 | 32 | off | 5% | NWT | 4.03 | 0.93 | c21 | 2x16.4 |  | 3.52 | 1.21 | 4.55 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 4.03 | 0.93 | c20 | 2x13.2 |  | 3.21 | 1.16 | 7.47 | BT | PASS |
|  | c34 | 1 | 48 | off | 5% | NWT | 4.14 | 0.84 | c23 | 2x32 |  | 3.76 | 1.09 | 3.74 | BT | EXCEED |
|  |  | 2 | 48 | off | 5% | BT | 4.14 | 0.84 | c22 | 2x24.4 |  | 3.83 | 1.04 | 3.08 | BT | PASS |
|  | c35 | 1 | 24.4 | on |  | NWT | 4.38 | 0.79 | c12 | 2x13.2 | off | 3.64 | 1.04 | 7.58 | BT | EXCEED |
|  |  | 2 | 24.4 | on |  | BT | 4.38 | 0.79 | c11 | 2x9.6 | off | 3.29 | 1.2 | 10.09 | BT | PASS |
|  | c36 | 1 | 13.2 | on | 5% | NWT | 3.09 | 1.15 | c18 | 2x8 | off | 1.88 | 0.99 | 10.65 | BT | EXCEED |
|  |  | 2 | 13.2 | on | 5% | BT | 3.09 | 1.15 | c17 | 2x7.2 | off | 1.83 | 1 | 11.09 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 9: Summary of the results of P800-3

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c24 | 13.2 | off |  | NWT c10 OR BT c09 | PASS |
|  | c25 | 16.4 | off |  | NWT c11 OR BT c10 | PASS |
|  | c26 | 24.4 | off |  | NWT c12 OR BT c11 | PASS |
|  | c27 | 32 | off |  | NWT c13 OR BT c12 | PASS |
|  | c28 | 48 | off |  | NWT c15 OR BT c14 | PASS |
|  | c29 | 64 | off |  | NWT c16 OR BT c15 | PASS |
|  | c30 | 13.2 | off | 5% | NWT c18 OR BT c17 | PASS |
|  | c31 | 16.4 | off | 5% | NWT c19 OR BT c18 | PASS |
|  | c32 | 24.4 | off | 5% | NWT c20 OR BT c19 | PASS |
|  | c33 | 32 | off | 5% | NWT c21 OR BT c20 | PASS |
|  | c34 | 48 | off | 5% | NWT c23 OR BT c22 | PASS |
|  | c35 | 24.4 | on |  | NWT c12 OR BT c11 | PASS |
|  | c36 | 13.2 | on | 5% | NWT c18 OR BT c17 | PASS |
| d | c24 | 13.2 | off |  | NWT c10 OR BT c09 | PASS |
|  | c25 | 16.4 | off |  | NWT c11 OR BT c10 | PASS |
|  | c26 | 24.4 | off |  | NWT c12 OR BT c11 | PASS |
|  | c27 | 32 | off |  | NWT c13 OR BT c12 | PASS |
|  | c28 | 48 | off |  | NWT c15 OR BT c14 | PASS |
|  | c29 | 64 | off |  | NWT c16 OR BT c15 | PASS |
|  | c30 | 13.2 | off | 5% | NWT c18 OR BT c17 | PASS |
|  | c31 | 16.4 | off | 5% | NWT c19 OR BT c18 | PASS |
|  | c32 | 24.4 | off | 5% | NWT c20 OR BT c19 | PASS |
|  | c33 | 32 | off | 5% | NWT c21 OR BT c20 | PASS |
|  | c34 | 48 | off | 5% | NWT c23 OR BT c22 | PASS |
|  | c35 | 24.4 | on |  | NWT c12 OR BT c11 | PASS |
|  | c36 | 13.2 | on | 5% | NWT c18 OR BT c17 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 3: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-3

## P800-4 (FOA, Clean speech)

The complete statistical evaluation of the requirement ToR tests for experiment P800-4 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 10: Statistical overview on the results of P800-4

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 16.4 | off |  | NWT | 3.13 | 1.06 | c09 | 3x7.2 | 2.47 | 1.14 | 5.66 | BT | EXCEED |
|  | c25 | 1 | 24.4 | off |  | NWT | 3.49 | 0.95 | c10 | 4x7.2 | 2.54 | 1.07 | 8.85 | BT | EXCEED |
|  | c26 | 1 | 32 | off |  | NWT | 3.74 | 0.98 | c12 | 4x9.6 | 3.4 | 0.98 | 3.33 | BT | EXCEED |
|  |  | 2 | 32 | off |  | BT | 3.74 | 0.98 | c11 | 4x8 | 2.73 | 1.11 | 9.24 | BT | PASS |
|  | c27 | 1 | 48 | off |  | NWT | 4.31 | 0.76 | c14 | 4x16.4 | 4.34 | 0.77 | -0.41 | NWT | PASS |
|  |  | 2 | 48 | off |  | BT | 4.31 | 0.76 | c13 | 4x13.2 | 4.1 | 0.88 | 2.43 | BT | PASS |
|  | c28 | 1 | 64 | off |  | NWT | 4.61 | 0.59 | c15 | 4x24.4 | 4.52 | 0.66 | 1.35 | NWT | PASS |
|  |  | 2 | 64 | off |  | BT | 4.61 | 0.59 | c14 | 4x16.4 | 4.34 | 0.77 | 3.68 | BT | PASS |
|  | c29 | 1 | 80 | off |  | NWT | 4.66 | 0.51 | c15 | 4x24.4 | 4.52 | 0.66 | 2.17 | BT | EXCEED |
|  |  | 2 | 80 | off |  | BT | 4.66 | 0.51 | c14 | 4x16.4 | 4.34 | 0.77 | 4.52 | BT | PASS |
|  | c30 | 1 | 96 | off |  | NWT | 4.72 | 0.47 | c16 | 4x32 | 4.52 | 0.64 | 3.46 | BT | EXCEED |
|  |  | 2 | 96 | off |  | BT | 4.72 | 0.47 | c15 | 4x24.4 | 4.52 | 0.66 | 3.32 | BT | PASS |
|  | c31 | 1 | 24.4 | off | 5% | NWT | 3.07 | 1.08 | c17 | 4x7.2 | 2.27 | 1.05 | 7.07 | BT | EXCEED |
|  | c32 | 1 | 32 | off | 5% | NWT | 3.31 | 0.97 | c19 | 4x9.6 | 2.93 | 1.09 | 3.53 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 3.31 | 0.97 | c18 | 4x8 | 2.38 | 1.14 | 8.34 | BT | PASS |
|  | c33 | 1 | 48 | off | 5% | NWT | 3.81 | 0.95 | c21 | 4x16.4 | 3.74 | 0.88 | 0.64 | NWT | PASS |
|  |  | 2 | 48 | off | 5% | BT | 3.81 | 0.95 | c20 | 4x13.2 | 3.52 | 0.99 | 2.83 | BT | PASS |
|  | c34 | 1 | 64 | off | 5% | NWT | 4.1 | 0.96 | c22 | 4x24.4 | 3.96 | 0.87 | 1.44 | NWT | PASS |
|  |  | 2 | 64 | off | 5% | BT | 4.1 | 0.96 | c21 | 4x16.4 | 3.74 | 0.88 | 3.66 | BT | PASS |
|  | c35 | 1 | 80 | off | 5% | NWT | 4.15 | 0.91 | c22 | 4x24.4 | 3.96 | 0.87 | 2.01 | BT | EXCEED |
|  |  | 2 | 80 | off | 5% | BT | 4.15 | 0.91 | c21 | 4x16.4 | 3.74 | 0.88 | 4.3 | BT | PASS |
|  | c36 | 1 | 96 | off | 5% | NWT | 4.02 | 0.92 | c23 | 4x32 | 3.85 | 0.87 | 1.83 | BT | EXCEED |
|  |  | 2 | 96 | off | 5% | BT | 4.02 | 0.92 | c22 | 4x24.4 | 3.96 | 0.87 | 0.65 | NWT | FAIL |
| c | c24 | 1 | 16.4 | off |  | NWT | 3.53 | 0.92 | c09 | 3x7.2 | 2.44 | 1 | 10.82 | BT | EXCEED |
|  | c25 | 1 | 24.4 | off |  | NWT | 3.68 | 0.86 | c10 | 4x7.2 | 2.44 | 0.89 | 13.45 | BT | EXCEED |
|  | c26 | 1 | 32 | off |  | NWT | 3.77 | 0.83 | c12 | 4x9.6 | 3.51 | 0.89 | 2.88 | BT | EXCEED |
|  |  | 2 | 32 | off |  | BT | 3.77 | 0.83 | c11 | 4x8 | 2.59 | 0.96 | 12.44 | BT | PASS |
|  | c27 | 1 | 48 | off |  | NWT | 4.39 | 0.63 | c14 | 4x16.4 | 4.29 | 0.67 | 1.46 | NWT | PASS |
|  |  | 2 | 48 | off |  | BT | 4.39 | 0.63 | c13 | 4x13.2 | 4.1 | 0.73 | 4.01 | BT | PASS |
|  | c28 | 1 | 64 | off |  | NWT | 4.61 | 0.54 | c15 | 4x24.4 | 4.5 | 0.62 | 1.81 | BT | EXCEED |
|  |  | 2 | 64 | off |  | BT | 4.61 | 0.54 | c14 | 4x16.4 | 4.29 | 0.67 | 5 | BT | PASS |
|  | c29 | 1 | 80 | off |  | NWT | 4.64 | 0.57 | c15 | 4x24.4 | 4.5 | 0.62 | 2.22 | BT | EXCEED |
|  |  | 2 | 80 | off |  | BT | 4.64 | 0.57 | c14 | 4x16.4 | 4.29 | 0.67 | 5.34 | BT | PASS |
|  | c30 | 1 | 96 | off |  | NWT | 4.66 | 0.51 | c16 | 4x32 | 4.46 | 0.62 | 3.35 | BT | EXCEED |
|  |  | 2 | 96 | off |  | BT | 4.66 | 0.51 | c15 | 4x24.4 | 4.5 | 0.62 | 2.69 | BT | PASS |
|  | c31 | 1 | 24.4 | off | 5% | NWT | 3.14 | 0.99 | c17 | 4x7.2 | 2.18 | 0.96 | 9.34 | BT | EXCEED |
|  | c32 | 1 | 32 | off | 5% | NWT | 3.51 | 0.89 | c19 | 4x9.6 | 3.08 | 1.02 | 4.19 | BT | EXCEED |
|  |  | 2 | 32 | off | 5% | BT | 3.51 | 0.89 | c18 | 4x8 | 2.26 | 0.92 | 13.1 | BT | PASS |
|  | c33 | 1 | 48 | off | 5% | NWT | 3.94 | 0.88 | c21 | 4x16.4 | 3.63 | 0.92 | 3.32 | BT | EXCEED |
|  |  | 2 | 48 | off | 5% | BT | 3.94 | 0.88 | c20 | 4x13.2 | 3.49 | 0.97 | 4.65 | BT | PASS |
|  | c34 | 1 | 64 | off | 5% | NWT | 4.13 | 0.76 | c22 | 4x24.4 | 4.09 | 0.75 | 0.49 | NWT | PASS |
|  |  | 2 | 64 | off | 5% | BT | 4.13 | 0.76 | c21 | 4x16.4 | 3.63 | 0.92 | 5.61 | BT | PASS |
|  | c35 | 1 | 80 | off | 5% | NWT | 4.18 | 0.7 | c22 | 4x24.4 | 4.09 | 0.75 | 1.23 | NWT | PASS |
|  |  | 2 | 80 | off | 5% | BT | 4.18 | 0.7 | c21 | 4x16.4 | 3.63 | 0.92 | 6.44 | BT | PASS |
|  | c36 | 1 | 96 | off | 5% | NWT | 4.14 | 0.8 | c23 | 4x32 | 3.81 | 0.76 | 4.05 | BT | EXCEED |
|  |  | 2 | 96 | off | 5% | BT | 4.14 | 0.8 | c22 | 4x24.4 | 4.09 | 0.75 | 0.61 | NWT | FAIL |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 11: Summary of the results of P800-4

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c24 | 16.4 | off |  | NWT c09 | EXCEED |
|  | c25 | 24.4 | off |  | NWT c10 | EXCEED |
|  | c26 | 32 | off |  | NWT c12 or BT c11 | PASS |
|  | c27 | 48 | off |  | NWT c14 or BT c13 | PASS |
|  | c28 | 64 | off |  | NWT c15 or BT c14 | PASS |
|  | c29 | 80 | off |  | NWT c15 or BT c14 | PASS |
|  | c30 | 96 | off |  | NWT c16 or BT c15 | PASS |
|  | c31 | 24.4 | off | 5% | NWT c17 | EXCEED |
|  | c32 | 32 | off | 5% | NWT c19 or BT c18 | PASS |
|  | c33 | 48 | off | 5% | NWT c21 or BT c20 | PASS |
|  | c34 | 64 | off | 5% | NWT c22 or BT c21 | PASS |
|  | c35 | 80 | off | 5% | NWT c22 or BT c21 | PASS |
|  | c36 | 96 | off | 5% | NWT c23 or BT c22 | PASS |
| c | c24 | 16.4 | off |  | NWT c09 | EXCEED |
|  | c25 | 24.4 | off |  | NWT c10 | EXCEED |
|  | c26 | 32 | off |  | NWT c12 or BT c11 | PASS |
|  | c27 | 48 | off |  | NWT c14 or BT c13 | PASS |
|  | c28 | 64 | off |  | NWT c15 or BT c14 | PASS |
|  | c29 | 80 | off |  | NWT c15 or BT c14 | PASS |
|  | c30 | 96 | off |  | NWT c16 or BT c15 | PASS |
|  | c31 | 24.4 | off | 5% | NWT c17 | EXCEED |
|  | c32 | 32 | off | 5% | NWT c19 or BT c18 | PASS |
|  | c33 | 48 | off | 5% | NWT c21 or BT c20 | PASS |
|  | c34 | 64 | off | 5% | NWT c22 or BT c21 | PASS |
|  | c35 | 80 | off | 5% | NWT c22 or BT c21 | PASS |
|  | c36 | 96 | off | 5% | NWT c23 or BT c22 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 4: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-4

## P800-5 (FOA, Speech+Background)

The complete statistical evaluation of the requirement ToR tests for experiment P800-5 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 12: Statistical overview on the results of P800-5

|  |  | Type | CuT |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 16.4 | off | NWT | 3.26 | 0.96 | c09 | 3x7.2 | 2.93 | 1.04 | 3.06 | BT | EXCEED |
|  | c25 | 1 | 24.4 | off | NWT | 3.68 | 1 | c10 | 4x7.2 | 3.04 | 1.08 | 5.82 | BT | EXCEED |
|  | c26 | 1 | 32 | off | NWT | 3.84 | 0.96 | c12 | 4x9.6 | 3.74 | 0.88 | 1.08 | NWT | PASS |
|  |  | 2 | 32 | off | BT | 3.84 | 0.96 | c11 | 4x8 | 3.15 | 1.03 | 6.6 | BT | PASS |
|  | c27 | 1 | 48 | off | NWT | 4.45 | 0.66 | c14 | 4x16.4 | 4.27 | 0.68 | 2.58 | BT | EXCEED |
|  |  | 2 | 48 | off | BT | 4.45 | 0.66 | c13 | 4x13.2 | 4 | 0.85 | 5.62 | BT | PASS |
|  | c28 | 1 | 64 | off | NWT | 4.54 | 0.59 | c15 | 4x24.4 | 4.47 | 0.71 | 1.12 | NWT | PASS |
|  |  | 2 | 64 | off | BT | 4.54 | 0.59 | c14 | 4x16.4 | 4.27 | 0.68 | 4.12 | BT | PASS |
|  | c29 | 1 | 80 | off | NWT | 4.56 | 0.65 | c15 | 4x24.4 | 4.47 | 0.71 | 1.31 | NWT | PASS |
|  |  | 2 | 80 | off | BT | 4.56 | 0.65 | c14 | 4x16.4 | 4.27 | 0.68 | 4.18 | BT | PASS |
|  | c30 | 1 | 96 | off | NWT | 4.54 | 0.65 | c16 | 4x32 | 4.38 | 0.68 | 2.38 | BT | EXCEED |
|  |  | 2 | 96 | off | BT | 4.54 | 0.65 | c15 | 4x24.4 | 4.47 | 0.71 | 1.07 | NWT | FAIL |
|  | c31 | 1 | 16.4 | on | NWT | 3.26 | 1 | c17 | 3x7.2 | 2.78 | 1.09 | 4.28 | BT | EXCEED |
|  | c32 | 1 | 24.4 | on | NWT | 3.49 | 0.98 | c18 | 4x7.2 | 2.96 | 1.01 | 5.09 | BT | EXCEED |
|  | c33 | 1 | 32 | on | NWT | 3.8 | 0.88 | c20 | 4x9.6 | 3.64 | 0.9 | 1.72 | BT | EXCEED |
|  |  | 2 | 32 | on | BT | 3.8 | 0.88 | c19 | 4x8 | 2.97 | 0.98 | 8.42 | BT | PASS |
|  | c34 | 1 | 48 | on | NWT | 4.26 | 0.7 | c22 | 4x16.4 | 4.13 | 0.66 | 1.79 | BT | EXCEED |
|  |  | 2 | 48 | on | BT | 4.26 | 0.7 | c21 | 4x13.2 | 3.87 | 0.88 | 4.69 | BT | PASS |
|  | c35 | 1 | 64 | on | NWT | 4.25 | 0.76 | c23 | 4x24.4 | 4.2 | 0.86 | 0.58 | NWT | PASS |
|  |  | 2 | 64 | on | BT | 4.25 | 0.76 | c22 | 4x16.4 | 4.13 | 0.66 | 1.56 | NWT | FAIL |
|  | c36 | 1 | 80 | on | NWT | 4.3 | 0.75 | c23 | 4x24.4 | 4.2 | 0.86 | 1.18 | NWT | PASS |
|  |  | 2 | 80 | on | BT | 4.3 | 0.75 | c22 | 4x16.4 | 4.13 | 0.66 | 2.26 | BT | PASS |
| b | c24 | 1 | 16.4 | off | NWT | 3.53 | 0.97 | c09 | 3x7.2 | 2.82 | 0.99 | 6.91 | BT | EXCEED |
|  | c25 | 1 | 24.4 | off | NWT | 3.78 | 1.01 | c10 | 4x7.2 | 2.93 | 1.04 | 7.88 | BT | EXCEED |
|  | c26 | 1 | 32 | off | NWT | 3.96 | 0.86 | c12 | 4x9.6 | 3.81 | 0.99 | 1.53 | NWT | PASS |
|  |  | 2 | 32 | off | BT | 3.96 | 0.86 | c11 | 4x8 | 2.87 | 1.05 | 10.75 | BT | PASS |
|  | c27 | 1 | 48 | off | NWT | 4.61 | 0.57 | c14 | 4x16.4 | 4.45 | 0.64 | 2.44 | BT | EXCEED |
|  |  | 2 | 48 | off | BT | 4.61 | 0.57 | c13 | 4x13.2 | 4.16 | 0.8 | 6.15 | BT | PASS |
|  | c28 | 1 | 64 | off | NWT | 4.77 | 0.46 | c15 | 4x24.4 | 4.63 | 0.55 | 2.6 | BT | EXCEED |
|  |  | 2 | 64 | off | BT | 4.77 | 0.46 | c14 | 4x16.4 | 4.45 | 0.64 | 5.4 | BT | PASS |
|  | c29 | 1 | 80 | off | NWT | 4.71 | 0.53 | c15 | 4x24.4 | 4.63 | 0.55 | 1.45 | NWT | PASS |
|  |  | 2 | 80 | off | BT | 4.71 | 0.53 | c14 | 4x16.4 | 4.45 | 0.64 | 4.21 | BT | PASS |
|  | c30 | 1 | 96 | off | NWT | 4.75 | 0.45 | c16 | 4x32 | 4.6 | 0.59 | 2.71 | BT | EXCEED |
|  |  | 2 | 96 | off | BT | 4.75 | 0.45 | c15 | 4x24.4 | 4.63 | 0.55 | 2.31 | BT | PASS |
|  | c31 | 1 | 16.4 | on | NWT | 3.52 | 1.04 | c17 | 3x7.2 | 2.72 | 0.99 | 7.5 | BT | EXCEED |
|  | c32 | 1 | 24.4 | on | NWT | 3.76 | 0.94 | c18 | 4x7.2 | 2.73 | 1.02 | 9.97 | BT | EXCEED |
|  | c33 | 1 | 32 | on | NWT | 3.79 | 0.99 | c20 | 4x9.6 | 3.83 | 0.88 | -0.34 | NWT | PASS |
|  |  | 2 | 32 | on | BT | 3.79 | 0.99 | c19 | 4x8 | 2.88 | 1.06 | 8.49 | BT | PASS |
|  | c34 | 1 | 48 | on | NWT | 4.52 | 0.58 | c22 | 4x16.4 | 4.33 | 0.72 | 2.73 | BT | EXCEED |
|  |  | 2 | 48 | on | BT | 4.52 | 0.58 | c21 | 4x13.2 | 4.02 | 0.84 | 6.65 | BT | PASS |
|  | c35 | 1 | 64 | on | NWT | 4.63 | 0.53 | c23 | 4x24.4 | 4.51 | 0.68 | 1.9 | BT | EXCEED |
|  |  | 2 | 64 | on | BT | 4.63 | 0.53 | c22 | 4x16.4 | 4.33 | 0.72 | 4.41 | BT | PASS |
|  | c36 | 1 | 80 | on | NWT | 4.73 | 0.47 | c23 | 4x24.4 | 4.51 | 0.68 | 3.69 | BT | EXCEED |
|  |  | 2 | 80 | on | BT | 4.73 | 0.47 | c22 | 4x16.4 | 4.33 | 0.72 | 6.22 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 13: Summary of the results of P800-5

| Lab | Cond. | Bitrate | DTX | ToR | Status |
| --- | --- | --- | --- | --- | --- |
| a | c24 | 16.4 | off | NWT c09 | EXCEED |
|  | c25 | 24.4 | off | NWT c10 | EXCEED |
|  | c26 | 32 | off | NWT c12 or BT c11 | PASS |
|  | c27 | 48 | off | NWT c14 or BT c13 | PASS |
|  | c28 | 64 | off | NWT c15 or BT c14 | PASS |
|  | c29 | 80 | off | NWT c15 or BT c14 | PASS |
|  | c30 | 96 | off | NWT c16 or BT c15 | PASS |
|  | c31 | 16.4 | on | NWT c17 | EXCEED |
|  | c32 | 24.4 | on | NWT c18 | EXCEED |
|  | c33 | 32 | on | NWT c20 or BT c19 | PASS |
|  | c34 | 48 | on | NWT c22 or BT c21 | PASS |
|  | c35 | 64 | on | NWT c23 or BT c22 | PASS |
|  | c36 | 80 | on | NWT c23 or BT c22 | PASS |
| b | c24 | 16.4 | off | NWT c09 | EXCEED |
|  | c25 | 24.4 | off | NWT c10 | EXCEED |
|  | c26 | 32 | off | NWT c12 or BT c11 | PASS |
|  | c27 | 48 | off | NWT c14 or BT c13 | PASS |
|  | c28 | 64 | off | NWT c15 or BT c14 | PASS |
|  | c29 | 80 | off | NWT c15 or BT c14 | PASS |
|  | c30 | 96 | off | NWT c16 or BT c15 | PASS |
|  | c31 | 16.4 | on | NWT c17 | EXCEED |
|  | c32 | 24.4 | on | NWT c18 | EXCEED |
|  | c33 | 32 | on | NWT c20 or BT c19 | PASS |
|  | c34 | 48 | on | NWT c22 or BT c21 | PASS |
|  | c35 | 64 | on | NWT c23 or BT c22 | PASS |
|  | c36 | 80 | on | NWT c23 or BT c22 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 5: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-5

## P800-6 (1 Object, Clean speech)

The complete statistical evaluation of the requirement ToR tests for experiment P800-6 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 14: Statistical overview on the results of P800-6

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 13.2 | Off |  | NWT | 4.49 | 0.77 | c11 | 4.46 | 0.75 | 0.48 | NWT | PASS |
|  | c25 | 1 | 16.4 | Off |  | NWT | 4.44 | 0.73 | c12 | 4.53 | 0.72 | -1.1 | NWT | PASS |
|  | c26 | 1 | 24.4 | Off |  | NWT | 4.58 | 0.67 | c13 | 4.61 | 0.6 | -0.49 | NWT | PASS |
|  | c27 | 1 | 32 | Off |  | NWT | 4.62 | 0.68 | c14 | 4.59 | 0.61 | 0.48 | NWT | PASS |
|  | c28 | 1 | 48 | Off |  | NWT | 4.58 | 0.65 | c15 | 4.63 | 0.62 | -0.67 | NWT | PASS |
|  | c29 | 1 | 64 | Off |  | NWT | 4.57 | 0.6 | c16 | 4.7 | 0.56 | -2.18 | WT | FAIL |
|  | c30 | 1 | 13.2 | Off | 5% | NWT | 3.67 | 1.14 | c17 | 3.56 | 1.15 | 0.96 | NWT | PASS |
|  | c31 | 1 | 16.4 | Off | 5% | NWT | 3.49 | 1.13 | c18 | 3.74 | 1.07 | -2.15 | WT | FAIL |
|  | c32 | 1 | 24.4 | Off | 5% | NWT | 3.82 | 1.07 | c19 | 3.77 | 1.06 | 0.49 | NWT | PASS |
|  | c33 | 1 | 32 | Off | 5% | NWT | 3.8 | 1.08 | c20 | 3.85 | 1.04 | -0.45 | NWT | PASS |
|  | c34 | 1 | 13.2 | On |  | NWT | 4.4 | 0.86 | c21 | 4.44 | 0.73 | -0.52 | NWT | PASS |
|  | c35 | 1 | 16.4 | On |  | NWT | 4.45 | 0.74 | c22 | 4.51 | 0.69 | -0.81 | NWT | PASS |
|  | c36 | 1 | 24.4 | On |  | NWT | 4.54 | 0.63 | c23 | 4.66 | 0.56 | -1.94 | WT | FAIL |
| c | c24 | 1 | 13.2 | Off |  | NWT | 4.23 | 0.71 | c11 | 4.14 | 0.76 | 1.09 | NWT | PASS |
|  | c25 | 1 | 16.4 | Off |  | NWT | 4.34 | 0.69 | c12 | 4.44 | 0.62 | -1.45 | NWT | PASS |
|  | c26 | 1 | 24.4 | Off |  | NWT | 4.49 | 0.66 | c13 | 4.52 | 0.6 | -0.42 | NWT | PASS |
|  | c27 | 1 | 32 | Off |  | NWT | 4.52 | 0.59 | c14 | 4.51 | 0.55 | 0.1 | NWT | PASS |
|  | c28 | 1 | 48 | Off |  | NWT | 4.48 | 0.64 | c15 | 4.59 | 0.6 | -1.69 | WT | FAIL |
|  | c29 | 1 | 64 | Off |  | NWT | 4.57 | 0.59 | c16 | 4.58 | 0.61 | -0.17 | NWT | PASS |
|  | c30 | 1 | 13.2 | Off | 5% | NWT | 3.4 | 0.82 | c17 | 3.43 | 0.89 | -0.37 | NWT | PASS |
|  | c31 | 1 | 16.4 | Off | 5% | NWT | 3.44 | 0.89 | c18 | 3.48 | 0.96 | -0.35 | NWT | PASS |
|  | c32 | 1 | 24.4 | Off | 5% | NWT | 3.58 | 0.89 | c19 | 3.68 | 0.89 | -1.07 | NWT | PASS |
|  | c33 | 1 | 32 | Off | 5% | NWT | 3.65 | 0.88 | c20 | 3.55 | 0.88 | 1.08 | NWT | PASS |
|  | c34 | 1 | 13.2 | On |  | NWT | 4.26 | 0.69 | c21 | 4.29 | 0.68 | -0.52 | NWT | PASS |
|  | c35 | 1 | 16.4 | On |  | NWT | 4.46 | 0.63 | c22 | 4.31 | 0.7 | 2.21 | BT | EXCEED |
|  | c36 | 1 | 24.4 | On |  | NWT | 4.56 | 0.58 | c23 | 4.6 | 0.61 | -0.7 | NWT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 15: Summary of the results of P800-6

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c24 | 13.2 | Off |  | NWT C11 | PASS |
|  | c25 | 16.4 | Off |  | NWT C12 | PASS |
|  | c26 | 24.4 | Off |  | NWT C13 | PASS |
|  | c27 | 32 | Off |  | NWT C14 | PASS |
|  | c28 | 48 | Off |  | NWT C15 | PASS |
|  | c29 | 64 | Off |  | NWT C16 | FAIL |
|  | c30 | 13.2 | Off | 5% | NWT C17 | PASS |
|  | c31 | 16.4 | Off | 5% | NWT C18 | FAIL |
|  | c32 | 24.4 | Off | 5% | NWT C19 | PASS |
|  | c33 | 32 | Off | 5% | NWT C20 | PASS |
|  | c34 | 13.2 | On |  | NWT C21 | PASS |
|  | c35 | 16.4 | On |  | NWT C22 | PASS |
|  | c36 | 24.4 | On |  | NWT C23 | FAIL |
| c | c24 | 13.2 | Off |  | NWT C11 | PASS |
|  | c25 | 16.4 | Off |  | NWT C12 | PASS |
|  | c26 | 24.4 | Off |  | NWT C13 | PASS |
|  | c27 | 32 | Off |  | NWT C14 | PASS |
|  | c28 | 48 | Off |  | NWT C15 | FAIL |
|  | c29 | 64 | Off |  | NWT C16 | PASS |
|  | c30 | 13.2 | Off | 5% | NWT C17 | PASS |
|  | c31 | 16.4 | Off | 5% | NWT C18 | PASS |
|  | c32 | 24.4 | Off | 5% | NWT C19 | PASS |
|  | c33 | 32 | Off | 5% | NWT C20 | PASS |
|  | c34 | 13.2 | On |  | NWT C21 | PASS |
|  | c35 | 16.4 | On |  | NWT C22 | EXCEED |
|  | c36 | 24.4 | On |  | NWT C23 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 6: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-6

## P800-7 (2 Objects, Clean speech)

The complete statistical evaluation of the requirement ToR tests for experiment P800-7 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 16: Statistical overview on the results of P800-7

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 16.4 | Off |  | NWT | 3.08 | 1.14 | c11 | 2 x 8.0 | 2.63 | 1.06 | 3.87 | BT | EXCEED |
|  | c25 | 1 | 24.4 | Off |  | NWT | 4.42 | 0.68 | c12 | 2 x 13.2 | 4.41 | 0.73 | 0.15 | NWT | PASS |
|  | c26 | 1 | 32 | Off |  | NWT | 4.59 | 0.59 | c13 | 2 x 16.4 | 4.51 | 0.72 | 1.2 | NWT | PASS |
|  | c27 | 1 | 48 | Off |  | NWT | 4.57 | 0.58 | c14 | 2 x 24.4 | 4.58 | 0.66 | -0.17 | NWT | PASS |
|  | c28 | 1 | 64 | Off |  | NWT | 4.55 | 0.66 | c15 | 2 x 32 | 4.5 | 0.74 | 0.67 | NWT | PASS |
|  | c29 | 1 | 16.4 | Off | 5% | NWT | 2.79 | 1.16 | c16 | 2 x 8.0 | 2.46 | 1.09 | 2.76 | BT | EXCEED |
|  | c30 | 1 | 24.4 | Off | 5% | NWT | 3.92 | 1.04 | c17 | 2 x 13.2 | 3.87 | 0.91 | 0.54 | NWT | PASS |
|  | c31 | 1 | 32 | Off | 5% | NWT | 4.05 | 0.84 | c18 | 2 x 16.4 | 3.94 | 0.91 | 1.2 | NWT | PASS |
|  | c32 | 1 | 48 | Off | 5% | NWT | 4.07 | 0.83 | c19 | 2 x 24.4 | 4.07 | 0.89 | 0.06 | NWT | PASS |
|  | c33 | 1 | 16.4 | On |  | NWT | 3.03 | 1.13 | c20 | 2 x 8.0 | 2.76 | 1.12 | 2.3 | BT | EXCEED |
|  | c34 | 1 | 24.4 | On |  | NWT | 4.45 | 0.69 | c21 | 2 x 13.2 | 4.36 | 0.74 | 1.25 | NWT | PASS |
|  | c35 | 1 | 32 | On |  | NWT | 4.58 | 0.67 | c22 | 2 x 16.4 | 4.47 | 0.7 | 1.47 | NWT | PASS |
|  | c36 | 1 | 48 | On |  | NWT | 4.54 | 0.65 | c23 | 2 x 24.4 | 4.54 | 0.6 | 0.08 | NWT | PASS |
| d | c24 | 1 | 16.4 | Off |  | NWT | 3.96 | 0.89 | c11 | 2 x 8.0 | 3.19 | 0.96 | 7.79 | BT | EXCEED |
|  | c25 | 1 | 24.4 | Off |  | NWT | 4.66 | 0.54 | c12 | 2 x 13.2 | 4.64 | 0.55 | 0.38 | NWT | PASS |
|  | c26 | 1 | 32 | Off |  | NWT | 4.7 | 0.52 | c13 | 2 x 16.4 | 4.63 | 0.59 | 1.23 | NWT | PASS |
|  | c27 | 1 | 48 | Off |  | NWT | 4.7 | 0.51 | c14 | 2 x 24.4 | 4.76 | 0.44 | -1.12 | NWT | PASS |
|  | c28 | 1 | 64 | Off |  | NWT | 4.74 | 0.49 | c15 | 2 x 32 | 4.71 | 0.49 | 0.54 | NWT | PASS |
|  | c29 | 1 | 16.4 | Off | 5% | NWT | 3.47 | 0.97 | c16 | 2 x 8.0 | 2.89 | 1.1 | 5.34 | BT | EXCEED |
|  | c30 | 1 | 24.4 | Off | 5% | NWT | 4.26 | 0.81 | c17 | 2 x 13.2 | 4.23 | 0.83 | 0.27 | NWT | PASS |
|  | c31 | 1 | 32 | Off | 5% | NWT | 4.32 | 0.75 | c18 | 2 x 16.4 | 4.33 | 0.79 | -0.14 | NWT | PASS |
|  | c32 | 1 | 48 | Off | 5% | NWT | 4.32 | 0.78 | c19 | 2 x 24.4 | 4.43 | 0.71 | -1.41 | NWT | PASS |
|  | c33 | 1 | 16.4 | On |  | NWT | 3.86 | 0.91 | c20 | 2 x 8.0 | 3.32 | 0.91 | 5.56 | BT | EXCEED |
|  | c34 | 1 | 24.4 | On |  | NWT | 4.63 | 0.6 | c21 | 2 x 13.2 | 4.64 | 0.53 | -0.1 | NWT | PASS |
|  | c35 | 1 | 32 | On |  | NWT | 4.64 | 0.57 | c22 | 2 x 16.4 | 4.62 | 0.57 | 0.36 | NWT | PASS |
|  | c36 | 1 | 48 | On |  | NWT | 4.66 | 0.54 | c23 | 2 x 24.4 | 4.73 | 0.47 | -1.35 | NWT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 17: Summary of the results of P800-7

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c24 | 16.4 | Off |  | NWT C11 | EXCEED |
|  | c25 | 24.4 | Off |  | NWT C12 | PASS |
|  | c26 | 32 | Off |  | NWT C13 | PASS |
|  | c27 | 48 | Off |  | NWT C14 | PASS |
|  | c28 | 64 | Off |  | NWT C15 | PASS |
|  | c29 | 16.4 | Off | 5% | NWT C16 | EXCEED |
|  | c30 | 24.4 | Off | 5% | NWT C17 | PASS |
|  | c31 | 32 | Off | 5% | NWT C18 | PASS |
|  | c32 | 48 | Off | 5% | NWT C19 | PASS |
|  | c33 | 16.4 | On |  | NWT C20 | EXCEED |
|  | c34 | 24.4 | On |  | NWT C21 | PASS |
|  | c35 | 32 | On |  | NWT C22 | PASS |
|  | c36 | 48 | On |  | NWT C23 | PASS |
| d | c24 | 16.4 | Off |  | NWT C11 | EXCEED |
|  | c25 | 24.4 | Off |  | NWT C12 | PASS |
|  | c26 | 32 | Off |  | NWT C13 | PASS |
|  | c27 | 48 | Off |  | NWT C14 | PASS |
|  | c28 | 64 | Off |  | NWT C15 | PASS |
|  | c29 | 16.4 | Off | 5% | NWT C16 | EXCEED |
|  | c30 | 24.4 | Off | 5% | NWT C17 | PASS |
|  | c31 | 32 | Off | 5% | NWT C18 | PASS |
|  | c32 | 48 | Off | 5% | NWT C19 | PASS |
|  | c33 | 16.4 | On |  | NWT C20 | EXCEED |
|  | c34 | 24.4 | On |  | NWT C21 | PASS |
|  | c35 | 32 | On |  | NWT C22 | PASS |
|  | c36 | 48 | On |  | NWT C23 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 7: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-7

## P800-8 (MASA, Clean speech)

The complete statistical evaluation of the requirement ToR tests for experiment P800-8 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 18: Statistical overview on the results of P800-8

|  |  | Type | CuT |  |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | FER | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c25 | 1 | 13.2 | Off |  | NWT | 3.61 | 0.91 | c09 | 3x7.2 | 2.91 | 1.06 | 6.7 | BT | EXCEED |
|  |  | 2 | 13.2 | Off |  | NWT | 3.61 | 0.91 | c15 | 2x7.2 | 2.93 | 1.24 | 5.91 | BT | EXCEED |
|  | c26 | 1 | 16.4 | Off |  | NWT | 3.91 | 0.83 | c10 | 4x7.2 | 2.9 | 1.09 | 9.89 | BT | EXCEED |
|  |  | 2 | 16.4 | Off |  | NWT | 3.91 | 0.83 | c15 | 2x7.2 | 2.93 | 1.24 | 8.78 | BT | EXCEED |
|  | c27 | 1 | 24.4 | Off |  | NWT | 4.08 | 0.78 | c11 | 4x8 | 2.96 | 1.08 | 11.28 | BT | EXCEED |
|  |  | 2 | 24.4 | Off |  | NWT | 4.08 | 0.78 | c16 | 2x8 | 3.12 | 1.23 | 8.94 | BT | EXCEED |
|  | c28 | 1 | 32 | Off |  | NWT | 4.28 | 0.77 | c12 | 4x9.6 | 3.62 | 1 | 6.96 | BT | EXCEED |
|  |  | 2 | 32 | Off |  | NWT | 4.28 | 0.77 | c17 | 2x9.6 | 3.67 | 1.1 | 6.09 | BT | EXCEED |
|  | c29 | 1 | 48 | Off |  | NWT | 4.46 | 0.67 | c13 | 4x16.4 | 4.28 | 0.76 | 2.36 | BT | EXCEED |
|  |  | 2 | 48 | Off |  | NWT | 4.46 | 0.67 | c18 | 2x16.4 | 4.45 | 0.68 | 0.08 | NWT | PASS |
|  | c30 | 1 | 64 | Off |  | NWT | 4.59 | 0.6 | c14 | 4x24.4 | 4.48 | 0.65 | 1.76 | BT | EXCEED |
|  |  | 2 | 64 | Off |  | NWT | 4.59 | 0.6 | c19 | 2x24.4 | 4.67 | 0.57 | -1.26 | NWT | PASS |
|  | c31 | 1 | 80 | Off |  | NWT | 4.68 | 0.5 | c14 | 4x24.4 | 4.48 | 0.65 | 3.27 | BT | EXCEED |
|  |  | 2 | 80 | Off |  | NWT | 4.68 | 0.5 | c19 | 2x24.4 | 4.67 | 0.57 | 0.11 | NWT | PASS |
|  | c32 | 1 | 13.2 | Off | 5% | NWT | 3.36 | 0.98 | c20 | 3x7.2 | 2.57 | 1.04 | 7.38 | BT | EXCEED |
|  | c33 | 1 | 16.4 | Off | 5% | NWT | 3.68 | 1 | c21 | 4x7.2 | 2.51 | 1.16 | 10.22 | BT | EXCEED |
|  | c34 | 1 | 24.4 | Off | 5% | NWT | 3.83 | 0.92 | c22 | 4x8 | 2.73 | 1.07 | 10.5 | BT | EXCEED |
|  | c35 | 1 | 48 | Off | 5% | NWT | 4.22 | 0.8 | c23 | 4x16.4 | 4 | 0.9 | 2.48 | BT | EXCEED |
|  | c36 | 1 | 64 | Off | 5% | NWT | 4.37 | 0.76 | c24 | 4x24.4 | 4.29 | 0.74 | 0.99 | NWT | PASS |
| b | c25 | 1 | 13.2 | Off |  | NWT | 3.36 | 1.03 | c09 | 3x7.2 | 2.37 | 0.98 | 9.32 | BT | EXCEED |
|  |  | 2 | 13.2 | Off |  | NWT | 3.36 | 1.03 | c15 | 2x7.2 | 2.41 | 0.94 | 9.11 | BT | EXCEED |
|  | c26 | 1 | 16.4 | Off |  | NWT | 3.78 | 0.92 | c10 | 4x7.2 | 2.36 | 1 | 14.09 | BT | EXCEED |
|  |  | 2 | 16.4 | Off |  | NWT | 3.78 | 0.92 | c15 | 2x7.2 | 2.41 | 0.94 | 13.97 | BT | EXCEED |
|  | c27 | 1 | 24.4 | Off |  | NWT | 3.98 | 0.89 | c11 | 4x8 | 2.43 | 0.91 | 16.43 | BT | EXCEED |
|  |  | 2 | 24.4 | Off |  | NWT | 3.98 | 0.89 | c16 | 2x8 | 2.57 | 1.05 | 13.84 | BT | EXCEED |
|  | c28 | 1 | 32 | Off |  | NWT | 4.04 | 0.92 | c12 | 4x9.6 | 3.54 | 1 | 4.94 | BT | EXCEED |
|  |  | 2 | 32 | Off |  | NWT | 4.04 | 0.92 | c17 | 2x9.6 | 3.62 | 1.16 | 3.78 | BT | EXCEED |
|  | c29 | 1 | 48 | Off |  | NWT | 4.46 | 0.74 | c13 | 4x16.4 | 4.13 | 0.92 | 3.73 | BT | EXCEED |
|  |  | 2 | 48 | Off |  | NWT | 4.46 | 0.74 | c18 | 2x16.4 | 4.46 | 0.68 | 0.07 | NWT | PASS |
|  | c30 | 1 | 64 | Off |  | NWT | 4.75 | 0.48 | c14 | 4x24.4 | 4.42 | 0.72 | 5.17 | BT | EXCEED |
|  |  | 2 | 64 | Off |  | NWT | 4.75 | 0.48 | c19 | 2x24.4 | 4.58 | 0.66 | 2.74 | BT | EXCEED |
|  | c31 | 1 | 80 | Off |  | NWT | 4.74 | 0.51 | c14 | 4x24.4 | 4.42 | 0.72 | 5 | BT | EXCEED |
|  |  | 2 | 80 | Off |  | NWT | 4.74 | 0.51 | c19 | 2x24.4 | 4.58 | 0.66 | 2.6 | BT | EXCEED |
|  | c32 | 1 | 13.2 | Off | 5% | NWT | 3.09 | 1.07 | c20 | 3x7.2 | 2.05 | 0.86 | 10.16 | BT | EXCEED |
|  | c33 | 1 | 16.4 | Off | 5% | NWT | 3.27 | 1.04 | c21 | 4x7.2 | 2.11 | 0.9 | 11.3 | BT | EXCEED |
|  | c34 | 1 | 24.4 | Off | 5% | NWT | 3.51 | 1.01 | c22 | 4x8 | 2.26 | 0.88 | 12.51 | BT | EXCEED |
|  | c35 | 1 | 48 | Off | 5% | NWT | 4.23 | 0.86 | c23 | 4x16.4 | 3.9 | 0.94 | 3.51 | BT | EXCEED |
|  | c36 | 1 | 64 | Off | 5% | NWT | 4.34 | 0.76 | c24 | 4x24.4 | 4.14 | 0.89 | 2.36 | BT | EXCEED |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 19: Summary of the results of P800-8

| Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- |
| a | c25 | 13.2 | Off |  | NWT c09 OR NWT c15 | EXCEED |
|  | c26 | 16.4 | Off |  | NWT c10 OR NWT c15 | EXCEED |
|  | c27 | 24.4 | Off |  | NWT c11 OR NWT c16 | EXCEED |
|  | c28 | 32 | Off |  | NWT c12 OR NWT c17 | EXCEED |
|  | c29 | 48 | Off |  | NWT c13 OR NWT c18 | PASS |
|  | c30 | 64 | Off |  | NWT c14 OR NWT c19 | PASS |
|  | c31 | 80 | Off |  | NWT c14 OR NWT c19 | PASS |
|  | c32 | 13.2 | Off | 5% | NWT c20 | EXCEED |
|  | c33 | 16.4 | Off | 5% | NWT c21 | EXCEED |
|  | c34 | 24.4 | Off | 5% | NWT c22 | EXCEED |
|  | c35 | 48 | Off | 5% | NWT c23 | EXCEED |
|  | c36 | 64 | Off | 5% | NWT c24 | PASS |
| b | c25 | 13.2 | Off |  | NWT c09 OR NWT c15 | EXCEED |
|  | c26 | 16.4 | Off |  | NWT c10 OR NWT c15 | EXCEED |
|  | c27 | 24.4 | Off |  | NWT c11 OR NWT c16 | EXCEED |
|  | c28 | 32 | Off |  | NWT c12 OR NWT c17 | EXCEED |
|  | c29 | 48 | Off |  | NWT c13 OR NWT c18 | PASS |
|  | c30 | 64 | Off |  | NWT c14 OR NWT c19 | EXCEED |
|  | c31 | 80 | Off |  | NWT c14 OR NWT c19 | EXCEED |
|  | c32 | 13.2 | Off | 5% | NWT c20 | EXCEED |
|  | c33 | 16.4 | Off | 5% | NWT c21 | EXCEED |
|  | c34 | 24.4 | Off | 5% | NWT c22 | EXCEED |
|  | c35 | 48 | Off | 5% | NWT c23 | EXCEED |
|  | c36 | 64 | Off | 5% | NWT c24 | EXCEED |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 8: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-8

## P800-9 (MASA, Speech+Background)

The complete statistical evaluation of the requirement ToR tests for experiment P800-9 is given in the following table. The evaluation is done separately for the data from the two listening laboratories.

Table 20: Statistical overview on the results of P800-9

|  |  | Type | CuT |  |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | DTX | Req. | MOS | Std. | Cond. | Bitrate | MOS | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |  |
| a | c24 | 1 | 13.2 | off | NWT | 3.46 | 1.1 | c09 | 3x7.2 | 3.44 | 1.1 | 0.15 | NWT | PASS |
|  | c25 | 1 | 16.4 | off | NWT | 3.69 | 0.97 | c10 | 4x7.2 | 3.47 | 1.11 | 2.07 | BT | EXCEED |
|  | c26 | 1 | 24.4 | off | NWT | 4.23 | 0.86 | c11 | 4x8 | 3.68 | 1.01 | 5.5 | BT | EXCEED |
|  | c27 | 1 | 32 | off | NWT | 4.31 | 0.79 | c12 | 4x9.6 | 4.04 | 0.9 | 3.04 | BT | EXCEED |
|  | c28 | 1 | 48 | off | NWT | 4.39 | 0.83 | c13 | 4x16.4 | 4.38 | 0.75 | 0.07 | NWT | PASS |
|  | c29 | 1 | 64 | off | NWT | 4.44 | 0.82 | c14 | 4x24.4 | 4.49 | 0.71 | -0.62 | NWT | PASS |
|  | c30 | 1 | 80 | off | NWT | 4.44 | 0.78 | c14 | 4x24.4 | 4.49 | 0.71 | -0.63 | NWT | PASS |
|  | c31 | 1 | 13.2 | on | NWT | 3.37 | 1.12 | c18 | 3x7.2 | 3.33 | 1.09 | 0.33 | NWT | PASS |
|  |  | 2 | 13.2 | on | NWT | 3.37 | 1.12 | c15 | 2x7.2 | 3.35 | 1.06 | 0.15 | NWT | PASS |
|  | c32 | 1 | 16.4 | on | NWT | 3.8 | 0.95 | c19 | 4x7.2 | 3.33 | 1.09 | 4.36 | BT | EXCEED |
|  |  | 2 | 16.4 | on | NWT | 3.8 | 0.95 | c15 | 2x7.2 | 3.35 | 1.06 | 4.22 | BT | EXCEED |
|  | c33 | 1 | 24.4 | on | NWT | 4.09 | 0.91 | c20 | 4x8 | 3.33 | 1.05 | 7.33 | BT | EXCEED |
|  | c34 | 1 | 32 | on | NWT | 4.04 | 0.96 | c21 | 4x9.6 | 4.06 | 0.92 | -0.17 | NWT | PASS |
|  |  | 2 | 32 | on | NWT | 4.04 | 0.96 | c16 | 2x9.6 | 4.16 | 0.91 | -1.19 | NWT | PASS |
|  | c35 | 1 | 48 | on | NWT | 4.29 | 0.8 | c22 | 4x16.4 | 4.31 | 0.77 | -0.21 | NWT | PASS |
|  |  | 2 | 48 | on | NWT | 4.29 | 0.8 | c17 | 2x16.4 | 4.33 | 0.78 | -0.53 | NWT | PASS |
|  | c36 | 1 | 64 | on | NWT | 4.4 | 0.79 | c23 | 4x24.4 | 4.34 | 0.8 | 0.67 | NWT | PASS |
| d | c24 | 1 | 13.2 | off | NWT | 3.44 | 0.89 | c09 | 3x7.2 | 2.49 | 0.8 | 10.57 | BT | EXCEED |
|  | c25 | 1 | 16.4 | off | NWT | 3.78 | 0.92 | c10 | 4x7.2 | 2.66 | 0.88 | 11.79 | BT | EXCEED |
|  | c26 | 1 | 24.4 | off | NWT | 3.71 | 0.86 | c11 | 4x8 | 2.67 | 0.87 | 11.35 | BT | EXCEED |
|  | c27 | 1 | 32 | off | NWT | 3.84 | 0.76 | c12 | 4x9.6 | 3.73 | 0.78 | 1.36 | NWT | PASS |
|  | c28 | 1 | 48 | off | NWT | 4.28 | 0.64 | c13 | 4x16.4 | 4.15 | 0.78 | 1.71 | BT | EXCEED |
|  | c29 | 1 | 64 | off | NWT | 4.24 | 0.71 | c14 | 4x24.4 | 4.32 | 0.74 | -1.02 | NWT | PASS |
|  | c30 | 1 | 80 | off | NWT | 4.29 | 0.67 | c14 | 4x24.4 | 4.32 | 0.74 | -0.38 | NWT | PASS |
|  | c31 | 1 | 13.2 | on | NWT | 3.25 | 0.88 | c18 | 3x7.2 | 2.52 | 0.8 | 8.24 | BT | EXCEED |
|  |  | 2 | 13.2 | on | NWT | 3.25 | 0.88 | c15 | 2x7.2 | 2.46 | 0.88 | 8.49 | BT | EXCEED |
|  | c32 | 1 | 16.4 | on | NWT | 3.58 | 0.86 | c19 | 4x7.2 | 2.57 | 0.86 | 11.13 | BT | EXCEED |
|  |  | 2 | 16.4 | on | NWT | 3.58 | 0.86 | c15 | 2x7.2 | 2.46 | 0.88 | 12.25 | BT | EXCEED |
|  | c33 | 1 | 24.4 | on | NWT | 3.65 | 0.84 | c20 | 4x8 | 2.61 | 0.87 | 11.63 | BT | EXCEED |
|  | c34 | 1 | 32 | on | NWT | 3.86 | 0.74 | c21 | 4x9.6 | 3.73 | 0.91 | 1.46 | NWT | PASS |
|  |  | 2 | 32 | on | NWT | 3.86 | 0.74 | c16 | 2x9.6 | 3.83 | 0.83 | 0.34 | NWT | PASS |
|  | c35 | 1 | 48 | on | NWT | 4.06 | 0.78 | c22 | 4x16.4 | 3.89 | 0.91 | 1.88 | BT | EXCEED |
|  |  | 2 | 48 | on | NWT | 4.06 | 0.78 | c17 | 2x16.4 | 3.96 | 0.81 | 1.14 | NWT | PASS |
|  | c36 | 1 | 64 | on | NWT | 4.22 | 0.73 | c23 | 4x24.4 | 4.09 | 0.79 | 1.6 | NWT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 21: Summary of the results of P800-9

| Lab | Cond. | Bitrate | DTX | ToR | Status |
| --- | --- | --- | --- | --- | --- |
| a | c24 | 13.2 | off | NWT c09 | PASS |
|  | c25 | 16.4 | off | NWT c10 | EXCEED |
|  | c26 | 24.4 | off | NWT c11 | EXCEED |
|  | c27 | 32 | off | NWT c12 | EXCEED |
|  | c28 | 48 | off | NWT c13 | PASS |
|  | c29 | 64 | off | NWT c14 | PASS |
|  | c30 | 80 | off | NWT c14 | PASS |
|  | c31 | 13.2 | on | NWT c18 or NWT c15 | PASS |
|  | c32 | 16.4 | on | NWT c19 or NWT c15 | EXCEED |
|  | c33 | 24.4 | on | NWT c20 | EXCEED |
|  | c34 | 32 | on | NWT c21 or NWT c16 | PASS |
|  | c35 | 48 | on | NWT c22 or NWT c17 | PASS |
|  | c36 | 64 | on | NWT c23 | PASS |
| d | c24 | 13.2 | off | NWT c09 | EXCEED |
|  | c25 | 16.4 | off | NWT c10 | EXCEED |
|  | c26 | 24.4 | off | NWT c11 | EXCEED |
|  | c27 | 32 | off | NWT c12 | PASS |
|  | c28 | 48 | off | NWT c13 | EXCEED |
|  | c29 | 64 | off | NWT c14 | PASS |
|  | c30 | 80 | off | NWT c14 | PASS |
|  | c31 | 13.2 | on | NWT c18 or NWT c15 | EXCEED |
|  | c32 | 16.4 | on | NWT c19 or NWT c15 | EXCEED |
|  | c33 | 24.4 | on | NWT c20 | EXCEED |
|  | c34 | 32 | on | NWT c21 or NWT c16 | PASS |
|  | c35 | 48 | on | NWT c22 or NWT c17 | PASS |
|  | c36 | 64 | on | NWT c23 | PASS |

The relation between the results for the two different languages and listening laboratories is illustrated by the following scatter plot. In this plot, only the test and not the anchor conditions are depicted.

Figure 9: Scatter plot of averaged test condition ratings for the two listening laboratories/languages of experiment P800-9

## Result overview

Across all P.800 experiments, there are 226 requirement ToR tests. The CuT failed four of them, it passed 166 times and exceeded the requirement 56 times. The four failed tests are all in P800-6.

Table 22: Overview of all failed tests in P.800 experiments

| Exp. | Lab | Cond. | Bitrate | DTX | FER | ToR | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P800-6 | a | c29 | 64 | Off |  | NWT C16 | FAIL |
|  |  | c31 | 16.4 | Off | 5% | NWT C18 | FAIL |
|  |  | c36 | 24.4 | On |  | NWT C23 | FAIL |
|  | c | c28 | 48 | Off |  | NWT C15 | FAIL |

# Results for BS.1534 MUSHRA experiments

## Overview

These listening tests were conducted with experienced listeners by three of the four listening laboratories. Each experiment was done at two different listening laboratories, an overview on the experiments and their allocation can be found in Tables 4 and 5 in IVAS-8a [1].

In the following, the individual results of the 28 experiments are presented along with combined results of experiments at different laboratories for the same experiment conditions. It is not explicitly stated in [1] if and how to combine data from different laboratories. The GAL also did not know whether the samples from the same experiment in each laboratory were partially or completely the same. Since there is no other known systematic difference (like the language in the P.800 experiments) between the data sets for the listening laboratories in the BS.1534 experiments, a combination of the data sets is analysed here as well.

For all data sets, the results of the statistical analyses are provided and the comparison with the ToR criteria is done. The confidence level t(M) for the statistical analyses is 1.654 or 1.649 due to the 168 or 336 votes per condition for these experiments when analysing the results from the listening laboratories individually or jointly, respectively.

As for the P.800-experiments, the full results for each experiment are available as an electronic attachment to this document.

BS1534-[X]_[L].xlsx

With:

X: BS1534 experiment identifier as per IVAS-8a (e.g., X=2A  BS1534-2A).

L: Lab identifier. The results are provided for both labs individually as well as combined for both, in that case, the identifier is [Lab identifier of the first LL + lab identifier of the second LL]

Example: 	BS1534-2A_a.xlsx for the results from LL a
BS1534-2A_b.xlsx for the results from LL b
BS1534-2A_b+a.xlsx for the combined results from LL b and LL a

Each Excel document includes three sheets:

-	Raw: The individual votes of each subject per listening sample.

-	Condition+Sample: Raw results aggregated for each sample and condition (14 votes).

-	Condition: Raw results aggregated for each condition (168 votes).

Standard deviations and confidence intervals for aggregated results were calculated according to Recommendation ITU-T P.1401 [4].

## BS1534-1a (Stereo, Headphones)

The averaged results per condition for experiment BS1534-1a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 10: Averaged results per condition for experiment BS1534-1a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-1a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 23: Statistical overview on the results of BS1534-1a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 48 | NWT | 63.8 | 27.3 | c04 | 2x32 | 48.9 | 29.2 | 4.81 | BT | EXCEED |
|  |  | 2 | 48 | BT | 63.8 | 27.3 | c03 | 2x24.4 | 42.9 | 28.1 | 6.92 | BT | PASS |
|  | c07 | 1 | 64 | NWT | 75.2 | 22.8 | c05 | 2x48 | 83.6 | 19.4 | -3.64 | WT | FAIL |
|  |  | 2 | 64 | BT | 75.2 | 22.8 | c04 | 2x32 | 48.9 | 29.2 | 9.18 | BT | PASS |
| d | c06 | 1 | 48 | NWT | 86.5 | 13.3 | c04 | 2x32 | 70.7 | 23.2 | 7.66 | BT | EXCEED |
|  |  | 2 | 48 | BT | 86.5 | 13.3 | c03 | 2x24.4 | 71.4 | 21.4 | 7.76 | BT | PASS |
|  | c07 | 1 | 64 | NWT | 86.8 | 11.6 | c05 | 2x48 | 89.5 | 11.8 | -2.1 | WT | FAIL |
|  |  | 2 | 64 | BT | 86.8 | 11.6 | c04 | 2x32 | 70.7 | 23.2 | 8.08 | BT | PASS |
| d+a | c06 | 1 | 48 | NWT | 75.1 | 24.3 | c04 | 2x32 | 59.8 | 28.5 | 7.5 | BT | EXCEED |
|  |  | 2 | 48 | BT | 75.1 | 24.3 | c03 | 2x24.4 | 57.1 | 28.7 | 8.77 | BT | PASS |
|  | c07 | 1 | 64 | NWT | 81 | 19 | c05 | 2x48 | 86.6 | 16.3 | -4.06 | WT | FAIL |
|  |  | 2 | 64 | BT | 81 | 19 | c04 | 2x32 | 59.8 | 28.5 | 11.36 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 24: Summary of the results of BS1534-1a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 48 | NWT c04 OR BT c03 | PASS |
|  | c07 | 64 | NWT c05 OR BT c04 | PASS |
| d | c06 | 48 | NWT c04 OR BT c03 | PASS |
|  | c07 | 64 | NWT c05 OR BT c04 | PASS |
| d+a | c06 | 48 | NWT c04 OR BT c03 | PASS |
|  | c07 | 64 | NWT c05 OR BT c04 | PASS |

## BS1534-1b (Stereo, Headphones)

The averaged results per condition for experiment BS1534-1b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 11: Averaged results per condition for experiment BS1534-1b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-1b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 25: Statistical overview on the results of BS1534-1b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c06 | 1 | 96 | NWT | 94.4 | 9 | c04 | 2x64 | 93.1 | 11.9 | 1.13 | NWT | PASS |
|  |  | 2 | 96 | BT | 94.4 | 9 | c03 | 2x48 | 94 | 9.5 | 0.37 | NWT | FAIL |
|  | c07 | 1 | 128 | NWT | 95 | 8.5 | c05 | 2x96 | 93.9 | 10.9 | 0.98 | NWT | PASS |
|  |  | 2 | 128 | BT | 95 | 8.5 | c04 | 2x64 | 93.1 | 11.9 | 1.68 | BT | PASS |
| d | c06 | 1 | 96 | NWT | 89.2 | 8.9 | c04 | 2x64 | 87.2 | 10.5 | 1.8 | BT | EXCEED |
|  |  | 2 | 96 | BT | 89.2 | 8.9 | c03 | 2x48 | 89.4 | 8 | -0.3 | NWT | FAIL |
|  | c07 | 1 | 128 | NWT | 90.4 | 7.5 | c05 | 2x96 | 90.2 | 8.7 | 0.26 | NWT | PASS |
|  |  | 2 | 128 | BT | 90.4 | 7.5 | c04 | 2x64 | 87.2 | 10.5 | 3.16 | BT | PASS |
| d+b | c06 | 1 | 96 | NWT | 91.8 | 9.3 | c04 | 2x64 | 90.2 | 11.6 | 1.98 | BT | EXCEED |
|  |  | 2 | 96 | BT | 91.8 | 9.3 | c03 | 2x48 | 91.7 | 9 | 0.06 | NWT | FAIL |
|  | c07 | 1 | 128 | NWT | 92.7 | 8.3 | c05 | 2x96 | 92.1 | 10 | 0.9 | NWT | PASS |
|  |  | 2 | 128 | BT | 92.7 | 8.3 | c04 | 2x64 | 90.2 | 11.6 | 3.24 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 26: Summary of the results of BS1534-1b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |
| d | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |
| d+b | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |

## BS1534-2a (5.1, 5.1)

The averaged results per condition for experiment BS1534-2a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 12: Averaged results per condition for experiment BS1534-2a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-2a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 27: Statistical overview on the results of BS1534-2a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 64 | NWT | 61.4 | 24.2 | c04 | 5x16.4 | 54.8 | 28.3 | 2.29 | BT | EXCEED |
|  |  | 2 | 64 | BT | 61.4 | 24.2 | c03 | 5x13.2 | 36.1 | 25.5 | 9.35 | BT | PASS |
|  | c07 | 1 | 96 | NWT | 80.6 | 21.9 | c05 | 5x24.4 | 64.7 | 26.7 | 5.95 | BT | EXCEED |
|  |  | 2 | 96 | BT | 80.6 | 21.9 | c04 | 5x16.4 | 54.8 | 28.3 | 9.32 | BT | PASS |
| b | c06 | 1 | 64 | NWT | 85.6 | 19 | c04 | 5x16.4 | 76.6 | 24.8 | 3.71 | BT | EXCEED |
|  |  | 2 | 64 | BT | 85.6 | 19 | c03 | 5x13.2 | 65.2 | 28.9 | 7.63 | BT | PASS |
|  | c07 | 1 | 96 | NWT | 88.9 | 15.5 | c05 | 5x24.4 | 86.1 | 18.6 | 1.51 | NWT | PASS |
|  |  | 2 | 96 | BT | 88.9 | 15.5 | c04 | 5x16.4 | 76.6 | 24.8 | 5.45 | BT | PASS |
| b+a | c06 | 1 | 64 | NWT | 73.5 | 24.9 | c04 | 5x16.4 | 65.7 | 28.7 | 3.75 | BT | EXCEED |
|  |  | 2 | 64 | BT | 73.5 | 24.9 | c03 | 5x13.2 | 50.6 | 30.9 | 10.57 | BT | PASS |
|  | c07 | 1 | 96 | NWT | 84.7 | 19.4 | c05 | 5x24.4 | 75.4 | 25.3 | 5.36 | BT | EXCEED |
|  |  | 2 | 96 | BT | 84.7 | 19.4 | c04 | 5x16.4 | 65.7 | 28.7 | 10.05 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 28: Summary of the results of BS1534-2a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 64 | NWT c04 OR BT c03 | PASS |
|  | c07 | 96 | NWT c05 OR BT c04 | PASS |
| b | c06 | 64 | NWT c04 OR BT c03 | PASS |
|  | c07 | 96 | NWT c05 OR BT c04 | PASS |
| b+a | c06 | 64 | NWT c04 OR BT c03 | PASS |
|  | c07 | 96 | NWT c05 OR BT c04 | PASS |

## BS1534-2b (5.1, 5.1)

The averaged results per condition for experiment BS1534-2b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 13: Averaged results per condition for experiment BS1534-2b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-2b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 29: Statistical overview on the results of BS1534-2b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 128 | NWT | 74.9 | 21.4 | c04 | 5x32 | 58.9 | 24.6 | 6.36 | BT | EXCEED |
|  |  | 2 | 128 | BT | 74.9 | 21.4 | c03 | 5x24.4 | 49.9 | 24.1 | 10.07 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 80.2 | 18.8 | c05 | 5x48 | 81 | 20.2 | -0.34 | NWT | PASS |
|  |  | 2 | 160 | BT | 80.2 | 18.8 | c04 | 5x32 | 58.9 | 24.6 | 8.93 | BT | PASS |
| b | c06 | 1 | 128 | NWT | 92.8 | 8.2 | c04 | 5x32 | 88.8 | 15.7 | 2.96 | BT | EXCEED |
|  |  | 2 | 128 | BT | 92.8 | 8.2 | c03 | 5x24.4 | 81.4 | 22.3 | 6.23 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 93.6 | 7.4 | c05 | 5x48 | 93.1 | 8.6 | 0.56 | NWT | PASS |
|  |  | 2 | 160 | BT | 93.6 | 7.4 | c04 | 5x32 | 88.8 | 15.7 | 3.59 | BT | PASS |
| b+a | c06 | 1 | 128 | NWT | 83.9 | 18.5 | c04 | 5x32 | 73.8 | 25.5 | 5.84 | BT | EXCEED |
|  |  | 2 | 128 | BT | 83.9 | 18.5 | c03 | 5x24.4 | 65.6 | 28 | 9.94 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 86.9 | 15.7 | c05 | 5x48 | 87 | 16.7 | -0.09 | NWT | PASS |
|  |  | 2 | 160 | BT | 86.9 | 15.7 | c04 | 5x32 | 73.8 | 25.5 | 8.01 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 30: Summary of the results of BS1534-2b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |
| b | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |
| b+a | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |

## BS1534-3a (7.1.4, 7.1.4)

The averaged results per condition for experiment BS1534-3a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 14: Averaged results per condition for experiment BS1534-3a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-3a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 31: Statistical overview on the results of BS1534-3a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 128 | NWT | 51.5 | 26.3 | c04 | 11x13.2 | 46.8 | 26.7 | 1.61 | NWT | PASS |
|  |  | 2 | 128 | BT | 51.5 | 26.3 | c03 | 11x9.6 | 39.8 | 23.9 | 4.25 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 77.5 | 17.7 | c05 | 11x16.4 | 61.1 | 24.6 | 7.01 | BT | EXCEED |
|  |  | 2 | 160 | BT | 77.5 | 17.7 | c04 | 11x13.2 | 46.8 | 26.7 | 12.42 | BT | PASS |
| b | c06 | 1 | 128 | NWT | 85.2 | 19.8 | c04 | 11x13.2 | 72.2 | 27.3 | 4.99 | BT | EXCEED |
|  |  | 2 | 128 | BT | 85.2 | 19.8 | c03 | 11x9.6 | 64.9 | 28.2 | 7.64 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 90.6 | 12.3 | c05 | 11x16.4 | 84.7 | 18.9 | 3.41 | BT | EXCEED |
|  |  | 2 | 160 | BT | 90.6 | 12.3 | c04 | 11x13.2 | 72.2 | 27.3 | 7.95 | BT | PASS |
| b+a | c06 | 1 | 128 | NWT | 68.4 | 28.8 | c04 | 11x13.2 | 59.5 | 29.8 | 3.91 | BT | EXCEED |
|  |  | 2 | 128 | BT | 68.4 | 28.8 | c03 | 11x9.6 | 52.4 | 29 | 7.18 | BT | PASS |
|  | c07 | 1 | 160 | NWT | 84.1 | 16.6 | c05 | 11x16.4 | 72.9 | 24.9 | 6.84 | BT | EXCEED |
|  |  | 2 | 160 | BT | 84.1 | 16.6 | c04 | 11x13.2 | 59.5 | 29.8 | 13.18 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 32: Summary of the results of BS1534-3a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |
| b | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |
| b+a | c06 | 128 | NWT c04 OR BT c03 | PASS |
|  | c07 | 160 | NWT c05 OR BT c04 | PASS |

## BS1534-3b (7.1.4, 7.1.4)

The averaged results per condition for experiment BS1534-3b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 15: Averaged results per condition for experiment BS1534-3b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-3b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 33: Statistical overview on the results of BS1534-3b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 384 | NWT | 87.3 | 16 | c04 | 11x48 | 85.9 | 17.6 | 0.72 | NWT | PASS |
|  |  | 2 | 384 | BT | 87.3 | 16 | c03 | 11x32 | 63.6 | 25 | 10.32 | BT | PASS |
|  | c07 | 1 | 512 | NWT | 88.6 | 15.8 | c05 | 11x64 | 88.7 | 16 | -0.04 | NWT | PASS |
|  |  | 2 | 512 | BT | 88.6 | 15.8 | c04 | 11x48 | 85.9 | 17.6 | 1.48 | NWT | FAIL |
| b | c06 | 1 | 384 | NWT | 90.1 | 11.5 | c04 | 11x48 | 89.3 | 12.4 | 0.6 | NWT | PASS |
|  |  | 2 | 384 | BT | 90.1 | 11.5 | c03 | 11x32 | 82.9 | 20.1 | 4.04 | BT | PASS |
|  | c07 | 1 | 512 | NWT | 91.5 | 10.1 | c05 | 11x64 | 93.3 | 8 | -1.76 | WT | FAIL |
|  |  | 2 | 512 | BT | 91.5 | 10.1 | c04 | 11x48 | 89.3 | 12.4 | 1.8 | BT | PASS |
| b+a | c06 | 1 | 384 | NWT | 88.7 | 14 | c04 | 11x48 | 87.6 | 15.3 | 0.94 | NWT | PASS |
|  |  | 2 | 384 | BT | 88.7 | 14 | c03 | 11x32 | 73.3 | 24.6 | 9.99 | BT | PASS |
|  | c07 | 1 | 512 | NWT | 90.1 | 13.3 | c05 | 11x64 | 91 | 12.8 | -0.9 | NWT | PASS |
|  |  | 2 | 512 | BT | 90.1 | 13.3 | c04 | 11x48 | 87.6 | 15.3 | 2.23 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 34: Summary of the results of BS1534-3b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 384 | NWT c04 OR BT c03 | PASS |
|  | c07 | 512 | NWT c05 OR BT c04 | PASS |
| b | c06 | 384 | NWT c04 OR BT c03 | PASS |
|  | c07 | 512 | NWT c05 OR BT c04 | PASS |
| b+a | c06 | 384 | NWT c04 OR BT c03 | PASS |
|  | c07 | 512 | NWT c05 OR BT c04 | PASS |

## BS1534-4a (FOA, Headphones)

The averaged results per condition for experiment BS1534-4a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 16: Averaged results per condition for experiment BS1534-4a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-4a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 35: Statistical overview on the results of BS1534-4a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 96 | NWT | 67.7 | 25 | c04 | 4x32 | 53.4 | 27.2 | 5 | BT | EXCEED |
|  |  | 2 | 96 | BT | 67.7 | 25 | c03 | 4x24.4 | 47.3 | 28.7 | 6.93 | BT | PASS |
|  | c07 | 1 | 128 | NWT | 72.3 | 23.6 | c05 | 4x48 | 68.4 | 23 | 1.53 | NWT | PASS |
|  |  | 2 | 128 | BT | 72.3 | 23.6 | c04 | 4x32 | 53.4 | 27.2 | 6.78 | BT | PASS |
|  | c08 | 1 | 160 | NWT | 75.4 | 21 | c05 | 4x48 | 68.4 | 23 | 2.93 | BT | EXCEED |
|  |  | 2 | 160 | BT | 75.4 | 21 | c04 | 4x32 | 53.4 | 27.2 | 8.28 | BT | PASS |
| d | c06 | 1 | 96 | NWT | 84.3 | 13.4 | c04 | 4x32 | 60.5 | 22.6 | 11.74 | BT | EXCEED |
|  |  | 2 | 96 | BT | 84.3 | 13.4 | c03 | 4x24.4 | 65.1 | 19 | 10.68 | BT | PASS |
|  | c07 | 1 | 128 | NWT | 85.7 | 12.3 | c05 | 4x48 | 87.5 | 10.3 | -1.48 | NWT | PASS |
|  |  | 2 | 128 | BT | 85.7 | 12.3 | c04 | 4x32 | 60.5 | 22.6 | 12.67 | BT | PASS |
|  | c08 | 1 | 160 | NWT | 89.2 | 8.7 | c05 | 4x48 | 87.5 | 10.3 | 1.6 | NWT | PASS |
|  |  | 2 | 160 | BT | 89.2 | 8.7 | c04 | 4x32 | 60.5 | 22.6 | 15.33 | BT | PASS |
| d+a | c06 | 1 | 96 | NWT | 76 | 21.7 | c04 | 4x32 | 57 | 25.2 | 10.48 | BT | EXCEED |
|  |  | 2 | 96 | BT | 76 | 21.7 | c03 | 4x24.4 | 56.2 | 25.9 | 10.72 | BT | PASS |
|  | c07 | 1 | 128 | NWT | 79 | 19.9 | c05 | 4x48 | 77.9 | 20.2 | 0.67 | NWT | PASS |
|  |  | 2 | 128 | BT | 79 | 19.9 | c04 | 4x32 | 57 | 25.2 | 12.53 | BT | PASS |
|  | c08 | 1 | 160 | NWT | 82.3 | 17.5 | c05 | 4x48 | 77.9 | 20.2 | 2.99 | BT | EXCEED |
|  |  | 2 | 160 | BT | 82.3 | 17.5 | c04 | 4x32 | 57 | 25.2 | 15.12 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 36: Summary of the results of BS1534-4a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |
|  | c08 | 160 | NWT c05 OR BT c04 | PASS |
| d | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |
|  | c08 | 160 | NWT c05 OR BT c04 | PASS |
| d+a | c06 | 96 | NWT c04 OR BT c03 | PASS |
|  | c07 | 128 | NWT c05 OR BT c04 | PASS |
|  | c08 | 160 | NWT c05 OR BT c04 | PASS |

## BS1534-4b (HOA2, Headphones)

The averaged results per condition for experiment BS1534-4b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 17: Averaged results per condition for experiment BS1534-4b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-4b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 37: Statistical overview on the results of BS1534-4b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c06 | 1 | 160 | NWT | 90 | 13.1 | c04 | 4x48 | 89.1 | 13.3 | 0.63 | NWT | PASS |
|  |  | 2 | 160 | BT | 90 | 13.1 | c03 | 4x32 | 82.2 | 20.2 | 4.21 | BT | PASS |
|  | c07 | 1 | 192 | NWT | 91 | 13.1 | c05 | 4x64 | 90.4 | 13.3 | 0.4 | NWT | PASS |
|  |  | 2 | 192 | BT | 91 | 13.1 | c04 | 4x48 | 89.1 | 13.3 | 1.29 | NWT | FAIL |
| d | c06 | 1 | 160 | NWT | 88.4 | 8.3 | c04 | 4x48 | 85.7 | 11.2 | 2.54 | BT | EXCEED |
|  |  | 2 | 160 | BT | 88.4 | 8.3 | c03 | 4x32 | 70 | 21.6 | 10.29 | BT | PASS |
|  | c07 | 1 | 192 | NWT | 90.3 | 8.9 | c05 | 4x64 | 87.8 | 10.1 | 2.4 | BT | EXCEED |
|  |  | 2 | 192 | BT | 90.3 | 8.9 | c04 | 4x48 | 85.7 | 11.2 | 4.16 | BT | PASS |
| d+b | c06 | 1 | 160 | NWT | 89.2 | 11 | c04 | 4x48 | 87.4 | 12.4 | 2.01 | BT | EXCEED |
|  |  | 2 | 160 | BT | 89.2 | 11 | c03 | 4x32 | 76.1 | 21.8 | 9.85 | BT | PASS |
|  | c07 | 1 | 192 | NWT | 90.6 | 11.2 | c05 | 4x64 | 89.1 | 11.9 | 1.72 | BT | EXCEED |
|  |  | 2 | 192 | BT | 90.6 | 11.2 | c04 | 4x48 | 87.4 | 12.4 | 3.54 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 38: Summary of the results of BS1534-4b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c06 | 160 | NWT c04 OR BT c03 | PASS |
|  | c07 | 192 | NWT c05 OR BT c04 | PASS |
| d | c06 | 160 | NWT c04 OR BT c03 | PASS |
|  | c07 | 192 | NWT c05 OR BT c04 | PASS |
| d+b | c06 | 160 | NWT c04 OR BT c03 | PASS |
|  | c07 | 192 | NWT c05 OR BT c04 | PASS |

## BS1534-5a (HOA3, Headphones)

The averaged results per condition for experiment BS1534-5a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 18: Averaged results per condition for experiment BS1534-5a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-5a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 39: Statistical overview on the results of BS1534-5a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c06 | 1 | 192 | NWT | 69.8 | 24.4 | c04 | 4x64 | 67.1 | 24.5 | 1.03 | NWT | PASS |
|  |  | 2 | 192 | BT | 69.8 | 24.4 | c03 | 4x48 | 58.8 | 23.6 | 4.22 | BT | PASS |
|  | c07 | 1 | 256 | NWT | 80.6 | 20.1 | c05 | 4x96 | 73 | 21.6 | 3.34 | BT | EXCEED |
|  |  | 2 | 256 | BT | 80.6 | 20.1 | c04 | 4x64 | 67.1 | 24.5 | 5.54 | BT | PASS |
| d | c06 | 1 | 192 | NWT | 88.1 | 10.4 | c04 | 4x64 | 87.1 | 10 | 0.86 | NWT | PASS |
|  |  | 2 | 192 | BT | 88.1 | 10.4 | c03 | 4x48 | 86.7 | 10.1 | 1.22 | NWT | FAIL |
|  | c07 | 1 | 256 | NWT | 87 | 9.7 | c05 | 4x96 | 90.4 | 10 | -3.13 | WT | FAIL |
|  |  | 2 | 256 | BT | 87 | 9.7 | c04 | 4x64 | 87.1 | 10 | -0.08 | NWT | FAIL |
| d+a | c06 | 1 | 192 | NWT | 78.9 | 20.8 | c04 | 4x64 | 77.1 | 21.2 | 1.14 | NWT | PASS |
|  |  | 2 | 192 | BT | 78.9 | 20.8 | c03 | 4x48 | 72.7 | 22.9 | 3.67 | BT | PASS |
|  | c07 | 1 | 256 | NWT | 83.8 | 16.1 | c05 | 4x96 | 81.7 | 18.9 | 1.57 | NWT | PASS |
|  |  | 2 | 256 | BT | 83.8 | 16.1 | c04 | 4x64 | 77.1 | 21.2 | 4.64 | BT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 40: Summary of the results of BS1534-5a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c06 | 192 | NWT c04 OR BT c03 | PASS |
|  | c07 | 256 | NWT c05 OR BT c04 | PASS |
| d | c06 | 192 | NWT c04 OR BT c03 | PASS |
|  | c07 | 256 | NWT c05 OR BT c04 | FAIL |
| d+a | c06 | 192 | NWT c04 OR BT c03 | PASS |
|  | c07 | 256 | NWT c05 OR BT c04 | PASS |

## BS1534-5b (HOA3, 7.1.4)

The averaged results per condition for experiment BS1534-5b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 19: Averaged results per condition for experiment BS1534-5b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-5b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 41: Statistical overview on the results of BS1534-5b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| a | c05 | 1 | 384 | NWT | 84.5 | 16.8 | c04 | 4x128 | 71.6 | 18.1 | 6.78 | BT | EXCEED |
|  |  | 2 | 384 | BT | 84.5 | 16.8 | c03 | 4x96 | 70.4 | 19.3 | 7.15 | BT | PASS |
|  | c06 | 1 | 512 | NWT | 85.8 | 16.1 | c04 | 4x128 | 71.6 | 18.1 | 7.62 | BT | EXCEED |
| b | c05 | 1 | 384 | NWT | 93.6 | 9.5 | c04 | 4x128 | 88 | 14.4 | 4.24 | BT | EXCEED |
|  |  | 2 | 384 | BT | 93.6 | 9.5 | c03 | 4x96 | 89.1 | 13.3 | 3.55 | BT | PASS |
|  | c06 | 1 | 512 | NWT | 94.2 | 8.5 | c04 | 4x128 | 88 | 14.4 | 4.8 | BT | EXCEED |
| b+a | c05 | 1 | 384 | NWT | 89.1 | 14.4 | c04 | 4x128 | 79.8 | 18.3 | 7.32 | BT | EXCEED |
|  |  | 2 | 384 | BT | 89.1 | 14.4 | c03 | 4x96 | 79.8 | 19 | 7.15 | BT | PASS |
|  | c06 | 1 | 512 | NWT | 90 | 13.5 | c04 | 4x128 | 79.8 | 18.3 | 8.23 | BT | EXCEED |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 42: Summary of the results of BS1534-5b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| a | c05 | 384 | NWT c04 OR BT c03 | PASS |
|  | c06 | 512 | NWT c04 | EXCEED |
| b | c05 | 384 | NWT c04 OR BT c03 | PASS |
|  | c06 | 512 | NWT c04 | EXCEED |
| b+a | c05 | 384 | NWT c04 OR BT c03 | PASS |
|  | c06 | 512 | NWT c04 | EXCEED |

## BS1534-6a (Objects, Headphones)

The averaged results per condition for experiment BS1534-6a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 20: Averaged results per condition for experiment BS1534-6a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-6a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 43: Statistical overview on the results of BS1534-6a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c06 | 1 | 48 | NWT | 91.4 | 10.2 | c03 | 3x16.4 | 88.6 | 14.6 | 1.97 | BT | EXCEED |
|  | c07 | 1 | 64 | NWT | 92.2 | 9.2 | c04 | 3x24.4 | 91.2 | 10.8 | 0.91 | NWT | PASS |
|  | c08 | 1 | 96 | NWT | 93.4 | 8.4 | c05 | 3x32 | 91.7 | 11.1 | 1.59 | NWT | PASS |
| d | c06 | 1 | 48 | NWT | 80.7 | 15.2 | c03 | 3x16.4 | 79.2 | 13.9 | 0.94 | NWT | PASS |
|  | c07 | 1 | 64 | NWT | 82 | 15.1 | c04 | 3x24.4 | 82.4 | 13.7 | -0.26 | NWT | PASS |
|  | c08 | 1 | 96 | NWT | 83.2 | 14.4 | c05 | 3x32 | 80.8 | 14.3 | 1.5 | NWT | PASS |
| d+b | c06 | 1 | 48 | NWT | 86 | 14 | c03 | 3x16.4 | 83.9 | 15 | 1.88 | BT | EXCEED |
|  | c07 | 1 | 64 | NWT | 87.1 | 13.5 | c04 | 3x24.4 | 86.8 | 13.1 | 0.28 | NWT | PASS |
|  | c08 | 1 | 96 | NWT | 88.3 | 12.9 | c05 | 3x32 | 86.2 | 13.9 | 1.97 | BT | EXCEED |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 44: Summary of the results of BS1534-6a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c06 | 48 | NWT c03 | EXCEED |
|  | c07 | 64 | NWT c04 | PASS |
|  | c08 | 96 | NWT c05 | PASS |
| d | c06 | 48 | NWT c03 | PASS |
|  | c07 | 64 | NWT c04 | PASS |
|  | c08 | 96 | NWT c05 | PASS |
| d+b | c06 | 48 | NWT c03 | EXCEED |
|  | c07 | 64 | NWT c04 | PASS |
|  | c08 | 96 | NWT c05 | EXCEED |

## BS1534-6b (Objects, Headphones)

The averaged results per condition for experiment BS1534-6b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 21: Averaged results per condition for experiment BS1534-6b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-6b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 45: Statistical overview on the results of BS1534-6b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c06 | 1 | 96 | NWT | 87.7 | 15.7 | c03 | 4x24.4 | 84.9 | 19.4 | 1.44 | NWT | PASS |
|  | c07 | 1 | 128 | NWT | 89 | 14 | c04 | 4x32 | 86 | 17.8 | 1.72 | BT | EXCEED |
|  | c08 | 1 | 256 | NWT | 91.7 | 11 | c05 | 4x64 | 92.4 | 9.9 | -0.64 | NWT | PASS |
| d | c06 | 1 | 96 | NWT | 85.2 | 10.7 | c03 | 4x24.4 | 84.1 | 11.3 | 0.97 | NWT | PASS |
|  | c07 | 1 | 128 | NWT | 85.7 | 10.4 | c04 | 4x32 | 81.6 | 11 | 3.51 | BT | EXCEED |
|  | c08 | 1 | 256 | NWT | 91.8 | 8.1 | c05 | 4x64 | 89.1 | 9.3 | 2.82 | BT | EXCEED |
| d+b | c06 | 1 | 96 | NWT | 86.5 | 13.5 | c03 | 4x24.4 | 84.5 | 15.9 | 1.73 | BT | EXCEED |
|  | c07 | 1 | 128 | NWT | 87.4 | 12.4 | c04 | 4x32 | 83.8 | 14.9 | 3.35 | BT | EXCEED |
|  | c08 | 1 | 256 | NWT | 91.7 | 9.6 | c05 | 4x64 | 90.8 | 9.7 | 1.31 | NWT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 46: Summary of the results of BS1534-6b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c06 | 96 | NWT c03 | PASS |
|  | c07 | 128 | NWT c04 | EXCEED |
|  | c08 | 256 | NWT c05 | PASS |
| d | c06 | 96 | NWT c03 | PASS |
|  | c07 | 128 | NWT c04 | EXCEED |
|  | c08 | 256 | NWT c05 | EXCEED |
| d+b | c06 | 96 | NWT c03 | EXCEED |
|  | c07 | 128 | NWT c04 | EXCEED |
|  | c08 | 256 | NWT c05 | PASS |

## BS1534-7a (MASA, Headphones)

The averaged results per condition for experiment BS1534-7a are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 22: Averaged results per condition for experiment BS1534-7a

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-7a is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 47: Statistical overview on the results of BS1534-7a

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c07 | 1 | 96 | NWT | 91.6 | 11.9 | c03 | 4x32 | 84.1 | 20.9 | 4.01 | BT | EXCEED |
|  |  | 2 | 96 | NWT | 91.6 | 11.9 | c05 | 2x32, unquantized MD | 90.5 | 12.6 | 0.85 | NWT | PASS |
|  | c08 | 1 | 128 | NWT | 93.9 | 7.7 | c04 | 4x48 | 90.2 | 13.1 | 3.14 | BT | EXCEED |
|  |  | 2 | 128 | NWT | 93.9 | 7.7 | c06 | 2x48, unquantized MD | 91.2 | 14.1 | 2.14 | BT | EXCEED |
| d | c07 | 1 | 96 | NWT | 88.5 | 7.9 | c03 | 4x32 | 77.2 | 14.3 | 9.05 | BT | EXCEED |
|  |  | 2 | 96 | NWT | 88.5 | 7.9 | c05 | 2x32, unquantized MD | 78.8 | 15 | 7.45 | BT | EXCEED |
|  | c08 | 1 | 128 | NWT | 90.6 | 7.5 | c04 | 4x48 | 87 | 8.9 | 3.91 | BT | EXCEED |
|  |  | 2 | 128 | NWT | 90.6 | 7.5 | c06 | 2x48, unquantized MD | 89.8 | 8.8 | 0.82 | NWT | PASS |
| d+b | c07 | 1 | 96 | NWT | 90.1 | 10.2 | c03 | 4x32 | 80.7 | 18.2 | 8.27 | BT | EXCEED |
|  |  | 2 | 96 | NWT | 90.1 | 10.2 | c05 | 2x32, unquantized MD | 84.6 | 15 | 5.49 | BT | EXCEED |
|  | c08 | 1 | 128 | NWT | 92.2 | 7.7 | c04 | 4x48 | 88.6 | 11.3 | 4.81 | BT | EXCEED |
|  |  | 2 | 128 | NWT | 92.2 | 7.7 | c06 | 2x48, unquantized MD | 90.5 | 11.8 | 2.2 | BT | EXCEED |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 48: Summary of the results of BS1534-7a

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c07 | 96 | NWT c03 OR NWT c05 | PASS |
|  | c08 | 128 | NWT c04 OR NWT c06 | EXCEED |
| d | c07 | 96 | NWT c03 OR NWT c05 | EXCEED |
|  | c08 | 128 | NWT c04 OR NWT c06 | PASS |
| d+b | c07 | 96 | NWT c03 OR NWT c05 | EXCEED |
|  | c08 | 128 | NWT c04 OR NWT c06 | EXCEED |

## BS1534-7b (MASA, Headphones)

The averaged results per condition for experiment BS1534-7b are depicted in the following figure. The three curves show the individual results for the two labs and the results for a joint evaluation, respectively. Condition c01 is the hidden reference while c02 is the 7 kHz lowpass anchor.

Figure 23: Averaged results per condition for experiment BS1534-7b

The complete statistical evaluation of the requirement ToR tests for experiment BS1534-7b is given in the following table. The evaluation is done separately for the data from the two listening laboratories and for a combination of the two data sets.

Table 49: Statistical overview on the results of BS1534-7b

|  |  | Type | CuT |  |  |  | EVS Reference |  |  |  | Evaluation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Value | Bitrate | Req. | Score | Std. | Cond. | Bitrate | Score | Std. | T-Stat | Result | State |
| Lab | Cond. | ToR# |  |  |  |  |  |  |  |  |  |  |  |
| b | c07 | 1 | 192 | NWT | 94.5 | 6.5 | c03 | 4x64 | 93.9 | 8 | 0.71 | NWT | PASS |
|  |  | 2 | 192 | NWT | 94.5 | 6.5 | c05 | 2x64, unquantized MD | 94.2 | 6.6 | 0.37 | NWT | PASS |
|  | c08 | 1 | 256 | NWT | 94.2 | 6.1 | c04 | 4x96 | 94.5 | 6 | -0.41 | NWT | PASS |
|  |  | 2 | 256 | NWT | 94.2 | 6.1 | c06 | 2x96, unquantized MD | 94.5 | 6.3 | -0.37 | NWT | PASS |
| d | c07 | 1 | 192 | NWT | 90.6 | 8.4 | c03 | 4x64 | 87.2 | 10.5 | 3.26 | BT | EXCEED |
|  |  | 2 | 192 | NWT | 90.6 | 8.4 | c05 | 2x64, unquantized MD | 89.4 | 8.8 | 1.33 | NWT | PASS |
|  | c08 | 1 | 256 | NWT | 90.4 | 8.6 | c04 | 4x96 | 88.8 | 9.3 | 1.64 | NWT | PASS |
|  |  | 2 | 256 | NWT | 90.4 | 8.6 | c06 | 2x96, unquantized MD | 90.7 | 9.1 | -0.23 | NWT | PASS |
| d+b | c07 | 1 | 192 | NWT | 92.5 | 7.7 | c03 | 4x64 | 90.6 | 9.9 | 2.87 | BT | EXCEED |
|  |  | 2 | 192 | NWT | 92.5 | 7.7 | c05 | 2x64, unquantized MD | 91.8 | 8.1 | 1.23 | NWT | PASS |
|  | c08 | 1 | 256 | NWT | 92.3 | 7.7 | c04 | 4x96 | 91.7 | 8.3 | 1.07 | NWT | PASS |
|  |  | 2 | 256 | NWT | 92.3 | 7.7 | c06 | 2x96, unquantized MD | 92.6 | 8.1 | -0.39 | NWT | PASS |

The following table provides a summary of the results. For this summary, the requirements that are defined as a disjunction of two separate checks have been combined into an overall status for this requirement as described before.

Table 50: Summary of the results of BS1534-7b

| Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- |
| b | c07 | 192 | NWT c03 OR NWT c05 | PASS |
|  | c08 | 256 | NWT c04 OR NWT c06 | PASS |
| d | c07 | 192 | NWT c03 OR NWT c05 | PASS |
|  | c08 | 256 | NWT c04 OR NWT c06 | PASS |
| d+b | c07 | 192 | NWT c03 OR NWT c05 | PASS |
|  | c08 | 256 | NWT c04 OR NWT c06 | PASS |

## Result overview

Across all BS.1534 experiments, there are 93 requirement ToR tests. The CuT failed one of them, it passed 77 times and exceeded the requirement 15 times. The failed test is in BS1534-5A.

Table 51: Overview of all failed tests in BS.1534 experiments

| Exp. | Lab | Cond. | Bitrate | ToR | Status |
| --- | --- | --- | --- | --- | --- |
| BS1534-5A | d | c07 | 256 | NWT c05 OR BT c04 | FAIL |

# Conclusion

The results of the listening tests for the IVAS Selection Phase were presented along with their statistical analyses. The results from the listening laboratories were analysed individually and (where applicable) jointly to provide a comprehensive overview as a basis for the next steps in the selection procedure.

In total, the CuT failed on four out of 226 requirement ToR tests for the P.800 experiments, it passed 166 times and exceeded the requirement 56 times. In the BS.1534 tests, the CuT failed on one of the 93 requirement ToR tests, it passed 77 times and exceeded the requirement 15 times. There is no instance in any experiment where the CuT failed a requirement ToR test for the data sets of both listening laboratories. Additionally, the CuT did not fail any requirement ToR test for the joint analyses of the BS.1534 data sets.

# References

| [1] | 3GPP S4-231084, „IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.0.0,“ VoiceAge Corporation. |
| --- | --- |
| [2] | 3GPP S4-231086, „IVAS Permanent Document IVAS-5: Selection Rules, v.1.0.0,“ Qualcomm France. |
| [3] | 3GPP S4aA230087, „IVAS-7a: Processing plan for selection phase,“ Ericsson. |
| [4] | ITU-T Recommendation P.1401, „Statistical analysis, evaluation and reporting guidelines of quality,“ 01/2020. |