3GPP TSG SA WG4 Meeting #125 	S4-231241

Goteborg, SE, 21 - 25 Aug 2023                                  revision of S4aV230031

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | pseudo | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Available Visualization Space |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 08/10/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Define available visualization space format |  |  |  |  |  |  |  |  |
| Summary of change: |  | A clause defining the format for available visualization space |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### 6.2.4	Available Visualization Space format

The Application may optionally define a 3-D space that is suitable for rendering visual objects, for instance, to avoid the rendered AR objects occluding with the real objects in the scene. This Available Visualization Space (AVS) may be communicated to the network so that the remote application can take advantage of the information for defining the rendering location of the composed AR objects. The method of calculating the AVS space is out of the scope of this document.

The content of the availableVisualizationSpace type shall follow the following format:

Table 6.2.4-1 – Available Visualization Space

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| availableVisualizationSpace | Object | 0..1 | An object defining the coordinate of the available visualization space (AVS) |
| cuboid | Object | 0..1* | The AVS in form of cuboid |
| x | float | 1 | Offset of the AVS starting point in the x direction |
| y | float | 1 | Offset of the AVS starting point in the y direction |
| z | float | 1 | Offset of the AVS starting point in the z direction |
| width | float | 1 | The width of AVS in the x direction |
| height | float | 1 | The height of AVS in the y direction |
| depth | float | 1 | The depth of AVS in the z direction |
| sphere | Object | 0..1* | The AVS in form of cuboid |
| x | float | 1 | Offset of the AVS center in the x direction |
| y | float | 1 | Offset of the AVS center in the y direction |
| z | float | 1 | Offset of the AVS center in the z direction |
| radius | float | 1 | The radius of AVS |
| - Only one of cuboid or sphere object shall exists. |  |  |  |

Start of Change 2

### B2.2 Available Visualization Space Implementation

### B2.2.1 Using OpenXR_XR_FB

The openXR XR_FB_scene extension allows to define the boundary room and also boundary space and objects in the space:

1. xrGetSpaceBoundingBox3DFB provides the defined rectangular cube XrRect3DfFB by defining the offset XrOffset3DfFB values x,y, z and the extend XrExtent3DfFB values width, height and depth in the x,y,z dimensions.

2. xrGetSpaceSemanticLabelsFB optionally provides a way to describe the semantic meaning of an space entity. It is recommended to use the label “3GPP-AvailableVisualizationSpace” when it is used to describe available visualization space.

#### B2.2.2 Using xrComputeNewSceneMSFT

The XR_MSFT_scene_understanding extension allows defining the bounding volume in 3 forms:

1.  XrSceneSphereBoundMSFT for defining a spherical available visualization space

2.  XrSceneOrientedBoxBoundMSFT for defining a cuboid available visualization space. Note that the bounding box is defined by its center and its edge to edge dimensions around its center. Therefore, these values shall be translated to the values defined in 6.2.4.

Also note that the scene components outside of the AVS may be excluded from rendering by the runtime.

End of Change 2