3GPP TSG SA WG4#125                                                                               Tdoc S4-231251

Gothenburg, Sweden, August 21-25, 2023

Source:	Dolby Laboratories, Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Title:                  Optional additional information on the IVAS Codec Candidate of the “IVAS Codec Public Collaboration”

Document for:  	Information

Agenda Item:   	7.5

## 1.           Introduction

The IVAS Codec Candidate of the “IVAS Codec Public Collaboration” provides all features required by the IVAS design constraints (IVAS-4 [1]). Beyond that, the IVAS Codec Candidate offers additional functionalities that are provided based on insights into market needs and opportunities. While part of the IVAS Codec Candidate and described in the High-Level Description [2], this functionality has not been assessed in the IVAS selection tests.

This document provides as additional information the results of in-house quality assessments and of complexity evaluations of such functionality that could not be considered in the IVAS selection tests.

## 2.	Additional information on functionality not considered in IVAS selection tests

### 2.1	Stereo operation with EVS compatible mono downmix stream

#### 2.1.1	Quality assessment results

For dynamic downmix tool for interoperable EVS the following in house listening test has been conducted. The results are shown in Fig 2.1.1.

- P.800 DCR 48 kHz  stereo input

  - Direct, MNRU, SDRU

  - CuT1: average (static downmix) + EVS

  - CuT2: trunk (dynamic downmix) + EVS

  - CuT3: tandem: mono output of IVAS stereo + EVS

- Bit rate

  - 13.2 kbps, 24.4 kbps, downmix only

- Input items:

  - Japanese speech with some music 60 items.

  - Including non-overlap, overlap talkers

  - Including 6 types of speaker and microphone position

- Listeners: 24 (naïve)

 

 

 

According to the results, dynamic downmix could provide better or not worse than static downmix and tandem coding. Note that tandem coding consists of EVS encoding of the mono output of IVAS stereo coding with additional delay.

Fig. 2.1.1  MOS scores of P.800 DCR test for downmix tools and tandem coding.

#### 2.1.2	Complexity evaluation results

Complexity and memory requirements of stereo downmix tool are summarized in the following tables. They are based on a specific audio test sequence used in the scope of the IVAS Public Collaboration.

Table 2.1.1 Summary of maximum computational complexity (WMOPS) at encoder

| function | 16 kHz | 32 kHz | 48 kHz |
| --- | --- | --- | --- |
| Stereo_dmx | 5.75 | 14.07 | 21.30 |
| EVS_enc | 59.33 | 80.71 | 94.75 |

Table 2.1.2 Summary of maximum memory requirements (Word) for downmix+ EVS encoder

| Program ROM (enc) | 93744 |
| --- | --- |
| Program ROM (com) | 47402 |
| Table ROM (enc) | 12314 |
| Table ROM (com) | 152318 |

Table 2.1.3 Summary of maximum RAM requirements (Word) due to downmix operations

|  | Downmix +EVS_enc | EVS_enc only | Increase due to downmix |
| --- | --- | --- | --- |
| RAM (stack+heap) | 63940 | 59626 | 4314 |
| Stack size | 24649 | 24649 | 0 |
| Intra-frame heap | 0 | 0 | 0 |
| Inter-frame heap | 39289 | 34977 | 4312 |

Stereo operation with EVS compatible mono downmix requires worst-case complexity of about 21.3 WMOPS (on top of about 128.9 WMOPS for EVS encoder+decoder) and  4.3 kWords of RAM (on top of 60 kWord of RAM for EVS). ROM and PROM are not provided here as the downmix tool is not isolated from the overall IVAS candidate. Formal evaluation of complexity and memory requirements should be conducted as part of IVAS characterization based on the latest IVAS source code.

### 2.2         Combined ISM and SBA operation

IVAS provides a combined format mode to code ISMs and an SBA scene in a single instance of the codec. At low bitrates, the objects are pre-rendered into the SBA scene, which is then coded and reproduced as usual. At high bitrates, a high-quality mode exists, which allocates extra downmix channels for the objects and codes their metadata separately.

#### 2.2.1	Quality assessment results

This high-quality mode can be shown to exhibit a strong quality benefit over both the separate coding of the two formats by separate instances of IVAS and pre-rendering. This is demonstrated by the results of a MUSHRA listening test plotted in Fig. X. The high-quality mode (osba_256kpbs) scores significantly higher than the separate-coding (ism_128kbps_sba_128kbps) and the pre-rendering (prerendered_256kbps).

Figure X – Comparison between combined vs. Separate operation of ISM + SBA at 256kbps

Further benefit can be observed at 512 kbps by the results of a separate MUSHRA listening test (6 expert listeners), shown in Figure X. The high-quality mode (osba_512) scores significantly higher than the separate-coding (base_2x256kbps) and the pre-rendering (prerend_512), bringing the average quality into the excellent range.

Figure X – Comparison between combined vs. Separate operation of ISM + SBA at 512kbps

#### 2.2.2	Complexity evaluation results

In Tab. X, the WMOPS for encoder and decoder in the high-quality OSBA mode at 256 kbps are shown. All supported object counts (1-4) are considered. One can see that in this mode there is a dependence of complexity on the number of objects.

| Max WMOPS/No. Of objects | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| encoder | 321.784 | 338.225 | 379.864 | 393.066 |
| decoder | 411.654 | 441.561 | 479.540 | 509.058 |

Tab. X show the RAM requirements of the encoder and decoder for 4 objects.

| RAM words / task | encoder | decoder |
| --- | --- | --- |
| Maximum RAM (stack + heap) size | 276719 | 470508 |
| Maximum stack size | 61481 | 57613 |
| Maximum inter-frame heap size | 215238 | 412895 |

### 2.3         Combined ISM and MASA operation

Combined ISM (objects) and spatial capture with MASA provide an efficient way to, e.g., capture and transmit the environment and talker(s) at the same time. Combined coding of ISMs and MASA provides both increased coding efficiency and reduced complexity and memory usage compared to separate codec instances. Combined ISM and MASA operation is possible with 1 to 4 ISMs at all bitrates from 13.2 to 512 kbps. At the lowest bitrates ISMs and MASA are pre-renderer into MASA format and with increasing bitrate objects are parametrically presented, most important object separately coded, and finally all ISMs separately coded with the accompanying metadata in addition to the spatial MASA signal. The combined format coding automatically distributes allowed bit budget optimally between ISMs, MASA, and spatial metadata.

#### 2.3.1	Quality assessment results

When comparing to separate coding of ISMs and MASA, combined coding (labeled ‘oMASA’ in figure below) provides significant quality improvement especially at lower bitrates.

#### 2.3.2	Complexity evaluation results

When comparing to separate coding of ISM and MASA, combined coding provides significant complexity and memory savings. Memory figure shown is the maximum stack and heap used. WMOPS figures correspond to the average WMOPS.

| 3 objects | Encoder + Decoder |  |  |  | 3 objects | Encoder + Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | bitrate | WMOPS | Memory kB |  |  | bitrate | WMOPS | Memory kB |
| ism_masa | 48 k | 341,4 | 297,3 |  | ism_masa | 64 k | 316,5 | 305,6 |
| masa | 24.4 k | 211,3 | 202,8 |  | masa | 32 k | 210,9 | 202,8 |
| ism | 24.4 k | 225,5 | 230,5 |  | ism | 32 k | 225,1 | 230,6 |
| Diff |  | -95,4 | -136,0 |  | Diff |  | -119,6 | -127,8 |
| 4 objects | Encoder + Decoder |  |  |  | 4 objects | Encoder + Decoder |  |  |
|  | bitrate | WMOPS | Memory kB |  |  | bitrate | WMOPS | Memory kB |
| ism_masa | 80 k | 335,2 | 308,5 |  | ism_masa | 128 k | 519,8 | 523,5 |
| masa | 32 k | 206,9 | 202,8 |  | masa | 64 k | 204,2 | 219,0 |
| ism | 48 k | 348,9 | 353,7 |  | ism | 64 k | 375,4 | 353,8 |
| Diff |  | -220,6 | -248,0 |  | Diff |  | -59,8 | -49,3 |

### 2.4         Binaural reverb operation

Room acoustics synthesis is essential for producing highly immersive experiences in virtual acoustic environments. This can be achieved by applying reverberation and spatialized early reflections. Reverberation and early reflections are available in binaural output modes of IVAS and can be controlled using parameters describing acoustic (late reverb) and physical (early reflections) room properties. These parameters can be used for modelling a large variety of room characteristics, as might be required by different applications, e.g., gaming or teleconferencing.

In the case of gaming, VR, and XR applications room acoustics should match the visual environment. For teleconferencing, users often prefer to apply room acoustics too, especially while using headset microphones resulting in dry input signal.

IVAS provides efficient implementation of late reverberation and spatialized early reflections algorithms that take listener position and orientation into account for realistic immersive effect.

#### 2.4.1	Complexity evaluation results

The complexity figures below compare reverb synthesis against BRIR room acoustic synthesis. Both maximum and average WMOPS figures are provided. The memory figures indicate maximum stack and heap.

|  | 48 kHz |  |  | 32 kHz |  |  | 16 kHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WMOPS |  | RAM kB | WMOPS |  | RAM kB | WMOPS |  | RAM kB |
|  | avg | max |  | avg | max |  | avg | max |  |
| HRTF | 113.3 | 125.0 | 215672 | 86.2 | 94.2 | 213372 | 41.9 | 54.7 | 211075 |
| BRIR | 169.8 | 181.4 | 285284 | 142.4 | 151.3 | 259780 | 79.4 | 96.1 | 235079 |
| Late reverb (diff vs BRIR) | 140.4-29.4 | 184.5 3.1 | 230323 -54961 | 110.2 -32.2 | 156.2 4.9 | 224197-35583 | 53.2 -26.2 | 85.4 -10.7 | 218104 -16975 |
| + Early reflections (diff vs BRIR) | 164.8 -5.0 | 211.8 30.4 | 240907 -40377 | 126.6 -15.8 | 175.3 24.0 | 231531 -28249 | 61.5 -17,9 | 96.2 0.1 | 222188 -12891 |

### 2.5         6 DoF Rendering and Directivity

For enriched 6 Degrees-of-Freedom (6 DoF) use cases, an extended metadata format is supported, adding source orientation (yaw and pitch) and radius distance from the origin. This optional feature is enabled for discrete ISM operations for bitrates equal to or higher than 64 kbps. When the extended metadata parameters are present, a source directivity and distance attenuation modeling is enabled. The rendering mode is suitable for 6 DoF applications, allowing the user to move away from one source and closer to another. An example use case scenario is a VR teleconference, where the listener may move from one conversation to another.

Figure 1: Example directivity pattern illustrated in the azimuth plane.

#### 2.5.1	Quality assessment results

A demo file is provided to show a use case with 4 sources. Default settings locate all sources on a unit circle as shown in Figure 2.a. Enabling the additional metadata parameters allow the sources to be located at different distances and orientations. Furthermore, this feature allows the listener to move between different sources as seen in Figure 2.b with the inclusion of additional head tracking parameters.

#### 2.5.2	Complexity evaluation results

A worst-case scenario was tested with the maximum number of objects (ISM 4) at 48 kHz, with a high bitrate of 64 kbps and metadata changing constantly in all directions. The complexity evaluation results for that case are presented in the following table.

| WMOPS | CuT-BSL |  |  |
| --- | --- | --- | --- |
| Operating Point | Encoder | Decoder | Total |
| ISM4@64 kbps FB | 0,021 | 1,098 | 1,119 |

## 6.           Conclusion/Summary

This additional information on the IVAS Codec Candidate of the IVAS Codec Public Collaboration demonstrates the added value of features not considered in the IVAS selection tests. Based on the evaluation results, it is believed that these features bring additional quality benefit or increase the versatility of the IVAS Codec Candidate at moderate complexity cost. Thus, they make the Candidate Codec even more attractive for future IVAS services.

## 7.           References

[1] 3GPP S4-231032, IVAS Design Constraints (IVAS-4) v1.2.0

[2] 3GPP S4-23xxxx, High-Level Description of the IVAS Codec Candidate of the “IVAS Codec Public Collaboration”