3GPP TSG-WG SA4 Meeting #125 	Tdoc S4-231271

Gothenburg, Sweden, August 20th – 25th, 2023

Source:	Nokia

Title:	[MP_RTT] Updates to IMS Data Channel Solution

Document for:	Agreement

Agenda Item:	14.9

Work Item / Release:	MP_RTT / Rel-18

# 1.	 Discussion

Current MP_RTT PD includes a Multi-party RTT over IMS Data Channel Solution based on the data channel architecture defined in TS 23.228. The current examples listed in 3.2.2.1 and 3.2.2.2 indicate that DCMF/MRF can be used as a mixer for supporting UE to create and join in the conference, and the conference application (AS) needs to manage the mapping relationship between the UE name and the steam ID of each participant, obtain the corresponding UE identity according to the stream ID when receiving the real-time text of each participant, and add the UE identity before the real-time text to correctly display the source. In this case, the interactions between AS and DCMF/MRF are needed to enable Multi-party RTT, which are not included in the PD yet.

# 2. Proposal

This contribution proposes to update the IMS Data Channel Solution by using DC AS for supporting UE to create and join in the conference as well as other functions need to be addressed by the conference application, and to incorporate the revised text into the MP_RTT PD as described below.

--------------------------------- first input starts --------------------------------

##### 3.2.2.1 Multi DC Streams

An example for three participants in a conference:

Figure 3.2.2.1-1 Multi DC Streams Example

Each UE has one uplink stream ID and two downlink stream IDs, if a new UE is added to the conference, a new downlink stream ID indicating the new UE should be added to all the existing participants.

The conference application (AS) needs to manage the mapping relationship between the UE identity and the steam ID of each participant, obtain the corresponding UE identity according to the stream ID when receiving the real-time text of each participant, and add the UE identity before the real-time text to correctly display the source.

Another example for three participants in a conference is to use DC Application Server as a mixer:

Figure 3.2.2.1-x Multi DC Streams Example

In this case, the conference application and mixer are enabled by the DC Application Server. The mixer can be implemented as part of the conference application, to integrated with other application functions, e.g., the mapping relationship between the UE ID and the steam ID of each participant, inviting new participants, etc.

--------------------------------- first input ends --------------------------------