3GPP TSG-SA WG4 Meeting #125	S4-231353

Gothenburg, Sweden, 21 – 25 August 2023

Source:	Fraunhofer Heinrich Hertz Institute (HHI), Nokia Corporation

Title:	[FS_AI4Media] MPEG Use Cases and Verification Results for NNC

Agenda item:	9.7

Document for:	Discussion and Agreement

- Introduction

An objective of the study item on AI/ML for Media (SP-220328) is to investigate the data traffic characteristics of AI/ML models for delivery over 5G systems, including whether there are any needs and potentials for data rate reduction.

A method that can achieve such data rate reductions is the NNC standard ISO/IEC 15938-17:2022, which is provided by MPEG and already documented in the permanent document in clause 6.

Concerning NNC, MPEG WG4 recently issued the output document MDS22894 [1], which reports different NNC use cases and achievable compression ratios. Although SA4 targets its own evaluation of compression methods, we believe that MPEG results are also of interest for 3GPP. They provide a comprehensive overview of scenarios where NNC can be applied and provide a rough estimate of achievable rate savings. Therefore, we propose to document these MPEG results in clause 8 of the permanent document. Proposed changes are shown in clause 2, which contains a table summarizing MPEG document MDS22894 [1]. This table is an extension of the table in [1], incorporating additional information from the same source.

Proposed changes

- Traffic characteristics

# Complete/Basic AI/ML model distribution

## Examples of AI/ML model distribution

### NNC use cases and validation results provided by MPEG WG4

Table 1-1 shows examples of AI/ML model distribution using MPEG’s NNC standard. These results have been reported by MPEG for the verification of NNC in different use cases [1].  Compression rates are given in percent of the original models at working points that have approximately the same performance as the original models. More detailed information on the different applications can be found in the documents referenced in [1].

| Application | Model / layer types | Datasets | Metrics | Codec | Compression rate at transparent performance |
| --- | --- | --- | --- | --- | --- |
| Image super-resolution | SWINv2 (Vision transformers) | DIV2K | PSNR, SSIM, LPIPS | NNCodec | 9-15% |
| Image super-resolution | EDSR (2D convolutions) | DIV2K | PSNR, SSIM, LPIPS | NNCodec | 15% |
| Image restoration | NAFNet (2D convolutions) | GoPro | PSNR | NNCodec | 18% |
| Learned quality metric (LPIPS) | AlexNet backbone (2D convolutions, fully connected) | DIV2K | LPIPS score | NNCodec | 9% |
| Image Compression | Autoencoder, 2D convolutions | CIFAR100 | PSNR, SSIM | NCTM | 17% |
| INVR (NERFs ) | DyNERF, MixVoxels | CBABasketball, Mirror | PSNR | NCTM | 10-20% |
| Point cloud compression | GRASP-Net (3D convolutions) | MPEG test sequences | D1/D2 PSNR | NNCodec | 20% |
| Visual object classification | VGG16, ResNet50, MobileNet v2 (2D convolutions, pooling, batch-normalisation, fully connected) | ImageNet | top-1, top-5 | NCTM | 3-12% |
| Visual object classification | SWIN (vision transformers) | MS COCO | top-1 | NNCodec | 10-12% |
| Object detection | SWIN (vision transformers) | ImageNet1K | mAP | NNCodec | 16% |
| Object detection | Yolo v3 (2D convolutions, pooling, batch-normalisation, fully connected) | MS COCO | F1 | NCTM | 10% |
| Acoustic scene classification | convolutions, fully connected | DCASE 2017 Task1 | classification accuracy | NCTM | 4% |
| Recommender system | Custom (feature embedding, fully connected) | MovieLens | top-100 | NNCodec | 2-4% |
| Adaptive bitrate selection using reinforcement learning | Pensieve (convolutions, fully connected) | Pensive-Pytorch | average reward | NCTM | 20% |
| NLP | BERT (transformer encoders) | SQuAD | F1 | NCTM | 15% |

Table 1-1: Application and verification of NNC in different use cases as reported by MPEG [1].

In summary, MPEG reports that some models can be compressed to 2% to 20% at transparent performance. According to [1], even greater bit rate reductions are possible when tolerating small performance reductions as a trade-off.

- References

- “Application and Verification of NNC in Different Use Cases”, MPEG document MDS22894 WG04 N00366, MPEG Video Coding ISO/IEC JTC 1/SC 29/WG 04, July 2023.