| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Packet-loss handling in iRTCW clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 15th August 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR proposes methods to handle packet-loss conditions in iRTCW clients in terminals. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Update to clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

The iRTCW technical specification 26.113 defines a set of functional components enabling immersive real-time communication (iRTC) services. These functional components include RGBD cameras, microphone array, and sensors supporting the capture of 3D video and spatial audio, and their complementary information. In iRTCW, iRTC clients use the WebRTC protocol stack for media transport.

The latest version of the technical specification (TS) document (S4-231062) is missing information on packet-loss handling in iRTCW clients in terminals.

This contribution proposes methods to handle packet-loss conditions in iRTCW clients in terminals.

# Changes

First Change

# Packet-loss handling

# General

This clause specifies some methods to handle conditions with packet losses.

The ‘a=bw-info’ attribute defined in clause 19 of TS 26.114 allows for negotiating how much additional bandwidth (if any) may be used for application layer redundancy in the session. When application layer redundancy is used, the media bandwidth negotiated for the session may need to be increased, e.g., by increasing the value used for the b=AS bandwidth modifier. The b=AS bandwidth modifier is however only a single value, which also applies only to the receiving direction. When an iRTCW client sends the SDP Offer/Answer, it is therefore not possible for the network and the other clients to know if the intention is to use the entire media bandwidth all the time (both with and without redundancy); or if the intention is to use the b=AS bandwidth only when redundancy is needed and to use a lower bandwidth when redundancy is not needed. It is also not possible to know what the iRTCW client can do in the sending direction. The ‘a=bw-info’ attribute defined in clause 19 of TS 26.114 offers an improved negotiation mechanism to better know what the iRTCW client can do and what it intends to do.

Improved error robustness can be enabled by packet-loss handling procedures of the client or the codec in the terminal.

# Video

iRTCW clients can use the following mechanisms to recover from packet losses:

-	AVPF Generic NACK and Picture Loss Indication (PLI) feedback messages,

-	RTP Retransmission, and

-	Forward Error Correction (FEC)

These mechanisms offer different performance trade-offs according to channel conditions such as end-to-end delay, bandwidth, rate and packet loss profile.

iRTCW clients offering video shall support the AVPF (RFC 4585) profile. The iRTCW client behaviour can be controlled by allocating enough RTCP bandwidth using "b=RR:" and "b=RS:" and setting the value of "trr-int". The attributes "b=RS:<bw>" and "b=RR:<bw>" as defined in RFC 4585 MAY be used to assign a different bandwidth (measured in bits per second) for RTCP messages to RTP senders and receivers, respectively. The attribute "trr-int" in SDP is used to specify the minimum time interval between two Regular (full compound) RTCP packets in milliseconds for a media session.

# NACK, PLI and SLI messages

AVPF NACK messages are used by iRTCW clients in terminal to indicate non-received RTP packets for video. iRTCW clients in terminal offering video shall support transmission and reception of NACK RTCP messages, as an indication of non-received media packets. iRTCW clients offering video shall also support transmission and reception of Picture Loss Indication (PLI) and Slice Loss Indication (SLI) AVPF RTCP messages. iRTCW clients supporting video shall also support Full Intra Request (FIR) codec control message (CCM) as defined in RFC 5104. Note that by setting the bitmask of following lost packets (BLP) the frequency of transmitting NACK can be reduced, but the repairing action by the iRTCW client receiving the message can be delayed correspondingly.

An iRTCW client transmitting video can use NACK information, as well as the PLI and SLI, at its earliest opportunity to take appropriate action and recover video from errors for the iRTCW client that sent the NACK, PLI or SLI messages. Recovery from error response is defined as sending a recovery picture that is equivalent to an Instantaneous Decoder Refresh (IDR) frame, sending Gradual Decoder Refresh (GDR), or retransmitting missing packets.

The usage of the AVPF and CCM feedback messages is negotiated by iRTCW clients using SDP offer/answer messages. Any AVPF or CCM feedback messages that have not been agreed in the SDP offer/answer negotiation shall not be used in the session.

An example of how an SDP offer/answer indicates support for feedback of PLI, SLI, negative acknowledgement, and FIR is as below,

v=0

m=video 51372 RTP/AVPF 99

a=rtpmap:99 H265/90000

a=rtcp-fb:99 nack pli sli

a=rtcp-fb:99 ccm fir

# RTP Retransmission

The RTP Retransmission Payload Format RFC 4588 supports retransmission of lost packets based on NACK feedback. Retransmission is useful if retransmitted packets arrive within the end-to-end delay requirements of the system. It is suitable for low RTT networks with relatively low observed packet loss.

If support for RTP retransmission payload format has been negotiated, the receivers shall support handling of RTP retransmission packets defined in RFC 4588 sent using SSRC multiplexing. Similarly, senders shall use RTP retransmission packets defined in RFC 4588 for packets they retransmit using SSRC multiplexing.

The following example specifies two original, AAC and HEVC, streams on ports 49170 and 49174 and their corresponding retransmission streams on ports 49172 and 49176, respectively:

m=audio 49170 RTP/AVPF 96

a=rtpmap:96 MP4A-LATM/90000

a=rtcp-fb:96 nack

a=mid:1

m=audio 49172 RTP/AVPF 97

a=rtpmap:97 rtx/90000

a=fmtp:97 apt=96;rtx-time=3000

a=mid:2

m=video 49174 RTP/AVPF 99

a=rtpmap:99 H265/90000

a=rtcp-fb:99 nack

a=fmtp:99 profile-level-id=8;config=01010000012000884006682C209\

0A21F

a=mid:3

m=video 49176 RTP/AVPF 100

a=rtpmap:100 rtx/90000

a=fmtp:100 apt=99;rtx-time=3000

a=mid:4

# Forward Error Correction (FEC)

Forward Error Correction (FEC) can provide effective error resiliency under certain packet loss and network RTT conditions. With FEC, more packets are going to be added to the actual data which can be used to restore other packets that are lost. If support for FEC is negotiated, then use of a separate SSRC multiplexed FEC stream with the RTP payload shall be supported at both the receiver and the sender. The receiver can demultiplex the incoming stream by the SSRC field and map it to the source by using the ssrc-group mechanism defined in RFC 5956. The systematic FEC scheme is a flexible parity FEC scheme that supports various signalling of source packets used to generate the parity packets.

An iRTCW client should support Forward Error Correction (FEC). An iRTCW client supporting FEC shall offer FEC for all media streams containing video. The MIME subtype name used for FEC stream is "flexfec". The "ssrc-group" attribute is used to designate FEC grouping association according to "ssrc" identifiers along with the "FEC-FR" grouping semantics for FEC Framework.  The "repair-window" parameter indicates the time span of the source and repair packets.

An example showing one source video stream (ssrc:1234) and one FEC repair streams (ssrc:2345) is as shown below. One FEC group is formed with the "a=ssrc-group:FEC-FR 1234 2345" line. The source and repair streams are multiplexed on different SSRCs. The repair window is set to 200 ms.

a=rtpmap:110 flexfec/90000

a=fmtp:110; repair-window:200000

a=ssrc:1234

a=ssrc:2345

a=ssrc-group:FEC-FR 1234 2345

FEC is still an expensive mechanism since it increases the bitrate considerably. So, it SHALL be used only sparingly:

- when it is known that there’s going to be packet losses on the network, and

- to protect only important video frames that many other frames are going to be dependent on.

End of First Change

# Proposal

We propose to agree the proposed change in clause 2 into clause 8 of iRTCW TS 26.113.

# References

- S4-230359, “Signalling the render pose and other related information”.

- S4-230390, “[MeCAR] On pose information”

- S4-230711, “RTP header extension for pose information.”

- S4-230708, “Pose information QoE”

- S4-230738, “MeCAR Permanent Document v7.0.”

- 3GPP TR 26.928, “Extended Reality (XR) in 5G”