3GPP TSG-SA WG4 Meeting #125	S4-231365

# Göteborg, Sweden, 21st August 2023 - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | <CR#> | rev | <Rev#> | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on Device types and media profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 11-08-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Defining the support of capabilities per device type |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - For each device type, defining two sub clause one for XR System capabilities and one for media capabilies - Defining the support of XR System capabilities by referring to the XR System capabilities defined in clause 4.1.3, outside of the device type section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3, B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE #1

# 10	Device types and media profiles

## 10.1	Introduction

AR experiences may be running on a variety of devices which have different characteristics and capabilities. Certain capabilities may be common to several devices while other capabilities may be unique to a specific device. Therefore, the present specification enables interoperability by collecting the media capabilities and profiles, defined in clauses 6 and 7, per device category. The device categories are comprised of:

- Device type 1: Thin AR glasses (see clause 10.2)

- Device type 2: AR glasses (see clause 10.3)

- Device type 3: XR phone (see clause 10.4)

- Device type 4: XR HMD (see clause 10.5)

CHANGE #2

## 10.2	Device type 1: Thin AR glasses

## 10.2.1	General

The thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 10.2.2	XR System support

An XR Device complying to the thin AR glasses device type shall support the following XR System capabilities:

-	orientationTracking and positionTracking

-	blendModeAdditive

-	viewConfigurationPrimaryMono and viewConfigurationPrimaryStereo

-	referenceSpaceView, referenceSpaceLocal and referenceSpaceStage

-	numberSwapchainImages equal to 2

-	compositionLayerProjection and compositionLayerQuad

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.2.2	Media capabilities support

TBD

CHANGE #3

## 10.3	Device type 2: AR glasses

### 10.3.1	General

The AR glasses device type represents a type of device which is considered to have higher computation power compared to the thin AR glasses device type. As a result, this device type has higher rendering capacities and is generally expected to be capable of rendering scenes without external support, even though remote rendering is not precluded to lower the power consumption on the device or enable the display of scenes beyond the device’s rendering capability. Lastly, the AR glasses offers AR experiences to the user via optical see-through display.

### 10.3.2	XR System support

An XR Device complying to the AR glasses device type shall support the following XR System capabilities:

-	orientationTracking and positionTracking

-	blendModeAdditive

-	viewConfigurationPrimaryStereo

-	referenceSpaceView, referenceSpaceLocal and referenceSpaceStage

-	numberSwapchainImages equal to 2

-	compositionLayerProjection and compositionLayerQuad

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.3.2	Media capabilities support

TBD

CHANGE #4

-	Minimum pixel with and height of 1k by 1k per eye

-	Frame rate: at least 60fps @1kx1k

-	16 Composition Layers

-	Orientation and position tracking

-	Blend mode: additive

-	View configuration: stereo

-	Reference space: View, local, stage

-	Swap chain formats: RGB

-	Swap Chain images: 2

-	Rendering capabilities: YUV to RGB conversion

## 10.4	Device type 3: XR phone

### 10.4.1	General

### The XR phone device type represents a type of device which corresponds to a smartphone with capacities and resources sufficient to offer AR experiences. As a result, this device type is capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.4.2	XR System support

An XR Device complying to the XR phone device type shall support the following XR System capabilities:

-	orientationTracking and positionTracking

-	blendModeOpaque and blendModeAlphaBlend

-	viewConfigurationPrimaryMono and viewConfigurationPrimaryStereo

-	referenceSpaceView, referenceSpaceLocal and referenceSpaceStage

-	numberSwapchainImages equal to 2

-	compositionLayerProjection and compositionLayerQuad

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.4.2	Media capabilities support

TBD

CHANGE #5

## 10.5	Device type 4: XR HMD

### 10.5.1	General

The XR HMD device type represents a type of device which corresponds to HMDs capable of offering at least AR experiences but not precluding other types of XR experiences. This device type is expected to be capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.5.2	XR System support

An XR Device complying to the XR HMD device type shall support the following XR System capabilities:

-	orientationTracking and positionTracking

-	blendModeAdditive, blendModeOpaque and blendModeAlphaBlend

-	viewConfigurationPrimaryMono and viewConfigurationPrimaryStereo

-	referenceSpaceView, referenceSpaceLocal and referenceSpaceStage

-	numberSwapchainImages equal to 2

-	compositionLayerProjection and compositionLayerQuad

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.5.2	Media capabilities support

TBD

Stereo display

The following XR Runtime functions are required to be supported

-	Minimum pixel with and height of 2k by 2k per eye

-	Frame rate: at least 60fps @2k

-	16 Composition Layers

-	Orientation and position tracking

-	Blend mode: alpha_blend, additive, opaque

-	View configuration: mono, stereo

-	Reference space: View, local, stage

-	Swap chain formats: RGBA

-	Swap Chain images: 2

-	Rendering capabilities: YUV to RGB conversion, advanced rendering

END OF CHANGES