3GPP TSG SA WG4#125 meeting                                                                      S4-231374

Goteborg, Sweden, 21-25 August 2023

Source:	Qualcomm

Title:	[5G_RTP] On the RTP header extension for the rendered pose

Document for:	Agreement

Agenda item	10.7

# Background

In the 5G_RTP PD [1], the RTP header extension format for the rendered pose misses the length field of the entire header extension (as defined in RFC 8285). We propose a correction for the error.

# Proposed changes

# * * * * 1st change * * * *

  - 4.3 RTP Header Extension for Rendered Pose

The split rendering server streams the rendered frame using one or more video streams, depending on the view and projection configuration that is selected by the UE. The server uses the proposed RTP header extension to associate the selected pose with the rendered frame. An RTP header extension is the most appropriate option to associate the rendered frame with its pose as it is carried as part of the RTP packets that carry the rendered images of a frame. The RTP header extension may also be used with audio streams of a split rendering process.

Header extensions are declared in the SDP using the “a=extmap” attribute as defined in RFC8285. The header extension is identified through an association between the URI of the header extension and an ID value that is contained as part of the extension. The rendered pose header extension should use the following URN: “urn:3gpp:xr-rendered-pose”.

The two-byte header format of the header extension is used for signaling the rendered pose. The 2-byte (RFC8285) RTP header extension format of the rendered pose header extension is as follows:

0                   1                   2                   3
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|     0x100            | appbits|             length            |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|       ID      | length=36+2n. |     x                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     y                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     z                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rx                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     ry                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rz                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rw                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                               |     timestamp              …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                     timestamp continued    …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          timestamp continued  |     action_id #1              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      action_id #2                |              ...          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The (x,y,z) provides the position of the rendered pose and the (rx,ry,rz,rw) provides the orientation of the rendered pose.

# * * * * End of 1st change * * * *

# References

[1] 3GPP S4-231025, 5G_RTP Permanent Document v. 0.0.6, SA4 #124, Berlin, May 2023.