3GPP TSG-SA WG4 Meeting #125	S4-231375

# Göteborg, Sweden, 21st August 2023 - 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | <CR#> | rev | <Rev#> | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on XR runtime capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 11-08-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Clause 4.1.3 is incomplete regarding XR Runtime capabilities definition - Clause 4.1.3 has editor’s notes |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - Introduce as defined term XR System as logical abstraction of an XR Runtime - The XR System capabilities are formalised as method names and associated parameters - The mapping of XR System capabilities onto OpenXR is provided in an Annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 4.1.3 remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3, B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE #1

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Frame of Reference: an abstract coordinate system whose origin, orientation, and scale are specified by a set of reference points

Reference Points: geometric points whose position is identified both mathematically and physically.

XR Application: application running on an XR Device which offers an XR experience based on an XR Runtime

XR Device: a device capable of offering an XR experience.

XR Runtime: Set of functions provided by the XR Device to the XR Application in order to create XR experiences.

XR Runtime API: the API to communicate with an XR Runtime

XR Session: an application’s intention to present XR content to the user.

XR System: a logical entity representing a collection of resources and capabilities that enables an XR experience.

XR View: a rendered view of the scene generated by the XR Application and passed on to the XR Runtime during a running XR Session

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

MR	Mixed Reality

VR	Virtual Reality

XR		eXtended Reality

CHANGE #2

### 4.1.3	XR System capabilities

Table 4.1.5-1 provides a summary of relevant capabilities for XR Runtimes exposed through an XR System. As such, an application does not interact deirectly with an XR Runtime but via an XR System exposed by XR Rutnime. A XR Runtime may expose one ore more XR Systems with different capabilities and resources. This table does not prescribe support for any specific capabilities of an XR Runtime, this is addressed for each device category individually. A mapping of these high-level capabilities to XR frameworks are provided in Annex B.

Table 4.1.5-1	XR System capabilities

| Capability | Description |  |  |  |  | Parameters |  | Parameter definitions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Create a XR System | An application can create a XR System from an XR Runtime. |  |  |  |  | xrSystemIdentifier |  | Identifier of a given XR System exposed by a XR Runtime. |  |  |  |  |
| Query XR System’s graphics properties | An application can query an XR System about its graphics capabilities. |  |  |  |  | maxSwapchainImageHeight |  | The maximum swapchain image pixel height supported by this XR system. |  |  |  |  |
|  |  |  |  |  |  | maxSwapchainImageWidth |  |  |  |  | The maximum swapchain image pixel height supported by this XR system. |  |
|  |  |  |  |  |  | maxLayerCount |  | The maximum number of composition layers supported by this XR system |  |  |  |  |
| Query XR System’s tracking properties | An application can query an XR System on the tracking capabilities. |  |  |  |  | orientationTracking |  |  |  |  | Indicates whether the XR system supports orientational tracking of the view pose(s), or not. |  |
|  |  |  |  |  |  |  | positionTracking |  |  |  |  | Indicates whether the XR system supports positional tracking of the view pose(s), |
| Enumerate XR System’s supported environment blend modes | An application can query an XR System about its supported environment blend modes, see clause [xxx]. |  |  |  |  | blendModeOpaque |  | Inditcates whether the XR System supports the opaque blend mode. |  |  |  |  |
|  |  |  |  |  |  | blendModeAdditive |  | Inditcates whether the XR System supports the additive blend mode. |  |  |  |  |
|  |  |  |  |  |  | blendModeAlphaBlend |  | Inditcates whether the XR System supports the alpha blend mode. |  |  |  |  |
| Enumarate supported view configuration types |  | An application can query an XR System about the its supported primary view configurations. | viewConfigurationPrimaryMono |  |  |  |  |  | Indicates that the primary view configuration of the XR System is a monoscopic, i.e. single view. |  |  |  |
|  |  |  | viewConfigurationPrimaryStereo |  |  |  |  |  | Indicates that the primary view configuration of the XR System is a stereroscopic, i.e. left and right-eye view. |  |  |  |
|  |  |  | viewConfigurationPrimaryOther |  |  |  |  |  | Indicates that the primary view configuration of the XR System is something else than the known view configurations. |  |  |  |
| Enumerate the view configuration properties | An application can list the properties associated with different view configurations advertised by an XR System. |  |  |  |  |  | recommendedImageRectWidth |  |  |  |  | The optimal width of imageRect to use when rendering this view into a swapchain. |
|  |  |  |  |  |  |  | maxImageRectWidth |  |  |  |  | The maximum width of imageRect supported when rendering this view into a swapchain. |
|  |  |  |  |  |  |  | recommendedImageRectHeight |  |  |  |  | The optimal height of imageRect to use when rendering this view into a swapchain |
|  |  |  |  |  |  |  | maxImageRectHeight |  |  |  |  | The maximum height of imageRect supported when rendering this view into a swapchain. |
|  |  |  |  |  |  |  | recommendedSwapchainSampleCount |  |  |  |  | The recommended number of sub-data element samples to create for each swapchain image that will be rendered into for this view. |
|  |  |  |  |  |  |  | maxSwapchainSampleCount |  |  |  |  | The maximum number of sub-data element samples supported for swapchain images that will be rendered into for this view. |
| Enumerate reference space types | An application can query an XR System about the supported reference space types, described in [xxx]. | referenceSpaceView | Indicates that the XR System supports the view reference space. |
|  |  | referenceSpaceLocal | Indicates that the XR System supports the local reference space. |
|  |  | referenceSpaceStage | Indicates that the XR System supports the stage reference space. |
|  |  | referenceSpaceUnbounded | Indicates that the XR System supports the unbounded reference space. |
|  |  | referenceSpaceUserDefined | Indicates that the XR System supports a user-defined reference space. |
| Query the spatial range boundaries | An application can query the spatial ranges in which an XR experience may be rendered. |  |  |  | 2DSpatialRangeBoundaries |  |  |  |  | Provides the rectangle centered on the origin of a given reference space in which the user can freely move. |  |  |
| Enumerate swapchain image formats | An application can query the swapchain image formats supported by an XR System. |  |  |  |  | swapchainImageFormatIdentifier |  |  |  |  | Provides an identifier of a swapchain image format that the XR System supports. |  |
| Enumerate swapchain images | An application can list the swapchain images allocated to a swapchain. | numberSwapchainImages | Provides the number of images allocated for a given swapchain. |
|  |  |  |  | swapchainImages |  |  |  | Provide the implementation-specific swapchain image objects for a given swapchain. |  |  |  |  |
| Enumerate composition layer type | An application can list the composition layer types supported by an XR System. |  |  | compositionLayerProjection |  |  |  | Indicates that the XR Sytems supports the projection composition layer. It represents planar projected images, one rendered for each eye using a perspective projection. |  |  |  |  |
|  |  |  |  | compositionLayerQuad |  |  |  | Indicates that the XR Sytems supports the quad composition layer. It is useful for rendering user interface elements or 2D content on a planar area in the world. |  |  |  |  |
|  |  |  |  | compositionLayerCylinder |  |  |  | Indicates that the XR Sytems supports the cylinder composition layer. It maps the texture onto the inside of a cylinder section. |  |  |  |  |
|  |  |  |  | compositionLayerCube |  |  |  | Indicates that the XR Sytems supports the cube composition layer. It consists of a cube map with six views to be rendered by the application. |  |  |  |  |
|  |  |  |  | compositionLayerEquirectangular |  |  |  | Indicates that the XR Sytems supports the equirectangular composition layer. It consists of an equirectangular image that is mapped onto the inside of a sphere in the world. |  |  |  |  |
|  |  |  |  | compositionLayerDepth |  |  |  | Indicates that the XR Sytems supports the depth composition layer. It allows for submitting depth maps as an extra composition layer which can be used by the XR Runtime for pose correction. |  |  |  |  |

CHANGE #3

# B.2	Capability mapping to OpenXR

### B.2.1	Mapping overview

| Capability |  |  | Corresponding OpenXR capability |  | Parameters |  |  | Corresponding OpenXR object |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Create an XR System |  | xrGetSystem() |  |  | xrSystemIdentifier |  | XrSystemId* systemId; |  |
| Query XR System’s graphics properties | xrGetSystemProperties() | maxSwapchainImageHeight |  | uint32_t maxSwapchainImageHeight; |
|  |  |  |  | maxSwapchainImageWidth |  |  | uint32_t maxSwapchainImageWidth; |  |
|  |  | maxLayerCount |  | uint32_t maxLayerCount; |
| Query XR System’s tracking properties |  |  | xrGetSystemProperties() |  | orientationTracking |  | XrBool32 orientationTracking; |  |
|  |  |  |  | positionTracking |  |  | XrBool32 positionTracking; |  |
| Enumerate XR System’s supported environment blend modes |  |  | xrEnumerateEnvironmentBlendModes() | blendModeOpaque |  |  | XrEnvironmentBlendMode* environmentBlendModes; There is one element of environmentBlendModes whose value is equal to XR_ENVIRONMENT_BLEND_MODE_OPAQUE. |  |
|  |  |  |  | blendModeAdditive |  |  | XrEnvironmentBlendMode* environmentBlendModes; There is one element of environmentBlendModes whose value is equal to XR_ENVIRONMENT_BLEND_MODE_ADDITIVE. |  |
|  |  |  |  | blendModeAlphaBlend |  |  | XrEnvironmentBlendMode* environmentBlendModes; There is one element of environmentBlendModes whose value is equal to XR_ENVIRONMENT_BLEND_MODE_ALPHA_BLEND. |  |
| Enumarate supported view configuration types | xrEnumerateViewConfigurations() |  |  | viewConfigurationPrimaryMono |  |  | XrViewConfigurationType* viewConfigurationTypes; There is one element of viewConfigurationTypes whose value is equal to XR_VIEW_CONFIGURATION_TYPE_PRIMARY_MONO. |  |
|  |  |  |  |  | viewConfigurationPrimaryStereo |  | XrViewConfigurationType* viewConfigurationTypes; There is one element of viewConfigurationTypes whose value is equal to XR_VIEW_CONFIGURATION_TYPE_PRIMARY_STEREO. |  |
|  |  |  |  | viewConfigurationPrimaryOther |  |  | XrViewConfigurationType* viewConfigurationTypes; There is one element of viewConfigurationTypes whose value is strictly greater than XR_VIEW_CONFIGURATION_TYPE_PRIMARY_STEREO and strictly lower than XR_VIEW_CONFIGURATION_TYPE_MAX_ENUM. |  |
| Enumerate the view configuration properties | xrEnumerateViewConfigurationViews() |  |  | recommendedImageRectWidth |  |  | uint32_t recommendedImageRectWidth; |  |
|  |  |  |  | maxImageRectWidth |  |  | uint32_t maxImageRectWidth; |  |
|  |  |  |  | recommendedImageRectHeight |  |  | uint32_t recommendedImageRectHeight; |  |
|  |  |  |  | maxImageRectHeight |  |  | uint32_t maxImageRectHeight; |  |
|  |  |  |  | recommendedSwapchainSampleCount |  |  | uint32_t recommendedSwapchainSampleCount; |  |
|  |  |  |  | maxSwapchainSampleCount |  |  | uint32_t maxSwapchainSampleCount; |  |
| Enumerate reference space types | xrEnumerateReferenceSpaces() |  |  | referenceSpaceView |  |  | XrReferenceSpaceType* spaces; There is one element of spaces whose value is equal to XR_REFERENCE_SPACE_TYPE_VIEW. |  |
|  |  | referenceSpaceLocal |  | XrReferenceSpaceType* spaces; There is one element of spaces whose value is equal to XR_REFERENCE_SPACE_TYPE_LOCAL. |
|  |  |  |  | referenceSpaceStage |  |  | XrReferenceSpaceType* spaces; There is one element of spaces whose value is equal to XR_REFERENCE_SPACE_TYPE_STAGE. |  |
|  |  | referenceSpaceUnbounded |  | XrReferenceSpaceType* spaces; There is one element of spaces whose value is equal to XR_REFERENCE_SPACE_TYPE_UNBOUNDED_MSFT. [Editor’s note: This requires the extension XR_MSFT_unbounded_reference_space ] |
|  |  |  |  | referenceSpaceUserDefined |  |  | [Editor’s note: This doesn’t seem to be mappable in OpenXR] |  |
| Query the spatial range boundaries | xrGetReferenceSpaceBoundsRect() | 2DSpatialRangeBoundaries |  | XrExtent2Df* bounds; |
| Enumerate swapchain image formats |  | xrEnumerateSwapchainFormats |  |  | swapchainImageFormatIdentifier |  | int64_t* formats; |  |
| Enumerate swapchain images | xrEnumerateSwapchainImages() |  |  | numberSwapchainImages |  |  | uint32_t* imageCountOutput; |  |
|  |  |  |  |  | swapchainImages |  | XrSwapchainImageBaseHeader* images; |  |  |
| Enumerate composition layer type | N/A | compositionLayerProjection |  | Part of the core specification |
|  |  | compositionLayerQuad |  | Part of the core specification |
|  | xrEnumerateInstanceExtensionProperties() | compositionLayerCylinder |  | XrStructureType type; The variable type has the value XR_TYPE_COMPOSITION_LAYER_CYLINDER_KHR. |
|  |  | compositionLayerCube |  | XrStructureType type; The variable type has the value XR_TYPE_COMPOSITION_LAYER_CUBE_KHR. |
|  |  | compositionLayerEquirectangular |  | XrStructureType type; The variable type has the value XR_TYPE_COMPOSITION_LAYER_EQUIRECT_KHR or XR_TYPE_COMPOSITION_LAYER_EQUIRECT2_KHR. |
|  |  | compositionLayerDepth |  | XrStructureType type; The variable type has the value XR_TYPE_COMPOSITION_LAYER_DEPTH_INFO_KHR. |

### B.2.2	XR views and rendering loop

Those composition layers are drawn in a specified order, with the 0th layer drawn first. Layers are drawn with a “painter’s algorithm,” with each successive layer potentially overwriting the destination layers whether or not the new layers are virtually closer to the viewer. Composition layers are subject to blending with other layers. Blending of layers can be controlled by the alpha channel information present in the image buffer of each layer. In addition, the image buffer of the layer may be limited by a maximum width and a maximum height when rendering them such that they fit into the capabilities of the swapchains.

For visual rendering, the following applies:

1)	To present images to the user, the runtime provides images organized in swapchains for the application to render into.

2)	The XR Runtime may support different swapchain image formats and the supported image formats may be provided to the application through the runtime API. XR Runtimes typically support at least sRGB formats. Details may depend on the graphics API specified when creating the session.

3) 	Swapchain images may be 2D or 2D Array. Arrays allow to extract a subset of the 2D images for rendering. Multiple swapchain handles may exist simultaneously, up to some limit imposed by the XR runtime. Swap chain parameters include:

-	texture format identifier, a graphics API specific version of a format, for example sRGB.

-	width and height, expressing the pixel count of the images sent to the swapchain

-	faceCount, being the number of faces, which can be either 6 (for cubemaps) or 1

-	indication whether the swapchain is dynamic, i.e. updated as part of the XR rendering loop or static, i.e. the application releases only one image to this swapchain over its entire lifetime.

-	access protection, indicating that the swapchain’s images are protected from CPU access

4)	Once a session is running and in focussed state as introduced in clause 4.1.2, the following rendering loop is executed following Figure 4.1.4

a)	The XR Application retrieves the action state, e.g. the status of the controllers and their associated pose. The application also establishes the location of different trackables.

b)	Before an application can begin writing to a swapchain image, it first waits on the image to avoid writing to it before the Compositor has finished reading from it. Then an XR application synchronizes its rendering loop to the runtime. In the common case that an XR application has pipelined frame submissions, the application is expected to compute the appropriate target display time using both the predicted display time and predicted display interval. An XR Runtime is expected to provide and operate a swapchain that supports a specific frame rate.

c) 	Once the wait time completes, the application initiates the rendering process. In order to support the application in rendering different views the XR Runtime provides access to the viewer pose and projection parameters that are needed to render the different views. The view and projection info is provided for a particular display time within a specified XR space. Typically, the target/predicted display time for a given frame.

D)	the application then performs its rendering work. Rendering work may be very simple, for example just directly copying data from the application into the swap chain or may be complex, for example iterating over the scene graph nodes and rendering complex objects. Once all views/layers are rendered, the application sends them to the XR Runtime for final compositing including the expected display time as well as the associated render pose.

e) 	An XR Runtime typically supports (i) planar projected images rendered from the eye point of each eye using a perspective projection, typically used to render the virtual world from the user’s perspective, and (ii) quad layer type describing a posable planar rectangle in the virtual world for displaying two-dimensional content. Other projection types such as cubemaps, equirectangular or cylindric projection may also be supported.

f)	The XR application offloads the composition of the final image to an XR Runtime-supplied compositor. By this, the rendering complexity is significantly lower since details such as frame-rate interpolation and distortion correction are performed by the XR Runtime. It is assumed that the XR Runtime provides a compositor functionality for device mapping. A Compositor in the runtime is responsible for taking all the received layers, performing any necessary corrections such as pose correction and lens distortion, compositing them, and then sending the final frame to the display. An application may use multiple composition layers for its rendering. Composition layers are drawn in a specified order, with the 0th layer drawn first. Layers are drawn with a “painter’s algorithm,” with each successive layer potentially overwriting the destination layers whether or not the new layers are virtually closer to the viewer. Composition layers are subject to blending with other layers. Blending of layers can be controlled by layer per-texel source alpha. Layer swapchain textures may contain an alpha channel. Composition and blending is done in RGBA.

g)	After the compositor has blended and flattened all layers, it then presents this image to the system’s display. The composited image is then blend with the user’s view of the physical world behind the displays in one of three modes, based on the application’s chosen environment blend mode:

-	OPAQUE. The composition layers are displayed with no view of the physical world behind them. The composited image is interpreted as an RGB image, ignoring the composited alpha channel. This is the typical mode for VR experiences, although this mode can also be supported on devices that support video passthrough.

-	ADDITIVE: The composition layers are additively blended with the real world behind the display. The composited image is interpreted as an RGB image, ignoring the composited alpha channel during the additive blending. This is the typical mode for an AR experience on a see-through headset with an additive display, although this mode can also be supported on devices that support video passthrough.

-	ALPHA_BLEND. The composition layers are alpha-blended with the real world behind the display. The composited image is interpreted as an RGBA image, with the composited alpha channel determining each pixel’s level of blending with the real world behind the display. This is the typical mode for an AR experience on a phone or headset that supports video passthrough.

h)	Meanwhile, while the XR Runtime uses the submitted frame for compositing and display, a new rendering process may be kicked off for a different swap chain image.

END OF CHANGES