Source:	Huawei Technologies Co., Ltd.

Title:	AR data transport analysis for IBACS

## Document for:	Discussion and Agreement

## Agenda Item:	10.6

# 1 Introduction

This contribution is a revision of S4aR230088. It now only focuses on AR data transport over IMS (or WebRTC) data channel for IBACS.

# 2	AR data transport analysis

## 2.1	Data Channel Solution

According to clause 6.5.5 of 3GPP TR 26.998 [1], IMS data channel can be used for AR/MR media (AR-specific data) transmission. IMS data channel over SCTP is derived from WebRTC data channel which is widely used web real-time communication.

Data channel can set flexible reliability modes for different streams id, by setting “max-retr” and “max-time” parameters (defined in RFC8864 [2]) in each a=dcmap during data channel establishment.

One approach is for the application to use stream id for different AR/MR media type identification. Here is an example:

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=fingerprint:SHA-1 \

4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB

a=tls-id:abc3de65cddef001be82

a=dcmap:1001 subprotocol="http"; max-retr=3; max-time=10 //scene description

a=dcmap:1002 subprotocol="http"; max-retr=5; max-time=15 // AR Anchor

a=dcmap:1003 subprotocol="http"; max-retr=0; max-time=0 // User Pose

Note: “max-retr” parameter indicates the maximal number of times a user message will be retransmitted. “max-time” parameter indicates that a user message will no longer be transmitted or retransmitted after a specified lifetime. If both of the value are set to 0, it indicates the unreliable mode.

For the scenario that different AR/MR media type (for example, both no timed and partial timed media are used in an application) have different Qos requirements, the application can set different “3gpp-qos-hint” attribute defined in 3GPP TS 26.114 [3] for different stream id with different m line, an example is shown as follows:

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=fingerprint:SHA-1 \

4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB

a=tls-id:abc3de65cddef001be82

a=dcmap:1001 subprotocol="http"; max-retr=3; max-time=10

a=3gpp-qos-hint:loss=0.001;latency=300

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=dcmap:1002 subprotocol="http"; max-retr=5; max-time=15

a=3gpp-qos-hint:loss=0.002;latency=400

## 2.2	Summary

The key advantages of IMS data channel can be summarized as:

1. The application can be decoupled from the terminal architecture, and the application can customize the transmitted data and data format, which can be expanded flexibly without terminal changes;

2. The application does not need to be pre-installed or built-in on the terminal.

While some drawbacks are:

1. The delay of SCTP link establishment is large due to its 4RTTs interaction.

2. The forced SCTP over DTLS encryption and decryption performance is poor.

3. SCTP has no FEC mechanism.

# Proposal

# The source believes that this analysis on the use of IMS data channel for AR data transport is a helpful input, it is therefore proposed to agree and integrate the clause 2 in to the IBACS PD.

# References

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- RFC 8864: “Negotiation Data Channels Using the Session Description Protocol (SDP)”.

- 3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”