3GPP TSG SA WG4 125 Meeting						   S4-231400

Goteborg, Sweden, 21st – 25th August 2023 			    Revision of S4-231253

Source:	Dolby Laboratories, Inc.

Title:	Requirements for Immersive Audio Split Rendering

Document for:	Discussion and Agreement

Agenda Item:	7.9

Introduction

According to the ISAR work plan [1], the work on identification of relevant requirements for immersive audio Split Rendering should be completed at the present SA4 meeting. Some progress was made at SA4#124 meeting, especially on interfaces. What is still lacking is physical and functional design constraints and performance requirements.

This contribution aims at closing that gap.

Discussion

Objective of Split Rendering

- Ideally, decoding and rendering of head-trackable immersive audio would be implementable and operational on any UE including XR end-devices like AR glasses or earbuds. However, lightweight end-devices of this class frequently operate under strict constraints in terms of computational complexity and memory consumption. Reasons are tight limits in terms of power consumption to reduce battery weight, power dissipation heat, and strict implementation cost constraints.

- Thus, it may not be possible to always ensure immersive decoding and rendering support consuming the native immersive audio format of a given codec in a lightweight XR end-device.

- It is thus the objective of Immersive Audio Split Rendering to solve this problem. For any Split Rendering solution to be viable, the following criteria should be met:

- Complexity of operation in end-rendering lightweight device is reduced substantially compared to the native decoding and head-tracked binaural rendering of the original coded audio format.

- Memory consumption in end-rendering lightweight device is reduced substantially compared to the native decoding and head-tracked binaural rendering of the original coded audio format.

- Minimum impact on QoS/QoE:

  - All required immersive audio formats of the original coding format, coding modes, and operating ranges (bit rates) should be supported.

  - Given that Split Rendering relies on pre-rendering on a capable first UE or network node to an intermediate representation, followed by coding and transmitting that representation for decoding and rendering on the lightweight device, it is unavoidably a transcoding approach. The transcoding impact on QoS/QoE, i.e., on quality and latency, should thus be as small as possible in comparison to a relevant reference system, which is to operate decoding and head-tracked rendering of the original coding format. This system is referred to as native decoding/rendering reference.

  - Head-tracked immersive audio attributes and especially DOF attributes of the audio formats of the original coding format should be retained. I.e., if the immersive audio of the original coding format is head-trackable in 3-DOF, Split Rendering should retain this property.

Design constraints

- In the following, design constraints are derived from the Split Rendering objective.

Reference systems

- The ‘golden’ reference system for any Split Rendering solution is to operate decoding and head-tracked rendering of the original coding format in the lightweight end-device. This system is referred to as native coding reference or native decoding/rendering reference.

- A further relevant reference system is a basic transcoding-based system with decoding and head-tracked binaural rendering of the native codec format carried out by the capable UE or network node. The rendered binaural audio signal is subsequently re-encoded using the most viable coding mode of the native codec that can code the binaural audio signal. Most viable would generally mean a coding mode with least complexity and memory footprint for decoding on the lightweight end device. This would typically be a stereo coding mode of that codec. Subsequently, the re-encoded binaural audio signal is transmitted to the lightweight end-device where it is decoded and output without final pose adjustment. As the end-device does not carry out any pose corrections matching the actual pose of the end-device, this reference configuration is referred to as 0-DOF native transcoding reference system. This approach is the only reasonable possibility (aside from Split Rendering) for lightweight end-devices to render binaural audio derived from the native codec format if full native decoding with head-tracked binaural rendering is not possible on that device.

Physical design constraints

- Physical attributes are complexity, memory consumption, algorithmic delay, motion-to-sound latency, bit rate. The following physical design constraints can be established based on the Split Rendering objective:

Table 1: Suggested physical design constraints for Split Rendering systems for immersive audio

| Physical attribute | Constraint | Comment |
| --- | --- | --- |
| Complexity of operation in end-rendering lightweight device | Complexity of operation in end-rendering lightweight device shall be reduced substantially compared to the native decoding/rendering reference. |  |
| Complexity of operation at pre-rendering device/node | No constraint. Objective: Complexity of operation should at maximum be at the complexity design constraint limit of the native decoding/rendering reference (if such limits are defined). If such a limit is not defined, there should be no excessive complexity increase over the native decoding/rendering reference. | Complexity benefits at the end-device are likely to come at the expense of increased complexity at the pre-rendering device/network node. The reason is that it requires decoding/rendering of the native format followed by re-encoding. |
| Memory footprint of operation in end-rendering lightweight device | Memory footprint in end-rendering lightweight device shall be reduced substantially compared to the native decoding/rendering reference and compared to the memory footprint of the final decoding operation of the 0-DOF native transcoding reference system. |  |
| Memory footprint of operation at pre-rendering device/node | No constraint. Objective: Memory footprint should at maximum be at the memory design constraint limit of the native decoding/rendering reference (if such limits are defined). If such a limit is not defined, there should be no excessive memory increase over the native decoding/rendering reference. | Memory benefits at the end-device are likely to come at the expense of increased memory consumption at the pre-rendering device/network node. The reason is that it requires decoding/rendering of the native format followed by re-encoding. |
| Motion-to-sound latency in head-tracked rendering operation | Motion-to-sound latency is no worse compared to native decoding/rendering reference. |  |
| Algorithmic audio delay | The total algorithmic end-to-end audio delay including the Split Rendering operation shall not substantially exceed the end-to-end delay of the native reference coding/rendering system. The total algorithmic end-to-end audio delay increase shall be below [20%] compared to the native reference coding/rendering system. In addition, the total algorithmic end-to-end audio delay shall be no worse than the algorithmic end-to-end audio delay of the 0-DOF native transcoding reference system. | A Split Rendering system involves transcoding of the native coding format to an intermediate representation used to transfer the audio to the lightweight device. Accordingly, the total algorithmic end-to-end audio delay including the Split Rendering operation can at best be the same as the one of the native decoding/rendering reference. It is expected that the Split Rendering approach would do algorithmic audio latency optimizations compared to the native transcoding reference system. Shared memory buffers during the transcoding from the native coding format to the intermediate representation can be assumed. |
| Bit rate of coded intermediate representation | Split Rendering operation should offer multiple bit rate options enabling different QoS/QoE levels. The supported bit rates of the coded intermediate representation should not substantially exceed the bit maximum bit rate of the native reference system and not exceed [150%] of it. | The bit rate supported on the interface between pre-rendering device/network node and end-rendering lightweight device may depend on the specific system and service configuration of a given service deployment. It is therefore desirable if Split Rendering operation offers flexible trade-offs between bit rate and QoS/QoE. Even if the bit rate of the coded intermediate representation is expected not to substantially exceed the bit maximum bit rate of the native reference system, the main priority is best possible QoS/QoE under a range of bit rates supported on the interface. |

Functional design constraints

- Functional design constraints are related to the Split Rendering objective that the required immersive audio formats, operation modes and ranges of the original (native) coding format be supported. A further functional attribute associated with immersive audio is head-trackability. This attribute needs to be retained as well.

- Accordingly, the following functional design constraints can be established based on the Split Rendering objective:

Table 2: Suggested functional design constraints for Split Rendering systems for immersive audio

| Functional attribute | Constraint | Comment |
| --- | --- | --- |
| Immersive audio formats of native coding format | All required immersive audio formats of the native coding format shall be supported by the Split Rendering operation. |  |
| Immersive audio coding modes of native coding format | All required immersive audio coding modes of the native coding format shall be supported by the Split Rendering operation. |  |
| Bit rates of required immersive audio coding modes of native coding format | All bit rates shall be supported. |  |
| Head-trackability of immersive audio formats | The head-trackability of the immersive audio formats of the native coding format shall be retained with the same DOF level. For scalable Split Rendering operation, reduced DOF levels may be provided in addition. | Explanation: an audio format supported by the native coding system may be 3-DOF head-trackable, i.e., around 3 axes (yaw, pitch, roll). The Split Rendering system shall retain this possibility. Complexity or bit rate reduced variants may though reduce this to lower DOF levels like yaw-only correction (1-DOF). |
| Packet loss concealment (PLC) | The Split Rendering operation shall be robust to packet loss on the transmission interface to the lightweight end-device. |  |
| Non-diegetic audio support | Split Rendering operation in non-diegetic mode (non-head-tracked) shall be possible. |  |

Performance Requirements

A key functional attribute to be retained by Split Rendering is head-trackability of immersive audio formats. If the round-trip latency between capable UE / network node performing the decoding and head-tracked rendering of the native audio coding format and the lightweight end-device is low, then the 0-DOF native transcoding reference system is a viable system at least in terms of retaining this attribute. However, given the large variety of connectivity scenarios, there is no guarantee that this latency will be sufficiently low to avoid that the pose deviation between the pose applied for head-tracked rendering at the capable UE / network node and the actual pose of the lightweight end-device will cause QoE impairments. Moreover, the amount of deviation will also depend on the speed of change of the pose. Large change speeds in combination with high round-trip delays will case large deviations of the assumed pose during rendering and the actual pose.

A safe approach when evaluating Split Rendering systems is thus to assess the potential quality impact when the pose correction scheme of the post-renderer adjusts the pose of the received binaural audio signal to compensate for a given static deviation between a pose available at the pre-renderer and the actual end-device pose. This is an assessment of the ability of the Split Rendering scheme to compensate for a given worst-case pose deviation that may be caused by a fast-changing end-device pose together with large transmission round-trip delay between pre-renderer on capable device or network node and post-renderer on lightweight end-device. Assuming a round-trip delay of 200 ms and a maximum pose change speed of 100 degrees / s would for instance lead to a maximum pose deviation to be compensated of 20 degrees.

- In addition to potential quality impairments due to audio degradations caused by imperfections of the pose correction scheme, there may be other factors influencing the quality of the final output audio signal. One important further impairment factor is the transcoding from the native audio coding format to the coded intermediate representation used for transmission to the lightweight end-device.

- Relevant reference systems for audio quality requirements are the native coding reference system and the 0-DOF native transcoding reference system. The former system assumes that the native coding format can be binaurally rendered at the end-device with the latest head-tracker pose. The quality of this system is the optimum which cannot be beaten by a Split Rendering system. The 0-DOF native transcoding reference system on the other hand will at least suffer from quality impairments as its underlying binaural rendering prior to transcoding is based on the pose available at the capable device/network node. The source of its quality impairments is the pose deviation between capable device/network node and end-device. For a Split Rendering system, it should be expected that the achievable quality exceeds the quality of that reference.

- This leads to the performance requirements of the following table:

Table 3: Suggested performance requirements for Split Rendering systems for immersive audio

| - Operation point of native coding system | - Static pose deviation | - Requirement | - Objective |
| --- | --- | --- | --- |
| - Head-trackable input audio format [x] encoded with mode [y] at bit rate [z] | - [20] degrees in 3 axes (yaw, pitch or roll) | - Better than 0-DOF native transcoding reference system using same operation point of native coding system and using best possible operation point for coding of binaural signal | - As close as possible to quality of native coding reference system using same operation point |
|  | - [20] degrees in two axes out of (yaw, pitch or roll) |  |  |
|  | - [20] degrees in single axis out of (yaw, pitch or roll) |  |  |

Conclusion and proposal

This contribution has established the main purpose or ‘objective’ of Split Rendering of immersive audio. Reference systems have been defined that should be used to assess if the purpose of the Split Rendering is achieved with a potential candidate solution. Based on the objective and the reference systems, physical and functional design constraints have been derived. In addition, following the same logic, performance requirements are proposed.

The source proposes to incorporate the suggested physical and functional design constraints and performance requirements into the TR on ISAR requirements [2]. An implementation of the proposal in [2] is provided in the Annex.

References:

[1] 	S4-230649: Work Plan for the ISAR v0.2

[2]	S4-231066: Draft TR 26.865 Immersive Audio for Split Rendering Scenarios; Requirements v0.2.0

| 3GPP TR 26.865 V0.2.0 (2023-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group SA; Immersive Audio for Split Rendering Scenarios; Requirements (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Interfaces	7

[4.1	Connectivity scenarios	7

4.1.1	Introduction	7

4.1.2	Device Type 1 – 5G AR glasses (standalone)	7

4.1.2	Device Type 2 – 5G UE-tethered AR glasses with AR/MR application on AR Glass	8

4.1.3	Device Type 3 - 5G UE-tethered AR glasses with AR/MR application on tethered device	8

4.1.4	Device Type 4 – 5G XR HMD UE (Standalone)	11

4.2	Audio Architectures for Split Rendering Scenarios	15

4.2.1	Introduction	15

4.2.2	Local Audio Rendering	16

4.2.3	Distributed Audio Rendering	16

4.2.4	Remote Audio Rendering	17

5	Objective of Split Rendering	18

6	Reference systems	18

7.	Design Constraints	19

7.1	 Physical Design Constraints	19

7.2	Functional Design Constraints	20

7	Performance Requirements	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

- Version x.y.z

- where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

[

An essential architectural characteristic of XR clients is the reliance on a functional split between a set of composite pre-renderers that are implemented as parts of a presentation engine and a set of post-rendering operations implemented on a lightweight end-device (AR glasses) prior to final output.

The functional split assumed in split renderer architectures is a result of stringent implementation and operational requirements applicable for rendering of XR media on XR devices. For head-tracked immersive audio, the need to rely on a split renderer architecture, may depend on various factors among which the round-trip latency between the renderer in the presentation engine and the lightweight device is a decisive parameter. There are scenarios where this latency may be substantial which may prefer a split rendering approach with pose correction in the end device for binaural audio in a similar way as for video unless decoding and head-tracked binaural audio rendering on the lightweight device does not exceed its strict complexity constraints. In other scenarios, that latency may be sufficiently low, in which case the head-tracked binaural rendering can exclusively be done in the presentation engine. It is notable that the transmission over the interface may generally be bit rate constrained and dependent on the specific physical interface.

Technical solutions for the provision of immersive audio in split rendering scenarios are largely dependent on the associated properties and imposed requirements, which are identified by the present TR.

]

# 1	Scope

[

The present document identifies relevant requirements associated with split rendering scenarios for immersive audio. It covers:

- Design constraints related to complexity and memory as well as constraints related to relevant interfaces between presentation engine and end device such as bit rate, latency, down- and upstream traffic characteristics.

- Design constraints related to functional capability requirements such as rendering of non-diegetic sounds, 3DoF rendering of diegetic immersive sounds, 6DoF rendering of diegetic immersive sounds, including simultaneous rendering of different sound categories, and room acoustics synthesis.

- Performance requirements.

- ]

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- -	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- -	For a specific reference, subsequent revisions do not apply.

- -	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: “<Title>”.

- It is preferred that the reference to TR 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

- This clause and its three (sub) clauses are mandatory. The contents shall be shown as “void” if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

- Definition format (Normal)

- <defined term>: <definition>.

Example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

- Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

- Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Interfaces

## [4.1	Connectivity scenarios

### 4.1.1	Introduction

Immersive Audio for Split Rendering may be present in a variety of connectivity scenarios between the presentation engine and audio producing devices. This clause provides a non-exhaustive list of the envisioned connectivity scenarios.

Device design types, ordered by media capability performances, are currently:

- Device type 1: 5G AR glasses (standalone)

- Device type 2: 5G UE-tethered AR glasses with AR/MR application on AR Glass

- Device type 3: 5G UE-tethered AR glasses with AR/MR application on tethered device

- Device type 4: 5G XR HMD UE (standalone)

Associate Editor’s note: Those device design types are further described in S4-230738 (MeCAR Permanent Document v7.0). The Device type names are aligned with the agreed clause 10 in S4-230920 (TS 26.119 v0.2.0). It is proposed to add a direct reference to those documents for the Device Type descriptions or copy those descriptions here. May have to consider a different architecture where the AR/MR application is on the EDGE (EDGAR).

NOTE:	With all device types, playback of the binaural acoustic signals can happen through transducers built in the glass/HMD or through a separate, connected device (true wireless stereo earbuds, wireless headphones or wired headphones/earbuds). While specification and requirements on the proprietary link are outside the scope of 3GPP, it is expected that a properly implemented system would allow for service continuity when the audio playback switches from the built-in transducers to the earbuds/headphones and vice-versa.

### 4.1.2	Device Type 1 – 5G AR glasses (standalone)

Device type 1 refers to standalone AR glasses. Such devices may be able to support simpler audio decoding capabilities [TBD] and can feature head-tracking but may not be capable of complex video or audio processing.

The connectivity scenarios are the same as those of Device Type 4 (see Clause 4.1.4), except that not all scenarios may be possible to implement due to the physical constraints.

Associate Editor’s note: It is expected that some guidance as to what scenarios from Device Type 4 are recommended once the physical constraints are better understood.

### 4.1.2	Device Type 2 – 5G UE-tethered AR glasses with AR/MR application on AR Glass

Device type 2 refers to an XR viewer that runs an AR/MR application and is tethered to a 5G device that provides connectivity and possibly additional processing. The tethering may be wireless or wired, and may be proprietary.

The audio connectivity scenarios are like those of Device Type 3 (see Clause 4.1.3), except that not all scenarios may be possible to implement due to the physical constraints.

Associate Editor’s note: It is expected that some guidance as to what scenarios from Device Type 3 are recommended once the physical constraints are better understood.

### 4.1.3	Device Type 3 - 5G UE-tethered AR glasses with AR/MR application on tethered device

The XR viewer is tethered to a 5G device (Remote Device) that includes the application and the XR functions. The tethering may be wireless or wired, and may be proprietary.

The 5G device runs the application that uses the Media access and rendering capabilities of the 5G device to run an AR/MR experience. The AR glass is connected to the 5G Device, but the XR runtime API is exposed to the 5G device/phone.

#### 4.1.3.1	Scenario 1 – Local Decoding and Rendering on 5G Phone

Associate Editor’s note: This is Scenario 4 in contribution S4-230842.

The fourth scenario is as follows in Figure 2-4, refer to a type of “5G WireLess Tethered AR UE” in TR26.998[2], the Phone UE connects to Cloud/Edge through an embedded 5G modem, the Phone UE and Lightweight UE connect through WiFi or 5G sidelink, maybe through Bluetooth for audio, and the Phone UE provides the capabilities of both decoding and rendering.

Figure 1 - Figure 2-4 Phone-dependent Lightweight UE

Associate Editor’s note: The figure needs rework to be aligned/mapped with the figures/functions from MeCAR/26.998 (shown in device types).

Variation A

Associate Editor’s note: Add text/figures in similar format as for Device Type 4. Audio playback through built-in speakers and pose estimation on glass.

Variation B

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback through earbuds and pose estimation on glass.

Variation C

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback and pose estimation through earbuds.

#### 4.1.3.2		Scenario 2 – Local Decoding and Pre-Rendering on 5G Phone

Associate Editor’s note: This is Scenario 5 in contribution S4-230842.

The fifth scenario is as follows in Figure 2-5, refer to a type of “5G WireLess Tethered AR UE” in TR26.998[2], the Phone UE connects to Cloud/Edge through an embedded 5G modem, the Phone UE and Lightweight UE connect through WiFi or 5G sidelink, maybe through Bluetooth for audio, Lightweight UE sends Pose Information to Phone UE if needed, and the Phone UE and Lightweight UE provide the capabilities of decoding and rendering together.

Figure 2 - Figure 2-5 Phone-dependent Lightweight UE

Variation A

Associate Editor’s note: Add text/figures in similar format as for Device Type 4. Audio playback through built-in speakers and pose estimation on glass.

Variation B

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback through earbuds and pose estimation on glass.Variation C

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback and pose estimation through earbuds.

#### 4.1.3.3		Scenario 3 – Decoding and Pre-Rendering on Cloud/Edge, 5G Phone as a pass-through device

Associate Editor’s note: This is Scenario 6 in contribution S4-230842.

The sixth scenario is as follows in Figure 2.6, refer to a type of “5G WireLess Tethered AR UE” in TR26.998[2], the Phone UE connects to Cloud/Edge through an embedded 5G modem, the Phone UE and Lightweight UE connect through WiFi or 5G sidelink, maybe through Bluetooth for audio, Lightweight UE sends Pose Information to Cloud/Edge if needed, and the Cloud/Edge and Lightweight UE provide the capabilities of decoding and rendering together, the Phone UE just acts as a relay device.

Figure 3 – Figure 2-6 Cloud/Edge-dependent Lightweight UE

Variation A

Associate Editor’s note: Add text/figures in similar format as for Device Type 4. Audio playback through built-in speakers and pose estimation on glass.

Variation B

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback through earbuds and pose estimation on glass.

Variation C

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback and pose estimation through earbuds.

#### 4.1.3.4		Scenario 4 – Decoding and Pre-Rendering on Cloud/Edge, further decoding, and pre-rendering on 5G Phone

Associate Editor’s note: This is Scenario 7 in contribution S4-230842. Is this a valid scenario? Seems to apply pre-rendering twice.

The seventh scenario is as follows in Figure 2-7, refer to a type of “5G WireLess Tethered AR UE” in TR26.998[2], the Phone UE connects to Cloud/Edge through an embedded 5G modem, the Phone UE and Lightweight UE connect through WiFi or 5G sidelink, maybe through Bluetooth for audio, Lightweight UE sends Pose Information to Phone UE and Cloud/Edge if needed, and the Cloud/Edge, Phone UE and Lightweight UE provide the capabilities of decoding and rendering together.

Figure 4 - Figure 2-7 Cloud/Edge and Phone UE-dependent Lightweight UE

Variation A

Associate Editor’s note: Add text/figures in similar format as for Device Type 4. Audio playback through built-in speakers

Variation B

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback through earbuds.

Variation C

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback and pose estimation through earbuds.

#### 4.1.3.5		Scenario 5 – Decoding and Pre-Rendering on Cloud/Edge, rendering on 5G Phone

Associate Editor’s note: This is Scenario 8 in contribution S4-230842. Is this a valid scenario? Seems to apply pre-rendering twice.

The eighth scenario is as follows in Figure 2-8, refer to a type of “5G WireLess Tethered AR UE” in TR26.998[2], the Phone UE connects to Cloud/Edge through an embedded 5G modem, the Phone UE and Lightweight UE connect through WiFi or 5G sidelink, maybe through Bluetooth for audio, Lightweight UE sends Pose Information to Phone UE and Cloud/Edge if needed, and the Cloud/Edge and Phone UE provide the capabilities of decoding and rendering together.

Figure 5 - Figure 2-8 Cloud/Edge and Phone UE-dependent Lightweight UE

Variation A

Associate Editor’s note: Add text/figures in similar format as for Device Type 4. Audio playback through built-in speakers

Variation B

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback through earbuds.

Variation C

Associate Editor’s note: Add text/figure in similar format as for Device Type 4. Audio playback and pose estimation through earbuds.

### 4.1.4	Device Type 4 – 5G XR HMD UE (Standalone)

For Device Type 4, the 5G device runs the XR application that uses the Media access and rendering capabilities of the 5G device to run an XR experience. All XR functionalities are included in a single device.

#### 4.1.4.1	Scenario 1 – Local Decoding and Rendering

The first scenario is as follows in Figure 2-1, (also referred as “5G Standalone AR UE” in TR26.998[2]). The 5G XR HMD UE connects directly to Cloud/Edge through an embedded 5G modem, and the 5G XR HMD UE has local capabilities of both decoding and rendering.

Variation A – Rendering, pose estimation and audio playback on HMD

In Variation A, the pose estimate, audio decoding, binaural rendering and audio playback are all performed by the 5G XR HMD UE.

Figure 6: 5G XR HMD UE with local decoding/rendering

Variation B – Decoding, Rendering and pose estimation on HMD, audio playback on earbuds.

In Variation B, the Standalone XR HMD Device is connected to a pair of True Wireless Stereo (TWS) earbuds or headphones through means of a wireless or wired proprietary link. The TWS earbuds/headphones plays back the binaural audio instead of the built-in speakers used in Variation A, but the pose estimation function is still performed by the 5G XR HMD UE.

Figure 7: 5G XR HMD UE with local decoding/rendering and audio playback through TWS earbuds/headphones

NOTE: Variation B is expected to be more prevalent than Variation C described below due to overall lower motion to sound latency and possibly a better pose estimation capability by the 5G XR HMD UE.

Variation C – Decoding and Rendering on HMD, pose estimation and audio playback on earbuds.

In Variation C, the Standalone XR HMD Device is connected to a pair of TWS Earbuds or headphones through means of a wireless or wired proprietary link. The TWS earbuds/headphones performs pose estimation that is provided to the 5G XR HMD UE. The 5G XR HMD UE performs binaural rendering using that pose estimate and the earbuds/headphones playback the binaural audio.

Figure 8: 5G XR HMD UE with local decoding/rendering and audio playback / pose estimation through TWS earbuds/headphones

Variation D – Decoding and Pre-Rendering on HMD, Rendering and audio playback on earbuds

If the overall motion to sound latency is expected to be high for variations A, B and C, use of audio split rendering between the 5G XR HMD UE and the earbuds/headphones may be appropriate.

Associate Editor’s note: Add figure for Variation D.

#### 4.1.4.2	Scenario 2 – Decoding and Rendering on Cloud/Edge

The second scenario is as follows in Figure 2-2 and refers to a type of “5G EDGe-Dependent AR UE” in TR26.998[2]. The Standalone XR HMD Device connects directly to Cloud/Edge through an embedded 5G modem, and the Cloud/Edge provides the capabilities of both decoding and rendering.

Variation A – Decoding and Rendering on Cloud/Edge, pose estimation and playback on HMD

In Variation A, the pose estimate is obtained at the Standalone XR HMD UE and sent all the way to the cloud/edge which performs audio decoding, binaural rendering, and stereo encoding for transmission to the Standalone XR HMD UE. Potentially high motion to sound latency can happen in this case since it includes the time it takes for the pose estimate to be transmitted to the cloud/edge and the binaural audio to be transmitted to the UE.

Figure 9 - 5G XR HMD UE with Cloud/Edge based decoding and rendering

Variation B - Decoding and Rendering on Cloud/Edge, pose estimation on HMD, playback on earbuds

In Variation B, a pair of earbuds/headphones is used to playback the binaural audio instead of the built-in speakers used in Variation A, but the pose estimation function is still performed by the 5G XR HMD UE. Variation B is expected to be more prevalent than Variation C described below, due to lower motion to sound latency and possibly better pose estimation capability by the 5G XR HMD UE.

The 5G XR HMD UE may be a pass-through device for the audio.

Figure 10 - 5G XR HMD UE with Cloud/Edge based decoding and rendering and audio playback through TWS Earbuds/headphones

Variation C - Decoding and Rendering on Cloud/Edge, pose estimation and playback on earbuds, HMD as a pass-through.

In Variation C, the 5G XR HMD UE is connected to TWS Earbuds or headphones through means of a wireless or wired proprietary link. The earbuds/headphones can perform pose estimation and the Standalone XR HMD Device may act as a pass-through device for both the downlink stereo audio and the uplink pose estimation data.

Figure 11 - 5G XR HMD UE with Cloud/Edge based decoding and rendering and pose estimation and audio playback through TWS Earbuds/headphones

#### 4.1.4.2	Scenario 3 – Decoding and Pre-Rendering on Cloud/Edge

The third scenario is as follows in Figure 2-3, refer to a type of “5G EDGe-Dependent AR UE” in TR26.998[2], the Lightweight UE connects directly to Cloud/Edge through an embedded 5G modem, and the Cloud/Edge and Lightweight UE provide the capabilities of decoding and rendering together.

Variation A - Decoding and Pre-Rendering on Cloud/Edge, pose estimation and playback on HMD

In Variation A, the pose estimate is obtained at the 5G XR HMD UE and sent all the way to the cloud/edge which performs audio decoding, pre-rendering, and re-encoding (ISAR Encoder) for transmission to the 5G XR HMD UE. Motion to sound latency can be partially compensated, since the 5G XR HMD UE can provide pose correction and binaural rendering (ISAR Decoder). Audio playback is through the built-in loudspeakers.

Figure 12 - 5G XR HMD UE with Cloud/Edge based pre-rendering and local rendering/pose correction and audio playback

Variation B - Decoding and Pre-Rendering on Cloud/Edge, pose estimation on HMD and playback on earbuds

In Variation B, a pair of earbuds/headphones is used to playback the binaural audio instead of the built-in speakers used in Variation A, but the pose estimation function is still performed by the 5G XR HMD UE. Variation B is expected to be more prevalent than Variation C described below due to possibly better pose estimation capability by the 5G XR HMD UE.

Figure 13- 5G XR HMD UE with Cloud/Edge based pre-rendering, local rendering/pose correction and audio playback through TWS Earbuds/headphones

Variation C - Decoding and Pre-Rendering on Cloud/Edge, pose estimation and playback on earbuds, HMD as a pass-through

In Variation C, a pair of earbuds/headphones performs pose estimation and playback the binaural audio instead of the built-in speakers used in Variation A. The Standalone XR HMD acts as a passh-through device between the Cloud/Edge and the earbuds.

Figure 14 - 5G XR HMD UE with Cloud/Edge based pre-rendering and rendering, pose correction and audio playback through TWS Earbuds/headphones

## 4.2	Audio Architectures for Split Rendering Scenarios

### 4.2.1	Introduction

An XR scene usually comprises both visual and audio media. Within the scope of ISAR the visual media follows a split rendering approach, where decoding and (pre-)rendering are performed by a capable device (e.g., an edge server), and limited processing with lower complexity is performed on the lightweight UE.

For the immersive audio media different constraints in terms of complexity and memory as well as constraints related to relevant interfaces between remote presentation engine and end device such as bit rate, latency (including motion-to-sound latency), down- and upstream link characteristics may apply.

The following generic architectures illustrate the separation of decoding and rendering of the downstream audio between lightweight UE and capable devices, limited to the data flow relevant to the application of the pose information for head-tracked binaural audio.

NOTE: 	Pose information may be required for the media generation and thus be sent upstream to the core network (and any media encoding instances beyond) independent of the three architectures.

Selection of an architecture has an impact on complexity and memory as well as applicability to relevant interfaces between remote presentation engine and end device due to bit rate, latency (including motion-to-sound latency), down- and upstream traffic characteristics.

### 4.2.2	Local Audio Rendering

The immersive audio data is streamed directly to the lightweight UE, which is responsible for decoding, rendering, and synchronizing the audio with the corresponding visual content. The lightweight UE processes the pose information locally and adjusts the audio rendering accordingly to create a convincing immersive experience.

NOTE: 	This architecture is not a split architecture for the audio media in the sense that complex operations are offloaded to a capable device. This also represents the case that should be considered as the reference.

Figure 3.2-1: Sequence of data flow for Architecture 1, Local Audio Rendering

### 4.2.3	Distributed Audio Rendering

The capable device performs decoding and pre-rendering of the immersive audio media, and the pre-rendered audio is transmitted to the lightweight UE. The pose information is sent to the capable device if needed, which adjusts the pre-rendering based on the pose data to generate an ‘intermediate representation’. The lightweight UE then performs decoding of the received intermediate representation and applies post-rendering for pose correction using a recent pose information.

Figure 3.3-1: Sequence of data flow for Architecture 2, Distributed Audio Rendering

### 4.2.4	Remote Audio Rendering

The capable device is responsible for decoding and fully rendering the immersive audio media and encoding the rendered audio into an ‘intermediate representation’, containing coded binaural audio. The intermediate representation is transmitted to the lightweight UE, which performs decoding of the rendered media. The lightweight UE synchronizes the binaural audio with the corresponding visual content.

Figure 3.4-1: Sequence of data flow for Architecture 3, Remote Audio Rendering]

Associate Editor’s note: This subclause does not yet cover multi-hop scenarios (e.g. from cloud to companion phone to glass to earbuds).

Editor’s Note:

- Identify relevant interfaces between presentation engine and end device.

# 5	Objective of Split Rendering

Ideally, decoding and rendering of head-trackable immersive audio would be implementable and operational on any UE including XR end-devices like AR glasses or earbuds. However, lightweight end-devices of this class frequently operate under strict constraints in terms of computational complexity and memory consumption. Reasons are tight limits in terms of power consumption to reduce battery weight, power dissipation heat, and strict implementation cost constraints.

Thus, it may not be possible to always ensure immersive decoding and rendering support consuming the native immersive audio format of a given codec in a lightweight XR end-device.

It is thus the objective of Immersive Audio Split Rendering to solve this problem. For any Split Rendering solution to be viable, the following criteria should be met:

- Complexity of operation in end-rendering lightweight device is reduced substantially compared to the native decoding and head-tracked binaural rendering of the original coded audio format.

- Memory consumption in end-rendering lightweight device is reduced substantially compared to the native decoding and head-tracked binaural rendering of the original coded audio format.

- Minimum impact on QoS/QoE:

  - All required immersive audio formats of the original coding format, coding modes, and operating ranges (bit rates) should be supported.

  - Given that Split Rendering relies on pre-rendering on a capable first UE or network node to an intermediate representation, followed by coding and transmitting that representation for decoding and rendering on the lightweight device, it is unavoidably a transcoding approach. The transcoding impact on QoS/QoE, i.e., on quality and latency, should thus be as small as possible in comparison to a relevant reference system, which is to operate decoding and head-tracked rendering of the original coding format. This system is referred to as native decoding/rendering reference.

  - Head-tracked immersive audio attributes and especially DOF attributes of the audio formats of the original coding format should be retained. I.e., if the immersive audio of the original coding format is head-trackable in 3-DOF, Split Rendering should retain this property.

# 6	Reference systems

The ‘golden’ reference system for any Split Rendering solution is to operate decoding and head-tracked rendering of the original coding format in the lightweight end-device. This system is referred to as native coding reference or native decoding/rendering reference.

A further relevant reference system is a basic transcoding-based system with decoding and head-tracked binaural rendering of the native codec format carried out by the capable UE or network node. The rendered binaural audio signal is subsequently re-encoded using the most viable coding mode of the native codec that can code the binaural audio signal. Most viable would generally mean a coding mode with least complexity and memory footprint for decoding on the lightweight end device. This would typically be a stereo coding mode of that codec. Subsequently, the re-encoded binaural audio signal is transmitted to the lightweight end-device where it is decoded and output without final pose adjustment. As the end-device does not carry out any pose corrections matching the actual pose of the end-device, this reference configuration is referred to as 0-DOF native transcoding reference system. This approach is the only reasonable possibility (aside from Split Rendering) for lightweight end-devices to render binaural audio derived from the native codec format if full native decoding with head-tracked binaural rendering is not possible on that device.

# 7.	Design Constraints

## In the following, design constraints are derived from the Split Rendering objective.

# 7.1		Physical Design Constraints

Editor’s Note:

- Identify design constraints related to complexity and memory

- Identify design constraints related to relevant interfaces such as bit rate, latency, down- and upstream traffic characteristics

- Identify implementation-specific design preferences

Physical attributes are complexity, memory consumption, algorithmic delay, motion-to-sound latency, bit rate. The following physical design constraints can be established based on the Split Rendering objective:

Table 1: Physical design constraints for Split Rendering systems for immersive audio

| Physical attribute | Constraint | Comment |
| --- | --- | --- |
| Complexity of operation in end-rendering lightweight device | Complexity of operation in end-rendering lightweight device shall be reduced substantially compared to the native decoding/rendering reference. |  |
| Complexity of operation at pre-rendering device/node | No constraint. Objective: Complexity of operation should at maximum be at the complexity design constraint limit of the native decoding/rendering reference (if such limits are defined). If such a limit is not defined, there should be no excessive complexity increase over the native decoding/rendering reference. | Complexity benefits at the end-device are likely to come at the expense of increased complexity at the pre-rendering device/network node. The reason is that it requires decoding/rendering of the native format followed by re-encoding. |
| Memory footprint of operation in end-rendering lightweight device | Memory footprint in end-rendering lightweight device shall be reduced substantially compared to the native decoding/rendering reference and compared to the memory footprint of the final decoding operation of the 0-DOF native transcoding reference system. |  |
| Memory footprint of operation at pre-rendering device/node | No constraint. Objective: Memory footprint should at maximum be at the memory design constraint limit of the native decoding/rendering reference (if such limits are defined). If such a limit is not defined, there should be no excessive memory increase over the native decoding/rendering reference. | Memory benefits at the end-device are likely to come at the expense of increased memory consumption at the pre-rendering device/network node. The reason is that it requires decoding/rendering of the native format followed by re-encoding. |
| Motion-to-sound latency in head-tracked rendering operation | Motion-to-sound latency is no worse compared to native decoding/rendering reference. |  |
| Algorithmic audio delay | The total algorithmic end-to-end audio delay including the Split Rendering operation shall not substantially exceed the end-to-end delay of the native reference coding/rendering system. The total algorithmic end-to-end audio delay increase shall be below [20%] compared to the native reference coding/rendering system. In addition, the total algorithmic end-to-end audio delay shall be no worse than the algorithmic end-to-end audio delay of the 0-DOF native transcoding reference system. | A Split Rendering system involves transcoding of the native coding format to an intermediate representation used to transfer the audio to the lightweight device. Accordingly, the total algorithmic end-to-end audio delay including the Split Rendering operation can at best be the same as the one of the native decoding/rendering reference. It is expected that the Split Rendering approach would do algorithmic audio latency optimizations compared to the native transcoding reference system. Shared memory buffers during the transcoding from the native coding format to the intermediate representation can be assumed. |
| Bit rate of coded intermediate representation | Split Rendering operation should offer multiple bit rate options enabling different QoS/QoE levels. The supported bit rates of the coded intermediate representation should not substantially exceed the bit maximum bit rate of the native reference system and not exceed [150%] of it. | The bit rate supported on the interface between pre-rendering device/network node and end-rendering lightweight device may depend on the specific system and service configuration of a given service deployment. It is therefore desirable if Split Rendering operation offers flexible trade-offs between bit rate and QoS/QoE. Even if the bit rate of the coded intermediate representation is expected not to substantially exceed the bit maximum bit rate of the native reference system, the main priority is best possible QoS/QoE under a range of bit rates supported on the interface. |

# 7.2	Functional Design Constraints

Editor’s Note:

- Identify functional design constraints such as rendering of non-diegetic sounds, 3DoF rendering of diegetic immersive sounds, 6DoF rendering of diegetic immersive sounds, including simultaneous rendering of different sound categories, and room acoustics synthesis

Functional design constraints are related to the Split Rendering objective that the required immersive audio formats, operation modes and ranges of the original (native) coding format be supported. A further functional attribute associated with immersive audio is head-trackability. This attribute needs to be retained as well.

Accordingly, the following functional design constraints can be established based on the Split Rendering objective:

Table 2: Functional design constraints for Split Rendering systems for immersive audio

| Functional attribute | Constraint | Comment |
| --- | --- | --- |
| Immersive audio formats of native coding format | All required immersive audio formats of the native coding format shall be supported by the Split Rendering operation. |  |
| Immersive audio coding modes of native coding format | All required immersive audio coding modes of the native coding format shall be supported by the Split Rendering operation. |  |
| Bit rates of required immersive audio coding modes of native coding format | All bit rates shall be supported. |  |
| Head-trackability of immersive audio formats | The head-trackability of the immersive audio formats of the native coding format shall be retained with the same DOF level. For scalable Split Rendering operation, reduced DOF levels may be provided in addition. | Explanation: an audio format supported by the native coding system may be 3-DOF head-trackable, i.e., around 3 axes (yaw, pitch, roll). The Split Rendering system shall retain this possibility. Complexity or bit rate reduced variants may though reduce this to lower DOF levels like yaw-only correction (1-DOF). |
| Packet loss concealment (PLC) | The Split Rendering operation shall be robust to packet loss on the transmission interface to the lightweight end-device. |  |
| Non-diegetic audio support | Split Rendering operation in non-diegetic mode (non-head-tracked) shall be supported. |  |

# 7	Performance Requirements

Editor’s Note:

- Identify performance requirements such as subjective and objective quality targets

A key functional attribute to be retained by Split Rendering is head-trackability of immersive audio formats. If the round-trip latency between capable UE / network node performing the decoding and head-tracked rendering of the native audio coding format and the lightweight end-device is low, then the 0-DOF native transcoding reference system is a viable system at least in terms of retaining this attribute. However, given the large variety of connectivity scenarios, there is no guarantee that this latency will be sufficiently low to avoid that the pose deviation between the pose applied for head-tracked rendering at the capable UE / network node and the actual pose of the lightweight end-device will cause QoE impairments. Moreover, the amount of deviation will also depend on the speed of change of the pose. Large change speeds in combination with high round-trip delays will case large deviations of the assumed pose during rendering and the actual pose.

A safe approach when evaluating Split Rendering systems is thus to assess the potential quality impact when the pose correction scheme of the post-renderer adjusts the pose of the received binaural audio signal to compensate for a given static deviation between a pose available at the pre-renderer and the actual end-device pose. This is an assessment of the ability of the Split Rendering scheme to compensate for a given worst-case pose deviation that may be caused by a fast-changing end-device pose together with large transmission round-trip delay between pre-renderer on capable device or network node and post-renderer on lightweight end-device. Assuming a round-trip delay of 200 ms and a maximum pose change speed of 100 degrees / s would for instance lead to a maximum pose deviation to be compensated of 20 degrees.

In addition to potential quality impairments due to audio degradations caused by imperfections of the pose correction scheme, there may be other factors influencing the quality of the final output audio signal. One important further impairment factor is the transcoding from the native audio coding format to the coded intermediate representation used for transmission to the lightweight end-device.

Relevant reference systems for audio quality requirements are the native coding reference system and the 0-DOF native transcoding reference system. The former system assumes that the native coding format can be binaurally rendered at the end-device with the latest head-tracker pose. The quality of this system is the optimum which cannot be beaten by a Split Rendering system. The 0-DOF native transcoding reference system on the other hand will at least suffer from quality impairments as its underlying binaural rendering prior to transcoding is based on the pose available at the capable device/network node. The source of its quality impairments is the pose deviation between capable device/network node and end-device. For a Split Rendering system, it should be expected that the achievable quality exceeds the quality of that reference.

This leads to the performance requirements of the following table:

Table 3: Performance requirements for Split Rendering systems for immersive audio

| Operation point of native coding system | Static pose deviation | Requirement | Objective |
| --- | --- | --- | --- |
| Head-trackable input audio format [x] encoded with mode [y] at bit rate [z] | [20] degrees in 3 axes (yaw, pitch or roll) | Better than 0-DOF native transcoding reference system using same operation point of native coding system and using best possible operation point for coding of binaural signal | As close as possible to quality of native coding reference system using same operation point |
|  | [20] degrees in two axes out of (yaw, pitch or roll) |  |  |
|  | [20] degrees in single axis out of (yaw, pitch or roll) |  |  |

Annex <A>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | SA4-e (AH) Audio SWG post 122 |  |  |  |  | Initial skeleton version | 0.0.1 |
| 2023-03 | SA4-e (AH) Audio SWG post 122 |  |  |  |  | Audio SWG agreed initial skeleton version | 0.0.2 |
| 2023-04 | SA4#123-e | S4-230 |  |  |  |  |  |