3GPP TSG-SA4 Meeting #125	S4-231410

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | 0002 | rev | 2 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_FGS] Proposed CR skeleton TR26955-h10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Film grain synthesis & analysis has become an important topic in the video codec standards community as a tool to (1) create artistic intent and (2) mask visual artifacts / save data bandwidth in video distribution. FS_FGS is seeing whether film grain synthesis is useful for the 5G Video scenarios defined in TR26.955 & provide relevant data points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add relevant information and details of film grain synthesis, analysis, including comparison test results to see whether film grain synthesis is useful for the 5G Video scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The objectives of FS_FGS are not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

Predictions on mobile video consumption are ever increasing. Different studies point to dominance of video traffic in 5G networks reaching from 65% total traffic in the short-term all the way to 90% by the end of the decade. Video is expected to be integral for services such as enhanced mobile media (such as mobile streaming services), home broadband and TV (for example in the context of 5G fixed wireless access services), immersive and interactive media in the context of eXtended Realities (XR) and cloud gaming as well as new media services from new verticals. This indicates that the user experience and efficiency of 5G networks will be heavily impacted by the quality of video compression technologies that are used with 5G services. Efficient video compression and decompression technologies required dedicated hardware for power and resource efficient real-time execution but are at the same time complex and costly in terms of implementation on integrated platforms. Hence, typically state-of-the-art video compression technologies last for several years and are used as generic service enablers for different applications and services, including traditional streaming and conversational services, but also new media services. This document analyzes the currently defined 3GPP-defined video compression technologies for their suitability for existing and emerging services in the context of 5G and identifies gaps and optimization potentials that would warrant the introduction of new video compression technologies.

<Comment> [In addition, a new clause for film grain synthesis is added to show that for certain 5G video scenarios the film grain synthesis effect can bring benefits in terms of preserving artistic intent, masking visual artifacts and/or saving bandwidth. ]

# Scope

The present technical report documents relevant interoperability requirements, performance characteristics and implementation constraints of video codecs in 5G services, and characterizes video codecs, in particular 3GPP defined codecs H.264/AVC and H.265/HEVC in order to have a benchmark for the addition of potential future video codecs. For this purpose, the document:

-	Collects a summary of the video coding capabilities in 3GPP services.

-	Collects a subset of relevant scenarios for video codecs in 5G-based services and applications, including video formats (resolution, frame rates, colour space, etc.), encoding and decoding requirements, adaptive streaming requirements.

-	Collects relevant and exemplary test conditions and material for such scenarios, including test sequences.

-	Defines performance metrics for such scenarios with focus on objective performance metrics.

-	Collects relevant interoperability functionalities and enabling elements for video codecs in different 5G services such as MTSI and Telepresence (i.e. RTP based conversational communications), or 5G media streaming (e.g. based on DASH/CMAF) supporting the identified scenarios.

-	Collects relevant criteria and key performance indicators for the integration of video codecs in 5G processing platforms, taking into account factors such as encoding and decoding complexity in the context of the defined scenarios.

-	Provides anchors for the existing codecs H.264/AVC and H.265/HEVC in the context of the above scenarios and document the findings in a consistent manner and characterizes H.265/HEVC against H.264/AVC

-	Provides initial information on EVC video codec developed in ISO/IEC SC29 WG 4 and H.266/VVC video codec developed in JVET (ITU-T SG16 Q6 and ISO/IEC SC29 WG 5) in the context of the above scenarios, including test streams and characterization results.

- 	Provides initial information AV1 video codec developed by the Alliance for Open Media in the context of the above scenarios, including test streams and characterization results.

-	Provides information and data on the film grain synthesis (FGS), related analysis and denoising with test results to see whether FGS is useful for the identified scenarios of 5G-based video services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

<snip>

[75]	JVET-AC2020-v2: “Film grain synthesis technology for video applications (Draft 4)”, Dan Grois, et al., Technical report draft, Joint Video Experts Team (JVET) of ITU-T SG16 WP3 and ISO/IEC JTC 1/SC 29, 29소 Meeting, by teleconference, 11-20 January 2023.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3GPP codec: a video codec defined in a 3GPP specification.

Anchor: combination of a reference sequence, a reference software of 3GPP codec and codec configuration to provide a baseline for codec testing and characterization.

Anchor Bitstream: a bitstream resulting from the encoding of a reference sequence with a reference software and a codec configuration assigned to an anchor.

Anchor Tuple: several anchors that differentiate only by one parameter in the codec configuration to change the bitrate/quality.

BD-Rate Gain: a measure of the bitrate reduction in percentage offered by a codec or codec feature under test compared to an anchor coded, while maintaining the same quality as measured by objective metrics.

Characterization: comparison of a codec under test with a 3GPP codec based on selected metrics.

Metric: A single number to provide a measurable quality of an anchor.

Test: combination of tools to test a codec for comparison with anchor, i.e. same reference sequence and comparable codec configuration.

Test Bitstream: a bitstream resulting from the encoding of a reference sequence with a reference software and a codec configuration assigned to a test.

Test Tuple: several tests that differentiate only by one parameter in the codec configuration to change the bitrate/quality.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

MSE_Y	Mean Square Error of the luma component

PSNRy	Peak-Signal to Noise Ratio of the luma component

PSNRu	Peak-Signal to Noise Ratio of the chroma u component

PSNRv	Peak-Signal to Noise Ratio of the chroma v component

PSNRyuv	Average PSNR over all colour components

MS_SSIM	Multi-Scale Structural Similarity Index Metric

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAC	Advanced Audio Coding

AAP	Alternative Approval Process

ABR	Adaptive BitRate

AFGS1	AOMedia Film Grain Synthesis Model

AOM	Alliance for Open Media

AOV	Arena Of Valor

ATSC	Advanced Television Systems Committee

AV1	AOMedia Video 1

AVC	Advanced Video Coding

AVCHD	AVC High Definition

AVI	Audio Video Interleave

BD	Bjöntegard-Delta

BDR	BD Rate

BMFF	Based Media File Format

CABAC	Context-Adaptive Binary Arithmetic Coding

CAE	Content Aware Encoding

CBP	Constrained Baseline Profile

CBR	Constant BitRate

CGI	Computed Generated Imaginary

CHP	Constrained High Profile

CMAF	Common Media Application Format

CRA	Clean Random Access

CSV	Comma-Separated Values

CTC	Common Test Conditions

CTU	Coding Tree Unit

DASH	Dynamic Adaptive Streaming over HTTP

DVB	Digital Video Broadcasting

EBU	European Broadcast Union

EFS	Effective File Size

EPZS	Enhanced Predictive Zonal Search

ERP	Equi-Rectangular Projection

EVC	Essential Video Coding

EXR	Extended Range

FHD	Full HD (i.e. 1080p)

FGC	Film Grain Characteristics

FGS	Film Grain Synthesis

FPS	Frames Per Second

GOP	Group-Of-Pictures

HDMI	High-Definition Multimedia Interface

HDR	High Dynamic Range

HDTV	High Definition TeleVision

HEVC	High-Efficiency Video Coding

HFR	High Frame Rate

HLG	Hybrid Log-Gamma

HLS	HTTP Live Streaming

HMD	Head-Mounted Display

HRD	Hypothetical Reference Decoder

HTML	HyperText Markup Language

HTTP	HyperText Transfer Protocol

IDR	Instantaneous Decoder Refresh

JCT-VC	Joint Collaborative Team on Video Coding

JSON	JavaScript Object Notation

JVET	Joint Video Experts Team

MCTF	Motion-Compensated Temporal Filtering

MIME	Multipurpose Internet Mail Extensions

MKV	MatrosKa Video

MMO	Massive Multiplayer Online

MMORPG	MMO Role-Playing Game

MMS	Multimedia Messaging Service

MOBA	Multiplayer Online Battle Arena

MOS	Mean Opinion Score

MPD	Media Presentation Description

MPEG	Moving Pictures Expert Group

MS-SSIM	Multi-Scale Structural Similarity Index

MSE	Mean Square Error

MTSI	Multimedia Telephony Service over IMS

OBS	Open Broadcaster Software

PSNR	Peak Signal to Noise Ratio

RA	Random Access

RAP	Random Access Point

RCS	Rich Communication Services

RDPCM	Residual Differential Pulse Code Modulation

RGB	Red Green Blue

RPG	Role-Playing Game

RTMP	Real-Time Messaging Protocol

RTP	Realtime Transport Protocol

RTS	RealTime Strategy

SCC	Screen Content Coding

SCM	Screen Content coding Model

SDK	Software Development Kit

SDR	Standard Definition Range

SEI	Supplemental Enhancement Information

SI 	Spatial perceptual Information

SMPTE	Society of Motion Picture & Television Engineers

SSIM	Structural Similarity Index Measure

SVOD	Subscription Video On Demand

TI 	Temporal perceptual Information

TIFF	Tagged Image File Format

UDP	User Datagram Protocol

UHD	Ultra High Definition

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

UVG	Ultra Video Group

VBR	Variable BitRate

VBS	Visual Basic Script

VCL	Video Coding Layer

VMAF	Video Multimethod Assessment Fusion

VSEI	Versatile Supplemental Enhancement Information

VTM	VVC Test Model

VVC	Versatile Video Coding

WCG	Wide Colour Gamut

# 6	Relevant Scenarios

## 6.1	Introduction

This clause collects relevant scenarios based on the template defined in Annex A. It also defines the anchors for each scenario based on the existing 3GPP codecs and profiles defined in clause 4. For each scenario, the following information is provided:

<Comment> Motivation: provides a context for the scenario, why it is relevant for 5G video services.

[For the 5G Video scenarios (#1 Full HD streaming, #2 4k TV, #4 social/online sharing and #5 online gaming) that are described in this clause, and potentially for new test sequences specifically developed to test for the film grain effect, some of the popular film grain synthesis and analysis are applied to test sequences encoded with H.265/HEVC, in order to ascertain whether there is benefit in (1) artistic intent and/or (2) masking artifacts / saving bitrate while maintaining visual acuity, etc.

## 6.7	Scenario 6: Film Grain Synthesis in 5G Video Services

6.7.1		Motivation and Use Cases

[Film grain synthesis & analysis has become an important topic in the video codec standards community as a tool to (1) create artistic intent and (2) mask visual artifacts / save data bandwidth in video distribution. This clause documents the findings on whether film grain synthesis is useful for the 5G Video scenarios defined in TR26.955 & provide relevant data points.]

[Public source code for use of film grain technologies for various use cases and video codecs, including HEVC, has been made available recently from several sources [ref].  Also, ISO/IEC and ITU-T have been documenting use of film grain technologies to improve compression efficiency of coded bitstreams (including HEVC) and have already performed preliminary subjective tests to quantify visual quality improvements attributable to film grain synthesis. See JVET Film grain synthesis technology for video applications [ref].]

[Film grain synthesis and modelling have diverse applications in video starting from content creation, compression, distribution, to end consumption.  A particular area of interest for film grain is the usage of Film Grain Characteristic (FGC) supplemental enhancement information (SEI) message which can be used as metadata along with any of ITU-T & ISO/IEC MPEG based AVC, HEVC or VVC coded video. The receivers can use the FGC SEI message to synthesize grain as per the specified parameters in the SEI and create the intended effect of the grain envisioned by the content creator or interim content distributor. Various use cases of FGC along with example implementations of film grain characterization and synthesis models are described in detail in the CD draft technical report - JVET-AC2020, “Film grain synthesis technology for video applications (Draft 4)” [5]]

[As FGC SEI based implementations are non-normative, the synthesized grain outputs of any two FGS implementations may not be identical (bit-exact) for the same FGC SEI applied on a decoded frame.  However, both the implementations are expected to have similar grain characteristics as conveyed in the SEI message to create the same intended perception of grain for human viewers. This makes it imperative to develop an FGS verification tool that can analyse and measure the grain characteristics and report possible deviations from the FGC parameters conveyed in the SEI as well as report other unintended, undesirable aftereffects such as mean luminance shift, colour shift, potential blocking artefacts introduced by grain, color saturation to name a few.]

<Comment> Film grain viewing tests using subjective and objective methodologies for test sequences w.r.t. the 5G Video scenarios #1 Full HD streaming, #2 4k TV service will be conducted.  Then #4 social/online sharing and #5 online gaming will be looked at, along with new test sequence(s) specifically added for film grain synthesis effects, in order to show (1) artistic intent, (2) artifact masking / bandwidth saving.

    - Film Grain Synthesis Technologies

<Comment>  WID objective 2.	Document relevant existing Film Grain Synthesis technologies that are not included in 3GPP today.

e.g. text from JVET FGS TR as introduction

[[FGS] technology can provide subjective quality benefits for certain video applications and, thus, could effectively achieve improved video compression. This can be achieved by commonly removing or reducing the amount of noise that is present in a video or image signal prior to compression and by resynthesizing and adding back an approximation of the removed noise, given a set of signalled parameters, during decoding. Such parameters can be signalled using appropriate mechanisms, such as the supplemental enhancement information messages that are supported by several video coding standards.]

<Comment> Add text about (1) generating film grain template, (2) randomization (initialization parameters, block size, offset randomness), (3) local adaptation, (4) deblocking, (5) blending.  A short description of the frequency filtering model and the autoregressive model.  The title of this clause may be “Film Grain Properties.”

      - Film Grain Technical Characteristics [75]

[With the advancements of digital camera sensors and their widespread utilization, the grainy look has been mostly eliminated. Although digital sensors have brought many possibilities in terms of visual quality and visual processing, the “film look” lives on among professionals and film enthusiasts. Within the new era, film grain has turned into a visual tool and not just a by-product of chemical processing as in the case of analogue film stock.

Note that the term film grain also includes synthetic film grain that is added in post-production to digitally captured high-value content for artistic effect or to mask imperfections in digital footage, which can otherwise look too sharp and clear sometimes. The term film grain can also be used to refer to image sensor noise, particularly in low light and high-speed captures.

Therefore, perception of moderate grain texture is a desirable, often sought after, characteristic in motion picture and video productions. Although the exact effect of the grain is not clear, it is considered a requirement in the motion picture industry to preserve the grainy appearance of images throughout the image processing and delivery chain. Intuitively, the presence of film grain helps to differentiate ‘real-world’ images from ‘computer-generated’ material, which are commonly created with no film grain. Furthermore, it is possible that film grain provides some visual cues that facilitate the correct perception of depth in two-dimensional pictures [6]. Even when movies are captured with digital cameras, artificial film grain is added at a post-processing stage to create a specific look, which artists qualify as “soft”, “organic”, or “living”. Synthetic film grain is also used to harmonize capture from different cameras, potentially mixing film and digital capture and different lighting conditions.

Film grain preservation during video distribution, and especially when targeting low bitrate applications, can be a challenging for two reasons. First, compression gains related to temporal prediction cannot be fully leveraged because of the random nature of the grain. Film grain noise is temporally independent, and as a result, motion compensation cannot be efficiently used for its prediction. Second, the grain commonly appears at high frequencies in the DCT domain, and it is typically filtered with other noise by in-loop filters, such as deblocking filters, or due to the quantization process [6]. This challenge is more severe with recent codecs, as bitrate gains have come along with noise elimination. In addition, introduction of pre-filtering in the video distribution chain can potentially remove film grain prior to compression. The use of quantization matrices [7],[8] could potentially assist in the preservation of some of the film grain within the video content, however this also can have severe limitations, especially at lower bitrates, and for streaming applications.]

    - Film Grain Characterization Methods, Performance Metrics and Subjective Testing Aspects

<Comment> Discuss potential characterization methods, performance metrics and subjective testing aspects for FGS performance evaluation.  This may include objective and subjective metrics such as:

  - Objective tests: BRISQUE, NIQE, VMAF metrics,

  - For subjective tests – ITU-TP910 recommendation based Pair-Wise Testing method will be used for the setup.

      - Film Grain Modelling

<Comment>  Draft text about film grain modelling will be added later.

      - Film Grain Performance Metrics

<Comment>  Draft text about film grain template generation, randomization, block size, offset, local adaptation, deblocking, blending, etc. will be added later from JVET FGS Technical Report draft.  [75]

      - Test Sequences for Film Grain Synthesis

<Comment> Describe the test sequences that are used for the FGS, analysis and verification.

[Partial list of test sequences for FGS tests: more will be added later.

| Content | 5G Video Scenario | Source | Online link | License |
| --- | --- | --- | --- | --- |
| In To Tree 1080p | Full HD Streaming | New content | Xiph.org :: Derf's Test Media Collection | SVT Multi Format Test Set Version 1.0 (xiph.org) |
| Old Town Cross 1080p | Full HD Streaming | New content | Xiph.org :: Derf's Test Media Collection | SVT Multi Format Test Set Version 1.0 (xiph.org) |
| Crowdrun 2160p | 4k TV | New content | Xiph.org :: Derf's Test Media Collection | SVT Multi Format Test Set Version 1.0 (xiph.org) |
| Chimera Dinner Scene 2160p | 4k TV | New content | S3 Bucket Listing Generator (netflix.com) | download.opencontent.netflix.com.s3.amazonaws.com/Netflix_test_conditions/creative-commons-attribution-4-intl-public-license.txt |
| BQTerrace 1080p | Full HD Streaming | New content | ftp://jvet@ftp.ient.rwth-aachen.de | These sequences and all intellectual property rights therein remain the property of NTT DOCOMO, INC. These sequences may only be used for the purpose of developing, testing and promulgating technology standards. NTT DOCOMO, INC. makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose. |
| Meridian 2160p | 4k TV | 3GPP | Index of /129021/dash/WAVE/3GPP/5GVideo/ReferenceSequences/Meridian (dash-large-files.akamaized.net) | download.opencontent.netflix.com.s3.amazonaws.com/Netflix_test_conditions/creative-commons-attribution-4-intl-public-license.txt |

]

    - Evaluation of Film Grain with HEVC

<Comment> WID objective 3.	Provide evaluation using HEVC of the benefits/drawbacks of corresponding solutions, including film grain characteristics SEI message (ITU-T H.274) including performance results, complexity and implementation aspects, interoperability, system integration, etc. following the example in TR26.955 based on selected scenarios.

<Comment> Add text about how the subjective tests will be conducted with naïve viewers and with expert viewers for each scenario being tested, with and without FGS.  Consider adding dithering methods especially if test devices have relatively low grade specifications.

<Comment> FGS tests will reuse many of the reference sequences for scenarios #1, #2, #4, #5.  There may be additional test sequences added if those are deemed beneficial for the purpose of testing and comparison.

- NIQE (Naturalness Image Quality Evaluator) scores etc. assessment methodology to be used.

- HM (HEVC reference encoder software) stream generation for testing – FHD – score computation being done.  4k TV streams work underway for subjective evaluation.

      - Film Grain Synthesis Bitstream Verification

<Initial draft text below – details will evolve and change>

[Create a set of HEVC coded FGC SEI test bitstreams to used as verification suite with following features:

  - FGC SEI messages to test following features:

    - Frequency Filtering model (FFM)

    - Auto-Regressive model (ARM)

    - Only Additive blending mode in initial verification suite. Multiplicative blending mode to be added later.

  - Use synthetic test video such as color bars, graded ramp patterns, video with flat areas of different block sizes and different intensity intervals for ease of test verification.

  - Video sizes, frame rates and bit depths:

    - Resolution: (360p, 480p,) 720p, 1080p, 2160p

    - Frame rates: 24, 30, 50, 60, (120)

    - Bit Depth: 8 and 10

    - SDR only.  (HDR may be added later)

    - Chroma formats: YUV420, (YUV422, YUV444)

  - Use of HM (and/or X265) reference software to generate FGS SEI bitstreams for random access configuration.]

      - FGS Verification Tool Specifications

<Comment>  Some high level considerations for the FGS verification tool:

- Develop FGS software utility that may contain these features:

  - Modular design where new features may be added later,

  - Initial software planned to be single threaded application,

  - Software will support analysis & assessment of the following grain characteristics:

    - Frequency filtering model (FFM) and autoregressive model (ARM).

    - Support additive blending mode.  Multiplicative blending mode to be added later.

  - Software will detect deviations of measured film grain parameters that go beyond acceptable deviations (beyond the user-configured limits).  Also detect unintended mean luminance shift (Y shift) / mean chrominance shift (UV shift).

6.7.6.3	FGS Verification Procedure

<Comment>  The fiture and text below is the initial draft, subject to further changes/edits.

Decoding

Film grain synthesis

Coded video bitstream containing film grain model parameters in FGC SEI

Decoded video

Film grain model parameters

Decoded video with simulated film grain

The above fiture shows a decoding process along with the film grain synthesis post-processing. At the decoder, the film grain model parameter values are parsed and input to a film grain synthesis process that generates simulated film grain and blends the grain with the decoded video to output decoded video with simulated film grain.

<Comment> A follow-on block diagram and text will be added to describe the grain verification procedure, using the decoded video.

Decoding

Film grain synthesis

Coded video bitstream containing film grain model parameters in FGC SEI

decoded video

Film grain model parameters

decoded video with simulated film grain

Film grain verification software

Film grain model parameters

Film grain verification test results

6.7.4		Potential UE Requirements

<Comment> This clause discusses the potential UE requirements to enable FGS in a consistent and interoperable way.

6.7.5		Interoperability Considerations on a System Level

<Comment> WID objective 7.	Identify relevant interoperability and system level aspects to potentially support Film Grain Synthesis.  Aspects about MPEG systems standards etc. that are directly related to the FGS technologies will be described and referenced here.

6.7.7		Conclusions

<Comment> Concluding summary of the findings from the FGS tests for 5G Video scenarios #1, #2, #4, #5, and a potential way forward.