| 3GPP TS 26.565 V0.6.0 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Split Rendering Media Service Enabler; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General	9

4.1	Overview	9

4.2	Typical Use Cases	9

5	Reference Architecture and Procedures	9

5.1	Reference Architecture	9

5.1.1	Introduction	9

5.1.2 	Client Architecture	9

5.1.3	End-to-End Architecture	10

5.1.5	User Plane Architecture	11

5.2	Procedures and Call Flows	11

5.2.1 	Call flow for Split Rendering instance discovery	11

5.2.1.1	Call flow for edge server and split rendering session setup	11

5.2.1.2 Client-driven procedures and call flows	12

5.2.2 	Call flow for Split Rendering session setup	13

6	Prerequisites	14

6.1	Requirements on 5G System	14

6.2	Requirements on Device APIs and Functionality	14

7	Split Rendering Application Function	15

7.1	Functionality	15

7.2	RESTful APIs	15

7.2.1	Split-Rendering Provisioning procedures (RTC-1)	15

7.2.1.1	General	15

7.2.1.2	Create Split-Rendering Configuration	15

7.2.1.3	Read Split-Rendering Configuration properties	15

7.2.1.4	Update Split-Rendering Configuration properties	15

7.2.1.5	Destroy Split-Rendering Configuration	15

7.2.2	 Split-Rendering Provisioning API	16

7.2.2.1	Overview	16

7.2.2.2	Resource structure	16

7.2.2.3	Data model	16

7.2.2.3.1	SplitRenderingConfiguration resource	16

7.2.2.4	Configuration Guidelines for Split Rendering	16

7.2.2.4.1	Guidelines on Provisioning Session	16

7.2.2.4.2	Guidelines on Edge Resource Configuration	17

7.2.2.4.3	Guidelines on Policy Template	17

7.2.3	Dynamic Policy API for Split Rendering	17

7.3	HTTP response codes	17

8	Split Rendering User Plane	17

8.1	Split Rendering Signalling Protocols	17

8.2	Split Rendering Formats	18

8.2.1 	General	18

8.2.2	Metadata Formats	19

8.2.2.1	General	19

8.2.2.2	Pose Format	19

8.2.2.3	Action Format	20

8.2.3	Metadata Data Channel Message Format	21

8.3	Split Rendering Transport Protocols	22

8.4 	Split Rendering Formats for Session Setup and Negotiation	22

8.4.1 	General	22

8.4.2 	Split Rendering Configuration Format	22

8.4.2.1	Introduction	22

8.4.2.2	 Split Rendering Configuration Format	22

9	Split Rendering Client	24

9.1	Functionality	24

9.2	Client API	24

9.3	Split Rendering Metrics	25

10 	Security and Privacy Aspects	25

10.1 	Security	25

10.2 	Privacy	26

Annex A	 (Informative): Implementation Guidelines	27

Annex A.1 Guidelines for Application Developers	27

Annex A.2 Guidelines for Split Rendering MSE Implementers	27

A.2.1	Guidelines for implementers of the Split Rendering Server	27

Annex A.3 Conformance Testing	27

Annex B (normative): IDL Definition of Client API	28

Annex C (normative): Split Rendering Profiles	29

C.1 	Pixel Streaming Profile	29

C.1 	Introduction	29

C.2 	2D Pixel Streaming Profile	29

C.2.1	Introduction	29

C.2.2	SRC Capabilities	29

C.2.2.1	Overview	29

C.2.2.2	Video decoding	29

C.2.2.3	Audio and Speech decoding	30

C.2.2.4	Video encoding	30

C.2.2.5	Audio and Speech encoding	30

C.2.2.6 	Metadata Formats	30

C.2.3	SRS Capabilities	30

C.2.3.1	Overview	30

C.2.3.2	Video encoding	30

C.2.3.3	Audio and Speech encoding	30

C.2.3.4	Video decoding	30

C.2.3.5	Audio and Speech decoding	30

C.2.3.6 	Metadata Formats	31

C.3 	3D Pixel Streaming Profile	31

C.3.1	Introduction	31

C.3.2	SRC Capabilities	31

C.3.2.1	Overview	31

C.3.2.2	Video decoding	31

C.3.2.3	Audio and Speech decoding	31

C.3.2.4	Video encoding	31

C.3.2.5	Audio and Speech encoding	32

C.3.2.6 	Metadata Formats	32

C.3.3	SRS Capabilities	32

C.3.3.1	Overview	32

C.3.3.2	Video encoding	32

C.3.3.3	Audio and Speech encoding	32

C.3.3.4	Video decoding	32

C.3.3.5	Audio and Speech decoding	32

C.3.3.6 	Metadata Formats	32

Annex X (informative): Change history	33

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This specification defines a media service enabler for split rendering in the 5G system.

# 1	Scope

The present document defines a Media Service Enabler for Split Rendering according to the guidelines of TR26.857 [1].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AF		Application Function

AS		Application Server

MAF	Media Access Function

MSH		Media Session Handler

5G-RTC		5G Real-Time Communication

SR 		Split Rendering

SRC		Split Rendering Client

SRS		Split Rendering Server

UE		User Equipment

# 4	General

## 4.1	Overview

The Split Rendering Media Service Enabler collects a set of 5G media functions to build a media service enabler that targets application developers, network operators, and application service providers, to enable the realization of split rendered applications.

The interfaces, formats, protocols, and APIs are either referenced or defined in this specification. This will allow for interoperability between multiple vendor implementations.

This specification targets primarily XR applications. However, it is not limited to XR applications and may be used for rendering for 2D displays.

## 4.2	Typical Use Cases

A typical use case for the split rendering MSE is immersive gaming. In this use case, the UE benefits from invoking split rendering by avoiding the download of the game to the phone and getting high quality graphics from edge rendering.

Another use case that can benefit from split rendering is immersive communication, where users gather in a shared space and interact with each other and with the environment. Users may be represented by sophisticated Avatars and as the number of users increases the rendering will become more complex.

# 5	Reference Architecture and Procedures

## 5.1	Reference Architecture

### 5.1.1	Introduction

In this clause, different variants of the reference architecture for the split rendering MSE are defined, each representing a different perspective and level of details.

The following functions are introduced:

- Split-Rendering Client (SRC): This function is responsible to acquire the UE media capabilities and negotiates with the RTC AS to agree on the split-rendering process at the RTC AS.

- Split-Rendering Server (SRS): This function is responsible for negotiation of SR session with SRC, monitoring the server’s edge resource usage, and managing/running the split rendering process.

- Application Function (AF): responsible for provisioning, QoS allocation, and edge resource discovery.

- Application Service Provider: The application provider that offers the service.

- Application: The application running on UE

- Media Session Handler (MSH): is the entity on UE that is responsible for the control plane communication with the AF.

### 5.1.2 	Client Architecture

The client architectural breakdown is based on the client architecture in TS26.119 clause 5.1.

The split rendering client consists of the following components:

- The Media Access Functions: allow for fetching and processing of the pre-rendered media in preparation of final display. The MAF is also responsible for the carriage of any metadata or local media to the split rendering server.

- The scene manager and thin Presentation Engine: is responsible for the negotiation of the split rendering session and the parsing of the description of the rendered media as provided by the SR server. It is also responsible for setting up and managing the XR session with the XR runtime.

- The XR source management is responsible for gathering timed metadata such as pose and action information and sending it to the SR server.

### 5.1.3	End-to-End Architecture

Figure 5.1-3 – Split management architecture

As shown in Figure 5.1.3:

1. The 5G Application Providers (AP) provisions the split-rendering through RTC-1.

2. In the use cases in which the AP is involved in the media delivery, the RTC-2 interface is used for this purpose.

3.The communication between AF and SRS is through RTC-3.  This interface is out of the scope of this document. This interface may for instance include the EDGE-3 interface.

4. The signaling as well as the media delivery between SRC and SRS is though RTC-4.

5. The AF may provide the split-rendering information to the Media Session Handler defined by RTC-5, defined in  TS26.506.

6. SRC in the UE discovers the application through RTC-6 and handles the XR runtime.

7. The SRC discovers the client media capabilities through the RTC-7 interface. This interface is out of the scope of this document.

8. The 5G Application and AP interact through RTC-8-8. This interface is out of the scope of this document.

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

Figure 5.1.5-1 – User Plane Architecture for Split management architecture

The SR interfaces are considered to be specializations of their parent RTC interfaces as defined in TS26.506.

In the context of split rendering, the SR-4 interface is further classified as SR-4s and SR-4m sub-interfaces. The SR-4s interface covers all user-plane signaling, including WebRTC and ICE signaling. The SR-4m serves for media and metadata exchange between the split rendering client and the split rendering server.

## 5.2	Procedures and Call Flows

### 5.2.1 	Call flow for Split Rendering instance discovery

#### 5.2.1.1	Call flow for edge server and split rendering session setup

Figure 5.2.1.1-1 demonstrates a general call flow for split-rendering.

Figure 5.2.1-1: High-level call flow for split-rendering

Steps:

- In this optional step, the Application Provider requests and sets up the edge server(s) used for the split-rendering as described in TS 26.506 clauses 6.1 or 6.2. The Application provider may use any other method to allocation edge servers, or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

- The Application Provider provisions the split-rendering session using SR-1 and SR-3, as defined in call flow of clauses 5.2.1.1. If the edge servers were provisioned in step 1, the edge servers ids are provided in this session to employ them for split-rendering.

NOTE: In the case of the client-driven edge management (TS 26.501 8.1), only the client-driven split-rendering (5.2.1.1) is applicable.

- The split-rendering session is set up according to clause 5.2.2.

#### 5.2.1.2 Client-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for initiating a split

Steps:

- The Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The Application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Session Handler about the client’s media capabilities.

- The SRC and SRS negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRS starts the split rendering process.

- The SRC establishes the WebRTC session.

8. The SRC informs the application that the split-rendering on edge is running.

9. The SRC sends uplink metadata, such as pose and action information.

10. The SRS sends the rendered media to the SRC.

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the as depicted in the call flow in the Figure 5.2.2-1.

Figure 5.2.2- 1 High-level call flow for split rendering session setup and operation

The steps are:

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering server.

- The split rendering server uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF.

# 6	Prerequisites

## 6.1	Requirements on 5G System

## 6.2	Requirements on Device APIs and Functionality

MSE-7

# 7	Split Rendering Application Function

## 7.1	Functionality

The SR application function is an RTC AF that supports the provisioning and configuration of split rendering sessions. The SR AF shall support the procedures for the RTC AF as defined in TS26.506.

## 7.2	RESTful APIs

### 7.2.1	Split-Rendering Provisioning procedures (RTC-1)

#### 7.2.1.1	General

These procedures are used by the Application Provider and the AF on SR-1 to provision for the split-rendering process.

#### 7.2.1.2	Create Split-Rendering Configuration

This procedure is used by the AP to create a new Split-Rendering Configuration. The AP shall use the HTTP POST method for this purpose and the request message body shall include a SplitRenderingConfiguration resource.

If the procedure is successful, the AF shall generate a resource identifier representing the new Split-Rendering Configuration. In this case, the AF shall respond with a 201 (Created) HTTP response message and shall provide the URL to the newly created resource in the Location header field. The response message body may include a SplitRenderingConfiguration resource that represents the current state of the Split Rendering Configuration, including any fields set by the AF.

If the procedure is not successful, the AF shall provide a response code as defined in clause 7.3.

#### 7.2.1.3	Read Split-Rendering Configuration properties

This procedure is used by the AP to obtain the properties of an existing Split-Rendering resource from theAF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the AF shall respond with a 200 (OK) response message that includes the SplitRenderingConfiguration resource in the response message body.

If the procedure is not successful, the AF shall provide a response code as defined in clause 7.3.

#### 7.2.1.4	Update Split-Rendering Configuration properties

The update operation is invoked by the AP to modify the properties of an existing SplitRenderingConfiguration resource. All writeable properties except domainNameAlias may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the AF shall respond with a 200 (OK) and provide the content of the resource in the response, confirming the successful update operation.

If the procedure is not successful, the AF shall provide a response code as defined in clause 7.3.

#### 7.2.1.5	Destroy Split-Rendering Configuration

This operation is used by AP to destroy a Split-Rendering Configuration resource and to terminate the related distribution. The HTTP DELETE method shall be used for this purpose. As a result, the AF will release any associated network resources, purge any cached content, and delete any corresponding configurations.

If the procedure is successful, the AF shall respond with a 200 (OK) response message.

If the procedure is not successful, the A.F shall provide a response code as defined in clause 7.3.

### 7.2.2		Split-Rendering Provisioning API

### 7.2.2.1	Overview

This clause specifies the API that the AP uses at interface SR-1 to create and manage Provisioning sessions for media services that use split rendering. Each split rendering configuration is represented by a SplitRenderingConfiguration, for which the resource structure is specified in 7.2.2.2 and the data model is specified in clause 7.2.2.3.

### 7.2.2.2	Resource structure

The Split-Rendering Provisioning API is accessible through this URL base path:

{apiRoot}/3gpp-rtc1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

Table 7.2.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.2.2-1: Operations supported by the Split-Rendering Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Split-Rendering Configuration | split-rendering-configuration | POST | Used to create a Split-Rendering Configuration resource. |
| Retrieve Split-Rendering Configuration |  | GET | Used to retrieve an existing Split-Rendering Configuration. |
| Update Split-Rendering Configuration |  | PUT, PATCH | Used to modify an existing Split-Rendering Configuration. |
| Delete Split-Rendering Configuration |  | DELETE | Used to delete an existing Split-Rendering Configuration. |

### 7.2.2.3	Data model

#### 7.2.2.3.1	SplitRenderingConfiguration resource

The data model for the SplitRenderingConfiguration resource is specified in table 7.2.2-2 below:

Table 7.2.2-2: Definition of SplitRenderingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| Name | String | 1..1 | A name for this Split Rendering Configuration. |
| Status | Boolean | 1..1 | Indicates whether to this split rendering configuration is active |
| edgeResourceConfigurationId | ResourceId | 0..1 | The identifier of the edge resource configuration that will be used for sessions of this split rendering configuration. |
| policyTemplateId | ResourceId | 1..1 | The identifier of the policy template that will be applied to the sessions of this split rendering configuration. |
| Configuration | Object | 1..1 | Describes the split-rendering configuration currently used by the SRS. Editor’s Note: The syntax and semantics of this element are TBD. |

### 7.2.2.4	Configuration Guidelines for Split Rendering

#### 7.2.2.4.1	Guidelines on Provisioning Session

The ProvisioningSessionType shall be set to “BIDIRECTIONAL”.

The aspId shall be configured and shall be a unique identifier for the Application Service Provider that offers split rendering.

The externalApplicationId shall be a URN that uniquely identifies the application and shall be terminated by the sub-string “+3gpp-sr”. An examples is as follows: “urn:com:example:game+3gpp-sr”.

#### 7.2.2.4.2	Guidelines on Edge Resource Configuration

A split rendering application may use the procedures defined in TS26.512 clause 7.10 to define an edge resource configuration to be used for split-rendering  session. In this case:

- The eligibilityCriteria shall be present and shall have appRequest set to true.

- The easRequirements shall indicate “SR” as the easType and shall include “3gpp-sr” among the easFeatures. The serviceKpi shall be present and indicate the SRS processing and networking capabilities and requirements.

- The easRelocationRequirements shall indicate “RELOCATION_INTOLERANT” in the tolerance field.

#### 7.2.2.4.3	Guidelines on Policy Template

A policy template for the split rendering shall be associated with the split rendering configuration. The QoS specification should include at least the following sub-streams:

- 2 configurations for left and right eye buffer streams

- 1 optional configuration for a depth buffer stream

- 1 configuration for an audio stream

### Dynamic Policy API for Split Rendering

The WebRTC Signaling Server (which potentially maybe part of the Split Rendering Server) shall support the dynamic policy API as defined in X.

Furthermore, the Split Rendering Server shall support the PDU Set marking and should support the End of Burst marking for the RTP streams that are generated by the Split Rendering Server.

## 7.3	HTTP response codes

Guidelines for error responses to the invocation of APIs of NF services are specified in clause 4.8 of TS 29.501 [22]. API-specific error responses are specified in the respective technical specifications.

# 8	Split Rendering User Plane

## 8.1	Split Rendering Signalling Protocols

Both SRC and SRS shall support the SWAP protocol as defined in TS26.113 clause 6.2.

The SWAP protocol allows for the definition of application-specific messages.

For Split Rendering, the following application-specific messages shall be supported:

- The configuration message carries the split rendering configuration information from the SRC to the SRS. It shall be identified by the type “urn:3gpp:sr-mse:sr-configuration” and the object shall be formatted according to clause 8.4.2.2.

- The rendering description message carries the description of the split rendered media from the SRS to SRC. It shall be identified by the type “urn:3gpp:sr-mse:sr-description” and the object shall be formatted according to clause 8.4.3. The rendering description message provides the semantics of the media that is delivered over WebRTC from the SRS to SRC.

The SWAP message exchange for the establishment of a split rendering session is depicted by the following call flow diagram:

Pre-requisites:

- The SRC has discovered the identifier of the SRS that it will use for its split rendering session.

- The SRC has retrieved the address of the SWAP server as part of the configuration.

The stpes are as follows:

- The SRC sends the configuration message as an application-specific SWAP message to the SWAP server. It provides the identifier of the target SRS as a matching criteria.

- The SWAP server uses the provided matching criteria to locate the SRS.

- The SWAP server forwards the configuration message to the target SRS.

- The SWAP server confirms the successful forwarding of the message to the SRC

- The SRS processes the SR configuration message. It may for instance verify application and resource availablity, launch the application, configure its rendering, and create a rendering description.

- The SRS sends the rendering description message as an application-specific SWAP message to the SWAP server.

- The SWAP server forwards the message to the SRC.

- The SWAP server acknowledges the successful forwarding of the message to the SRS.

- The SRC processes the rendering description and identifies the required data channel and media sessions.

- SRC sends a connect message with the SDP offer to the SRS. The offer reflects the negotiated media and data channel streams.

- The SWAP server acknowledges the forwarding of the message to the SRS

- The SRS replies with an accept message that includes the SDP answer. The SDP answer reflects the information that was provided in the split rendering description.

- The SWAP server acknowledges the forwarding of the message to the SRC

## 8.2	Split Rendering Formats

### 8.2.1 	General

This clause defines media and metadata formats that are common to one or more split rendering profiles.

### 8.2.2	Metadata Formats

#### 8.2.2.1	General

In the “3gpp-sr” data channel sub-protocol, the message content depends on the type of the message. The data channel sub-protocol is defined in clause X.

Message types shall be unique identifiers in the URN format. This clause defines a set of message types and their formats. The messages are derived from the OpenXR API to ensure smooth operation with AR devices that support OpenXR. In case other XR APIs are used, mapping the message payload to the appropriate XR API structures shall be performed by the split rendering client.

Editor’s Note: This following sections will potentially reference the corresponding formats in 26.119.

#### 8.2.2.2	Pose Format

The split rendering client on the XR device periodically transmits a set of pose predictions to the split rendering server. The type of the message shall be set to “urn:3gpp:split-rendering:v1:pose”.

Each predicted pose shall contain the associated predicted display time and an identifier of the XR space that was used for that pose.

Depending on the view configuration of the XR session, there could be different pose information for each view.

The payload of the message shall be as follows:

Table 8 - Pose Prediction Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| poseInfo | Object | 1..n | An array of pose information objects, each corresponding to a target display time and XR space. |
| displayTime | number | 1..1 | The time for which the current view poses are predicted. |
| xrSpace | number | 0..1 | An identifier for the XR space in which the view poses are expressed. The set of XR spaces are agreed on between the split rendering client and the split rendering server at the setup of the split rendering session. The set of XR spaces is negotiated as part of the split rendering configuration as defined in clause 8.4.2.2. |
| viewPoses | Object | 0..n | An array that provides a list of the poses associated with every view. The number of views is determined during the split rendering session setup between the split rendering client and server, depending on the view configuration of the XR session. |
| pose | Object | 1..1 | An object that carries the pose information for a particular view. |
| orientation | Object | 1..1 | Represents the orientation of the view pose as a quaternion based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the quaternion. |
| y | number | 1..1 | Provides the y coordinate of the quaternion. |
| z | number | 1..1 | Provides the z coordinate of the quaternion. |
| w | number | 1..1 | Provides the w coordinate of the quaternion. |
| position | Object | 0..1 | Represents the location in 3D space of the pose based on the reference XR space. For eye gaze poses, the position is not required. |
| x | number | 1..1 | Provides the x coordinate of the position vector. |
| y | number | 1..1 | Provides the y coordinate of the position vector. |
| z | number | 1..1 | Provides the z coordinate of the position vector. |
| confidence | number | 0..1 | This optional parameter provides a confidence score that reflects the probability for this pose prediction to be correct. For the current pose or a pose in the past, the confidence value would be 1. The confidence can take a value between 0 and 1. If not provided by the XR runtime, this field may be estimated by the SRC or omitted. |
| fov | Object | 1..1 | Indicates the four sides of the field of view used for the projection of the corresponding XR view. |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |

#### 8.2.2.3	Action Format

Actions are grouped into action sets, which may be activated and deactivated during the lifetime of an XR session. The action sets and actions are negotiated at the start of the split rendering session.

The split rendering client reports any changes to action state as soon as it occurs by sending a message of the type “urn:3gpp:split-rendering:v1:action”.

The content of the action message type shall follow the following format:

Table 9 - Action Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| actionSets | Object | 1..n | An array of active action sets, for which there is at least an action that has a state change. |
| actions | Object | 1..n | An array of objects that conveys information about the actions of the parent action set. |
| identifier | string | 1..1 | A unique identifier of the action that was agreed upon during split rendering session setup. |
| subactionPath | string | 1..1 | The sub-action path for which the state has changed. It abstracts a binding between an action and the hardware input associated to it by the XR runtime. |
| state | object | 1..1 | The state of the action that had a change in state. |
| lastChangeTime | number | 1..1 | The timestamp of the last change to the state of this action. |
| currentStateBool | Bool | 0..1 | The current Boolean state of the action |
| currentStateNum | number | 0..1 | The current numerical state of the action. |
| currentStateVec2 | Array | 0..1 | An array of numerical state values for the action. |

### 8.2.3	Metadata Data Channel Message Format

For the carriage of metadata defined in clause 8.2, such as pose and action information, the SRS and SRC shall use the WebRTC data channel. The data channel sub-protocol shall be identified as “3gpp-sr-metadata”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The split rendering metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

A data channel message may carry one or more split rendering messages as defined in Table 8.2.3-1.

Table 8.2.3-1 Split Rendering Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of split rendering metadata messages. Each message shall be formatted according to the Message data type as defined in Table |

Each split rendering message shall follow the format specified in Table 8.2.3-2.

Table 8.2.3-2 Split Rendering Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | An unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | A urn that identifies the message type. |
| message | object | 1..1 | The message content depends on the message type. |

## 8.3	Split Rendering Transport Protocols

Split Rendering shall use WebRTC for the real-time transport of the rendered media. The RTP restrictions for WebRTC as specified in RFC8834 shall apply. The usage of the WebRTC data channel shall be in accordance with RFC8831.

Editor’s Note: applicable guidelines for the usage of the PDU Set Marking is pending completion of TS26.522.

## 8.4 	Split Rendering Formats for Session Setup and Negotiation

### 8.4.1 	General

In Figure 5.2.1-1 and 5.2.1-2, step 5 defines the negotiation between the SRC and SRS for the split-rendering configuration. In most simple case, the SRC provides SRS the capabilities of the device and if SRS can accommodate the split-rendering processing that addresses the device, it confirms the configuration. In such scheme, the SRS is responsible to make the decision and no back-and-forth negotiation occurs.

### 8.4.2 	Split Rendering Configuration Format

#### 8.4.2.1	Introduction

The Split Rendering client establishes an XR session locally based on the device configuration and user selection. The SR client defines the view configuration (e.g. mono or stereo views), the projection format (such as projection, equirectangular, quad, or cubemap), the swap chain image configuration, etc.

In addition, XR space and action configurations are negotiated between the SR client and server. This includes defining common XR spaces and defining and selecting actions and action sets.

The format is extensible to support the exchange of additional/future configuration information.

#### 8.4.2.2	 Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name |  | Type | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) |  |  | 0..1 |  | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |
| spaceConfiguration |  | Object | 0..1 |  | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |  |
| referenceSpaces |  | Array | 0..1 |  | An array of reference spaces and their identifiers. |  |
| id |  | number | 1..1 |  | A unique identifier of the XR space in the context of the split rendering session. |  |
| refSpace |  | enum | 1..1 |  | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |  |
| actionSpaces |  | Array | 0..1 |  | An array of action spaces that need to be defined by the split rendering client in the XR session. |  |
| id |  | number | 1..1 |  | A unique identifier of the XR space in the context of the split rendering session. |  |
| actionId |  | number | 1..1 |  | Provides the unique identifier of the action. |  |
| subactionPath |  | string | 1..1 |  | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |  |
| initialPose |  | Pose | 0..1 |  | Provides the initial pose of the new XR space’s origin. |  |
| viewConfiguration |  | Object | 0..1 |  | Conveys the view configuration that is configured for the XR session. |  |
| type |  | Enum | 1..1 |  | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |  |
| width |  | number | 1..1 |  | The recommended width of the swapchain image. |  |
| height |  | number | 1..1 |  | The recommended height of the swapchain image. |  |
| compositionLayer |  | string | 1..1 |  | An identifier of the selected composition layer. |  |
| environmentBlendMode |  | enum | 1..1 |  | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |  |
| actionConfiguration |  | Array | 0..1 |  | This contains a list of the actions that are to be defined by the SR client. |  |
| action |  | Object | 1..n |  | A definition of a single action object. |  |
| id |  | number | 1..1 |  | A unique identifier of the action. |  |
| actionType |  | enum | 1..1 |  | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |  |
| subactionPaths |  | string | 1..n |  | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |  |
| extraConfigurations |  | Object | 0..1 |  | A placeholder for addition configuration information. |  |

# 9	Split Rendering Client

## 9.1	Functionality

The Split Rendering Client (SRC) is a function that runs on the UE to provide split rendering functionality to applications. The SRC is designed to be offered as an SDK to application developers. The SRC abstracts the details of the split rendering operation and provides a simple to use API to the application to facilitate the usage of split rendering.

The SRC performs the following functions:

- Creates and manages the XR session

- Discovers and selects a split rendering server (SRS) in the network

- Establishes a split rendering session with the SRS

- Communicates the necessary information about the session to the MSH/AF to benefit from dynamic policy, network assistance, consumption reporting, etc.

- Operates the rendering loop on the UE

## 9.2	Client API

As described in clause 5.1.3, the SRC exposes an API over RTC-7 interface to the application. The SRC defines the following interface:

| Method | State after Success | Description |
| --- | --- | --- |
| SplitRenderer() | STATE_READY | Creates a SplitRenderer object, which can subsequently be used to connect to an SRS and perform split rendering. |
| connect() | STATE_CONNECTED | Instructs the SRC to discover and connect to an SRS. |
| disconnect() | STATE_DISCONNECTED | Terminates the connection to the SRS. |
| getMetrics() | - | Retrieves a set of metric reports for the split rendering session that describe the quality of experience of the session. |

The SplitRenderer interface is defined using the IDL syntax (according to ISO/IEC 19516) as follows:

interface SplitRenderer { readonly attribute SRState state; attribute EventHandler onstatechange; attribute EventHandler onerror; attribute EventHandler onqualitychange; void SplitRenderer(String application_id); void connect(); void disconnect(); Metrics getMetrics(); };

The application is able to subscribe to events related to the split rendering session by setting the corresponding event handler. The supported events are:

- State change: the state of the SR session has changed

- Error: an error has occurred during the split rendering session. The error is not severe enough to cause a state change to the STATE_ERROR state.

- Quality change: the SRC has observed a change in the quality of the split rendering session. This may involve one or more SR metrics.

## 9.3	Split Rendering Metrics

Editor’s Note: to be updated once the AR/MR QoE metrics are specified.

[

Relevant metrices related to split rendering depend upon the sepcific application category, that is, VR, AR and/or MR.

Split rendering QoE metrics may include, but is not limited to:

- QoE metrics related with device information, e.g. Eye Relief, lifetime, gaze-based metrics and head-motion-based metrics, etc.

- QoE metrics related to the network transmission quality.

- metrics describing the characteristics of the AR/MR content creation, content rendering and content encoding (e.g. the quality for the generated or rendered AR/MR content, the motion-to-render-to-photon latency, etc).

- metrics describing the characteristics of immersiveness and presence for AR/MR, such as tracking (e.g. tracking errors), world-scale experience (map world-scale experience to QoE metrics), persistence (map persistence to QoE metrics) etc.

For specific use cases of VR, Split Rendering QoE metrices may be based on the 3GPP TR 3GPP TR 26.928 [2] and TR 26.929 [3].

For specific use cases AR and MR, Split Rendering QoE metrices may be based on the 3GPP TR 26.812 [4].

]

# 10 	Security and Privacy Aspects

## 10.1 	Security

Signaling for session establishment and exchange of application-specific messages shall use a secure transport channel based on WebSockets as defined in TS26.113.

Media transport shall be secured by the usage of WebRTC.

## 10.2 	Privacy

Editor’s Note: Privacy considerations are FFS.

# Annex A	 (Informative):
Implementation Guidelines

## Annex A.1 Guidelines for Application Developers

## Annex A.2 Guidelines for Split Rendering MSE Implementers

### A.2.1	Guidelines for implementers of the Split Rendering Server

If the use of eye gaze tracking is activated, the SRS may use this confidence information to perform foveated rendering. In foveated rendering, areas of the picture are encoded with a higher SNR quality than other areas, to match the user’s current gaze.

With gaze predictions, the SRS should create an importance map for the picture based on the confidence values associated with the gaze predictions. Additionally, the SRS may also use other information to produce the importance map, such as content Regions of Interest. A low confidence score may indicate that the estimation on the device is not adequate. In this case the server can try to re-estimate the pose prediction prior to rendering.

This importance map is passed to the encoder to properly allocate bits for the encoding of the picture.

## Annex A.3 Conformance Testing

Annex B (normative):
IDL Definition of Client API

Annex C (normative):
Split Rendering Profiles

## C.1 	Pixel Streaming Profile

## C.1 	Introduction

This Annex defines split rendering profiles to define requirements for SRC and SRS for different scenarios. At this stage the following two profiles are defined:

- 2D Pixel Streaming Profile in clause C.2 to support split rendering to 2D screens

- 3D Pixel Streaming Profile in clause C.3 to support split rendering to MeCAR glasses

## C.2 	2D Pixel Streaming Profile

### C.2.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support split rendering to 2D screens.

### C.2.2	SRC Capabilities

#### C.2.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### C.2.2.2	Video decoding

Based on the definitions in MeCAR PD, clause 6.6, an SRC shall support

-	the AVC-UHD-Dec decoding capability.

-	the HEVC-UHD-Dec decoding capability.

An SRC should support

-	the AVC-UHD-Dec-4 decoding capability.

-	the HEVC-UHD-Dec-4 decoding capability.

-	the UHD-Dec-4 decoding capability.

A SRC Client may support

-	the HEVC-8K-Dec decoding capability.

-	the 8K-Dec-8 decoding capability.

#### C.2.2.3	Audio and Speech decoding

The SRC client shall support the capabilities required for speech and audio decoding as defined in clause 6.7.2 (of MeCAR document).

#### C.2.2.4	Video encoding

Editor’s Note: Video Encoding Capabilities are for further study

#### C.2.2.5	Audio and Speech encoding

Editor’s Note: Audio and Video Encoding Capabilities are for further study

#### C.2.2.6 	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

### C.2.3	SRS Capabilities

#### C.2.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

#### C.2.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

#### C.2.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

#### C.2.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

#### C.2.3.5	Audio and Speech decoding

Editor’s Note: Audio and Video Decoding Capabilities are for further study

#### C.2.3.6 	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.

## C.3 	3D Pixel Streaming Profile

### C.3.1	Introduction

This profile defines required capabilities for UE-based SRC functionalities as network-side SRS capabilities to support MeCAR devices.

### C.3.2	SRC Capabilities

#### C.3.2.1	Overview

Requirements for UE-based SRC functionalities for following functions are defined in this clause:

-	Media Decoding

-	Media Encoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### C.3.2.2	Video decoding

Based on the definitions in MeCAR PD, clause 6.6, an SRC shall support

-	the AVC-UHD-Dec decoding capability.

-	the HEVC-UHD-Dec decoding capability.

-	the AVC-UHD-Dec-4 decoding capability.

-	the HEVC-UHD-Dec-4 decoding capability.

-	the UHD-Dec-4 decoding capability.

A SRC Client may support

-	the HEVC-8K-Dec decoding capability.

-	the 8K-Dec-8 decoding capability.

#### C.3.2.3	Audio and Speech decoding

The SRC client shall support the capabilities required for speech and audio decoding as defined in clause 6.7.2 (of MeCAR document).

#### C.3.2.4	Video encoding

Editor’s Note: Video Encoding Capabilities are for further study

#### C.3.2.5	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding Capabilities are for further study

#### C.3.2.6 	Metadata Formats

XR-Pose-Cap 1: the SRC shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered. The pose predication shall be formatted according to clause 8.2.2.2.

XR-Pose-Cap 2: the SRC shall be able to retrieve and collect the user actions that occurred during an identified time interval. The action information shall be formatted according to clause 8.2.2.3.

### C.3.3	SRS Capabilities

#### C.3.3.1	Overview

Requirements for network-based SRS functionalities for following functions are defined in this clause:

-	Media Encoding

-	Media Decoding

-	Metadata Formats

Editor’s Note: Additional Media Capabilities are for further study

The capabilities of the SRC are shared with the SRC prior to the start of the split rendering session.

Editor’s Note: Signaling of capabilities and configurations are for further study.

#### C.3.3.2	Video encoding

Editor’s Note: Video Encoding capabilities is for further study to match the SRC capabilities.

#### C.3.3.3	Audio and Speech encoding

Editor’s Note: Audio and Speech Encoding capabilities is for further study to match the SRC capabilities.

#### C.3.3.4	Video decoding

Editor’s Note: Video Decoding Capabilities are for further study

#### C.3.3.5	Audio and Speech decoding

Editor’s Note: Audio and Speech Decoding Capabilities are for further study

#### C.3.3.6 	Metadata Formats

Editor’s Note: Metadata capabilities are for further study to match the SRC capabilities.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| April 2023 | 123-e | S4-230726 |  |  |  | Improvements and Corrections to edge and dynamic policy procedures in SR |  |
| May 2023 | 124 | S4-121075 |  |  |  | General updates to TS26.565 |  |
| May 2023 | 124 | S4-121004 |  |  |  | SR Rendering API |  |
| May 2023 | 124 | S4-231005 |  |  |  | Pixel Streaming Media Profile |  |
| May 2023 | 124 | S4-231003 |  |  |  | pCR on signaling for SR session control |  |
| May 2023 | 124 | S4-230925 |  |  |  | On SR configuration API and view configuration |  |
| August 2023 | 125 | S4-231449 |  |  |  | [SR_MSE] Transport protocols |  |
| August 2023 | 125 | S4-231518 |  |  |  | [SR_MSE] Rendering optimization |  |
| August 2023 | 125 | S4-231432 |  |  |  | [SR_MSE] Updates to Media Capabilities |  |
| August 2023 | 125 | S4-231324 |  |  |  | Split rendering Metrics |  |
| August 2023 | 125 | S4-231434 |  |  |  | Editorial corrections on SR MSE architectures |  |