3GPP TSG-WG SA4 Meeting #125	S4-231524

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S4-231275)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PESUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | xxxx | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guidelines for PDU Set identification without PDU Set RTP HE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the LS S2-2303849 as shown below, SA4 is asked to provide how the PDU Set information can be derived from the existing RTP/SRTP headers, header extensions and/or payloads when the PDU Set based RTP header extension is unavailable. Hence this paper intends to provide the guidelines for that as informative annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Annex of guidelines on PDU Set information identification based on existing RTP/SRTP headers and payloads. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to derive the PDU Set information based on the existing RTP/SRTP header and payloads are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T Rec H.264: "Advanced video coding specification".
https://www.itu.int/rec/T-REC-H.264-202108-I/en

[3]	ITU-T Rec H.265: "High efficiency video coding specification ".
https://www.itu.int/rec/T-REC-H.265-202108-I/en

[x]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[y]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[z]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# * * * * Second change * * * *

Annex X	Guildelines for PDU Set identification without PDU Set RTP HE

When the PDU Set based RTP Header Extension is not available, some or all of PDU Set information can be derived by the RTP/SRTP header, header extension and/or payloads. The possible PDU Set information to be derived based on the RTP/SRTP header, header extension and the payloads are provided as following.

X.1 RTP/SRTP header

Figure 1 RTP header fields as defined in RFC 3550 [x]

When the RTP/SRTP is used to convey the video content and the PDU Set represents a video frame, the video frame may be identified based on the RTP header fields as following:

-	The "marker (M)" bit is used with the video payload formats in clause 2.2 to indicate the frame boundary, by setting the M bit on the last PDU of a frame. With the "M" bit and the sequence number in RTP header, the Indication of End PDU of a PDU Set and PDU SN within a PDU Set/frame can be derived.

-	The "timestamp" field indicates the sampling instant of the first octet in the RTP data packet and the video frame is generally with the same timestamp. Therefore, with the "timestamp" field and the sequence number in RTP header, the Indication of End PDU of a PDU Set and PDU SN within a PDU Set/frame can be derived.

NOTE 1:	When multiple RTP streams multiplexed over a single RTP session, the "M" bit/"timestamp" field, and sequence number information can be used together with the ssynchronization source (SSRC) in the RTP header to identify the boundary of video frame for each of the RTP streams that can be separated by their different SSRC values.

NOTE 2:	For the timestamp-based solution, generally, the end PDU of the PDU Set can only be determined when a PDU with new RTP timestamp arrives, which may introduce additional latency.

The PDU Set/frame size and the PDU Set importance cannot be derived by the RTP header fields.

X.2 RTP payload

    - X.2.1 General

The PDU Set information identification based on the RTP payload format is present in this clause, including the RTP payload format for H.264/AVC and H.265/HEVC codecs. It is assumed that the 5GC (i.e. UPF) is aware of the RTP payload format in advance.

NOTE:	This clause applies only when the RTP payload is not encrypted.

    - X.2.2	RTP payload for H.264/AVC codec

For a video slice with H.264 RTP payload, the PDU Set Information can be realized by following approach.

According to RFC 6184 [y], within the RTP packet, the NAL unit type in the NAL unit header can indicate the content of NAL unit, e.g. coded slice of an I frame, coded slice of a P frame, and also the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet and fragmentation unit (FU).

- For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet can be treated as a single PDU Set when the NAL unit type is less than 28.

- When NAL unit type is 28 or 29, one NAL unit is carried over multiple RTP packets. In this case, the first byte of RTP payload is also named the fragmented unit (FU) indicator and the following byte is the FU header. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the PDU Set can be identified.

Figure 3 NAL unit header format for H.264 [y]

With the RTP payload (i.e. NAL unit header and optionally FU header) and the sequence number in the RTP header, the indication of the End PDU of the PDU Set and the PDU SN within a PDU Set can be derived.

As described in clause 4.4.2.6.2.2, the Type and NRI value in the NAL unit header indicates the relative transport priority and can be used to set the PDU Set importance. Besides, different NRI values can also indicate different requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

The PDU Set Size in bytes cannot be derived based on the RTP payload for a H.264/AVC codec.

    - X.2.3	RTP payload for H.265/HEVC codec

For a video slice with H.265 RTP payload, the identification of the PDU Set can be realized by following approach.

According to RFC 7798 [z], within the RTP packet, the NAL unit header can indicate the content of NAL unit. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet (APs), fragmentation unit (FUs) and Payload Content Information (PACI) carrying RTP packet.

-	For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet can be treated as a single PDU Set when the NAL unit type is less than 49.

-	When NAL unit type is 49, one NAL unit is carried over multiple RTP packets. In this case, the first two-byte of RTP payload is also named the payload header (denoted as NAL U header) and the following byte is the FU header. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the PDU Set can be identified.

-	When NAL unit type is 50, this is a PACI packet which may carry a single NAL unit packet or FU. In this case, the first two-byte of RTP payload is also named as the PACI header (denoted as NAL Unit header). In the following two bytes, the "A" bit is the copy of "F" bit and cType field is the copy of Type field in the PACI payload NAL unit. Then the following is the PHES field, whose length is determined by the PHSize. Finally, the following is the PACI payload NAL unit, during which the first byte is FU header when cType is 49. Therefore, based on the PACI header and PACI payload NAL unit, the start/end of the PDU Set can be identified.

Figure 4 The Structure of the HEVC NAL Unit Header [z]

Figure 5 The Structure of FU Header

With the RTP payload (i.e. NAL unit header and optionally FU header) and the sequence number in the RTP header, the indication of the End PDU of the PDU Set and the PDU SN within a PDU Set can be derived.

As described in clause 4.4.2.6.2.3, the Type field and the TID field in the NAL unit header indicates the relative transport priority and can be used to be mapped to the PDU Set importance. While they can also indicate different requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

The PDU Set Size in bytes cannot be derived based on the RTP payload for a H.265/HEVC codec.

# * * * * End of changes * * * *