3GPP TSG-SA4 Meeting #125	S4-231578 (rev of S4-231512 rev of S4-231414

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023					         rev of S4-231314)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0066 | rev | 5 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMSA, TEI18] Clarifications to Network Assistance feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, BBC, Dolby |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA, TEI18 |  |  |  |  |  | Date: |  |  | 2023-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some unclarities and missing inputs have been identified regarding the Network Assistance feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Harmonize the use of the same terminology (i.e. throughput estimation, delivery boost) throughout clauses 4.0.5 and 4.7.4.7, as the rest of the document. - The expressions “bit rate recommendation” and “throughput estimation” are used interchangeably in TS 26.501. We harmonized and propgated the same in clause 4.7.3. - A descriptive sentence is added in clause 5.9.1 to clarify the definition of the “recommended bit rate” concept. - Clause 5.9.1 only applies to Network Assistance for downlink media streaming. A symmetric statement about uplink Network Assistance is added in clause 6.1. - The missing interactions with the PCF added to the high-level Procedure in 6.5. - Removal of the following requirement in Metrics Collection Process: an error message shall be sent by the Media Session Handler to the appropriate network entity. - Change in Downlink network Assistance: nominal time period to valid until further notice. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The definition of the Network Assistance feature is incomplete. - Unclarities for implementers. - 5GMS stage 2 and Stage 3 specifications are misaligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0.5, 4.7.3, 4.7.3.1, 4.7.3.2, 4.7.4.7, 5.5.3, 5.9.1, 5.9.2, 6.1, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-230887 -> S4aI230112 -> S4-231314 -> S4-231414 |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.0.5	Network assistance

The network assistance feature is applicable to both downlink media streaming and uplink media streaming. It enables the 5GMS Client in the UE to interrogate or manipulate the network Quality of Service for an ongoing media streaming session.

High-level procedures for this feature are defined in clause 5.9 (downlink media streaming) and in clauses 6.1, 6.5 and 6.7 (uplink media streaming). The network assistance feature is not explicitly provisioned by the 5GMS Application Provider. It is either available for a particular media streaming session or not, depending on system pre-configuration and/or policy.

Two mechanisms for obtaining network assistance are defined in the present document: one based on interactions with the PCF via network-based components of the 5GMS System (AF-based network assistance), the other based on ANBR signalling interactions between the UE modem and the RAN (ANBR-based network assistance).

Figure 4.0.5-1: High-level arrangement for network assistance feature

The following AF-based network assistance sub-features are defined in this release:

1.	Bit rate recommendation (or throughput estimation). The 5GMS Client requests an estimate from a network-side component of the 5GMS System of the bit rate that can currently be offered by a media streaming session. The network-side component interrogates the PCF on behalf of the 5GMS Client to obtain this information about the PDU session corresponding to the media streaming session.

The 5GMS Client uses this information to adjust its own streaming bit rate to fit within the Quality of Service (QoS) envelope that the network is able to offer, for example by switching to a different representation listed in its Media Entry Point, or by adjusting the encoding bit rate for uplink streaming to fits within this bit rate budget. The media streaming Quality of Experience (QoE) is more stable and consistent as a consequence.

2.	Delivery boost. The 5GMS Client speculatively requests a temporary boost to the bit rate of a media streaming session from a network-side component of the  5GMS System. The network-side component requests a modification to the PDU session corresponding to the media streaming session from the PCF on behalf of the 5GMS Client. If there is sufficient spare network capacity to accommodate the requested bit rate, it is granted by the 5GMS System on a temporary basis.

The 5GMS Client uses this temporary boost to speed up media streaming data transfer, for example to replenish a depleted downlink streaming buffer or to complete a download/upload faster than would otherwise be possible.

ANBR-based bit rate recommendation is also defined for downlink media streaming (see clause 5.9.3).

ANBR-based delivery boost is also defined for uplink media streaming (see clause 6.7).

In addition, the use of network assistance by 5GMS Clients is logged by the 5GMS System and, if suitably provisioned, is exposed by it to subscribing 5GMS Application Providers in the form of events (see also clause 4.0.12).

NEXT CHANGE

### 4.7.3	UE data processing for 5GMS

#### 4.7.3.1	UE data processing procedures for downlink media streaming

The following restriction dimensions and aggregation functions defined in clause 4.5.2 of TS 26.531 [22] may be provisioned in a Data Access Profile as part of a 5GMSd Provisioning Session and shall, as a consequence, be applied to reported UE data prior to exposing it to event consumers.

Table 4.7.3.1-1: Valid processing of downlink media streaming UE data by the Data Collection AF

|  | Restriction dimension |  |  | Aggregation function |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time | User | Location | None | Count | Mean | Maximum | Minimum | Sum |
| QoE metrics for downlink media streaming | Yes | Yes | Yes | Yes | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) |
| Consumption of downlink media streaming | Yes | Yes | Yes | Yes | Yes (NOTE 2) | No | No | No | No |
| Downlink dynamic policy invocations | Yes | Yes | Yes | Yes | Yes (NOTE 3) | No | No | No | No |
| AF-based downlink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 3) | Yes (NOTE 4) | Yes (NOTE 4) | Yes (NOTE 4) | No |
| ANBR-based downlink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 3) | Yes (NOTE 4) | Yes (NOTE 4) | Yes (NOTE 4) | No |
| Downlink media streaming access activity | Yes | Yes | Yes | Yes | Yes (NOTE 2) | No | No | No | No |
| NOTE 1: Aggregation functions applied individually to all exposed metrics within the scope of the applicable restriction dimension(s). NOTE 2: Number of downlink media streaming sessions within the scope of the applicable restriction dimension(s). NOTE 3: Number of invocations within the scope of the applicable restriction dimension(s). NOTE 4: Aggregation functions applied to bit rate recommendations within the scope of the applicable restriction dimension(s). |  |  |  |  |  |  |  |  |  |

#### 4.7.3.2	UE data processing procedures for uplink media streaming

The following restriction dimensions and aggregation functions defined in clause 4.5.2 of TS 26.531 [22] may be provisioned in a Data Access Profile as part of a 5GMSu Provisioning Session and shall, as a consequence, be applied to reported UE data prior to exposing it to event consumers.

Table 4.7.3.2-1: Valid processing of uplink media streaming UE data by the Data Collection AF

|  | Restriction dimension |  |  | Aggregation function |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time | User | Location | None | Count | Mean | Maximum | Minimum | Sum |
| AF-based uplink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 1) | Yes (NOTE 2) | Yes (NOTE 2) | Yes (NOTE 2) | No |
| ANBR-based uplink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 1) | Yes (NOTE 2) | Yes (NOTE 2) | Yes (NOTE 2) | No |
| NOTE 1: Number of invocations within the scope of the applicable restriction dimension(s). NOTE 2: Aggregation functions applied to bit rate recommendations within the scope of the applicable restriction dimension(s). |  |  |  |  |  |  |  |  |  |

NEXT CHANGE

#### 4.7.4.7	Event exposure of Network Assistance UE data

For both AF-based Network Assistance and ANBR-based Network Assistance, the Data Collection AF shall include the following parameters (derived from the baseline parameters defined in table 4.7.4.3-1) in each event it exposes to event consumers:

Table 4.7.4.7-1: Parameters for Network Assistance invocations collection

| Parameter | Cardinality | Description |
| --- | --- | --- |
| Collection timestamp | 1..1 | The date–time at which the collection was exposed by the Data Collection AF as an event to its subscribed event consumers. |
| Start timestamp | 1..1 | Date–time of the earliest Network Assistance invocation included in or summarised by this collection. |
| End timestamp | 1..1 | Date–time of the latest Network Assistance invocation included in or summarised by this collection. |
| Sample count | 1..1 | The number of Network Assistance invocations included in or summarised by this collection. Where summary records are included in the collection, the number of records in the collection differs from this number. |
| Media streaming direction | 1..1 | Indicating whether this collection of Network Assistance invocations relates to downlink media streaming sessions or to uplink media streaming sessions. |
| Record summarisation descriptor | 1..* | Nature of the records included in this collection. One or more of the following: - None: Event includes a separate record for each Network Assistance invocation. - Count: Event describes only the number of Network Assistance invocations over a certain time period with no individual records provided. - Mean: Event includes a record summarising the mean average value of each QoS parameter over a certain time period. - Minimum: Event includes a record summarising the smallest value of each QoS parameter over a certain time period. - Maximum: Event includes a record summarising the largest value of each QoS parameter over a certain time period. |
| Records | 0..* | A set of records, each one describing a Network Assistance invocation or summarising a set of Network Assistance invocations. |

And for each record in the collection the following parameters shall be included, extending the baseline parameter set defined in table 4.7.4.3-2:

Table 4.7.4.7-2: Parameters for Network Assistance invocation record

| Parameter |  | Cardinality | Description |
| --- | --- | --- | --- |
| Record type |  | 1..1 | Indicating the nature of information carried in this Network Assistance invocation record: - Individual invocation record. - Mean parameter values summary record. - Minimum parameter values summary record. - Maximum parameter values summary record. |
| UE identification |  | 0..1 | GPSI and/or IP address of the UE seeking Network Assistance. Present only for individual invocation record type. |
| Data Network Name |  | 0..1 | Identifying the Data Network of the M4 media streaming session for which Network Assistance was sought. Present only for individual invocation record type. |
| Slice identification |  | 0..1 | The S-NSSAI identifying the Network Slice of the M4 media streaming session on which Network Assistance was sought. Present only for individual invocation record type. |
| UE location |  | 0..1 | The location of the UE when Network Assistance was sought. Present only for individual invocation record type. |
| Record subtype |  | 1..1 | Indicating which of the following Network Assistance features was invoked by the UE: - Bit rate recommendation solicited from 5GMS AF by 5GMS Client. - Delivery boost requested from 5GMS AF by 5GMS Client. - Bit rate recommendation solicited from RAN modem by 5GMS Client. - Delivery boost requested from RAN modem by 5GMS Client. |
| Requested QoS parameters |  | 0..1 | The network QoS parameters (if any) requested from the 5GMS AF or RAN by the 5GMS Client. |
|  | Maximum requested bit rate | 1..1 | The maximum bit rate requested. |
|  | Minimum desired bit rate | 0..1 | The minimum bit rate desired. |
|  | Minimum requested bit rate | 1..1 | The minimum bit rate requested. |
|  | Desired packet latency | 0..1 | The packet latency requested. |
|  | Desired packet loss rate | 0..1 | The packet loss rate requested. |
| Recommended QoS parameters |  | 0..1 | The network QoS parameters (if any) recommended by the 5GMS AF or RAN to the 5GMS Client. |
|  | Maximum recommended bit rate | 1..1 | The maximum bit rate recommended. |
|  | Minimum recommended bit rate | 1..1 | The minimum bit rate recommended. |

NEXT CHANGE

### 5.5.3	5GMSd AF-based reporting procedure

The second use-case, shown in figure 5.5.3-1 below, illustrates a scenario where the metrics collection and reporting is configured by the 5GMSd AF. In this example, it is assumed that the metrics configuration provided by the 5GMSd AF comprises instructions/rules regarding metrics collection (i.e. measurement and logging) and reporting for two different schemes - one that is non-3GPP-defined and initially sourced from the 5GMSd AS, and the other scheme that is defined by 3GPP and initially sourced from an OAM Server belonging to the 5GMS System operator. Each metrics scheme requires the 5GMSd Client to perform metrics collection and subsequent metrics reporting to the 5GMSd AF according to the configuration rules of that scheme. Furthermore, it is assumed that the 5GMSd AF is required to deliver metrics reports obtained from the 5GMSd Client to separate destination entities, upon optionally having performed post-processing of the original report information, according to the configuration rules of each metrics scheme. The 5GMSd AF and 5GMSd AS functions can be either trusted or untrusted.

Figure 5.5.3-1: Metrics collection and reporting via 5GMSd AF-based configuration

The message sequence steps are described below:

1:	The 5GMSd AF is provisioned with two separate sets of metrics reporting configuration information - metrics scheme_1 as defined by the 5GMSd Application Provider and metrics scheme_2 as defined by 3GPP, each pertaining to metrics collection by the Media Player, retrieval of those collected metrics from the Media Player by the Media Session Handler, and the generation and delivery of metrics reports from the Media Session Handler to the 5GMSd AF. The 5GMSd AF is similarly provisioned with two separate sets of metrics reporting configuration information, associated with the metrics schemes 1 and 2, regarding required post-processing functionality and subsequent and separate delivery of processed metrics reports to the 5GMSd Application Provider and to the OAM Server.

2:	The 5GMSd-Aware Application triggers the Service Announcement and Content Discovery procedure. The Service Announcement includes the whole Service Access Information that contains metric configuration info for metrics schemes 1 and 2, with their associated metrics collection configurations acquired by the Media Player and metrics collection configurations acquired by the Media Session Handler.

3:	Time passes until the 5GMSd UE initiates session establishment and media playback.

4:	The 5GMSd-Aware Application informs the Media Player of impending media playback.

5a:	The Media Player requests the establishment of a streaming session with the Media Session Handler which acknowledges the request.

5b:	The Media Session Handler requests the establishment of a streaming session with the 5GMSd AF which confirms the streaming session establishment.

5c:	The Media Session Handler informs the Media Player the successful set-up of the streaming session.

6:	Media playback pipeline is set up between the Media Player, the 5GMSd AS and the 5GMSd Application Provider.

7:	The Media Session Handler queries the Media Player on its capability to perform metrics collection (measurement and logging function) in accordance with both schemes 1 and 2 as defined by its metrics configuration.

8:	The Media Player acknowledges its support for the collection of the required metrics of all configured schemes.

During the course of media playback, steps 9-18 below may be repeated, depending on the duration of the playback and the frequency of metrics reporting as set by the metrics configuration for each of the two schemes.

9:	Assuming a live streaming service, media content is delivered via push ingest from the 5GMSd Application Provider to the 5GMSd AS.

10:	The Media Player fetches media content from the 5GMSd AS and begins media playback.

11:	The Media Player notifies the Media Session Handler of the start of media playback, causing the Media Session Handler to initialize and begin countdown of separate metrics reporting interval timers for schemes 1 and 2.

12:	Upon expiration of timer_1 (associated with scheme_1), the Media Session Handler retrieves the logged metrics measurements from the Media Player according to scheme_1.

13:	In accordance with its metrics reporting configuration as provisioned in step 2, a metrics report for scheme_1 is sent from the Media Session Handler to the 5GMSd AF.

14:	Upon expiration of timer_2 (associated with scheme_2), the Media Session Handler retrieves the logged metrics measurements from the Media Player according to scheme_2.

15:	In accordance with its metrics reporting configuration as provisioned in step 2, a metrics report for scheme_2 is sent from the Media Session Handler to the 5GMSd AF.

16:	In accordance with its metrics reporting configuration as provisioned in step 1, the 5GMSd AF performs separate post-processing in accordance with schemes 1 and 2 (e.g. filtering, aggregation, reformatting) of the received types of metrics reports.

Furthermore, in accordance with its metrics reporting configuration as provisioned in step 1:

17:	The 5GMSd AF sends a processed metrics report in accordance with scheme_1 to the 5GMSd Application Provider.

18:	The 5GMSd AF sends a processed metrics report in accordance with scheme_2 to the OAM Server.

Upon the termination of media playback (as notified by the 5GMSd-Aware Application to the Media Player) a final round of metrics collection and reporting is performed:

19:	The Media Session Handler obtains the latest metrics measurements from the Media Player in accordance with schemes 1 and 2. These procedures might occur prior to the nominal expiration of the metrics reporting interval timers.

20:	Final metrics reports, in accordance with schemes 1 and 2 are sent by the Media Session Handler to the 5GMSd AF. These procedures might occur prior to the expiration of the nominal metrics reporting intervals.

21:	The 5GMSd AF performs post-processing of the received final metrics reports in accordance with schemes 1 and 2.

22:	Same as step 17.

23:	Same as step 18.

NOTE:	Although not explicitly shown or described in Figure 5.3.3-1, should the 5GMSd AF represent an untrusted network entity and the OAM Server represent a trusted network entity, the delivery of processed metrics reports from the 5GMSd AF to the OAM Server, as depicted in steps 18 and 23, must be mediated by the NEF (Network Exposure Function).

NEXT CHANGE

### 5.9.1	Introduction

The Network Assistance (NA) feature enables a UE that is receiving a downlink media stream to improve the QoE of the media streaming session, by being able to make use of two distinct facilities.

The first facility is bit rate recommendation (throughput estimation). This enables the UE to start a downlink streaming session at the most appropriate bit rate for the network conditions at hand, or to obtain a recommendation from the network which will remain valid until further notice during a media streaming session. The recommended bit rate is based on network estimations or predictions of available link bandwidth. This function is provided as an additional tool to support the UE, in addition to the common approach of the UE performing its own estimation based on measurement of the downlink traffic in the past.

The second facility is the delivery boost. The 5GMSd Client uses this function to indicate to the network that a temporary boost, i.e., a temporary increase of network throughput for this client is needed in order to avoid the risk of media playback stalling due to buffer under-run, which could otherwise occur during the next media segment or soon after. Throughput boosting may also be used at the start of a playback session to shorten the time to start media playback, giving a better experience for the user.

Network Assistance for downlink media streaming may be offered to the UE in one of two ways:

-	Based on interaction between the UE and the 5GMSd AF, with a subsequent interaction between the 5GMSd AF and the PCF (or the NEF);

-	Based on interaction between the UE and the RAN, re-using the ANBR-based RAN signalling.

The UE shall not use both approaches on the same Network Assistance session.

Figure 5.9.1-1 depicts the Network Assistance feature in the context of the 5GMS architecture, showing the scope of both approaches.

Figure 5.9.1-1: Downlink Network Assistance alternative approaches

NEXT CHANGE

### 5.9.2	5GMSd AF-based downlink Network Assistance

The Network Assistance (NA) feature enables a UE to receive a bit rate recommendation from the 5GMSd AF that provides the NA server function. The 5GMSd AF provides the response with an estimation of throughput, or the recommendation of a bit rate which will remain valid until further notice. The UE uses this estimation to derive the most suitable bit rate for its downlink content session from the versions that might be available. The network is expected to commit resources to be able to fulfil the recommendation, although no guarantee for the bandwidth estimation can be assumed. The 5GMSd Client may provide additional information, such as available media versions, in terms of the required bit rates, to the 5GMSd AF.

The second facility is the delivery boost. The 5GMSd Client uses this function to indicate to the network that a temporary boost, i.e. a temporary increase of network throughput for this client, is needed.

Each interaction for the 5GMSd AF-based downlink Network Assistance procedures consists of two steps in sequence:

1.	Between the UE (Media Session Handler) and the 5GMSd AF using a 5GMS API at interface M5d;

2.	Between the 5GMSd AF and the PCF (or the NEF):

-	using the Npcf_PolicyAuthorization procedure or Nnef_AFSessionWithQoS procedure to request modification of the PDU session with the requested QoS information.

-	using the Npcf_PolicyAuthorization notification service or Nnef Monitoring Event procedure to receive QoS changes. The 5GMSd AF receives these policy change notifications asynchronously.

Network Assistance is performed within dedicated NA sessions that are a part of the Media Session Handler procedures. Only 5GMSd Clients that have been granted an NA session by the 5GMSd AF may execute the NA bit rate recommendation and boost request procedures. This enables the 5GMSd AF to enforce policies that could include the access to NA by certain 5GMSd Clients only, such that authorisation to use NA facilities can be verified once at the granting of an NA session, and does not need to be verified for each NA request from the 5GMSd Client.

The procedures for 5GMSd AF-based downlink Network Assistance are:

1.	NA session initiation. The 5GMSd Client requests to initiate an NA session with the 5GMSd AF. If the request fulfils any pre-requisites for access to NA functionality, for example policy and charging, then the 5GMSd AF responds with a confirmation that the NA session has been established.

2.	NA throughput estimation. The 5GMSd Client requests a throughput estimation for a downlink media session from the 5GMSd AF. A unique identifier for the downlink media session is provided by the 5GMSd Client.

If the set of available bit rates in the downlink media session is provided with the request then the 5GMSd AF responds with the recommended bit rate based on its throughput estimation. If the throughput estimation is lower than the lowest value from the set of available bit rates, then the actual throughput estimation is provided.

If no set of available bit rates is provided with the request then the 5GMSd AF responds with the throughput estimation.

3.	NA delivery boost. The 5GMSd Client requests from the AF a downlink delivery boost. A unique identifier for the downlink media session is provided by the 5GMSd Client.

4.	NA session termination. The 5GMSd Client requests to terminate an NA session with the 5GMSd AF. The unique identifier of the NA session to be terminated is provided by the 5GMSd Client.

The AF responds positively if the indicated session could be terminated, and negatively otherwise.

NEXT CHANGE

## 6.1	General

The procedures for uplink media streaming allow a 5GMSu Application Provider to create, modify, establish and delete sessions. Uplink media streaming sessions exist between a 5GMSu Client and a 5GMSu AS.

The uplink streaming procedures follow a general high-level workflow, starting from provisioning to the actual uplink streaming sessions. The Egest Session refers to the time period during which media content is uplink streamed to the 5GMSu AS and optionally egested from there to the 5GMS Application Provider. The Provisioning Session refers to the time period during which the 5GMSu Client is permitted to upload stream media content. Interactions between the 5GMSu AF and the 5GMSu Application Provider may occur at any time while the Provisioning Session is active.

The 5GMSu Provisioning API allows the selection of Media Session Handling (M5u) and Uplink Streaming (M4u) options, including whether the media content is published to trusted 5GMSu ASs. The 5GMSu AF selects the M5u interface according to the provisioning option. The Media Session Handling interface exposed by the 5GMSu AF can be used for metrics reporting, requesting different policy and charging treatments, or 5GMSu AF-based Network Assistance.

When the 5GMSu AF and 5GMSu AS reside in the same DN, then the 5GMSu AF selects the 5GMSu AS. Interactions between a 5GMSu AF and a 5GMSu AS (M3u interactions) take place for Content Egest (M2u) and Uplink Streaming (M4u) resource reservations. The 5GMSu AS allocates M2u and M4u resources and communicates resource identifiers back to the 5GMSu AF. The 5GMSu AF provides information about the provisioned resources (in the form of resource identifiers) for Media Session Handling, Content Egest, and Uplink Streaming to the 5GMSu Application Provider. The resource identifiers for Media Session Handling and Uplink Streaming are needed by the 5GMSu Client to access the selected features.

5GMSu Client can (in principle) start the uplink streaming by activating its uplink streaming session. The uplink streaming session for a given UE (or for each UE) is active from the time at which the 5GMSu-Aware Application activates the transmission of an uplink streaming service until its termination.

The 5GMSu-Aware Application receives application metadata from the 5GMSu Application Provider before transmitting the uplink streaming media. The application metadata contains Service Access Information, which acts as an entry point for the 5GMSu Client to start the uplink streaming session. The 5GMSu Client may either receive the Service Access Information from the 5GMSu Application Provider (using a not standardized interface) or instructions for a remote control session through the 5GMS-Aware Application.

The Network Assistance (NA) feature enables a UE that is receiving an uplink media stream to improve the QoE of the media streaming session, by being able to make use of two distinct facilities.

The first facility is bit rate recommendation (throughput estimation). This enables the UE to start an uplink streaming session at the most appropriate bit rate for the network conditions at hand, or to obtain a recommendation from the network which will remain valid until further notice during a media streaming session. The recommended bit rate is based on network estimations or predictions of available link bandwidth. This function is provided as an additional tool to support the UE, in addition to the common approach of the UE performing its own estimation based on measurement of the uplink traffic in the past.

The second facility is the delivery boost. The 5GMSu Client uses this function to indicate to the network that a temporary boost, i.e., a temporary increase of network throughput for this client is needed, for example in order to prevent the uplink media streaming buffer in the Media Streamer from overflowing.

Network Assistance may be offered to the UE in one of two ways:

-	Based on interaction between the UE and the 5GMSu AF, with a subsequent interaction between the 5GMSu AF and the PCF (or the NEF), as defined in clause 6.5.

-	Based on interaction between the UE and the RAN, re-using the ANBR-based RAN signalling as defined in clause 6.7.

The UE shall not use both approaches on the same Network Assistance session.

Figure 6.1-1 provides a basic high-level call flow for uplink streaming. The corresponding collaboration scenario is defined in clause A.10.

Figure 6.1-1: High-level call flow for uplink media streaming

Steps:

1.	The 5GMSu Application Provider provisions the 5GMSu AF at reference point M1u, including a Content Publishing Configuration for content egest and, optionally, Content Preparation Templates.

2.	When Content Publishing is offered and selected, there may be interactions between the 5GMSu AF and the 5GMSu AS at reference point M3u, e.g. to configure Server Certificates and/or Content Preparation Templates and to confirm the availability of resources for Content Preparation and Content Egest by providing a Content Publishing Configuration (defined in clause 6.2.3). The 5GMSu AS provides resource identifiers for the allocated resources to the 5GMSu AF, which then provides the information to the 5GMSu Application Provider.

At some later point in time:

3.	The 5GMSu Application Provider provides Service Access Information to the 5GMS-Aware Application at reference point M8u.

4.	When the 5GMSu-Aware Application decides to activate an uplink media streaming session, the Service Access Information is provided to the 5GMSu Client.

5.	The 5GMSu Client requests the 5GMSu AF to initialise uplink media streaming (M5u).

Alternatively:

6.	The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

7.	The 5GMSu Client requests Service Access Information from the 5GSMu AF at reference point M5u.

Then:

8.	The 5GMSu Client starts the Egest Session by activating the uplink streaming session.

9.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu AS via reference point M4u.

10.	If content preparation was provisioned in step 1, the uplinked media may be manipulated by the 5GMSu AS prior to egest.

11.	Media streaming egest starts from the 5GMSu AS to the 5GMSu Application Provider at reference point M2u.

Clauses A.11 to A.15 define additional collaboration scenarios for uplink streaming. The call flow for each collaboration scenario is also included in each clause.

last CHANGE

## 6.5	Providing 5GMSu AF-based Network Assistance

The procedure a 5GMSu Client uses to obtain network assistance from a 5GMSu AF is defined in figure 6.5-1 below.

Figure 6.5-1: Providing 5GMSu AF-based Network Assistance

Steps:

1:	An uplink streaming session is active. A Network Assistance Session is established with the 5GMSu AF.

2:	The Media Session Handler in the 5GMSu Client requests assistance information from the 5GMSu AF.

2a:	The 5GMSu AF interacts with the PCF (or, if the AF is deployed outside the Trusted DN, with the PCF via the NEF) to obtain the requested assistance information.

3:	The assistance information is returned to the Media Session Handler in the 5GMSu Client.

4:	The Media Streamer in the 5GMSu Client takes an appropriate action based on the information received.

END OF CHANGES