| 3GPP TR 26.812 V0.7.1 (2023-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on QoE Metrics for AR/MR Services (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Current state of AR/MR QoE metrics outside 3GPP	9

4.1	AR/MR QoE related work in ITU-T	9

4.2	AR/MR QoE related work in IEEE	12

4.3	AR/MR QoE related work in MPEG	12

5	Relevant AR/MR metrics	13

5.1	Head-motion aware viewport quality metric (HMAVQ)	13

5.1.1	Background	13

5.1.2	Metric description	13

6	Identification of AR/MR QoE Metrics for 3GPP	14

6.1	General	14

6.2	AR/MR QoE reference model	15

6.2.1	Observation Point 1	15

6.2.1.1	Viewer pose and Projection parameters	16

6.2.1.2	Camera information	16

6.2.1.3	Gesture	17

6.2.1.4	Body action	17

6.2.1.5	Tracking pose prediction parameters	17

6.2.1.6	Pose prediction parameters	18

6.2.1.7	Eye gaze pose prediction parameters	18

6.2.1.8	Parameters monitored by OP1	20

6.2.2	Observation Point 2	21

6.2.3	Observation Point 3	21

6.2.4	Observation Point 4	21

6.3	Introduction of AR/MR QoE metrics	21

6.3.1	Registration latency	22

6.3.2	Scene startup latency and Interaction latency	22

6.3.3	Tracking pose prediction error	23

6.3.4	One-way delay and RTT	23

6.3.4.1	Background	23

6.3.4.2	Metric description	24

6.3.5	Pose error and time error	24

6.3.5.1	Background	24

6.3.5.2	Metric description	24

6.3.5.3	Measurement procedure	25

6.3.6	Device related QoE metrics	27

6.3.6.1	Background	27

6.3.6.2	Metric description	29

6.3.7	Spatial Anchors and Trackables	29

6.3.7.1	Background	29

6.3.7.2	Impact on latencies	30

6.3.7.3	Retrieval of the AR anchoring information	30

6.3.7.4	Anchor Creation Delay (ACD)	31

6.3.7.4.1	Measurement of the local Anchor Creation Delay	31

6.3.7.4.2	Measurement of the remote Anchor Creation Delay	31

6.3.7.5	Anchor Detection to Render to Photon QoE	32

6.3.7.5.1	Measurement of the Anchor Detection to Render to Photon (ADRP)	32

6.3.7.6	Anchor Untracked Ratio QoE	34

6.3.7.6.1	Measurement of the Anchor Untracked Ratio (AUR)	34

7	Other QoE metrics and collaborations with other 3GPP groups	35

7.1	General NWDAF based metrics configuration and collection	35

7.2	RRC based metrics configuration and collection	35

7.3	Summary	36

8	Conclusions and Recommendations	36

Annex <A> (informative): <Normative annex for a Technical Specification>	36

Annex <X> (informative): Change history	37

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

In Rel-18, simple QoE Metrics for AR media will be specified in MeCar WI, which focus on which VR QoE metrics can be reused and enhanced for AR media. This study intends to complement MeCar WI by studying additional and new information related to QoE metrics which are different from VR QoE metrics. QoE metrics typically reflect the expected user experience based on measurable parameters. XR (e.g. AR/MR) user experience is impacted by different aspects, such as the media quality, the rendering capabilities of the device, the tracking capabilities of the devices, etc. In order to identify the impact of different QoE factors, as well as in order to identify how QoE related parameters can be measured in devices, additional detailed study is necessary.

Based on this, the present document collects available information in other organizations such as the ITU-T, MPEG-I groups, and provides recommendation on normative work for new XR QoE metrics in such that XR (e.g. AR/MR) user experience can be enhanced with the complete defined QoE metrics.

1	Scope

This document addresses information collection and QoE metrics definition aspects when the XR (e.g. AR/MR) devices are based on the architecture defined in MeCAR WI. The following details are in scope:

-	Collect relevant external information on QoE Metrics for AR and XR services, for example taking into account information in ITU-T, MPEG or other groups dealing with quality measurements, include device related QoE metrics, network transmission related QoE metrics, content handling related QoE metrics, and other immersiveness/presence related QoE metrics.

-	Documentation of subjective tests results on XR QoE metrics, if considered relevant.

-	Identification of relevant XR QoE Metrics and their impacts on the user experience.

-	Identification of relevant observation points and define the measurement and derivation of relevant XR QoE metrics in the device architecture based on MeCAR.

-	Collaboration with relevant groups or specifications on NWDAF, RRC-based metrics configuration and collection.

-	Provide recommendation on normative work for new XR QoE metrics based on the findings in this study.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[3]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[4]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[5]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[6]	ITU-T G.1036, “Quality of experience (QoE) influencing factors for augmented reality (AR) services”.

[7]	ITU-T P.1320, “QoE assessment of extended reality (XR) meetings”.

[8]	Technical Report ITU-T GSTR-5GQoE, QoE requirements for real-time multimedia services over 5G networks.

[9]	ITU-T H.430.1 Recommendation ITU-T H.430.3 (2018), Service scenario of immersive live experience (ILE).

[10]	ITU-T RGM-Q8-DOC10-R2 (2022-06)	H.IIS-reqts: “Requirements of Interactive Immersive Services”

[11]	IEEE P3333.1.1/D2, May 2022 - IEEE Draft Standard for Quality of Experience (QoE) and Visual-Comfort Assessments of Three-Dimensional (3D) Contents Based on Psychophysical Studies

[12]	IEEE P3333.1.2/D3, Apr 2022 - IEEE Draft Standard for the Perceptual Quality Assessment of Three Dimensional (3D), Ultra High Definition (UHD) and High Dynamic Range (HDR) Contents

[13]	IEEE Std 3333.1.3-2022 “IEEE Standard for the Deep Learning-Based Assessment of Visual Experience Based on Human Factors”

[14]	ISO/IEC 23090-6:2021 Information technology — Coded representation of immersive media — Part 6: Immersive media metrics

[15]	ISO/IEC JTC 1/SC 29/WG 03 N0777	Technologies under Consideration for ISO/IEC 23090-6	WG 03, MPEG Systems

[16]	ISO/IEC JTC 1/SC 29/WG 03 N00710. “Text of ISO/IEC 23090-6 FDAM 1 Immersive Media Metrics for V3C data and OMAF”.

[17]	ISO/IEC JTC 1/SC 29/WG 03 N00826. “Preliminary WD of ISO/IEC 23090-6 AMD 2 Additional latencies and Other Improvements”.

[18]	Kara, B., Akcay, M. N., Begen, A. C., Ahsan, S., Curcio, I. D., & Aksu, E. B. (2022). Could Head Motions Affect Quality When Viewing 360-Degree Videos?. IEEE MultiMedia.

[19]	Van der Hooft, J., Vega, M. T., Petrangeli, S., Wauters, T., & De Turck, F. (2019). Quality assessment for adaptive virtual reality video streaming: A probabilistic approach on the user’s gaze. In 2019 22nd conference on Innovation in Clouds, Internet and Networks (ICIN) (pp. 19-24). IEEE.

[20]	Hu, Z., Bulling, A., Li, S., & Wang, G. (2021). Ehtask: Recognizing user tasks from eye and head movements in immersive virtual reality. IEEE Transactions on Visualization and Computer Graphics.

[21]	Rai, Y., Le Callet, P., & Guillotel, P. (2017, May). Which saliency weighting for omni directional image quality assessment?. In 2017 Ninth International Conference on Quality of Multimedia Experience (QoMEX) (pp. 1-6). IEEE.

[22]	The OpenXR Specification, Copyright (c) 2017-2023, The Khronos Group Inc., Version 1.0.27: from git ref release-1.0.27

[23]	ETSI GS ARF 004-2 V1.1.1(2021-08) - Augmented Reality Framework (ARF) Interoperability Requirements for AR components, systems and services Part 2: World Storage and AR Authoring functions

[24]	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[25]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[26]	3GPP TS 25.331 "Radio Resource Control (RRC); Protocol specification".

[27]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".3	Definitions, abbreviations and conventions.

[29]	3GPP TS 38.300: "NR Overall Description; Stage 2".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

MR 	Mixed Reality

4	Current state of AR/MR QoE metrics outside 3GPP

4.1	AR/MR QoE related work in ITU-T

In ITU-T, there are many study groups focusing on the quality of experience (QoE) for XR services, e.g. Study Group 12 (SG12), Study Group 16 (SG16). Typically, Study Group 12 (SG12) majoring in the P​erformance, QoS & QoE​ is quite relevant to the study of AR/MR QoE in SA4.

In SG12, there are several ongoing or completed work about the AR/MR QoE.

1) ITU-T G.1036 “Quality of experience (QoE) influencing factors for augmented reality (AR) services” [6].

This Recommendation has been consented by the recent ITU-T Study Group 12 meeting, June 2022. It lists typical use cases of augmented reality services and identifies the key QoE factors in it, and also gives a suggested scheme for AR QoE assessment in future works.

Different from Virtual Reality, which aims to completely replace the user’s real world environment with a simulated one, Augmented Reality keeps the real world and puts additional information to the natural environment.

Due to addition of new ways to locate the self-position of users and new display mode of perceptual information, a set of new requirements to QoE assessment to characterize AR’s immersive video, spatial audio, and interactivity are emerging. Besides, it’s important to address the requirements and basic factors affecting the VR QoE before benchmarking work is executed. This document identifies the key factors affecting user-perceived experience.

The identified AR QoE metrics can be shown as following:

Figure 4.1-1: AR QoE assessment scheme [6]

Table 4.1-1: Identified QoE factors for AR services [6]

| Human Factors | Vision | myopia, hyperopia, astigmatism, and amblyopia. |
| --- | --- | --- |
|  | Hearing | the audio perceptibility |
|  | Touch/Force Sensation | tactile/force and other types of perceptual enhancement. |
| System Influence Factors | Content Related | Real Environment, virtual content, Superimposition of Real Environment and Virtual Content |
|  | Media and Coding Related | Codec, Bitrate, Resolutionl, Framerate, coding delay, streaming quality, stordage and transport. |
|  | Network and Transmission related | Bandwidth, latency, packet loss, jitter. |
|  | Recognition related | Marker attributes, Recognition response. |
|  | Consistency | Geometry consistency, Lighting consistency, Time consistency |
|  | Harware Related | Comfortableness, FoV, stereo, depth range |
|  | Interaction Related | Hand Gesture, Speech, Body Posture, Tangible Interface, Eye/head based interfaces, Brain-computer interfaces |
| Content Influence Factors | Physical context | physical environment condition |
|  | Colloboration context | collaborative environment for multi-person AR services. |
|  | Task context | tasks and goals users are aiming to in their minds. |
|  | Temporal context | the frequency and duration is to use AR services. |
| General factors | Human factors | Olfactory, Multisensory integration, Simulator sickness, Static and Dynamic Human Factor |
|  | Hardware related factors | Colour Space, Dynamic Range, Refresh Rate |

2) ITU-T P.1320 “QoE assessment of extended reality (XR) meetings” [7].

This Recommendation has been consent by the recent ITU-T Study Group 12 meeting, June 2022. It advises on aspects of importance for QoE assessment of telemeetings with extended reality elements. The goal is to define the human, context, and system factors that affect the choice of the QoE assessment procedure and metrics when extended reality telemeeting systems are under evaluation.

This Recommendation focuses on aspects of importance for the assessment of Quality of Experience (QoE) of different types of eXtended Reality (XR) telemeetings, which may comprise a combination of telemeetings taking place in virtual reality (VR), augmented reality (AR), or mixed reality (MR) environments. It targets XR services aiming to immerse the user and augment the exchange of information by delivering interactive real-time uni- or multimodal sensory information for two-party and multiparty communication. The services include telemeetings taking place in a virtual location or a combination of virtual and real locations; telemeetings with mixtures of real and virtual participants; telemeetings with augmented elements for collaboration; virtual conferences; and joint teleoperation of equipment.

While multiple QoE evaluation methodologies for telemeetings have been developed, novel XR telemeeting services may result in cognitive effects that are not covered by the existing Recommendations. These effects may include simulator sickness, fatigue, immersion, or presence, for example. This document advises on the key QoE factors affecting the user experience of an XR telemeeting service.

The identified QoE metrics for XR meetings can be found as the following.

Table 4.1-2: Factors influencing QoE of XR telemeetings [7]

| Human Influence Factors | Perceptual and congnitive characteristics | Visual, audio, olfactory, tactile acuity. |
| --- | --- | --- |
|  | Internal state of individual participants | Simulator sickness, immersion, level of expertise, spatial intelligence and introversion, etc. |
|  | Conversation behaviour | Conversation behaviour, degrees of involvement. |
|  | Relations between participants | A closer (e.g., romantic) relationship may be preferred for this kind of tactile stimulation. |
|  | Language and body language aspects | linguistic social cues and body language. |
| Content Influence Factors | Communication environment | Real Environment, virtual content, Superimposition of Real Environment and Virtual Content |
|  | Communication scenarios | Codec, Bitrate, Resolution, Framerate, coding delay, streaming quality, storage and transport. |
|  | Time aspects | Bandwidth, latency, packet loss, jitter. |
| System Influence Factors | Human/world-related factors | Degree of freedom, representation of users, realism/style, locamotion, position, proxemics |
|  | Rendering | Rendering per client, multi-party effects on rendering, redering errors, resolutions, foveated rendering, overlaying rendering images. |
|  | Network and compression | Media encoding/decoding, latency, bandwidth, synchronization |
|  | Temporal context | the frequency and duration is to use AR services. |

3) Technical Report ITU-T GSTR-5GQoE, QoE requirements for real-time multimedia services over 5G networks [8]

This report has been produced by the 5G-KPI working group of the Video Quality Experts Group (VQEG) and defines a scope for the analysis of QoE in 5G services and several use cases where this scope is applicable.

This Technical Report defines a scope for the analysis of QoE in 5G services and several use cases where this scope is applicable. Such use cases are: Tele-operated Driving, Wireless Content Production, Mixed Reality Offloading, and First Responder Networks. For all the targeted use cases, the work item aims to study the specific QoE requirements, as well as the required performance and features from the network. By addressing them in parallel, it is possible to find synergies between them and, more relevantly, extract the common information that can be used to also analyse other use cases that may arise outside the scope of this work item.

Specially, the MR Offloading case assumes that the immersive environment where the application takes place is either the real world around the user (AR) or a completely virtual environment (VR). 5G-network capabilities (i.e. Edge Computing) are used to enable running state-of-the-art MR algorithms while wearing a lightweight HMD.

The identified QoE factors for MR Offloading can be found in the following:

-	Motion-to-photon latency

-	Responsiveness in human-virtual interaction

-	Embodiment feeling

-	RTT time

-	Peak uplink/downlink throughputs

-	Coding/processing delay

-	Mean network throughput

-	Type of MR application

-	Temporal context

Study Group 16 (SG16) majoring in the “immersive live experience (ILE) ” and “interactive immersive services (IIS)” shares some views on the requirements of immersive services, e.g. extended reality service. The related details are listed as below:

1) ITU-T H.430.1 “Requirements for immersive live experience (ILE) services” [9] provide some requirements of ILE as follows:

General requirements: Displaying real-sized objects, Direction of sounds, Reconstruction of stage effects, Spatial environment, Synchronous media representation of multiple assets, Augmented information attachment ability.

High-level requirements: Real-time object extraction, Synchronous transmission of multiple media streams, Media processing, High-realistic auditory lateralization, Video stitching.

2) ITU-T H.430.6 H.IIS-reqts: “Requirements of Interactive Immersive Services (IIS)” work item is under study [10]. The baseline text provides some service and capability requirements of IIS service as follows:

Service and capability requirements: Support for interactive capabilities, Support for synchronous transmission of concurrent streams, Intelligent distribution of massive multimedia data, Media processing, Network status awareness and QoS scheduling, Multimedia stream embellishing.

Both the ILE and IIS work item in ITU-T SG 16 are mainly focusing on the requirements of immersive service without any information related to how to map the requirements to the AR/MR QoE metrics or factors.

4.2	AR/MR QoE related work in IEEE

In IEEE, P3333.1 series standards focus on the experience assessment of 3D contents, and visual experience assessment based on human factors. The details are listed as below.

1) IEEE P3333.1.1/D2, May 2022 - IEEE Draft Standard for Quality of Experience (QoE) and Visual-Comfort Assessments of Three-Dimensional (3D) Contents Based on Psychophysical Studies [11] captures the subjective assessment for quantifying the visual discomfort and quality of experience (QoE) of 3D image and video.

2) IEEE P3333.1.2/D3, Apr 2022 - IEEE Draft Standard for the Perceptual Quality Assessment of Three Dimensional (3D), Ultra High Definition (UHD) and High Dynamic Range (HDR) Contents [12] defines quality metrics for 3D, UHD and HDR quality assessment.

3) IEEE Std 3333.1.3-2022 “IEEE Standard for the Deep Learning-Based Assessment of Visual Experience Based on Human Factors” [13] defines deep learning-based metrics of content analysis and quality of experience (QoE) assessment for visual contents. The standards mainly cover quality assessment of visual contents and cybersickness assessment of visual contents for VR.

The above IEEE standards firstly provide the methods of video quality assessment (e.g. 3D contents quality), and then cover deep learning models considering human factors for various QoE assessments from an objective point of view. However, those information are not related to AR/MR QoE metrics definition, and even the IEEE Std 3333.1.3 only covers QoE assessment for VR cybersickness.

4.3	AR/MR QoE related work in MPEG

MPEG has defined a series of standards for immersive media with a project of MPEG-I (ISO/IEC 23090 Coded Representation of Immersive Media). It contains 26 parts related with immersive media components. In the part 6, the immersive media metrics and the measurement framework are specified in ISO/IEC 23090-6:2021 [14]. This standard also includes a VR client reference model with observation and measurement points for collection of the metrics. The immersive media metrics in [14] are listed as below:

-	Rendered FOV set metric

-	Display information set metric

-	Rendered viewports metric

-	Comparable quality viewport switching latency metric

In addition, Viewpoint Mismatch Duration Metric is also agreed to be added into the specification ISO/IEC 23090-6 [15].

Furthermore, the following metrics are defined in ISO/IEC 23090-6 AMD1 [16] and proposed in ISO/IEC 23090-6 AMD2 [17]:

-	Omnidirectional Viewpoint Switching Latency metric [16]

-	V3C Viewpoint Switching Latency [16]

-	Viewpoint Switching Latency [17]

All the 8 immersive media metrics are defined for the VR service based on VR client model, not relevant to AR/MR QoE metrics.

5	Relevant AR/MR metrics

5.1	Head-motion aware viewport quality metric (HMAVQ)

5.1.1	Background

Perceptual quality is defined as a user’s degree of satisfaction while viewing a video. Various objective quality metrics (e.g. average viewport quality, motion-to-high-quality delay) were proposed to estimate the quality of a 360-degree video. TS 26.118 specified a metric where the viewport quality is calculated by multiplying the quality ranking of each tile in the viewport by the percentage of the viewport it covers.

According to some studies [19, 20], the methods that best estimate the actual user experience are the ones that take into account the human visual system (HVS). While these studies considered user’s eye gaze, they largely ignored the effects of the head motion velocity, direction and duration. For instance, the content rendered in the viewport may be unfocused, or even blurry, to a human eye if the head is turning too fast (that is, the user can’t focus on the content during a fast head motion). In this case, this individual viewport’s quality may have only a slight, if any, effect on the overall user experience since the low-quality video is shown to the user only for a brief amount of time.

Also, the head motion may affect the region-of-interest (ROI) the user is paying attention to inside the viewport. Since the head and eye gaze mainly move in the same direction [20], the ROI shifts from the viewport center to the edges of the viewport, in the direction of the head motion. Consequently, the quality of the tiles in the opposite direction are expected to matter less and have a smaller effect on the overall user experience. Studies also show that users tend to view the content with little focus during a fast head motion and the actual quality of the viewport during this motion is not as relevant [20, 21]. This implies that not every viewport a user has viewed has an equal weight in determining the user’s overall experience.

A QoE metric that takes into account the above considerations can facilitate more advanced algorithms for viewport-dependent streaming of XR experiences by evaluating these algorithms more accurately. In the next section, a metric that brings “head motion awareness” in quality assessment is described. Inclusion of head motion exemplified in this metric can be useful for development of more effective AR/MR metrics.

5.1.2	Metric description

Computation of the head-motion aware viewport quality metric (HMVAQ) comprises two steps. In the first step, the quality of an individual viewport is calculated during head motion. In the second step, the overall viewport quality over a period of time is calculated from the individual viewport qualities. Details of these two steps are described below.

Individual viewport qualities

Quality of the individual viewports is calculated using sampled points that are shifted and weighted based on head motion speed and direction. Each viewport is sampled with n circles, where each circle has m sample points. The circles are indexed from 1 to n, and each circle’s diameter grows from the viewport center towards the viewport edges. Depending on head movements, the circles are shifted by xixi degrees horizontally and  yiyi degrees vertically, where these shift amounts depend on the radius  riri of each circle, head motion velocities in horizontal and vertical axes vx, vyvx, vy and a speed threshold vthreshvthresh beyond which the viewport starts becoming unfocused (blurry). The shift amounts are calculated as:

xi=w2-ri*vxvthresh ,    yi=h2-ri*vyvthresh

where w and h are the width and height of the viewport, respectively. The circle weights are assigned linearly in descending order from viewport center towards the edges. The weight of circle i (ωi) is calculated by dividing its reversed order by the sum of the circle indexes:

ωi=n-i+1i=1ni

Subsequently, the points on each circle are distributed with an angular distance of 2π/m2π/m. The coordinates of each point are calculated using the parametric equation of the circle. These coordinates are then used to find the tile T with the projection of the point. The quality of the tile T, on which a point is projected, is recorded as the quality of that point. The same operation is applied for all points, and the average across all points determines the quality of the circle i. The circle quality is then multiplied by the weight of the circle ωiωi, and the weighted sum of all weight circle qualities yields the individual viewport quality.

Example sampled viewports at different head motion velocities are shown in Figure 5.1.2-1, where vertical and horizontal lines represent the tile boundaries. Fig. 5.1.2-1 (a) shows the case here the viewport is stationary (no head movement). Figures 5.1.2-1 (b) and 5.1.2-1 (c) show the shifted circles for slow and fast head movements, respectively. Fig. 5.1.2-1 (d) shows the case where the head movement is faster than the speed threshold (vthreshvthresh) in the horizontal axis.

Figure 5.1.2-1: Sampled viewports at different head motion speeds

Overall viewport quality

Overall viewport quality is calculated as the weighted average of the individual viewport qualities over a period of time. The weights are assigned based on head motion velocity. The idea is to assign smaller weights to the viewports viewed at faster head motions, whereas those viewed in stationary head positions or at slower head motions are assigned a larger weight. Weight assignment is not a trivial task and the best one can do is to find a good method empirically. After tests, a thresholding approach is adopted such that a weight equal to 1 is used for viewports where vthresh is reached or exceeded, and 2 for the other viewports. More sophisticated weight assignment functions might perform better, and this deserves further testing and validation.

For further details on the metric, please refer to [18].

6	Identification of AR/MR QoE Metrics for 3GPP

6.1	General

There are 22 core use cases identified for AR/MR devices in TR 26.998 [2] and they are further clarified into the several categories based on the similar requirements for media flow and device functional structure:

-	Immersive media downlink streaming

-	5G interactive immersive service

-	5G cognitive immersive service

-	AR conversational service

-	Shared AR conversational service

The AR/MR QoE metrics can be studied based on the following aspects:

1)	Content part: study needs to be conducted on factors of content part which would help analyse user experience.

2)	Delivery part: changing network conditions may lead to problems in user experience, especially the impact of transmission latency on user experience.

3)	Device part: device capabilities also have impact on user experience.

QoE metrics relevant with the above aspects need to be studied under this study item, and based on the result of this study, user experience of AR/MR service could be evaluated.

6.2	AR/MR QoE reference model

According to the clause 4.1.2 of MeCAR PD, a defined AR/MR QoE framework and the observation points can be reused as baseline for the AR QoE reference model, which is illustrated in Figure 6.2-1.

NOTE:	The observation points can also be used to identify the advanced AR/MR QoE metrics.

Figure 6.2-1: AR/MR QoE reference model and Metrics Observation Points

It’s also noted that the above observation points may be further updated based on the agreements of the AR/MR QoE metrics identification and definition.

6.2.1	Observation Point 1

XR Runtime is a set of functions that interface with a platform to perform commonly required operations, such as accessing the controller/peripheral state, getting current and/or predicted tracking positions, performing spatial computing, and submitting rendered frames to the display processing unit. The XR Runtime provides the viewer pose and projection parameters needed to render each view for use in a composition projection layer.

XR Source Management addresses the management of data sources provided through the XR runtime such as microphones, cameras, trackers, etc. The XR Source Management may expose information to the application or may provide a subset to the media access function to be sent remote.

Presentation Engine is a set of composite renderers, rendering the component of the scenes, based on the input from the Scene Manager. The Scene Manager together with the Presentation Engine that includes functions such as scene composition and possible complex audio or visual rendering.

Observation point 1 is derived from the XR Runtime API which exchanges information between XR Runtime and XR Source Management/Presentation Engine. In addition, MeCar PD clarify that the OP1 (can also called IF1) is implemented as an API-1 that exposes functions provided by the XR Runtime. An example of this API is the Khronos OpenXR API. So the key is to define parameters that may be exposed (or monitored) through the runtime.

6.2.1.1	Viewer pose and Projection parameters

Viewer pose is to present the user position and orientation, which can be defined as quaternion (X, Y, Z, W) for orientation and three vectors (X, Y, Z in cartesian coordinate system) for position. Projection parameters are parameters associated to the perspective/orthogonal/omnidirectional projection to the 3D scene.

It’s noted that OpenXR is the interface between an application and an in-process or out-of-process "XR runtime system", or just "runtime" hereafter [22]. In OpenXR [22], an XR application uses xrLocateViews to retrieve the viewer pose and projection parameters needed to render each view for use in a composition projection layer. xrLocateViews returns an array of XrView elements and the XrView data structure is defined as below [22]:

typedef struct XrView {

XrStructureType	type;

void*				next;

XrPosef			pose;

XrFovf				fov;

} XrView;

In XrView structure, it’s defined that pose is an XrPosef indicating the location and orientation of the view in the space specified by the xrLocateViews function, fov is the XrFovf for the four sides of the projection. And it also clarifies the XrView structure contains view pose and projection state necessary to render a single projection view in the view configuration.

Viewer pose and projection parameters may be monitored or observed via the OP1.

6.2.1.2	Camera information

Camera information including the attribute of the camera and everything external to the camera, such as resolution, FOV, relative pose, attached to,etc [22].

The section 12.117 of XR_OCULUS_external_camera in OpenXR [22] clarifies this extension enables the querying of external camera information for a session. This extension is intended to enable mixed reality capture support for applications. For details,

XR_OCULUS_external_camera API supports returning camera intrinsics and extrinsics.

The intrinsic parameters are the attributes of the camera and include [22]:

-	fov is the XrFovf for this camera’s viewport.

-	virtualNearPlaneDistance is the near plane distance of the virtual camera used to match the external camera

-	virtualFarPlaneDistance is the far plane distance of the virtual camera used to match the external camera

-	imageSensorPixelResolution is the XrExtent2Di specifying the camera’s resolution (in pixels).

The extrinsic parameters are everything external to the camera: relative pose, attached to, etc.

Camera information parameter, including the camera intrinsic and extrinsic, may be monitored or observed via the OP1.

6.2.1.3	Gesture

Gesture can trigger specific actions during an AR experience, it can be provided as a list of hand joint poses which represent the current configuration of the tracked hands.

Clause 12.30, XR_EXT_hand_tracking in OpenXR [22] enables applications to locate the individual joints of hand tracking inputs. It enables applications to render hands in XR experiences and interact with virtual objects using hand joints.

The section 12.57, XR_FB_hand_tracking_aim in OpenXR [22], clarifies that the XR_EXT_hand_tracking extension provides a list of hand joint poses which represent the current configuration of the tracked hands. This extension adds a layer of gesture recognition that is used by the system. That means an application is allowed to get a set of basic gesture states for the hand when using the XR_EXT_hand_tracking extension. Hand gesture parameter may be monitored or observed via the OP1.

6.2.1.4	Body action

Body action parameters includes body joints and joint locations. The section 12.44 of XR_FB_body_tracking in OpenXR [22] clarifies that this extension enables applications to locate the individual body joints that represent the estimated position of the user of the device. It enables applications to render the upper body in XR experiences. When create a body tracker handle, this handle can be used to locate body joints using xrLocateBodyJointsFB function, and a body tracker provides joint locations with an unobstructed range of human body motion.

Body action parameters may be monitored or observed via the OP1.

6.2.1.5	Tracking pose prediction parameters

Tracking pose prediction parameters includes space location information.

Section 7.4 of Locating Spaces in OpenXR [22] clarifies that applications use the xrLocateSpace function to find the pose of an XrSpace’s origin within a base XrSpace at a given historical or predicted time.

The structure of xrLocateSpace is describe as below [22]:

XrResult xrLocateSpace(

XrSpace                                     space,

XrSpace                                     baseSpace,

XrTime                                      time,

XrSpaceLocation*                    location);

The detailed parameters description are listed as below:

-	space identifies the target space to locate.

-	baseSpace identifies the underlying space in which to locate space.

-	time is the time for which the location should be provided.

-	location provides the location of space in baseSpace.

It also described that for a time in the past, the runtime should locate the spaces based on the runtime’s most accurate current understanding of how the world was at that historical time. For a time in the future, the runtime should locate the spaces based on the runtime’s most up-to-date prediction of how the world will be at that future time. The minimum valid range of values for time are described in Prediction Time Limits. With respect to backward prediction, the application can pass a prediction time equivalent to the timestamp of the most recently received pose plus as much as 50 milliseconds in the past to retrieve accurate historical data.

Tracking pose prediction parameters may be monitored or observed via the OP1.

6.2.1.6	Pose prediction parameters

Pose prediction parameters include viewer pose information and the target display time and XR space.

In clause 6.2.1.1, it’s described that xrLocateViews function defined in the OpenXR can retrieve the viewer pose and projection parameters needed to render each view for use in a composition projection layer. The data structure of xrLocateViews function is defined as below in section 10.2 of View and Projection State in OpenXR [22].

The structure of xrLocateViews is describe as below [22]:

XrResult xrLocateViews(

XrSession 						session,

const XrViewLocateInfo* 	viewLocateInfo,

XrViewState* 					viewState,

uint32_t						viewCapacityInput,

uint32_t* 						viewCountOutput,

XrView* 						views);

The XrViewLocateInfo data structure is defined as below [22]:

typedef struct XrViewLocateInfo {

XrStructureType 				type;

const void* 					next;

XrViewConfigurationType 	viewConfigurationType;

XrTime 						displayTime;

XrSpace 						space;

} XrViewLocateInfo;

In XrViewLocateInfo structure, it contains the display time and space used to locate the view XrView function. The displayTime is the time for which the view poses are predicted, and space is the XrSpace in which the pose in each XrView is expressed. So pose information corresponding to each view can be predicated based on the target display time for a given frame.

Pose prediction parameters may be monitored or observed via the OP1.

6.2.1.7	Eye gaze pose prediction parameters

Eye gaze typically consists of a gaze origin (a point positioned between the user’s eyes) and a gaze direction (a ray pointing towards where the user is looking at), and gaze point (a three-dimensional position where the user is looking at). There are two different cases to use eye gaze information, e.g. for eye interacting and for eyes rendering in XR experience.

It’s noted that AR/MR QoE metrics collection is sensitive to user privacy, e.g. eye gaze related QoE metrics.

In the eye interaction case, it’s described in the OpenXR [22] that applications can get eye gaze input from an eye tracker to enable eye gaze interactions by XR_EXT_eye_gaze_interaction extension. With this extension, an application can discover if the XR runtime has access to an eye tracker, bind the eye gaze pose to the action system, determine if the eye tracker is actively tracking the users eye gaze, and use the eye gaze pose as an input signal to build eye gaze interactions [22]. In this case, eye gaze pose information is directly got by the application, not used in the rendering.

The eye gaze pose is natively oriented with +Y up, +X to the right, and -Z forward and not gravity-aligned. The eye gaze pose may originate from a point positioned between the user’s eyes. At the time both the position and direction of the eye pose is tracked. The runtime must set both XR_SPACE_LOCATION_POSITION_TRACKED_BIT and XR_SPACE_LOCATION_ORIENTATION_TRACKED_BIT. To allow for an application to reason about high accuracy eye tracking, the application can chain in an XrEyeGazeSampleTimeEXT to the next pointer of the XrSpaceLocation structure passed into the xrLocateSpace call. The XrEyeGazeSampleTimeEXT structure is defined as below:

typedef struct XrEyeGazeSampleTimeEXT {

XrStructureType				type;

void*							next;

XrTime							time;

} XrEyeGazeSampleTimeEXT;

In XrEyeGazeSampleTimeEXT structure, it’s defined that time is when in time the eye gaze pose is expressed. The time in the XrEyeGazeSampleTimeEXT structure can be set to the clamped, predicted or interpolated time. The application may inspect the time field to understand when in time the pose is expressed. The time field may be in the future if a runtime can predict gaze poses. So eye gaze poses can be predicated based on the future time.

In eye interaction case, eye gaze pose prediction parameters may be monitored or observed via the OP1.

In the case that applications needs to render eyes in XR experience and obtain position and orientation of the user’s eyes, such as driving the animation of avatar eyes. In this case, eye gaze information may be used for foveated rendering.

The XrEyeTrackerFB handle can be used to represent the resources for eye tracking. This handle is used for getting eye gaze using xrGetEyeGazesFB function. The xrGetEyeGazesFB function is defined as below:

XrResult xrGetEyeGazesFB(

XrEyeTrackerFB				eyeTracker,

const XrEyeGazesInfoFB*	gazeInfo,

XrEyeGazesFB*				eyeGazes);

In the xrGetEyeGazesFB function, gazeInfo is the information to get eye gaze, and eyeTracker is a pointer to XrEyeGazesFB receiving the returned eye poses and confidence. The XrEyeGazesInfoFB structure describes the information to get eye gaze directions, whose structure is defined as below:

typedef struct XrEyeGazesInfoFB {

XrStructureType				type;

const void*						next;

XrSpace						baseSpace;

XrTime							time;

} XrEyeGazesInfoFB;

In the xrGetEyeGazesFB function, baseSpace is an XrSpace within which the returned eye poses will be represented. The time is an XrTime at which the eye gaze information is requested.

The application should request a time equal to the predicted display time for the rendered frame. The system will employ appropriate modeling to provide eye gaze at this time.

The XrEyeGazesFB structure returns the state of the eye gaze directions, which structure is defined as below:

typedef struct XrEyeGazesFB {

XrStructureType				type;

void*							next;

XrEyeGazeFB					gaze[XR_EYE_POSITION_COUNT_FB];

XrTime							time;

} XrEyeGazesFB;

The gaze is an array of XrEyeGazeFB receiving the returned eye gaze directions, and the time is an XrTime time at which the returned eye gaze is tracked or extrapolated to.

XrEyeGazeFB structure describes the validity, direction, and confidence of a social eye gaze observation, which structure is defined as below:

typedef struct XrEyeGazeFB {

XrBool32						isValid;

XrPosef						gazePose;

Float							gazeConfidence;

} XrEyeGazeFB;

The gazePose is an XrPosef describing the position and orientation of the user’s eye. XrPosef structure is defined as below:

typedef struct XrPosef {

XrQuaternionf					orientation;

XrVector3f						position;

} XrPosef;

The pose is represented in the coordinate system provided by XrEyeGazesInfoFB::baseSpace. The gazeConfidence is a float value between 0 and 1 that represents the confidence for eye pose. A value of 0 represents no confidence in the pose returned, and a value of 1 means maximum confidence in the returned eye pose. The eye gaze pose is natively oriented with +Y up, +X to the right, and -Z forward and not gravity-aligned, similar to the XR_REFERENCE_SPACE_TYPE_VIEW in [22].

Eye gaze pose parameters, such as user’s eye position and orientation parameter can be predicted at a future time base on the field of XrTime in XrEyeGazesFB structure.

In the eye rendering in XR experience cases, eye gaze pose prediction parameters may be monitored or observed via the OP1.

In summary, for either eye interaction or eye rendering cases, the eye gaze pose prediction parameters may be monitored or observed via the OP1.

Gaze pose parameters may be used to be identified as two potential QoE metrics. Firstly, the eye gaze pose prediction error can be derived by calculating the deviations between the actual gaze position/orientation information and the predicated gaze position/orientation information. The eye gaze pose prediction error affects user's experience of looking at an XR world area or content. e.g. in the foveated rendering scenario, eye tracked foveated rendering renders lower pixel density in the periphery of the user’s gaze, taking advantage of low peripheral acuity. If the eye gaze pose prediction error is a lot, it may happen that people will see low-pixel density content in the center of the gaze, this will reduce the user's perception of the experience.

Secondly, the eye gaze interaction latency may be also derived to indicate the time from the eye gaze information is requested until the eye gaze pose is displayed (i.e., the content reflecting the eye gaze information is displayed). e.g. in the foveated rendering scenario, if the eye gaze interaction latency is large and when the user's gaze changes, people will find that the content he is looking at is still low pixelated for a longer period of time, which will degrade user experience.

Editor’s Note: Eye gaze parameters monitoring and potential eye gaze related QoE metrics can be further discussed if there are detailed eye gaze rendering information imported from MeCar WI”.

6.2.1.8	Parameters monitored by OP1

To summarize, observation point 1 is defined to monitor:

-	Viewer pose

-	Projection parameters

-	Camera information

-	Gesture

-	Body action

-	Tracking pose prediction parameters

-	Pose prediction parameters

-	Registration latency

6.2.2	Observation Point 2

Scene Manager is a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the XR Runtime. XR Scene Manager has access to the latest pose and tracking information from the XR Runtime which is then provided. Based on this information, the Scene Manager may for example determine the objects visible to the user at a given point in time or more generally the objects that may be needed to be rendered in the next rendering cycles.

Media Access Function is a set of functions that enables access to media and other XR-related data that is needed in the Scene manager or XR Runtime to provide an XR experience. The media access function accesses the network resources or sends data to the network using the established media pipelines.

Observation point 2 is derived from the API which exchanges information between Scene Manager and Media Access Function and is defined to monitor:

Editor’s Note: Parameters to be monitored in OP2 are FFS, which can be aligned with MeCar WI TS 26.119.

6.2.3	Observation Point 3

Observation point 3 is derived from the API which exchanges information between Media Access Function and 5G System and is defined to monitor:

Editor’s Note: Parameters to be monitored in OP3 are FFS, which can be aligned with MeCar WI TS 26.119.

6.2.4	Observation Point 4

Observation point 4 is derived from the API which exchanges information between XR Source Management and Metrics Access Functions and is defined to monitor:

Editor’s Note: Parameters to be monitored in OP4 are FFS, which can be aligned with MeCar WI TS 26.119.

6.3	Introduction of AR/MR QoE metrics

The typical procedures can be shown as below [2].

1.	Application Started

2.	Initial AR/MR object retrieval

3.	XR Spatial Mapping

4.	The AR/MR objects are rendered and displayed at the right place based on the reconstructed 3D map.

Based on the above typical procedures, the following QoE metrics can be introduced and measured.

6.3.1	Registration latency

Registration latency indicates the time from the application is started until the 3D reconstructed map is obtained by the XR runtime and it can be observed in the OP-1. The whole 3D map reconstruction includes following aspects:

Figure 6.3.1-1: Functional diagram for XR Spatial computing with network/cloud support [2]

1)	Surrounding sensing latency;

2)	Sensor information delivery from the MAF to the XR Spatial Computing Server;

3)	3D Map reconstruction and Spatial Description generation;

4)	Spatial Description delivery from the XR Spatial Computing Server to the MAF;

5)	Spatial Description parsing and the local/remote AR object displaying;

This metric is also available for the local AR/MR experience without network assistance.

6.3.2	Scene startup latency and Interaction latency

Scene startup latency indicates the time from the application is started until the remote initial AR scene is displayed in the right place of the reconstructed 3D space. For instance, once the AR application is started, an initial AR scene is requested by the client and further sent back to the AR runtime.

The interaction latency indicates the time from the new AR scene is requested until the remote new AR scene is displayed. For example, when user clicks to request a specific AR object in the front, the AR scene is then requested by the client and further sent back to the AR runtime for rendering and display. Render to photon time is calculated using predicted displayTime minus startRenderTime. Predicted displayTime can be monitored by the observation point 1, and startRenderTime is defined as the time when the renderer starts to render the scene according to the viewer pose.

These can be observed in the OP-1. This can include following aspects:

Figure 6.3.2-1: Functional structure for AR UE

1)	Optionally, AR scene request sent from the MAF to the remote scene server;

2)	AR scene generation and rendering the remote scene server;

3)	AR scene delivery from the scene server to the MAF;

4)	AR scene rendering and display.

This metric is also available for the local AR/MR experience without network assistance.

6.3.3	Tracking pose prediction error

This metric belongs to the device part, which mainly depends on the tracking pose prediction accuracy.

Tracking pose prediction error mainly refers to the relative pose error which indicates the deviation of the relative pose in the real world and the predicted pose. This can be observed at OP-1 and derived by comparing the predicated spaces locations and real space locations, and detailed QoE metric is defined in the Table 6.3.3-1.

Table 6.3.3-1: Tracking pose prediction error

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| TrackingPosePredictionErrorSet |  |  | Set | Set of tracking pose prediction error. |
|  | Entry |  | Object |  |
|  |  | Time | Integer | The time for which the location should be provided. |
|  |  | SpacePredictionError | Set | The deviation between the actual and predicted space location. |

Note that the actual location may not be known in an XR session.

6.3.4	One-way delay and RTT

6.3.4.1	Background

The motion-to-render-to-photon delay has a significant impact on the QoE. The delay consists of the uplink one-way delay, the downlink one-way delay, or the RTT. One of the issues of measuring these delays is that the measurements may not be representative of the delays experienced by the media. The issue has been considered in SmarTAR [23], and an in-band delay measurement method with RTP header extension has been agreed. The method is beneficial to improving the accuracy of the measured the one-way delays and RTT.

6.3.4.2	Metric description

Each delay of the uplink one-way delay, the downlink one-way delay and RTT includes the delays in the communication networks as well as the processing delay such as encryption (e.g., in the case of SRTP) and the queueing delay in the processing pipeline.

6.3.5	Pose error and time error

6.3.5.1	Background

The rendering process may use a predicted pose for rendering. The pose error (the difference between the pose used for rendering and the pose at the actual display time) affects the match can cause motion sickness, although the XR Runtime can mitigate the impact of pose errors to some extent by reprojection. Thus the pose error is a relevant metric for QoE.

The pose error depends on the time error (for a rendered frame, how much the predicted display time is off from the actual display time). The time error can be used as a control knob by the rendering process and the communication network to adjust the respective delays in optimizing the QoE. Therefore, the time error is a relevant metric for QoE optimization.

6.3.5.2	Metric description

As described in clause 6.2.1.1, a pose can be described by a position and an orientation in space relative to an XR Space.  Viewer Pose Prediction Error QoE metric is defined in the below table 6.3.5.2-1.

Table 6.3.5.2-1: Viewer Pose Prediction Error

| Key |  |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ViewerPosePredictionErrorSet |  |  |  |  |  | Set | Set of viewer pose prediction errors. |
|  | Entry |  |  |  |  | Object |  |
|  |  | Time |  |  |  | Integer | The time when the predicted viewer pose is used for. |
|  |  |  | view |  |  | Integer | The view index (0 for left eye and 1 for right eye) |
|  |  |  |  | Pose prediction error |  | Set | The deviation between the actual and predicted pose. |
|  |  |  |  |  | Position prediction error | Vector | Vector distance between the actual and predicted position |
|  |  |  |  |  | Orientation prediction error | Vector | Quaternion distance between the actual and predicted position |
|  |  |  |  | FoV prediction error |  | Set | The deviation between the actual and predicted FoV. |
|  |  |  |  |  | Left error | float | Difference between the actual and predicted left angle of FoV |
|  |  |  |  |  | Right error | float | Difference between the actual and predicted right angle of FoV |
|  |  |  |  |  | Up error | float | Difference between the actual and predicted Up angle of FoV |
|  |  |  |  |  | Down error | float | Difference between the actual and predicted Down angle of FOV |

The view is an integer value specifying left or right eye. In OpenXR this corresponds to view index in XrViewConfigurationProperties and XrCompositionLayerProjection. As an example, the deviation of actual and predicated pose information can be summarized into a single metirc as formula 6.3-1. In this formula, DevposPredError means the deviation of actual and predicated pose information. α and β represent the weights of the deviation of position and orientation respectively, the weights may be set based on the implementation or application. PA, PP refer to the actual position and the predicted position respectively, with (x,y,z) indicating their respective Cartesian coordinates , and QA ,QP refer to the actual orientation and the predicted orientation respectively, expressed as unit quaternions and Q-1 indicates the quaternion conjugation operation.

DevPosPredError=α(x PA-xPP)2+(y PA-yPP)2+(z PA-zPP)2+β ||ln⁡(QA-1QP)||                                          6.3-1

Note that the actual pose may not be known during an XR session.

6.3.5.3	Measurement procedure

A measurement procedure for the scenario of cloud-based rendering is shown in Figure 5.2.3-1. The XR Runtime and the XR Application may be on a same device such as a UE, or on difference devices such as an AR glasses (which hosts the XR Runtime) and a UE (which hosts the XR Application). The steps are as follows:

1)	The XR Application estimates the round-trip time (RTT) between the XR application and the Edge Application Server (EAS).

2)	The XR Application queries for the next display time. This (and step 3) can be achieved by calling the xrWaitFrame function in OpenXR.

3)	The XR Runtime replies with the next display time.

4)	The XR application predicts a display time – an initial prediction – and the use of initial is because a second prediction/estimation will be made later. This predicted display time is called T2.predicted1.

5)	The XR application queries for a predicted pose at the initial predicted display time T2.predicted1. Calling the function xrLocateViews in OpenXR can achieve this step and step 7.

6)	The XR Runtime predicts the pose, and the prediction occurs at time T1.

7)	The XR Runtime returns with the predicted pose (P.predicted1) including status flags information.

7bis)	The XR application computes the accuracy level (AL.predicted1) of the predicted pose based on the status flags information returned with that pose.

8)	The XR application sends the predicted pose (P.predicted1) and the associated initial predicted display time (T2.predicted1) to the EAS.

9)	The EAS renders for the predicted pose (P.predicted1), and compresses the rendered frame.

10)	The EAS returns the rendered frame along with the initial predicted display time (T2.predicted1) to the XR Application.

11)	The XR Application sends the rendered frame to the XR Runtine, e.g., via swapchain. This can be achieved by calling the xrReleaseSwapchainImage function in OpenXR. The XR Application passes the display time used for the rendering the frame, and this can be achieved by calling the xrEndFrame function in OpenXR.

12)	The XR Application queries for the predicted display time. This is intended to get a more accurate prediction of the display time than the one in step 4, because there is less time to predict into the future at this moment.

13)	The XR Runtime returns an updated prediction of the display time (T2.predicted2).

14)	The XR Runtime performs reprojection for pose correction. The actual display play time is called T2.actual.

15)	The XR Application queries for the pose associated with the updated prediction of the display time (T2.predicted2). This can be achieved by calling the xrLocateViews function in OpenXR.

16)	The XR Runtime does pose estimation.

17)	The XR Runtime returns a pose estimate (P.predicted2) including status flags information.

18)	The XR Application computes the aggregated accuracy level (AL.predicted2) using the status information returned with the pose (P.predicted2) from the step 17 and the accuracy level (AL.predicted1) from the step 7bis. Then the XR Application computes a pose error estimate (P.predicted1 – P.predicte2) and a time error estimate(T2.predicted1 – T2.predicted2) according to the aggregated accuracy level (AL.predicted2).

Figure 6.3.5.3-1: The procedure for measuring the pose error and time error in pose prediction

Note that two queries are used to predict the display time of a same frame. The first query occurs in step 2, and the query result is used to determine a target display time for the rendering process in step 4. The second query occurs much closer to the actual display time, as shown in steps 12-13, and thus provides higher accuracy. This is shown in Figure 6.3.5.3-2.

Figure 6.3.5.3-2: The use of a second prediction (T2.predicted2) of the display time for better accuracy

Note: to compute the accuracy level in step 7bis and 18 with the Khronos OpenXR API [22], the xrLocateViews function returns the status information related to the predicted/estimated pose in the XrViewState structure. XrViewStateFlags in the XrViewState are flags that give information validity and tracking of position and orientation.

The following XrViewStateFlags may be used to compute an accuracy level on the predicted/estimated pose:

-	XR_VIEW_STATE_ORIENTATION_VALID_BIT

-	XR_VIEW_STATE_POSITION_VALID_BIT

-	XR_VIEW_STATE_POSITION_TRACKED_BIT

-	XR_VIEW_STATE_ORIENTATION_TRACKED_BIT

Editor’s note: the computation of the accuracy level using the XrViewStateFlags is FFS.

6.3.6	Device related QoE metrics

6.3.6.1	Background

Some property information of AR/MR device have impacts on user experience, e.g. resolution, hand tracking, eye tracking, spatial mapping…, which are the part of AR/MR device-related QoE metrics.

For AR/MR terminals, the UE can get the system properties by xrGetSystemProperties function defined in openXR [22]. The structure of xrGetSystemProperties is defined as below [22]:

XrResult xrGetSystemProperties(

XrInstance						instance,

XrSystemId					systemId,

XrSystemProperties*			properties);

The properties points to an instance of the XrSystemProperties structure, that will be filled with returned information, and the XrSystemProperties structure is defined as [22]:

typedef struct XrSystemProperties {

XrStructureType 					type;

void*                        				next;

XrSystemId                    			systemId;

uint32_t                      				vendorId;

char                          				systemName[XR_MAX_SYSTEM_NAME_SIZE];

XrSystemGraphicsProperties    	graphicsProperties;

XrSystemTrackingProperties   	 trackingProperties;

} XrSystemProperties;

The “next” is NULL or a pointer to the next structure in a structure chain, such as XrSystemEyeTrackingPropertiesFB, XrSystemHandTrackingPropertiesEXT, XrSystemSpatialEntityPropertiesFB…

The XrSystemEyeTrackingPropertiesFB structure is defined as below in [22]:

typedef struct XrSystemEyeTrackingPropertiesFB {

XrStructureType				type;

void*							next;

XrBool32						supportsEyeTracking;

} XrSystemEyeTrackingPropertiesFB;

The “supportsEyeTracking” is an XrBool32, indicating if the current system is capable of receiving eye tracking input. So, eye tracking capability can be got and indicated from the runtime when the AR/MR terminals check the value of “supportsEyeTracking”. When the value is true, the user may get a better experience in comparison with the case that AR/MR UE doesn’t support eye tracking capability.

The XrSystemHandTrackingPropertiesEXT structure is defined as below in [22]:

typedef struct XrSystemHandTrackingPropertiesEXT {

XrStructureType				type;

void*							next;

XrBool32						supportsHandTracking;

} XrSystemHandTrackingPropertiesEXT;

The “supportsHandTracking” is an XrBool32, indicating if the current system is capable of hand tracking input. So, hand tracking capability can be got and indicated from the runtime when the AR/MR terminals check the value of “supportsHandTracking”. When the value is true, the user may get a better experience in comparison with the case that AR/MR UE doesn’t support hand tracking capability.

The XrSystemSpatialEntityPropertiesFB structure is defined as below in [22]:

typedef struct XrSystemSpatialEntityPropertiesFB {

XrStructureType				type;

const void*						next;

XrBool32						supportsSpatialEntity;

} XrSystemSpatialEntityPropertiesFB;

The “supportsSpatialEntity” is an XrBool32, indicating if spatial entities are supported by the system. An application can inspect whether the system is capable of spatial entity operations by extending the XrSystemProperties with XrSystemSpatialEntityPropertiesFB structure when calling xrGetSystemProperties. The XR_FB_spatial_entity can provide the XrSpatialAnchorCreateInfoFB whose structure is defined as in [22]:

typedef struct XrSpatialAnchorCreateInfoFB {

XrStructureType				type;

const void*						next;

XrSpace						space;

XrPosef						poseInSpace;

XrTime							time;

} XrSpatialAnchorCreateInfoFB;

The space entity identifies world-locked frames of reference. It enables applications to persist the real world location of content over time and contains definitions. The spatial mapping capability can be got and indicated from the runtime when the AR/MR terminals check the value of “supportsSpatialEntity”. When the value is true, the user may get a better experience in comparison with the case that AR/MR UE doesn’t support spatial mapping capability.

6.3.6.2	Metric description

Base on the above analysis in clause 6.3.6.1, the QoE metrics relevant with AR/MR device as listed in Table 6.3.6.2-1 is necessary for assessment of device impact on user experience.

Table 6.3.6.2-1: QoE metrics relevant with AR/MR device

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| DeviceInformationList |  |  |  | List | A list of device information objects. |
|  | Entry |  |  | Object | A single object containing new device information. |
|  |  | resolution |  | Object | Display resolution for each eye |
|  |  |  | videowidth | Integer | Number of pixels in display width |
|  |  |  | videoheight | Integer | Number of pixels in display height |
|  |  | eyetrackingCapability |  | Boolean | Indication of end device eye tracking capability. |
|  |  | handtrackingCapability |  | Boolean | Indication of end device hand tracking capability. |
|  |  | spatialmappingCapability |  | Boolean | Indication of end device spatial mapping capability. |
|  |  | … |  |  |  |

Editor’s Note: Additional AR/MR device-related QoE metrics are FFS.

6.3.7	Spatial Anchors and Trackables

6.3.7.1	Background

To establish the pose of the virtual objects in the user real environment, the concept of AR anchoring has been defined based on trackable and spatial anchor entities.

A trackable [24] is a model of an element of the real world of which features are available and/or could be extracted. Each trackable provides a local reference space in which a spatial anchor pose can be expressed.

A spatial anchor [24] corresponds to a real-world pose, identified using one or more trackables. Each spatial anchor provides a local reference space in which a pose can be expressed.

Figure 6.3.7.1-1 illustrates an AR anchoring example. A trackable (2D marker type) provides a local reference space. The spatial anchor refers to this trackable, with the pose TRS#1, for accurate positioning relative to the real world. An AR Asset (Virtual chest) is attached to this spatial anchor with the pose TRS#2.

Figure 6.3.7.1-1: Spatial relationships between trackable, spatial anchor and virtual asset

6.3.7.2	Impact on latencies

AR anchoring has an impact on the user experience, for example, a virtual object may not be correctly positioned in the user's real environment.

AR anchoring latencies also directly impacts the scene start-up latency and the interaction latency:

-	The initial scene start-up latency corresponds to the sum of the first initialization step delay (fetching content entry point, initialization of the scene manager, retrieval of the scene description file, the Anchor Creation Delay, the delay until the trackable is first detected and the Anchor Detection-to-Render-to-Photon delay in the case of a single spatial anchor for the whole scene.

-	The interaction latency corresponds to the Anchor Detection-to-Render-to-Photon latency metric for the first detection of the trackable. After a first detection, the spatial computing function may predict the pose of the trackable even if it is no more tracked, enabling the anchoring, the positioning, and the display of the virtual content (even if the tracking pose prediction error may increase).

6.3.7.3	Retrieval of the AR anchoring information

Once the Scene description file is received by the UE, the scene manager parses the file and retrieves the AR anchoring information required for that AR experience.

The AR anchoring information consists of:

-	The different types of trackable to be supported.

-	For each trackable, the spatial relationship between the trackable, its related spatial anchor and the virtual content to be anchored.

-	Some optional metadata specifying how to handle the AR anchoring process at runtime. E.g., displaying or not the virtual content at a default location until the trackable is detected, defining a minimum available space in the user’s real environment to allow the anchoring of virtual content.

For example, the MPEG-I Scene Description AMD2 [24] detailed and specified an extension to carry the AR anchoring information in a glTF file.

From a QoE metric perspective, the most relevant AR anchoring information is the type(s) of trackable(s) required for that AR experience as some types of trackable may not be supported locally in the UE, leading to Cloud or Edge delegation for the spatial computing function for that trackable. This delegation has a direct impact on the configuration and the measurement of QoE metric.

Several spatial anchors may be defined in one scene to anchor different virtual content. The QoE metric should be measured for each spatial anchor.

6.3.7.4	Anchor Creation Delay (ACD)

This metric corresponds to the delay between the time of the spatial anchor creation request and the time when the related XR space (i.e., the frame of reference in which the 3D coordinates are expressed) is created.

6.3.7.4.1	Measurement of the local Anchor Creation Delay

If the type of trackable is supported locally in the UE, this metric is measured at the interface between the XR runtime and the scene manager. It corresponds to the OP-1 Observation Point.

To measure the ACD with the Khronos OpenXR API [22]:

-	The ACD start time which corresponds to the time of the spatial anchor creation request i.e., when calling the xrCreateReferenceSpace, xrCreateActionSpace, xrCreateSpatialAnchorFB, xrCreateSpatialAnchorMSFT or xrCreateSpatialAnchorFromPersistedNameMSFT function depending on the type of trackable to support.

-	The ACD end time which corresponds to the time when receiving a XR_SUCCESS returned value.

Then the ACD for that spatial anchor = end time - start time.

6.3.7.4.2	Measurement of the remote Anchor Creation Delay

In the case of remote spatial computing (i.e., the type of trackable is not supported locally in the UE), it is relevant to measure the ACD metric.

The measurement procedure of the ACD in case of remote spatial computing is provided in Figure 6.3.7.4-1.

Figure 6.3.7.4-1: The procedure for measuring the ACD with remote XR spatial computing

1)	The UE and the Server configures the spatial anchor creation QoE. A delegation to a XR Spatial Computing server is established for that spatial anchor as the trackable cannot be supported by the UE.

2)	The request of the spatial anchor creation is sent by the UE Scene Graph Handler to the XR Spatial Computing Server. A unique spatial anchor identifier (anchor-id) and the anchor-creation-request-time are recorded.

3)	The spatial anchor creation request is received by the XR server, and it starts to the creation of the anchor.

4)	Once the XR Server has created the spatial anchor, it transmits the acknowledgement to the UE.

5)	The Scene manager receives the acknowledgment of the anchor creation and records the anchor-creation-end-RX-time.

Based on this ACD measurement call flow, the UE can measure the ACD as follows:

Then the ACD for that spatial anchor = anchor-creation-end-time – anchor-creation-start-time.

The Roundtrip ACD for that spatial anchor = anchor-creation-end-RX-time – anchor-creation-request-time.

6.3.7.5	Anchor Detection to Render to Photon QoE

This metric corresponds to the delay between the time of the spatial anchor pose request leading to the detection of the trackable and the time when the virtual content is displayed in the user’s real environment.

6.3.7.5.1	Measurement of the Anchor Detection to Render to Photon (ADRP)

This step occurs each time a detection process of the trackable related to that spatial anchor needs to be launched.

Typically, it occurs:

-	At the beginning, after the spatial anchor creation and after a potential first adjustment of the AR anchoring process

-	When the trackable associated with that spatial anchor was not visible and becomes visible again.

To measure the ADRP with the Khronos OpenXR API [22]:

-	The ADRP start time which corresponds to the time of the spatial anchor pose request leading to the detection of the trackable. The xrLocateSpace() function is used to request a spatial anchor pose at a current or predicted time. The flags XrSpaceLocationFlags in the XrSpaceLocation structure returned by the xrLocateSpace() function are used to check the trackable detection state:

a)	The detection of the trackable corresponds to ((XR_SPACE_LOCATION_POSITION_VALID_BIT Flags is set) AND (XR_SPACE_LOCATION_POSITION_TRACKED_BIT Flags are changed from unset to set)) AND ((XR_SPACE_LOCATION_ORIENTATION_VALID_BIT Flags is set) AND (XR_SPACE_LOCATION_ORIENTATION_TRACKED_BIT Flags are changed from unset to set))

-	The ADRP end time which corresponds to the actual display time for that frame.

The measurement procedure of the ADRP for UE device with local spatial computing is provided in Figure 6.3.7.4 2.

Figure 6.3.7.4-2: The procedure for measuring the ADRP with local spatial computing

1)	The Scene Graph Handler requests the pose of the spatial anchor to the XR Spatial Compute function. The UE records the anchor-pose-request-time.

2)	The XR Spatial Computing function detects of the trackable related to that spatial anchor.

3)	The XR Spatial Computing function sends the spatial anchor pose to the Scene Graph Handler. The UE records the anchor-detection-time.

4)	The UE updates the scene by enabling the virtual assets related to that spatial anchor and by placing them with respect to the detected spatial anchor pose.

5)	The UE records the render-start-time when the updated scene is ready to be rendered.

6)	The UE renders the scene with the predicted pose related to the predicted display time provided by the XR Runtime.

7)	The UE records the render-end-time when the rendering task is done. The rendered frame is provided to the XR Runtime.

8)	The XR Runtime performs further post-processing such as late pose correction and final composition.

9)	The rendered frame is presented to the display. The actual display time is called T2.actual.

Based on this ADRP measurement procedure, the UE can measure the ADRP as follows:

ADRP for that spatial anchor = T2.actual – anchor-pose-request-time.

6.3.7.6	Anchor Untracked Ratio QoE

This metric corresponds to the ratio between the number of frames where the trackable associated with the spatial anchor is not tracked and the total number of frames during the observation period in which no detection process is launched.

The tracking is lost, so untracked, when the position and/or orientation are no more actively tracked. The XR runtimes may continue to provide valid but untracked position and/or orientation values that are inferred or last-known, so long as it’s still meaningful for the application to use that position and/or orientation.

Editor’s Note: Impact on the AUR metric when the spatial anchor relies on several trackables is FFS.

Editor’s Note: Impact on the AUR metric when a trackable creates multiple spatial anchors is FFS.

6.3.7.6.1	Measurement of the Anchor Untracked Ratio (AUR)

This metric is measured at the interface between the XR runtime and the scene manager. It corresponds to the OP-1 Observation Point.

This measurement occurs after the trackable detection. The measurement parameters (e.g., periodicity, observation period) of the AUR metrics have been defined during the configuration step.

Two counters are used to measure the Anchor Untracked Ratio:

-	Nf: length in number of frames of the observation period in which no detection process is launched.

-	Nu: Number of untracked in the observation period. This counter is incremented for each rendered frame where the trackable associated with the spatial anchor is not tracked.

The Khronos OpenXR API [22] may be used to know the tracked/untracked status of a trackable with the xrLocateSpace() function. The flags XrSpaceLocationFlags in the XrSpaceLocation structure returned by the xrLocateSpace() function are used to know the tracked/untracked status of a trackable:

-	The spatial anchor is said as untracked and Nu is incremented when the tracking of the position or orientation are lost but the XR runtime still provides valid inferred or last-known position and orientation: 
((XR_SPACE_LOCATION_POSITION_TRACKED_BIT Flag is unset) OR (XR_SPACE_LOCATION_ORIENTATION_TRACKED_BIT Flag is unset)) AND (XR_SPACE_LOCATION_ORIENTATION_VALID_BIT Flags is set) AND (XR_SPACE_LOCATION_POSITION_VALID_BIT Flags is set)

-	The measurement is aborted when the XR runtime stops to provide valid position or orientation: 
(XR_SPACE_LOCATION_ORIENTATION_VALID_BIT Flags toggles from set to unset) OR (XR_SPACE_LOCATION_POSITION_VALID_BIT Flags toggles from set to unset)

-	New observation period is launched when the XR runtime starts to provide valid position and orientation: 
(XR_SPACE_LOCATION_ORIENTATION_VALID_BIT Flags toggles from unset to set) AND (XR_SPACE_LOCATION_POSITION_VALID_BIT Flags toggles from unset to set)

Then the AUR for that spatial anchor = Nu / Nf

7	Other QoE metrics and collaborations with other 3GPP groups

7.1	General NWDAF based metrics configuration and collection

The Network Data Analytics Function (NWDAF) is a core network function performing

-	Inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and analytics results exposure.

-	Training Machine Learning (ML) models and new training services (e.g. providing trained ML model) exposure.

The analytics information is either statistical information of the past events, or predictive information. The NWDAF can collect the service experience, e.g. subscribing to the DC-AF for the QoE metrics. Based on the collected QoE metrics, the NWDAF may provide the estimation of the service experience for a specific application, a specific network slice or a specific UE as defined in clause 6.4 of TS 23.288 [25]. The AR/MR QoE metrics in present document also can be used by NWDAF based metrics configuration and collection.

7.2	RRC based metrics configuration and collection

The RRC based QoE metrics configuration and reporting is also named as the QoE Measurement Collection (QMC) functionality. In this case, the OAM provides the QoE configuration to the RAN and then the RAN sends the QoE configuration to the UE via specific RRC messages for UMTS [26], LTE [27] and NR [28] over the control plane. Similarly, the UE collects and reports the QoE metrics via the specific RRC messages to RAN and RAN further sends the QoE reports to the OAM for service experience evaluation, as shown below.

Figure 7.2-1: Example signalling diagram for NR

The service types supported by the QMC functionality include the DASH, VR and the MTSI [29]. According to the AR/MR QoE metrics definition, the QMC functionality may need to support the collection and reporting of AR/MR QoE by existing mechanism. The AR/MR QoE metrics study in the present document will not impact the QMC functionality.

7.3	Summary

Based on the analysis above, both the NWDAF based and the RRC based QoE metrics configuration and collection are not in scope of this AR/MR QoE study.

8	Conclusions and Recommendations

Editor’s Note: Provide recommendation on normative work for new XR QoE metrics based on the findings in this study

Annex <A> (informative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA4#120e | S4-221164 |  |  |  | Initial version | 0.0.1 |
| 2022-08 | SA4#120e | S4-221165 |  |  |  | S4-221163: Collect information from ITU-T | 0.1.0 |
| 2022-11 | SA4#121 | S4-221564 |  |  |  | Add QoE-related activity in the external standardizations | 0.2.0 |
| 2023-02 | SA4#122 | S4-230294 |  |  |  | Add QoE information collection from MPEG, relevant VR metrics, basic architecture and observation points. | 0.3.0 |
| 2023-04 | SA4#123-e | S4-230699 |  |  |  | Clarify the parameters that can be monitored by the observation point 1. Identify some QoE metrics. Replace the figures with the original figure or figures in visio format. Change the equations to mathtype format. | 0.4.0 |
| 2023-05 | SA4#124 | S4-230987 |  |  |  | Delete the reference to MeCar PD and S4-230393; Add the pose prediction parameters that can be observed in OP1; Define the Pose error and time error QoE metric, Tracking position prediction error, device related QoE metric, and one-way delay RTT metric. | 0.5.0 |
| 2023-06 | SA4#125 | S4-231539 |  |  |  | Remove the Editor’s Note in clause 4. Revise “Tracking position prediction parameters” to “Tracking pose prediction parameters”, and update “Tracking position prediction error” metric to “Tracking pose prediction error”. Add the Eye gaze pose prediction parameters” that may be observed via OP1. Remove the Editor’s Note in clause 6.2.1.8. Update the Editor’s Note in clause 6.2.2, 6.2.3 and 6.2.4 by indicating the parameters tha may be monitored by OP2~OP4 depends on the progress of MeCar WI. Remove the Editor’s note in clause 6.,3.2 by clarifying Render to photon time. Remove the Editor’s note in clause 6.3.5 by defining how to calculate the Viewer Pose Prediction Error metric. Update the Pose error and time error measurement procedure. Add the Spatial Anchors and Trackables related QoE metrics, which include Anchor Creation Delay (ACD) metric, Anchor Detection-to-Render-to-Photon (ADRP) metric, and Anchor Untracked Ratio (AUR) metric. Remove the subjective assessment clause. Add some clarification that both the NWDAF based and the RRC based QoE metrics configuration and collection are not in scope of this AR/MR QoE study. Correct some typo error and add an Editor’s note in clause 6.3.7.6. | 0.7.1 |