3GPP TSG SA WG4 Meeting #125	S4-231350

# Gothenburg, SE, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR |  | rev |  | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] Transitioning clause from PD to TS 26.522 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 15 August 2023 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pCR transitions the agreed clause describing the RTP header extension for rendered pose from the 5G_RTP PD to TS 26.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 4.4	RTP Header Extensions

### 4.4.1	General

TBA

### 4.4.2	RTP Header Extension for PDU Set Marking

### 4.4.3	RTP Header Extension for Rendered Pose

The server streams the rendered frame using one or more video streams, depending on the view and projection configuration that is selected by the UE. The server should use the RTP header extension for rendered pose to associate the selected pose with the rendered frame. The rendered pose RTP header extension may also be used with audio streams.

The frequency of RTP header extension for rendered pose shall be at least once in a frame. It may be sent more often but not necessarily in every RTP packet.

The rendered pose header extension shall be declared in the SDP using the “a=extmap” attribute as defined in RFC8285 with the following URN: “urn:3gpp:xr-rendered-pose”.

The 2-byte (RFC8285) RTP header extension format shall be used for signalling the rendered pose header extension as follows:

Figure X. 2-byte RTP header extension format for rendered pose

The (x,y,z) provides the position of the rendered pose and the (rx,ry,rz,rw) provides the orientation of the rendered pose.

The timestamp represents the predicted time for the pose. This timestamp uses the XR system clock. There is no requirement to synchronize the timestamps of the RTP stream to the XR system clock. The timestamp is passed to the XR runtime together with the rendered swapchain images (e.g. as part of the xrEndFrame call in OpenXR).

A list of actions that were processed for the rendering of the frame are listed using action id as shown in Figure X. The number of action id values in one RTP header extension for rendered pose shall be no more than 10. Hence, the maximum size of the header extension is 4 × (7 + 2 × 2) + 2 × n, where n is the number of action identifiers in the header extension.

When both video and audio are delivered to the UE, or when either audio or video is delivered using multiple real-time streams (e.g., left eye + right eye), multiple RTP streams may be associated with the same header extension data, e.g., the same pose may have been used for generating multiple streams. This may lead to sending the same header extension data multiple times in different streams.

A sender may reuse the rendered pose RTP header extension of one stream for multiple RTP streams. For example, only the video stream carries the rendered pose RTP header extension, but the rendered pose is applicable also for the audio bitstream. In this case, the sender shall include the extension attribute media followed by a semi-colon separated list of media ID (MID) values in the "a=extmap” attribute. The MID values indicate all media streams for which the rendered pose RTP header extension is applicable to. If the extension attribute media is present, then the media description of all bitstreams that reuse the header extension shall include the attribute “mid” as defined in RFC 5888.

NOTE: In case there is a mismatch between the frame rates of the streams, the UE may use the few most recent samples from the source RTP stream to obtain a synchronized sample in the dependent stream via interpolation. Alternatively, the UE may choose to not perform any interpolation and simply use the last available sample from the source RTP stream for the dependent stream. It is left to the discretion of the UE application to select an appropriate synchronization method.

The syntax for the "a=3gpp:xr-rendered-pose" attribute shall conform to the following ABNF:

extmap-attr="a=extmap:" 1*5DIGIT ["/" direction] SP "urn:3gpp:xr-rendered-pose" [SP ”media:” 1*(SP token)]

## End of First change

## Second change

Annex A (informative):
Examples of SDP offers and answers

A.1 SDP example for rendered pose RTP header extension

An example SDP description using the rendered pose RTP header extension is presented below. Using the extension attribute media, the rendered pose RTP header extension with URI urn:3gpp:xr-rendered-pose provided in the video stream with MID m1 is also applicable to another video stream with MID m3.

v=0

o=alice 2890844526 2890844526 IN IP4 host.atlanta.example.com

s=SDP Session

c=IN IP4 host.atlanta.example.com

t=0 0

m=application 1001 UDP/DTLS/SCTP webrtc-datachannel

a=sendonly

m=video 23458 RTP/AVP 96

a=mid:m1

a=recvonly

a=rtpmap:96 H264/90000

a=extmap:1 urn:3gpp:xr-rendered-pose media:m3

m=audio 23468 RTP/AVP 97

a=mid:m2

a=recvonly

a=rtpmap:97 PCMU/8000

m=video 23478 RTP/AVP 97

a=mid:m3

a=recvonly

a=rtpmap:96 H264/90000

## End of Second change