3GPP TSG-SA4 Meeting # 125	S4-231451

# Gothenburg, Sweden,  21st - 25th August 2023                            (revision of S4-231193)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR |  | rev | 1 | Current version: | 0.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Rendering Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 15-08-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding support for negotiating rendering features and indicating a confidence value for pose predictions. To support eye gaze tracking, the position part of the pose is made optional. Guidelines for foveated rendering based on user eye gaze predictions are also added to Annex A. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 8.4.2.2	 Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| RenderingFlags | Array(SR_CONFIG_FLAGS) | 0..1 | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING |
| spaceConfiguration | Object | 0..1 | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |
| referenceSpaces | Array | 0..1 | An array of reference spaces and their identifiers. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| refSpace | enum | 1..1 | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE. |
| actionSpaces | Array | 0..1 | An array of action spaces that need to be defined by the split rendering client in the XR session. |
| id | number | 1..1 | A unique identifier of the XR space in the context of the split rendering session. |
| actionId | number | 1..1 | Provides the unique identifier of the action. |
| subactionPath | string | 1..1 | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |
| initialPose | Pose | 0..1 | Provides the initial pose of the new XR space’s origin. |
| viewConfiguration | Object | 0..1 | Conveys the view configuration that is configured for the XR session. |
| type | Enum | 1..1 | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |
| width | number | 1..1 | The recommended width of the swapchain image. |
| height | number | 1..1 | The recommended height of the swapchain image. |
| compositionLayer | string | 1..1 | An identifier of the selected composition layer. |
| environmentBlendMode | enum | 1..1 | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |
| actionConfiguration | Array | 0..1 | This contains a list of the actions that are to be defined by the SR client. |
| action | Object | 1..n | A definition of a single action object. |
| id | number | 1..1 | A unique identifier of the action. |
| actionType | enum | 1..1 | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |
| subactionPaths | string | 1..n | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |
| extraConfigurations | Object | 0..1 | A placeholder for addition configuration information. |

2nd Change

#### 8.2.2.2	Pose Format

The split rendering client on the XR device periodically transmits a set of pose predictions to the split rendering server. The type of the message shall be set to “urn:3gpp:split-rendering:v1:pose”.

Each predicted pose shall contain the associated predicted display time and an identifier of the XR space that was used for that pose.

Depending on the view configuration of the XR session, there could be different pose information for each view.

The payload of the message shall be as follows:

Table 8 - Pose Prediction Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| poseInfo | Object | 1..n | An array of pose information objects, each corresponding to a target display time and XR space. |
| displayTime | number | 1..1 | The time for which the current view poses are predicted. |
| xrSpace | number | 0..1 | An identifier for the XR space in which the view poses are expressed. The set of XR spaces are agreed on between the split rendering client and the split rendering server at the setup of the split rendering session. The set of XR spaces is negotiated as part of the split rendering configuration as defined in clause 8.4.2.2. |
| viewPoses | Object | 0..n | An array that provides a list of the poses associated with every view. The number of views is determined during the split rendering session setup between the split rendering client and server, depending on the view configuration of the XR session. |
| pose | Object | 1..1 | An object that carries the pose information for a particular view. |
| orientation | Object | 1..1 | Represents the orientation of the view pose as a quaternion based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the quaternion. |
| y | number | 1..1 | Provides the y coordinate of the quaternion. |
| z | number | 1..1 | Provides the z coordinate of the quaternion. |
| w | number | 1..1 | Provides the w coordinate of the quaternion. |
| position | Object | 0..1 | Represents the location in 3D space of the pose based on the reference XR space. For eye gaze poses, the position is not required. |
| x | number | 1..1 | Provides the x coordinate of the position vector. |
| y | number | 1..1 | Provides the y coordinate of the position vector. |
| z | number | 1..1 | Provides the z coordinate of the position vector. |
| confidence | number | 0..1 | This optional parameter provides a confidence score that reflects the probability for this pose prediction to be correct. For the current pose or a pose in the past, the confidence value would be 1. The confidence can take a value between 0 and 1. If not provided by the XR runtime, this field may be estimated by the SRC or omitted. |
| fov | Object | 1..1 | Indicates the four sides of the field of view used for the projection of the corresponding XR view. |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |

3rd Change

# A.2 Guidelines for Split Rendering MSE Implementers

### A.2.1	Guidelines for implementers of the Split Rendering Server

If the use of eye gaze tracking is activated, the SRS may use this information to perform foveated rendering. In foveated rendering, areas of the picture are encoded with a higher SNR quality than other areas, to match the user’s current gaze.

With gaze predictions, the SRS should create an importance map for the picture based on the confidence values associated with the gaze predictions. Additionally, the SRS may also use other information to produce the importance map, such as content Regions of Interest.

This importance map is passed to the encoder to properly allocate bits for the encoding of the picture.