3GPP TSG SA WG4#125	S4-231538

Gothenburg, Sweden, 21 – 25 August 2023	revision of S4-231215

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 8.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Video Capabilites for MeCAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 15/08/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Decoding capabilities were unclear and unaligned with TS 26.511 This is for video |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align with TS 26.511 Define multiple decoding instances |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | S4-231215; [MeCAR] Split Rendering profile updates; Qualcomm Incorporated; Thomas Stockhammer. Presenter: Thomas Stockhammer Online Discussion: - 2023/08/23: Thomas presents - Thomas: ok to add IVAS instead of EVS. Same availability on common cloud gaming servers. - Andre: also good - Stefan: also ok - Emmanuel: MeCAR is included in the device/UE, so it seems that this is about the information we can provide you. Also ok to work on IVAS according to work item plan - Andre: do we have to differentiate audio and speech? - Thomas: no, would be good to get rid of this audio/speech - Stefan B: ok to also get rid of speech and do audio only - Stephane: MECAR would be good to refer to speech - Stefan: change capabilities to encode => decode - Thomas: ok - Thomas: is there a profile for IVAS that would refer to, for example 2 or 4 decoder instances for EVS - Tomas: yes, this is the case. Supports HOA, objects, stereo - Emmanuel: would be confusing to add IVAS now, we have a CR planned for MECAR that would address it. - Some decisions: - Refer to audio/speech - Add a placeholder for IVAS - Good baseline - Emmanuel: can we support cross-codecs? - Thomas: generally yes, but harder to document, use case - Emmanuel: left and right eye HEVC, depth AVC - Thomas: believe we can document something - Clearly a set of revisions is needed |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 6.5	MeCAR Media Capabilities - Overview

A MeCAR device can query the capabilities of the XR Runtime, the Scene Manager and the Presentation Engine for audio and visual media, and the MAF.

A MeCAR device at the minimum shall support:

- A set of XR Runtime functionalities

  - allowing to establish a XR Session

  - enabling 6DoF and AR video rendering based on swapchain APIs (as defined for example in OpenXR) with pose correction

  - enabling rendering of audio signals with pose correction

  - providing a 6DoF pose of the user to the application

- A subset of 2D video codecs to support display characteristics as defined in 6.4.1.2.1

- A subset of audio codecs (to be defined)

- Basic delivery formats

- A media playback entry point (e.g. a scene description)

- A security framework

A MeCAR device may in addition support:

- Additional functionalities of XR Runtime or other APIs beyond the minimum set supported

- 3D rendering capabilities that, for example, allow to render point clouds, meshes, depth, etc. present in the scenes

- Additional audio and video encoding and decoding capabilities

- Decoding of streaming formats encapsulated in CMAF

- Delivery of data with RTP and SRTP

- Capabilities to be exchanged with the network in order to support cloud/edge rendering

=====  CHANGE  =====

## 6.6	Visual functions and capabilities

### 6.6.1	Video Decoding

### In alignment with TS 26.511, clause 4.2

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [2] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264/AVC Progressive High Profile Level 5.1 [2] bitstreams for H.264/AVC with the following additional restrictions and requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

-	the bitstream does not contain more than 10 slices per picture.

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following further limitations:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

### 6.6.2	Video Encoding

### In alignment with TS 26.511

-	AVC-FullHD-Enc: the capability to encode a video signal with:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 6.6.1

-	HEVC-FullHD-Enc: the capability to encode a video signal with

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 6.6.1.

-	HEVC-UHD-Enc: the capability to encode a video signal with

-	up to 534,773,760 luma samples per second;

-	up to a luma picture size of 8,912,896 samples;

-	up to 480 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 6.6.1.

### 6.6.3	Simultaneous Video Decoding instances

#### 6.6.3.1	Definition

Simultaneous Video Decoding Instances are defined as follows.

For N video bitstreams encoded according to a video codec profile, decoding units flow into the coded picture buffer (CPB) for each stream according to a specified arrival schedule and are delivered by the common Hypothetical Stream Scheduler (HSS) that schedules the N bitstreams for decoding each of the units. For each access unit

- all data associated with an access unit is removed and decoded instantaneously by the instantaneous decoding process at CPB removal time of the access unit.

- Each decoded picture is placed in the Decoded Picture Buffer (DPB) for being referenced by the decoding process of this stream as well as for output and cropping.

- A decoded picture is removed from the DPB at the time that it becomes no longer needed for inter-prediction reference as well as the output time of the access unit is the largest of all decoded pictures remaining in the group of N decoders

Then at any point time,

- each of the individual streams conforms to the signaled profile/level/tier and HRD parameters of the individual stream.

- The sum of the CPB size conforms to common profile/level/tier signaling

- The aggregate decoder processing speed (samples per seconds) conforms to common profile/level/tier signaling.

- The sum of the DPB size conforms to common profile/level/tier signaling

- The common DPB size conforms to common profile/level/tier signaling

A set of N Simultaneous Video Decoding Instances conforms to a common profile/level, if a set of up to N bitstreams encoded to be decodable by the HRD above, is decodable within the timing limits.

In addition, the simultaneous decoding of different video codecs is defined.

#### 6.6.3.2	Relevant Capabilities

Based on this definition, the following Simultaneous Video Decoding Instances are defined

AVC-UHD-Dec-4: The capability of supporting up to N=4 Simultaneous Video Decoding Instances with the aggregate capabilities of AVC-UHD-Dec

HEVC-UHD-Dec-4: The capability of supporting up to N=4 Simultaneous Video Decoding Instances with the aggregate capabilities of HEVC-UHD-Dec

UHD-Dec-4: The capability supporting up to N=4 Simultaneous Video Decoding Instances with either

-	the aggregate capabilities of AVC-UHD-Dec-4, or

-	the aggregate capabilities of HEVC-UHD-Dec-4, or

-	the capability of decoding up to 4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec

HEVC-8K-Dec-8: The capability of supporting up to N=8 Simultaneous Video Decoding Instances with the aggregate capabilities of HEVC-UHD-Dec

8K-Dec-8: The capability supporting up to N=8 Simultaneous Video Decoding Instances with either

-	the aggregate capabilities of AVC-UHD-Dec-4, or

-	the aggregate capabilities of HEVC-8K-Dec-8, or

-	the capability of decoding up to

-	8 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec or,

-	4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-UHD-Dec or HEVC-UHD-Dec,

### 6.6.4	Minimum Requirements

A MeCAR Client shall support

-	the AVC-UHD-Dec decoding capability.

-	the HEVC-UHD-Dec decoding capability.

-	the AVC-UHD-Dec-4 decoding capability.

-	the HEVC-UHD-Dec-4 decoding capability.

-	the UHD-Dec-4 decoding capability.

A MeCAR Client may support

-	the HEVC-8K-Dec decoding capability.

-	the 8K-Dec-8 decoding capability.