TSG-SA WG4 Meeting #125	S4-231548

Goteburg, 21– 25 August 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | - | rev | 1 | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Proposed clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 28/07/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Although the current versioning, TS 26.119 is in an early draft state (v0.2.0) as first version available beyond skeleton, some changes are proposed along with comments that target to improve the readability and structure of the specification in general. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some clauses are proposed to be renamed and new subclauses introduced by breaking existing larger clauses. Further clarification comments are provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The noted conflicts and issues will remain the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4.1, 4.1.1, 4.1.2, 4.1.2.1, 4.1.2.2, 4.1.3, 6.2.2, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 1	Scope

The present document defines the supported media formats, codecs, processing functions per AR device category. The present document addresses the interoperability gaps identified in the conclusions of TR 26.998 [3].

* * * Next Change * * * *

## 4.1	XR concepts

## 4.1.1	General

Extended Reality (XR) refers to a continuum of experiences combine real-a and- virtual combined environments in which the user is immersed through one or more devices capable of audio, visual and haptics rendering generated by computers through human-machine interaction. XR encompasses technologies associated with vVirtual rReality (VR), aAugmented rReality (AR) and mMixed rReality (MR) which constitute the so-called XR continuum. A detailed overview of definitions, concepts and background on XR and AR is provided in TR 26.928 [2] and TR 26.998 [3], respectively.

The clause 4 documents the core assumptions for a device capable of offering an XR experience. In the context of this document, such a device will be referred to an XR Device. An XR Device is assumed to have one or several displays, speakers, sensors, cameras, microphones, actuators, controllers and/or other peripherals that allow to create XR experiences, i.e. experiences for which the user interacts with the content presented in virtual world and/or augmented to the real-world. Example of XR Devices are AR Glasses, a VR/MR Head-Mounted Display (HMD) or a regular smartphone, etc.

An application which offers an XR experience by making use of the hardware capabilities, including media capabilities, of the XR Device it runs on as well as the network connectivity to retrieve the asset being used by the application is referred to as an XR Application. In the context of this specification, it is primarily assumed that access to the network is provided by 5G System functionalities.

To enable XR experiences, the hardware on an XR Device typically offers a set of functions to perform commonly required XR operations. These operations include, but are not limited to:

-	accessing controller/peripheral state,

-	getting current and/or predicted tracking positions and pose information of the user,

-	receiving or generating pre-rendered views of the scene for final presentation to the user, taking into account the latest user position and pose. Adaptation to the latest user position and pose is also referred to as warping.

* * * Next Change * * * *

### 4.1.2	XR Runtime

#### 4.1.2.1 General

XR Runtime provides a set of functionalities to XR applications including but not limited to peripheral management, runtime functions as tracking, SLAM, composition and warping etc. The functions are accessible to the XR Application via an API exposed by the XR Runtime referred to as the XR Runtime Application Programming Interface (API). The XR Runtime typically handles functionalities such as composition, peripheral management, tracking, Spatial Localization and Mapping (SLAM), capturing and audio-related functions. Further, it is assumed that the hardware and software capabilities of the XR Device are accessible through well-defined device APIs, and in particular the media capabilities are accessible through media APIs.

In the remainder of the specification, the XR prefix with Runtime or Application or other defined XR-prefixed terms may be omitted for better readability.

An overview of an XR Device logical components is shown in Figure 4.1.1-1.

Figure 4.1.2-1 Logical components of an XR Device

The primary scope of this specification is the definition of a minimum amount of media capabilities that an XR Application can rely on when deployed targeting a certain category of XR Device. Media capabilities include, but are not limited to, media encoders and decoders, parsing and writing of media encapsulation format, security functions, synchronization information, spatial alignment information, metadata formats, graphics capabilities, etc.

The logic and behaviour of the XR Application is not specified in this specification. The media capabilities may also be referenced as part of a Media Session Enabler as defined in TR 26.857 [4].

This specification relies on a hypothetical XR Runtime and its API in order to define the media capabilities. This way, different implementation of XR runtimes may be compatible with this specification. However, for the purpose of developing this specification, the minimal set of expected functionalities of the XR Runtime have been aligned with the core Khronos’ OpenXR specification [5]. Support for other XR Runtime environments are not precluded by this approach. Lastly, a mapping of general functionalities to OpenXR is provided in Annex B.

* * * Next Change * * * *

### 4.1.2.2	XR session and rendering loop (informative)

At startup, the XR Application creates an XR Session via the XR Runtime API and allocates the necessary resources from the available resources on the XR Device. Upon success, the XR Runtime begins the life cycle of the XR Session whose cycle is typically made of several states. The purpose of those states is to synchronise the rendering operations controlled by the XR Application with the display operations controlled by the XR Runtime. The rendering loop is thus a task jointly executed by the XR Runtime and the XR Application and synchronised via the states of the XR Session.

The XR Application is responsible of generating a rendered view of the scene from the perspective of the user. To this end, the XR Application produces XR Views which are passed to the XR Runtime at iterations of the rendering loop. The XR Views are generated for one or more poses in the scene for which the XR application can render images. From those views, the view corresponding to the viewer’s pose is typically called the primary view. There may be other XR Views defined in the scene, for instance for spectators.

The XR Views are configured based on the display properties of the XR Device. A typical head-mounted XR System has a stereoscopic view configuration, i.e. two views, while a handheld XR Device has a monoscopic view configuration, i.e. a single view. Other view configurations may exist. At the start of session, the XR Application configures the view type based on those device properties which remains the same for the duration of the XR Session.

A XR View may also comprise one more composition layers associated with an image buffer. Those layers are then composed together by the XR Runtime to form the final rendered images.

In addition to layers containing visual data, an XR View may be complemented with a layer provided depth information of the scene associated with this XR View. This additional information may help the XR Runtime to perform pose correction when generating the final display buffer. Another type of layer can be an alpha channel layer useful for blending the XR View with the real environment for video-see through XR devices, e.g. which is the case for AR applications running on smartphones.

For the XR Application to render the XR Views, the XR Runtime provides the viewer pose as well as projection parameters which are typically taken into account by applications to render those different XR Views. The viewer pose and projection parameters are provided for a given display time in the near future. The XR Runtime accepts repeated calls for prediction updates of the pose, which may not necessarily return the same result for the same target display time. Instead, the prediction gets increasingly accurate as the function is called closer to the given time for which a prediction is made. This allows an application to prepare the predicted views early enough to account for the amount of latency in the rendering while at the same time minimising the prediction error when pre-rendering the views.

In addition, the XR Application communicates with input devices in order to collect actions. Actions are created at initialization time and later used to request input device state, create action spaces, or control haptic events. Input action handles represent ‘actions’ that the application is interested in obtaining the state of, not direct input device hardware.

Figure 4.1.4-1 Rendering loop for visual data

* * * Next Change * * * *

### 4.1.3	XR runtime capabilities

Table 4.1.3-1 provides a summary of relevant capabilities for XR Runtimes. This table does not prescribe support for any specific capabilities, this aspect is addressed for each device category individually. A mapping of these high-level capabilities to OpenXR is provided in Annex A.

Table 4.1.3-1	XR Runtime Capabilities

| Capability | Description and Reference | Parameters | OpenXR (will be moved to Annex) |
| --- | --- | --- | --- |
| XR System Properties | An application can query the XR Runtime to retrieve information about the system such as a system identifier, graphics properties or tracking properties. | System identifier Tracking Properties Graphics Properties | xrGetSystemProperties |
| XR System Graphics Properties | Information on the graphics capabilities, namely the maximum image pixel height and width of the swapchain as well as the maximum number of composition layers | maxSwapchainImageHeight maxSwapchainImageWidth maxLayerCount | xrSystemGraphicsProperties minMaxLayerCount = 16 |
| XR System Tracking Properties | Information on the tracking capabilities, namely support of orientation and position tracking. | orientationTracking positionTracking | XrSystemTrackingProperties |
| Blend Mode | The supported blend modes of the XR System, see clause 4.1.4 | Opaque, additive, alpha_blend | XrEnvironmentBlendMode |
| Supported view configuration types | Supported primary view configurations by the XR System | Mono, Stereo, others | xrEnumerateViewConfigurations xrViewConfigurationType |
| View Configuration Properties | specifies properties related to rendering of an individual view within a view configuration | Recommended and maximum height/width and swapchain sample count | XrViewConfigurationView |
| Reference Space Type | XR Runtimes implement different reference spaces as described in clause 4.1.3 | View, Local, Stage, unbounded, user-defined | xrEnumerateReferenceSpaces |
| Spatial Range Boundaries | XR systems may have limited real world spatial ranges in which users can freely move around while remaining tracked | dimensions of an axis-aligned bounding box | xrGetReferenceSpaceBoundsRect |
| Swapchain Formats | Swapchain image format support by the runtime | For example R8G8B8A8 | xrEnumerateSwapchainFormats |
| Swapchain Images | number of images allocated to swapchain | For example 1 or 2 | xrEnumerateSwapchainImages |
| Projection Layer Type | Provides the supported layer type that is used in the projections for the layer | Projection Composition Layer: represents planar projected images, one rendered for each eye using a perspective projection. Quad Composition Layer: is useful for rendering user interface elements or 2D content on a planar area in the world. Cylinder Composition Layer: the XR runtime maps a texture stemming from a swapchain onto the inside of a cylinder section. Cube Composition Layer: consists of a cube map with 6 views to be rendered by the application. Equirectangular Composition Layer: consists of an equirectangular image that is mapped onto the inside of a sphere in the world. Depth Composition Layer: provides an extra composition layer to allow applications to submit depth maps to assist with the pose correction of projected images of a project layer. | XrStructureType |
| Frame rate |  |  |  |
| ACTIONS |  |  |  |

[Add a table of capabilities of the XR Runtime and what is expected to available and what is optional needs to be queried.

Basic concept of specification:

-	Capability query

-	[Editor’s note: Description of the pipelines, sensors, AR runtime, decoders… identify for what entities capabilities are defined]

Collected Requirements]

* * * Next Change * * * *

### 6.2.2	Pose prediction format

The split rendering client on the XR device may periodically transmit a set of pose predictions to the split rendering server. The type of the message shall be set to “urn:3gpp:split-rendering:v1:pose”.

Each predicted pose shall contain the associated predicted display time and an identifier of the XR space that was used for that pose.

Depending on the view configuration of the XR session, there could be different pose information for each view.

The payload of the message shall be as follows:

Table 5.1.2-1 - Pose Prediction Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| poseInfo | Object | 1..n | An array of pose information objects, each corresponding to a target display time and XR space. |
| displayTime | number | 1..1 | The time for which the current view poses are predicted. |
| xrSpace | number | 0..1 | An identifier for the XR space in which the view poses are expressed. The set of XR spaces are agreed on between the split rendering client and the split rendering server at the setup of the split rendering session. |
| viewPoses | Object | 0..n | An array that provides a list of the poses associated with every view. The number of views is determined during the split rendering session setup between the split rendering client and server, depending on the view configuration of the XR session. |
| pose | Object | 1..1 | An object that carries the pose information for a particular view. |
| orientation | Object | 1..1 | Represents the orientation of the view pose as a quaternion based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the quaternion. |
| y | number | 1..1 | Provides the y coordinate of the quaternion. |
| z | number | 1..1 | Provides the z coordinate of the quaternion. |
| w | number | 1..1 | Provides the w coordinate of the quaternion. |
| position | Object | 1..1 | Represents the location in 3D space of the pose based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the position vector. |
| y | number | 1..1 | Provides the y coordinate of the position vector. |
| z | number | 1..1 | Provides the z coordinate of the position vector. |
| fov | Object | 1..1 | Indicates the four sides of the field of view used for the projection of the corresponding XR view. |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |

* * * Next Change * * * *

### 6.2.3	Action format

Actions are grouped into action sets, which may be activated and deactivated during the lifetime of an XR session. The action sets and actions are negotiated at the start of the split rendering session.

The split rendering client may report any changes to action state as soon as it occurs by sending a message of the type “urn:3gpp:split-rendering:v1:action”.

The content of the action message type shall follow the following format:

Table 5.1.3-1 - Action Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| actionSets | Object | 1..n | An array of active action sets, for which there is at least an action that has a state change. |
| actions | Object | 1..n | An array of objects that conveys information about the actions of the parent action set. |
| identifier | string | 1..1 | A unique identifier of the action that was agreed upon during split rendering session setup. |
| subactionPath | string | 1..1 | The sub-action path for which the state has changed. It abstracts a binding between an action and the hardware input associated to it by the XR runtime. |
| state | object | 1..1 | The state of the action that had a change in state. |
| lastChangeTime | number | 1..1 | The timestamp of the last change to the state of this action. |
| currentStateBool | Bool | 0..1 | The current Boolean state of the action |
| currentStateNum | number | 0..1 | The current numerical state of the action. |
| currentStateVec2 | Array | 0..1 | An array of numerical state values for the action. |

* * * End of Changes * * * *