Source: 				 Interdigital Finland Oy

Title: 	[FS_AI4Media] Metadata for Split AI/ML operations

Agenda item: 	9.7

Document for:	Discussion and Agreement

#### 1 Discussion

The contribution proposes a set of metadata information communicated between the UE and the network when a first device (UE or Network endpoint) transmits data to a second device (respectively Network endpoint or UE) according to a split configuration.

When a device dynamically selects or reselects the model or a model subset to be executed on an input media segment, metadata may need to be passed alongside the output data to the other device.

#### 2 Proposed changes

--------------------------------------------- Begin change ------------------------------------------------------------------------

## 6.4	Metadata

### 6.4.1	Distributed/Federated Learning

### 6.4.2	Split AI/ML operations

Table 6.4-2-1 shows a set of metadata information communicated between the UE and the network when a first device (UE or Network endpoint) transmits data to a second device (respectively Network endpoint or UE) according to a split configuration.

When a device dynamically selects or reselects the model or a model subset to be executed on an input media segment, metadata may need to be passed alongside the output data transmitted to the other device.

Note: The metadata types below are descriptive and optional information that may be redundant.

| Metadata category | Metadata type | Definition | Metadata type description (Examples) |
| --- | --- | --- | --- |
| Model information (Information on model selected for split operations) | Model identifier | An identifier of the split model that sends intermediate data. Identifier may be a name, a number, a combination thereof, a hash value. The identifier is defined during the configuration stage. | Example: model_1, model_2 |
|  | Number of Parameters | Total number of parameters in the neural network | Example: 11 millions (ResNet 18) |
| Split Point information (Information on the selected split point corresponding to output data being sent) | Split point identifier | An identifier of the model split point in a description of a computing graph, may be generated by a neural network description language such as ONNX/NNEF. For example, this can be a layer number. The layer number may belong to limited list or set of identified layer numbers defined at the configuration stage. | Example: 10, Layer_10, L0-2 at points 75, 90, and 105 |
| Output data format (Format Information of output data being sent) | Data type | The type of output data format. Depending on the split configuration, the type of output data can be media data when the split point is configured to the first layer of the model, result data when the split point is set to the first layer, or otherwise intermediate data when the split point is an intermediate layer. The data type representation or identification (e.g., number) is defined at the configuration stage. | Example: media, intermediate data, result, or predefined type e.g... 1, 2, 3 |
|  | Tensor shape | The tensor shape(s) when the output is intermediate data. Tensor shape is a tuple of positive integers, where the size of the tuple represents the dimension of the tensor, and each value represents the size in each dimension. | Example: [1,64,64,64]. |
|  | Tensor structure information | The exact underlying tensor structure of the intermediate data tensors including the exact version of it. | Example: PyTorch 2.0, tensor flow v2.13.0, NumPy v1 .25 |
| Processing capabilities (Information on processing capabilities recommended or estimated for processing the other part of the split model) | Processing capabilities identifier | An identifier of the allocated or recommended processing capabilities. Processing capabilities may be negotiated at the configuration stage where each processing capabilities setting may be stored and identified with a processing capabilities identifier. | Example: |
|  | Parameters precision | Number of bits for storing the parameter. Additionally, use “I” for indicating an integer parameter and use “F” to indicate a floating-point number. | Example: If the proposed method uses 16-bit integer to represent a parameter, parameter precision may be “16 (I)”. |
|  | Multiply–accumulate (MAC) | Number of multiply–accumulate (MAC) operations in the worst case for the inference stage, where the multiply–accumulate operation is a common step that computes the product of two numbers and adds that product to an accumulator. | Example: ResNet-18 for (32x32 (1x1024) 36 million units |
|  | Temporary memory | Indicates the memory needed to store the output feature map for all intermediate layers (forward pass). | Example: Resnet18 (Conv2d output): 3.2MB |
| Framework information (Framework information on processing of part of the model) | Framework configuration set identifier | An identifier of the framework configuration set used for processing the first part of the model and required for processing the next part for compatibility reasons. A set of identifiers may have been defined during the configuration stage. | Example |
|  | Framework description | This identifies the framework configuration including the exact version (language, name, version, specific component name) used by the UE or the network producing intermediate data. | Example: Pytorch 2.0 |

Table 6.4-2-1: Metadata for split AI/ML operations

--------------------------------------------- End change ----------------------------------------------------------------------------------

#### 3 Proposal

We propose to add the text in a new clause §6.4.2 of the permanent document.