3GPP TSG-S4 Meeting #125	S4-231574

# Gothenburg, Sweden,  21. August. – 25. August 2023				revision of S4-231554

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0039 | rev | 4 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_EDGE_3] Correction of EAS Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_EDGE_3 |  |  |  |  |  | Date: |  |  | 26.7.2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EASDiscoveryTemplate is refering to the wrong specification and binding properties incorrectly. The Content Hosting Configuration is missing a reference to the related edge configuration. The EAS Registration and EAS Discovery procedures, as defined by SA6, allow the usage of the EASID as parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EASDiscoveryTemplate is corrected to point to the correct specifications. The EASID is added to the EASRequirements and the EASDiscoveryTemplate data types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Edge Computing feature cannot be used by 5G Media Streaming Services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3.1, 7.10.3.3, 11.2.3.2, C.3.5, C.3.9, C.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-231554, S4-231514, S4-231357 |  |  |  |  |  |  |  |  |

**** First Change ****

#### 7.6.3.1	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| name | String | 1..1 | A name for this Content Hosting Configuration. |
| ingestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider's origin server from which media resources will be ingested via interface M2d. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | Uri | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content ingest protocol. The set of supported protocols is defined in clause 8. |
| baseURL | AbsoluteUrl | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is ingested at reference point M2d for this ingest configuration. In the case of Pull-based content ingest (pull flag is set to True), the base URL shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled for this Content Hosting Configuration. A request received at reference point M4d is mapped by the 5GMSd AS to a URL at reference point M2d whose base is the value of this property. In the case of Push-based content ingest (pull flag is set to False), this property is populated by the 5GMSd AF and returned to the 5GMSd Application Provider to indicate the base URL to which content for this Content Hosting Configuration is to be published. |
| distributionConfigurations | Array(Object) | 1..1 | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| entryPoint | M1‌Media‌Entry‌Point | 0..1 | The Media Entry Point when this distribution configuration is used to describe a single content item. Omitted when this distribution configuration describes multiple content items. |
| relativePath | RelativeUrl | 1..1 | A relative path (i.e. without a scheme or any leading forward slash characters) to the resource for the Media Entry Point. The semantics are dependent on the value of ingestConfiguration.protocol, as specified in clause 8. The path shall be valid at reference point M2d when appended to the ingest base URL and at reference point M4d when appended to the distribution base URL. |
| contentType | String | 1..1 | The MIME content type of the Media Entry Point. Used by the 5GMS Client to select a distribution configuration. |
| profiles | Array(Uri) | 0..1 | An optional list of conformance profile identifiers associated with the Media Entry Point, each one expressed as a URI. A profile URI may indicate an interoperability point, for example. Used by the 5GMS Client to select a distribution configuration. If present, the array shall contain at least one item. |
| contentPreparationTemplateId | ResourceId | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4. |
| edgeResourcesConfigurationId | ResourceId | 0..1 | When present, the 5GMSd AS supporting this content distribution shall be deployed as a set of one or more EAS instances. |
| supplementary‌Distribution‌Networks | Array(<Distribution‌NetworkType, DistributionMode> | 0..1 | Specifies that the content for this distribution configuration is to be distributed via one of more supplementary networks. Each member of the array is a duple mapping a type of distribution network to a mode of distribution. The same DistributionNetworkType value shall appear at most once in the array. |
| canonicalDomainName | String | 0..1 | All resources of the current distribution shall be accessible through this default Fully Qualified Domain Name assigned by the 5GMSd AF. |
| domainNameAlias | String | 0..1 | The 5GMSd Application Provider may assign another Fully-Qualified Domain Name through which media resources are additionally accessible at M4d. This domain name is used by the 5GMSd AS to select an appropriate Server Certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. If this property is present, the 5GMSd Application Provider is responsible for providing in the DNS a CNAME record that resolves domainNameAlias to canonicalDomainName. |
| baseURL | AbsoluteUrl | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is made available to 5GMS Clients at reference point M4d for this distribution configuration. The value is chosen by the 5GMSd AF when the Content Hosting Configuration is provisioned. It is an error for the 5GMSd Application Provider to set this. |
| pathRewriteRules | Array(Object) | 0..1 | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the 5GMSd AS. If multiple rules match a particular resource's path, only the first matching rule, in order of appearance in this array, shall be applied. |
| requestPathPattern | String | 1..1 | A regular expression [5] against which the path part of each 5GMSd AS request URL, including the leading "/", and up to and including the final "/", shall be compared. (Any leaf path element following the final "/" shall be excluded from this comparison.) In the case of Pull-based ingest, the M4d download request path is used in the comparison. In the case of Push-based ingest, the M2d upload request path is used in the comparison. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
| mappedPath | String | 1..1 | A replacement for the portion of the 5GMSd AS request path that matches requestPathPattern. In the case of Pull-based ingest, ingestConfiguration.entryPoint is concatenated with the mapped path and any leaf path element from the original M4d download request to form the M2d origin request URL. In the case of Push-based ingest, canonicalDomainName (and, optionally, domainNameAlias) are concatenated with the mapped path and any leaf path element from the original M2d upload request to form the distribution URL(s) exposed over M4d. |
| cachingConfigurations | Array(Object) | 0..1 | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| cachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided cachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | Array(Integer) | 0..1 | The set of HTTP origin response status codes to which these cachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the CachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| geoFencing | Object | 0..N | Limit access to the content to the indicated geographic areas. |
| locatorType | Uri | 1..1 | The type of the locators shall be indicated using a fully-qualified term identifier URI from the controlled vocabulary urn:3gpp:5gms:‌locator-type, as specified in clause 7.6.4.6, or else from a vendor-specific vocabulary. |
| locators | Array(String) | 1..1 | Array of locators from which access to the resources is to be allowed. The format of the locator strings shall be determined by the value of locatorType, as specified in clause 7.6.4.6. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used by the 5GMSd AS to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token calculated over the portion of the M4d request URL that matches this pattern. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this distributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | ResourceId | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

**** Next Change ****

#### 7.10.3.3	EASRequirements type

The EASRequirements type is specified in table 7.10.3.3-1 below:

Table 7.10.3.3-1: Definition of EASRequirements type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| easProviderIds | array(string) | 0..1 | The set of acceptable providers of 5GMS EAS instances associated with this Provisioning Session. If empty, EAS instances from any provider are acceptable. |
| easId | string | 0..1 | The Application Identifier (e.g., in the form of a URI or Fully-Qualified Domain Name) of a set of EAS instances, or of a particular EAS instance associated with this Provisioning Session. |
| easType | string | 0..1 | The type of 5GMS EAS instances associated with this Provisioning Session. |
| easFeatures | array(string) | 0..1 | 5GMS EAS service features required to be supported by EAS instances associated with this Provisioning Session. If empty, 5GMS EAS instances of the specified easType with any feature set are acceptable. |
| serviceKpi | EASServiceKPI | 0..1 | Service characteristics required to be satisfied by 5GMS AS EAS instances associated with this Provisioning Session. If absent, 5GMS EAS instances with any service characteristics are acceptable. |
| serviceArea | Geographical‌Service‌Area | 0..1 | The list of geographical areas that 5GMS EAS instances associated with this Provisioning Session are required to serve. If absent, 5GMS EAS instances shall serve all geographical areas whenever possible. |
| service‌Availability‌Schedule | array(Scheduled‌Communication‌Time) | 0..1 | The required availability schedule for 5GMS EAS instances associated with this Provisioning Session. If omitted, 5GMS EAS instances are required to be available at all times. |
| service‌Continuity‌Scenarios | array(ACRScenario) | 0..1 | The Application Context Relocation scenarios that 5GMS EAS instances associated with this Provisioning Session are required to support for service continuity. If omitted 5GMS EAS instances are not required to support service continuity across EAS relocation. |
| NOTE: Data types ScheduledCommunicationTime, GeographicalServiceArea, EASServiceKPI, and ACRScenario are defined in TS 29.558 [43]. |  |  |  |

**** Next Change ****

#### 11.2.3.2	EASDiscoveryTemplate type

Table 6.4.3.10-1  Definition of EASDiscoveryTemplate type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| easId | string | 0..1 | The application identifier of the EAS, e.g. FQDN, URI. If omitted, any 5GMS EAS instance matching the other criteria specified in the template are acceptable. Corresponding to EasCharacteristics.easId, as specified in clause 6.3.5.2.7 of TS 24.558 [43]. |
| easType | string | 0..1 | If present, a non-empty string indicating the type of 5GMS EAS required to support media streaming sessions in the scope of this discovery template. Corresponding to EasCharacteristics.easType, as specified in clause 6.3.5.2.7 of TS 24.558 [43]. |
| easProviderIds | array(string) | 0..1 | The set of acceptable EAS provider identifiers. If omitted, 5GMS EAS instances of the specified easType from any provider are acceptable. Corresponding to EasCharacteristics.easProvId, as specified in clause 6.3.5.2.7 of TS 24.558 [43]. |
| easFeatures | array(string) | 0..1 | The required service features for the EAS to serve this session. If omitted, 5GMS EAS instances of the specified easType with any feature set are acceptable. Corresponding to EasCharacteristics.svcFeats, as specified in clause 6.3.5.2.7 of TS 24.558 [43]. |
| NOTE: At least one of the properties shall contain a value. |  |  |  |

**** Next Change ****

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 2.3.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V17.6.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'The regular expression' type: string responses: '200': # OK description: 'Content Purged' content: application/json: schema: description: 'The aggregate number of cache entries purged in all 5GMSd AS instances distributing content for the requested Provisioning Session.' type: integer minimum: 1 '204': # No Content description: 'No Content Purged' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '413': # Payload Too Large $ref: 'TS29571_CommonData.yaml#/components/responses/413' '414': # URI Too Long $ref: 'TS29571_CommonData.yaml#/components/responses/414' '415': # Unsupported Media Type $ref: 'TS29571_CommonData.yaml#/components/responses/415' '422': # Unprocessable Entity (e.g. syntactically invalid regular expression in request body) description: 'Unprocessable Entity' content: application/problem+json: schema: $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails' '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: schemas: IngestConfiguration: type: object description: 'A configuration for content ingest.' properties: pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' M1MediaEntryPoint: description: "A typed entry point for downlink or uplink media streaming." type: object required: - relativePath - contentType properties: relativePath: $ref: 'TS26512_CommonData.yaml#/components/schemas/RelativeUrl' contentType: type: string profiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 PathRewriteRule: type: object description: 'A rule to manipulate URL paths.' required: - requestPathPattern - mappedPath properties: requestPathPattern: type: string mappedPath: type: string CachingConfiguration: type: object description: 'A content caching configuration.' required: - urlPatternFilter properties: urlPatternFilter: type: string cachingDirectives: type: object required: - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object description: 'A content distribution configuration.' properties: entryPoint: $ref: '#/components/schemas/M1MediaEntryPoint' contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' edgeResourcesConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 urlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' supplementaryDistributionNetworks: type: array items: type: object description: "A duple tying a type of supplementary distribution network to its distribution mode." required: - distributionNetworkType - distributionMode properties: distributionNetworkType: $ref: '#/components/schemas/DistributionNetworkType' distributionMode: $ref: '#/components/schemas/DistributionMode' # Schema for the resource itself ContentHostingConfiguration: type: object description: "A representation of a Content Hosting Configuration resource." required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration' DistributionNetworkType: description: "Type of distribution network." anyOf: - type: string enum: [NETWORK_EMBMS] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DistributionMode: description: "Mode of content distribution." anyOf: - type: string enum: [MODE_EXCLUSIVE, MODE_HYBRID, MODE_DYNAMIC] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

**** Next Change ****

## C.3.9	M1_EdgeResourcesProvisioning API

openapi: 3.0.0 info: title: M1_EdgeResourcesProvisioning version: 2.1.2 description: ¦ 5GMS AF M1 Edge Resources Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_EdgeResourcesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Edge Resources Provisioning' externalDocs: description: 'TS 26.512 V17.6.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 7.10. paths: /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createEdgeResourcesConfiguration summary: 'Create an Edge Resources Configuration within the scope of the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '201': description: 'Edge Resources Configuration Created' headers: Location: description: 'URL of the newly created Edge Resources Configuration.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations/{edgeResourcesConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: edgeResourcesConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Edge Resources Configuration.' get: operationId: retrieveEdgeResourcesConfiguration summary: 'Retrieve the Edge Resources Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' put: operationId: updateEdgeResourcesConfiguration summary: 'Update an Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '204': description: 'Updated Edge Resources Configuration' '404': description: 'Not Found' patch: operationId: patchEdgeResourcesConfiguration summary: 'Patch the Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Edge Resources Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '200': description: 'Patched Edge Resources Configuration' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' '404': description: 'Not Found' delete: operationId: destroyEdgeResourcesConfiguration responses: '204': description: 'Destroyed Edge Resources Configuration' '404': description: 'Not Found' components: schemas: EdgeResourcesConfiguration: type: object description: 'A representation of an Edge Resources Configuration resource.' required: - edgeResourcesConfigurationId - edgeManagementMode - easRequirements properties: edgeResourcesConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' edgeManagementMode: $ref: '#/components/schemas/EdgeManagementMode' eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easRequirements: $ref: '#/components/schemas/EASRequirements' easRelocationRequirements: $ref: '#/components/schemas/M1EASRelocationRequirements' M1EASRelocationRequirements: type: object description: 'Relocation requirements of an EAS.' required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' maxResponseTimeDifference: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' EASRequirements: type: object description: 'Requirements of an EAS.' properties: easProviderIds: type: array items: type: string minItems: 1 easId: type: string easType: type: string easFeatures: type: array items: type: string minItems: 1 serviceKpi: $ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASServiceKPI' serviceArea: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/GeographicalServiceArea' serviceAvailabilitySchedule: type: array items: $ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime' minItems: 1 serviceContinuityScenarios: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' minItems: 1 serviceContinuitySupport: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' minItems: 1 EdgeManagementMode: description: 'The management mode of an EAS.' anyOf: - type: string enum: [EM_AF_DRIVEN, EM_APP_DRIVEN] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

**** Next Change ****

## C.4.1	M5_ServiceAccessInformation API

openapi: 3.0.0 info: title: M5_ServiceAccessInformation version: 2.2.2 description: ¦ 5GMS AF M5 Service Access Information API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ServiceAccessInformation description: '5G Media Streaming: Media Session Handling (M5) APIs: Service Access Information' externalDocs: description: 'TS 26.512 V17.6.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /service-access-information/{provisioningSessionId}: parameters: - name: provisioningSessionId description: 'The resource identifier of an existing Provisioning Session.' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveServiceAccessInformation summary: 'Retrieve the Service Access Information resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ServiceAccessInformationResource' '404': description: 'Not Found' components: schemas: M5MediaEntryPoint: description: "A typed entry point for downlink or uplink media streaming." type: object required: - locator - contentType properties: locator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' contentType: type: string profiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 ServerAddresses: description: "A set of application endpoint addresses." type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' minItems: 1 ServiceAccessInformationResource: description: "A representation of a Service Access Information resource." type: object required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' streamingAccess: type: object properties: entryPoints: type: array items: $ref: '#/components/schemas/M5MediaEntryPoint' eMBMSServiceAnnouncementLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' clientConsumptionReportingConfiguration: type: object required: - serverAddresses - locationReporting - samplePercentage properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' serverAddresses: $ref: '#/components/schemas/ServerAddresses' locationReporting: type: boolean accessReporting: type: boolean samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' dynamicPolicyInvocationConfiguration: type: object required: - serverAddresses - validPolicyTemplateIds - sdfMethods properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' validPolicyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 0 sdfMethods: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/SdfMethod' minItems: 0 externalReferences: type: array items: type: string minItems: 1 clientMetricsReportingConfiguration: type: array items: type: object required: - serverAddresses - scheme - samplePercentage - urlFilters - metrics properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 0 metrics: type: array items: type: string networkAssistanceConfiguration: type: object required: - serverAddresses properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' clientEdgeResourcesConfiguration: type: object required: - easDiscoveryTemplate properties: eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easDiscoveryTemplate: $ref: '#/components/schemas/EASDiscoveryTemplate' easRelocationRequirements: $ref: '#/components/schemas/M5EASRelocationRequirements' M5EASRelocationRequirements: description: 'Relocation requirements of an EAS.' type: object required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' EASDiscoveryTemplate: description: 'A template for discovering an EAS instance .' type: object properties: easId: type: string easType: type: string easProviderIds: type: array items: type: string minItems: 1 serviceFeatures: type: array items: type: string minItems: 1

**** Last Change ****