3GPP TSG-S4 Meeting #126	S4-231779

Chicago, US,  13th - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26. | CR | 10 | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Scene Description for MeCAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc., InterDigital Europe, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 7th November 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Scene description is a key enabler for immersive experiences. Capabilities for the support of scene description are expected to be defined in MeCAR clause 7.3. MPEG-I Scene Description has been described in the MeCAR PD and is already adopted in different 3GPP specifications for scene description. XR device types 3 and 4 need this capability to offer immersive XR experiences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR specifies the capability for scene description in MeCAR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for 3D scenes will be avilable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

…

[9]		ISO/IEC 23090-14:2023, Information technology, Coded representation of immersive media, Part 14: Scene description

[10]	ISO/IEC 23090-14:2023/Amd 1, Information technology, Coded representation of immersive media, Part 14: Scene description

[11]	ISO/IEC 23090-14:2023/CD Amd 2, Information technology, Coded representation of immersive media, Part 14: Scene description

2nd Change

## 7.3	Scene processing capabilities

SD-Rendering-Core: the capability to process and render glTF2.0 scenes with the following restrictions:

- the glTF 2.0 file may be an independent JSON file or an encapsulated GLB file

- An aggregate geometry complexity, including all mesh primitives of the scene, of at least 100k faces and/or points

- A total of at least 20 materials using static image textures

- Support for glTF 2.0 animations and skinning

- Support for the following extensions: KHR_ligths_punctual, KHR_materials_specular

SD-Rendering-Ext1: the capability to process and render the a document that conforms to the requirements in SD-Rendering-Core. In addition, the capability to process and render MPEG-I Scene Description [9-11] documents as specified in [9-11] with restrictions as describe here.

In addition, this capability includes the simultaneous decoding of scene object components and the rendering of scenes with at least the following characteristics:

- At least 4 materials using video textures, which implies conformance to AVC-HEVC-4-Dec

- A total of at least 5 object audio sources

This capability shall include the capability to process the following glTF 2.0 extensions defined in [9-11]:

- The MPEG_media extension: used to reference the media streams.

-	The MPEG_accessor_timed and the MPEG_buffer_circular: used to bind timed media.

-	The MPEG_texture_video: used to define video textures.

- 	The MPEG_audio_spatial: used to define spatial audio in the scene.

-	The scene description update mechanism as defined in clause 5.2.4 of [9].

SD-Rendering-Ext2: the capability to process the following glTF 2.0 extensions defined in [9-11]:

- The AR anchoring and the MPEG_anchor extension for support of AR,

- The EXT_ligths_image_based and the MPEG_lights_texture_based for support of lighting in the scene.

In addition, gener

ating scene description updates may be supported to allow the device to contribute to a scene description of a dynamic shared scene.

3rd Change

## 10.2	Device type 1: Thin AR glasses

[Editor’s note: For each device type, it is expected that rendering capabilities will also be added before completing the TS.]

### 10.2.1	General

The thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 10.2.2	XR System support

An XR Device complying to the thin AR glasses device has an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.2.5	Scene processing capabilities support

TBD

3rd Change

## 10.3	Device type 2: AR glasses

### 10.3.1	General

The AR glasses device type represents a type of device which is considered to have higher computation power compared to the thin AR glasses device type. As a result, this device type has higher rendering capacities and is generally expected to be capable of rendering scenes without external support, even though remote rendering is not precluded to lower the power consumption on the device or enable the display of scenes beyond the device’s rendering capability. Lastly, the AR glasses offers AR experiences to the user via optical see-through display.

### 10.3.2	XR System support

An XR Device complying to the AR glasses device type has offers XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Value 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.3.5	Scene processing capabilities support

An XR Device complying to the AR glasses device type shall support at least the following capabilities:

- SD-Rendering-Core

An XR Device complying to the AR glasses device type may support the following capabilities:

- SD-Rendering-Ext1

4th Change

## 10.4	Device type 3: XR phone

### 10.4.1	General

The XR phone device type represents a type of device which corresponds to a smartphone with capacities and resources sufficient to offer AR experiences. As a result, this device type is capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.4.2	XR System support

An XR Device complying to the XR phone device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.4.5	Scene processing capabilities support

An XR Device complying to the XR phone device type shall support at least the following capabilities:

- SD-Rendering-Core

- SD-Rendering-Ext1

An XR Device complying to the XR phone device type should support the following capabilities:

- SD-Rendering-Ext2

5th Change

## 10.5	Device type 4: XR HMD

### 10.5.1	General

The XR HMD device type represents a type of device which corresponds to HMDs capable of offering at least AR experiences but not precluding other types of XR experiences. This device type is expected to be capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.5.2	XR System support

An XR Device complying to the XR HMD device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' or values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE	For the definition of those capabilities, please refer to clause 4.1.3.

[Editor’s note: This list of capabilities is a starting point and more can be added after being defined in clause 4.1.3]

### 10.5.5	Scene processing capabilities support

An XR Device complying to the XR HMD  device type shall support at least the following capabilities:

- SD-Rendering-Core

- SD-Rendering-Ext1

An XR Device complying to XR HMD  device type should support the following capabilities:

- SD-Rendering-Ext2