3GPP TSG-S4 Meeting # 127-bis-e	S4-240685

Online,  April 8th - 12th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Usage of Server Certificates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IBACS |  |  |  |  |  | Date: |  |  | 2nd April 2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This pCR clarifies the usage of the scene description for AR calls. It removes the clause on split rendering as it is considered out of scope and will be handled in Rel-19 under the SR_IMS work item. It also defines the format for the pose and action information. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4, 6.5, 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 6.4	Scene descriptions

An AR-MTSI client in terminal shall comply with the capabilities requirements for scene description as described in clause 10 of TS26.119 for their respective device type.

When used in an AR call, the scene description shall be the entry point to the AR session (after establishment of a regular call/conference) and shall be exchanged over the bootstrap data channel. Based on the information in the scene description, the UE may decide to add additional media streams through a re-INVITE. However, at least the RTP session for the voice of every participant should be present and should be linked to an audio source in the scene description.

Each participant should be associated with their own camera node, identified through the node name and the SDP through the “sd-nodes” attribute.

The “sd-nodes” attribute shall conform to the following ABNF syntax:

sd-nodes = “a=sd-nodes:” SP participant-label SP 1*(node-name [“,”])

The AR MF/MRF should apply pose updates from the received pose information of each participant to their respective camera nodes, as negotiated by the SDP sd-nodes attribute.

A scene description of an AR session is sent from the AR MF/MRF to the AR-MTSI clients in terminal.

An AR MF/MRF shall support:

- The capability to generate a scene description file that conforms to the SD-Rendering-glTF-Core capability as defined in [TS26.119].

- The capability to generate and update a scene description file that conforms to the SD-Rendering-glTF-Ext1 as specified in [TS26.119].

An AR MF/MRF may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Ext2 capabilities as defined in [TS26.119].

An AR MF/MRF may additionally support the generation of scene description files and updates that conform to the SD-Rendering-glTF-Interactive capabilities as defined in [TS26.119].

In addition, an AR MF/MRF shall support the referencing of RTP streams in the scene description through the MPEG_media extension as defined in [TS26.962] clause 7.3. The external media shall be RTP media streams supported by an AR-MTSI client and signalled in the SDP.

The scene description shall be sent by the AR MF/MRF to the AR-MTSI client in terminal over the data channel. The type of the message shall be set to “urn:3gpp:ar:v1:sd”.

2nd Change

3rd Change

## 6.X Metadata Support

Both AR-MTSI client and AR MF/MRF shall support the exchange of pose information over the data channel. The pose information shall be formatted according to [TS26.119] clause 12.2.

Both AR-MTSI client and AR MF/MRF should support the exchange of action information over the data channel. The action information shall be formatted according to [TS26.119] clause 12.3.