# 3GPP TSG-SA4 Meeting Sophia-Antipolis,

# France, 29th Jan 2024 - 2nd Feb 2024														S4-240123

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | - | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Improvements to TS 26.119 addressing general and editorial comments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, Xiaomi, Tencent, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 2024-01-20 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 26.119 needs general and editorial improvements as agreed in S4aV230116 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New Table of contents with more logical chapters - References fixed - Terms and definitions extended - Editorial improvements in clause 4, 5, 6, 7, 8, 9 and 10 - scene description in separate section - Metadata in separate section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Technical Specification is harder to read and understand, incorrect references and definitions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All except annex and scope |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Abbreviations	8

4	XR Concepts and Device Types	10

4.1	XR concepts	10

4.1.1	XR Device	10

4.1.2	XR Application	10

4.1.3	XR Runtime	10

4.2	Media pipelines and rendering loop 	13

4.3	AR Device Types	13

4.3.1	Device type 1: Thin AR glasses	13

4.3.2	Device type 2: AR glasses	14

4.3.3	Device type 3: XR phone	14

4.3.4	Device type 4: XR Head Mounted Display (HMD)	14

5	Device reference architecture and interfaces	14

5.1	Architecture	14

5.2	Description of the functional blocks	15

5.3	Interfaces and APIs	15

6	General system functions and capabilities	16

6.1	XR system capabilities	16

7	Visual functions and capabilities	20

7.1	Decoding capabilities	21

7.1.1	Single decoder instance	21

7.1.2	Concurrent decoding capabilities	21

7.2	Encoding capabilities	23

7.2.1	Single encoder instance	23

8	Audio functions and capabilities	23

8.1	Audio/Speech Decoding	23

8.2	Audio/Speech Encoding	24

9	Scene processing capabilities	24

9.1	General	24

9.2	glTF-based Scene Description capabilities	24

10	Device types and media profiles	26

10.1	Introduction	26

10.2	Device type 1: Thin AR glasses	26

10.2.1	General	26

10.2.2	XR System support	26

10.2.3	Video capabilities support	27

10.2.4	Audio/Speech capabilities support	27

10.2.5	Scene Description capabilities support	27

10.3	Device type 2: AR glasses	27

10.3.1	General	27

10.3.2	XR System support	28

10.3.3	Video capabilities support	28

10.3.4	Audio/Speech capabilities support	28

10.3.5	Scene Description capabilities support	29

10.4	Device type 3: XR phone	29

10.4.1	General	29

10.4.2	XR System support	29

10.4.3	Video capabilities support	29

10.4.4	Audio/Speech capabilities support	30

10.4.5	Scene Description capabilities support	30

10.5	Device type 4: XR HMD	31

10.5.1	General	31

10.5.2	XR System support	31

10.5.3	Video capabilities support	31

10.5.4	Audio/Speech capabilities support	32

10.5.5	Scene Description capabilities support	32

11	QoE Metrics	32

11.1	Metrics and Observation Points	33

11.1.1	Overview	33

11.1.2	Observation Point 1: XR Runtime information	33

11.1.3	Observation Point 2	34

11.1.4	Observation Point 3	34

11.1.5	Observation Point 4	34

11.2	Metrics Definitions	34

11.2.1	Latency metrics	34

12	Metadata formats	36

12.1	General	36

12.2	Pose information format	36

10.3	Action format	37

10.4	Available Visualization Space format	38

A.1	Introduction	40

B.1	Introduction	40

B.2	Capability mapping to OpenXR	40

B.2.1	Mapping overview	40

B.2.2	XR views and rendering loop	43

B.2.3	Available Visualization Space implementation	45

B.2.3.1	Using OpenXR_XR_FB	45

B.2.3.2	Using xrComputeNewSceneMSFT	45

[B.3	Capability mapping to WebXR]	45

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document provides technologies for the deployment of Augmented Reality (AR) and eXtended Reality services (XR) as defined in TR 26.928 [2] and the execution of Augmented Reality applications on targeted devices such as those identified in TR 26.998 [3].

On the spectrum of eXtended Reality (XR) experiences, Augmented Reality overlay virtual information on top of the user’s perception of the real environment. Those virtual and real components of the scene seamlessly blend together from the user’s perspective. Additionally, some AR experiences can enable interactivity between the user and the virtual components of the scene.

In the present document, the focus lies in the definition of the media capabilities for AR devices, including media format encapsulation capabilities, media codec capabilities, processing function capabilities.  The related minimum required performances for different device types are also defined.

# 1	Scope

The present document defines the supported media formats, codecs, processing functions for XR Devices in UE per XR device type category. The present document addresses the interoperability gaps identified in the conclusions of TR 26.998 [3].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[3]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[4]	3GPP TR 26.857: "5G Media Service Enablers".

[5]	Khronos, "The OpenXR Specification", https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html.

[6]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[7]	ITU-T Recommendation H.264 (08/2021): "Advanced video coding for generic audiovisual services".

[8]	ITU-T Recommendation H.265 (08/2021): "High efficiency video coding".

[9]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[10]	ISO/IEC 12113:2022 Information technology Runtime 3D asset delivery format Khronos  glTF™2.0

[11]                      ISO/IEC 23090-14:2023 Information technology Coded representation of immersive media Part 14: Scene description

[12]                      ISO/IEC 23090-14:2023/Amd 1:2023 Information technology Coded representation of immersive media Part 14: Scene description

[13]                      ISO/IEC 23090-14:2023/DAmd 2 Information technology Coded representation of immersive media Part 14: Scene description

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]

AR Glasses:

Composition: TBD

Media Session Handler: a set of functions responsible for handling all 5G control plane operations, such as requesting network assistance, discovering and allocating edge resources, etc.

Presentation Engine: a set of composite renderers, rendering the component of the scenes,

Reference Points: geometric points whose position is identified both mathematically and physically.

SD-Rendering: Scene Description Rendering

Swapchain image: a queue of images that are waiting to be presented to the screen

XR Application: a software application that integrates audio-visual content into the user’s real-world environment

XR Device: a device capable of offering an XR experience.

XR Head Mounted Display:

XR Runtime: Set of functions provided by the XR Device to the XR Application in order to create XR experiences.

XR Runtime API: the API to communicate with an XR Runtime

XR Scene Manager: a set of functions that supports the application in arranging the logical and spatial representations

XR Session: an application’s intention to present XR content to the user.

XR Source Management: management of data sources provided through the XR runtime.

XR Phone: TBD

XR System: a collection of resources and capabilities from the XR Runtime exposed to the XR Application for the duration of the XR Session.

XR View: a rendered view of the scene generated by the XR Application and passed on to the XR Runtime during a running XR Session

XR Space: TBD

Warping: TBD

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AR	Augmented Reality

AAC	Advanced Audio Coding

AVC	Advanced Video Coding

CPB	Coded Picture Buffer

DPB	Decoded Picture Buffer

ELD	Enhanced Low Delay

EVS	Enhanced Voice Services

glTF	graphics library Transmission Format

GLB	glTF Binary

HEVC	High Efficiency Video Coding

HMD	Head-Mounted Display

HRD	Hypothetical Reference Decoder

HSS	Hypothetical Stream Scheduler

IVAS	Immersive Voice and Audio Services

JSON	JavaScript Object Notation

MPEG	Moving Picture Experts Group

MR	Mixed Reality

OP	Observation Point

MPEG SD	MPEG Scene Description

SLAM	Simultaneous localisation and mapping

UE	User Equipment

VCL	Video Coding Layer

VR	Virtual Reality

XR		eXtended Reality

XR API	XR Runtime Application Programming Interface

# 4	XR Concepts and Device Types

## 4.1	XR concepts

### 4.1.1	XR Device

Extended Reality (XR) refers to a continuum of experiences combine real-a and- virtual combined environments in which the user is immersed through one or more devices capable of audio, visual and haptics rendering generated by computers through human-machine interaction. XR encompasses technologies associated with Virtual Reality (VR), Augmented Reality (AR) and Mixed Reality (MR) which constitute the so-called XR continuum. A detailed overview of definitions, concepts and background on XR and AR is provided in TR 26.928 [2] and TR 26.998 [3], respectively.

Clause 4.1 of 26.928 [2] defines the use the terms Augmented Reality, Virtual Reality, Mixed Reality and eXtended Reality as used throughout this document.

An XR device is a device capable of offering an XR experience. An XR Device is assumed to have one or several displays, speakers, sensors, cameras, microphones, actuators, controllers and/or other peripherals that allow to create XR experiences, i.e. experiences for which the user interacts with the content presented in virtual world and/or augmented to the real-world. Example of XR Devices are AR Glasses, a VR/MR Head-Mounted Display (HMD) or a regular smartphone, etc.

### 4.1.2	XR Application

An application which offers an XR experience by making use of the hardware capabilities, including media capabilities, of the XR Device it runs on as well as the network connectivity to retrieve the asset being used by the application is referred to as an XR Application. In the context of this specification, it is primarily assumed that access to the network is provided by 5G System functionalities.

To enable XR experiences, the hardware on an XR Device typically offers a set of functions to perform commonly required XR operations. These operations include, but are not limited to:

-	accessing controller/peripheral state,

-	getting current and/or predicted tracking positions and pose information of the user,

-	receiving or generating pre-rendered views of the scene for final presentation to the user, taking into account the latest user position and pose. Adaptation to the latest user position and pose is also referred to as warping.

### 4.1.3	XR Runtime

#### 4.1.3.1	General

XR Runtime provides a set of functionalities to XR applications including but not limited to peripheral management, runtime functions as tracking, SLAM, composition and warping etc. The functions are accessible to the XR Application via an API exposed by the XR Runtime referred to as the XR Runtime Application Programming Interface (XR API). The XR Runtime typically handles functionalities such as composition, peripheral management, tracking, Spatial Localization and Mapping (SLAM), capturing and audio-related functions. Further, it is assumed that the hardware and software capabilities of the XR Device are accessible through well-defined device APIs, and in particular the media capabilities are accessible through media APIs.

An overview of an XR Device logical components is shown in Figure 4.1.2-1.

Figure 4.1.2-1 Logical components of an XR Device

This specification relies on a hypothetical XR Runtime and its API in order to define the media capabilities. This way, different implementation of XR runtimes may be compatible with this specification. However, for the purpose of developing this specification, the minimal set of expected functionalities of the XR Runtime has been aligned with the core Khronos’ OpenXR specification [5]. Support for other XR Runtime environments is not precluded by this approach. Lastly, a mapping of general functionalities to OpenXR is provided in Annex B.

#### 4.1.3.2	XR session and rendering loop using XR Runtime (informative)

At startup, the XR Application creates an XR Session via the XR Runtime API and allocates the necessary resources from the available resources on the XR Device. Upon success, the XR Runtime begins the life cycle of the XR Session whose cycle is typically made of several states. The purpose of those states is to synchronise the rendering operations controlled by the XR Application with the display operations controlled by the XR Runtime. The rendering loop is thus a task jointly executed by the XR Runtime and the XR Application and synchronised via the states of the XR Session.

The XR Application is responsible of generating a rendered view of the scene from the perspective of the user. To this end, the XR Application produces XR Views which are passed to the XR Runtime at iterations of the rendering loop. The XR Views are generated for one or more poses in the scene for which the XR application can render images. From those views, the view corresponding to the viewer’s pose is typically called the primary view. There may be other XR Views defined in the scene, for instance for spectators.

The XR Views are configured based on the display properties of the XR Device. A typical head-mounted XR System has a stereoscopic view configuration, i.e. two views, while a handheld XR Device has a monoscopic view configuration, i.e. a single view. Other view configurations may exist. At the start of session, the XR Application configures the view type based on those device properties which remains the same for the duration of the XR Session.

A XR View may also comprise one more composition layers associated with an image buffer. Those layers are then composed together by the XR Runtime to form the final rendered images.

In addition to layers containing visual data, an XR View may be complemented with a layer provided depth information of the scene associated with this XR View. This additional information may help the XR Runtime to perform pose correction when generating the final display buffer. Another type of layer can be an alpha channel layer useful for blending the XR View with the real environment for video-see through XR devices, e.g. which is the case for AR applications running on smartphones.

For the XR Application to render the XR Views, the XR Runtime provides the viewer pose as well as projection parameters which are typically taken into account by applications to render those different XR Views. The viewer pose and projection parameters are provided for a given display time in the near future. The XR Runtime accepts repeated calls for prediction updates of the pose, which may not necessarily return the same result for the same target display time. Instead, the prediction gets increasingly accurate as the function is called closer to the given time for which a prediction is made. This allows an application to prepare the predicted views early enough to account for the amount of latency in the rendering while at the same time minimizing the prediction error when pre-rendering the views.

In addition, the XR Application communicates with input devices in order to collect actions. Actions are created at initialization time and later used to request input device state, create action spaces, or control haptic events. Input action handles represent ‘actions’ that the application is interested in obtaining the state of, not direct input device hardware.

Figure 4.1.4-1 Rendering loop for visual data

### XR system capabilities

The XR Runtime comprises functions and hardware components present on the XR Device. However, those functions and hardware components are not directly exposed to the XR Application. Instead, the XR Runtime offers its functions and hardware components via an XR System. A single XR Runtime may expose more than one XR Systems for targeting different purposes, e.g., a handheld device may have two XR Systems, one when the user holds the device and one when the device is inserted into an HMD. At the start of the XR Application, the XR Application is expected to query what XR Systems are available on the XR Device and select one of them to create the XR Session.

Table 4.1.3-1 provides capabilities for XR Runtimes exposed through an XR System. This table does not prescribe support for any specific capabilities of an XR System. The support of XR System capabilities is defined per device category in clause 10. A mapping of these high-level capabilities to XR frameworks are provided in Annex B.

Table 4.1.3-1	XR System capabilities

[Editor’s note: Add a table of capabilities of the XR Runtime and what is expected to be available and what is optional needs to be queried.

Basic concept of specification:

-	Capability query

-	[Editor’s note: Description of the pipelines, sensors, AR runtime, decoders… identify for what entities capabilities are defined]

Collected Requirements]

## 4.2	Media pipelines and rendering loop

[Editor’s note: Description of the pipelines, sensors, AR runtime, decoders… identify for what entities capabilities are defined]

In the context of this specification, media to be rendered and displayed by the XR Device through the XR Runtime is typically available in a compressed form on the device. In contrast, media is accessed using a 5G System, decoded in the device using media capabilities, and the decoded media is rendered to then be provided through swapchains to the XR Runtime as shown in Figure 4.2.1-1.

Figure 4.2.1-1 Media pipelines: Access, decoding and rendering

The rendering function is responsible to adapt the content to be presentable by the XR Runtime by making use of a rendering loop and using swapchains. The application configures pipeline of different processes, namely the media access, the decoding and the rendering. The static information provided to the rendering step needs to be sufficient to configure the number of layers as well as each layer appropriately including:

-	View configuration

-	Blend modes

-	XR spaces

-	swap chain formats and images

-	projection layer types

[Frame rates: https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html#XR_FB_display_refresh_rate

Rendering supported by the XR runtime

-	Visual

-	Audio]

## 4.3	AR Device Types

### 4.3.1	Device type 1: Thin AR glasses

The thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 4.3.2	Device type 2: AR glasses

The AR glasses device type represents a type of device which is considered to have higher computation power compared to the thin AR glasses device type. As a result, this AR device type has higher rendering capacities and is generally expected to be capable of rendering scenes without external support, even though remote rendering is not precluded to lower the power consumption on the device or enable the display of scenes beyond the device’s rendering capability. Lastly, the AR glasses offers AR experiences to the user via optical see-through display.

### 4.3.3	Device type 3: XR phone

The XR phone device type represents a type of device which corresponds to a smartphone with capacities and resources sufficient to offer AR experiences. As a result, this device type is capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 4.3.4	Device type 4: XR Head Mounted Display (HMD)

The XR HMD device type represents a type of device which corresponds to HMDs capable of offering at least AR experiences but not precluding other types of XR experiences. This device type is expected to be capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

# 5	Device reference architecture and interfaces

## 5.1	Architecture

The XR Baseline Client represents the functionalities, the peripherals, and the interfaces that are present on a generic XR Device. The XR Baseline Client reference architecture is shown in Figure 5.1-1 which reflects the architecture of XR Devices.

Figure 5.1-1 XR Baseline clientarchitecture and interfaces

## 5.2	Description of the functional blocks

In terms of functionalities, an XR Baseline Client is composed of:

-	An XR Application: a software application that integrates audio-visual content into the user’s real-world environment

-	An XR Runtime: a set of functions that interface with a platform to perform commonly required operations, such as accessing the controller/peripheral state, getting current and/or predicted tracking positions, performing spatial computing, as well as submitting rendered frames to the display processing unit and rendered audio to the speakers with a late stage re-projection to the latest pose (see clause 4.1.3).

-	An XR Source Management: management of data sources provided through the XR runtime such as microphones, cameras, trackers, etc.  For instance, making the information available to the XR application or providing it to the MAF for sending in the uplink.

-	A Media Access Function: A set of functions that enables access to media and other XR-related data that is needed in the Scene manager or XR Runtime to provide an XR experience as well to create delivery formats for information provided by the XR Source Management.

-	A Scene Manager: a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the XR Runtime.

-	A Presentation Engine: a set of composite renderers, rendering the component of the scenes, based on the input from the Scene Manager.

-	A Media Session Handler: a set of functions responsible for handling all 5G control plane operations, such as requesting network assistance, discovering and allocating edge resources, etc. This may be realized as a 5G-RTC MSH, 5GMS Media Session Handler, or any other function. In addition, those functional blocks are integrated together via interfaces. Interfaces may be made of APIs and/or data formats and collectively act as a contract between the two sides of the interface.

In addition, those functional blocks are integrated together via interfaces. Interfaces may be made of APIs and/or data formats and collectively act as a contract between the two sides of the interface.

## 5.3	Interfaces and APIs

The XR Baseline Client contains the following interfaces:

-	IF-1 for the XR Runtime on one side and the Application (1a), the XR Source Management (1b) and the Presentation Engine (1c). IF-1a-c is implemented as an API (API-1) that exposes functions provided by the XR Runtime. An example of this API is the Khronos OpenXR API.

-	IF-2 describes the functions exposed by the XR Source Management that can be accessed and controlled by the XR application, or possibly other functions in the device. IF-2 is typically implemented as an API.

-	IF-3 lies between the XR Source Management and the Media Access Function and provides serialized information accessible on XR Runtime to the MAF.

-	IF-4 between the Media Access Function and the 5G System for user plane data, such as application data or other graphics data needed by the XR application.

-	IF-5 lies between the UE and the 5G System, implementing control sessions. An example instance of  this interface is RTC-5 interface as defined by TS 26.506 [6].

-	IF-6 connects the Media Session Handler and the Application/MAF. It offers the tools for them to activate 5G media functionality such as network assistance and edge resource discovery. The IF-6 is realized through an API (API-6).

-	IF-7 lies between the XR Application and the Media Access function to configure Media Access. This is typically implemented as an API (API-7) that exposes functions of the MAF.

-	IF-8 is an interface that allows the XR application to make use of 5G System connectivity.

-	IF-9 between the Scene Manager and the Media Access Function.

-	IF-10 between the Scene Manager and the XR Application.

# 6	General system functions and capabilities

[Ed note: Description of general functions such as sensors, runtime and their different capabilities, same for system aspects including protocols…]

## 6.1	XR system capabilities

The XR Runtime comprises functions and hardware components present on the XR Device. However, those functions and hardware components are not directly exposed to the XR Application. Instead, the XR Runtime offers its functions and hardware components via an XR System. A single XR Runtime may expose more than one XR Systems for targeting different purposes, e.g., a handheld device may have two XR Systems, one when the user holds the device and one when the device is inserted into an HMD. At the start of the XR Application, the XR Application is expected to query what XR Systems are available on the XR Device and select one of them to create the XR Session.

Table 6.1-1 provides capabilities for XR Runtimes exposed through an XR System. This table does not prescribe support for any specific capabilities of an XR device as defined in clause 4.3. The support of XR System capabilities is defined per device category in clause 10. A mapping of these high-level capabilities to XR frameworks are provided in Annex B.

Table 6.1-1	XR System capabilities

| Capability | Description | Parameters | Value type | Parameter definitions |
| --- | --- | --- | --- | --- |
| Create a XR System | An application can create a XR System from an XR Runtime. | xrSystemIdentifier | integer | Identifier of a given XR System exposed by a XR Runtime. |
| Query XR System’s graphics properties | An application can query an XR System about its graphics capabilities. | swapchainSupported | boolean | Indicates whether the XR System supports the swapchains. |
|  |  | maxSwapchainImageHeight | integer | The maximum swapchain image pixel height supported by this XR system. |
|  |  | maxSwapchainImageWidth | integer | The maximum swapchain image pixel height supported by this XR system. |
|  |  | maxLayerCount | integer | The maximum number of composition layers supported by this XR system |
| Query XR System’s tracking properties | An application can query an XR System on the tracking capabilities. | orientationTracking | boolean | Indicates whether the XR System supports orientational tracking of the view pose(s), or not. |
|  |  | positionTracking | boolean | Indicates whether the XR system supports positional tracking of the view pose(s), |
| Enumerate XR System’s supported environment blend modes | An application can query an XR System about its supported environment blend modes, see clause [xxx]. | blendMode | ['opaque', 'additive', 'alpha_blend'] | Indicates the type of blend mode supported by the XR System. The value 'opaque' relates to the opaque blend mode, the value 'additive' to the additive blend mode and 'alpha_blend' to the alpha blend mode. |
| Enumarate supported view configuration types | An application can query an XR System about the its supported primary view configurations. | viewConfigurationPrimary | ['monoscopic', 'stereoscopic', 'other'] | Indicates the type of primary view configuration of the XR System. The value 'monoscopic' relates to a single view, the value 'stereoscopic' to the left and right-eye views and 'other' to a type undefined in the scope of this specification. |
| Enumerate the view configuration properties | An application can list the properties associated with different view configurations advertised by an XR System. | recommendedImageRectWidth | integer | The optimal width of imageRect to use when rendering this view into a swapchain. |
|  |  | maxImageRectWidth | integer | The maximum width of imageRect supported when rendering this view into a swapchain. |
|  |  | recommendedImageRectHeight | integer | The optimal height of imageRect to use when rendering this view into a swapchain |
|  |  | maxImageRectHeight | integer | The maximum height of imageRect supported when rendering this view into a swapchain. |
|  |  | recommendedSwapchainSampleCount | integer | The recommended number of sub-data element samples to create for each swapchain image that will be rendered into for this view. |
|  |  | maxSwapchainSampleCount | integer | The maximum number of sub-data element samples supported for swapchain images that will be rendered into for this view. |
| Enumerate reference space types | An application can query an XR System about the supported reference space types, described in [xxx]. | referenceSpaceView | ['view', 'local', 'stage', 'unbounded', 'user_defined'] | Indicates the type of reference spaces supported by the XR System. The value 'view' relates to view reference space, the value 'local' to the local reference space, the value 'stage' to the stage reference space, the value 'unbounded'. |
| Query the spatial range boundaries | An application can query the spatial ranges in which an XR experience may be rendered. | 2DSpatialRangeBoundaries | tbd | Provides the rectangle centered on the origin of a given reference space in which the user can freely move. |
| Enumerate swapchain image formats | An application can query the swapchain image formats supported by an XR System. | swapchainImageFormatIdentifier | enumeration | Provides an identifier of a swapchain image format that the XR System supports. |
| Enumerate swapchain images | An application can list the swapchain images allocated to a swapchain. | numberSwapchainImages | enumeration | Provides the number of images allocated for a given swapchain. |
|  |  | swapchainImages | object | Provide the implementation-specific swapchain image objects for a given swapchain. |
| Enumerate composition layer type | An application can list the composition layer types supported by an XR System. | compositionLayerProjection | ['projection', 'quad', 'cylinder', 'cube', 'equirectangular', 'depth'] | Indicates the type of composition layers supported by the XR Systems supports. The value 'projection' represents planar projected images, one rendered for each eye using a perspective projection. The value 'quad' represents quad composition layers which are useful for rendering user interface elements or 2D content on a planar area in the world. The value 'cylinder' represents cylinder composition layers which maps the texture onto the inside of a cylinder section. The value 'cube' represents cube composition layer which consists of a cube map with six views to be rendered by the application. The value 'equirectangular' represents equirectangular composition layers which consists of an equirectangular image that is mapped onto the inside of a sphere in the world. The value 'depth' represents depth composition layers which allows submitting depth maps as an extra composition layer to be used by the XR Runtime for pose correction. |

[Editor’s note: Add a table of capabilities of the XR Runtime and what is expected to be available and what is optional needs to be queried.

Basic concept of specification:

-	Capability query

-	[Editor’s note: Description of the pipelines, sensors, AR runtime, decoders… identify for what entities capabilities are defined]

Collected Requirements]

## 6.2	Metadata formats

### 6.2.1	General

TBD

### 6.2.2	Pose information format

The pose information is described by the poseInfo object.

The structure and the attributes of the poseInfo object are defined in Table 6.2.2-1. [Ed.note: table to be aligned with split rendering spec]

Table 6.2.2-1 - Pose information format

### 6.2.3	Action format

Actions are grouped into action sets which may be activated and deactivated during the lifetime of an XR session.

The action information is described by the actionSets object. The structure and the attributes of the actionSets object are defined in Table 6.2.3-1. [Ed.note: table to be aligned with split rendering spec]

Table 6.2.3-1 - Action format

### 6.2.4	Available Visualization Space format

The XR Application may define a three-dimensional space within the user’s real-word space that is suitable for rendering virtual objects called the Available Visualization Space. Such a space is defined with a shape which is either cube or sphere with the corresponding size and coordinates. In the case that the virtual scene is rendered by a remote entity (e.g. split rendering), this Available Visualization Space may be transmitted to this remote entity so that the composed AR objects remain within the defined Available Visualization Space. The method of calculating the Available Visualization Space is out of the scope of this document.

The content of the availableVisualizationSpace type shall follow the format defined in Table 6.2.4-1.

Table 6.2.4-1 – Available Visualization Space

With this Available Visualization Space, a user may for instance avoid the virtual objects to be occluding other real objects in the scene, e.g. TV sets, people, etc., since an AR experience is achieved by the integration of visual objects into the user environment.

# 7	Visual functions and capabilities

[Ed note: eg description of video formats and codecs, same for GPU capabilities and formats]

## 7.1	Decoding capabilities

### 7.1.1	Single decoder instance

The following video decoding capabilities are defined:

-	AVC-FullHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [7] bitstreams.

-	AVC-UHD-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 5.1 [7] bitstreams with the following additional requirements:

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and,

-	the bitstream does not contain more than 10 slices per picture.

-	AVC-8K-Dec: the capability to decode H.264 (AVC) Progressive High Profile Level 6.1 [7] bitstreams [TBD: possible additional requirements]

[Editor’s note: not present in TS 26.511].

-	HEVC-FullHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

-	HEVC-UHD-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1 and general_frame_only_constraint_flag equal to 1.

-	HEVC-8K-Dec: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 6.1 [8] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1 with the following additional requirements:

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432; and,

-     the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

### 7.1.2	Concurrent decoding capabilities

[Editor’s note: The technical details of the concurrent decoder instances definition in this clause will be further checked.]

#### 7.1.2.1	Definition

Concurrent video decoder instances are defined as follows.

For N video bitstreams encoded according to a video codec profile, decoding units flow into the coded picture buffer (CPB) for each stream according to a specified arrival schedule and are delivered by the common Hypothetical Stream Scheduler (HSS) that schedules the N bitstreams for decoding each of the units. For each access unit

-	all data associated with an access unit is removed and decoded instantaneously by the instantaneous decoding process at CPB removal time of the access unit.

-	Each decoded picture is placed in the Decoded Picture Buffer (DPB) for being referenced by the decoding process of this stream as well as for output and cropping.

-	A decoded picture is removed from the DPB at the time that it becomes no longer needed for inter-prediction reference as well as the output time of the access unit is the largest of all decoded pictures remaining in the group of N decoders

Then at any point time,

-	each of the individual streams conforms to the signaled profile/level/tier and HRD parameters of the individual stream.

-	The sum of the CPB size conforms to common profile/level/tier signaling

-	The aggregate decoder processing speed (samples per seconds) conforms to common profile/level/tier signaling.

-	The sum of the DPB size conforms to common profile/level/tier signaling

-	The common DPB size conforms to common profile/level/tier signaling

A set of N concurrent decoder instances conforms to a given capabilities (defined in clause 7.1.2.2), if a set of up to N bitstreams encoded to be decodable by the HRD above, is decodable within the timing limits.

#### 7.1.2.2	Capabilities

Based on the definition in clause 7.1.2.1, the following capabilities are defined:

-	AVC-FullHD-Dec-2: The capability of supporting up to two (N=2) concurrent decoder instances with the aggregate capabilities of AVC-FullHD-Dec.

-	AVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of AVC-UHD-Dec.

-	HEVC-UHD-Dec-4: The capability of supporting up to four (N=4) concurrent decoder instances with the aggregate capabilities of HEVC-UHD-Dec.

-	UHD-Dec-4: The capability supporting up to four (N=4) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-UHD-Dec-4;

-	the aggregate capabilities of HEVC-UHD-Dec-4; or,

-	the capability of decoding up to 4 bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec.

-	AVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of AVC-8K-Dec.

-	HEVC-8K-Dec-8: The capability of supporting up to eight (N=8) concurrent decoder instances with the aggregate capabilities of HEVC-8K-Dec.

-	8K-Dec-8: The capability supporting up to eight (N=8) concurrent decoder instances with either:

-	the aggregate capabilities of AVC-8K-Dec-8;

-	the aggregate capabilities of HEVC-8K-Dec-8; or,

-	the capability of decoding up to:

-	eight bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-FullHD-Dec or HEVC-FullHD-Dec; or,

-	four bitstreams for which each bitstream does not exceed the capability of being decodable either with AVC-UHD-Dec or HEVC-UHD-Dec.

## 7.2	Encoding capabilities

### 7.2.1	Single encoder instance

The following video encoding capabilities are defined:

-	AVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is AVC-FullHD-Dec capable as defined in clause 7.1.1.1 with the following additional constraints:

-	up to 245,760 macroblocks per second;

-	up to a frame size of 8,192 macroblocks;

-	up to 240 frames per second;

-	the chroma format being 4:2:0; and

-	the bit depth being 8 bit;

-	HEVC-FullHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 7.1.1 with the following additional constraints:

-	up to 133,693,440 luma samples per second;

-	up to a luma picture size of 2,228,224 samples;

-	up to 240 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

-	HEVC-UHD-Enc: the capability to encode a video signal to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 7.1.1 with the following additional constraints:

-	up to 534,773,760 luma samples per second;

-	up to a luma picture size of 8,912,896 samples;

-	up to 480 frames per second;

-	the Chroma format being 4:2:0; and

-	the bit depth being either 8 or 10 bit;

## 7.3	Scene processing capabilities

### 7.3.1	General

This clause defines scene processing capabilities that enable AR and XR experiences.

In clause 7.3.2, glTF-based Scene Description capabilities are defined.

NOTE: Functionalities of this specification can be used w/o a standardized scene description, for example by an application providing an interoperability on Scene Description. However, in this case, AR experiences may be limited to the functionalities of the application and may be restricted.

### 7.3.2	glTF-based Scene Description capabilities

This clause defines client cababilities for rendering and presenting Scene Description based on glTF2.0 [12]. Additional extended capabilites are defined, primarily to support real-time media based on the extensions defined in the MPEG-I Scene Description in ISO/IEC 23090-14 [9].

The following scene processing capabilities are defined.

-	SD-Rendering-glTF-Core: The capability to process glTF2.0 [12] scene description files and to render the described scenes with the following restrictions:

-	The glTF 2.0 scene description can either be a standalone JSON file or an encapsulated GLB file.

-	The glTF 2.0 scene description can have an aggregate geometry complexity, including all mesh primitives of the scene, of at least 100k faces and/or points. The requirement on the number of faces in mesh nodes allows for moderate complexity scenes.

-	The glTF 2.0 scene description can have at least 20 materials using static image textures. The requirement on the number of materials with image texture maps allows for moderate complexity scenes.

-	The glTF 2.0 scene description can use glTF2.0 animations and skinning.

-	The glTF 2.0 scene description can use the KHR_lights_punctual and KHR_materials_specular extensions.

-	SD-Rendering-glTF-Ext1: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [9-11] and to render the described scenes with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_media extension.

-	The MPEG-I Scene Description document can use the MPEG_accessor_timed and the MPEG_buffer_circular extensions.

-	The MPEG-I Scene Description document can use the MPEG_texture_video extension.

- 	The MPEG-I Scene Description document can use the MPEG_audio_spatial extension.

-	The MPEG-I Scene Description document can use the scene description update mechanism as defined in clause 5.2.4 of [9].

-	Processing of scene description updates as defined in ISO/IEC 23090-14 [9] to allow the device to contribute to a scene description of a dynamic shared scene.

-	The MPEG-I Scene Description document can use at least 4 materials using video textures

NOTE: This functionality implies that at least 4 concurrent video decoder instances are available

[-	The MPEG-I Scene Description document can use at least 4 object-based mono audio sources, which allows for relatively simple audio support.

NOTE: This functionality implies that at least 4 concurrent mono sources are available as the output of the audio decoding engine instances are available, potentially requiring concurrent audio decoder instances]

Editor’s Note: the audio-specific minimal support, such as the type of audio sources and the number of audio sources to support as part of this capability, are to be agreed with the Audio SWG.

-	SD-Rendering-glTF-Ext2: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [9-11] with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_anchor extension.

-	The MPEG-I Scene Description document can use the EXT_lights_image_based and the MPEG_lights_texture_based extensions.

-	SD-Rendering-glTF-Interactive: On top of the SD-Rendering-glTF-Core capability, the capability to process the following glTF 2.0 extensions defined in [11] to enable user input and local interaction processing.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with the following triggers: TRIGGER_USER_INPUT, TRIGGER_PROXIMITY, TRIGGER_COLLISION, TRIGGER_VISIBILITY.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with actions: ACTION_ACTIVATE, ACTION_TRANSFORM, ACTION_BLOCK, ACTION_ANIMATION, ACTION_MEDIA, ACTION_MANIPULATE, ACTION_SET_MATERIAL

# 8	Audio functions and capabilities

## 8.1	Audio/Speech Decoding

The following audio/speech decoding capabilities are defined:

- 	EVS-Dec: the decoding capability as defined in TS 26.117 [9] clause 5.2.

- 	IVAS-[Editor’s note: IVAS level TBD]-Dec: the decoding capability as defined in TS 26.117 [9] clause 5.2.

- 	EVS-Dec-2: the decoding capability as defined TS 26.117 [9] clause 5.2.

- 	EVS-Dec-4: the decoding capability as defined TS 26.117 [9] clause 5.2.

- 	AAC-ELDv2-Dec: the decoding capability as defined TS 26.117 [9] clause 5.2.

- 	AAC-ELDv2-Dec-2: the decoding capability as defined TS 26.117 [9] clause 5.2.

## 8.2	Audio/Speech Encoding

The following audio/speech encoding capabilities are defined:

- 	EVS-Enc: the sender requirements for the EVS Operation Point as defined in TS 26.117 [9] clause 6.2.4.3.

- 	IVAS-[Editor’s note: IVAS level TBD]-Enc: the sender requirements for the IVAS Operation Point as defined in TS 26.117 [9] clause 6.3.5.3.

- 	AAC-ELDv2-Enc: the sender requirements for the AAC-ELDv2 Operation Point as defined in TS 26.117 [9] clause 6.3.6.3.

# 9	Scene processing capabilities

### 9.1	General

This clause defines scene processing capabilities that enable AR and XR experiences.

In clause 9.2, glTF-based Scene Description capabilities are defined.

NOTE: Functionalities of this specification can be used w/o a standardized scene description, for example by an application providing an interoperability on Scene Description. However, in this case, AR experiences may be limited to the functionalities of the application and may be restricted.

### 9.2	glTF-based Scene Description capabilities

This clause defines client capabilities for rendering and presenting Scene Description based on glTF2.0 [10]. Additional extended capabilites are defined, primarily to support real-time media based on the extensions defined in the MPEG-I Scene Description in ISO/IEC 23090-14 [11-13].

The following scene processing capabilities are defined.

-	SD-Rendering-glTF-Core: The capability to process glTF2.0 [10] scene description files and to render the described scenes with the following restrictions:

-	The glTF 2.0 scene description can either be a standalone JSON file or an encapsulated GLB file.

-	The glTF 2.0 scene description can have an aggregate geometry complexity, including all mesh primitives of the scene, of at least 100k faces and/or points. The requirement on the number of faces in mesh nodes allows for moderate complexity scenes.

-	The glTF 2.0 scene description can have at least 20 materials using static image textures. The requirement on the number of materials with image texture maps allows for moderate complexity scenes.

-	The glTF 2.0 scene description can use glTF2.0 animations and skinning.

-	The glTF 2.0 scene description can use the KHR_lights_punctual and KHR_materials_specular extensions.

-	SD-Rendering-glTF-Ext1: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [11-13] and to render the described scenes with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_media extension.

-	The MPEG-I Scene Description document can use the MPEG_accessor_timed and the MPEG_buffer_circular extensions.

-	The MPEG-I Scene Description document can use the MPEG_texture_video extension.

- 	The MPEG-I Scene Description document can use the MPEG_audio_spatial extension.

-	The MPEG-I Scene Description document can use the scene description update mechanism as defined in clause 5.2.4 of [9].

-	Processing of scene description updates as defined in ISO/IEC 23090-14 [11-13] to allow the device to contribute to a scene description of a dynamic shared scene.

-	The MPEG-I Scene Description document can use at least 4 materials using video textures

NOTE: This functionality implies that at least 4 concurrent video decoder instances are available

[-	The MPEG-I Scene Description document can use at least 4 object-based mono audio sources, which allows for relatively simple audio support.

NOTE: This functionality implies that at least 4 concurrent mono sources are available as the output of the audio decoding engine instances are available, potentially requiring concurrent audio decoder instances]

Editor’s Note: the audio-specific minimal support, such as the type of audio sources and the number of audio sources to support as part of this capability, are to be agreed with the Audio SWG.

-	SD-Rendering-glTF-Ext2: On top of the SD-Rendering-glTF-Core capability, the capability to process MPEG-I Scene Description documents as specified in [11-13] with the following restrictions:

-	The MPEG-I Scene Description document can use the MPEG_anchor extension.

-	The MPEG-I Scene Description document can use the EXT_lights_image_based and the MPEG_lights_texture_based extensions.

-	SD-Rendering-glTF-Interactive: On top of the SD-Rendering-glTF-Core capability, the capability to process the following glTF 2.0 extensions defined in [11-13] to enable user input and local interaction processing.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with the following triggers: TRIGGER_USER_INPUT, TRIGGER_PROXIMITY, TRIGGER_COLLISION, TRIGGER_VISIBILITY.

-	The MPEG-I Scene Description document can use the MPEG_scene_interactivity extension with actions: ACTION_ACTIVATE, ACTION_TRANSFORM, ACTION_BLOCK, ACTION_ANIMATION, ACTION_MEDIA, ACTION_MANIPULATE,  ACTION_SET_MATERIAL

# 9	QoE metrics

[Editor’s note: related WID objectives

Identify which QoE metrics from VR QoE metrics can be reused or enhanced for AR media (e.g., resolution per eye, Field of view (FOV), round-trip interaction delay, etc.) and define relevant KPIs that are dedicated to AR/MR

Specify additional relevant KPIs and simple QoE Metrics for AR media]

## 9.1	Metrics and Observation Points

### 9.1.1	Overview

The Observation Points (OPs) are defined to support the definition of the corresponding metrics. This specification defines four observation points as shown in Figure 9.1.1-1. The metrics collection function, as part of the Media Session Handler, is responsible of collecting specific information observed at each OP in order to generate the metrics. This function has also access to the 5G System such that the metrics can be reported to an external entity.

Figure 9.1.1-1 - Observation Points in the XR Baseline Client

### 9.1.2	Observation Point 1: XR Runtime information

Observation point 1 (OP-1) is derived from the XR Runtime API. The OP-1 observes information exchanged between the XR Runtime on one side and the XR Source Management, the Presentation Engine and the application on the other side, i.e. on IF-1.

On observation point 1, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	XR runtime clock

-	Actual presentation/display time

-	Actual playout frame rate

-	Viewer pose prediction and pose prediction parameters

-	Projection parameters

-	Tracking pose prediction parameters

-	user input actions and the time when the action is made

-	Rendering loop status

-	Camera information

]

### 9.1.3	Observation Point 2

Observation point 2 (OP-2) observes information at the input of the Scene Manager, i.e. on IF-9 for data received from the Media Access Function and the IF-10 for information exchanged between the Scene Manager and the application.

On observation point 2, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	Media resolution

-	Media codec

-	Media frame rate

-	Media decoding time

]

### 9.1.4	Observation Point 3

Observation point 3 (OP-3) is derived from the API which exchanges information between the XR Source Management and the Media Access Functions. It corresponds to the IF-3 interface.

On observation point 3, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition]

### 9.1.5	Observation Point 4

Observation point 4 (OP-4) observes information between the Media Access Function and the 5G System, i.e. on IF-4 interface.

On observation point , the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	The media type

]

## 9.2	Metrics Definitions

### 9.2.1	Latency metrics

To enable good XR experiences, it is relevant to monitor latencies such as the pose-to-render-to-photon.

Beyond the sense of presence and immersiveness, the age of the content and user interaction delay are of the uttermost importance for immersive and non-immersive interactive experiences, i.e. experiences for which the user interaction with the scene impacts the content of scene (such as online gaming).

Table 9.2.1-1 provides time information that may be gathered to compute the latency metrics. The observation points to collect the time information are indicated per device type.

Table 9.2.1-1: Time information for latency metrics

# 10	Device types and media profiles

## 10.1	Introduction

AR experiences may be running on a variety of devices which have different characteristics and capabilities. Certain capabilities may be common to several devices while other capabilities may be unique to a specific device. Therefore, the present specification enables interoperability by collecting the media capabilities and profiles, defined in clauses 6 and 7. 8 and 9 per device type. The four device types defined are:

-	Device type 1: Thin AR glasses (see clause 10.2)

-	Device type 2: AR glasses (see clause 10.3)

-	Device type 3: XR phone (see clause 10.4)

-	Device type 4: XR HMD (see clause 10.5)

NOTE:	A given physical device can correspond to more than one device type.

## 10.2	Device type 1: Thin AR glasses

[Editor’s note: For each device type, it is expected that rendering capabilities will also be added before completing the TS.]

### 10.2.1	General

As defined in 4.3.1, the thin AR glasses device type represents a type of device which is considered to be power-constrained and with limited computing power with respect to the other device types. These limitations typically come from the requirement to design a device with a small and lightweight form factor. Regarding rendering capacity, this device type is expected to rely on remote rendering to be able display complex scenes to the user. For example, such device type may run a split rendering session where the split rendering server delivers pre-rendered views of the scene. However, devices in this category can still operate without external support for applications that do not require complex rendering capabilities, for instance, text messaging, 2D video communication, etc. Lastly, the thin AR glasses offers AR experiences to the user via optical see-through display.

### 10.2.2	XR System support

An XR Device complying to the thin AR glasses device has an XR System with at least the following capabilities as based on clause 6.1:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 6.1.

### 10.2.3	Video capabilities support

An XR Device complying to device type 1 shall support at least the following decoding capabilities from clause 7:

-	AVC-FullHD-Dec

-	[AVC-FullHD-Dec-2]

An XR Device complying to device type 1 shall support at least the following encoding capabilities:

-	AVC-FullHD-Enc

An XR Device complying to device type 1 should support at least the following decoding capabilities:

-	HEVC-FullHD-Dec

An XR Device complying to device type 1 should support at least the following encoding capabilities:

-	HEVC-FullHD-Enc

### 10.2.4	Audio/Speech capabilities support

[Editor’s note: The audio/speech capabilities for this device type are expected to be further adapted based on the characteristics of this device type.]

An XR Device complying to device type 1 device shall support the following decoding capabilities from clause 8:

-	EVS-Dec

-	AAC-ELDv2-Dec

An XR Device complying to device type 1 should support the following decoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Dec

-	EVS-Dec-2

An XR Device complying to device type 1 may support the following decoding capabilities:

-	EVS-Dec-4

-	AAC-ELDv2-Dec-2

An XR Device complying to device type 1 shall support the following encoding capabilities:

-	EVS-Enc

An XR Device complying to device type 1 should support the following encoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Enc

-	AAC-ELDv2-Enc

### 10.2.5	Scene Description capabilities support

TBD

## 10.3	Device type 2: AR glasses

### 10.3.1	General

As defined in 4.3.2, The AR glasses device type represents a type of device which is considered to have higher computation power compared to the thin AR glasses device type. As a result, this device type has higher rendering capacities and is generally expected to be capable of rendering scenes without external support, even though remote rendering is not precluded to lower the power consumption on the device or enable the display of scenes beyond the device’s rendering capability. Lastly, the AR glasses offers AR experiences to the user via optical see-through display.

### 10.3.2	XR System support

An XR Device complying to the AR glasses device type has offers XR System with at least the following capabilities from clause 6.1:

-	orientationTracking is ‘true’

-	positionTracking is ‘true’

-	Value ‘additive’ of the enumeration blendMode

-	Value ‘stereoscopic’ of the enumeration viewConfigurationPrimary

-	Values ‘view’, ‘local’ and ‘stage’ of the enumeration referenceSpace

-	If swapchainSupported is ‘true’, numberSwapchainImages is equal to 2

-	Values ‘projection’ and ‘quad’ of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 6.1.

### 10.3.3	Video capabilities support

An XR Device complying to device type 2 shall support at least the following decoding capabilities from clause 7:

-	AVC-UHD-Dec

-	[AVC-UHD-Dec-4]

-	HEVC-UHD-Dec

An XR Device complying to device type 2 shall support at least one of the following encoding capabilities:

-	AVC-FullHD-Enc

-	HEVC-FullHD-Enc

An XR Device complying to device type 2 should support at least the following decoding capabilities:

-	[HEVC-UHD-Dec-4]

-	[UHD-Dec-4]

### 10.3.4	Audio/Speech capabilities support

[Editor’s note: The audio/speech capabilities for this device type are expected to be further adapted based on the characteristics of this device type.]

An XR Device complying to device type 2 device shall support at least the following decoding capabilities:

-	EVS-Dec

-	AAC-ELDv2-Dec

An XR Device complying to device type 2 should support the following decoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Dec

-	EVS-Dec-2

An XR Device complying to device type 2 may support the following decoding capabilities:

-	EVS-Dec-4

-	AAC-ELDv2-Dec-2

An XR Device complying to device type 2 shall support at least the following encoding capabilities:

-	EVS-Enc

An XR Device complying to device type 2 should support the following encoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Enc

-	AAC-ELDv2-Enc

### 10.3.5	Scene Description capabilities support

A device of type 2 should support glTF-based scene description as defined in clause 9.2.

If gltf-based scene description is supported, the following requirements and recommendation hold.

-	The SD-Rendering-gltf-Core capabilities shall be supported

-	The SD-Rendering-gltf-ext1 capabilities should be supported

-	The SD-Rendering-gltf-ext2 capabilities may be supported

-	The SD-Rendering-gltf-interactive capabilities may be supported

## 10.4	Device type 3: XR phone

### 10.4.1	General

As defined in 4.3.3, the XR phone device type represents a type of device which corresponds to a smartphone with capacities and resources sufficient to offer AR experiences. As a result, this device type is capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.4.2	XR System support

An XR Device complying to the XR phone device type offers an XR System with at least the following capabilities:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 6.1.

### 10.4.3	Video capabilities support

An XR Device complying to device type 3 shall support at least the following decoding capabilities from clause 7:

-	AVC-UHD-Dec

-	[AVC-UHD-Dec-4]

-	HEVC-UHD-Dec

-	[HEVC-UHD-Dec-4]

-	[UHD-Dec-4]

An XR Device complying to device type 3 shall support at least one of the following encoding capabilities:

-	AVC-UHD-Enc

-	HEVC-UHD-Enc

An XR Device complying to device type 3 should support at least the following decoding capabilities:

-	AVC-8K-Dec

-	HEVC-8K-Dec

-	[8K-Dec-8]

### 10.4.4	Audio/Speech capabilities support

[Editor’s note: The audio/speech capabilities for this device type are expected to be further adapted based on the characteristics of this device type.]

An XR Device complying to device type 3 device shall support at least the following decoding capabilities from clause 8:

-	EVS-Dec

-	AAC-ELDv2-Dec

An XR Device complying to device type 3 should support the following decoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Dec

-	EVS-Dec-2

An XR Device complying to device type 3 may support the following decoding capabilities:

-	EVS-Dec-4

-	AAC-ELDv2-Dec-2

An XR Device complying to device type 3 shall support at least the following encoding capabilities:

-	EVS-Enc

An XR Device complying to device type 3 should support the following encoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Enc

-	AAC-ELDv2-Enc

### 10.4.5	Scene Description capabilities support

A device of type 3 should support gltf-based scene description as defined in clause 9.2.

If gltf-based scene description is supported, the following requirements and recommendation hold.

-	The SD-Rendering-gltf-Core capabilities shall be supported

-	The SD-Rendering-gltf-ext1 capabilities should be supported

-	The SD-Rendering-gltf-ext2 capabilities should be supported

-	The SD-Rendering-gltf-interactive capabilities should  be supported

## 10.5	Device type 4: XR HMD

### 10.5.1	General

As defined in clause 4.3.4, the XR HMD device type represents a type of device which corresponds to HMDs capable of offering at least AR experiences but not precluding other types of XR experiences. This device type is expected to be capable of rendering scenes without external support. Lastly, the XR phone offers AR experiences to the user via video see-through display.

### 10.5.2	XR System support

An XR Device complying to the XR HMD device type offers an XR System with at least the following capabilities from clause 6.1:

-	orientationTracking is 'true'

-	positionTracking is 'true'

-	Value 'additive' or values 'opaque' and 'alpha_blend' of the enumeration blendMode

-	Values 'monoscopic' and 'stereoscopic' of the enumeration viewConfigurationPrimary

-	Values 'view', 'local' and 'stage' of the enumeration referenceSpace

-	If swapchainSupported is 'true', numberSwapchainImages is equal to 2

-	Values 'projection' and 'quad' of the enumeration compositionLayer

NOTE:	For the definition of those capabilities, please refer to clause 6.1.

### 10.5.3	Video capabilities support

An XR Device complying to device type 4 shall support at least the following decoding capabilities from clause 7

-	AVC-UHD-Dec

-	[AVC-UHD-Dec-4]

-	HEVC-UHD-Dec

-	[HEVC-UHD-Dec-4]

-	[UHD-Dec-4]

An XR Device complying to device type 4 shall support at least one of the following encoding capabilities:

-	AVC-UHD-Enc

-	HEVC-UHD-Enc

An XR Device complying to device type 4 should support at least the following decoding capabilities:

-	AVC-8K-Dec

-	HEVC-8K-Dec

-	[8K-Dec-8]

### 10.5.4	Audio/Speech capabilities support

[Editor’s note: The audio/speech capabilities for this device type are expected to be further adapted based on the characteristics of this device type.]

An XR Device complying to device type 4 device shall support at least the following decoding capabilities from clause 8:

-	EVS-Dec

-	AAC-ELDv2-Dec

An XR Device complying to device type 4 should support the following decoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Dec

-	EVS-Dec-2

An XR Device complying to device type 4 may support the following decoding capabilities:

-	EVS-Dec-4

-	AAC-ELDv2-Dec-2

An XR Device complying to device type 4 shall support at least the following encoding capabilities:

-	EVS-Enc

An XR Device complying to device type 4 should support the following encoding capabilities:

-	IVAS-[Editor’s note: IVAS level TBD]-Enc

-	AAC-ELDv2-Enc

### 10.5.5	Scene Description capabilities support

A device of type 4 should support gltf-based scene description as defined in clause 9.2.

If gltf-based scene description is supported, the following requirements and recommendation hold.

-	The SD-Rendering-gltf-Core capabilities shall be supported

-	The SD-Rendering-gltf-ext1 capabilities should be supported

-	The SD-Rendering-gltf-ext2 capabilities should be supported

-	The SD-Rendering-gltf-interactive capabilities should be supported

# 11	QoE Metrics

[Editor’s note: related WID objectives

Identify which QoE metrics from VR QoE metrics can be reused or enhanced for AR media (e.g., resolution per eye, Field of view (FOV), round-trip interaction delay, etc.) and define relevant KPIs that are dedicated to AR/MR

Specify additional relevant KPIs and simple QoE Metrics for AR media]

## 11.1	Metrics and Observation Points

### 11.1.1	Overview

The Observation Points (OPs) are defined to support the definition of the corresponding metrics. This specification defines four observation points as shown in Figure 9.1.1-1. The metrics collection function, as part of the Media Session Handler, is responsible of collecting specific information observed at each OP in order to generate the metrics. This function has also access to the 5G System such that the metrics can be reported to an external entity.

Figure 9.1.1-1 - Observation Points in the XR Baseline Client

### 11.1.2	Observation Point 1: XR Runtime information

Observation point 1 (OP-1) is derived from the XR Runtime API. The OP-1 observes information exchanged between the XR Runtime on one side and the XR Source Management, the Presentation Engine and the application on the other side, i.e. on IF-1.

On observation point 1, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	XR runtime clock

-	Actual presentation/display time

-	Actual playout frame rate

-	Viewer pose prediction and pose prediction parameters

-	Projection parameters

-	Tracking pose prediction parameters

-	user input actions and the time when the action is made

-	Rendering loop status

-	Camera information

]

### 11.1.3	Observation Point 2

Observation point 2 (OP-2) observes information at the input of the Scene Manager, i.e. on IF-9 for data received from the Media Access Function and the IF-10 for information exchanged between the Scene Manager and the application.

On observation point 2, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	Media resolution

-	Media codec

-	Media frame rate

-	Media decoding time

]

### 11.1.4	Observation Point 3

Observation point 3 (OP-3) is derived from the API which exchanges information between the XR Source Management and the Media Access Functions. It corresponds to the IF-3 interface.

On observation point 3, the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition]

### 11.1.5	Observation Point 4

Observation point 4 (OP-4) observes information between the Media Access Function and the 5G System, i.e. on IF-4 interface.

On observation point , the following observed information is defined:

[Editor’s note: define the observed information that is later used in the metrics definition

-	The media type

]

## 11.2	Metrics Definitions

### 11.2.1	Latency metrics

To enable good XR experiences, it is relevant to monitor latencies such as the pose-to-render-to-photon.

Beyond the sense of presence and immersiveness, the age of the content and user interaction delay are of the uttermost importance for immersive and non-immersive interactive experiences, i.e. experiences for which the user interaction with the scene impacts the content of scene (such as online gaming).

Table 9.2.1-1 provides time information that may be gathered to compute the latency metrics. The observation points to collect the time information are indicated per device type.

Table 9.2.1-1: Time information for latency metrics

| Observation Point | Time information | Definition |
| --- | --- | --- |
| OP-1 | estimatedAtTime (ref. T1) | The time when the viewer pose prediction is made. It corresponds to the time when the predicted viewer pose is collected using the XR runtime API-1 by the application or the XR Source Manager. |
| OP-1 | lastChangeTime | The time when the user action is made. It corresponds to the lastChangeTime field defined in the action format in Table 5.1.3-1. |
| OP-2 | sceneUpdateTime (ref. T6) | The time when the Scene Manager starts to update the 3D scene graph according to the viewer pose and the user actions. |
| OP-1 | startToRenderAtTime (ref. T3) | The time when the renderer starts to render the scene according to the viewer pose. |
| OP-1 | actualDisplayTime (ref. T2.actual) | The actual display time of the rendered frame in the swapchain. The estimation of the actual display time is available through the XR runtime. |

[Editor’s note: the reference time used for the timing information is for FFS]

The latency metrics are specified in Table 9.2.1-2. The formula to compute the latencies are defined using the collected time information.

Table 9.2.1-2: Latency metrics

| Latency metric | Description |
| --- | --- |
| poseToRenderToPhoton | The time duration, in units of milliseconds, between the time to query the pose information from the XR runtime to the renderer (the renderer uses this pose to generate the rendered frame) and the display time of the rendered frame. It can be computed as follows: actualDisplayTime – estimatedAtTime |
| renderToPhoton | The time duration, in units of milliseconds, between the start of the rendering by the Presentation Engine and the display time of the rendered frame. It can be computed as follows: actualDisplayTime – startToRenderAtTime |
| roundtripInteractionDelay | The time duration, in units of milliseconds, between the time a user action is initiated and the time the action is presented to the user. It can be computed as follows: actualDisplayTime – lastChangeTime |
| userInteractionDelay | The time duration, in units of milliseconds, between the time a user action is initiated and the time the action is taken into account by the content creation engine in the scene manager. It can be computed as follows: sceneUpdateTime – lastChangeTime |
| ageOfContent | The time duration, in units of milliseconds, between the time the content is created in the scene by the Scene Manager and the time it is presented to the user. It can be computed as follows: actualDisplayTime – sceneUpdateTime |
| sceneUpdateDelay | The time duration, in units of milliseconds, spent by the Scene Manager to update the scene graph. It can be computed as follows: startToRenderAtTime – sceneUpdateTime |

# 12	Metadata formats

### 12.1	General

TBD

### 12.2	Pose information format

The pose information is described by the poseInfo object.

The structure and the attributes of the poseInfo object are defined in Table 10.2.2-1. [Ed.note: table to be aligned with split rendering spec]

Table 10.2.2-1 - Pose information format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| poseInfo | Object | 1..n | An array of pose information objects, each corresponding to a target display time and XR space. |
| displayTime | number | 1..1 | The time for which the current view poses are predicted. |
| xrSpace | number | 0..1 | An identifier for the XR space in which the view poses are expressed. The set of XR spaces are agreed on between the split rendering client and the split rendering server at the setup of the split rendering session. The set of XR spaces is negotiated as part of the split rendering configuration as defined in clause 8.4.2.2. [Ed: spec?] |
| viewPoses | Object | 0..n | An array that provides a list of the poses associated with every view. The number of views is determined during the split rendering session setup between the split rendering client and server, depending on the view configuration of the XR session. |
| pose | Object | 1..1 | An object that carries the pose information for a particular view. |
| orientation | Object | 1..1 | Represents the orientation of the view pose as a quaternion based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the quaternion. |
| y | number | 1..1 | Provides the y coordinate of the quaternion. |
| z | number | 1..1 | Provides the z coordinate of the quaternion. |
| w | number | 1..1 | Provides the w coordinate of the quaternion. |
| position | Object | 1..1 | Represents the location in 3D space of the pose based on the reference XR space. |
| x | number | 1..1 | Provides the x coordinate of the position vector. |
| y | number | 1..1 | Provides the y coordinate of the position vector. |
| z | number | 1..1 | Provides the z coordinate of the position vector. |
| confidence | number | 0..1 | This optional parameter provides a confidence score that reflects the probability for this pose prediction to be correct. For the current pose or a pose in the past, the confidence value would be 1. The confidence can take a value between 0 and 1. If not provided by the XR runtime, this field may be estimated by the SRC or omitted. |
| fov | Object | 1..1 | Indicates the four sides of the field of view used for the projection of the corresponding XR view. |
| angleLeft | number | 1..1 | The angle of the left side of the field of view. For a symmetric field of view this value is negative. |
| angleRight | number | 1..1 | The angle of the right side of the field of view. |
| angleUp | number | 1..1 | The angle of the top part of the field of view. |
| angleDown | number | 1..1 | The angle of the bottom part of the field of view. For a symmetric field of view this value is negative. |

### 12.3	Action format

Actions are grouped into action sets which may be activated and deactivated during the lifetime of an XR session.

The action information is described by the actionSets object. The structure and the attributes of the actionSets object are defined in Table 6.2.3-1. [Ed.note: table to be aligned with split rendering spec]

Table 10.3-1 - Action format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| actionSets | Object | 1..n | An array of active action sets, for which there is at least an action that has a state change. |
| actions | number | 1..n | An array of objects that conveys information about the actions of the parent action set. |
| identifier | number | 1..1 | A unique identifier of the action that was agreed upon during split rendering session setup. |
| subactionPath | string | 1..1 | The sub-action path for which the state has changed. It abstracts a binding between an action and the hardware input associated to it by the XR runtime. |
| state | object | 1..1 | The state of the action that had a change in state. |
| lastChangeTime | number | 1..1 | The timestamp of the last change to the state of this action. |
| currentStateBool | Bool | 0..1 | The current Boolean state of the action |
| currentStateNum | number | 0..1 | The current numerical state of the action. |
| currentStateVec2 | Array | 0..1 | An array of numerical state values for the action. |

### 12.4	Available Visualization Space format

The XR Application may define a three-dimensional space within the user’s real-word space that is suitable for rendering virtual objects called the Available Visualization Space. Such a space is defined with a shape which is either cube or sphere with the corresponding size and coordinates. In the case that the virtual scene is rendered by a remote entity (e.g. split rendering), this Available Visualization Space may be transmitted to this remote entity so that the composed AR objects remain within the defined Available Visualization Space. The method of calculating the Available Visualization Space is out of the scope of this document.

The content of the availableVisualizationSpace type shall follow the format defined in Table 10.4-1.

Table 10.4-1 – Available Visualization Space

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| availableVisualizationSpace | Object | 0..1 | An object defining the coordinate of the available visualization space. |
| cuboid | Object | 0..1* | The available visualization space in form of cuboid |
| x | float | 1 | Offset of the available visualization space starting point in the x direction ias defined by the Open XR coordinate system in meters. The value is in meters. |
| y | float | 1 | Offset of the available visualization space starting point in the y direction as defined by the Open XR coordinate system. The value is in meters. |
| z | float | 1 | Offset of the available visualization space starting point in the z direction as defined by the Open XR coordinate system. The value is in meters. |
| width | float | 1 | The width of available visualization space in the x direction as defined by the Open XR coordinate system. The value is in meters. |
| height | float | 1 | The height of available visualization space in the y direction as defined by the Open XR coordinate system. The value is in meters. |
| depth | float | 1 | The depth of available visualization space in the z direction as defined by the Open XR coordinate system. The value is in meters. |
| sphere | Object | 0..1* | The available visualization space in form of cuboid |
| x | float | 1 | Offset of the available visualization space center in the x direction as defined by the Open XR coordinate system. The value is in meters. |
| y | float | 1 | Offset of the available visualization space center in the y direction as defined by the Open XR coordinate system. The value is in meters. |
| z | float | 1 | Offset of the available visualization space center in the z direction as defined by the Open XR coordinate system. The value is in meters. |
| radius | float | 1 | The radius of available visualization space as defined by the Open XR coordinate system. The value is in meters. |
| *Only one of cuboid or sphere object shall exists. |  |  |  |

With this Available Visualization Space, a user may for instance avoid the virtual objects to be occluding other real objects in the scene, e.g. TV sets, people, etc., since an AR experience is achieved by the integration of visual objects into the user environment.

End of First Change