3GPP SA4#127	S4-240233

29 January – 3 Feburary, 2024, Sophia Antipolis, France

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | pseudo | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] On the ABNF syntax for the RTP header extension for PDU Set Marking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 29/01/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The ABNF sytax for the SDP for the RTP header extension fo the PDU Set marking is presented differently from those for the other two RTP header extensions in TS26.522, and this may cause confusions. - To represent this line: extmap-attr="a=extmap:" 1*5DIGIT ["/" direction] SP "urn:3gpp:pdu-set-marking:rel-18" SP extensionattributes we only need to define extensionname and extensionattributes. 2. The meaning of the “short”/”long” indicator being absent is not defined. 3. The SDP signlaing for NPDS is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The definitions of extensionname and extensionattributes replaced the current syntax for the RTP header extension fo the PDU Set marking. 2. The semantics for the absence of “short”/”long” indicator is defined. 3. An example is added. 4. The SDP signlaing for NPDS is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different styles in describing the ABNF sytax for RTP header extensions in the same spec; the semantics is not clear when “short”/”long” is absent; SDP signaling is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

In the Clause 4.4.2.5:

# * * * * 1st change * * * *

#### 4.4.2.5	SDP Signaling

An AS or sender UE capable of sending PDU set marking HE shall use the SDP attribute extmap for PDU set marking HE in the media description of the RTP stream(s) carrying the PDU set HE. A receiver that does not support PDU set marking HE can ignore the RTP header when included. The signaling of the PDU Set and End-of-Burst marking RTP header extension shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The URN for the PDU Set marking shall be set to "urn:3gpp:pdu-set-marking:rel-18".

The header extension identifier shall be registered with IANA as maintained in Real-Time Transport Protocol (RTP) Parameters (iana.org).

The ABNF syntax for the extmap attribute for the signaling of PDU Set Information and End of Burst marking is defined as follows:

extensionname = "urn:3gpp:pdu-set-marking:rel-18"

extensionattributes = *3(format / "pdu-set-size"/ “num-pdus-in-pdu-set”)

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP header extension for PDU Set and End-of-Burst marking uses the 1-byte (short) or the 2-byte (long) format. If absent, it indicates that the 1-byte (short) format is used.

-	pdu-set-size: if present, this attribute indicates that the RTP sender will provide the PDU Set size in bytes in the RTP header extension with every RTP packet. This results in an additional 3 bytes of length for the RTP header extension

-   num-pdus-in-pdu-set: if present, it indicates that the RTP sender will provide the NPDS in the RTP header extension with every RTP packet. This results in an additional 2 bytes of length fo the RTP header extension.

Below is an example:

a=extmap:7 urn:3gpp:pdu-set-marking:rel-18 short pdu-set-size

# * * * * End of 1st  change * * * *