Source:	Fraunhofer IIS

Title:	On the characterization testing of OSBA

Document for:	Discussion and Agreement

Agenda Item:	7.5 – IVAS_Codec

# Introduction

In the working version of IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase [1], the subjective testing of the combined formats including OSBA is considered.

In contribution [2] a proposal for the P.800Suppl DCR experiment for the combined format OMASA was presented.

This document aims to outline a P.800Suppl DCR experiment for the combined format OSBA in a similar manner.

# Discussion

In general, the source agrees with the considerations made in section 3 of [2]. Same considerations apply to the combined format OSBA.

Furthermore, to facilitate the task of item collection and selection and since background noise for OMASA can be obtained from FOA or HOA format, the same exact items can be used as in the proposed OSBA and OMASA tests, similarly as was done for the Selection Phase testing for SBA and MASA.

# P.SUPPL800 experiment proposal for OSBA

It is proposed for OSBA inputs to have following conditions for the IVAS characterization listening test.

Table 1 Overview of test conditions

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 15 | IVAS OSBA operated at all bitrates 13.2- 512 kbps with DTX off at 0% FER |
|  | 3 | IVAS OSBA operated at 32, 64, 256 kbps with 5% FER |
|  | 4 | IVAS OSBA fixed point / floating point interoperability conditions at 32, 64, 128 and 256 kbps |
| Codec references |  |  |
| Codec references | 6 | IVAS operation in two separate instances (SBA + ISM) |
| Other references |  |  |
| Direct | 1 | Fixed point IVAS_rend. Nominal input level |
| P.50 MNRU (applied to SBA transport streams) | 4 | Q = 34, 30, 26, 22 dB (all: nominal input level) |
| ESDRU [ITU-T P.811] | 4 | α = 0.8, 0.6, 0.4, 0.2 (output loudness set to nominal level) |
| Common Conditions |  |  |
| Test item generation | 1 | Model-based generation according to processing scripts. |
| Binaural rendering | 1 | IVAS codec internal binaural renderer and for references IVAS external renderer (IVAS_rend) |
| Audio sampling frequency / bandwidth | 1 | 48 kHz / maximum available audio bandwidth (WB, SWB, FB) |
| Content types / categories | 6 | Scenes as described in Table2 of [1] |
| Number of talkers | 6 | ISM signals are generated so that different objects are uttered by different talkers (one each of 3 male and 3 female talkers). All talkers are tried to be used equal amount. |
| Number of speech samples | 7 | 6 for tests + 1 for preliminaries per category |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Background signal (SBA) input loudness | 1 | -36 LKFS ([ITU-R BS.1770-4]) |
| ISM speech input loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | DCR with instructions according to IVAS 8a [3] |
| Languages | 1 | [tbd] |
| Listening System | 1 | High-quality headphones, diotic presentation |
| Listening Environment | 1 | No noise |

Table 2 Sample Categories

| Category | Number of objects | Speech Level [dB] | Background signal type** | Background Level | Overtalk [s] | Talker positions |
| --- | --- | --- | --- | --- | --- | --- |
| cat 1 | 1 | -26 | Indoors 1 | [-36] | No overtalk | 2 fixed, 4 with movement |
| cat 2 | 2 | -26 | Indoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 3 | 3 | -26 | Outdoors 1 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 4 | 4 | -26 | Outdoors 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |
| cat 5 | 2 | -26 | Background with music 1 | [-36] | No overtalk | 2 fixed, 4 with movement* |
| cat 6 | 3 | -26 | Background with music 2 | [-36] | Overtalk | 2 fixed, 4 with movement* |

*for 2 samples one ISM is moving, for the last 2 samples two or more objects are moving. For practice sample one ISM is moving.

** Background type signal is HOA3

Table 3 Test conditions for P.SUPPL800 OSBA, speech and background environments.

| Label | Condition | Bitrate [kbps] | FER/float | Notes |
| --- | --- | --- | --- | --- |
| c01 | Reference IVAS_rend | - | - |  |
| c02 | MNRU Q=34 dB | - | - |  |
| c03 | MNRU Q=30 dB | - | - |  |
| c04 | MNRU Q=26 dB | - | - |  |
| c05 | MNRU Q=22 dB | - | - |  |
| c06 | ESDRU α=0.8 | - | - |  |
| c07 | ESDRU α=0.6 | - | - |  |
| c08 | ESDRU α=0.4 | - | - |  |
| c09 | ESDRU α=0.2 | - | - |  |
| c10 | OSBA | 32 | Float |  |
| c11 | OSBA | 64 | Float |  |
| c12 | OSBA | 128 | Float |  |
| c13 | OSBA | 256 | Float |  |
| c14 | OSBA | 32 | 5% FER |  |
| c15 | OSBA | 64 | 5% FER |  |
| c16 | OSBA | 256 | 5% FER |  |
| c17 | OSBA | 13.2 | No errors |  |
| c18 | OSBA | 16.4 | No errors |  |
| c19 | OSBA | 24.4 | No errors |  |
| c20 | OSBA | 32 | No errors |  |
| c21 | OSBA | 80 | No errors |  |
| c22 | OSBA | 160 | No errors |  |
| c23 | OSBA | 384 | No errors |  |
| c24 | OSBA | 512 | No errors |  |
| c25 | OSBA | 48 | No errors | C31 |
| c26 | OSBA | 64 | No errors | C32 |
| c27 | OSBA | 96 | No errors | C33 |
| c28 | OSBA | 128 | No errors | C34 |
| c29 | OSBA | 192 | No errors | C35 |
| c30 | OSBA | 256 | No errors | C36 |
| c31 | ISM + SBA (HOA3) | 24.4 + 24.4 | No errors |  |
| c32 | ISM + SBA (HOA3) | 32 + 32 | No errors |  |
| c33 | ISM + SBA (HOA3) | 48 + 48 | No errors |  |
| c34 | ISM + SBA (HOA3) | 64 + 64 | No errors |  |
| c35 | ISM + SBA (HOA3) | 96 + 96 | No errors |  |
| c36 | ISM + SBA (HOA3) | 128 + 128 | No errors |  |

# Summary

The specific proposal outlines a P.SUPPL800 DCR experiment for the IVAS OSBA combined format to be discussed and accepted in the IVAS-8b Permanent Document [1].

# References

[1]	S4aA230127-IVAS Permanent Document IVAS-8b - Test Plan for Characterization Phase, Version v.0.1.2

[2]	Tdoc S4aA23013 - Characterization of Combined formats, Nokia Corporation

[3]	S4aA-230086: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.1.1.0