Source:	NTT

Title:	Integration of RTC-X in RTC Architecture

Agenda Item:		10.10

Document for:		Discussion and Agreement

Abstract:

This discussion paper provides supplementary explanations regarding the integration of service control functions and RTC-X reference point into RTC architecture for the proposed new Study Item (FS_iRTCW_Ph2), "Enhancements of RTC architecture variant for collaboration scenario 3," as proposed in SA4#128.

- Introduction

According to 3GPP TR 26.930, Service control is a set of the features for RTC services, and its functionalities and possible APIs are documented in the TR. Service control can be used in scenarios where the RTC Application Provider utilizes the operator network functional blocks in RTC AS (e.g., WebRTC Signalling Function and Media Function) to provide RTC services.

The functionality of service control is assumed to be utilized by Content Provider (CP) through APIs that the Application Supporting Web Function (ASWF) within the operator's network's RTC AS exposes to the CP, as stated in 3GPP TR 26.930. However, the interface between ASWF and CP is currently FFS as RTC-X, and it is encouraged to organize early-stage discussions for future architectural extensions considering the current discussion on the common elements between RTC and Media Streaming in 3GPP TS 26.510.

This discussion paper provides supplementary explanations regarding the integration of RTC-X and service control functions, which are being considered in the proposed new SID, and presents a plan how to proceed the integration.

2. Discussion

2.1. Motivation for service control

RTC Application Providers that utilize operator assistance are referred to as Content Providers (CP) in 3GPP TR 26.930. By utilizing the service control functionalities provided by the operator, CPs can request the operator network functions to operate according to the requirements of the RTC services that CPs intend to provide. Service control also enables CPs to receive the notifications of the status changes of the resources allocated for CPs' services within the operator network.

Specifically, service control refers to RTC service-specific control functionalities such as:

- Flexible configuration of the rules for media and data forwarding performed in RTC AS based on RTC service requirements (such as video and audio routing for regular conference rooms, asymmetric media distribution for one-to-many webinars, private chats between participants).

- Allocating and releasing resources in RTC AS corresponding to VR spaces

- Controlling user connections for each VR space

- Event notification (e.g., changes in corresponding resources' availability in RTC AS, connection of new users).

In RTC services, it is necessary to manage complex media/data flows between multiple WebRTC endpoints, as well as the connections to individual media data and the state management of VR space.

The motivation behind introducing the service control functionality and its associated interface, RTC-X, is to present these service controls as APIs accessible from the CP. This enables the realization of RTC services aligned with the requirements of the CP by using operator assistance.

2.2. Options for architectural enhancement realizing RTC-X

2.2.1. General

Two options for reference points equivalent to RTC-X between CP and ASWF are compared in following clauses.

2.2.2. Option 1: RTC-1 with RTC-3

2.2.2.1. General

In terms of provisioning session, service control seems to have similarities with Provisioning Session. Since Provisioning Session in RTC is provided via RTC-1, RTC-X mapped to RTC-1.

The service control requests sent via RTC-1 are received by the RTC AF. To relay the requests to the RTC AS which ASWF belongs to, functional extensions of RTC AF and extensions of RTC-3 are required.

Fig 2.2.1-1: Architectural enhancement for option 1

In Option 1, service control procedures are performed through RTC AF. Below is an example call flow for when CP requests resource allocation corresponding to VR space to ASWF.

Fig 2.2.1-2: Option 1 example service control flow

2.2.2.2. Evaluation

RTC-1 is already available for use. However, to relay requests from RTC AF to the ASWF in RTC AS, there is a need for specifications corresponding to RTC-3, which is currently not covered by existing specification. Furthermore, enhancements to the functionality of RTC AF are necessary.

Also, due to the following reason, it is anticipated that service control will be implemented as a separate procedure rather than an extension of Provisioning Session. As understood from the content described in clause 4.2.2 and 4.2.4 of TS 26.506, RTC sessions is controlled via the WebRTC Signalling Function in RTC architecture, and information from the Provisioning Session is used by the RTC MSH to acquire necessary information when initiating session establishment. However, RTC applications operating within web browsers do not have reference point and access to RTC-MSH. Integrating Provisioning Session, which manages information acquired by the MSH, with service control seems not reasonable for comprehensive RTC applications.

Therefore, as service control operates independently from Provisioning Session, a new functional block for RTC AF distinct from the Provisioning Function within RTC AF is believed to be required to relay service control requests.

2.2.3. Option 2: RTC-2

2.2.3.1. General

In analogy of Media Delivery architecture, RTC-2 is potential interface between CP and RTC-AS although it is not currently specified in TS 26.506. In this option, RTC-X will be mapped to the interface.

Fig 2.3.1-1: Architectural enhancement for option 2

In Option 2, service control procedures are performed directly on the RTC AS. Below is an example call flow for when CP requests resource allocation corresponding to VR space to ASWF.

Fig 2.3.1-2: Option 2 example service control flow

2.2.3.2. Evaluation

Although not currently specified, it is anticipated that the interface between RTC Application Provider and RTC AS will be designated as RTC-2, considering comparisons with M2.

In this option, Stage-2 extensions in the functional block are only performed on ASWF. Additionally, direct request-response between the related functional blocks allows avoiding redundant extensions across multiple interfaces.

2.2.4. Summary

Considering the extensions required for Stage-2 specifications in both options, Option 1 appears to involve numerous and redundant extensions, making it less efficient compared to the other option. Furthermore, as mentioned in clause 2.2.2.2, it is anticipated that Provisioning Session and Service Control will remain separate APIs rather than being integrated. Then, it is believed that there is no need to map them to RTC-1.

Also, considering the need to harmonize the current functionality deployment of the RTC architecture within the mapping of RTC-X, Option 2 is thought to be the more concise and suitable.

Therefore, RTC-2 is thought to be better option for RTC-X integration.

3. Proposal

It is proposed that the new study item (FS_iRTW_Ph2) consider the integration of RTC-X based on the summary of the discussion. For integrating RTC-X as RTC-2, followings are encouraged to be addressed in the new study:

-	Identifying the definition of RTC-2 for TS 26.506

-	Adjusting the name of RTC-2 (currently named as "media hosting interface") for TS 26.113

-	Clarifying the relation between RTC-2 and M2, and to make a consensus with MBS group

Appendix

Here is the RTC Architecture and Media Delivery Architecture in Release 18.

Fig 1: Referenced RTC architecture specified in 3GPP TS 26.506

Fig 2: Referenced Media Delivery architecture specified in 3GPP TS 26.501