3GPP TSG-SA WG4 Meeting #129e	S4-241769

# Only – 19-23 July 2024																 revision of S4-241507

Source:	InterDigital Europe, Tencent

Title:	Haptics input formats

Spec:	3GPP TR 26.854

Agenda item:	15.11

Document for:	Agreement

1. Introduction

This contribution proposes a first text and information on Haptics input format.

2. Reason for Change

Provide initial text for Haptics input formats to populate section 6, some references for section 2 and abbreviations in section 3 of TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

-

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

LRA	Linear Resonant Actuators

ERM	Eccentric Rotating Mass

* * * Third Change * * * *

# 6	Haptic input formats and device types

## 6.1	introduction

Haptics refers to the sense of touch, and encompasses the generation, manipulation, and perception of tactile sensations, forces, and motions. In the media domain it relates to the capture and rendering of physical information about objects and environments due to interactions with the user. The capture is generally performed with sensors and the rendering with hardware actuators allowing to render different modalities.

In general, three types of modalities are considered, tactile (vibration, temperature, pression), kinaesthetic (force) and proprioception (motion, acceleration). Those modalities are rendered in the human body through different mechanoreceptors. The density and properties of those receptors are different depending on the modality and the body parts. In addition, each individual has different sensibility, which can be managed through users’ profiles and adaptation.

Finally, the rendering is performed with specific hardware actuators embedded into user devices. The different Haptic modalities are rendered with different actuators. Typically, tactile sensations can be rendered with vibrotactile (e.g. linear resonant actuators (LRA), eccentric rotating mass (ERM), temperature (piezzo), wind (fan), force) … devices.

## 6.2		Haptic input formats

### 6.2.1	Introduction

Haptic input formats are essential in capturing and transmitting tactile sensations in a variety of applications, ranging from simple vibrations in mobile devices to complex force feedback in virtual reality systems. Two types of input formats exist, time-sampled signals (typically PCM encoded) captured by sensors, and parametric description of haptic effects.

* * * End of Changes * * * *