3GPP TSG-SA4 Meeting #131	S4-250017

# Geneva, , 17th Feb 2025 - 21st Feb 2025									revision of S4aI250024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0006 | rev | 1 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Selective Unicast Requests in MBS/MBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 AMD study, several cases for potential hybrid services have been identified, and are only partially addressed: - In-session unicast repair as documented in clause 5.9 of TR 26.802, - Application Services including hybrid services, as documented in clause 5.11.3.2 of TR 26.802, - Reporting and metrics as documented in clause 5.11.3.4 of TR 26.802, - Support for DRM in clause 5.10 of TR 26.804, in particular as mentioned in clause 5.10.4: "The mapping to 5GMS via MBS/MBMS is for further study", - Common Media Client Data as introduced in clause 5.16 of TR 26.804, in particular as mentioned in clause 6.16, it is recommended "to further study the usage of CMCD when 5GMS is deployed over MBS and/or MBMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This clause addresses the above aspects generalizes the architectural and procedural aspects when combining unicast and broadcast. This aspect also allows to add new functionalities that require selective unicast requests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Then the open issue on reporting and metrics collection from applications in MBS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.12 (new), 8.4.2, 8.4.3, 8.4.4, 8.4.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI250024; [FS_AMD] Selective Unicast Requests in MBS/MBMS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: none Presenter: Thomas Stockhammer Online Discussion: (January 9 2025) - Richard: Little reluctant to add a new key issue. We have agreed on a few topics already. Technical content is good. What are the implications of postponing this to the next release? Prefer to postpone to next release. - Thomas: Point of MBS moving independent is not good. Idea was always to use MBS as a framework with feature match to MBMS. Today we are not using MBMS for hybrid services. There is interest from 5G-MAG. Procedurally, no argument - Richard: I think we may be rushing. I’ll be comfortable taking slowly. - Thomas: This is in line with existing study in terms of timeline. I hoped to have technical discussion on this - Julien: In addition to existing examples on CMCD and watermarking, we prefer to include example on targeted advertising and fast startup. - Thomas: These are all good use cases. I hoped we could do in 3GPP in this release - Fred: There is another possibility to attempt during Feb 6 call - Richard: Have a problem to accept now because we are overloaded. At high level, this seems reasonable. But need to look into details. Decision: - January 9 2025: no agreement. noted. S4aI250024 is noted. This contribution addressed some of the above comments. Currently it is still proposed to address this in this release. If we can document the issue in the TR and refrain from making it a Rel-19 work, maybe the CR could be added. |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[A331]	ATSC A/331, "Signaling, Delivery, Synchronization, and Error Protection".

[103972]	ETSI TR 103 972: "Deployment Guidelines for DVB-I services over 5G Systems".

[26804]	3GPP TR 26.804: " Study on 5G media streaming extensions"

[AWS-WM]	Kevin Yao, "Combining dynamic ad insertion and A/B watermarking", 22 FEB 2023 in AWS Elemental MediaTailor, Direct-to-Consumer & Streaming, Media & Entertainment, Media Services, available here: https://aws.amazon.com/blogs/media/combining-dynamic-ad-insertion-and-a-b-watermarking/

[104002]	ETSI TS 104 002: " Publicly Available Specification (PAS); DASH-IF Forensic A/B Watermarking An interoperable watermarking integration schema".

[DSS-2023]	DASH-IF Special Session: "A/B Watermarking", February 2023, slides available here: https://github.com/Dash-Industry-Forum/Dash-Industry-Forum.github.io/files/10720660/DASH-IF_ServerSideWM_2023.pdf

[CTA5007]	CTA 5007-A: "Web Application Video Ecosystem - Common Access Token"

[WM-API]	DASH-IF Candidate Specification, "Watermarking Encoder API", Community Review version available here https://dashif.org/news/cr-watermarking-api/

## =====  CHANGE =====

## 5.12	Key Issue #11: Selective Unicast Requests in MBS and MBMS

### 5.12.1	Description

In several deployment scenarios and specifications, the distribution of unicast streaming content such as DASH/HLS/CMAF content over a broadcast/multicast system is considered. Examples include, but are not limited to:

-	5GMS via eMBMS (see TS 26.501 [9], clause 4.9 and 5.10)

-	5GMS via MBS (see TS 26.501 [9], clause 4.11 and 5.12)

-	DVB ABR Multicast in ETSI TS 103 769 [12]

-	ATSC3.0 specification in A/331 [A331]

-	ETSI TS 103 720 [31] for 5G Broadcast

-	ETSI TR 103 972 [103972] for DVB-I over 5G Broadcast

Hybrid services have been well established, and examples include alternative languages requested over unicast, targeted ad insertion, coverage extensions, and several others.

In the context of the Rel-19 AMD study, several cases for potential hybrid services have been identified, and are only partially addressed:

-	In-session unicast repair as documented in clause 5.9 of this document,

-	Application Services including hybrid services, as documented in clause 5.11.3.2 of this document,

-	Reporting and metrics as documented in clause 5.11.3.4 of this document,

-	Support for DRM in clause 5.10 of TR 26.804 [26804], in particular as mentioned in clause 5.10.4: "The mapping to 5GMS via MBS/MBMS is for further study",

-	Common Media Client Data as introduced in clause 5.16 of TR 26.804 [26804], in particular as mentioned in clause 6.16, it is recommended "to further study the usage of CMCD when 5GMS is deployed over MBS and/or MBMS.

This clause addresses the above aspects generalizes the architectural and procedural aspects when combining unicast and broadcast. This aspect also allows to add new functionalities that require selective unicast requests.

In hybrid media cases, the Media Player may simultaneously request and consume resources from a unicast server (CDN) and a local Media Server, attached to the UE. In the example shown in Figure 5.12.1-1 taken ETSI TR 103 972 [103972], three options are considered, how the combination of unicast and multicast are carried out to support hybrid services.

1.	The broadcast receiver accesses unicast network

2.	The DASH client selects content from different networks

3.	The application (in this case DVB-I client) selects content from different networks

Figure 5.12.1-1: Different options to combine multicast/broadcast with unicast (from ETSI TR 103 972 [103972])

Many streaming services and experiences nowadays have a certain amount of personalization. In this case, two aspects are of importance:

-	Requests from the Media Player may include personalized information such as identifiers for the user, specific tokens, device identifiers, tracking data or other client metadata such as defined in CMCD.

-	Responses to requests may be customized by the CDN or some edge server based on information included in requests.

Prominent examples are provided in Table 5.12.1-1.

Table 5.12.1-1: Examples for personalized unicast requests and responses

| Customized requests | Customized responses |
| --- | --- |
| Ad tracking and beaconing | Monetization incentives, ad skipping possible |
| DRM and License keys requests | License keys |
| Common Media Client Data | - |
| Identifiers used by Ad Decision systems | Targeted dynamic ad content |
| Tokens used by A/B Watermarking schemes | Variants of A/B watermarking |

For more details on A/B watermarking schemes and Ad Insertion, see for example [AWS-WM]. To fully support 5G Media Streaming and general media streaming services over MBS/MBMS, the above functionalities need to be supported as well in a hybrid manner.

Two high-level solutions may be considered to address the handling unicast requests.

- Solution 1: Media Player handles unicast requests

-	The manifest discriminates requests to the local media server (fed by broadcast/multicast) and the unicast requests by creating different service locations and typically different URLs

-	The Media Player handles the unicast requests directly to the network and deals with request issues

-	Generally, this is the preferred option for several use cases (alternative language requests, handoff to unicast for coverage extensions, etc.), but there are limitations

-	If the request to a broadcast resource is bundled with unicast related data, the information terminates in the local Media Server (CMCD, Tokens, etc.)

-	If the differentiated request needs to be obfuscated to the client and the response logic is provided be a network server, instead of done by the client.

-	Scalability, too many requests may go to unicast, so only subset needs to be addressed through unicast.

- Solution 2: MBS/MBMS/5G Broadcast client handles unicast request

- In this case, the Media Player sends conditioned requests to the local or gateway server

- The local/gateway server not only serves the requested resource, but also processes the additional information provided with the request and potentially issues unicast requests based on the resources.

-	The requested resource may be served from the broadcast distribution, or it may be served by issuing a unicast request or, as in the case of unicast repair, it may even be a combination of the two.

Solution 1 is well established in MBS through differentiating URLs in the manifest. For high-level solution 2, no mechanism is yet provided. The remainder of this clause addresses a mapping of the solution 2 to MBS/MBMS architectures. The solution may be combined with 5G Media Streaming, or may support third-party media streaming services.

5.12.2	Collaboration scenarios and architecture mappings

#### 5.12.2.1	General

In order to address the functionality of handling a combination of personalized requests from media players, a new function in the Multicast Broadcast client is added that collects information and may communicate with the unicast Application Provider. Figure 5.12.2.1-1 provides a basic overview of the considered architecture, initially independent of a specific implementation for MBS or MBMS.

Figure 5.12.2.1-1: Extension to Multicast Broadcast client to support selected unicast requests

In this case, the application provider configures the MC/BC network function for distribution. In addition, it provides a configuration instruction, such that clients Multicast Broadcast clients and be configured, such that they manage personalized requests and responses from the Media clients.  This configuration information is provided in a service announcement to the MC/BC client. The MC/BC client uses the request information, possibly adapts the responses and possibly selectively requests information from the unicast server

#### 5.12.2.2	Example 1: Common Media Client Data (CMCD)

A detailed description of Common Media Client Data (CMCD) as well as for the integration into 5GMS is provided in TR 26.804 [26804], clause 5.16. A basic overview of CMCD is provided in Figure 5.12.2.2-1 illustrating header option in green and query option in red. With every HTTP request, typically and primarily for the request of a media segment, some custom data is sent either as HTTP header or as query string appended to the segment request URL.

Figure 5.12.2.2-1: CMCD basic architecture and call flow.

In the call flow adapted to 5G Media Streaming as shown in Figure 5.12.2.2-2, the information is collected in the 5GMSd AS (network server) and then exposed to the AF by aggregating the information. The information in itself may be sent to the NWDAF or used for optimizing delivery.

Figure 5.12.2.2-2: High-level call flow for CMCD in 5G Media Streaming according to clause 5.16.

In a mapping to a multicast broadcast distribution and to the architecture provided in clause 5.12.2.1, the instantation and adaptation for CMCD shown in Figure 5.12.2.2-3 may be considered.

Figure 5.12.2.2-3: CMCD Adaptation to Multicast/Broadcast.

In the case above the general architecture and call flow is adapted to configure the DASH/HLS for CMCD data collection (e.g. in the manifest). The client requests data with CMCD configuration. The MC/BC client is configured accordingly to collect the data.  The MC/BC client provides the aggregated information to the network function, for example NWDAF – note that this would then align with unicast.

#### 5.12.2.3	Example 2: A/B Watermarking

In order to also provide a more recent mapping not even discussed yet for 5G Media Streaming, this clause provides a mapping of A/B Watermarking (WM) to Multicast Broadcast. As an example, A/B Watermarking is specified in DASH-IF ETSI TS 104 002	 [104002]. For more background, there was also a DASH-IF Special session in February 2023 with information available here [DSS-2023].

In this case, forensic watermarking is used to embed an imperceptible unique identifier within each content copy for the purpose of tracking individual copies to deter unauthorized redistribution. Forensic watermarking remains in the content after the other content protection features are removed (i.e., after DRM/CAS decryption is applied, after the content has left the Secure Video Path, etc.). Main market drivers are live sports and premium VoD. The major industry trend is server-side watermarking as it minimizes the trade off for cache overhead and watermarking and does not require integration into the client. The basic architecture is showin in Figure 5.12.2.3-1. In this case, a transcoder generates two variants (A and B variant), and both variants are uploaded to an origin server. Each client gets a assigned a watermarking token, and based on this token, the intelligent edge server distributes either the A or B variant for each segment.

Figure 5.12.2.3-1: Server-side A/B Watermarking

ETSI TS 104 002 defines relevant exchange formats along the above architecture to support A/B watermarking, primarily to support the A/B routing logic in the edge server, namely:

-	A watermark token containing a unique A/B pattern transmitted to the Edge by the device

-	an explicit metadata stream (WMPaceInfo metadata) from the encoder to the edge to signal which variant is associated to a given egress segment

The formalized exchange of information is shown in Figure 5.12.2.3-2.

Figure 5.12.2.3-2: Formalized information exchange for A/B watermarking

The WM token follows the logic of a CDN Access Token (CAT) as defined in CTA-5007 [CTA5007]. The token carries the WM vendor and either may directly carry the WM pattern, or indirectly with parameters to generate the WM pattern. In the above diagram, the edge having available both variants, decides based on the WM token received from the device, which variant to send to the client, A or B. The WMPaceInfo provides the information to edge server, on which variant to deliver to the client. ETSI TS 104 002 provides full semantics and syntax for WMPaceInfo. Different options are considered to delivery the information, for example inband SEI messages, HTTP headers, ISO BMMF boxes or sidecar files. An additional specification is provided with the DASH-IF Watermarking Encoder API [WM-API] that specifies APIs for the encoder/watermarking integration.

NOTE: The integration of A/B Watermarking into 5G Media Streaming is for further study.

In a mapping to a multicast broadcast distribution and to the architecture provided in clause 5.12.2.1, the instantation and adaptation for A/B watermarking shown in Figure 5.12.2.3-3 may be considered.

Figure 5.12.2.3-3: A/B Watermarking Adaptation to Multicast/Broadcast.

In this case, the application provider provides the WMPaceInfo to the Mulicast/Broadcast network function, and this information is included into the service announcement. Further, only one variant is distributed over broadcast, the other variant(s) are on unicast only. In the case the information on mapping requests to variants is provided through the side car info data shared to the client (via service announcement config). In combination with the unicast token, provided typically with the manifest in a personalized request, the Multicast/Broadcast client decides whether it uses the information from the MC/BC distribution, or if it selectively requests the content variant via the unicast server.

In order to minimize the required unicast data, some variants may be considered:

-	The differentiating watermarking data may be concentrated to some byte range of the segments. In this case, not the entire segment may be requested from unicast, but only a well-defined byte range and this byte range may replace the portion of the broadcast encoding. Note that this would require a watermark that does not propagate over predicted frames.

-	Typically, in unicast A/B variants may be selected with equal likelihood. However, if only the A variant is distributed over broadcast, the other variants should be selected less often to minimize unicast traffic. This obviously reduces the amount of possible unique watermark patterns. It is a deployment choice to balance the duration of unique patterns and the detection speed in forensic watermarking and the unicast requests.

-	In another option, more than 2 variants could be created. This allows to have more variants for the unicast request to create more unique sequences

This example shows that client or edge devices can be configured properly in order to selectively apply unicast requests for specific personalized information from regular Media Players.

#### 5.12.2.4	Mapping to MBS User Services

Based on the general introduction and the examples in this clause, the MC/BC client as shown in Figire 5.12.2.1-1 issues selective requests for unicast based on information from the media player and the configuration received in the service announcement. The selective requests may combine information in

-	http headers, query strings, or based on a pattern in the request URL

-	With information provided in the configuration information, for example a filter when to issue unicast requests

-	Unicast requests may also be issued based on certain randomization

The selective requests may be forwarded as is or may be modified by the MC/BC client The configuration needs to take into account the ability to scale the unicast traffic.

In TS 26.502, the MBS AS is already defined, but at this stage only has a single functionality, namely object repair. In order to support selective unicast delivery, the MBS AS is extended to host data not only for object repair.

NOTE: 	the physical location of the MBS AS may be collocated with a unicast server that is also used for regular unicast communication. The MBS AS is primarily considered a logical function.

Figure 5.12.2.4-1 now extends the MBS AS to support selective unicast requests.

Figure 5.12.2.4-1: Extensions to MBS User Services network architecture: Selective unicast functionality in MBS AS

The above extension does not yet include the proposed extensions in considered in clause 5.9, namely definition of the reference point between MBSTF and MBS AS. However, it is considered a generalization of the discussion in clause 5.9. In addition, the client architecture may stay primarily unmodified, but some of the extensions are provided.

Figure 5.12.2.4-2: Extensions to MBS User Services reference architecture: Selective unicast functionality in MBSTF client

In Figure 5.12.2.4-2, the MBS User Services reference architecture is extended also in the MBSTF client to support selective unicast requests. In this case, the MBSF client may configure the logic for selective unicast requests via MBS-6', and the media server in the MBSTF client may also provide a configuration API, for example which requests are served via unicast or which ones are served via MBS-4-MC.

The extensions for the high-level baseline procedures in TS 26.502 [26502] are a generalization to the procedure extensions in clause 5.9 for selective unicast requests and are provided in Figure 5.12.2.4-3 in bold.

NOTE:	In the interests of brevity, the prefix MBS is omitted from the numbered steps in the figure.

Figure 5.12.2.4-3: Extended MBS User Service high-level baseline procedures

The following steps are updated or extended compared to the original high-level call flow:

-	The user service provisioning step 1 includes selective unicast provisioning

-	The distribution session provisioning step 2 includes selective unicast provisioning

-	A new step NEW1 (after step 2) is added to provision selective unicast ingest

-	The user service announcement step 3 is extended to include instructions for selective unicast

-	A new step NEW2 (after step 5) is added to ingest selective unicast request content to the MBS AS

-	Step 8 and 9 are extended to provision the MBSTF client to support selective unicast.

-	A new step NEW3 is added (after step 10) to support personalized requests from the application

-	Step 11 is extended such that in the distribution session also uses the MBS AS selectively for unicast requests.

#### 5.12.2.4	Example 3: Targeted Ad Insertion

The integration of targeted ad insertion is for further study based on potential enhancements in 5G Media Streaming.

NOTE: 	ISO/IEC 23009-1 6th edition addresses details of this functionality for DASH as Alternative MPD Events and is aligned with HLS interstitials

#### 5.12.2.5	Example 4: Fast Start-up using unicast

The integration of fast start-up with is for further study based on potential enhancements in 5G Media Streaming.

NOTE: 	ISO/IEC 23009-1 6th edition addresses details of this functionality for DASH using Segment Sequences.

### 5.12.3	Gap analysis and requirements

The following aspects are identified to be missing:

1)	Formal definition of a named reference point between the MBSTF and the MBS AS in order to publish objects to the MBS AS for the purpose of selective unicasts.

2)	User service provisioning to support selective unicast requests.

3)	Distribution session provisioning to support selective unicast requests.

4)	Provisioning of the MBS AS in order to support selective unicast requests.

5)	User service announcements to support provisioning for selective unicast requests for different purposes.

6)	Ingest session to support hosting unicast data on MBS AS

7)	Functional extension of MBSTF Client to support selective unicast requests based on application requests and configuration information in user service information.

8)	Support for personalized requests from application to MBS client and filtering those requests by MBSTF client.

9)	Support for selective unicast requests from MBSTF client to MBS AS.

### 5.2.4	Candidate solutions

In order to address the gaps identified in clause 5.12.3, the following solutions may be considered

1)	Formal definition of a named reference point between the MBSTF and the MBS AS in order to publish objects to the MBS AS for the purpose of selective unicasts.

-	The same reference point as defined for object repair in clause 5.9 is expected to be used.

2)	User service provisioning to support selective unicast requests.

-	the provisioning may be supported by template functions that are otherwise provided to a proxy/edge server to configure personalized requests.

3)	Distribution session provisioning to support selective unicast requests.

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

4)	Provisioning of the MBS AS in order to support selective unicast requests.

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

5)	User service announcements to support provisioning for selective unicast requests for different purposes.

-	the provisioning may be supported by forwarding a template function that are otherwise provided to a proxy/edge server to configure personalized requests.

6)	Ingest session to support hosting unicast data on MBS AS

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

7)	Functional extension of MBSTF Client to support selective unicast requests based on application requests and configuration information in user service information.

-	the functionality may be supported by providing a template to process special application requests.

8)	Support for personalized requests from application to MBS client and filtering those requests by MBSTF client.

-	no specific extensions are needed except for parameters that are processed, including http header parameters, query parameters or other parts of HTTP requests.

9)	Support for selective unicast requests from MBSTF client to MBS AS.

-	no specific extensions are needed except for parameters that are processed, including http header parameters, query parameters or other parts of HTTP requests.

### 5.12.5	Summary and conclusions

In order to address the functionality of handling a combination of personalized requests from media players, a new function in the Multicast Broadcast client is added that collects information and may communicate with the unicast Application Provider.

For stage-2 impact:

-	Gap#1-9 in clause 5.12.3 are expected to be addressed by:

a.	Defining a new reference point in TS 26.502 [29] (already considered in clause 5.9)

b.	Documenting call flows and procedures for selective unicast repair based on the details in clause 5.12.4

Stage-3 impact is expected to address gaps #1 to #9 based on the candidate solutions in clause 5.12.4 and the stage-3 aspects.

## =====  CHANGE =====

### 8.4.2	Recommendations for stage 2 normative work arising from version 19

It is recommended to provide relevant extensions to TS 26.502 [29] to extend the MBS User Service architecture based on the updated conclusions in clause 5. Candidates for these extensions are:

1.	For Key Issue #8: In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 and based on the conclusions in clause 5.9.7:

-	Address Gap #1 identified in clause 5.9.5 by the candidate solution described in clause 5.9.6:

i.	Define a new reference point in TS 26.502 [29] between the MBSTF and the MBS AS.

ii.	Document call flows and procedures for both post-session and in-session unicast repair.

2.	For Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 and based on the conclusions in clause 5.10.6:

a.	Fully specify support for the Joint BM-SC and MBSF Functionality. For this purpose, the gap identified in clause 5.10.4.1 of the present document needs to be addressed by documenting additional procedures and baseline parameters as required in TS 26.502 [29] and permitting the signalling of MBMS sessions.

b.	Document in an informative annex to TS 26.502 [29] the deployment architectures, client architectures and high-level call flows in clauses 5.10.2.3 and 5.10.2.4.

3.	For Key Issue #10: Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 and based on the conclusions in clause 5.11.4:

a.	Add the necessary functional extensions and call flows to support the generic Application Service as defined in clause 7.6 of TS 26.346 [16] based on the discussion in clause 5.11.3.2,

b.	Add the necessary functional extensions and call flows to support partial file handling as defined in clause 7.9 of TS 26.346 [16] based on the discussion in clause 5.11.3.3,

c.	Add the necessary functional extensions and call flows to support reporting of metrics based on the discussion in clause 5.11.3.4,

d.	Add the necessary functional extensions and call flows to support time Synchronization as defined in TS 26.346 [16] in clause 4.6 based on the discussion in clause 5.11.3.6.

4.	For Key Issue #11: Selective Unicast Requests in MBS and MBMS as introduced in clause 5.12 and based on the conclusions in clause 5.12.4:

a.	Documenting call flows and procedures for selective unicast repair based on the details in clause 5.12.4

## =====  CHANGE =====

### 8.4.3	Recommendations for stage 3 normative work arising from version 19

It is recommended to provide relevant extensions to MBS User service protocols and formats specified in TS 26.517 [30] based on the conclusions in clause 5 and the stage-2 extensions above, if applicable. Candidates for these extensions are:

1.	For Key Issue #8: In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 and based on the conclusions in clause 5.9.7:

-	Address Gaps #2, #3, #4, and #5 in clause 5.9.5 by the candidate solution in clause 5.9.6:

i.	On gap #2 identified in clause 5.9.5, both of the following signalling options are expected to be supported:

-	Using FDT parameters to signal the time when repairs can be requested using the Expires attribute).

-	Using LCT header information to signal the time when repairs can be requested using the B-Flag.

ii.	On Gap #3 identified in clause 5.9.5, the following signalling options exist in the FLUTE File Delivery Table (FDT):

-	Defining a new FDT extensions parameter to signal the availability time when the object needs to be released.

iii.	On gap #4 identified in clause 5.9.5, the execution of MBS object delivery and in-session unicast repair can run in parallel in the MBS Client. However, this should be validated if there are cases this is not the case and whether these cases need to be explicitly stated, for example reduced capability (RedCaP) UEs.

iv.	On gap #5 identified in clause 5.9.5, time synchronization can reuse functionalities defined in TS 26.346 [16], but tighter synchronization that 1 second. This work is aligned with the findings and work in clause 5.11.3.6.

2.	for Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 and based on the conclusions in clause 5.10.6:

-	Address the relevant stage-3 aspects based on stage-2 work.

3.	For Key Issue #10: Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 and based on the conclusions in clause 5.11.4:

a.	Address the relevant stage-3 aspects based on stage-2 work.

b.	Adapt the generic Application Service as defined in clause 7.6 of TS 26.346 [16] to MBS User Services,

c.	Adapt partial file handling as defined in clause 7.9 of TS 26.346 [16] to MBS User Services

d.	Adapt time synchronization as defined in clause 4.6 of TS 26.346 [16] to MBS User Services.

4.	For Key Issue #11: Selective Unicast Requests in MBS and MBMS as introduced in clause 5.12 and based on the conclusions in clause 5.12.4:

-	Stage-3 impact is expected to address gaps #1 to #9 based on the candidate solutions in clause 5.12.4 and the stage-3 aspects.

## =====  CHANGE =====

### 8.4.4	Recommendations for further study arising from version 19

It is recommended to continue the study of additional extensions to MBS User Services. Candidate topics based on the present document are:

1.	For Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 and based on the conclusions in clause 5.10.6:

a.	Validate the approaches by implementation, for example in 5G-MAG Reference Tools, and identify if the functionality is fully supported or any further specification updates are needed.

b.	Going forward, ensure that enhancements to the MBSTF and delivery methods in MBS can also be leveraged and deployed for eMBMS.

2.	For Key Issue #10: Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 based on the conclusions in clause 5.11.4:

-	Further study MBMS features that are not yet supported in based on the analysis in clause 5.11.2.

3.	For Key Issue #11: Selective Unicast Requests in MBS and MBMS as introduced in clause 5.12 and based on the conclusions in clause 5.12.4:

-	Study details of use cases including A/B watermarking, Ad insertion, CMCD reporting, etc..

## =====  CHANGE =====

### 8.4.5	Recommendations for coordination arising from version 19

It is recommended to coordinate work with other working groups and organizations as follows:

1.	For Key Issue #9: MBS User Service and Delivery Protocols for eMBMS as introduced in clause 5.10 and based on the conclusions in clause 5.10.6:

a.	Validate the approaches by implementation, for example in 5G-MAG Reference Tools.

b.	Validate with other working groups whether the TMGI allocation in the MBS User Service Announcement is achievable via MB-SMF.

2.	For Key Issue #10: Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 and based on the conclusions in clause 5.11.4:

-	Validate the approaches by implementation, for example in 5G-MAG Reference Tools.

3.	For Key Issue #11: Selective Unicast Requests in MBS and MBMS as introduced in clause 5.12 and based on the conclusions in clause 5.12.4:

-	Validate the approaches by implementation, for example in 5G-MAG Reference Tools.