3GPP TSG-SA4 Meeting #131	S4-250028

# Geneva, , 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0025 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Improvement to DRM and Conditional Access. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Spain |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The work and call flow presented in the initial document is not fully aligned with common practices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The call flow is updated to address the additional information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion for normative work phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.10.3, 5.10.4, 6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[187]	"Introduction to CPIX", Unified Streaming online blog, https://docs.unified-streaming.com/documentation/drm/cpix_intro.html

[188]	"What is DRM", Castlabs online documentation, https://castlabs.com/faq/drm/

## =====  CHANGE =====

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ABR	Adaptive Bit Rate

ACK	ACKnowledgment

ACM	Association for Computing Machinery

ALPN	Application-Layer Protocol Negotiation

AMF	Access and Mobility Management Function

ANBR	Access Network Bitrate Recommendation

ANTS	Advanced Network Technologies Symposium

API	Application Programming Interface

AQM	Active Queue Management

ARPA	Advanced Research Projects Agency

ARQ	Automatic Repeat reQuest

ASP	Application Service Provider

ATSSS	Access Traffic Steering, Switching, and Splitting

AUS	AUthentication Server

AVC	Advanced Video Coding

AWS	Amazon Web Services

BBC	British Broadcasting Corporation

BBR	Bottleneck Bandwidth and Round-trip propagation time

BDT	Background Data Transfer

BMFF	Base Media File Format

BMSC	Broadcast/Multicast Service Center

CAE	Content-Aware Encoding

CBR	Constant Bit Rate

CCF	Call Control Function

CDF	Cumulative Distribution Function

CDN	Content Delivery Network

CDP	Content Delivery Protocol

CERN	European Organization for Nuclear Research

CIRR	Carrier-Independent Routing Registry

CMAF	Common Media Application Format

CMAS	Commercial Mobile Alert System

CMCD	Common Media Client Data

CMMF	Coded Multisource Media Format

CMSD	Content Media Server Data

CPI	Content Protection Information

CPIX	Content Protection Information Exchange

CPT	Content Preparation Template

CRUD	Create, Read, Update, Delete

CTA	Consumer Technology Association

CWR	Congestion Window Reduced

DANE	DASH-Aware Network Element

DASH	Dynamic Adaptive Streaming over HTTP

DCSM	Data Collection and Storage Management

DNS	Domain Name System

DRM	Digital Rights Management

DSCP	Differentiated Services Code Point

DTT	Digital Terrestrial Television

DVB	Digital Video Broadcasting

DVR	Digital Video Recorder

EAS	Edge Application Server

ECE	Explicit Congestion Notification

ECN	Explicit Congestion Notification

ECP	Enhanced Content Protection

ECT	Explicit Congestion Notification-Capable Transport

EDGE	Enhanced Data rates for GSM Evolution

EDL	Enhanced Data Link

EEC	Edge-Enabled Client

EEL	End-to-End Latency

EES	Edge Enabler Server

EFDT	Enhanced File Delivery Table

EME	Encrypted Media Extensions

ENP	Enhanced Network Performance

EPS	Evolved Packet System

EPT	Enhanced Packet Transport

EVEX	EVent EXposure

FAR	Forward Action Rule

FDIS	Final Draft International Standard

FDT	File Delivery Table

FEC	Forward Error Correction

FHD	Full High Definition

FLUS	Framework for Live Uplink Streaming

FLUTE	File Delivery over Unidirectional Transport

FQDN	Fully Qualified Domain Name

GBR	Guaranteed Bit Rate

GRO	Generic Receive Offload

GSO	Geostationary Satellite Orbit

GTP	GPRS Tunneling Protocol

GUID	Globally Unique Identifier

HDR	High Dynamic Range

HEVC	High Efficiency Video Coding

HLS	HTTP Live Streaming

HTTP	Hypertext Transfer Protocol

HTTPS	Hypertext Transfer Protocol Secure

IANA	Internet Assigned Numbers Authority

ICCE	International Conference on Consumer Electronics

IEC	International Electrotechnical Commission

IEEE	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IMS	IP Multimedia Subsystem

IOP	Interoperability Points

IPC	Inter-Process Communication

IPTV	Internet Protocol Television

ISO	International Organization for Standardization

JSON	JavaScript Object Notation

JTC	Joint Technical Committee

KID	Key IDentifier

KPI	Key Performance Indicator

LPT	Low-Power Transceiver

LSD	Low-Speed Data

LTE	Long-Term Evolution

MABR	Multicast Adaptive Bit Rate

MANO	Management and Orchestration

MAR	Multi-Access Rule

MBMS	Multimedia Broadcast Multicast Service

MBS	Multicast Broadcast Service

MBSF	Multicast Broadcast Service Function

MBSTF	Multicast Broadcast Service Transport Function

MHV	Mile-High Video

MIME	Multipurpose Internet Mail Extensions

MNO	Mobile Network Operator

MoQ	Media over QUIC

MPD	Media Presentation Description

MPEG	Moving Picture Experts Group

MPQUIC	Multipath QUIC

MPTCP	Multipath TCP

MQTT	Message Queuing Telemetry Transport

MSE	Media Source Extensions

MTSI	Multimedia Telephony Service for IMS

NAT	Network Address Translation

NBMP	Network-Based Media Processing

NEF	Network Exposure Function

NFV	Network Functions Virtualization

NRF	Network Repository Function

NRM	Network Resource Model

NSACF	Network Slice Admission Control Function

NSCALE	Network Slice Capability Exposure

NSSAI	Network Slice Selection Assistance Information

NWDAF	Network Data Analytics Function

OAM	Operations, Administration, and Maintenance

OMA	Open Mobile Alliance

OTI	Object Transmission Information

OTT	Over-The-Top

PAS	Publicly Available Specification

PCC	Policy and Charging Control

PCF	Policy Control Function

PDF	Portable Document Format

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PFCP	Packet Forwarding Control Protocol

PFD	Packet Flow Description

PFDF	Packet Flow Description Function

PFS	Packet Flow Set

PHB	Per-Hop Behaviour

PING	Packet Internet Groper

PLMN	Public Land Mobile Network

PSA	Public Service Announcement

PSDB	Packet Set Delay Budget

PSER	Packet Set Error Rate

PSIHI	Packet Set Integrated Information

PSNR	Peak Signal-to-Noise Ratio

PSS	Packet-Switched Streaming

PSSH	Protection System Specific Header

QER	QoS Enforcement Rule

QFI	QoS Flow Identifier

QLOG	QUIC Logging

QoS	(Network) Quality of Service

QPACK	QUIC Header Compression

QRT	QUIC RTP Tunnelling

QUIC	Quick UDP Internet Connections

RAN	Radio Access Network

RFC	Request for Comments

RIST	Reliable Internet Stream Transport

RLC	Radio Link Control

ROM	Receive-Only Mode

RSFEC	Reed-Solomon Forward Error Correction

RTC	Real-Time Communication

RTCP	Real-Time Control Protocol

RTMP	Real-Time Messaging Protocol

RTP	Real-Time Protocol

RTT	(Network) Round-Trip Time

SAND	Server and Network Assisted DASH

SAP	Service Access Point

SBI	Service-Based Interface

SCTE	Society of Cable Telecommunications Engineers

SDAP	Service Data Adaptation Protocol

SDF	Service Data Flow

SEAL	Service Enabler Architecture Layer

SEI	Supplemental Enhancement Information

SIB	System Information Block

SIM	Subscriber Identity Module

SLA	Service Level Agreement

SMF	Session Management Function

SMS	Short Message Service

SMTP	Simple Mail Transfer Protocol

SRT	Secure Reliable Transport

SSD	Seek Start-up Delay

SST	Slice Selection Type

STS	Secure Token Service

SVC	Scalable Video Coding

SVTA	Streaming Video Technology Alliance

TBD	To Be Determined

TBF	Token Bucket Filter

TCP	Transmission Control Protocol

TEE	Trusted Execution Environment

TEID	Tunnel Endpoint Identifier

TLS	Transport Layer Security

TMGI	Temporary Mobile Group Identity

TOI	Transmission Object Identifier

TSG	Technical Specification Group

TSI	Transport Stream Identifier

TZM	Time Zone Management

UDP	User Datagram Protocol

UDR	Unified Data Repository

UHD	Ultra-High Definition

UMTS	Universal Mobile Telecommunications System

UPF	User Plane Function

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

URN	Uniform Resource Name

URR	Usage Reporting Rule

USIM	Universal Subscriber Identity Module

UTC	Coordinated Universal Time

UUID	Universally Unique Identifier

VBR	Variable Bit Rate

VSF	Video Services Forum

WAVE	Web Application Video Environment

WDD	Workflow Description Document

XML	Extensible Markup Language

ZSM	Zero-touch network and Service Management

## =====  CHANGE =====

### 5.10.3	Deployment Architectures

The core components of a DRM workflow are provided in figure 5.10.3-1 based on Part 6 of the DASH-IF Interoperability Points specification [144]. Similar workflows apply also for other streaming technologies, but DASH is used in the below analysis for illustrative purposes.

Figure 5.10.3-1: Core elements in content protection according to DASH-IF IOP Part 6 [144]

The definition of the functions is as follows:

-	Authorization Server: provides authorization tokens that may be required for requesting a license from a license server.

-	DASH client: a function using the Media platform and the DRM system to playback encrypted content.

-	DASH Presentation: A server hosting DASH resources, i.e. MPDs and Segments primarily, and includes information on the used DRM System.

-	License Server: A license server provides licenses that are data structures in a DRM system specific format that contains one or more content keys and associates them with a policy that governs the usage of the content keys (e.g. expiration time).

-	Media Platform: enables playback of encrypted content while protecting the decrypted samples and content keys against potential attacks.

-	DRM System: an implementation of content keys management cooperating with the device’s media platform to enable playback of encrypted content while protecting the decrypted samples and content keys against potential attacks, consisting of two main components: a license server and a DRM client.

-	DRM Client: processes licenses and enforcing the associated policies. Either the DRM client handles the decryption of samples, or the DRM client interacts with the hardware elements that address the decryption.

A DRM system cooperates with the device’s media platform to enable playback of encrypted content while protecting the decrypted samples and content keys against potential attacks. The same encrypted DASH presentation can be decrypted by different DRM systems if a DASH client is provided the DRM system configuration for each DRM system, either in the MPD or at runtime. A content key is a key used by a DRM system to make content available for playback. A content key and its identifier can be shared between all DRM systems, whereas the mechanisms used for key acquisition and content protection are largely DRM system specific. DASH adaptation sets are often protected by different content keys. The encapsulated content keys are typically encrypted and only readable by the DRM system.

A more detailed DRM workflow is provided in figure 5.10.3-2 based on the Content Protection Information Exchange Format (CPIX) as specified in ETSI TS 103 799 [143]. It complements Part 6 of the DASH-IF Interoperability Points specification [144] by putting more emphasis on the back-end aspects. The following additional functions are defined:

-	Content Provider: A publisher who provides the rights and rules for delivering protected media, also possibly source media (mezzanine format, for transcoding), asset identifiers, key identifiers (KID), content key values, encoding instructions, and content description metadata.

-	Encoder: A function that encodes media in a specified set of formats with different bitrates and resolutions etc., possibly determined by the publisher.

-	Packager/Encryptor: A function that who encrypts and packages media, inserting DRM signalling and metadata into the media files.

-	Manifest Creator: A function that generates the media manifests which group the various media files into a coherent presentation. These manifest files may contain DRM signalling information.

-	DRM Client: Obtains information from different sources: media manifest files, media files, and DRM licenses.

An example architecture is provided in figure 5.10.3-2.

Figure 5.10.3-2: Example backend architecture and workflow for encrypted live content
based on ETSI TS 103 799 [143]

In this case, content is continuously received, transcoded in the desired format and encrypted if any type of protection is required. One or multiple content keys can be used regardless of whether key rotation is used or not. Keys are generated by the encryption engine or the DRM system and are available to all DRM systems and encryption engines to support multi-DRM with a shared key The MPD Generator requests to the DRM systems their specific signalling, if any, to be added in the MPD. Encrypted segments and the media manifest can be uploaded on a CDN making it available to users.

Figure 5.10.3-3 illustrates the usage of the encrypted content in a realistic workflow comprising multiple cooperating components. In ETSI TS 103 799 [143], a standardised data format for content protection information exchange is defined, collected in a document that can be signed.

Figure 5.10.3-3: Incremental update and extension of the document based on ETSI TS 103 799 [143]

Also, in ETSI TS 103 799 [143] a workflow for use with CPIX is presented for which multiple producers are included. This workflow is shown in figure 5.10.3-4.

Figure 5.10.3-4: Multiple producer example steps based on ETSI TS 103 799 [143]

Based on the collection of data above, as well as further deployment guidelines, the following functions are defined when distributing DRM-protected content using multiple DRMs and common encryption:

-	DRM System: an implementation of content keys management cooperating with the device’s media platform to enable playback of encrypted content while protecting the decrypted samples and content keys against potential attacks, consisting of two main components: a license server and a DRM client.

-	Authorization Server: provides authorization tokens that may be required for requesting a license from a license server.

-	License Server: A license server provides licenses that are data structures in a DRM system specific format that contains one or more content keys and associates them with a policy that governs the usage of the content keys (e.g. expiration time). The license server interacts with the authorization server to verify user credentials and with the key server to obtain the necessary decryption keys. When a user requests to play DRM-protected content, the player's request is sent to the license server. The license server verifies the user's credentials and entitlements, and if authorized, it retrieves the decryption keys from the key server and issues a license to the player.

-	Key Server: The key server is responsible for generating, storing, and managing encryption keys used to encrypt the content. It provides the encryption keys to the encryptor during the content encryption process. These keys are then used to encrypt the content before distribution. The key server typically interacts with the encryptor and the license server to ensure that the correct keys are used for both encryption and decryption processes.

-	Encoder: The encoder converts raw video and audio content into a compressed format suitable for streaming. It typically generates multiple versions of the content at different bitrates to support adaptive streaming. It may also produce different variants of the content, for example an HD version, a UHD version, and HDR Version, etc. The encoder does not interact DRM system.

- 	Encryptor: The encryptor applies encryption to the encoded content using encryption keys. This ensures that the content is protected and can only be accessed by authorized users. The encryptor communicates with the Key Server to obtain the encryption keys needed to encrypt the content.

-	Packager: The packager receives encrypted content from the encryptor and formats it into streaming protocols like DASH (MPD) and HLS (M3U8). It also inserts DRM metadata into the media segments.

-	Media entry point creator: The media entry point creator generates manifest files (e.g., DASH MPD, HLS M3U8) that describe how the content is organized and how it should be played back. These manifest files include DRM metadata that informs the player about the encryption and how to obtain the decryption keys.

-	Content Hosting Server: A server hosting Streaming resources, i.e. Media Entry Points and Segments primarily, and includes information on the used DRM System.

-	Media Platform: enables playback of encrypted content while protecting the decrypted samples and content keys against potential attacks. The Media Platform summarizes the functions of media decapsulation, media decryption, media decoding and media rendering as defined in clause 4.2.2 of TS 26.501 [15].

-	DRM Client: processes licenses and enforcing the associated policies. Either the DRM client handles the decryption of samples, or the DRM client interacts with the hardware elements that address the decryption.

-	Media player: a function using the Media platform and the DRM system to playback encrypted content.

Also note that in many DRM workflows, only the key IDs (KIDs) are exchanged rather than the actual encryption keys. This approach enhances security by minimizing the exposure of the actual keys.

A typical example workflow of encrypted content is shown in figure 5.10.3-5.

Figure 5.10.3-5 Typical workflow for generating, distributing and playing back encrypted content

The following call flow is provided:

Content Preparation:

1.	A content provider provides encoding instructions and rules for using the content to the content preparation and hosting system

2.	The content provider provides raw content to the content preparation and hosting system.

Trust Establishment:

3.	The Encryptor/Packager and the DRM System (typically the License and Authorization Server as well as the Key Server) exchange public signing keys to establish a trusted communication.

Content Protection Information construction:

4.	The Packager generates initial content protection information that includes identification of the receivers and the various stream encoding criteria (usage rules).

5.	The Encryptor retrieves this information from the packager and requests one or several content keys from the key server by sending the signed content protection information to a key server.

6.	The Key server generates content keys according to the request. It adds these content keys to the content protection information and encrypts the content protection information and sends it to the encoder/packager.

7.	The encryptor/packager extracts the content keys and stores in a data base.

Content Protection Information distribution:

8.	The Encryptor/Packager sends the content protection information to the DRM System.

9. 	The License Server decrypts the content protection information and imports keys or key ids into its data base.

10.	The License Server sends the updated content protection information to the Encryptor/Packager and the Manifest Creator.

Presentation manifest and media segment generation:

11.	The Manifest Creator generates the presentation manifest (e.g. DASH MPD) and adds the content protection information (DRM System specific information, key identifiers, etc.).

12.	The Manifest Creator uploads the presentation manifest to the Content Hosting.

13.	The Encryptor/Packager generates encrypted segments and adds the content protection information (e.g. DRM System specific information).

14.	The Encryptor/Packager provides the encrypted segments to the Content Hosting.

Client requests and authorisation:

15.	The device application requests an authentication for the user with the content provider.

16:	The application provider providers authorisation tokens.

17:	The application provides the tokens to the DASH Player and the DRM client.

18:	The DASH Client requests the presentation manifest including content protection information from the Content Hosting.

19:	The DASH Client requests one or several licenses from the licensing server using key ids, possibly including authorization tokens.

20:	The License Servers asks for user verification to the authorisation server

21: The user is verified.

22:	The License Server requests the decryption keys from the key server based on the key ids.

23: The Key Server provides the relevant content keys.

24.	The License server provides a DRM license with one or multiple keys to the DASH client and DRM Client.

Content delivery and decryption:

25.	The DASH Client requests encrypted segments from the Content Hosting.

26.	The DASH Client provides the encrypted segments to the Media Platform.

27.	The DASH Client provides the encrypted samples to the DRM Client.

28.	The DRM System decrypts the samples using the DRM licence and content keys.

29.	The DRM System provides the decrypted samples to the Media Platform.

As mentioned above, ETSI TS 103 799 [143] provides a format that permits a DRM system to provide content protection information to an encryptor, packager and manifest creator according to step 10 above. An example if shown in Listing 5.10.3-1.

Figure 5.10.3-1 CPIX example

<?xml version="1.0" encoding="UTF-8"?> <CPIX xmlns="urn:dashif:org:cpix" xmlns:pskc="urn:ietf:params:xml:ns:keyprov:pskc"> <ContentKeyList> <ContentKey kid="e82f184c-3aaa-57b4-ace8-606b5e3febad"> <Data> <pskc:Secret> <pskc:PlainValue>wvr2bihSzExKdR8KKpQf2w==</pskc:PlainValue> </pskc:Secret> </Data> </ContentKey> <ContentKey kid="087bcfc6-f7a5-5716-b840-6aa6eba3369e"> <Data> <pskc:Secret> <pskc:PlainValue>8281ce8db9083697d9770d87db962835</pskc:PlainValue> </pskc:Secret> </Data> </ContentKey> </ContentKeyList> <DRMSystemList> <DRMSystem kid="e82f184c-3aaa-57b4-ace8-606b5e3febad" systemId="edef8ba9-79d6-4ace-a3c8-27dcd51d21ed"> <PSSH> AAAAxnBzc2gBAAAA7e+LqXnWSs6jyCfc1R0h7QAAAAINw+xPdoNUi4HnPGTlguE2FEe37S9mVyu9EwbOfPNhDQAAAIISEBRHt+0vZlcrvRMGznzzYQ0SEFrGoR6qL17Vv2aMQByBNMoSEG7hNRbI51h7rp9+zT6Zom4SEPnsEqYaJl1Hj4MzTjp40scSEA3D7E92g1SLgec8ZOWC4TYaDXdpZGV2aW5lX3Rlc3QiEXVuaWZpZWQtc3RyZWFtaW5nSOPclZsG </PSSH> </DRMSystem> <DRMSystem kid="087bcfc6-f7a5-5716-b840-6aa6eba3369e" systemId="9a04f079-9840-4286-ab92-e65be0885f95"> <PSSH> AAAAxnBzc2gBAAAA7e+LqXnWSs6jyCfc1R0h7QAAAAINw+xPdoNUi4HnPGTlguE2FEe37S9mVyu9EwbOfPNhDQAAAIISEBRHt+0vZlcrvRMGznzzYQ0SEFrGoR6qL17Vv2aMQByBNMoSEG7hNRbI51h7rp9+zT6Zom4SEPnsEqYaJl1Hj4MzTjp40scSEA3D7E92g1SLgec8ZOWC4TYaDXdpZGV2aW5lX3Rlc3QiEXVuaWZpZWQtc3RyZWFtaW5nSOPclZsG </PSSH> </DRMSystem> </DRMSystemList> <ContentKeyUsageRuleList> <ContentKeyUsageRule kid="e82f184c-3aaa-57b4-ace8-606b5e3febad"> <VideoFilter minPixels="0" maxPixels="442368"/> <BitrateFilter maxBitrate="500000"/> </ContentKeyUsageRule> <ContentKeyUsageRule kid="087bcfc6-f7a5-5716-b840-6aa6eba3369e"> <VideoFilter minPixels="442369" maxPixels="2073600"/> </ContentKeyUsageRule> </ContentKeyUsageRuleList> </CPIX>

The example provides the following

-	ContentKeyList: This element defines the content keys used to encrypt the content. Each ContentKey element includes a @kid attribute for the Key ID and the actual encryption key in the PlainValue element.

-	DRMSystemList: This element defines the DRM systems associated with each content key. Each DRMSystem element includes a @kid (Key ID) and a @systemId that identifies the DRM system (e.g., Widevine™, PlayReady™). The PSSH element includes the Protection System Specific Header (PSSH) containing DRM-specific data required for license acquisition and playback.

-	ContentKeyUsageRuleList: This element defines usage rules for each content key. Each ContentKeyUsageRule element includes a @kid (Key ID) and filters that specify how the content can be used. For example, the VideoFilter and BitrateFilter elements define restrictions on video resolution and bitrate.

-	Usage Rules and Filters

-	VideoFilter: Specifies the range of video resolutions that can be decrypted with the associated key. For example, @minPixels="0" and @maxPixels="442368" restrict the key to standard definition (SD) video.

-	BitrateFilter: Specifies the maximum bitrate that can be decrypted with the associated key. For example, @maxBitrate="500000" restricts the key to content with a bitrate of up to 500 kbps.

This CPIX document ensures that different keys are used for different content tracks and that specific usage rules are enforced, enhancing the security and flexibility of the DRM-protected content.

Step 8 above may only include a subset of the information, namely the DRM System information is absent and is only added in step 9.

## =====  CHANGE =====

### 5.10.4	Mapping to high-level call flows

Based on the generalised Media Delivery architecture from TS 26.501 [15], as reproduced in figure 5.15.1-1 below, different mapping options of the components of the above DRM architecture to the downlinkdownlink media streaming architecture are provided in table 5.10.4-1.

NOTE	Since this Key Issue deals only with downlink media streaming, figure 4.2.1-2 from TS 26.501 may be applicable as well, with "5GMSd" instead of "Media" throughout.

Figure 5.10.4-1 Media Delivery architecture as defined in figure 4.1.2.2-1 of TS 26.501 [15]

Table 5.10.4-1 provides different deployment options on how the DRM network functions are mapped to the downlink media streaming functions.

Table 5.10.4-1 Possible deployment options to map DRM network functions
to downlink media streaming functions

| DRM Function | 5GMSd Application Provider | 5GMSd AS | 5GMSd AF | Media Player | Media Session Handler |
| --- | --- | --- | --- | --- | --- |
| Authorization Server | 1, 2, 3, 4, 5, 6 | 2 |  |  |  |
| Key Server | 1, 3, 4 | 2 | 5, 6 |  |  |
| License Server | 1, 3, 4 | 2 | 5, 6 |  |  |
| Encoder | 1, 4, 6 | 2, 3, 5 |  |  |  |
| Encryptor/packager | 1, 4, 6 | 2, 3, 5 |  |  |  |
| Manifest Creator | 1, 6 | 2, 3, 4, 5 |  |  |  |
| Content Hosting |  | 1, 2, 3, 5, 6 |  |  |  |
| DRM Client |  |  |  | 1, 2, 3, 4, 5, 6 |  |
| DASH Client |  |  |  | 1, 2, 3, 4, 5, 6 |  |
| Media Platform |  |  |  | 1, 2, 3, 4, 5, 6 |  |

The following different deployment options are discussed:

Option 1:	The 5GMSd Application Provider runs all DRM and packaging related functions, and the 5GMSd AS only caches the DASH Presentation as a CDN. For DRM acquisition, the UE contacts the 5GMSd Application Provider.

NOTE:	This option corresponds to downlink media streaming in Release 18 in which DRM is external to the 5GMSd AS. Other options address the internalization of certain DRM-related functions.

Option 2:	The 5GMSd System runs a DRM and packaging service, including a License Server, on the 5GMSd AS. Content is ingested by the 5GMSd AS and all functions of licence hosting, content encoding, content encryption and so on run in the 5GMSd AS.

Option 3:	The 5GMSd System runs a content encoding and packaging service, but the DRM System including License and Key Servers are external in the 5GMSd Application Provider domain. The 5GMSd AS needs to communicate with the License Server for content encoding and packaging.

Option 4:	The 5GMSd System generates the presentation manifest, but content encoding, content packaging and content encryption are external in the 5GMSd Application Provider. The 5GMSd AS needs to acquire relevant information for presentation manifest generation.

Option 5:	This is similar to Option 2 but, being quite specific in operation, the License Server is handed to the 5GMSd AF alongside the Authorization Server. The main communication is between 5GMSd AS and 5GMSd AF at reference point M3d.

Option 6:	This is similar to option 5, but only the License Server is offered by the 5GMSd System, hosted in the 5GMSd AF. Similar to option 4, content encoding, content packaging and content encryption are external in the Media Application Provider. The main communication is between the 5GMSd Provider and the 5GMsd AF at reference point M1d for the exchange of Content Protection Information.

Based on current deployments, licence acquisition is a user plane communication and no APIs in the media access client exist to delegate the licence acquisition to a third-party function, such as the Media Access Function (e.g. Media Player). Hence, options 5 and 6 are not further discussed.

Now, for the different options 1-4, table 5.10.4-2 maps the steps in figure 5.10.3-5 to components and/or interfaces defined in the media delivery architecture as shown in figure 5.10.4-1.

Table 5.10.4-2: Mapping of steps in figure 5.10.3-5 to functions and/or interfaces defined in the media delivery architecture as shown in Figure 5.10.4-1

| Steps | Option 1 (External DRM and packaging) | Option 2 (Internal DRM and packaging) | Option 3 (Internal packaging, external DRM) | Option 4 (only manifest internal) |
| --- | --- | --- | --- | --- |
| 1 | 5GMSd AP | M1d/M3d (Content Template) | M1d/M3d (CT) | M1d/M3d (CT) |
| 2 | 5GMSd AP | M2d (Ingest) | M2d (Ingest) | M2d (Ingest) |
| 3 | 5GMSd AP | 5GMSd AS | M1d/M3d | 5GMSd AP |
| 4 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AP |
| 5 | 5GMSd AP | 5GMSd AS | M2d (CPI) | 5GMSd AP |
| 6 | 5GMSd AP | 5GMSd AS | M2d (CPI) | 5GMSd AP |
| 7 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AP |
| 8 | 5GMSd AP | 5GMSd AS | M2d (CPI) | 5GMSd AP |
| 9 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AP |
| 10 | 5GMSd AP | 5GMSd AS | M2d (CPI) | M2d (CPI) |
| 11 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AS |
| 12 | M2 (Ingest) | 5GMSd AS | 5GMSd AS | 5GMSd AS |
| 13 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AP |
| 14 | M2 (Ingest) | 5GMSd AS | 5GMSd AS | M2d (ingest) |
| 15 | M8d | M8d | M8d | M8d |
| 16 | M8d | M8d | M8d | M8d |
| 17 | Media Player | Media Player | Media Player | Media Player |
| 18 | M4d | M4d | M4d | M4d |
| 19 | new (License Req) | M2d | new (License Request) | M4d |
| 20 | 5GMSd AP | M2d | 5GMSd AS | 5GMSd AP |
| 21 | 5GMSd AP | M2d | 5GMSd AS | 5GMSd AS |
| 22 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AS |
| 23 | 5GMSd AP | 5GMSd AS | 5GMSd AS | 5GMSd AS |
| 24 | new | M4d | New (License Response) | new |
| 25 | M4d | M4d | M4d | M4d |
| 26 | M4d | M4d | M4d | M4d |
| 27 | Media Player | Media Player | Media Player | Media Player |
| 28 | Media Player | Media Player | Media Player | Media Player |
| 29 | Media Player | Media Player | Media Player | Media Player |

Of the options documented in table 5.10.4-2, Option 1 is most prominent because it is the default option for downlink media streaming in Release 18. Of the remaining options:

-	In particular Option 3 is expected to be of interest for 5G Media Streaming deployments, because it addresses the scenario for which a specific encoding or transcoding for 5G Media Streaming is carried out by the Application Server.

-	Option 4 is a subset of Option 3, and hence not discussed explicitly.

-	We also exclude option 2 for now, as DRM servers are generally handled outside MNO networks.

NOTE:	The mapping to 5GMS via MBS/MBMS is for further study

## =====  CHANGE =====

## 6.10	Support for distributing DRM-protected, encrypted and high-value content

DRM and Conditional Access are commonly used by third-party streaming services. However, in case streaming is done through MBS or MBMS, a more careful management of the encryption/decryption keys needs to be checked. Scalability of key delivery is an issue. The support for encrypted content in unicast/multicast and broadcast is relevant. Integration of Content Protection interfaces in the provisioning of 5G Media Streaming – for example using CPIX back-end interfaces – is of high relevance for the industry and needs to be addressed accordingly. With the inclusion of DRM support in 5GMS, and the support of 5GMS via MBS and MBMS, these use cases need to be supported not only for unicast but also for multicast/broadcast User Services.

Based on the considerations in clause 5.10, it is recommended to:

-	Support the Content Protection Information Exchange Format (CPIX) as specified in ETSI TS 103 799 [143] at reference point M2d by specifying the necessary stage-2 and stage-3 extensions in TS 26.501 [15] and TS 26.512 [16] respectively.

-	Support the DASH-IF Interoperability Points specified in [144] at reference point M4d for both DASH and HLS by specifying the necessary stage-2 and stage-3 extensions in TS 26.501 [15] and TS 26.512 [16], respectively.

-	Specification of a Content Preparation Template format in TS 26.512 [16] or TS 26.510 [108] that can configure encryption content preparation tasks in the 5GMS AS.

NOTE:	Maintaining functional symmetry with uplink media streaming (where applicable) is generally considered important, but DRM is considered to be applicable only for 5GMS downlink.

In particular, stage 2 extensions are needed to address the open issues:

-	definition of a new interface between Media Player any 5GMS Application provider (see also clause 6.19)

-	Functional updates to the definition of the 5GMS AS to support:

-	Ingest, delivery, and contribution of encrypted content

-	Content preparation tasks for:

-	Decrypting content ingested at reference point M2d.

-	(Re-)encrypting content prior to distribution at reference point M4d.

-	Applying usage filters and restrictions

-	Updates to the definitions of reference points to support:

-	Carriage of Content Protection information at reference point M2d.

-	Delivery of Content Protection information in presentation manifests at reference point M4d.

Stage 3 aspects are provided above.