3GPP TSG-SA4 Meeting #131	S4-250035

# Geneva, Switzerland, 17th Feb 2025 - 21st Feb 2025

Source:	Qualcomm Germany

Title:	[VOPS] Interoperable MV-HEVC Experiences - how?

Agenda item:	9.5

Document for:	Decision

# 1	Introduction

There are multiple efforts to add MV-HEVC to 3GPP and MPEG Systems standards to support Spatial video experiences. This document provides an overview of the activities in 3GPP and the industry, as well as experiences in our company to get conforming media streams.

The following use cases are considered:

-	File playback and Messaging

-	DASH/HLS/CMAF based streaming

# 2	Messaging in 3GPP

## 2.1	Introduction

In TR 26.841 v1.0.0, key topic #5 "Additional media experiences" is introduced. A specific format of interest is the ability to share 3D video with audio. Existing and emerging capturing systems on mobile devices are used to capture video formats that go beyond regular 2D video formats. The captured scene is recorded and shared with another UE, for example another mobile device with a 2D screen, a mobile device with an Autostereoscopic Display, a VR or MR HMD, or it consumed on AR glass. While sharing may be done by different means (upload and download, stream, communication), the focus in the context of this document is message exchange.

This issue addresses the analysis and requirements to support 3D video in advanced messaging based on MV-HEVC as defined in 3GPP TS 26.265 [26265].

## 2.2	Gap Analysis and Requirements

In order to identify the gaps to add 3D video to advanced messaging, a high-level flow chart is provided in Figure 1 and also elaborated further below based on the architecture shown in Figure 4.1-1 of TR 26.841.

The call flow is based on the generic description in clause 4.1 of TR 26.841.

Figure 1 High-Level Call flow for 3D video messaging

The following high-level call flow is executed to address the 3D video messaging and gaps are indicated in bold:

1)	A Message Service Sender instructs and MMBP generator to generate a message with 3D format where the format is defined through a set of parameters handed over on an MMBP-GEN-API.

2)	The MMBP generator creates an MMBP that includes 3D video formats encoded with MV-HEVC, and adds relevant static and dynamic metadata.

3)	The generated file with associated relevant static and dynamic metadata is provided in a container message that is uploaded to a messaging server, together with a signaling in the media type and associated parameters. The media signaling may include that the file is suitable for different clients, for example it includes a 2D and a Beyond 2D experience.

4)	A messaging service client is informed about the availability of a message in this format using the media type signaling together with appropriate sub-parameters.

5)	A messaging service client uses the MMBP-PLAY-API to query the MMBP player with it can process, decode and render the message, possibly in 2D legacy or as 3D video.

6)	If the MMBP player indicates that it supports the capability, the messaging service clients selects and downloads the message and hands it to the MMBP player for playback.

7)	The MMBP player processes, decodes and renders the 3D message based on its decoding and rendering capabilities together with other media types.

Based on the analysis in the call flow, the following aspects are missing:

1)	the definition of the 3D formats that are supported

2)	a generator for a 3D video message including API parameters

3)	a 3D video messaging format including MV-HEVC with relevant metadata, including depth, alpha. projection formats, disparity information – for details refer to TR 26.956 [26956].

4)	the signaling of the media type and the associated parameters, typically a MIME type and the codecs parameter.

5)	an API to query the MMBP player that it can process, decoder and render the message

6)	an MMBP player capability that processes, decodes and renders the message.

7)	an MMBP player capability renders the video message together with other media components in a synchronized and spatially aligned matter.

## 2.3	Considered Solutions

### 2.3.1	Overview

To address the issues, a set of solutions are discussed in the following clause.

### 2.3.2	3D Video Formats

For the 3D video formats according to a), the parameters are defined in TR 26.956 [26956], including different type of Beyond 2D video using projections to left and right eye.

TS 26.265 is expected to define the Representation Format, similar to the one defined in clause 3, but the content may be richer as it is user generated.

### 2.3.3	3D Video Encoding and Decoding capabilities

To address issue 2) as well as partially 5) and 6), video encoding and decoding capabilities, as video operation points need to be defined. A candidate solution will be available in 3GPP TS 26.265 [26265].

Decoding capabilities are defined currently.

-	MV-HEVC-UHD-Dec: the capability to decode bitstreams with an HEVC/ITU-T H.265 Main 10 Profile base layer (layer_id=0), and a single HEVC/ITU-T H.265 Multiview Main 10 [or Multiview Extended 10] layer (layer_id=1) [h265]. Each layer shall conform to Main Tier, Level 5.1, while the device should be capable of supporting single layer decoding of HEVC/ITU-T H.265 Main 10 Profile bitstreams at Main Tier, Level 5.2. All layers shall have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

Editor’s Note: Adding operating point(s) for 8k stereoscopic is FFS.

Editor’s Note: Adding operating point(s) for frame packed stereoscopic video is FFS.

-	the bitstream does not exceed the maximum luma picture size in samples of 33,554,432,

-	the maximum VCL Bit Rate is constrained to be 80 Mbps with CpbVclFactor and CpbNalFactor being fixed to be 1000 and 1100, respectively.

A video operation point is defined.

Table 6.1-1	Video Operation Points

| Name | Video Format | Decoding Capabilities | Definition |
| --- | --- | --- | --- |
| 3GPP-AVC-HDTV | 3GPP-HDTV (see clause 4.4.3.2) | AVC-FullHD-Dec (see clause 5.4) | 6.2 |
| 3GPP-HEVC-HDTV | 3GPP-HDTV (see clause 4.4.3.2) | HEVC-FullHD-Dec (see clause 5.4) | 6.3 |
| 3GPP-HEVC-HD-HDR | 3GPP-HDR (see clause 4.4.3.3) | HEVC-FullHD-Dec (see clause 5.4) | 6.4 |
| 3GPP-HEVC-UHD-HDR | 3GPP-HDR (see clause 4.4.3.3) | HEVC-UHD-Dec (see clause 5.4) | 6.5 |
| 3GPP-HEVC-3DTV | 3GPP-3DTV (see clause 4.4.3.4) | HEVC-UHD-Dec-2 (see clause 5.5) | 6.6 |
| 3GPP-MVHEVC-3DTV | 3GPP-3DTV (see clause 4.4.3.4) | MVHEVC-UHD-2 (see clause 5.5) | 6.7 |

Editor’s note:	It is expected that TS 26.265 defines encoding and decoding capabilities for 3D video.

The capabilities are expected to include abstract APIs to provide encoding parameters.

### 2.3.4	3D Video Systems Operation Point

To address the integration of the video elementary stream into a container message, together with the metadata, a comprehensive 3D track format is required.

ISO/IEC 14496-12:2022 (ISOBMFF) defines the multiplexed metadata track format ('mebx'), which is capable of carrying multiple metadata items over a time range in a single track.

Additional metadata signaling for 3D video operation points are expected to be completed in 3GPP TS 26.265 [26265].

Editor’s note:	It is expected that TS 26.265 defines a track format for 3D video.

Details are below.

### 2.3.4	3D Video Media Type Signaling

One of the possibilities to carry MV-HEVC video in mp4 is by using the 'hvc1' or 'hev1' sample entry type as specified in clause 9 of ISO/IEC 14496-15 in a backward compatible manner. Some existing products in the market are using this concept to carry stereoscopic content and alpha using the Multiview extensions of the HEVC standard as L-HEVC in mp4. Such profiles with existing support in the mobile ecosystem are expected to be defined in TS 26.265 [26265]. However, when constructing the MIME types 'codecs' parameter, according to Annex E of ISO/IEC 14496-15, it does not provide the necessary signalling for all layers. Even if additional information can be specified with other MIME type parameters, these may not be processed by certain APIs. For example, the W3C API accepts a MIME type with no extra MIME parameters except for codecs.

Furthermore, the current signalling inside the codecs string does not expose other important information such as the types of auxiliary information related to rendering the stream. Given these challenges, it is important that a standardized solution be developed to enhance interoperability, accuracy, and efficiency of multi-layer video stream handling. The following high-level requirements for such a solution have been identified:

-	Comprehensive Layer Signalling: Enable the signalling of multiple video layers to be used for 3GPP-based services, including DASH MPDs (see TS 26.511), capability checks in the context of 5G Media Streaming (see TS 26.511), as well as for Media Messaging Services (see TS 26.143). This should include but not be limited to the number of layers, types of each layer, their inter-dependencies, etc.

-	Backward Compatibility: Ensure that the solution maintains backward compatibility, allowing existing players and systems to continue functioning without modifications, while enabling enhanced capabilities for updated systems.

-	Adaptability and Extensibility: Design the solution to be adaptable for future extensions and new types of layers or enhancements without requiring significant overhauls. Consider making the signalling codec agnostic.

Editor’s note:	It is expected that MPEG provides signaling on this matter to be added here.

### 2.3.5	System integration

The 3D video needs to be synchronized and spatially aligned with other media types. Time synchronization is provided by using the ISO BMFF/MP4 file format.

### 2.3.6	Summary and Conclusions

Editor’s note:	It is expected that all functionalities are available to support 3D video and general media experiences in a future messaging service.

# 3	DASH/HLS/CMAF based streaming

## 3.1	Introduction

In order to add content into streaming environments, DASH and HLS, CMAF profiles are the baseline. Some recommendations on media profiles for DASH/CMAF are provided on what needs to be defined.

1)	The name of the media profile

2)	The specification where the media profile is defined

3)	The static mapping of parameters to a DASH MPD, in particular to the MPD parameters, such as @mimeType, @codecs, etc.

4)	The dynamic mapping of parameters to a DASH MPD from a CMAF Principal Header, in particular to the MPD parameters, such as @width, @height, etc.

In addition, for 3GPP, an operation point includes the definition of content options.

In the following, the summary and needs are defined.

## 3.2	Formats

In 3GPP, a content format is provided as below. This is expected to be specific for 3GPP and can does not have to be defined in MPEG.

| Parameter | Restrictions |
| --- | --- |
| Picture aspect ratio | 16:9 |
| Spatial Resolution width x height | 3840 × 2160, 1920 × 1080 NOTE: For 1080, typically the encoded signal has 1088 lines and cropping is applied to remove spatial samples that are not presented. |
| Scan Type | the source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273 is progressive |
| Chroma format indicator | The chroma format indicator is 4:2:0. |
| Bit depth | The permitted values are 8 or 10 bit. 8 bit is only permitted for SDR. |

| Colour primaries Transfer Characteristics Matrix Coefficients | Only the following value combinations are permitted: (1, 1, 1), (9,14, 9), (9, 16, 9), and (9, 18, 9) for SDR HD, SDR UHD, HDR PQ, and HDR HLG, respectively. |
| --- | --- |
| Frame rates | The permitted values are 60, 60/1.001, 48, 48/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 fps. |
| Frame packing | No frame packing is applied. |
| Projection | No projection is used. |
| Sample aspect ratio | The pixel aspect ratio is 1 (square pixel), i.e. only the value 1 as defined in clause 7.3 of Rec. ITU-T H.273 is permitted. |
| Chroma sample location type | For SDR HD, the location of chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7 is set to 0. For SDR UHD, HDR PQ, and HLG, the location of chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7 is set to 2. |
| Range | The restricted video range is used. |
| Stereoscopic Video | A signal for the Left and for the Right Eye is provided whereby the signals have the identical parameters as above and are timely synchronized. |

## 3.3	CMAF Profile

A CMAF profile is defined in

| MDS24412 | Text of ISO/IEC 23000-19 3rd edition CDAM 2 New Structural CMAF Brand Profile | WG 03 MPEG Systems | MDS24412_WG03_N01349. |
| --- | --- | --- | --- |

The profile is in early stage and all details that are discussed in clause 2 and clause 5 need to be handled as well.

## 3.4	Codecs Types and Manifest signaling

The following issues are discussed:

-	Layered codecs strings

-	Signaling "multiple codecs" in one track.

-	Extraction of metadata from CMAF Principal header to DASH MPD.

# 4	Stereo Content in Scene Description

A contribution is provided in m71320 [SD] Support for multi-view video and multi-channel audio sources.

# 5	Identified issues in implementations

Internally, we attempted to create MV-HEVC based files that can be produced and played back on different devices, and be generated in different environments. This is initially only for files, but likely also applies for CMAF/Streaming content.

One example is provided, producing content following the description here: https://blog.mikeswanson.com/spatial-video/

We also recognized that some content includes and requires the presence of 3D SEI (three_dimensional_reference_displays_info) to be recognized as spatial video in playback.

We observed the issue that there is a 4CC box code floating that conflicts with an ISOBMFF registered box, namely the "stri". Another conflict is with the "proj" box, which is a Youtube box for VR video that is widely supported but not registered. This was observed when using gpac. We need to make sure that we fix this issue.

(MV-)HEVC bitstreams that change parameter sets every GOP cause issues. It causes the creation of multiple sample entries and would result in duplication of the MV-HEVC related boxes. A single sample entry box is likely required to be defined.

A writer is expected to detect the three_dimensional_reference_displays_info SEI message (176) and the presence of L-HEVC layer and automatically add these boxes.

A writer packaging the elementary stream needs information on what is needed to populate the extra box content seen above. It seems that this information cannot be derived from the elementary stream.

In some content, the 'hero' box is present, in others not. It is unclear whether the box is needed, required, etc. and if so, what would be the source from the elementary stream.

We also observed that content on HMDs not consistently plays as stereo/spatial. There is another indication, that is related, but not identical. it does not permit an immersive mode.

There are also questions on the exact details of the encoding of the content, for example closed and open GOPs provide different compatibility.

# 6	Conclusion and Proposal

We recommend taking the information into account.

In particular, we propose to address

-	core interoperability in TS 26.265

-	any adaptation to streaming formats based on MV-HEVC is added to TS 26.511 or TS 26.118

-	create a messaging format that is compatible in TS 26.143, completing the study in TS 26.841.

To complete the work in FS_MeMe and VOPS, more time is needed to address all issues. We propose to extend the timelines accordingly and address all relevant issues.

# 7	Reference

[1]	 3GPP TR 26.841v1.0.0, "Study on Media Messaging Enhancements",