3GPP SA4 131	S4-250043

Geneva, February 17th 2025 – February 21th 2025

Source:	Orange

Title:	Pseudo-CR on Proposed Conclusions

Spec:	3GPP TR 26.942 v1.0.1

Agenda item:	8.8 – FS_MediaEnergyGREEN (Study on Media enerGy consumption exposuRE and EvaluatioN framework)

Document for:	Agreement

1.	Introduction

The goal of this pCR is to progress on clause 8 Conclusions and proposed next steps. More solutions will be probably added during SA4#131. The goal of this document is to propose a basis which will be modified/complemented during the meeting to complete the Technical Report at the end of the meeting as expected.

2.	Reason for Change

Conclusions and proposed next steps are missing.

3.	Conclusions

Without this pCR, no instructions will be provided for next steps.

4.	Proposal

It is proposed to agree the following changes to 3GPP TR 26.942 v1.0.1.

* * * First Change * * * *
(all new text)

# 8	Conclusions and proposed next steps

## 8.1 	General

The present document highlights ongoing challenges in accurately estimating energy consumption of media delivery and consumption in the 5G system, including mobile network as well as end-user devices. Main difficulties are:

- Data Accuracy: Difficulty in obtaining precise power data at the hardware level.

- Energy Attribution: Challenges in assigning energy usage to specific data throughput across multiple stakeholders, complicated by non-linear relationships between energy and data.

- Regulatory Pressures: Legal requirements for net zero emissions necessitate clear energy allocation strategies, which can lead to disputes among network operators, service providers and consumers.

- Measurement Boundaries: Defining system boundaries for energy measurements affects responsibility and accountability.

Current assessments of energy use are often too coarse for effective policymaking or system design, and do not meet reporting standards set by the Greenhouse Gas Protocol and regulatory bodies. It is suggested to collect fine-grained energy consumption data in real-time could address these challenges and proposes a framework for future measurements.

## 8.2	Conclusion for Key Issue #1

Two different solutions have been submitted to address this Key Issue about Energy-related Information exposure:

- Solution#4 proposes to extend UE data collection, reporting and event exposure defined in TS 26.531 [21] by having the Direct Data Collection Client collecting and reporting per-application UE energy-related information to the Data Collection AF.

- Solution#5 proposes new architecture and procedures allowing to provide energy-related information from the device and the network to a UE application during media consumption for exposure to the user with the definition of two new components, the Energy Information AF and the Energy Information Collector.

The following aspects are concluded:

- For exposure of end-to-end Energy-related Information to the network, Direct Data Collection Client and Data Collection AF specified in TS 26.532 [22] will take into consideration UE energy-related information in addition of their existing data.

- For exposure of end-to-end Energy-related Information to UE application, two new entities and their associated interfaces will be defined to provide energy-related information from the device, the network and other components of the Media Delivery system to a UE application: The Energy Information AF and the Energy Information Collector.

In conclusion, those principles will be addressed by the normative work:

- Provide relevant extensions to TS 26.532 to extend the role of Direct Data Collection Client and Data Collection AF to take into consideration UE energy-related information.

- Create of new TS to define the architecture allowing to expose end-to-end Energy-related Information to UE application in media context.

- Address the relevant stage-3 aspects based on stage-2 work.

## 8.3 	Conclusion for Key Issue #2

Two solutions have been submitted to address this Key Issue about Energy-related monitoring and measurement:

- Solution#3 indicates how applications currently measure energy consumption of a smartphone but listing major limitations (information not publicly available on all UE OS, not limited to media consumption and with uncertain accuracy).

- Solution#6 proposes a method leveraging energy consumption information to monitor and measure the way the media content is handled and delivered to the users, and to provide better Quality of Experience (QoE) for users.

The following aspects are concluded:

- On the device side, there is no consensus allowing to define a UE entity in the 5G system managing the measurement of UE energy-related information. UE energy-related information can still be used on some UE Operating System providing these kinds of metrics but as the metrics differ from one OS to another, there will not be normative work on UE energy-related information collection and metrics.

- On the network side, the Energy Information Function (EIF) in TS 23.501 [23501] is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, PDU Session and/or QoS flow granularity and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF). No additional information has been identified in context of media delivery or consumption.

In conclusion, no normative work is planned to address this Key Issue.

## 8.4 	Conclusion for Key Issue #3

Two different solutions have been submitted to address this Key Issue about Evaluation framework:

- Solution#1, based on work done by the French regulator, indicates how API available on some UE OS can be used to evaluate the impact of the use of some technologies on energy efficiency on devices in a lab.

- Solution#2 proposes a method to evaluate the energy efficiency of the entire UE as a whole device, which is UE implementation specific.

The following aspects are concluded:

- No methodology allowing to evaluate the energy usage/savings of multimedia standards features and proposals, following best practices from these earlier characterization frameworks, has been identified. The main difficulties are on UE metrics related to media consumption which are not available on all Operating Systems, and the lake of implementation of 5G network entities allowing to provide Energy-related information from the network with the good granularity. It seems this characterization framework will have to wait for Energy Information Function implementations before being able to evaluate the energy usage/savings of a multimedia standards feature on a specific device in a lab.

In conclusion, no normative work is planned to address this Key Issue.