3GPP SA4 TSG-SA WG4 Meeting #131	S4-250050

# Switzerland, Geneva, 17-21 February 2025                             revision of S4aR250032r1

Source:	InterDigital France

Title:	Pseudo-CR on OpenXR API for Haptic Media

Spec:	3GPP TS 26.854 v1.0.0

Agenda item:	10.8

Document for:	Agreement

1. Introduction

During the previous RTC call, the section 7.3.4 was empty with an editor note: TBD and a text was proposed to fill the section. It was agreed with square bracket.

This contribution removes the square bracket around the provided and unchanged text for the OpenXR API.

R1 addresses comment received on the list.

S4-250050 is the version agreed in the RTC adhoc call.

2. Proposal

Agree to remove the “[]” and editorial changes.

* * * First Change  * * * *

### 7.3.4	OpenXR APIs

The OpenXR API [6] standardizes the use of cross-platform XR device capabilities, including haptic devices. Current support mainly addresses XR and game controllers handled by the user hands and most commercial controllers are supported. Functions such as trigger, click, touch, squeeze, grip are input actions with a potential haptic output specified by an hapticsAction and hapticPath. The hapticsAction specifies the type of haptic feedback (e.g. vibrations) and hapticPath specifies where the effect is applied (e.g. left hand). Haptic feedback is sent to a device using the ApplyHapticFeedback and StopHapticFeedback functions.

The only haptic type supported by the OpenXR API v 1.1.43 is XrHapticVibration for haptics vibrations (vibration amplitude, duration, frequency). Some proprietary extensions also provide support for vibrations described by a haptic amplitude envelope or PCM signal (XrHapticAmplitudeEnvelopeVibrationFB, XrHapticPcmVibrationFB).

In 2022, the HIF (Haptic Industry Forum) submitted a high-level proposal for advanced haptics APIs to the OpenXR consortium. The advanced haptics APIs are designed to extend the OpenXR API to other haptics modalities and process several haptics standards among which the MPEG HJIF, IEEE P2861.3 standards as well as other proprietary formats (AHAP, IVS)..

* * * End of Changes * * * *