3GPP TSG-SA WG4 Meeting #131							S4-250051

CH, Geneva, 17 – 21 February 2025

Source:	Nokia

Title:	V-DMC web player demo

Agenda item:	6.2

Document for:	Information

# Introduction

Dynamic mesh sequences are being increasingly used in a growing number of applications such as gaming, extended

Reality (XR), photorealistic avatars, automotive, movie production, digital twins, and 3D visualization. Dynamic mesh sequences contain frames, each consisting in a textured 3D mesh that may contain additional attribute data such as normal data. With increasing mesh resolutions and frame rates, such dynamic sequences require a large amount of data for both storage and streaming.

Video-based dynamic mesh coding (V-DMC) [1] is a recent standard from MPEG that supports the lossy and lossless coding of dynamic mesh sequences of arbitrary topology and connectivity that may be time-varying. MPEG issued a call for proposals for dynamic mesh coding in 2021, and several responses showed improvements over the state of the art. Since then, the selected technology [2] has been continuously improved, and V-DMC has reached the ISO Draft Internal Standard (DIS) stage in December 2024.

For widespread adoption of the V-DMC standard, it is important to have implementations that can run even on low-power devices. Nokia developed a prototype of V-DMC compatible player for the web platform. This contribution provides an overview and key aspects of its implementation.

# Brief overview of V-DMC

V-DMC follows the principles of the V3C standard family (MIV, V-PCC) [3] organizing the coded volumetric representation into metadata that guides the decoder-side reconstruction process, and a number of video component sub-streams that represent the attributes (e.g., color, normal) and geometry information (i.e., displacement). Additionally, V-DMC introduces a new type of data that is not present in other applications of V3C, i.e., base connectivity that is provided through an additional base mesh sub-stream.

At the encoder side, the base mesh is obtained by mesh decimation (i.e., a process that reduces the number of vertices and triangles to generate a coarse mesh approximation, typically with a few percents of the original triangle and vertex counts) of the original mesh. Texture UV coordinates are generated for the base mesh, and the base mesh connectivity is subdivided to better approximate the original mesh. (Subdivision is the process of dividing the mesh faces into a number of sub-faces and interpolating the geometry of newly created vertices.) The base mesh is coded by a static mesh codec (intra coding), or it can be differentially coded using motion estimation between meshes in a sequence (inter coding, or animation coding). Estimated distance vectors between the subdivided base mesh surface and the original mesh frame surface are called displacements and are further hierarchically predicted and updated  using a lifting scheme. Displacement data is represented by a 1D scalar that provides the projection of the displacement vector onto the normal vector. Once predicted and quantized, displacement data is typically packed into a video frame per Level-of-Detail (LoD) following a Morton order (Z-order), alternatively, they can also be coded using arithmetic coding into a dedicated sub-stream. Attributes of dynamic mesh are mapped using texture coordinates and recolorization into a video frame and encoded with a video codec such as H.265/HEVC. Metadata used to describe the encoding and reconstruction processing is organized in an atlas as an extension of V3C specifications [3].

At the decoder side, the process is split into two steps: 1) demultiplexing and decoding all sub-streams (atlas data, base mesh data, displacement data and attribute data), 2) reconstructing the 3D mesh frames from the decoded sub-streams with the decoded atlas metadata. The reconstruction process involves subdividing the base mesh frames and their attributes, adding decoded and reconstructed inverse transformed displacement data to reconstruct the mesh connectivity and geometry, and finally associating the reconstructed texture (attribute) map to render or store the reconstructed mesh frames.

An example of encoder and decoder flow is shown in Figures 1 and 2.

Figure 1 –V-DMC encoder processing chain

Figure 2 – V-DMC decoder processing chain

# Prototype V-DMC implementation overview

The V-DMC prototype implementation is web based. The selection of the web platform was driven by several key advantages:

- The application is platform-agnostic, without requiring separate distributions per platform/OS.

- With access to technologies like WebGL2 and WebCodecs, the application can take advantage of the underlying hardware capabilities of devices.

- The application can be integrated with other technologies such as AR/XR frameworks, web services, and multimedia content, providing a richer and more interactive user experience.

Figure 3 illustrates the high-level processing chain of the V-DMC prototype player.

Figure 3 – The high-level processing chain in the V-DMC prototype player

Some remarks on the processing chain:

- Basemesh and Atlas bitstreams are decoded in CPU utilizing TMMv9.0 [4], ported for the Web.

- Displacement and Attribute video bitstreams are decoded utilizing WebCodecs (HW decoder in GPU).

  - NOTE: The WebCodecs API gives web developers low-level access to the individual frames of a video stream.

- Rendering is done utilizing WebGL2.

The demo device is a smartphone with the following specifications:

| OS | Android™ 14 |
| --- | --- |
| CPU | Qualcomm SM8550-AB Snapdragon 8 Gen 2 (4 nm), Octa-core (1x3.2 GHz Cortex-X3 & 2x2.8 GHz Cortex-A715 & 2x2.8 GHz Cortex-A710 & 3x2.0 GHz Cortex-A510) |
| GPU | Adreno 740 (700 MHz, 768 SIMD (ALUs), 2089.0 GFLOPs (float32)) |
| RAM | 8GB/16GB LPDDR5X |
| APIs | HTML5, WebGL2, WebCodecs, WebAssembly |

Figures 4 to 6 provide several illustrations of the prototype running on a mobile device, on a desktop browser and on a mobile device in AR mode, respectively.

Figure 4 – (left) V-DMC playback in mobile device, (right) closelook: arm of “Mitch” in wireframe mode

Figure 5 – V-DMC decoding in a desktop browser

Figure 6 – V-DMC decoding in AR (hit-test demo)

A performance analysis of the V-DMC decoder executed on a target device is presented in the table below.

| CPU (JS/WASM) Parsing/Decoding | Base-mesh can be decoded using a single CPU core. For modern devices, this allows decoding 4-5 base-meshes in real time. |
| --- | --- |
| GPU (WebGL2) Rendering | Up to 40 million triangles per second can be rendered, including: - Fetching displacements - Inverse quantization - Subdivision - Shading This is sufficient to render up to 8 V-DMC streams @ 30 FPS (~150,000 triangles per frame). |
| HW Decoding (HEVC) | Up to 5 streams in real-time, with resolutions of 2048x2048 for textures and 256x512 for displacements using the hardware HEVC decoder. |

The current prototype is the first public demonstrator to decode and render one V-DMC-coded dynamic mesh sequence in real-time on a mobile device. The performance metrics reported in the table show the potential to significantly increase the performance of this demonstrator towards rendering multiple dynamic mesh sequences in parallel and in real-time on the same device.

# Conclusions

In FS_Beyond2D, SA4 is considering dynamic meshes as a representation format for enabling beyond 2D experiences. V-DMC is a recent MPEG standard that enables efficient compression of such dynamic meshes.

This contribution describes a prototype of a V-DMC compatible web player. The prototype shows that the V-DMC standard is implementable for the web platform and can run on already existing hardware. The associated demo will be available during SA4#131.

We believe that this contribution provides an understanding of real-world scenarios and requirements for the V-DMC standard and decoders.

# References

[1] ISO/IEC DIS 23090-29, Information technology – Coded Representation of Immersive Media - Part 29: Video-based dynamic mesh coding (V-DMC).

[2] Mammou, K., Kim, J., Tourapis, A. M., Podborski, D., & Flynn, D. (2022, September). Video and subdivision based mesh coding. In 2022 10th European Workshop on Visual Information Processing (EUVIP) (pp. 1-6). IEEE.

[3] ISO/IEC DIS 23090-5, Information technology – Coded Representation of Immersive Media - Part 5: Visual volumetric video-based coding (V3C) and video-based point cloud compression".

[4] “V-DMC TMM 9.0”, ISO/IEC JTC 1/SC 29/WG 7, July 2024, Sapporo.
https://mpeg.expert/software/MPEG/3dgh/v-dmc/software/mpeg-vmesh-tm.