3GPP SA4 RTC SWG Telco Post #130	S4aR250034r1

# Online, 5 February 2025                                                              Revision of S4aR250018

Source:	InterDigital France

Title:	Pseudo-CR on Haptics Media in messaging service

Spec:	3GPP TS 26.854 v1.0.0

Agenda item:	4.9

Document for:	Agreement

1. Introduction

This contribution provides for the integration of haptics media in 3GPP messaging services taking in account comments received during the January 5th telco. and addresses the comments received during the previous adhoc call.

R1 addresses the comments received on the list.

2. Proposal

Agree to the additional details to finalize section 8.3

* * * First Change All new * * * *

### 8.3.4	Messaging Media services

#### 8.3.4.1 System description

To support the use-cases in section 5 for haptics media enhanced messaging services, the following figures from TS 26.143 [143] are modified to illustrate haptics media functionalities in the overall Messaging Media system, Figure 8.3.4-1, and in the Messaging Media player model, Figure 8.3.4.-2.

Haptic Media Engine

Haptic Media Engine

Figure 8.3.4-1 Example system for Haptics media enhanced Messaging multimedia message exchange

Haptic media   Renderer r

Haptic media Decoder

Figure 8.3.4-2 Haptics media enhanced MMBP Player Model

#### 8.3.4.2 Media Capabilities and Profiles

The MMBP media capabilities in [143] currently do not support haptics media types. The inclusion of a top top-level haptics media type would happen at the same level as other media formats (text, speech, audio, image, etc.) and would not require modification to the multipart MMBPs and container formats (single, mixed, alternative, parallel, related, and 3GP9). A new sample entry for haptics media needs to be added to the supported sample entries in TS 26.244 [244].  . For Messaging services, it is expected that a single File Format track is used for haptic media.

In addition, when the haptic media effect is included in a scene description in association with a particular node, the scene description capabilities of the messaging service need to be extended to support MPEG_haptic and MPEG_haptic_material as well as SD-Rendering-glTF-Interactive [mecar]

8.3.4.2.1 Player and Decoding capabilities

For instance, the capability 26143_HAPTICSMEDIA_MIHS can be defined as the capability of playing back (decoding and rendering) a file that:

- is decodable by a decoder capable of the MIHS decoding and rendering capabilities as defined in [7],

- is encapsulated in an ISO BMFF track with the sample entry ‘mih1’ as defined in [25]

- is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability defined in clause 5.2 of [143] and extended with  haptics media capabilities.

The media type for files with this capability 26143_HAPTICSMEDIA_MIHS shall be signalled with haptics/mp4, profile="3gp9" codecs="mih1" or an equivalently compatible media type.

Multiple media files may be handled by creating multiple instances of the player or combining the files to output a single haptic signal for rendering through the onboard or connected actuator. Multi-technology feedback (e.g., thermal+vibrotactile) can be played simultaneously through different onboard or connected devices [ISO/IEC 23090-31].

##### 8.3.4.2.2 MMBP content generator capabilities:

For instance, the capability 26143_HAPTICSMEDIA_ENC_MIHS for a content generator can be defined as the capability of:

- Generate a haptics media file from an actuator or from a local stored file, such that the file can be played back by a player with the capability 26143_HAPTICSMEDIA_MIHS

- Generate an ISO BMFF track that conforms with the requirements of the sample entry 'mih1' as defined in [7].

- Generate a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability extended with haptics media capabilities

- Signal the haptic media capability with the generated file haptics/mp4, profile="3gp9" codecs="mih1" or an equivalently compatible media type

Note: TR 26.841 [841] studies media messaging enhancement, evaluation of haptics media as part of MMBP media capabilities in IETF MIMI [MIMI], and in the advance file format would need to be done.

* * * End of Changes * * * *