# 3GPP TSG-SA WG4 Meeting #131	S4-250070

# Geneva, CH, 17 – 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | pseudo | rev | - | Current version: | 0.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Scenario 1: UE-to-UE Stereoscopic Video Live Streaming - Reference Sequences |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The public datasets, generation software, and capturing tools for creating source sequences for UE-to-UE stereoscopic live streaming has been discussed and agreed (S4aV240042) to be added to Clause 2.5.9 of the permanent document of TR 26.956. This content is expected to be reviewed and approved as candidate source sequences for the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document provides test and reference sequences for UE-to-UE Stereoscopic Video Live Streaming scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete 7.2.7 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.7, Annex D (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[LS-12]	Wei Bao, Wei Wang, Yuhua Xu, Yulan Guo, Siyu Hong, Xiaohu Zhang. InStereo2K: A large real dataset for stereo matching in indoor scenes. SCIENCE CHINA Information Sciences. 2020.

[LS-13]	 D. Scharstein, H. Hirschmüller, Y. Kitajima, G. Krathwohl, N. Nesic, X. Wang, and P. Westling. High-resolution stereo datasets with subpixel-accurate ground truth. In German Conference on Pattern Recognition (GCPR 2014), Münster, Germany, September 2014.

=====  CHANGE =====

7.2.7	Test Sequences

7.2.7.1	Candidate Source Stereoscopic 3D Video Sequences

This section introduces candidate source stereoscopic 3D video sequences available for testing. Some sequences are freely available under a license agreement, while others are self-generated using AI-based 2D-to-3D conversion algorithms. Additionally, some sequences are self-captured using commercially available mobile devices.

7.2.7.1.1	Public Datasets

InStereo2K [LS-12], an indoor real scene stereo dataset. It contains 2000 pairs of images with high accuracy disparity maps. The author kindly grants the permission to use this dataset under the license in 3GPP.

Middlebury 2021 mobile datasets [LS-13], 24 datasets obtained with a mobile device on a robot arm, using the technique described in  [LS-13]. The authors kindly grant permission to use and publish all images and disparity maps on their website. However, they request to cite the appropriate paper  [LS-13] for using 2021 datasets.

Middlebury 2014 stereo datasets [LS-13], 33 datasets obtained using the technique described in [LS-13]. The authors kindly grant permission to use and publish all images and disparity maps on their website. However, they request to cite the appropriate paper [LS-13] for using 2014 datasets.

The table below summarizes the characteristics of the sequences in the public datasets.

Table 7.2.7.1.1-1 Test material datasets

| Name | Generated /Natural | Stereo | Depth | Resolution | Color Space | Download Link |
| --- | --- | --- | --- | --- | --- | --- |
| InStereo2K | Natural | Y | Y | 1080 × 860 | BT.709 SDR | https://github.com/YuhuaXu/StereoDataset |
| Middlebury 2021 Mobile stereo datasets | Natural | Y | Y | 1920 × 1080 | BT.709 SDR | https://vision.middlebury.edu/stereo/data/scenes2021/ |
| Middlebury 2014 stereo datasets | Natural | Y | Y | 2964 × 1988 | BT.709 SDR | https://vision.middlebury.edu/stereo/data/scenes2014/ |

7.2.7.1.2	Self-Converted Sequences

The AI-based conversion of existing 2D images and Video to stereo3D is proving commercially viable and fulfills the growing need for high quality stereoscopic images. This approach is particularly effective when creating content for the new generation of autostereoscopic displays that require multiple stereo images.Various open-source algorithms and platforms use deep neural networks to perform real-time end-to-end conversion of 2D videos and images to stereoscopic 3D video format.

As 2D-to-Stereo3D conversion algorithms usually take RGB video format, the Python scripts can be found in Annex D.2.2 to convert between YUV and RGB formats.

The test sequence sample ([link: https://pan.baidu.com/s/1_AHuebYRH33IqiqzdHsj5Q?pwd=mv57 ;Password: mv57]), provided by CMCC, is a stereoscopic 3D video (1920×1080 per eye, 60fps) generated from a 2D video frame (BQTerrace, 1920×1080, 60fps) using an AI-based conversion algorithm.

Editor’s NOTE: More sequences can be provided using this method if the quality of sample sequence is acceptable.

7.2.7.1.3		Self-Captured Sequences

A dual-lens camera can be used to directly capture stereo 3D video. There are many mobile devices on the market with this capability. For example, SpatialLabs EyesTM provided by Acer is a stereoscopic camera capable of capturing up to 8-MP (4K) per eye at 30 fps or 2K per eye at 60 fps. Or the ZTE Nubia Pad 3D IITM can capture stereo 3D video with the specifications in Table 7.2.7.1.3-1:

The main camera setup is the dual-camera systems includes two identical 13 MP lenses. These cameras capture slightly different perspectives of the same scene, mimicking the way human eyes perceive depth. The AI then processes these images to produced a coherent 3D representation. The selfie camera setup features two lenses positioned near the center of the top bezel when the tablet is oriented horizontally (with the longer side on the top).

Table 7.2.7.1.3-1 Specification of Capturing Device

| Rear Camera | Specification |
| --- | --- |
| Number of Cameras | 2 (Dual) |
| Resolution | 13 MP (wide); 13 MP (wide) |
| Autofocus | AF, AF |
| Video Recording | 1200 @ 30 fps |
| Others | LED Flash, panorama, HDR, Stereoscopic AI-powered 3D capture |
| Front Camera | Specification |
| Number of Cameras | 2 (Dual) |
| Resolution | 8 MP (ultra wide); 8 MP (ultra wide) |
| Aperture | f/2.2, f/2.2 |
| Field of View | 105°, 105° |
| Video Recording | 1200 @ 30 fps |

The captured videos (in mp4 file) need further processing (e.g., reading the right/left views and concatenate them into one video frame) the scripts can be found in Annex D.2.3. An FFmpeg command described in Annex D.2.4 can be used to save each frame into a proper test sequence,

The test sequence sample ([link: https://pan.baidu.com/s/1A0mTN97gsqQDaNpNjkBS-Q?pwd=wqdi;Password: wqdi]), provided by CMCC, is a stereoscopic 3D video (1920*1200 per eye, 30fps) captured from ZTE Nubia Pad 3D IITM main camera and post-processed by the above tools.

Editor’s NOTE: More sequences can be provided using this method if the quality of sample sequence is acceptable.

=====  CHANGE =====

Annex D: Software Package

D.1	Introduction

<TBD>

D.2	Video Processing

D.2.1 	Overview

<TBD>

D.2.2 	Common Color Conversion

A Python script for converting YUV to RGB format can be found blow:

- import cv2

- import numpy as np

- def read_yuv_frame(file, width, height):

- # calculate size of YUV frame

- frame_size = width * height * 3 // 2

- # Read YUV frames

- yuv = np.fromfile(file, dtype=np.uint8, count=frame_size)

- if yuv.size != frame_size:

- return None

- # reconstruct YUV data to YUV420 format

- y = yuv[0:width*height].reshape((height, width))

- u = yuv[width*height:width*height + (width//2)*(height//2)].reshape((height//2, width//2))

- v = yuv[width*height + (width//2)*(height//2):].reshape((height//2, width//2))

- return y, u, v

- def yuv_to_rgb(y, u, v):

- # convert YUV420 to YUV444

- u = cv2.resize(u, (y.shape[1], y.shape[0]), interpolation=cv2.INTER_LINEAR)

- v = cv2.resize(v, (y.shape[1], y.shape[0]), interpolation=cv2.INTER_LINEAR)

- yuv = np.stack((y, u, v), axis=-1)

- # convert yuv to rgb

- rgb = cv2.cvtColor(yuv, cv2.COLOR_YUV2RGB)

- return rgb

- def main():

- # read YUV file path

- yuv_file_path = 'video.yuv'

- # set width and height

- width, height = 1920, 1080

- with open(yuv_file_path, 'rb') as file:

- while True:

- # red yuv frame

- yuv_frame = read_yuv_frame(file, width, height)

- if yuv_frame is None:

- break

- y, u, v = yuv_frame

- # convert yuv to rgb

- rgb_frame = yuv_to_rgb(y, u, v)

- # display rgb frame

- cv2.imshow('RGB Frame', rgb_frame)

- if cv2.waitKey(30) & 0xFF == ord('q'):

- break

- cv2.destroyAllWindows()

- if __name__ == "__main__":

- main()

A Python script for converting RGB to YUV format can be found blow:

- import cv2

- import numpy as np

- import time

- def rgb_to_yuv420(frame):

- # Using OpenCV for converting RGB to YUV format

- yuv = cv2.cvtColor(frame, cv2.COLOR_RGB2YUV_I420)

- return yuv

- def main():

- # read RGB video file path

- rgb_video_path = 'video.mp4'

- # set output YUV file path

- yuv_file_path = 'output.yuv'

- # open video file

- cap = cv2.VideoCapture(rgb_video_path)

- # check if video file is open

- if not cap.isOpened():

- print("Error: Could not open video.")

- return

- # get width and height of video file

- width = int(cap.get(cv2.CAP_PROP_FRAME_WIDTH))

- height = int(cap.get(cv2.CAP_PROP_FRAME_HEIGHT))

- print(f'Video dimensions: {width}x{height}')

- # set framerate

- fps = 60

- frame_duration = 1.0 / fps

- with open(yuv_file_path, 'wb') as yuv_file:

- while cap.isOpened():

- start_time = time.time()

- # read RGB frame

- ret, frame = cap.read()

- if not ret:

- break

- # convert RGB to YUV

- yuv_frame = rgb_to_yuv420(frame)

- # write YUV frame to yuv file

- yuv_file.write(yuv_frame)

- elapsed_time = time.time() - start_time

- wait_time = max(1, int((frame_duration - elapsed_time) * 1000))

- if cv2.waitKey(wait_time) & 0xFF == ord('q'):

- break

- cap.release()

- cv2.destroyAllWindows()

- if __name__ == "__main__":

- main()

D.2.3 	Video Composition

A Python script for reading the right/left views and concatenating them into one video frame can be found below:

- import numpy as np

- import json

- import subprocess

- def videoInfo(filename):

- proc = subprocess.run([

- *"ffprobe -v quiet -print_format json -show_format -show_streams".split(),

- filename

- ], capture_output=True)

- proc.check_returncode()

- return json.loads(proc.stdout)

- def readVideo(filename):

- cmd = ["ffmpeg", "-i", filename]

- streams = 0

- for stream in videoInfo(filename)["streams"]:

- index = stream["index"]

- if stream["codec_type"] == "video":

- width = stream["width"]

- height = stream["height"]

- cmd += "-map", f"0:{index}"

- streams = streams + 1

- cmd += "-f", "rawvideo", "-pix_fmt", "rgb24", "-"

- shape = np.array([streams, height, width, 3])

- with subprocess.Popen(cmd, stdout=subprocess.PIPE) as proc:

- while True:

- # One byte per each element

- data = proc.stdout.read(shape.prod())

- if not data:

- return

- yield np.frombuffer(data, dtype=np.uint8).reshape(shape)

- if __name__ == '__main__':

- import matplotlib.pyplot as plt

- from PIL import Image

- idx = 1

- for left, right in readVideo("./StereoCaptured.mp4"):

- # concatenate left and right

- img = np.concatenate((left[0:1080,:], right[0:1080,:]), axis=1)

- img = Image.fromarray(img)

- img.save(f"imgs/{idx}.jpg")

- idx += 1

D.2.4 	FFmpeg Tools

FFmpeg is a universal media converter. It can read a wide variety of inputs - including live grabbing/recording devices - filter, and transcode them into a plethora of output formats.

-	An FFmpeg command for storage each frame into .yuv:

- ffmpeg -start_number 1 -r 30 -i %d.jpg -pix_fmt yuv420p -s 1920x1200 out.yuv