# 3GPP TSG-SA WG4 Meeting #131	S4-250071

# Geneva, CH, 17 – 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | pseudo | rev | - | Current version: | 0.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Scenario 1: UE-to-UE Stereoscopic Video Live Streaming - Performance Metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The verified Objective Metrics (e.g., HV3D Quality Metrics) and Subjective Assessment methods for stereoscopic video (S4-241711) has been discussed and agreed to be added to Clause 4.2 of the permanent document of TR 26.956. This content is expected to be reviewed and approved as performance metrics for the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document provides performance metrics for UE-to-UE stereoscopic video live streaming scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete 7.2.5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[LS-1]	Wang, Y., Lu, Z., Cao, P. et al. How Live Streaming Changes Shopping Decisions in E-commerce: A Study of Live Streaming Commerce. Comput Supported Coop Work 31, 701–729 (2022). https://doi.org/10.1007/s10606-022-09439-2

[LS-2]	Xie, Junyuan et al. “Deep3D: Fully Automatic 2D-to-3D Video Conversion with Deep Convolutional Neural Networks.” European Conference on Computer Vision (2016).

[LS-3]	Dumic, E. et al.. “Transmission of 3D Video Content. In: Assunção, P., Gotchev, A. (eds) 3D Visual Content Creation, Coding and Delivery.” Signals and Communication Technology (2019). Springer, Cham. https://doi.org/10.1007/978-3-319-77842-6_8

[LS-4]	Schierl, Thomas and Sam Narasimhan. “Transport and Storage Systems for 3-D Video Using MPEG-2 Systems, RTP, and ISO File Format.” Proceedings of the IEEE 99 (2011): 671-683.

[LS-5]	3GPP TR 26.905 V 18.0.0: "Mobile stereoscopic 3D video"

[LS-6]	ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 - JCT3V-C0032: A human visual system based 3D video quality metric

[LS-7]	A. Banitalebi-Dehkordi, M. T. Pourazad and P. Nasiopoulos, "A human visual system-based 3D video quality metric," 2012 International Conference on 3D Imaging (IC3D), Liege, Belgium, 2012, pp. 1-5, doi: 10.1109/IC3D.2012.6615146.

[LS-8]	Banitalebi-Dehkordi, Amin, Mahsa T. Pourazad, and Panos Nasiopoulos. "An efficient human visual system based quality metric for 3D video." Multimedia Tools and Applications 75, no. 8 (2016): 4187-4215.

[LS-9]	Recommendation ITU-R BT.500-13, “Methodology for the subjective assessment of the quality of the television pictures”, 2012.

[LS-10]	ISO/IEC JTC1/SC29/WG11 (MPEG), Document N12036, “Call for proposals on 3D video coding technology,” 96th MPEG meeting, Geneva, March 2011.

[LS-11]	Q. Hyunh-Thu, P. L. Callet, and M. Barkowsky, “Video quality assessment: from 2D to 3D challenges and future trends,” IEEE 17th International Conference on Image Processing, (ICIP), pp.4025-4028, 2010.

=====  CHANGE =====

7.2.5	Performance Metrics

7.2.5.1		Objective Metrics

Objective evaluation of stereoscopic video can be conducted by applying 2D video quality metrics (e.g., PSNR, SSIM, SNR...) to each source view position and computing their average.

A full-referenced human visual-system-based quality metric for stereoscopic videos called HV3D had been proposed in ITU-T [LS-6]. As shown in Figure 7.2.5.1-1, HV3D metric takes into account the quality of individual views, the quality of the cyclopean view (fusion of the right and left view, what the viewer perceives), as well as the quality of the depth information. The HV3D quality metric is taking values between 0 and 1 (because it is optimized to be correlated with MOS/10), and higher than 1 in case quality is improved.

Figure 7.2.5.1-1 HV3D Flowchart

7.2.5.1.1	Quality of individual views

The metrics are computed for the quality of individual views that form the stereo pair. The quality of the distorted right view with respect to its matching reference view is calculated as followings. The quality of the left view is calculated in the same fashion.

w1QR'= w1VIF(YR,YR')+w4VIF(UR,UR')+w4VIF(VR,VR')

Where YR and YR’ are luma information of the reference and distorted right views respectively, UR and VR are the chroma information of the reference right-view, UR’ and VR’ are the chroma information of the distorted right-view, w1 and w4 are weighting constants.

7.2.5.1.2	Quality of cyclopean view

A 3D-DCT transform is then applied to each pair of matching blocks, generating two 16×16 DCT-blocks containing the DCT coefficients of the fused blocks. Given the human visual system’s sensitivity to contrast, a 16×16 Contrast Sensitivity Function (CSF) modeling mask is applied to the 16×16 DCT block, assigning greater weights to frequencies that are more perceptually significant. This process is illustrated as follows:

XC=i=116j=116Ci,jXi,j

Where XC is the cyclopean-view model for a pair of matching blocks in the right and left views, Xi, j are the low frequency 3D-DCT coefficients of the fused view, i and j are the horizontal and vertical indices of coefficients, and Ci, j is the CSF modeling mask.

Once the cyclopean-view model for all the blocks within the distorted and reference stereoscopic views is obtained, the quality of the cyclopean view is calculated as follows:

QR'L'=VIF(D,D')βi=1NSSIM(IDCT (XCi),IDCT (XCi’))N

7.2.5.1.3	Quality of depth maps

The quality of depth information plays an important role in the perceptual of the stereoscopic video content. In HV3D, the quality of depth map is chosen to be variance-dependent, and it is formulated as follows:

QD'=VIF(D,D')βi=1Nσdi2N.max(σdi2 | i =1,2,..N)

Where σdiis the variance of block i in the depth map of the 3D reference view, β is a constant = 0.7 and N is the total number of blocks. The local disparity variance, σdi2, is defined as follows:

σdi2=164×64−1k, l=164(Md−Rk,l)2

Where Md is the mean of the depth value of each 64×64 block in the normalized reference depth map. The reference depth map is normalized with respect to its maximum value per frame, ranging between 0 and 1. Rk,l is the depth value of pixel (k, l) in the outer 64×64 block within the normalized reference depth map.

[7.2.5.1.4	Weighting constants

Weighting constants are found using least mean square technique such that the difference between the HV3D metric values and the MOS values was minimized for the training video set.

minwi, i=1,2,3,4{|| HV3D−MOS||2}

In study [LS-7], and experiment has been conducted to to determine the best values for the weighting constants w1, w2, w3 and w4 which result in the minimum mean of square errors between our HV3D index and the MOS. Table 7.2.5.1.4-1 shows the resulting values for these constants.

Table 7.2.5.1.4-1 Weighting Constants

| w1 | w2 | w3 | w4 |
| --- | --- | --- | --- |
| 0.14 | 0.1208 | 0.05 | 0.1353 |

]

7.2.5.2		Subjective Evaluation

According to [LS-8], the viewing conditions for subjective tests were set according to the ITU-R Recommendation BT.500-13 [LS-9]. The evaluation was performed using a 46” Full HD Hyundai 3D TV (Model: S465D) with passive glasses. The peak luminance of the screen was set at 120 cd/m2 and the color temperature was set at 6500K according to MPEG recommendations for the subjective evaluation of the proposals submitted in response to the 3D Video Coding Call for Proposals [LS-10]. The wall behind the monitor was illuminated with a uniform light source (not directly hitting the viewers) with the light level less than 5 % of the monitor peak luminance.

A total of 88 subjects participated in the subjective test sessions, ranging from 21 to 32 years old. All subjects had none to marginal 3D image and video viewing experience. They were all screened for color blindness (using Ishihara chart), visual acuity (using Snellen charts), and stereovision acuity (via Randot test– graded circle test 100 seconds of arc). Subjective evaluations were performed on both training and validation data sets.

Test session started after a short training session, where subjects became familiar with video distortions, the ranking scheme, and test procedure. Test sessions were set up using the single stimulus (SS) method where videos with different qualities were shown to the subjects in random order (and in a different random sequence for each observer). Each test video was 10 seconds long and a four-second gray interval was provided between test videos to allow the viewers to rate the perceptual quality of the content and relax their eyes before watching the next video. There were 11 discrete quality levels (0-10) for ranking the videos, where score 10 indicated the highest quality and 0 indicated the lowest quality. Here, the perceptual quality reflects whether the displayed scene looks pleasant in general. In particular, subjects were asked to rate a combination of “naturalness”, “depth impression” and “comfort” as suggested by [LS-11]. After collecting the experimental results, we removed the outliers from the experiments (there were seven outliers) and then the mean opinion scores (MOS) from the remaining viewers were calculated. Outlier detection was performed in accordance to ITU-R BT.500-13, Annex 2 [LS-9].

7.2.5.3	Correlation between the objective and subjective metrics

The performance of the objective metric, HV3D, described in section 7.2.5.1 is validated by subjective test described in section 7.2.5.2, using 88 subjects, following the ITU-R BT.500-13 recommendation. Performance evaluation results showed that HV3D quality metric quantifies quality degradation caused by several representative types of distortions very accurately, with Pearson correlation coefficient of 90.8 % [LS-8].

7.2.5.4	Verification and crosscheck

All produced bitstreams, metric results and produced videos shall be crosschecked by at least one other 3GPP SA4 member to ensure that results are correct.