| 3GPP TS 26.567 V0.4.1 (2025-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Split Rendering over IMS; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	System description	8

4.1	Overview	8

4.2	Reference Architecture	10

4.3	Reference Points	10

4.4 	Split Rendering DCMTSI Client (SR-DCMTSI)	11

4.5	Media Function (MF)	11

4.6      DC Application Server (DC AS)	12

5	Media codecs, configuration, and data transport	12

5.1	General	12

5.2	Media codecs	12

5.3	Media configuration	13

5.4	Data transport	13

5.4.1	General	13

5.4.2	Metadata Formats	13

5.4.2.1	General	13

5.4.2.2	Pose Format	13

5.4.2.3	Action Format	14

5.4.3	  Metadata Data Channel Message Format	14

6	Split Rendering Metrics	14

6.1 	Metrics definition and formats	15

6.2 	Metrics Configuration	15

6.3 	Metrics Reporting	15

6.3.1 	General	15

6.3.2 	Report format	15

7	Procedures	18

7.1	Procedures for session establishment	18

7.1.1 	Procedures for P2P session establishment	19

7.1.2 	Procedures for P2A(/2P) session establishment	21

7.2	Procedures for session modification	22

7.2.1 	Procedures for P2P session modification	23

7.3	Network support procedures	25

7.3.1	Processing Delay adaptation based on QoE metrics	27

Annex A (normative):	30

A.1	Supported Metadata Formats and Message Types	30

A.1.1	Adaptation Split	30

A.1.2	Seamless Adaptive Split	31

A.1.3	Processing Delay Adaptation based on QoE metrics	32

Annex B(informative):	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document specifies functional entities, reference points and protocols for IMS-based split rendering. It also provides codecs for delivery of split-render media and metadata of split rendered content for both uplink and downlink. The present document also contains procedures of split rendering session establishment, session management, adaption, as well as other procedures to support split rendering process based on network support functions.

Key use cases enabled by this specification include XR services incorporating real-time and non-real-time media in industrial (e.g., for monitoring, maintenance, collaboration and tele-operation), enterprise and educational environments; entertainment use-cases, including cloud-gaming, and shared and collaborative entertainment and productivity XR services.

# 1	Scope

The present document specifies functional entities, reference points and protocols for IMS-based split rendering. It also provides codecs for delivery of split-render media and metadata of split rendered content for both uplink and downlink. The present document also contains procedures of split rendering session establishment, session management, adaption, as well as other procedures to support split rendering process based on network support functions.

Key use cases enabled by this specification include XR services incorporating real-time and non-real-time media in industrial (e.g., for monitoring, maintenance, collaboration and tele-operation), enterprise and educational environments; entertainment use-cases, including cloud-gaming, and shared and collaborative entertainment and productivity XR services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 26.264: "IMS-based AR Real-Time Communication".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[6]	3GPP TS 26.119: "Device Media Capabilities for Augmented Reality Services".

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[8]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[9]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[10]	3GPP TS 28.405; "Management of Quality of Experience (QoE) measurement collection; Control and configuration"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

SR-DCMTSI Client	: a Split-Rendering capable DCMTSI Client

Split rendering session: a media session running the split rendering process between two or more entities. A split rendering session may include a data channel, consisting of application data channel, and one or more RTP streams for delivering split rendered media.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DC	Data Channel

DC AS	Data Channel Application Server

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Core Network Subsystem

MF	Media Function

MTSI	Multimedia Telephony Service for IMS

P2A	Person to Application

P2P	Person to Person

QoE	Quality of Experience

UE	User Equipment

# 4	System description

## 4.1	Overview

The IMS stage-2 service description is specified in TS 23.228. The IMS architecture enhancements to support data channel services can be found in TS 23.228 Annex AC [2], which supports separation of signalling function and media function supporting data channel services.

Figure 4.1.1: Architecture of IMS supporting DC usage with MF

Editor’s note: above content will be updated to align with TS 23.228 as well as SA2 R19 IMS related work.

Based on the architecture in Figure 4.1.1, TS 26.264 [3] provides the IMS-based conversational AR (Augmented reality) services, including the generalized end-to-end architecture to support AR communication over IMS DC, as shown in Figure 4.1.2.

Figure 4.1.2: Generalized IMS DC Architecture to support AR communication (TS 26.264)

Accordingly, AR Application Server (AR AS) is responsible for AR service control related to AR communication, including AR session media control and AR media capability negotiation with the UE.

The DCSF receives event reports from the IMS AS, and decides whether AR communication service is allowed to be provided during the IMS session. Additionally, the DCSF interacts with the AR AS for DC resource control.

MF/MRF supports AR conversational service by providing transcoding for terminals with limited capabilities. Additionally, the MF/MRF may collect spatial and media descriptions from UEs and create scene descriptions for symmetrical AR call experiences. MF/MRF also provide remote rendering for AR-MTSI clients in terminals with limited capabilities based on rendering negotiation. For remote rendering the AR-MTSI client provides AR metadata.

The IMS AS receives the media control instructions from the DCSF and accordingly interacts with the UE for connecting the UE's audio/video media termination to the MF/MRF and interacts with MF/MRF for data channel media resource management for AR media processing.

According to the architectures above, the present document introduces a mapping to the IMS architecture for IMS-based split rendering.

## 4.2	Reference Architecture

The generalized IMS DC architecture to support split rendering is shown in Figure 4.2.1.

Figure 4.2.1: Generalized IMS DC Architecture to support split rendering

Split-Rendering DCMTSI Client (SR-DCMTSI Client): A DCMTSI Client, that is responsible for acquiring the UE media capabilities and interacting with the MF during the split-rendering process.

Media Function (MF): A Media Function, that is responsible for interacting with SR-DCMTSI Client during split-rendering process, monitoring resource usage, and managing/running the split rendering process, etc.

Data Channel Application Server (DC AS): A DC Application Server, that is responsible for service control related to split-rendering, including session media control and media capability negotiation with the UE via MF, etc.

Editor’s note: the definition of SR-DCMTSI Client will be revisited (legacy device or new device) according to the detailed functions defined in this specification.

## 4.3	Reference Points

As shown in Figure 4.2.1, the following reference points are used to enable split rendering over IMS:

Mb: Reference point to enable split rendering between UE and MF, as specified in TS 23.228 [2].

Gm: Reference point to support communication between UE and IMS, as specified in TS 23.228 [2].

MDC1: Reference point for transport of data channel media between data channel media function and DCSF. MF terminates the bootstrap data channel from the UE and forward HTTP traffic between UE and DCSF via MDC1, as specified in TS 23.228 [2].

MDC2: Reference point for transport of data channel media between data channel media function and DC Application Server, for split rendering functions between application server to MF. MF relay traffic on A2P/P2A application data channels between the UE and the DC Application Server via MDC2, as specified in TS 23.228 [2].

The following reference points are also used to support split rendering related procedures:

DC1: Reference point between the DCSF and the IMS AS, as specified in TS 23.228 [2].

DC2: Reference point between the IMS AS and MF, for split rendering related data channel media resource management, as specified in TS 23.228 [2].

DC3: Reference point between the DCSF and NEF, as specified in TS 23.228 [2].

DC4: Reference point between the DCSF and DC Application Server, as specified in TS 23.228 [2].

N33: Reference point between NEF and DC Application server, network exposure to enable split rendering related applications, as specified in TS 23.501 [x].

Note: DC5, ISC, and N5 are out of the scope of this specification.

## 4.4 	Split Rendering DCMTSI Client (SR-DCMTSI)

An SR-DCMTSI is a DCMTSI client in terminal defined as per clause 3.1 of TS 26.114 which supports split rendering. It is responsible for acquiring media capabilities and interacting with IMS functions during a split rendering session. It is responsible for the signalling with DCSF via IMS AS to set-up a split rendering session and negotiating with the MF and DCAS about the parameters of the split rendering session, for example, the scene to be rendered and the split of rendering operations.

The SR-DCMTSI Client performs the following functions:

-	Initiate a request of split rendering session establishment,

-	Establish split rendering session, including signalling with DCSF via IMS AS to set up the data channel, and MF and DC AS to set up the split rendering,

-	Operate the rendering loop on the UE,

-	Update the rendering loop according to the network parameters and media capabilities with IMS functionalities,

-	Stop the split rendering session or switch from a split rendering session to a local rendering session.

## 4.5		Media Function (MF)

MF is a Media Function which supports split rendering. It is responsible for interacting with SR-DCMTSI client and the DC AS during split-rendering process, monitoring resource usage, managing and running the split rendering process, etc.

The MF performs the following functions:

- Negotiate and interact with SR-DCMTSI Client (via IMS AS) and DC AS to establish the split rendering session,

- Operate the rendering loop, including, receiving meta-data from the SR-DCMTSI client, rendering frames, and forwarding requests to DC AS based on the split rendering logic,

- Expose its capabilities, capacity and media processing and rendering capabilities to the DC AS,

- Update the rendering loop according to the split rendering logic and requests from UE. If supported, the update can be based on the network parameters and interactions with other IMS functionalities,

- Stop/Pause/Resume the split rendering session according to the requests from UE.

## 4.6    DC Application Server (DC AS)

DC Application Server (DC AS) is responsible for service control related to split-rendering, including session media control and, session setup and media capability negotiation with the SR-DCMTSI client via MF and DCSF where applicable. The DC-AS may provide split rendering application source data to the MF. The DC AS may be in the media path via MDC2, for example in P2A, A2P or P2A2P scenarios as defined in clause AC.7.2.2 and AC.7.2.3 of TS 23.228 [2]. The DC-AS may perform split rendering when it is in the media path.

The DC AS performs the following functions:

- Provide the DC application service, including the IMS DC application, to the DCSF.

- Negotiate and interact with SR-DCMTSI Client via MF and DCSF to establish the split rendering session,

- Query MF capabilities and capacity.

- Provide application source data for split rendering to the MF and to the SR-DCMTSI client

- If the DC AS is performing split rendering,

- Operate the rendering loop, including receiving meta-data from the SR-DCMTSI client via the MF, rendering frames based on the split rendering logic.

- Update the rendering loop according to the requests from UE via the MF

- Stop/Pause/Resume the split rendering session according to the requests from the SR-DCMTSI client.

# 5	Media codecs, configuration, and data transport

## 5.1	General

Editor’s note: this clause provides media formats, signalling protocols to support split rendering, etc.

## 5.2	Media codecs

Editor’s note:	media format, where possible, references to TS 26.114, TS 26.264, TS 26.119, TS 26.522 and TS 26.565.

An SR-DCMTSI client that supports audio shall support the codec requirements for MTSI clients as specified in clause 5.2.1 of TS 26.114 [7].

An SR-DCMTSI client that supports video shall support the codecs requirements for MTSI clients as specified in clause 5.2.2 of TS 26.114[7].

An SR-DCMTSI client that supports real-time text shall support the codec requirements for MTSI clients as specified in clause 5.2.3 of TS 26.114[7].

An SR-DCMTSI client that supports still images shall support the codec requirements for MTSI clients as specified in clause 5.2.4 of TS 26.114[7].

SR DCMTSI clients conforming to device types defined in clause 10 of TS 26.119 [6] should conform to media capabilities recommended for the respective device type in clause 10 of TS 26.119 [6].

Editor’s Note: Media capabilities of SR DCMTSI clients may be further defined based on further study.

## 5.3	Media configuration

SR-DCMTSI clients shall support media configuration requirements specified in clause 6 of TS 26.114 [7].

## 5.4	Data transport

## 5.4.1	General

Editor’s note: This clause defines media and metadata formats that are used for IMS-based split rendering.

An SR-DCMTSI client shall support data channel media and support procedures in clause 6.2.10 of TS 26.114 [7].

An SR-DCMTSI client shall support the data transport requirements specified in clause 7 of TS 26.114 [7].

Application data channels over which meta-data is transported shall support the IMS DC requirements in TS 23.228 [2], and requirements specified in clause 5.4.2.

## 5.4.2	Metadata Formats

#### 5.4.2.1	General

SR-DCMTSI client and Media Function shall support the usage of the IMS data channel for the exchange of split rendering metadata with the MF. The data channel shall declare “3gpp-sr” as the data channel sub-protocol. The message content format depends on the type of the message. The data channel sub-protocol is defined in clause 8.3.3 of TS 26.565 [5].

#### 5.4.2.2	Pose Format

For XR services, the pose information format that is used for IMS-based split rendering [shall] comply with the format defined in clause 12.2 of TS 26.119 [6]. The pose information [shall] be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause 8.3.3 of TS 26.565 [5]. The message type [shall] be “urn:3gpp:split-rendering:v1:pose”.

#### 5.4.2.3	Action Format

The action information format that is used for IMS-based split rendering [shall] comply with the format defined in clause 12.3 of TS 26.119 [6]. The action information [shall] be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause 8.3.3 of TS 26.565 [5]. The message type [shall] be “urn:3gpp:split-rendering:v1:action”.

## 5.4.3	  Metadata Data Channel Message Format

For the carriage of metadata defined in clause 5.4.2, such as pose and action information, the SRDCMTSI client and MF shall use an IMS application data channel. The data channel sub-protocol shall be identified as “3gpp-sr”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The split rendering metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

A data channel message may carry one or more split rendering messages as defined in Clause 8.3.3 of TS 26.565 [5] and reproduced in Table 5.4.3-1.

Table 5.4.3-1 Split Rendering Metadata Messages Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| messages | Array(Message) | 1..n | A list of split rendering metadata messages. Each message shall be formatted according to the Message data type as defined in Table |

Each split rendering message shall follow the format specified in Clause 8.3.3 of TS 26.565 [5] and reproduced in Table 5.4.3-2.

Table 5.4.3-2 Split Rendering Metadata Message Data Type

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | An unique identifier of the message in the scope of the data channel session. |
| Type | string | 1..1 | A urn that identifies the message type. |
| Message | object | 1..1 | The message content depends on the message type. |
| sendingAtTime | number | 0..1 | The time when the split rendering metadata message is transmitted from the split rendering client to the split rendering server. |

# 6	Split Rendering Metrics

Editor’s note: this clause defines a set of metrics that are relevant to the operation of a split rendering session.

## 6.1 	Metrics definition and formats

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall support the collection and reporting of the metrics in clause 16.2 of TS 26.114 [7], which include corruption duration metric, successive loss of RTP packets, frame rate, jitter duration, sync loss duration, round-trip time, average codec bitrate, codec information, call setup time.

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall also support the collection and reporting of the split-rendering latency metrics defined in clause 9.3.4 of TS 26.565 [8] according to the QoE metrics configuration, which include pose-to-render-to-photon, render-to-photon, round-trip interaction delay, user interaction delay, age of content, scene update delay, metadata delay, and data frame delay metrics. A Media Function may use the “QoE timing information” defined in clause 9.3.2 of TS 26.565 [8] to transmit the timing information required for measuring the QoE latency metrics to an SR-DCMTSI client.

The metrics listed above are valid for one or more media types such as speech/audio, video, and text, and are calculated for each measurement resolution interval "Measure-Resolution". They are reported to the OAM [10] or QoE server as shown in figure 16.5.1-1 of TS 26.114 [7] according to the measurement reporting interval "Sending-Rate" and after the end of the session.

## 6.2 	Metrics Configuration

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall support the usage of an OMA-DM solution [9] for configuration of QoE metrics and their activation. As an alternative, metrics configuration can be specified by the QoE Measurement Collection (QMC) functionality.

## 6.3 	Metrics Reporting

## 6.3.1 	General

The metrics reporting procedure specified in clause 16.4 of TS 26.114 [7] allows the SR-DCMTSI client to send QoE metrics reports to the QoE server.

An SR-DCMTSI Client shall report QoE metrics specified in clause 6.2 for the real-time media it has received using the protocol specified in clause 16.4 of TS 26.114 [7] according to the QoE metrics reporting configuration obtained in a 3GPP MTSIQOE (MTSI QoE metrics) management object (see clause 16.3.1 of TS 26.114 [7]) or in an RRC message (see clause 16.5.1 of TS 26.114 [7]).

The quality metrics report follows the XML-based report format defined in clause 6.3.2.

SR-DCMTSI Clients shall use the MIME type "application/3gprtc-qoe-report+xml" for an XML-formatted QoE report. The metrics report format is defined in clause 6.3.2.

## 6.3.2 	Report format

The QoE report is formatted as an XML document that complies with the XML schema in listing 16.4.1of TS 26.114 [7].

The schema in listing 6.3.2-1 allows the SR-DCMTSI client to report QoE metrics using the metrics reporting mechanism specified in clause 16.4 of TS 26.114 [7].

The filename of this schema is "TS26567_SR_IMSQoEMetrics.xsd".

Listing 6.3.2-1: SR_IMS QoE Metrics XML schema

<?xml version="1.0" encoding="UTF-8"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2024:RTC:SR_IMSQoEMetrics" xmlns="urn:3gpp:metadata:2024:RTC:SR_IMSQoEMetrics" xmlns:sv="urn:3gpp:metadata:2017:MTSI:schemaVersion" elementFormDefault="qualified"> <xs:element name="QoeReport" type="QoeReportType"/> <xs:complexType name="QoeReportType"> <xs:sequence> <xs:element name="statisticalReport" type="starType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="starType"> <xs:sequence> <xs:element name="mediaLevelQoeMetrics" type="mediaLevelQoeMetricsType" minOccurs="1" maxOccurs="unbounded"/> <xs:element ref="sv:delimiter"/> <xs:element name="latencyQoeMetric" type="QoeMetricType" minOccurs="0" maxOccurs="unbounded"/> <xs:element ref="sv:delimiter"/> </xs:sequence> <xs:attribute name="startTime" type="xs:unsignedLong" use="required"/> <xs:attribute name="stopTime" type="xs:unsignedLong" use="required"/> <xs:attribute name="callId" type="xs:string" use="required"/> <xs:attribute name="clientId" type="xs:string" use="required"/> <xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/> <xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/> <xs:attribute name="dnn" type="xs:string" use="optional"/> <xs:attribute name="snssai" type="xs:unsignedLong" use=”optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="mediaLevelQoeMetricsType"> <xs:sequence> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="mediaId" type="xs:integer" use="required"/> <xs:attribute name="totalCorruptionDuration" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfCorruptionEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="corruptionAlternative" type="xs:string" use="optional"/> <xs:attribute name="totalNumberofSuccessivePacketLoss" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfSuccessiveLossEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="numberOfReceivedPackets" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="framerate" type="doubleVectorType" use="optional"/> <xs:attribute name="totalJitterDuration" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfJitterEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="totalSyncLossDuration" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfSyncLossEvents" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="networkRTT" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="internalRTT" type="unsignedLongVectorType" use="optional"/> <xs:attribute name="codecInfo" type="stringVectorType" use="optional"/> <xs:attribute name="codecProfileLevel" type="stringVectorType" use="optional"/> <xs:attribute name="codecImageSize" type="stringVectorType" use="optional"/> <xs:attribute name="averageCodecBitrate" type="doubleVectorType" use="optional"/> <xs:attribute name="callSetupTime" type="xs:unsignedLong" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:element name="latencyQoeMetric" type="QoeMetricType"/> <xs:complexType name="QoeMetricType"> <xs:sequence> <xs:choice> <xs:element name="poseToRenderToPhoton" type="PoseToRenderToPhotonType"/> <xs:element name="renderToPhoton" type="RenderToPhotonType"/> <xs:elementname="roundTripInteractionDelay" type="RoundTripInteractionDelayType"/> <xs:element name="userInteractionDelay" type="UserInteractionDelayType"/> <xs:element name="ageOfContent" type="AgeOfContentType"/> <xs:element name="sceneUpdateDelay" type="SceneUpdateDelayType"/> <xs:element name="metadataDelay" type="MetadataDelayType"/> <xs:element name="dataFrameDelay" type="DataFrameDelayType"/> </xs:choice> <xs:element ref="sv:delimiter"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PoseToRenderToPhotonType"> <xs:attribute name="avgPoseToRenderToPhoton" type="doubleVectorType" use="optional"/> <xs:attribute name="minPoseToRenderToPhoton" type="unsignedIntVectorType" use=" optional"/> <xs:attribute name="maxPoseToRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RenderToPhotonType"> <xs:attribute name="avgPoseToRenderToPhoton" type="doubleVectorType" use="optional"/> <xs:attribute name="minRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxRenderToPhoton" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RoundTripInteractionDelayType"> <xs:attribute name="avgRoundTripInteractionDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfUserActions" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minRoundTripInteractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minActionIds" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxRoundTripInteractionDelay" type="UnsignedIntVectorType" use="optional"/> <xs:attribute name="maxActionIds" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="UserInteractionDelayType"> <xs:attribute name="avgUserInetractionDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfUserActions" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minUserInetractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minActionIds" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxUserInteractionDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxActionIds" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="AgeOfContentType"> <xs:attribute name="avgAgeOfContent" type="doubleVectorType" use="optional"/> <xs:attribute name="mumberOfSceneEvents" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minageOfContent" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxAgeOfContent" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="SceneUpdateDelayType"> <xs:attribute name="avgSceneUpdateDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfSceneUpdates" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minsceneUpdateDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxsceneUpdateDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="MetadataDelayType"> <xs:attribute name="avgMetadataDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfMetadataMessages" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minMetadataDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxMetadataDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="DataFrameDelayType"> <xs:attribute name="avgDataFrameDelay" type="doubleVectorType" use="optional"/> <xs:attribute name="numberOfDataFrames" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="minDataFrameDelay" type="unsignedIntVectorType" use="optional"/> <xs:attribute name="maxDataFrameDelay" type="unsignedIntVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="doubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="stringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <xs:simpleType name="unsignedLongVectorType"> <xs:list itemType="xs:unsignedLong"/> </xs:simpleType> </xs:schema>

# 7	Procedures

## 7.1	Procedures for session establishment

The figure 7.1-1 indicates the general procedures of split rendering session establishment including session setup and negotiation.

Figure7.1-1: General procedures for split rendering session establishment

The steps are as follows:

Step 1: The UE1 initiates an IMS session including bootstrap data channel establishment with the originating MF.

Step 2: UE1 sends a request to IMS AS to establish an application data channel for a split rendering session:

The UE1 can initiate a request of split rendering session establishment if its media capabilities cannot meet the related media rendering requirements. Then the UE1 calculates which objects can be rendered by itself based on its status and decides which part of the objects to be rendered in the UE1 and the others to be rendered in the IMS network.

Step 3: The IMS AS sends DCSF the event notifications including split rendering related information.

Step 4: The DCSF receives event notifications from the IMS AS and  processes the session establishment request based on the information in the notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the split rendering request, to instruct IMS AS to terminate the media flow of the UE1 to the MF.

Step 5 and 6: The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2.

Editor’s Note: MF discovery for split rendering is FFS.

Step 7: The IMS AS sends a split rendering session establishment request to the DC AS via the DCSF, , the request may include the information of the objects to be rendered in IMS network.

Step 8: The DC AS sends a description of the split rendering output to the IMS AS via the DCSF.

Step 9: The IMS AS sends the media resource allocation request to the MF, to reserve XR media rendering resource for the UE1.

Step 10: When the resources are allocated successfully, the MF returns a successful response to the IMS AS.

Step 11: The IMS AS returns a successful response to the UE1.

Step 12: Successful split rendering session is established between UE1 and MF through the application data channel.

Editor’s note: more details of session establishments for P2P and P2A scenarios are provided in clause 7.1.1 and 7.1.2.

## 7.1.1 	Procedures for P2P session establishment

The figure 7.1.1-1 indicates the procedures of split rendering session establishment for P2P scenarios.

Figure7.1.1-1: Procedures for P2P split rendering session establishment

The steps are as follows:

Steps 1: IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

Steps 2-12: Split rendering session establishment between UE1 and originating MF according to the steps 2-12 of clause 7.1.

Step 13: Split rendering session established for UE2 between the originating MF and UE2. Similarly, seps 2 to 12 are followed for UE2. Step 2 may be a reply to the application data channel establishment (re)INVITE from UE1.

Step 14: Successful split rendering session is established between UE1 and UE2 through the application data channel anchored by the MF.

## 7.1.2 	Procedures for P2A(/2P) session establishment

Figure7.1.2-1: Procedures for P2A(/2P) split rendering session establishment

The steps are:

Steps 1: IMS session establishment between UE1 and MF, including bootstrap session establishment by UE1 and UE2 with originating MF.

Steps 2-12: Split rendering session establishment between UE1 and originating MF, Same as in steps 2-12 of clause 7.1.1

Step 13: The MF exposes its current capabilities and resources to the DC-AS.

The information exposed may be the hardware and software stack and the resources currently available at the MF.

Step 14: DC-AS selects resources at the MF and asks MF to reserve these resources.

Editor's Note: Details for steps 13 and 14 are FFS.

Step 15: DC-AS transfers application source data, for example scripts and scene graph, graphical assets and split logic to the MF.

Step 16: UE relevant application source data like scene graph, application and split logic is sent to the UE.

## 7.2	Procedures for session modification

A UE may trigger split rendering session modification during an established split rendering session e.g., if the current session no longer meets the media rendering requirements. If the existing MF does not meet the new requirements, the session is migrated to a new MF. The session modification procedure in such a case is shown in Figure 7.2-1.

Figure7.2-1: General procedures for session modification

The steps are as follows:

Step 1: IMS session establishment including bootstrap data channel.

Step 2: split rendering session establishment between UE1 and the MF1 (details see clause 7.1)

Step 3: split rendering session is running.

Step 4: During a running split rendering session, when the UE1 discovers that its media capabilities cannot meet the related media rendering requirements, the UE1; can send a request to modify the split rendering session to the IMS AS.

Step 5: The IMS AS sends updated event notifications to DCSF to modify the current split rendering session.

Step 6: The DCSF receives event notification from the IMS AS and  process the session modification request based on the information in the received notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the new split rendering request; to instruct IMS AS to terminate the media flow of the UE1 to the new MF(s).

Step 7: The IMS AS receives the updated data channel control instructions from the DCSF.

Note: here it is assumed that there are more than one MF in the network. It is also assumed that the serving MF does not satisfy the new request (due to unavailability).

Step 8: The (new) MF2 is discovered (if the existing serving MF1 does not satisfy the UE1 request for session modification).

Editor’s Note: MF discovery for split rendering is FFS.

Step 9: UE1 sends application request to the newly discovered MF2 to request an application list.

Step 10: Accordingly, the MF2 responds with application list including an identifier that if the application is capable to be split rendered. DCSF receives the list and forward to the UE1.

Step 11: The IMS AS sends a split rendering session modification request to the DC AS via DCSF through the established application data channel, the request may include the information of the objects to be rendered in IMS network.

Step 12: The DC AS sends a description of the split rendering output to the IMS AS via DCSF.

Step 13: The IMS AS sends the media resource allocation request to the MF2, to reserve media rendering resource for the UE1.

Step 14: When the resources are allocated successfully, the MF2 returns a successful response to the IMS AS.

Step 15: The IMS AS returns a successful response to the UE1.

Step 16: The modified split rendering session is established between UE1 and MF2 through the data channel.

Note: MF1 should serve the split rendering session until MF2 takes over.

Editor’s note: more details of session modification for P2P and P2A scenarios are provided in clause 7.2.1 and 7.2.2.

## 7.2.1 	Procedures for P2P session modification

The figure 7.2.1-1 indicates the procedures of split rendering session modification for P2P scenarios.

Figure 7.2.1-1: Procedures for P2P split rendering session modification

The steps are as follows:

Steps 1: IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

Steps 2 to 4: Split rendering session establishment between UE1 and UE2 anchored by MF1 (details see clause 7.1.1).

Step 5 to 17: The split rendering session is modified; the new session is established between UE1 and MF2 through the data channel.(details see clause 7.2).

Step 18: The new split rendering session is established between UE1 and UE2 anchored by MF2.

## 7.3	Network support procedures

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering, One such procedure may be to adapt the split of rendering operations between the SR-DCMTSI client and the MF during a split rendering session. This split may be due to change in operating conditions of the split rendering session, for example operating conditions of the UE, the MF or changes in the application or scene being rendered, for example changes in the scene description. Split adaptation may include data exchange, for example, exchange of adaptation messages, application state information and assets needed for the split rendering of an DC application. The following generic procedure shall apply, while the exact details may depend on the DC-application being rendered.

Figure7.3-1: General procedures for adaptation of split rendering

The steps are as follows:

Step 1: The IMS session is established between the SR-DCMTSI client in terminal and a terminating SR-DCMTSI Client which may be in a terminal. For Person to Person calls, procedures in clause 7.1 are followed.

Step 2: A split rendering session is set up between the SR-DCMTSI client and a serving MF.

Step 3: Assets related to the application being split rendered may be delivered to participants of the split rendering session. The asset delivery may include javascript assets, scene descriptions and graphical objects needed for the session.

Rendering Loop:

The rendering loop is executed continuously during the duration of the split rendering session, for each frame.

Step 4: The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. The metadata may include pose, pose predictions, user inputs etc.

Step 5 and 6: The SR-DCMTSI client in terminal and the MF render the frame.

Step 7: The frame rendered by the MF is transmitted to the SR-DCMTSI client in terminal as well as possible metadata.

Step 8: The SR-DCMTSI client in terminal composes a display frame from the received rendered media and media rendered locally.

NOTE: Steps 5,6,7 although ordered above, may occur in any order. Step 8 may include pose-correction. Step 8 and 6 may be executed as a single step.

Further Procedures:

Split Adaptation:

NOTE: Split Adaptation refers to adaptation of split rendering operations in an ongoing split rendering session between the SR-DCMTSI client and MF, without impacting the MF resources provisioned by IMS AS in step 9 of clause 7.1

Step 9: A trigger to adapt the split occurs at the SR-DCMTSI client in terminal; the trigger may be, for example, a change in available UE resources (e.g. battery, compute), changes in QoE of the split rendering session, changes in the scene/application being rendered.

Step 10: The SR-DCMTSI client in terminal decides if a new split of the rendering operations is needed and determines the new split.

Step 11: The SR-DCMTSI client in terminal sends a request to the MF to adapt the split to the new split.

Step 12: The MF actuates the new split of the rendering operations.

Step 13: The MF sends an acknowledgment of the new split to the SR-DCMTSI client in terminal.

Step 14: The MF and UE may exchange messages and data to support the new split of operations. This may include exchange of messages, for example, for synchronization of the state of the scene being split rendered or exchange of assets, for example, those in Step 3.

The meta-data messages exchanged shall follow the formats specified in clause 5.4

Step 15: The rendering loop (steps 4 through 8) continues.

Note:  Split adaptation is shown to be initiated by the SR-DCMTSI client in terminal for clarity, the procedure may be triggered by the MF. Further, other procedures to actuate the new split may be executed during the split rendering session.

## 7.3.1	Processing Delay adaptation based on QoE metrics

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering. An additional procedure may include adjusting various round-trip delays between the SR-DCMTSI client and the MF during a split rendering session. The delay adaptation information may be sent from the SR-DCMTSI client to the MF periodically when the measured QoE metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) goes out of the target delay range. Upon receiving the notification, an MF may adjust the delay involved in some of the processing tasks. For example, an MF may change the Level of Detail (LoD) of the objects in an XR scene which impacts the processing complexity of the XR scene. Delay adaptation procedure may include data exchange, for example, exchange of information messages for delay adaptation. The following generic procedure may apply, while the exact details may depend on the DC-application being rendered.

Figure 7.3.1-1: General procedures for processing delay adaptation based on QoE metrics information

Steps 1 to 3 are as described in clause 7.3.

The steps for processing delay adaptation based on QoE metrics information during the rendering loop are as follows:

The SR-DCMTSI client measures and collects the QoE metrics negotiated in the delay adaptation configuration message. The QoE metrics considered for delay adaptation may contain all or a subset of the QoE latency metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) negotiated in the metrics configuration message in clause 6.3.1.

Step 4: The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. In addition to the pose, pose predictions and user input metadata, the metadata may include a delay adaptation information message to the MF. The trigger to send the delay adaptation information message is based on the configured periodicity to inform the MF that the measured delay of a QoE latency metric is out of the target delay range. For example, the measured poseToRenderToPhoton QoE latency metric goes out of the target delay range due to new network conditions.

Step 5a: The MF may adjust the processing delay based on the delay adaptation information message.

NOTE: For example, the MF may change the LoD of the objects that are part of the scene for the delay adaptation.

Step 6 and 5b: The SR-DCMTSI client in terminal and the MF render the frame.

Steps 7 to 8 are as described in clause 7.3.

Annex A (normative):

## A.1	Supported Metadata Formats and Message Types

This annex defines the metadata and metadata message types supported by this specification. Metadata formats and meta data channel message types and formats supported in this specification re-use or modify formats and message types defined in other 3GPP specifications as defined in Table A.1-1.

Table A.1-1 Formats and relationship with 3GPP specifications

| Format | Source specification | Clause in source specification | Modified for this specification | Clause in this specification |
| --- | --- | --- | --- | --- |
| Pose | TS 26.119 | 12.2 | No | 5.4.2.2 |
| Action | TS 26.119 | 12.3 | No | 5.4.2.3 |
| Metadata Data Channel Message | TS 26.565 | 8.3.3 | No | 5.4.3 |
| Split Adaptation Message | TS 26.565 | C.2.3.2 | Yes | A.2.X |
| Seamless Adaptive Split | TS 26.565 | C.2.3.2 | Yes | A.2.Y |
| State Synchronization Message | TS 26.565 | C.2.3.3 | No | To be filled |

Editor’s Note: The clause may contain all message types for XR and other services. The suitable message types from TS 26.567 need to be referred or modified and imported to this spec as appropriate. If there is a need to further define profiles with support for specific messages as mandatory/optional is FFS.

## A.1.1	Adaptation Split

An SR-DCMTSI client that supports the adaptive split rendering shall support the split adaptation message as defined in table A.1.1-1 below based on the split adaptation message defined clause C.2.3.2 of TS 26.565,

Table A.1.1-1 Message format for split adaptation messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:asrp:sr-split |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | A object identifying objects to be rendered and where they are to be rendered (MF or UE). The message shall be a dictionary object. with keys “MF” and “UE”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The keys shall indicate where the objects named in the corresponding value list are rendered. |

## A.1.2	Seamless Adaptive Split

An SR-DCMTSI client that supports the adaptive split rendering with seamless adaptation shall support the seamless adaptive split message format defined below.

Table A.1.2-1 Message format for seamless adaptive split messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| Id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp: split-rendering: v1: asrp:sr-split-seamless |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | An object identifying objects to be rendered and where they are to be rendered (MF or UE). The message shall be a dictionary object. with keys “MF” and “UE”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The key ‘UE is used for objects that are to be rendered by the UE and key ‘MF is used for objects that are to be rendered by the MF, when seamlessSplit conditions are not met. |
| seamlessSplit | object | 0..1 | An object that if present indicates a seamless adaptation of the rendering process when possible. |
| radius | number | 1..1 | A distance in meters that defines a sphere centered at the UE, such that preferential rendering is used for objects that lie within this sphere or intersect with it. An object lies within the preferential rendering sphere if for all points defining the collider associated with the object the distance from the UE is less than the radius. An object intersects with the preferential rendering sphere if the distance between the UE and some points on the collider associated with the object is less than the radius and for some other points on the collider associated with the object, the distance from the UE is greater than or equal to the radius. |
| type | string | 1..1 | A string that indicates the type of preferential rendering to be used for the objects defined by the key ‘UE’ in renderingSplit. The following values are supported: “Local”: The objects are rendered at the UE when they are within the sphere define by R and rendered by the MF when they are outside of it. “LOD”: The MF renders the objects when they are within the radius R at a high fidelity. For example, 3D models with a higher Level-of-detail may be rendered. |

Editor’s Note: We need to check if the conditions are deterministic enough for both UE and MF to function seamlessly.

## A.1.3	Processing Delay Adaptation based on QoE metrics

## A.1.3.1	Configuration format

The MF shall share the configuration information of the delay adaptation procedure with the SR-DCMTSI client in terminal during the split rendering session negotiation and establishment processes. The configuration may be updated during the rendering loop. The configuration information of the delay adaptation procedure shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:configuration”.

Table A.1.3.1-1 – Configuration message format for Processing Delay adaptation based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:configuration |
| message | Object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is considered. This qoeMetrics array may contain all or a subset of the QoE latency metrics negotiated in the metrics configuration message in clause 6.3.1. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. The name of that QoE metric is chosen as unique ID, this name should be consistent with the name provided in the metrics reporting configuration defined in clause 6.3.1. |
| periodicity | string | 1..1 | The periodicity of the delay adaptation information for that QoE metric. It may be expressed as a multiple of the "Measure-Resolution" defined in clause 16.3.2 of TS 26.114 [7]. Whenever a delay adaptation information message is sent, the SR-DCMTSI client in terminal shall reset its timer to the value of the periodicity property and it shall begin countdown of the timer again. |

## NOTE: 	The target delay range for a QoE metric is delimited by a minimum threshold and a maximum threshold delay values. The thresholds and the target delay, for each QoE metric in the qoeMetrics array, can be provided by the DC Application Server to the SR-DCMTSI client in terminal.A.1.3.2	Metadata format

During a IMS-based split rendering session, the operating environment of the serving MF, the resources of SR-DCMTSI client or the network conditions may change. Consequently, the roundtrip delay may need to be adjusted to deliver a consistent QoE.

When delay adaptation procedure is enabled, the SR-DCMTSI client in terminal checks for the QoE metrics being monitored whether the measured delays are within the target delay range or not. When a measured delay is outside the target delay range for a QoE metric, the SR-DCMTSI client in terminal may report the measured delay based on the configured periodicity.

The delay adaptation information message format that is used for IMS-based split rendering shall comply with the format defined in Table A.1.3.2-1. The delay adaptation information message shall be carried as part of the data channel messaging mechanism. The metadata data channel message shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:information”.

Table A.1.3.2-1 Metadata format for Processing Delay Adaptation information message based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the IMS-based split rendering session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:information |
| message | object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is needed. This qoeMetrics array may contain all or a subset of the QoE metrics negotiated in the configuration message in clause A.1.X.1. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. |
| delayValue | number | 1..1 | The measured delay value of that QoE metric. |

Annex B(informative):

Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04-2024 | 127b-e | S4-240645 |  |  |  | [SR_IMS]TS 26.567 Skeleton | 0.0.1 |
| 04-2024 | 127b-e | S4-240790 |  |  |  | [SR_IMS] version agreed during SA4#127bis-e (including S4-240583 and S4-240651) | 0.1.0 |
| 05-2024 | 128 | S4-241272 |  |  |  | [SR_IMS] version agreed during SA4#128 (including S4-241160, 241161, 241274 and S4-241213) | 0.2.0 |
| 08-2024 | 129-e | S4-241739 |  |  |  | [SR_IMS] version agreed during SA4#129 (including S4-241550, 241731, 241735 and S4-241728) | 0.3.0 |
| 11-2024 | 130 | S4-241953 |  |  |  | [SR_IMS] version included agreement during Post SA4#129 RTC SWG Telco (S4aR240081, S4aR240063) | 0.3.1 |
| 11-2024 | 130 | S4-242172 |  |  |  | [SR_IMS] version agreed during SA4#130 (including S4-241947, 241962, 241963, 241972, 242041, 242043 and S4-242092) | 0.4.0 |
| 02-2025 | 131 | S4-250089 |  |  |  | [SR_IMS] version included agreement during Post SA4#130 RTC SWG Telco (S4aR250009 with editor’s note in meeting minutes, S4aR250056, S4aR250057, S4aR250061, S4aR250062, S4aR250069) | 0.4.1 |