Agenda item: 	10.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] On the ending time of the Time To the Next Data Burst (TTNB)

Document for	Discussion and Agreement

# Introduction

The definition of TTNB is not just for the sake of definition, but also needs to facilitate what the definition is to be used in the 5G system. In this regard, there has been discussion on the starting time of the time to the next data burst (TTNB) in clause 6.25 of TR26.822 [1] and in [2]. The ending time of TTNB has been assumed to be clear, e.g., in 6.6.3 of [1], the ending time refers to the time of transmission of the first packet of the next burst.

TTNB is the time interval between the transmission of the last packet in the current burst and first packet of the next burst, i.e., inter-burst time.

However, with this interpretation of the ending time, the resulting TTNB may be highly random due the random delay introduced by video encoding and rate adaptation as explained in clause 6.24.2.2 of TR26.822[1], and we will show the randomness from experimental data. Thus, it may be impossible for the traffic source to signal an accurate TTNB value, although an accurate value is desirable for the 5G system.

# TTNB variability

### 2.1 Conversational video

Experimental results for TTNB, termed Inter Burst Time (ibt) in clause 16 of [1], are copied below, where the shaded columns show the mean and standard deviation of ibt:

Table 6.13.2.5-3:	Statistical results of experiments in different scenarios in scenario #13

| Scenario (direction) | ibt (mean) | Ibt (stdev) | Bd (mean) | Bs (mean) | Ppb (mean) | %lp (%) | %bp (%) | %lb (%) | %bb (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w_h (a) | 0.0352 | 0.01437 | 0.0002 | 3463 | 3.55 | 23.2 | 76.7 | 6.6 | 93.4 |
| w_h (b) | 0.01592 | 0.019 | 0.00016 | 3756 | 3.3 | 19.2 | 80.8 | 17.4 | 82.6 |
| u_h (a) | 0.03132 | 0.00873 | 0.00017 | 13895 | 12,3 | 40.2 | 59.8 | 22.6 | 77.4 |
| u_h (b) | 0.42321 | 0.37918 | 0.00007 | 213.92 | 2 | 72.2 | 27.8 | 73.8 | 26.2 |
| g_v_a | 0.01642 | 0.01175 | 0.00052 | 2864 | 7.73 | 2.4 | 97.6 | 0.7 | 99.3 |
| g_v_b | 0.01796 | 0.00967 | 0.00029 | 3109 | 8.6 | 0.2 | 99.8 | 0.2 | 99.8 |
| g_va_a | 0.01252 | 0.0114 | 0.00071 | 4144 | 6.6 | 14.2 | 85.8 | 20.1 | 79.9 |
| g_va_b | 0.01414 | 0.00955 | 0.00033 | 4472 | 7.5 | 14.8 | 85.2 | 24.7 | 75.3 |
| w_s (a) | 0.08732 | 0.03483 | 0.00079 | 24822 | 42.3 | 0.2 | 99.8 | 0.0 | 100.0 |
| w_r (a) | 0.03778 | 0.04627 | 0.00054 | 10671 | 18.24 | 0.7 | 99.3 | 0.1 | 99.9 |
| m_rv | 0.02749 | 0.0404 | 0.00084 | 6630 | 6.38 | 2.5 | 97.5 | 0.5 | 99.5 |
| m_sav (a) | 0.04777 | 0.03838 | 0.00056 | 11906 | 12.18 | 12.6 | 87.4 | 3.3 | 96.7 |
| m_rav (a) | 0.02573 | 0.03412 | 0.00086 | 6692 | 6.8 | 12.2 | 87.8 | 3.2 | 96.5 |

The standard deviation is comparable to the mean for most cases, and for some scenarios, e.g., w_h (b) and m_rv, the standard deviation is greater than the mean. This shows a quite spread-out distribution of TTNB and TTNB is highly random. This raises the question how the sender could get an accurate TTNB value.

To be specific, consider the first scenario, w_h (a), the mean is 35.2ms, and the standard deviation is 14.37ms. For simplicity, assuming the distribution is Gaussian, this means, 31.73% of the TTNB values are either less than 20.82ms or greater than 49.57ms.

Observation 1: For conversational video, TTNB may be highly random, and it is probably infeasible to predict TTNB accurately without intentional buffering.

### 2.2 XR split rendering

The experimental setup for the XR split rendering is described in Clause 6.24.2.3 of [1]. The video is in two streams, one for the left eye and the other for the right eye. The packets of each video frame are grouped as a separate data burst. The TTNB is plotted in Figure 1. A portion of the curve is zoomed in Figure 2.

Figure 1 The TTNB as a function of the data burst index.

Again, it is observed that TTNB is quite random, and Observation 1 based on conversational video still applies.

However, there is some predictable pattern. This is evident in Figure 2:

- For odd bursts, TTNB is above the mean, and greater 6.3ms,

- For even bursts, TTNB is below the mean, and greater than 4.0ms.

This pattern is easy to understand, because with a fixed frame rate, if video frame n experiences extra delay in video encoding, it will increase the TTNB between video frame n-1 and video frame n, and decrease the TTNB between video frame n and video frame n+1.

Figure 2 Zoom in of the TTNB as a function of the data burst index in Figure 1

For the base station, when it receives the ending packet of a data burst, it can let the UE go to sleep, but it needs to determine when the UE needs to wake up as not to miss the next data burst. The primary motivation of defining TTNB is to assist the base station in making this decision.

Observation 2: The purpose of indicating TTNB is to assist the base station in determining the time for the UE to wake up when the UE goes to sleep.

Sending an inaccurate TTNB to the base station may cause a UE to wake up too late. An alternative is to send the predicted TTNB plus the standard deviation and let the base station to decide what to do. While more information is available, the base station still cannot fully determine when the UE needs to wake up. Moreover, this increases the complexity to the base station.

Therefore, we propose that the traffic source sends an amount of time before the transmission of the first packet of the next data burst, and we call this idle period. That is, the traffic source will not send any packet during the idle period, and may send packets any moment after the idle period.

The idle period can be considered as a lower bound of TTNB, where the ending time of the TTNB is the time of transmission of the first packet of the next data burst.

Alternatively, if we redefine the TTNB such that the ending time is a time at which the first packet of the next data burst has not been transmitted yet, then the idle period is the same redefined TTNB with the new ending point.

Proposal 1: Given the highly random nature of TTNB, to make the indication of the timing information of the data bursts useful to the 5G system, the traffic source indicates the idle period instead of TTNB, where the idle period is defined as an amount of time from the end of the current data burst to a time before the transmission of the first packet of the next data burst.

NOTE 1: the idle period is a lower bound of the TTNB with the current definition of the ending point of the TTNB.

NOTE 2: the application at the traffic source determines the idle period based on its implementation.

Proposal 2: SA4 sends an LS to SA2 and RAN2 on Proposal 1.

# Proposals

Based on the analysis of the traffic patterns for both conversational video and XR split rendering, we propose the following.

Proposal 1: Given the highly random nature of TTNB, to make the indication of the timing information of the data bursts useful to the 5G system, the traffic source indicates the idle period instead of TTNB, where the idle period is defined as a time interval during which the traffic source is guaranteed not to send packets.

NOTE 1: the idle period is a lower bound of the TTNB with the current definition of the ending point of the TTNB.

NOTE 2: the application at the traffic source determines the idle period based on its implementation.

Proposal 2: SA4 sends an LS to SA2 and RAN2 on Proposal 1.

# References

[1] 	TR26.822, Study on 5G Real-time Transport Protocol Configurations, V1.2.0, Rel-19, Nov 2024.

[2]		S4-250065, On the start time of the Time To the Next Data Burst, 3GPP SA4 #131, February 17-21, 2025.