| 3GPP TR 26.847 V0.4.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Evaluation of Artificial Intelligence and Machine Learning in 5G media services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	AI/ML Evaluation Testbed Architectures and Anchors	7

4.1	General	7

4.2	Split inference intermediate data testbed architecture	7

4.3	Model data testbed architecture	7

4.4	Metrics	7

5	Evaluation Scenarios	7

5.1	General	7

5.2	Scenario 1:	7

5.3	Scenario 2:	7

6	Conclusions and Proposed Next Steps	7

Annex A: Scenario Template	8

A.1	Introduction	8

A.2	Template	8

Annex B (informative): Change history	9

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document provides details on feasibility evaluations for a given set of scenarios related to AI/ML in media services. In particular, aspects related the feasibility of split inferencing between different entities and the characteristics of the subsequent intermediate data are documented. A set of anchors and related metrics are defined to evaluate the feasibility and characteristics of such scenarios in terms of service requirements (such as service latency, inference accuracy, network bandwidth etc.). The transmission of relevant data in AI/ML media services is also considered, with evaluations on the compression technologies of data components such as AI/ML models. The functional aspects related to the AI/ML media services given in this document are presented in TR 26.927 [bb].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[aa]	3GPP TR 22.874: “Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS”.

[bb]	3GPP TR 26.927: “Study on Artificial Intelligence and Machine Learning in 5G media services”.

[cc]	A. Baevski, H. Zhou, A. Mohamed and M. Auli, “wav2vec 2.0: A Framework for Self-Supervised Learning of Speech Representations”, arXiv, 2006.11477, 2020

[dd]	W.-N. Hsu, B. Bolte, Y.-H. Tsai, K. Lakhotia, R. Salakhutdinov, and A. Mohamed. “Hubert: self-supervised speech representation learning by masked prediction of hidden units”, arXiv, 2106.07447, 2021

[ee]	TorchAudio: An audio library for Pytorch [Computer software], https://github.com/pytorch/audio, V2.0.1

[ff]	TorchAudio: WAV2VEC2_ASR_BASE_960H, [Computer software], https://pytorch.org/audio/stable/generated/torchaudio.pipelines.WAV2VEC2_ASR_BASE_960H.html

[fa]	TorchAudio: HUBERT_ASR_LARGE, [Computer software], https://pytorch.org/audio/stable/generated/torchaudio.pipelines.HUBERT_ASR_LARGE.html

[gg]	V. Panayotov, G. Chen, D. Povey and S. Khudanpur, "Librispeech: An ASR corpus based on public domain audio books," 2015 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), Australia, 2015

[hh]	OpenSLR, LibriSpeech ASR corpus [Online], https://www.openslr.org/12

[ii]	J. Kahn, M. Rivière, W. Zheng, E. Kharitonov, Q. Xu, P. E. Mazaré, J. Karadayi, V. Liptchinsky, R. Collobert, C. Fuegen, T. Likhomanenko, G. Synnaeve, A. Joulin, A. Mohamed, and E. Dupoux. “Libri-light: a benchmark for ASR with limited or no supervision”. IEEE Int. Conf. on Acoustics, Speech and Sig. Proc. (ICASSP), 7669–7673. 2020. https://github.com/facebookresearch/libri-light.

[jj]	Pytorch: Speech Recognition on Android with Wav2Vec2 Low complexity implementation [Computer Software], https://github.com/pytorch/android-demo-app/tree/master/SpeechRecognition

[kk]	S. Gondi, "Wav2vec2.0 on the edge: Performance evaluation", arXiv, 2202.05993, 2022

[x1]	Ssd_resnet model: https://github.com/nikheelpandey/SSD_RESNET

[x2]	Retinanet model : : https://github.com/onnx/models?tab=readme-ov-file#object_detection

[x3]	5G-MAG repository for AI4Media evaluations: https://github.com/5G-MAG/rt-ai-ml-evaluation-framework

[x4]	[x4]	VGG16 model: https://pytorch.org/vision/main/models/generated/torchvision.models.vgg16.html

[x5]	ONNX/ https://onnx.ai/onnx/api/utils.html

[x6]	SFU object repository https://dash-large-files.akamaized.net/WAVE/3GPP/AIML/ReferenceDataSets/sfuhwobjects.tar.gz

[x7]	5G-MAG scripts for ssd_resenet: https://github.com/5G-MAG/rt-ai-ml-evaluation-framework/blob/development/scripts/objectdetection/ssd300/

[x8]	5G-MAG split multibranch repository: https://github.com/5G-MAG/rt-ai-ml-evaluation-framework in a branch called split_multibranch.

[x9]	NNC codec: https://github.com/fraunhoferhhi/nncodec

[x10]	Netron: Visualization tool for neural network, deep learning and machine learning models. https://netron.app/

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

AI/ML model split point: the points in a DNN AI/ML model where it is split into multiple AI/ML model subsets.

intermediate data: output from the inference process of an AI/ML model that is not considered the final inference result.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MSH	Media Session Handler

RTC	Real-Time Media Communication

SDP		Session Description Protocol

# 4	AI/ML Evaluation Testbed Architectures and Anchors

## 4.1	General

[Editor’s note: Introduction the common set of architectures for the evaluation framework, including anchors, metrics and test methodology expected for each test, according to the scenarios].

A common set of architectures is assumed for the evaluation framework, unless specified otherwise, irrespective of the scenario.

The anchor architectures are as follows:

- Running inference completely on the device

- Receiving a compressed video (e.g. from the device), and running inference completely at the network and potentially sharing the inference results with the device.

These anchor architectures are depicted by the figure 4.1-1:

Figure 4.1-1: Anchor architecture

In figure 5-1, the left hand side represents the anchor for running the inference at the device side. The right hand side shows the architecture for the anchor where the inference is run on the network side. The anchor model for running on the device should be derived from the anchor model running on the network.

The derivation process may include:

- Quantization to match the device’s inference engine, e.g. converting the weights and inputs to fixed point or unsigned integers.

- Re-training of the converted model to optimize for the inference platform. This is allowed but should be accompanied by results without re-training.

- Conversion to an exchange format such as ONNX

The supported model libraries are PyTorch and Keras/Tensorflow2.

## 4.2	Split inference intermediate data testbed architecture

[Editor’s note: Present the common testbed architecture for split inference related scenarios].

A testbed architecture for the evaluation of split inference intermediate data is represented in figure 4.2-1. The anchor model is split into two, split model part 1 and 2, each existing and inferenced at two different nodes respectively (for example a local and the remote compute node), according to scenarios defined. The local to remote direction simulates an uplink communication while the remote to local direction simulates a downlink communication. The sending of data via the network encompasses both unlink and downlink communication, depending on the scenarios defined. Likewise, the sender of the intermediate data may be the local inference node or the remote inference node.

Figure 4.2-1 Split inference intermediate data testbed architecture

The testbed architecture includes the following main functional blocks:

- Anchor model: A pre-trained model with a documented architecture and pre-trained weights, to be used as the anchor model for the test. Optionally, the use of untrained anchor models should be provided with anchor training input data sets and training parameters in order to build a trained anchor model.

- AI framework/library: The AI framework/library used for the testbed, for example, TensorFlow, Pytorch, etc.

- Model split configuration: The configuration of split points for the anchor model which are to be evaluated. The decision for split points may take into consideration the configuration factors, constraints and settings as described in clause 2.

  - Local inferencing: Where the anchor model fully runs on the local node.

  - Remote inferencing: Where the anchor model fully runs on the remote node.

  - Split inferencing: Where an anchor model is split into two parts, each run on a local and a remote node respectively.

- Test dataset: Media data to be input into the anchor model. Depending on the use case and scenario, such data may be video data, audio data, or other media data. In a given scenario, such data may originate from either a local or remote node.

- Test dataset pre-processor: A function which processes the test dataset media data such that it is compatible with the input requirements of the anchor model.

- Inference output processor: A function which processes the inference output of the anchor and/or split model (if necessary), for metric computation.

- Test split model: The outputs of the model split configuration model 1 and model 2 running on the same or different inference nodes. An inference node may be a:

  - Local inference node: Typically emulating an end-device such as a UE.

  - Remote inference node: Typically emulating a network node such as edge/cloud/5G CN Application server.

- Test bitstream (intermediate data): The output as a result of the inference of test split model #1, typically to be sent via the Network, and used as the input to test split model #2.

- Test encoder/decoder: Encoder and decoder for the intermediate data to be sent via the Network. This may include serialization, optimization or compression technologies.

- Network configuration: This defines the network simulation configuration. This may include the type of the Wireless/wired network, network protocols, lossless/lossy emulation, network throttling (e.g., for uplink simulation).

- Test network: The network over which output data from certain functions are delivered. In use cases, this is typically the 5GS.

- Metrics Logs/Computation: A function which logs or computes the metrics on corresponding output data from certain functions, relevant for the scenario. Such metrics may include those described in clause 4.4.

- Test metrics: The metrics used for the evaluation of the scenario.

## 4.3	Model data testbed architecture

[Editor’s note: Present the common testbed architecture for model data (e.g. compression) related scenarios].

A testbed architecture for the evaluation of model data compression is represented in figure 4.3-1. The anchor model is compressed by a test encoder, which may include optimization and/or compression technologies. In the case of sender only compression approaches, the test decoder may be optional.

Figure 4.3-1 Model data testbed architecture

The testbed architecture includes the following main functional blocks:

- Anchor model: A pre-trained model with a documented architecture and pre-trained weights, to be used as the anchor model for the test. Optionally, the use of untrained anchor models should be provided with anchor training input data sets and training parameters in order to build a trained anchor model.

- Test configuration: The configuration of the test encoder to be used for the scenario.

- Test encoder: A function which encodes the anchor model according to that detailed in the test configuration. Encoding may include optimization and/or compression technologies.

- Test decoder: A function which decodes the compressed model. This function may be absent for sender only approaches.

- Test dataset: Media data to be input into the anchor model. Depending on the use case and scenario, such data may be video data, audio data, or other media data. In a given scenario, such data may originate from either a local or remote node.

- Test dataset pre-processor: A function which processes the test dataset media data such that it is compatible with the input requirements of the anchor model.

- Inference output processor: A function which processes the inference output of the anchor model (if necessary), for metric computation.

- Test bitstream (compressed model): The compressed test model of the anchor model, typically to be sent via the network.

- Test model: The test model which was encoded and subsequently decoded. The inference performance of this test model is compared with the anchor model to evaluate the impacts of the test encoder and decoder.

- Test network: The network over which output data from certain functions are delivered. For model compression scenarios, the compressed model is sent over the network. In use cases, this network is typically the 5GS.

- Metrics Logs/Computation: A function which logs and computes the metrics on corresponding output data from certain functions, relevant for the scenario. Such metrics may include those described in clause 4.4.

- Test metrics: The metrics used for the evaluation of the scenario.

## 4.4	Metrics

[Editor’s note: Introduce relevant metrics for the related evaluations, according to different scenario AIML work tasks].

In the process of AI/ML, no matter on the training set or on the new sample, there is always some difference between the output result of the model and the real value. Model evaluation is a process of using different evaluation metrics to understand the performance of artificial intelligence/machine learning models and its advantages and disadvantages. It is an indispensable part of the model development phases which can help to discover the appropriate model to express the data and evaluate the performance of the selected model.

Different AI/ML work tasks have different evaluation metrics, and the same machine learning task will also have different evaluation metrics, each metric has different emphasis, e.g., classification, regression, ranking, clustering, recommendation, etc.

Given that most scenarios that we’re dealing with in the scope of this study involve computer vision tasks, for model performance metrics, the evaluation framework should reuse existing metrics that are well-established in the research community. There exists different metrics depending on the type of task performed by the model.

Classification model evaluation is the process of assessing and measuring the performance of a machine learning model that has been used for classification tasks. its goal is to divide different images into different categories, to achieve the minimum classification error.

Confusion matrix is a table used in classification tasks that summarizes the performance of a machine learning model on a set of data for which the true values are known. It consists of rows and columns where each row represents the true class of the samples and each column represents the predicted class. The confusion matrix displays the number of samples that are classified correctly (true positives and true negatives) and incorrectly (false positives and false negatives) by the model.

Table 4.4-1: Confusion matrix

| Confusion Matrix |  | Predicted Value |  |
| --- | --- | --- | --- |
|  |  | Positive | Negative |
| True Value | Positive | True Positives (TP) | False Negatives (FN) |
|  | Negative | False Positives (FP) | True Negatives (TN) |

True Positives (TP): predict an observation belongs to a class and it actually does belong to that class;

True Negatives (TN): predict an observation does not belong to a class and it actually does not belong to that class;

False Positives (FP): predict an observation belongs to a class but it does not belong to that class;

False Negatives (FN): predict an observation does not belong to a class but it does belong to that class.

For object classification tasks, the following metrics are used to evaluate or measure the performance of a classification model:

- Accuracy: Accuracy is the simplest metric for evaluating classification performance. It measures the percentage of correctly classified objects out of the total number of objects in the dataset. While accuracy is easy to understand and compute, it can be misleading if the dataset is imbalanced, or the cost of misclassifying different categories is not equal. Accuracy measures how often the classifier makes the correct predictions, it is defined as the ratio between the number of correct predictions and the number of total predictions.

Accuracy=TP+TNTP+TN+FP+FN

- Precision: Precision measures the proportion of true positives among all the objects that the model classified as positive. It is useful when the cost of false positives is high, and it is essential to avoid misclassifying objects. Since precision measures the proportion of predicted positive results that are actually positive, it is defined as the fraction of examples (true positives) among all of the examples which were predicted to belong in a certain class (positive).
Precision=TPTP+FP

- Recall: Recall measures the proportion of true positives among all the objects that belong to the positive class in the dataset. It is useful when the cost of false negatives is high, and it is essential to detect all objects in the dataset. Since recall measures how much the classifier can predict in an actual positive sample, it is defined as the fraction of examples which were predicted to belong to a class with respect to all of the examples that truly belong in the class.

Recall=TPTP+FN

- F1 Score: The F1 score is the harmonic mean of precision and recall and provides a balanced view of the model's performance. F1-score is a combination of precision and recall, providing a balanced measure of the model's ability to find all true positive cases and its ability to avoid false positives.

F1=2*Precision*RecallPrecision+Recall

For object detection tasks, the metrics are:

- Intersection over Union (IoU): IoU is one of the most commonly used metrics for evaluating object detection algorithms. It measures the overlap between the ground truth bounding box and the predicted bounding box. IoU is computed as the ratio of the intersection of the two boxes to the union of the two boxes. A higher IoU score indicates better object detection accuracy.

- Precision and Recall: Precision measures the fraction of true positives (correctly identified objects) out of all predicted positives (objects identified by the algorithm). Recall measures the fraction of true positives out of all ground truth positives (objects that should have been identified). A high precision score indicates that the algorithm is correctly identifying objects, while a high recall score indicates that the algorithm is not missing any objects.

- Average Precision (AP): AP is a commonly used metric for evaluating object detection algorithms. It measures the precision at different levels of recall and then averages them. AP provides a single number that summarizes the overall performance of the algorithm. A higher AP score indicates better object detection accuracy.

- F1 Score: The F1 score is the harmonic mean of precision and recall. It provides a single number that summarizes the overall performance of the algorithm. A higher F1 score indicates better object detection accuracy.

For object tracking tasks, the metrics are:

- Intersection over Union (IoU): IoU is also commonly used for evaluating object tracking algorithms. In this case, it measures the overlap between the ground truth bounding box and the predicted bounding box for each frame in the sequence. A higher IoU score indicates better object tracking accuracy.

- Precision and Recall: Precision and recall can also be used to evaluate object tracking algorithms. In this case, precision measures the fraction of frames where the algorithm correctly identified the object, while recall measures the fraction of frames where the algorithm correctly tracked the object.

- Mean Average Precision (mAP): mAP is a commonly used metric for evaluating object tracking algorithms. It measures the average precision at different levels of overlap between the ground truth and predicted bounding boxes over the entire sequence. A higher mAP score indicates better object tracking accuracy.

- Tracking Precision (TP) and Tracking Recall (TR): TP measures the fraction of frames where the predicted bounding box overlaps with the ground truth bounding box by a certain threshold, while TR measures the fraction of ground truth bounding boxes that were successfully tracked. A high TP score indicates that the algorithm is accurately tracking the object, while a high TR score indicates that the algorithm is not losing track of the object.

AI regression model evaluation is the process of measuring the accuracy and performance of a regression model developed using artificial intelligence (AI) techniques. Regression analysis is a statistical method used to predict the relationship between dependent and independent variables. Some of the most commonly used evaluation metrics for regression models are listed as following:

- Mean Squared Error (MSE): measures the average squared error between the predicted and actual values. It's represented as the average of the squared differences between the predicted and actual values.

- Root Mean Squared Error (RMSE): the square root of the mean squared error, this metric indicates the deviation of the predicted values from the actual values.

- Mean Absolute Error (MAE): measures the average absolute difference between the predicted and actual values. This metric is robust to outliers.

- R-squared (R2): determines how well the regression line fits the data by measuring the proportion of the variance explained by the model.

For other non-object related tasks, examples model performance metrics may include:

- Ranking Model Metrics (MRR, DCG, NDCG)

- Statistical Model Metrics (Correlation)

- Computer Vision Model Metrics (PSNR, SSIM, IoU)

- NLP Model Metrics (Perplexity, BLEU score)

For split inference and model compression related scenarios, other feasibility/performance metrics that should also be considered are:

- Video quality: depending on the scenario, the input or output video quality should also be documented. For example, a video super resolution scenario has to evaluate the quality of the resulting video. For the tasks, the impact of the quality of the input video on the accuracy should also be evaluated.

- Complexity: complexity of the entire process, including video compression and decompression, model compression and decompression (where relevant), and inference process.

- Bitrate: the total bitrate needed for performing the task. This may be 0 for the device anchor. For the network anchor, this includes the video bitrate for the uplink and the bitrate for sharing the task results back to the device. For split inference related scenarios, this should include the intermediate data bitrate.

- Split model size: model size and comparison ratio of the test split model to be delivered (compared to anchor model)

- Intermediate data size or bitrate: a comparison ratio of the intermediate data to be delivered (compared to the data size or bitrate of the relevant data from the anchors)

- Compressed model size: the compression ratio of the compressed model compared to the original reference model.

- Compressed intermediate data ratio: compression ratio of the compressed intermediate data bitstream compared to the original intermediate data bitstream

- Latency: the latency requirements for each scenario must also be taken into account to evaluate the feasibility of the proposed solutions, in particular for split inference scenarios, such as:

  - Inference latency metrics

    - local inference time

    - Remote inference time

    - Total local and inference time

    - End to end latency

  - Other latency metrics

    - Encoding/decoding time.

    - intermediate data delivery time

- Resources metrics of UE and/or DN:

  - Computing power consumption on node

      - CPU time

      - GPU time

  - Memory usage

  - Energy consumption

# 5	Evaluation Scenarios

## 5.1	General

[Editor’s note: Introduce information provided for each scenario, based on template in Annex A].

## 5.2	Scenario 1: Transmission of compressed AI/ML model data for automatic speech recognition

### 5.2.1	Motivation and use case relevance

AI/ML model data distribution and sharing over 5G system has been identified in TR 22.874 [aa] as one of the three key operations for AI/ML related services. Reason for this is that UEs might need a great variety of AI/ML models to respond to different tasks and environments, while not being able to store all needed AI/ML models due to memory storage constraints, so that a frequent context adaptive down-loading of AI/ML model data is necessary.

To tackle this problem, methods for model compression have been proposed (clause 6 of TR 26.927 [bb]), which provide the benefits that they 1) lower bandwidth requirements or latencies for model data distribution, and 2) reduce the memory footprint of the AI/ML models on the UEs. However, besides the reduction of the model size, compression can also lead to a decrease of the AI/ML model performance. Which performance-compression trade-offs can be reached by different AI/ML model compression methods is thus an important question when defining AI/ML related services and is thus investigated in this scenario.

From the media-based AI/ML use cases defined in clause 4 of TR 26.927 [bb], the following require the transmission of AI/ML model data and thus could benefit from model compression:

- Full or partial transfer of models for object recognition in image and video

- Transfer of models for post-filtering for video coding

- Transfer of models for crowd-sourcing media capture

- Transfer of models for NLP on speech

This scenario evaluates the transmission of the wav2vec 2.0 [cc] and the HuBERT [dd] AI/ML models for automatic speech recognition (ASR), which derive a transcript of a given speech sequence. The transmission of compressed AI/ML models for ASR is relevant in the following use cases defined in clause 4 of TR 26.927 [bb]:

- Crowd-Sourcing Media Capture: To adapt to background noise or for lyrics recognition, specialized AI/ML models for ASR need to be transferred to a huge number of UEs for device inference.

- NLP on Speech: An initial ASR model needs to be down-loaded to the UE; then updated model data needs to be shared frequently with other UEs for distributed/federated learning.

### 5.2.2	Description of scenario

In this scenario, a pre-trained AI/ML model for ASR, wav2vec 2.0 [cc] or HuBERT [dd], is transmitted to an UE as shown in figure 5.2.2-1. To reduce bandwidth requirements or latencies the model is compressed before transmission. The compression method might be implemented as sender-only compression/optimization technique or might comprise an encoder at the sender-side and a decoder at the receiver-side.

Figure 5.2.2-1: Transmission of the ASR model

How the ASR model can be employed by an UE to derive a transcript of a speech sequence is shown in figure 5.2.2-2, which comprises the following entities:

Figure 5.2.2-2: Prediction of a transcript with the reconstructed ASR model

### 5.2.3	Supporting companies and 3GPP members

- Fraunhofer HHI

- Nokia Corporation

### 5.2.4	Anchor AI/ML DNN model(s) for the scenario

Several pre-trained AI/ML models for ASR are provided by the TorchAudio library [ee] under MIT License. For evaluation, the models listed in table 5.2.4-1 are used.

Table 5.2.4-1: Number of parameters (numParam), size (sizeAnc) and word error rates (werAnc) of the anchor models

| TorchAudio name | numParam [M] | sizeAnc [Mbit] | werAnc[%] |
| --- | --- | --- | --- |
| WAV2VEC2_ASR_BASE_960H | 94.4 | 3021 | 3.4 |
| HUBERT_ASR_LARGE | 315.5 | 10095 | 2.1 |

The WAV2VEC2_ASR_BASE_960H [ff] model consist of several convolutional layers for feature extraction and a transformer. It is pre-trained 960 hours of audio data from the Librispeech data set [gg][hh] and has been fine-tuned on 960 hours of audio data from the same set.

The HUBERT_ASR_LARGE [fa] is a modified version of the wav2vec 2.0 model. It is pre-trained on 60.000 hours of unlabeled audio data from the Libri-Light [ii] dataset and has been fine-tuned on 960 hours of audio data from the Librispeech data set [gg][hh]. It achieves a lower word error rate, but has more parameters.

### 5.2.5	Testbed architecture and anchors

The testbed architecture corresponds to the example testbed architecture defined in clause 4.3 and shown in figure 4.3-1. The following applies for the shown functional blocks:

- The test encoder can also be a sender-only optimization/compression technique.

- The test decoder might be absent for sender-only optimization techniques.

- The reference data set is the test-clean dataset as shown in table 5.2.8-1.

- The anchor model is one of the models shown in table 5.2.4-1.

- The inference output processor corresponds to the pipeline shown in figure 4.3-1.

- Metrics computation derives the word error rate (wer) and the model size (size) as defined in clause 4.4.

### 5.2.6	Test configuration factors, constraints and settings

For encoding, data-dependent optimization techniques might be used. The Librispeech dev-clean dataset, as shown in table 5.2.8-1, might be used for optimization.

### 5.2.7	Feasibility/performance evaluation metrics and requirements

The anchor model and test bitstream are provided as files containing the model parameters. The file size (size) combined with the word error rate (wer) achieved by the reconstructed ASR model after inference are employed to determine the efficiency of a compression method.

File Size

The anchor model and test bitstream can be stored as follows:

- The anchor model is provided as data file containing numParam uncompressed model parameters individually represented as N-byte floating-point values.

- For encoder-only compression methods, the test bitstream is provided as data file containing numParam quantized and/or reduced model parameters individually represented as N-byte values.

- For methods requiring a decoder, the test bitstream is a coded representation encoding the parameters jointly.

For all cases, size can be determined by measuring the file size. For cases a) and b), the size in bit can also be determined as numParam * 8 * N.

Word Error Rate

To quantify the performance of the anchor and the reconstructed model, the word error rate (wer) is used, which has also been applied in the original publication of the wav2vec 2.0 model [cc]. The word error rate is determined on a set of data pairs. Each pair comprises:

- a speech sequence stored as uncompressed audio file, and

- a reference transcript of the audio sequence stored as text file.

Using the dataset, the wer value is determined in two steps:

- A word error rate weri is derived for each pair i of the dataset as follows:

  - The AI/ML model is applied as shown in figure 5.2.2-2 using the speech sequence as input and obtaining a predicted transcript as output.

  - The predicted and reference transcripts are split into a predicted and a reference list of words, respectively.

  - The word error rate 〖wer〗_i of the predicted word list with respect to the reference word list is derived as follows
weri=Si+Di+IiNi

with Si, Di, and Ii denoting the number of word substitutions, word deletions, and word insertions in the predicted word list and Ni denoting the number of words in the reference list.

- The total word error rate wer is derived as follows:

wer= iweriNiiNi

### 5.2.8	Test dataset(s) and scripts for the scenario

Evaluations use the Librispeech [gg][hh] datasets, which are available under Creative Commons Attribution 4.0 International License and shown in table 5.2.8-1. To quantify the performance of the anchor and the test model, the word error rate (wer) is determined based on the test-clean dataset. For data-dependent encoder optimizations, the dev-clean dataset might be used. The datasets can be automatically down-loaded, e.g. by using the exemplary python-script shown in figure 5.2.8-1.

Table 5.2.8-1: Datasets considered in the scenario

| Name | Number of sequences | Hours of audio |
| --- | --- | --- |
| test-clean | 2620 | 5.4 |
| dev-clean | 2864 | 5.4 |

The exemplary script derives word error rate and file size of the anchor models. Further scripts to create and evaluate the reconstructed models can be obtained from https://github.com/5G-MAG/rt-ai-ml-evaluation-framework. They can be generically extended by different compression methods.

import torch            # Version 2.0.0 required
import torchaudio       # Version 2.0.1 required
import torchaudio.datasets as datasets
from torchaudio.functional import resample
from torcheval.metrics import WordErrorRate


test_dir = "D:\\data" # This directory should exist, datasets will be stored here.
device   = "cpu" # or "cuda"

def eval_test_case( test_case, bundle ):
    print('Evaluating test case {test_case}'.format( test_case=test_case) )

    ####### Get Model ##############################
    model       = bundle.get_model()
    sample_rate = bundle.sample_rate
    labels      = bundle.get_labels()

    ####### Get Data Loader Model ##################
    val_loader = torch.utils.data.DataLoader(
                datasets.LIBRISPEECH(test_dir, "test-clean", "LibriSpeech", True ),
                batch_size=1, shuffle=False, num_workers=1, pin_memory=True)

    ####### Evaluate Model #########################
    model.eval()
    model.to( device )
    metric = WordErrorRate()
    blank  = 0

    with torch.inference_mode():
        for speech_sequence, cur_sample_rate, reference_transcript, *dump in val_loader:

            # Resample speech sequence if necessary
            if cur_sample_rate != sample_rate:
                speech_sequence = resample(speech_sequence, cur_sample_rate, sample_rate)

            speech_sequence = speech_sequence.reshape( (1,-1) )
            speech_sequence = speech_sequence.to(device)

            # Apply the ASR model
            vetor_sequence, _ = model(speech_sequence)

            # Select labels
            idcs = torch.argmax(vetor_sequence[0], dim=-1)
            idcs = torch.unique_consecutive(idcs, dim=-1)
            idcs = [i for i in idcs if i != blank]
            predicted_transcript = "".join([labels[i] for i in idcs])
            predicted_transcript = predicted_transcript.replace("|"," ")

            # Update error
            metric.update( predicted_transcript, reference_transcript[0] )

    wer_Anc = metric.compute()
    print('   wer_Anc: {wer_Anc:.3f} %'.format(wer_Anc=wer_Anc*100))

    ####### Get Model Size #########################
    num_parameters = 0
    for param in model.parameters():
        num_parameters += param.numel()

    # Each parameter is stored as 32 bit float, so multiply by four
    size_Anc = num_parameters * 4 * 8
    print('   size_Anc: {size_Anc:.3f} Mbit'.format(size_Anc=size_Anc/1000/1000))

if __name__ == '__main__':
    eval_test_case( 1, torchaudio.pipelines.WAV2VEC2_ASR_BASE_960H )
    eval_test_case( 2, torchaudio.pipelines.HUBERT_ASR_LARGE       )

import torch            # Version 2.0.0 required
import torchaudio       # Version 2.0.1 required
import torchaudio.datasets as datasets
from torchaudio.functional import resample
from torcheval.metrics import WordErrorRate


test_dir = "D:\\data" # This directory should exist, datasets will be stored here.
device   = "cpu" # or "cuda"

def eval_test_case( test_case, bundle ):
    print('Evaluating test case {test_case}'.format( test_case=test_case) )

    ####### Get Model ##############################
    model       = bundle.get_model()
    sample_rate = bundle.sample_rate
    labels      = bundle.get_labels()

    ####### Get Data Loader Model ##################
    val_loader = torch.utils.data.DataLoader(
                datasets.LIBRISPEECH(test_dir, "test-clean", "LibriSpeech", True ),
                batch_size=1, shuffle=False, num_workers=1, pin_memory=True)

    ####### Evaluate Model #########################
    model.eval()
    model.to( device )
    metric = WordErrorRate()
    blank  = 0

    with torch.inference_mode():
        for speech_sequence, cur_sample_rate, reference_transcript, *dump in val_loader:

            # Resample speech sequence if necessary
            if cur_sample_rate != sample_rate:
                speech_sequence = resample(speech_sequence, cur_sample_rate, sample_rate)

            speech_sequence = speech_sequence.reshape( (1,-1) )
            speech_sequence = speech_sequence.to(device)

            # Apply the ASR model
            vetor_sequence, _ = model(speech_sequence)

            # Select labels
            idcs = torch.argmax(vetor_sequence[0], dim=-1)
            idcs = torch.unique_consecutive(idcs, dim=-1)
            idcs = [i for i in idcs if i != blank]
            predicted_transcript = "".join([labels[i] for i in idcs])
            predicted_transcript = predicted_transcript.replace("|"," ")

            # Update error
            metric.update( predicted_transcript, reference_transcript[0] )

    wer_Anc = metric.compute()
    print('   wer_Anc: {wer_Anc:.3f} %'.format(wer_Anc=wer_Anc*100))

    ####### Get Model Size #########################
    num_parameters = 0
    for param in model.parameters():
        num_parameters += param.numel()

    # Each parameter is stored as 32 bit float, so multiply by four
    size_Anc = num_parameters * 4 * 8
    print('   size_Anc: {size_Anc:.3f} Mbit'.format(size_Anc=size_Anc/1000/1000))

if __name__ == '__main__':
    eval_test_case( 1, torchaudio.pipelines.WAV2VEC2_ASR_BASE_960H )
    eval_test_case( 2, torchaudio.pipelines.HUBERT_ASR_LARGE       )

Figure 5.2.8-1: Exemplary python script for determining sizeAnc and werAnc.

### 5.2.9	Detailed test conditions

A compression method under test is evaluated using the test cases shown in table 5.2.9-1. werAnc and sizeAnc are given in table 5.2.4-1.

Table 5.2.9-1: Test cases and respective wer ranges

| Test case | Model | wer range |
| --- | --- | --- |
| 1 | WAV2VEC2_ASR_BASE_960H | 3.4% to 8.4% |
| 2 | HUBERT_ASR_LARGE | 2.1% to 7.1% |

To characterize a compression method under test in a given test case, it is evaluated using different test configurations T, which might be produced by varying encoder parameters, e.g. quantization parameters or sparsification ratios. More specifically, for each test configuration T from a set of test configurations, a data pair (cSize, wer) is derived with:

- cSize denoting the size of the test bitstream size divided by the size of anchor model sizeAnc and

- wer denoting the word error rate of the test model.

If possible, the set of test configurations should contain at least 5 test configurations T that produce word error rates in the range of werAnc to werAnc+0.05 as shown in table 5.2.4-1.

For comparison, (cSize, wer) pairs, as well as werAnc, might be reported graphically, as shown in figure 5.2.9-1.

Figure 5.2.9-1: Example for the characterization of a compression method for different test configurations T

### 5.2.10	Interoperability considerations for the scenario

Download (possibly via TCP) of the model data is expected.

### 5.2.11	External performance data

None.

### 5.2.12	Expected time plan for the scenario completion

Evaluations are expected to be completed within the time plan of the feasibility study on AI/ML for Media.

### 5.2.13	Additional information

The wav2vec 2.0 model has been successfully employed for ASR on mobile devices: An Android-based implementation can be downloaded from [jj]. An evaluation of the wav2vec 2.0 model on a device with limited computational performance can be found in [kk].

### 5.2.14	Coding results for NNC without data-dependent tools

To quantify possible data rate reductions with NNC [aa], the weight tensors of the WAV2VEC2_ASR_BASE_960H model and the HUBERT_ASR_LARGE model (as described in clause 5.2.4) have been encoded with the software framework described in Annex B.2. This means, NNCodec [xx], which is an open implementation of the NNC standard, has been used.

The models have been encoded without enabling enhanced or data-dependent encoding tools and also without employing encoder-only pre-processing techniques to the model. Consequently, results are representative for a straightforward use-case, which does not require test or training data for additional encoder-side re-training or additional inference steps for encoder-decisions. Table 5.2.14-1 provides details on the enabled NNCodec tools.

Table 5.2.14-1: Enabled NNC tools as described in [xx], other parameters are set to NNCodec’s default values

| NNCodec parameter | Value | Description |
| --- | --- | --- |
| use_dq | True | Dependent scalar quantization |
| codebook_mode | False | Integer codebook for transmission |
| param_opt | True | Parameter optimization for DeepCabac |
| cabac_unary_length_minus1 | 10 | Length of unary binarization part |
| opt_qp | True | QP optimization based on tensor statistics |
| ioq | False | Inference-optimized quantization |
| bnf | False | Batch-norm folding |
| lsa | False | Training-based local scaling adaptation |
| fine_tune | False | Training-based tuning of non-weight tensors |

To achieve different trade-offs between compressed model size and model performance, the models have been encoded with different quantization step sizes. More specifically, NNCodec’s quantization parameter (QPs) has been varied in the range from −15 and −45.

Figure 5.2.14-1 shows the results: The performance of the compressed model is reported as word error rate (wer). The size of the compressed model (cSize) is reported in percent of the original uncompressed model size (sizeAnc). In summary, the results show that NNC reduces the model size to about 10% to 15% with negligible model performance losses in a setup without any data-driven tools, or optimization techniques that modify the models before encoding. Higher reductions can might be possible when enabling more sophisticated encoding tools, as e.g. also data-dependent tools or additional encoder-only model optimization techniques.

Figure 5.2.14-1: Compressed model size and model performance achieved for different QPs. 
For reference, the anchor performance werAnc is shown as red line.

## 5.3	Scenario 2: Split inferencing for object detection and labelling

### 5.2.1	Motivation and use case relevance

Object detection and tracking find prevalent applications in today’s world. These applications range from surveillance, image-based gallery and web search, media annotation, autonomous driving and more.

TR 22.874 [aa] clause 5.2 describes these scenarios where deep learning-based object detection and tracking is performed.

### 5.3.2	Description of the scenario

In this scenario, a pre-trained model is used to detect objects in a video sequence. The output of the inference may consist of the following:

- Detected object labels per image

- Bounding boxes for the detected objects

- Masks describing pixel-accurate location of the object

In this scenario, it is assumed that the end device is resource constrained and may not have sufficient memory/processing capabilities, or battery power to perform the object detection task.

It is proposed that by splitting the model into 2 parts, where one part is inferred in the end device and the other part is inferred in the network, the end device will be able to perform the inference within its capabilities.

Two configurations are possible, based on the exact use cases:

- The image/video is captured on the device and inference is run on the image/video to produce feature maps that are then sent to the network for further inference. This step may be performed to protect user privacy. The device will then receive the results once the inference is finalized on the network. An example of such a use case is image/video-based web search, where the user captures an image/video and receives web search results. Another such use case is where the user captures an image/video and attempts to remove a specific object from the image/video.

- The image/video is provided by a content provider and processed by the network to enable the user to perform different tasks. The video is processed by a deep network to produce distilled features, which are then used by the device to perform task-specific inference. Different users viewing the same image/video may run different tasks. An example of such a use case is a sports game streaming service, where different users may have different interests in the game. One user may configure their application to track and annotate the players of their favourite team. Another user may be interested in extracting statistics about the ball. The core of the network produces a set of features that can be used to perform both tasks, where each user will run the model head specific to their selected task.

### 5.3.3	Supporting companies and 3GPP members

- Qualcomm,

- Interdigital.

### 5.3.4	Anchor AI/ML DNN model(s) for the scenario

The evaluation using the PyTorch framework includes several DNN models belonging to table 5.3.4-1:

- retinanet [x2]

- ssd_resnet: The SSD300 model from Nvidia  [x1]

Table 5.3.4-1: DNN models used for scenario 2

| Model | Size (MB) | No. of parameters |
| --- | --- | --- |
| retinanet | 146 MB | About 34 to 45 millions |
| ssd_resnet SSD300 (ResNet-50) | 89 MB | About 23 millions |

### 5.3.5	Testbed architecture and anchors

The testbed architecture for this scenario is shown in figure 5.3.5-1.

Figure 5.3.5-1: Testbed architecture for scenario 2

The split configurations for the scenario are compared to three anchors:

- Where the anchor model is fully inferenced on the device.

- Where the anchor model is fully inferred on the network.

- Where the anchor model is split between the device and the network for at least the first layers of the model to meet the privacy requirements as described in 10.X.1.

The anchor model used is shown in clause 10.3.4.

Test network latencies are not considered to ensure scenario reproducibility.

Multiple model split configurations are considered as described in clause 10.2.6.

### 5.3.6	Test configuration factors, constraints, and settings

Split configurations can include different computational capabilities (CPU/GPU), encoding/decoding functions (optimization and/or compression/decompression), as well as serialization/deserialization functions.

Figure 5.3.6-1: Testbed configuration

### 5.3.7 Feasibility/performance evaluation metrics and requirements

We evaluate the performances according to the following metrics for each split point configuration: inference latency, output data size, resulting accuracy. The evaluation may include the impact of encoding/decoding functions and/or serialization/deserialization functions on the measured metrics. The delivery latency is estimated from the output data size according to the different bandwidths of the 5G network.

### 5.3.8	Test dataset(s) and scripts for the scenario

The SFU-HW-Objects and the SFU-HW-Tracking datasets are used for this evaluation scenario.

A set of scripts is made available under the 5G-MAG rt-ml-ai-evaluation-framework repository [x3]. Detailed scripts API is provided in Annex B.3

Two models were evaluated with different scripts adapted for each model (ssd_resnet, retinanet).

### 5.3.9	Results for single and multi-branch split on ONNX models

#### 5.3.9.1	Introduction

ONNX provides a function extract_model() enabling the extraction of a sub-model from an ONNX model [x5] as shown in figure 5.3.9.1-1.

Figure 5.3.9.1-1: ONNX extract_model function

This function is already used by scripts in 5G-MAG repository [x3], such as split_onnx.py and split_retinanet.py.

Single branch or multi-branches split script making use of extract-model function are described in clause 5.3.9.2 and 5.3.9.3. Specific issues regarding input tensors and output results applied to multi-branch split are also showed, when Part II needs model input tensor and, in §2.4, when Part I generates partial output result of the full outputs results.

#### 5.3.9.2	Bottleneck/single branch split

For some models, or some parts of a model, a node is connected with only one input node and one output node. For example, it is the case for all nodes of the VGG16 model [x4], and for some nodes of ssd_resnet [x1]. An overview of the beginning of VGG16 opened with Netron [x10] is provided in figure 5.3.9.2-1.

Figure 5.3.9.2-1: VGG16 layers visualisation with Netron

In order to split just before the node 5, “vgg0_conv2_fwd”:

Figure 5.3.9.2-2: Split VGG16 at node 5 “vgg0_conv2_fwd” split

The intermediate data communicated between the two sub models will be the tensor “vgg0_pool0_fwd”.

To get the two sub models, you need to give the tensor name of the input and the tensor name of the output of each part to the extract_model function.

For the first part, the following needs to be provided:

- input: [“data”] (model input)

- output: [“vgg0_pool0_fwd”] (split tensor name)

For the second part, the following needs to be provided:

- input: [“vgg0_pool0_fwd”] (split tensor name)

- output: ['vgg0_dense2_fwd'] (model output)

A single branch script ‘split_onnx.py’ is used to split the model and detailed in clause B.3.3. The multi branch script ‘split_onnx_multi.py’ detailed in clause B.3.4 can be used as well

#### 5.3.9.3	Multi branches split

For some models, or some parts of a model, a node is connected to several nodes for its input, and/or several nodes for its output. Figure 5.3.9.3-1 is an example for the resnet model [x1] split at node 6 with an overview of part I and Part II opened with Netron [x10].

Figure 5.3.9.3-1: Split illustration of resnet model at node 6 with Netron

The intermediate data communicated between the two submodels will be the tensors:[“/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/conv2/Conv_output_0”, “/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0”].

To get the two sub models, you need to give the tensor name of the input and the tensor name of the output of each part to the extract_model function.

For the first part, the following data needs to be provided

- input: [“input”] (model input)

- output: [“/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/conv2/Conv_output_0”, “/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0”]

For the second part, the following  needs to be provided:

- input: [“/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/conv2/Conv_output_0”, “/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0”]

- output: [“output1”,” output2”] (model output)

The multi branch script ‘split_onnx_multi.py’ is used to split the model and detailed in clause B.3.4 can be used as well

#### 5.3.9.4	AI/ML model splitability from ONNX model assessment

Table 5.3.9.4-1 gives a status of the current assessed models regarding the split operations with ONNX [x5]. Process inference was run on both models ssd-resnet and retinanet.

ONNX multi-branch split function APIs and scripts were used on different models as follows:

Table 5.3.9.4-1: Split operations with ONNX model files

| Model name | task | Splitted | Verified with Inference | Multi-branch | Nb Maximum branches |
| --- | --- | --- | --- | --- | --- |
| ssd_resnet | Object detection | yes | yes | Yes | 13 |
| retinanet | Object detection | yes | yes | Yes | 85 |
| yolo-v2 | Object detection | yes | no | Yes | 3 |
| yolo-v4 | Object detection | yes | no | Yes | 7 |
| tinyyolov2 | Object detection | yes | no | no | 1 |
| mobilenetv3 | Image classification | yes | no | Yes | 3 |
| Resnet152 | Image classification | yes | no | Yes | 2 |
| Resnet18 | Image classification | yes | no | Yes | 2 |
| ResNext50FF | Image classification | yes | no | Yes | 2 |
| squeezenet | Image classification | yes | no | Yes | 2 |
| efficientnet | Image classification | yes | no | Yes | 4 |
| caffenet | Image classification | yes | no | no | 1 |
| inception | Image classification | yes | no | Yes | 5 |
| vgg16 | Image classification | yes | no | No | 1 |
| nerf | 3D model rendering | yes | no | Yes | 36 |

Table 5.3.9.4-1 demonstrates that a large set of ONNX files was successfully split.

#### 5.3.9.5	Multi-branch split operation analysis

SSD_RESNET

- Intermediate data contains between 1 and 13 tensors (number of branches)

- Input anchor and input of part I is [“input”]

- Output anchor is [“output1” ,“output2”]

- From nodes 1 to 153

  - Input of Part II is exactly the output of Part I

  - Part II is generating anchor outputs [“output1” ,“output2”]

- Node 154

  - Part I is generating one of the anchor outputs: “output1”.

  - Part I output is ['/Reshape_1_output_0', '/Reshape_3_output_0', '/Reshape_5_output_0', '/Reshape_7_output_0', '/Reshape_9_output_0', '/Reshape_11_output_0', 'output1']

  - Input of Part II is the output of Part I except ‘output1”
['/Reshape_1_output_0', '/Reshape_3_output_0', '/Reshape_5_output_0', '/Reshape_7_output_0', '/Reshape_9_output_0', '/Reshape_11_output_0']

  - Part II is generating anchor outputs [“output2”]

  - A consolidation is needed to rebuild the output anchor [“output1” ,“output2”]

RETINANET

- Intermediate data contains between 1 and 85 tensors (number of branches)

- Input anchor and input of part I is [“input_images”]

- Output anchor is [“2734”, “2712”,”2713”]

- From nodes 1 to 65:

  - Part I is generating intermediate data,

  - Part II needs to have the input “input_images” in addition to output of part I,

  - Part II is generating is [“2734”, “2712”,”2713”].

- From nodes 66 to 2230:

  - Part I is generating only intermediate data, between 7 and 85 tensors

  - Input of Part II is exactly the output of Part I

- Node 2230

  - Part I is generating 8 tensors including anchor outputs “2712”:
['/Concat_27_output_0', '/Slice_50_output_0', '/Gather_output_0', '/Cast_4_output_0', '/Gather_1_output_0', '/Cast_3_output_0', '/Gather_68_output_0', '2712']

  - Part II input is the part I output except the anchor output ‘2712’
['/Concat_27_output_0', '/Slice_50_output_0', '/Gather_output_0', '/Cast_4_output_0', '/Gather_1_output_0', '/Cast_3_output_0', '/Gather_68_output_0']

  - Part II is generating anchor outputs [“2734”, “2713”].

  - A consolidation is needed to rebuild the output anchor [“2734”, “2712”,”2713”]

- Nodes 2231 to 2248

  - Part I is generating between 6 and 8 tensors including anchor outputs [“2712”,”2713”]

  - Part II input is the part I output except the anchor outputs [“2712”,”2713”]

  - Part II is generating anchor outputs [“2734”]

  - A consolidation is needed to rebuild the output anchor [“2734”, “2712”,”2713”]

#### 5.3.9.6	Intermediate data size evaluation

##### 5.3.9.6.1	Evaluation of the extraction of intermediate data size from ONNX model file

In this clause the size of the intermediate data is determined by parsing ONNX model file, without any inferencing. This evaluation is made on models ssd_resnet [x1] and retinanet [x2].

SSD_RESNET

First step:

The ONNX file is parsed to extract the tensors, their types and their shape information and evaluated the tensor size for each tensor.

An example of size calculations for some tensors are shown in table 5.3.9.6.1-1.

Table 5.3.9.6.1-1: Example tensor size calculations

| Tensor name | Tensor shape | Number of Data | Data type | Data size (bytes) | Tensor size (bytes) |
| --- | --- | --- | --- | --- | --- |
| “input” | ['1', '3', '300', '300'] | 270000 | float32 | 4 | 1080000 |
| “Conv_output_0” | ['1', '64', '150', '150'] | 1440000 | float32 | 4 | 5760000 |
| “output1” | ['1', '4', '8732'] | 34928 | float32 | 4 | 139712 |
| “output2” | ['1', '81', '8732'] | 707292 | float32 | 4 | 2829168 |

Second step:

The list of the tensors are that composes the intermediate data at each potential split point are extracted. The intermediate data size for each split points is calculated by adding up the size of each tensor (first step).

Figure 5.3.9.6.1-1 shows the intermediate data size in blue and, the number of tensors (i.e. number of branches) of the split points in green.

Figure 5.3.9.6.1-1: Intermediate data size and number of branches per node

Third step

The intermediate data sizes are cross-checked with the ones obtained by inference.

RETINANET

First step:

The ONNX file is parsed to extract the tensors, their types and their shape information and evaluated the tensor size for each tensor.

An example of the tensors obtained is shown in table 5.3.9.6.1-2.

Table 5.3.9.6.1-2: Example tensors obtained

| Tensor name | Tensor shape | Number of Data | Data type | Data size (bytes) | Tensor size (bytes) |
| --- | --- | --- | --- | --- | --- |
| input_images | ['bs', '3', 'h', 'w'] | bs*3*h*w | float32 | 4 | 4*bs*3*h*w |
| /Split_output_0 | ['unk__0', '3', 'h', 'w'] | Unk_0*3*h*w | float32 | 4 | 4*Unk_0*3*h*w |
| 2712 | ['Gather2712_dim_0'] | Gather2712_dim_0 | float32 | 4 | 4*Gather2712_dim_0 |
| 2713 | ['Gather2712_dim_0'] | Gather2712_dim_0 | int64 | 8 | 8*Gather2712_dim_0 |
| 2734 | ['Concat2734_dim_0', '4'] | Concat2734_dim*4 | float32 | 4 | 4*Concat2734_dim*4 |

However, unlike ssd_resnet, the shape information contains variable parameters:

- Some of these are the size of the input data: height(h) and width(w).

- Others are unknown: “Unk_0”, “Concat2734_dim”, “Gather2712_dim_0”, “Gather2712_dim_0”.

Consequently, it is not possible to carry out the second step to calculate the intermediate size of all the split point configurations.

##### 5.3.9.6.2	Evaluation of the correlation between the size of the input data size and the size of the intermediate data at inference time

In this clause, inference on images of different dimensions is performed at several split points. This evaluation is made on models ssd_resnet [x1] and retinanet [x2].

SSD_RESNET

Figure 5.3.9.6.2-1 shows the evaluation of intermediate data size at inference stage.

Figure 5.3.9.6.2-1: Inference experimentation with ssd_resnet with images having various dimensions

The same values were obtained as in section 5.3.9.6.1 since the image is resized at a fix dimension (300,300) at the pre-processing stage.

RETINANET

The size of intermediate data was calculated for a set of different image dimensions.

Figure 5.3.9.6.2-2 shows the size of the intermediate data obtained after inferencing images with different dimensions at different split points node.

Figure 5.3.9.6.2-3 also shows the size of the intermediate data obtained after inferencing images with different dimensions at different split points node with a bar graph representation.

Figure 5.3.9.6.2-2: Inference experimentation with retinanet with images having various dimensions

Figure 5.3.9.6.2-3: Inference experimentation with retinanet with images having various dimensions (Bar graph)

A variation in the intermediate size was observed with the variation of the input image dimension. However, no direct link between the size of the input image in number of pixels and the size of the intermediate data was observed.

By analyzing the retinanet tensors, the presence of a tensor “'/transform/Resize_output_0'” at node 126 was observed, which explains some clipping effects after this node. For example, intermediate data size is the same for images having dimension (3840*2160, 1920*1080,1280*720) for node 150 (so after 126). Differences for other dimensions were still observed, meaning that other processing is done beside the resizing.

##### 5.3.9.6.3	Conclusion for evaluation of intermediate data size

From these experimentations, it is observed that it is not always possible to obtain the intermediate data size before the inference phase, for two reasons:

- Models can be designed to infer media data with variable size.

- Models can produce intermediate tensor data of varying shape whose relationship to the characteristics of the input data is unknown.

Consequently, for some models, the size of the intermediate tensors can only be calculated at inference stage. In this case, the size of the tensors composing the intermediate data must be transmitted as metadata with the intermediate data.

#### 5.3.9.7	Multi-branch script results

Table 5.3.9.7-1 shows the intermediate data size for some split points of ssd_resnet. Results are independent of the original image/frame size as the model requests an image resized at a fix dimension of 300x300.

Table 5.3.9.7-1: Multi-branch script results for ssd_resnet

| Nodes |  | Intermediate data size (Mbytes) | Nb branches | Estimated network latency (ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Split | Node name |  |  | at 500 Mbps | at 1Gbps | at 5 Gbps |
| Anchor (full network inference) |  | 1.08 (RGB) |  | 17,28 | 8,64 | 1,73 |
| split_node_1 | /feature_extractor/feature_extractor/feature_extractor.2/Relu | 5.76 | 1 | 92,16 | 46,08 | 9,22 |
| split_node_3 | /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/conv1/Conv | 1.44 | 1 | 23,04 | 11,52 | 2,30 |
| split_node_10 | /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/relu_2/Relu | 5.76 | 1 | 92,16 | 46,08 | 9,22 |
| split_node_17 | /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.1/relu_2/Relu | 5.76 | 1 | 92,16 | 46,08 | 9,22 |
| split_node_24 | /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.2/relu_2/Relu | 5.76 | 1 | 92,16 | 46,08 | 9,22 |
| split_node_25 | /feature_extractor/feature_extractor/feature_extractor.5/feature_extractor.5.0/conv1/Conv | 5.76 | 1 | 92,16 | 46,08 | 9,22 |
| split_node_39 | /feature_extractor/feature_extractor/feature_extractor.5/feature_extractor.5.1/relu_2/Relu | 2.96 | 1 | 47,36 | 23,68 | 4,74 |
| split_node_46 | /feature_extractor/feature_extractor/feature_extractor.5/feature_extractor.5.2/relu_2/Relu | 2.96 | 1 | 47,36 | 23,68 | 4,74 |
| split_node_54 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.0/conv1/Conv | 2.96 | 1 | 47,36 | 23,68 | 4,74 |
| split_node_62 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.1/conv1/Conv | 5.91 | 1 | 94,56 | 47,28 | 9,46 |
| split_node_68 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.1/relu_2/Relu | 5.91 | 1 | 94,56 | 47,28 | 9,46 |
| split_node_69 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.2/conv1/Conv | 5.91 | 1 | 94,56 | 47,28 | 9,46 |
| split_node_75 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.2/relu_2/Relu | 5.91 | 1 | 94,56 | 47,28 | 9,46 |
| split_node_76 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.3/conv1/Conv | 5.91 | 1 | 94,56 | 47,28 | 9,46 |
| split_node_82 | /feature_extractor/feature_extractor/feature_extractor.6/feature_extractor.6.3/relu_2/Relu | 5.91 | 1 | 94,56 | 47,28 | 9,46 |

Table 5.3.9.7-2 shows the intermediate data size for some split points of retinanet. For this model, results are dependent of the original image/frame size. Results are for an image of size 640x428

Table 5.3.9.7-2: Multi-branch script results for retinanet – input image dimension 640x428

| Nodes |  | Intermediate data size (Mbytes) | Nb branches | Estimated network latency (ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Split | Node name |  |  | at 500 Mbps | at 1Gbps | at 5 Gbps |
| Anchor (full network inference) |  | 0,82 |  | 13,15 | 6,57 | 1,31 |
| split_node_100 | /transform/Cast_2 | 3,35 | 72 | 53,64 | 26,82 | 5,36 |
| split_node_200 | /transform/Cast_19 | 11,55 | 74 | 184,76 | 92,38 | 18,48 |
| split_node_300 | /backbone/body/layer3/layer3.5/relu_2/Relu | 58,43 | 75 | 934,94 | 467,47 | 93,49 |
| split_node_400 | /head/classification_head/conv/conv.2/conv.2.1/Shape | 63,62 | 80 | 1017,95 | 508,98 | 101,80 |
| split_node_500 | /head/classification_head/Transpose_1 | 94,73 | 73 | 1515,65 | 757,83 | 151,57 |
| split_node_600 | /head/classification_head/conv/conv.2/conv.2.1_3/InstanceNormalization | 98,33 | 65 | 1573,35 | 786,68 | 157,34 |
| split_node_700 | /head/classification_head/Reshape_8 | 98,85 | 53 | 1581,67 | 790,84 | 158,17 |
| split_node_800 | /head/regression_head/conv/conv.2/conv.2.1_1/Constant_1 | 108,82 | 46 | 1741,15 | 870,57 | 174,11 |
| split_node_900 | /head/regression_head/Unsqueeze_10 | 101,71 | 40 | 1627,38 | 813,69 | 162,74 |
| split_node_1000 | /head/regression_head/conv/conv.2/conv.2.1_4/Reshape | 101,81 | 25 | 1628,89 | 814,45 | 162,89 |
| split_node_1100 | /anchor_generator/Cast_7 | 69,31 | 25 | 1109,01 | 554,51 | 110,90 |
| split_node_1200 | /anchor_generator/Reshape_5 | 69,43 | 31 | 1110,96 | 555,48 | 111,10 |
| split_node_1300 | /anchor_generator/Cast_45 | 72,19 | 28 | 1154,98 | 577,49 | 115,50 |
| split_node_1400 | /Div | 72,23 | 18 | 1155,71 | 577,85 | 115,57 |
| split_node_1500 | /Reshape_1 | 22,44 | 27 | 359,05 | 179,53 | 35,91 |
| split_node_1600 | /Slice_21 | 18,08 | 28 | 289,22 | 144,61 | 28,92 |
| split_node_1700 | /Constant_93 | 4,58 | 32 | 73,23 | 36,61 | 7,32 |
| split_node_1800 | /Gather_37 | 1,45 | 26 | 23,28 | 11,64 | 2,33 |
| split_node_1900 | /Max_4 | 1,21 | 26 | 19,31 | 9,65 | 1,93 |
| split_node_2000 | /Constant_195 | 0,33 | 30 | 5,23 | 2,62 | 0,52 |
| split_node_2100 | /Constant_225 | 0,09 | 27 | 1,47 | 0,73 | 0,15 |
| split_node_2200 | /Min_9 | 0,08 | 21 | 1,21 | 0,61 | 0,12 |

Results show that intermediate data size may be relatively large resulting in a relatively large network latency. Intermediate data compression can be considered to significantly reduce intermediate data size, in return for potential additional latency due to compression/decompression time.

#### 5.3.9.8	Example of predictions with ssd_resnet and retinanet

Table 5.3.9.8-1 shows and example of predictions with ssd_resnet and retinanet models applied on the first frame of the video FourPeople_1280x720_60.mp4 from SFU-HW-Objects.

Table 5.3.9.8-1: Scripts predictions for ssd_resnet and retinanet

| Ground Truth | Predictions |  |
| --- | --- | --- |
|  | ssd_resnet | retinanet |
| person 427 226 704 720 person 717 200 988 485 person 933 163 1281 517 person 89 151 130 196 person 131 154 159 191 person 104 201 135 238 person 144 192 169 227 person 196 123 277 261 person 328 216 363 244 person 384 244 422 337 person 518 123 590 236 person 429 166 460 195 person 451 200 465 224 person 476 192 493 214 person 466 158 479 187 person 602 220 618 242 person 600 242 623 265 person 636 242 659 265 person 634 217 653 238 person 678 242 710 319 person 658 253 684 308 potted_plant 1 216 175 566 cup 537 449 589 503 cup 682 443 730 501 cup 1051 446 1098 509 person 189 223 441 721 chair 921 310 1012 480 chair 658 325 733 481 cup 343 401 399 455 | bottle 502 438 36 62 0.51 dining table 32 481 674 214 0.52 dining table 7 464 1237 213 0.55 person 1052 169 219 345 0.67 person 180 202 255 313 0.80 person 741 195 245 288 0.82 person 434 162 275 | person 999 166 280 327 0.83 person 190 221 239 288 0.83 person 749 205 233 285 0.82 person 446 232 232 324 0.82 potted_plant 0 198 249 331 0.74 cup 1053 451 33 51 0.66 cup 684 448 43 50 0.64 cup 723 428 36 62 0.60 cup 541 456 41 48 0.57 chair 931 318 70 149 0.57 laptop 95 148 140 135 0.52 chair 651 336 71 148 0.50 cup 500 442 36 64 0.50 chair 447 290 248 204 0.45 dining_table 0 483 798 223 0.45 cup 347 463 40 73 0.40 |
|  | mAP Score |  |
|  | Ssd_resnet | retinanet |

Predictions and mAP scores are the same for anchor predictions and for split predictions, whatever the split point used when the intermediate data is not compressed with loss. When intermediate data is compressed with loss, predictions and mAP scores may evolve according to the level of loss, as described in the next clause.

#### 5.3.9.9	Intermediate data compression for ssd_resnet and retinanet

Intermediate data compression is performed with the same multi-branch scripts (Annex B.3.4) using encode_algo parameter for numpy quantization or nnc [x9] with different qc parameters.

- Models

  - ssd_resnet: ssd_resnet.onnx generated by https://github.com/5G-MAG/rt-ai-ml-evaluation-framework/blob/development/scripts/objectdetection/ssd300/convert_ssd300_to_onnx.py (develop branch)

  - retinanet : retinanet.onnx available at https://github.com/5G-MAG/rt-ai-ml-evaluation-framework/tree/main/models

- Split points:

  - ssd_resnet: This model has 154 nodes. 
We split this model at index nodes [10, 30,50,70,90,110,130].

  - retinanet: This model has 2248 nodes. 
We split this model at index nodes [100,400,700,1000,1300,1600,1900]

- Compression

  - Compressed data:

    - Intermediate data generated by the first part of the model are compressed using one of the following algorithms.

    - Compressed Intermediate data is then decompressed and provided for inference of the second part of the model.

  - Compressions algorithm:

    - Numpy quantization from float32 bits to float 16 bits
numpy.float16 : Half precision float: sign bit, 5 bits exponent, 10 bits mantissa

    - Neural Network Encoder Decoder by Fraunhofer HHI [x9]
This library includes a key parameter in the encoder function that controls the rate-performance trade-off, called Quantization Parameter (QP) that controls the quantization stepsize and thus the rate-performance trade-off for all weight parameters. A lower qp is related to a lower quantization stepsize, which yields a higher bitrate but also a lower model performance degradation. Conversely, increasing the qp value results in a lower bitrate but also in a higher model performance degradation. https://github.com/fraunhoferhhi/nncodec/wiki/Usage

- Metrics

  - The accuracy of the results is measured by computing the mAP metrics using the function calculate_map() of the script calc_map.py at [x3]

Note: For practical reasons, we create a version calc_map_image_dataset.py calling the function calculate_map() on the whole dataset of images

- Summary of the experiments process:

Evaluation of the effects of the quantization only (32 bits to 16 bits) and the effects of quantization and entropy coding (using various qp of Fraunhofer’s nncodec)

  - First step: test with one input image

Evaluation on one image to design and set up the testbed pipeline. Implementation of a first prototype integrating the compression library to quickly present results and get quick feedback on results. These first results were encouraging regarding the reasonable impact of the compression on the final task accuracy.

Outcome:

    - The studied models were resilient to loss of accuracy with intermediate data degradations.

    - More images should be used for greater statistical robustness on results

  - Second Step:  tests have been extended to a full video.

Evaluation on a full video to average the result on all the frames. The video FourPeople_1280x720_60.mp4 was used and ground-truth available at [x3]. This video contains 600 frames.

Outcome:

    - The processing takes a lot of time (several weeks) with very few differences to the results with one image regarding accuracy and compressed intermediate data size.

    - There was not enough disparity between the frames on the selected video (FourPeople).

  - Third Step: test with a selection of various images.

Evaluation on a large dataset of images based on existing evaluation datasets. The coco dataset 2017 validation images with the 2017 Train/Val annotations was used.

The dataset contains 5000 images with a large diversity on the objects.

The performance of ssd_resnet on the first 200 images of the dataset was checked.

Outcome:

    - For some of the images in the dataset, ssd_resnet yields poor results. As the objective was to measure the impact of lossy compression, it was important to ensure good initial accuracy (e.g. If the model does not detect the objects on the images we start at a mAP of 0 and the impact of compression  will not be measured.)

  - Fourth step: test with a selection of various images on which ssd_resnet and retinanet have good accuracy
The first 50 images of coco dataset were selected, where ssd_resnet performs perfectly. As a result, the mAP score of ssd_resnet on this dataset is 100.

The same dataset was kept for the evaluation with retinanet.

The score of retinanet on this dataset was very high, 94.21, and considered high enough for the experiments

- Experiments with a dataset of 50 selected images

  - Test set conditions:

    - 50 selected images from coco dataset 2017 where ssd_resnet score is perfect

- List of 50 selected images in table 5.3.9.9-1:

Table 5.3.9.9-1: List of 50 selected images for the experiment

| 000000000285.jpg | 000000042889.jpg | 000000052462.jpg | 000000004395.jpg | 000000029187.jpg |
| --- | --- | --- | --- | --- |
| 000000000802.jpg | 000000044652.jpg | 000000052507.jpg | 000000004495.jpg | 000000016598.jpg |
| 000000032081.jpg | 000000046804.jpg | 000000052565.jpg | 000000004765.jpg | 000000017031.jpg |
| 000000032285.jpg | 000000049259.jpg | 000000054164.jpg | 000000007784.jpg | 000000017115.jpg |
| 000000032570.jpg | 000000049269.jpg | 000000055072.jpg | 000000007888.jpg | 000000018519.jpg |
| 000000033005.jpg | 000000050844.jpg | 000000031749.jpg | 000000009448.jpg | 000000018737.jpg |
| 000000038070.jpg | 000000051314.jpg | 000000031050.jpg | 000000010764.jpg | 000000020059.jpg |
| 000000038825.jpg | 000000051326.jpg | 000000030675.jpg | 000000010995.jpg | 000000020107.jpg |
| 000000039480.jpg | 000000051712.jpg | 000000015278.jpg | 000000011122.jpg | 000000022396.jpg |
| 000000039670.jpg | 000000051961.jpg | 000000001490.jpg | 000000015746.jpg | 000000022623.jpg |

  - Compression Algorithms

    - No encoding

    - Numpy quantization float32 bits to float 16 bits

    - Lossy compression with nnc, with QP=-38

    - Lossy compression with nnc, with QP=-26

    - Lossy compression with nnc, with QP=-18

    - Lossy compression with nnc, with QP=-14

    - Lossy compression with nnc, with QP=-10

    - Lossy compression with nnc, with QP=-6

    - Lossy compression with nnc, with QP=-4

    - Lossy compression with nnc, with QP=0

  - Results

    - mAP score in function of split points and compression algorithm:

Figure 5.3.9.9-1: ssd_resnet map score prediction on dataset 50 selected images

Figure 5.3.9.9-2: retiananet map score prediction on dataset 50 selected images

Outcome:

    - We notice that mAP score is affected by the choice of the algorithm and the choice of the selected split point

    - We notice that according to the split point it is interesting to not always select the same algorithm to reach a given mAP score

    - Representation of mAP score according to the size of intermediate data:
The curve is built by plotting for each compression algorithm the mAP score as a function of the compressed intermediate size.

Figure 5.3.9.9-3: ssd_resnet map score prediction on dataset 50 selected images with split at node 10

Figure 5.3.9.9-4: retinanet map score prediction on dataset 50 selected images with split at node 1000

- Representation of mAP score according to the size of intermediate data and split point:
A curve per split point.
For each split point, the curve is built by plotting for each compression algorithm the point (compressed intermediate size, mAP score).

Figure 5.3.9.9-5: ssd_resnet map score prediction on dataset 50 selected images with 7 splits

Figure 5.3.9.9-6: ssd_resnet map score prediction on dataset 50 selected images with 7 splits -zoom X-axis

Figure 5.3.9.9-7: retinanet map score prediction on dataset 50 selected images with 7 splits

Figure 5.3.9.9-8: retinanet map score prediction on dataset 50 selected images with 7 splits - zoom X-axis

- Compression performance 
Representation of mAP score according to ratio compressed intermediate size / uncompressed intermediate size

For each split point, and each compression algorithm compute the ratio :

compression ratio= uncompressed intermediate size - compressed intermediate size uncompressed intermediate size

compression ratio percentage=compression ratio*100

Then plot each point (compressed ratio percentage, mAP score).

Figure 5.3.9.9-9: Compression performance with ssd_resnet

Figure 5.3.9.9-10: Compression performance with retinanet

- Outcome:

- Quantization 32 bits to 16 bits offers a compression ratio at 50% with no impact on the accuracy

- Thanks to nnc it is possible to reach higher compression ratio up to 80% and same accuracy

- Above 80% of compression ratio, compression may impact differently the accuracy according to the model

- Analysis

- The mAP score is affected by both the choice of the algorithm and the choice of the selected split point

- The selection of the compression algorithm needs to be made according to the selected split point to maintain a given mAP score

- These results using on-the-shelf compression tools offer interesting compression rates with limited impact on the accuracy

NOTE: MPEG (MPEG-FCM: Feature Coding for Machines) are designing codecs for intermediate data in the context of split computer vision models.  The expected compression ratio of the uncompressed feature size verses the compressed feature size in near lossless setting ranges from 6000:1 to 40000:1 on instance segmentation, object detection and object tracking. The obtained compression ratio of intermediate data while preserving near lossless accuracy is defined within a tolerance of 1% drop in task accuracy, relative to the performance achieved by the original task model operating directly on the input data.

NOTE: The MPEG-FCM results have not been evaluated in the context of 3GPP.

# 6	Traffic characteristics

## 6.1	General

[Editor’s note: Based on the architectures, identify for the relevant data components for each of the scenarios, the corresponding traffic characteristics (burst size, delay/bandwidth/reliability requirements etc.). Tentative subclauses for Complete/Basic AI/ML model distribution, Split AI/ML operation, Distributed/federated learning]

# 7	KPIs

## 7.1	General

## 7.2	List of KPIs

[Editor’s note: E.g. Latency, data rate, reliability, accuracy…]

# 8	Conclusions and Proposed Next Steps

Annex A: Scenario Template

## A.1	Introduction

This annex provides a proposed template to introduce a scenario for AI/ML evaluation.

## A.2	Template

A scenario should provide the following information (aligned with TR 26.955, Annex A):

- Scenario name <give the scenario a catchy name>

- Motivation for the scenario and its use case relevance:

Why is the scenario relevant for AI/ML multimedia services? Under which of the following use cases does the scenario fall?

- Object Recognition in Image and Video

- Video Quality Enhancement in Streaming

- Crowd-Sourcing Media Capture

- NLP on Speech

- Description of the scenario:

This provides a description of the scenario addressing potentially the relation to the three AI/ML evaluation framework objectives, including AI/ML model split points, AI/ML model checkpoints and updates, and AI/ML model data compression. The description should be more specific than the use case description as provided in TR 26.927. Predominantly the description should allow to develop a baseline solution.

- Supporting companies and 3GPP members:

a.	This documents the 3GPP members that support this scenario in terms of providing the information, test material, test requirements and the characterization for the tests. For each of the identified necessities, a tick box is created in the template.

b.	Preferably several 3GPP members are included in the support.

c.	Cross-verification is preferably done by the supporters of the scenario

- Anchor AI/ML DNN model(s) for the scenario:

Give the name and details of the trained AI/ML DNN model(s) that will serve for building anchors for this scenario, as well as the data set used for its training. Such trained AI/ML models are not only limited to readily available base AI/ML models, but can also include models developed using transfer learning. There may be more than one candidate anchor AI/ML model for the scenario. As an example, details may include:

a.	Base model used (including links to such base model)

b.	Framework language used (e.g. TFLite, Pytorch)

c.	Architecture/model type (e.g. CNN, RNN)

d.	Number of layers

e.	Number of parameters

f.	Model size

g.	Details of data set used for training

- Testbed architecture and anchors

Describe and detail the testbed architecture and anchors to be used for the scenario. The architecture and anchors should be based on the ones as defined in clause 5, with modifications matched to the scenario.

- Test configuration factors, constraints and settings:

Describe the test configuration factors, constraints and settings for the scenario. Depending on the nature of the scenario, examples are shown below.

AI/ML model split configuration factors, constraints and settings:

For scenarios considering the feasibility of AI/ML split points, many factors may contribute to the split point decision for the scenario, including those related to device/network status and conditions, as well those related to the AI/ML model used, such as its architecture and complexity. Possible split point decision factors may include those shown in table A.2-1.

Table A.2-1: Split point decision factors

| Categories | Parameters | Details |
| --- | --- | --- |
| Devices Involved | CPU/GPU | Device processor capabilities |
|  | Battery | Device battery status |
|  | Heat | Device heating / user health considerations |
| Network | Cellular | Network selection, bandwidth, latency |
|  | Mobility | Network handover and mobility |
| Intermediate Data | Size | Data transmission decision, data weights |
|  | Type | Video, Audio/Speech, Text, Binary etc. |
| Model Type | Architecture | CNN, RNN, GAN, LSTM, etc.1 |
| User focus | APP KPI | Latency Requirement , Service criticality |
|  | Data Privacy | Data transmission allowed or not |
|  | Cost of hosting | Deployment cost at cloud/server |
| Data flow | Topologies2 | Media data source, intermediate data in uplink or downlink |

1 Studies and experiments about splitting operations shall focus on CNN. Splitting for GAN/RNN/LSTM is FFS.

2 Topologies comprise the next cases:

- Local source data – local initial inference

- Local source data – remote initial inference

- Remote source data – remote initial inference

The scenario may also describe split point constraints, such as limiting split points to those that do not change the model topology and its parameters, splitting only at the layers of the AI/ML model, etc.

Compression or optimization constraints and settings:

For scenarios considering the compression or optimization of the AI/ML model, and/or the intermediate data (where applicable to split inference scenarios), describe the compression or optimization constraints and settings.

- Feasibility/performance evaluation metrics and requirements:

Depending on the scenario, feasibility and performance metrics may be either related to model performance, or to the test bitstream (the nature of which depends on the use case scenario).

List and describe the relevant feasibility/performance evaluation metrics for the scenario. A list of possible metrics is detailed in clause 4.4.

- Test dataset(s) and scripts for the scenario:

Describe and provide data sets that will be used for the evaluation of this scenario. This should include a description of the license, access procedure, and the dataset annotation format. Same test datasets may be used for similar scenarios falling under the same use case.

Also provide scripts that will be used for performing the evaluation and calculating the metrics.

Further details are provided in clause 4.4.

- Detailed test conditions:

Provide the detailed test conditions, in particular the descriptions of the input and outputs of the task.

- Interoperability considerations for the scenario:

Interoperability considerations for the scenario may include those related to the delivery considerations for the AI model and other corresponding data (such as intermediate data), including delivery methods, protocols and packetization methods.

- AI/ML model delivery formats, methods and pipelines: encapsulation formats for AI model data (if necessary), related to the delivery methods and pipelines which may be considered (e.g. download, streaming). This may be related to model update requirements and constraints.

- AI/ML model optimization methods: methods of model optimization which are not considered under the evaluation methods described under the AI/ML model data compression evaluation defined.

- Intermediate data compression, delivery formats, methods and pipelines.

- Related to a and c above: streaming protocols such as TCP / UDP

- Related to a and c above: packetization methods such as RTP

- External performance data

References to external performance data that can be added, for example other SDOs, public documents and so on.

- Expected time plan for the scenario completion

- Additional information

Annex B: Scripts and Datasets

## B.1	Introduction

[Editor’s note: Describe the general aspects regarding the AI/ML software framework, including use of docker container with scripts and datasets].

## B.2	Scripts for the evaluation of compressed AI/ML model transmission

### B.2.1	General

This clause describes the software framework for the evaluation of compressed AI/ML model transmission. The framework comprises python scripts implementing three main functionalities:

- A codec pipeline for encoding, decoding, and evaluation of AI/ML models with different parameters to obtain their performances and sizes before and after encoding (pipeline/run.py). The pipeline can be extended with new scenarios and compression methods in a modular way.

- An NNC evaluation script to evaluate the compression of the ASR scenario (as defined in clause 5.2) with NNC. The script invokes the codec pipeline multiple times with different QPs and stores results in csv-files (evaluations/evalNncAsr.py).

- A graph plotter creating pdfs from the csv-files obtained by the NNC evaluation script (evaluations/plotGraphs.py).

Furthermore, the framework includes bash-scripts to a) create a docker image comprising the python scripts and required dependencies and to b) run docker containers from the docker image.

### B.2.2	Software repository and installation

The software is available in the following git-repository:

- https://github.com/5G-MAG/rt-ai-ml-evaluation-framework/tree/main/scripts/asr/compression

The scripts can be used with or without a docker. Without docker, the framework can be installed as follows:

- Install a python 3.10 environment.

- Install packages listed in requirements.txt

pip install -r requirements.txt

pip install -r requirements.txt

- Install NNCodec as follows:

git clone --branch v0.3.1 https://github.com/fraunhoferhhi/nncodec.git

cd nncodec

pip install .

git clone --branch v0.3.1 https://github.com/fraunhoferhhi/nncodec.git

cd nncodec

pip install .

To run the scripts with docker, the framework comprises a Dockerfile to build an image with Ubuntu 22.04, python 3.10, packages in the requirements.txt, Nvidia GPU support, and NNCodec.

The image can be built as follows:

cd docker

./buildContainer.sh

cd docker

./buildContainer.sh

Containers have been tested with CUDA version 12.0 and docker engine version 24.0.5.

### B.2.3	Codec pipeline

Figure B.2.3-1 shows the process executed by the codec pipeline as simplified pseudo-code. First, the process instantiates a scenario object and a coder object. Then, the process obtains the anchor model from the scenario object. It derives the size of the anchor model and uses the scenario object to derive the anchor model’s performance. Subsequently, the coder object encodes the anchor model to a bitstream and decodes the bitstream to obtain the reconstructed model. Finally, the process derives the size of the bitstream, uses the scenario object to derive the reconstructed model’s performance and writes the results to a file as comma separated values (csv).

scenario  = scenario_factory.get( cfg )
  coder     = coder_factory.get( cfg, scenario )

  anc_model = scenario.get_model()

  results["anc_size"] = get_size( anc_model )
  results["anc_perf"] = scenario.get_performance( anc_model )

  bit_stream  = coder.encode( anc_model )
  rec_model   = coder.decode( bit_stream )

  results["rec_size"] = get_size( bit_stream )
  results["rec_perf"] = scenario.get_performance( rec_model )

  write_to_csv( results )

scenario  = scenario_factory.get( cfg )
  coder     = coder_factory.get( cfg, scenario )

  anc_model = scenario.get_model()

  results["anc_size"] = get_size( anc_model )
  results["anc_perf"] = scenario.get_performance( anc_model )

  bit_stream  = coder.encode( anc_model )
  rec_model   = coder.decode( bit_stream )

  results["rec_size"] = get_size( bit_stream )
  results["rec_perf"] = scenario.get_performance( rec_model )

  write_to_csv( results )

Figure B.2.3-1: The main evaluation process (simplified pseudo-code)

### B.2.4	Configuration

The codec pipeline can be configured as shown in table B.2.4-1. Marks C, S, and R in the last column indicate that the parameters are directly forwarded to the coder object, the scenario object, and the result csv-file, respectively.

Table B.2.4-1: Configuration parameters

| Parameter name | Description | Forward |
| --- | --- | --- |
| coder_name | Name of the compression method | C,R |
| scenario_name | Name of the scenario | S,R |
| data_set_name | Name of the dataset | S,R |
| model_name | Name of the model (valid values depend on the scenario) | S,R |
| enc_cfg_file_name | Name of a config-file for the compression method | C |
| unique_tag | Unique tag added to output file-names | C,R |
| out_dir | Directory to store the csv-file, the bitstream, and other data | C |
| data_dir | Directory to model data and datasets | S |
| batch_size | Evaluation batch size (currently ignored) | S |
| workers | Number of workers for the data loader | S |
| disable_progress_bar | Disable progress bar | C, S |
| eval_compression | Compress and evaluate reconstructed model | R |
| eval_anchor | Evaluate anchor model | R |
| download_only | Only download models and datasets |  |

The available scenarios, model and compression methods that are currently available are shown in table B.2.4-2.

Table B.2.4-2: Implemented scenarios and compression methods

| Type | Name | Description |
| --- | --- | --- |
| Scenario | asr | Automatic speech recognition. Available models: wav2vec_asr_base_960h and hubert_asr_large |
| Coder | dummy | Dummy methods. Writes parameters as unmodified 32-bit floating point values. Copies the anchor model to the reconstructed model. |
| Coder | nnc | NNCodec |

The pipeline can, for example be started with:

python ./run --scenario_name="asr" --model_name="hubert_asr_large" 
       --coder_name="dummy"`

python ./run --scenario_name="asr" --model_name="hubert_asr_large" 
       --coder_name="dummy"`

### B.2.5	Encoder configuration files

Encoder configuration files for NNC are provided in evaluations/cfg and can be specified using the parameter enc_cfg_file_name. They can for example be used as follows:

python ./run --scenario_name="asr" --model_name="wav2vec_asr_base_960h"
       --coder_name="nnc" --enc_cfg_file_name="../evaluations/cfg/QP_-22.json"

python ./run --scenario_name="asr" --model_name="wav2vec_asr_base_960h"
       --coder_name="nnc" --enc_cfg_file_name="../evaluations/cfg/QP_-22.json"

A description of the parameters can be found at https://github.com/fraunhoferhhi/nncodec/wiki/Usage. The parameters bitstream_path, model_name, and dataset_path are programmatically set by the codec pipeline. Since the framework currently only supports non-data driven tools, the parameters ioq, lsa, and fine_tune must always be equal to 0.

### B.2.6	Result csv file

Table B.2.6-1 shows the results that are written to the result csv-file. Additionally, the configuration parameters marked with R in table B.2.4-1 are added.

Table B.2.6-1: Results written to the csv-file

| Name | Description | Unit |
| --- | --- | --- |
| anc_size | Size of the anchor model | byte |
| rec_size | Size of the bitstream | byte |
| compress_ratio | rec_size / anc_size | - |
| metric_name | Name of the metric | - |
| anc_perf | Performance of anchor model | Unit of metric_name |
| rec_perf | Performance of reconstructed model | Unit of metric_name |
| anc_eval_time | Evaluation time for anchor model | seconds |
| rec_eval_time | Evaluation time for reconstructed model | seconds |
| enc_time | Encoding time | seconds |
| dec_time | Decoding time | seconds |

### B.2.7	Running the codec pipeline with docker

The codec pipeline can be run within a docker container by calling docker/runPipeline.sh from within the docker directory, as follows:

cd docker

runPipeline.sh host_directory [other_parameters...]

cd docker

runPipeline.sh host_directory [other_parameters...]

with the following parameters:

- host_directory is a directory on the host system to which models/datasets are downloaded and to which results will be written.

- other_parameters are the other codec pipeline parameters, as specified above.

The script operates as follows:

- It mounts host_directory to the container.

- It sets the parameters out_dir and data_dir to host_directory\out and host_directory\data, respectively.

- It forwards other parameters to the invocation of pipeline/run.py within the container.

Example:

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
  --coder_name="dummy"

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
  --coder_name="dummy"

Configuration files for NNC, which are stored in evaluations/cfg, are also available within the docker image in directory framework/evaluations/cfg. The container can be run with a specific cfg-file, e.g. as follows:

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
   --coder_name="nnc" --eval_anchor=0 --unique_tag="test_hubert -22"
   --enc_cfg_file_name="/framework/evaluations/cfg/QP_-22.json"

runPipeline.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large"
   --coder_name="nnc" --eval_anchor=0 --unique_tag="test_hubert -22"
   --enc_cfg_file_name="/framework/evaluations/cfg/QP_-22.json"

### B.2.8	Extending the codec pipeline

The software framework allows to add new scenarios in a modular way. New scenarios must be provided as package containing a python class having the interface shown in figure B.2.8-1. The parameters marked with S in table B.2.4-1 are forwarded to the init function of the Scenario class within the opts variable.

class Scenario():
  def __init__(self, opts):
    self.metric_name = "MetricOfScenario"
    # Input:
    # - opts: an object with members defining the scenario configuration
    # Should:
    # - define self.metric_name as string denoting the performance metric of

#   the scenario, which will be forwarded to the result csv-file
    # - init object from opts

  def get_model(self, pre_trained):
    # Input:

# - pre_trained a boolean indicating whether to provide the pre-trained model
    # Should download model data and datasets, when not already done
    # Output:
    # - If pre_trained is true, model should be a pre-trained model,
    #   Otherwise, model should be an un-initialized model
    return model

  def download_data_and_models(self):
    # Should download model data and datasets, when not already done

  def get_perf(self, model, partition, enforce_higher_is_better=False):
    # Inputs:
    # - model: the model to get the performance for
    # - partition: the partition of the dataset used for evaluation:
    #   - "test"  The test partition for final performance measurement should be used
    #   - "valid" The validation partition for data-driven methods should be used
    # - enforce_higher_is_better: if true perf should be increasing with increasing 
    #   model performance
    # Outputs:
    # - perf: the performance
    # - infer_time: the inference plus measurement time
    return perf, infer_time

class Scenario():
  def __init__(self, opts):
    self.metric_name = "MetricOfScenario"
    # Input:
    # - opts: an object with members defining the scenario configuration
    # Should:
    # - define self.metric_name as string denoting the performance metric of

#   the scenario, which will be forwarded to the result csv-file
    # - init object from opts

  def get_model(self, pre_trained):
    # Input:

# - pre_trained a boolean indicating whether to provide the pre-trained model
    # Should download model data and datasets, when not already done
    # Output:
    # - If pre_trained is true, model should be a pre-trained model,
    #   Otherwise, model should be an un-initialized model
    return model

  def download_data_and_models(self):
    # Should download model data and datasets, when not already done

  def get_perf(self, model, partition, enforce_higher_is_better=False):
    # Inputs:
    # - model: the model to get the performance for
    # - partition: the partition of the dataset used for evaluation:
    #   - "test"  The test partition for final performance measurement should be used
    #   - "valid" The validation partition for data-driven methods should be used
    # - enforce_higher_is_better: if true perf should be increasing with increasing 
    #   model performance
    # Outputs:
    # - perf: the performance
    # - infer_time: the inference plus measurement time
    return perf, infer_time

Figure B.2.8-1: Interface required to be implement for new scenarios

The software framework allows to add new compression methods in a modular way. New compression methods must be provided as package containing a python class having the interface shown in figure B.2.8-1. The parameters marked with C in table B.2.4-1 are forwarded to the init function of the Coder class within the opts variable.

class Coder():
  def __init__(self, opts ):
    self.__opts = opts
    # Inputs:
    # - opts: an object with members defining the coder configuration:
    #   - opts.file_names["bit"]: the bitstream filename
    #   - opts.file_names["dec"]: the decoded model filename
    #   - opts.scenario: the scenario object
    # Should init the coder object from the opts object

  def encode(self, model ):
    # Inputs:
    # - model: the model to encode
    # Should:
    # - Encode the state_dict() of model to the file given in
    #   self.__opts.file_names["bit"]

  def decode(self, rec_model):
    # Inputs:
    # - rec_model: the model to write the reconstructed parameters to
    # Should:
    # - decode the bitstream file given in self.__opts.file_names["bit"]
    # - store the decoded parameters in the state_dict of rec_model

class Coder():
  def __init__(self, opts ):
    self.__opts = opts
    # Inputs:
    # - opts: an object with members defining the coder configuration:
    #   - opts.file_names["bit"]: the bitstream filename
    #   - opts.file_names["dec"]: the decoded model filename
    #   - opts.scenario: the scenario object
    # Should init the coder object from the opts object

  def encode(self, model ):
    # Inputs:
    # - model: the model to encode
    # Should:
    # - Encode the state_dict() of model to the file given in
    #   self.__opts.file_names["bit"]

  def decode(self, rec_model):
    # Inputs:
    # - rec_model: the model to write the reconstructed parameters to
    # Should:
    # - decode the bitstream file given in self.__opts.file_names["bit"]
    # - store the decoded parameters in the state_dict of rec_model

Figure B.2.8-2: Interface required to be implemented for new compression methods

Encoder-only optimization methods might use:

- the encode function to write optimized model parameters in a raw-byte format to the bitstream

- the decode function to read them back to rec_model.

runContainer.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large" \

--coder_name="dummy"

runContainer.sh ~/myAiMlData --scenario_name="asr" --model_name="hubert_asr_large" \

--coder_name="dummy"

## B.3	Scripts for the evaluation of split inferencing for object detection and labelling

### B.3.1	Introduction

The scripts corresponding to the evaluation of Split and Compression are available in [x8]

The code is available for both Linux and Windows platforms. Tests have been done on different machines with ubuntu 22.04.4 LTS and Git Bash for Windows 2.45.2.

Model sources:

  - ssd_resnet.onnx is generated by convert_ssd300_to_onnx.py and available at [x4].

  - retinanet.onnx is available at [x1] under “models” directory.

### B.3.2	Repository and installation

FPN/retinanet and single branch SSD300 scripts

The command to clone the git repository is:

git clone https://github.com/5G-MAG/rt-ai-ml-evaluation-framework.git

Multibranch scripts

The command to clone the git repository is:

git clone -b split_multibranch https://github.com/5G-MAG/rt-ai-ml-evaluation-framework.git

A readme file scripts/objectdetection/multibranch/Readme.md described how it works for splitting operation and inferencing with or without compression using quantization and/or NNCodec [x9]. Scripts may be used without installation of NNCodec.

The documented scripts are the following:

- split_onnx_multi.py

- split_ssd_resnet.sh

- split_retinanet.sh

- infer_onnx_multi.py

- infer_image.sh

- infer_video.sh

- infer_dataset_image.sh

- calc_map_image.py

- calc_map_video.py

- calc_map_image_dataset.py

Note: to use NNCodec, a specific installation shall be done following [x9].

Additional bash scripts located in scripts/objectdetection/tool_for_dataset are provided to download a set of images and associated annotations from the cocodataset repository, as well as a script to convert annotations to conform to calc_map scripts. A readme file scripts/objectdetection/tool_for_dataset /Readme.md details the following scripts usage:

- get_cocodataset_annotations.sh

- get_cocodataset_selection.sh

### B.3.3	Single branch APIs and scripts applied to SSD300 model

The scripts are:

- convert_ssd300_to_onnx.py

This script converts the PyTorch ssd_300 model to ONNX.

Usage: python convert_ssd300model.py <output_path_to_directory> Output: <output_path_to_directory>/ssd_resnet.onnx Example: From rt-ml-ai-evaluation-framework directory python scripts/objectdetection/ssd300/convert ssd300model.py ./models

- split_onnx.py

This script splits an ONNX file at identified bottlenecks points.

Usage: python split_onnx.py <path_to_onnx file> <split_point_name> <split_flag> split_flag :’before’ to split before the split_point_name , ‘after’ to split after the split_point_name

Example: python split_onnx.py ./models/ssd_resnet.onnx /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/relu_2/Relu before Output : First and second part of the split in “./models” Special character “/” in split_point_name is replaced with a “_”. Output example: /models/ssd_resnet_Part_I__feature_extractor_feature_extractor_feature_extractor.4_feature_extractor.4.0_relu_2_Relu.onnx ./models/ssd_resnet_Part_II__feature_extractor_feature_extractor_feature_extractor.4_feature_extractor.4.0_relu_2_Relu.onnx

- infer_onnx.py

This script is used to run the inference of ssd300 model on an image or on a video. It infers the first part and the second part of the model sequentially in GPU or in CPU. The predictions are saved with the format [label top_left_x top_left_y bottom_right_x bottom_right_y confidence_score], compatible with the scripts visualize.py and calc_map.py. Intermediate data are saved in numpy binary format .npz. The visual prediction results, the image with the boxes, are saved with the .png format. For video, only the first visual prediction is saved.

Usage: python infer_onnx.py [-h] [-c PATH_TO_CONFIG] [-s INPUT_SOURCE] [-loop LOOP] [-partI PARTI] [-partII PARTII] [-anchor ANCHOR] [-results_filename RESULTS_FILENAME] -results_dir RESULTS_DIR [-no_CPU_anchor] [-no_GPU_anchor] [-ref_split REF_SPLIT] [-no_split] Help: infer_onnx is a script that run the inference of a ssd resnet model, full model or split. Options: -h, --help show this help message and exit -c PATH_TO_CONFIG, --path_to_config PATH_TO_CONFIG Path to config file -s INPUT_SOURCE, --input_source INPUT_SOURCE Path to input source -loop LOOP loop inference -partI PARTI Path to model part I -partII PARTII Path to model part II -anchor ANCHOR Path to model anchor -results_filename RESULTS_FILENAME Path to results file -results_dir RESULTS_DIR Path to results directory hosting predictions -no_CPU_anchor no inference with CPU on model anchor -no_GPU_anchor no inference with GPU on model anchor -ref_split REF_SPLIT reference split label -no_split no split (just anchor for instance)

### B.3.4	Multi branch split APIs and scripts (ssd_resnet and retinanet) with compression

The scripts are:

- split_onnx_multi.py

This script splits an ONNX file at any node. Split point may be referenced by the node index/rank or by the node name.

| Usage Help split_onnx_multi is a script that split a ONNX model at a rank or at a node name |  |
| --- | --- |
| optional arguments: |  |
| -h, --help | show this help message and exit |
| -a ANCHOR, --anchor ANCHOR | Path to model anchor |
| -r RANK, --rank RANK | Rank of the node where to split the model; e.g., with '-r 7' Model I will contain nodes [0-6] and model II will contain nodes [7-48] |
| -n NAME, --name NAME | Name of the node where to split the model |
| -f FLAG, --flag FLAG | Split flag indicating if the split occurs 'before' or 'after' the given node (default is 'before') |

Script outputs are the two model subsets part I and part II, suffixed with “_part_I_node_#rank” (resp. _part_II_node_#rank), located in the same directory as the anchor.

Usage: python split_onnx_multi.py -a ./models/ssd_resnet.onnx -r 7 Output: Model will be split at rank 7, node name /feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/conv3/Conv (before) Model Part I: Extraction at level 0 to 7 (excluded): input=['input'] output=['/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/relu_1/Relu_output_0', '/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0'] Model Part II: Extraction at level 7 (included) to 154 : input=['/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/relu_1/Relu_output_0', '/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0'] output=['output1', 'output2'] Split done. part I: ./models/ssd_resnet_part_I_node_7.onnx part II: ./models /ssd_resnet_part_II_node_7.onnx Intermediate data to transfer: ['/feature_extractor/feature_extractor/feature_extractor.4/feature_extractor.4.0/relu_1/Relu_output_0', '/feature_extractor/feature_extractor/feature_extractor.3/MaxPool_output_0']

- infer_onnx_multi.py

This script infers the first ONNX model subset (part I) from a first ONNX runtime, encodes the output of the inference, decodes the encoded output, and passes the decoded output as an input for inference of the second ONNX subset (part II) with a second ONNX runtime. It emulates the two split inferences from a first and a second endpoint. It supports models “ssd_resnet” and “retinanet” or any other models having same pre-processing and same post-processing as these models.

| Usage Help infer_onnx is a script that run the inference of either a ssd resnet model or retinanet model, full model or split |  |
| --- | --- |
| optional arguments: |  |
| -h, --help | show this help message and exit |
| -s INPUT_SOURCE, --input_source INPUT_SOURCE | Path to input source (Image, video, directory of images) |
| -loop LOOP | loop inference (for image) |
| -partI PARTI | Path to model part I |
| -partII PARTII | Path to model part II |
| -anchor ANCHOR | Path to model anchor |
| -f FAMILY, --family FAMILY | Model family i.e., 'ssd_resnet' or 'retinanet' |
| -results_filename RESULTS_FILENAME | Path to .csv results file (time measurement) |
| -results_dir RESULTS_DIR | Path to results directory hosting predictions |
| -no_CPU_anchor | no inference with CPU on model anchor |
| -no_GPU_anchor | no inference with GPU on model anchor |
| -ref_split REF_SPLIT | reference split label |
| -no_split | no split (just anchor for instance) |
| -no_check_model | do not check the model partI and partII |
| -save_intermediate_data | to save intermediate data |
| -nb_frames NB_FRAMES | Limit the inference to the nb_frames first frames for a video |
| -encode_algo ENCODE_ALGO | algo for intermediate_data encoding 0=no encoding; 1= convert to float16; 8xx for nnc with xx = abs(qp) (e.g., 838 for qp=-38) |
| -PU_partI PU_PARTI | Processing Unit for part I (CPU or GPU) |
| -PU_partII PU_PARTII | Processing Unit for part II (CPU or GPU) |
| --labels_coco LABELS_COCO | Path to labels file coco_labels.csv |

According to the parameters the script is doing:

- Verification of the ONNX model by using checker.check_model() function. This verification can be skipped especially for the splitted models that may sometimes raise an unfounded error (i.e., an error which does not prevent a correct inference with the ONNX runtime)

- inference of the anchor using GPU (except if flag -no_GPU_anchor is activated)

- inference of part I and part II using processing units indicating by flags (-PU_partI  and -PU_partII)(except if flag -no_split is activated). Intermediate data are encoded and decoded using the algorithm indicated by the identifier -encode_algo.

- inference of the anchor using CPU (except if flag -no_CPU_anchor is activated)

Script outputs are:

- A .csv file containing information on time measurement, intermediate data size, predictions (described below)

- A folder containing the image (or frame) with the predictions on overlay (bounding boxes with label and prediction score)

- A folder containing the .txt predictions file compatible with format expected by calc_map scripts

Results .csv file have following columns:

| Column name | Type | Unit | Description |
| --- | --- | --- | --- |
| Source | String | - | path to the input data source |
| Nodes | String | - | node reference (e.g., split_node_0010) |
| Inference_loop | Int | - | number of inference run on the source |
| UE_inference_time_CPU (avg; std) | Float | ms | inference time of the part I (average and standard deviation on all measures) when the processing unit of part I is CPU |
| UE_inference_time_CPU_steady_state (avg;std) | Float | ms | inference time of the part I (average and standard deviation on all measures except the first one) when the processing unit of part I is CPU |
| UE_inference_time_GPU (avg;std) | Float | ms | inference time of the part I (average and standard deviation on all measures) when the processing unit of part I is GPU |
| UE_inference_time_GPU_steady_state (avg; std): | Float | ms | inference time of the part I (average and standard deviation on all measures except the first one) when the processing unit of part I is GPU |
| Server_inference_time_CPU(avg; std): | Float | ms | inference time of the part II (average and standard deviation on all measures except the first one) when the processing unit of part II is CPU |
| Server_inference_time_CPU_steady_state(avg; std): | Float | ms | inference time of the part II (average and standard deviation on all measures except the first one) when the processing unit of part I is CPU |
| Server_inference_time_GPU(avg; std); | Float | ms | inference time of the part II (average and standard deviation on all measures except the first one) when the processing unit of part II is GPU |
| Server_inference_time_GPU_steady_state (avg; std); | Float | ms | inference time of the part II (average and standard deviation on all measures except the first one) when the processing unit of part I is GPU |
| Total_inference_time_steady_state (avg; std); | Float | ms | Sum of “UE inference time steady state” + “Server inference time steady state “ (average and standard deviation) |
| Encoding_time (min,avg,max); | Float | ms | Encoding time of the intermediate data |
| Decoding_time (min,avg,max); | Float | ms | Decoding time of the intermediate data |
| nb_inference; | Int | - | number of inference run on a video source (nb frames) |
| intermediate_data_size (min,avg,max); | Int | bytes | Intermediate data size (uncompressed) |
| encoded_intermediate_data_size (min,avg,max); | Int | bytes | Intermediate data size (encoded/compressed) |
| encoding_algo; | Int | - | Encoding Algorithm identifier |
| predictions_size (min,avg,max); | Int | Bytes | Size of the post-processed predictions |
| predictions | String | - | Post-processed predictions (label, boxes coordinates (top_x, top_y, bottom_x, bottom_y), confidence score) (e.g., bear 13 28 573 620 0.49) |

- calc_map_image.py

This script calculates the mAP score for an image. This script is based on the existing calc_map.py script, and is a simple adaptation to be able to compute the mAP score on a single image. It uses the same calculate_map() function as calc_map.py. It can compute the mAP score for several splits.

| Usage Help Calculate the mAP for the object detection prediction |  |
| --- | --- |
| optional arguments: |  |
| -h, --help | show this help message and exit |
| -i IMAGE_PATH, --image_path IMAGE_PATH | Path to the image |
| -p PREDICTION_PATH, --prediction_path PREDICTION_PATH | Path to the prediction file or directory containing prediction files (for several splits) |
| -g GROUNDTRUTH_PATH, --groundtruth_path GROUNDTRUTH_PATH | Path to the ground-truth annotation file |
| -r RESULTS_FILENAME, --results_filename RESULTS_FILENAME | Path to .csv results file |
| --threshold THRESHOLD | The threshold for the prediction confidence to consider the prediction. |
| --no_plot | do not display the plot. |

- calc_map_video.py

This script calculates the mAP score for a video. This script is based on the existing calc_map.py script and is a simple adaptation to be able to compute the mAP score on a single video. It uses the same calculate_map() function as calc_map.py. It can compute the mAP score for several splits.

| Usage Help Calculate the mAP for the object detection prediction |  |
| --- | --- |
| optional arguments: |  |
| -h, --help | show this help message and exit |
| -v VIDEO_PATH, --video_path VIDEO_PATH | Path to the video |
| -p PREDICTION_PATH, --prediction_path PREDICTION_PATH | Path to the directory containing prediction files (one split) or Path to the directory containing directories of each split (several splits) |
| -g GROUNDTRUTH_PATH, --groundtruth_path GROUNDTRUTH_PATH | Path to the directory containing groundtruth annotation file |
| -r RESULTS_FILENAME, --results_filename RESULTS_FILENAME | Path to .csv results file |
| --labels_imagenet LABELS_IMAGENET | Path to labels file imagenet_coco.csv |
| --labels_coco LABELS_COCO | Path to labels file coco_labels.csv |
| -o IMAGE_PATH, --image_path IMAGE_PATH | Path to the output file containg the mAP outplot plot |
| --threshold THRESHOLD | The threshold for the prediction confidence to consider the prediction. |

- calc_map_image_dataset.py

This script calculates the mAP score for a set of images. This script is based on the existing calc_map.py script and is a simple adaptation to be able to compute the mAP score on a set of images. It uses the same calculate_map() function than calc_map.py. It can compute the mAP score for several splits.

| Usage Help Calculate the mAP for the object detection prediction |  |
| --- | --- |
| optional arguments: |  |
| -h, --help | show this help message and exit |
| -d IMAGE_DATASET_PATH, --image_dataset_path IMAGE_DATASET_PATH | Path to the directory containing the images |
| -p PREDICTION_PATH, --prediction_path PREDICTION_PATH | Path to the directory containing prediction files (one split) or Path to the directory containing directories of each split (several splits) |
| --multisplit | indicates if prediction path contains several directories for multi split |
| -g GROUNDTRUTH_PATH, --groundtruth_path GROUNDTRUTH_PATH | Path to the directory containing groundtruth annotation file |
| -r RESULTS_FILENAME, --results_filename RESULTS_FILENAME | Path to .csv results file |
| --labels_imagenet LABELS_IMAGENET | Path to labels file imagenet_coco.csv |
| --labels_coco LABELS_COCO | Path to labels file coco_labels.csv |
| --threshold THRESHOLD | The threshold for the prediction confidence to consider the prediction. |

### B.3.5	FPN/RPN retinanet scripts

The scripts are:

- convert_model.py

a script to convert a pre-trained model into an ONNX model

- inferonnx.py:

this script is used to run an object detection inference model and produce predication results in the following format [label top_left_x top_left_y bottom_right_x bottom_right_y confidence_score]. The model is used to produce results for the anchors, where the full model is run locally on the device or completely in the network.

usage: inferonnx.py [-h] [--mask] dataset_name model_location inferonnx.py: error: the following arguments are required: dataset_name, model_location

- split_retinanet.py:

this script is used to split the retinanet represented in the ONNX format. It takes the model at models/retinanet.onnx and splits at the four feature pyramid network (FPN) feature maps, as shown by the 4 nodes with red arrows pointed to in Figure 2.3-1, together with four other auxiliary operations (two of which are pointed to by the blue arrows in Figure 2.3-1 and there are two similar ones on the right side of the graph but not shown) that provide the input image shape information for later stages of the network. Note that the split needs 8 split points, rather than a single split point, due to branching and joining present in the structure of retinanet.

The splitting results in two partial models, called retinanet_part1.onnx and retinanet_part2.onnx, also in ONNX format. The input to part 1 is the input image. The feature maps in the output of part 1 is part of the input to part 2. The correct operation of part 2 needs additional input which is the shape of the input image. However, it makes no sense to feed the input image (together with the feature maps) as input to part 2. To resolve this problem, a dummy image of the same shape as the input image is used to generate the shape needed by part2. As a result, there is an overlap between part 1 and part 2. The overlap is chosen in such a way that only the portion of the graph directly contributing to generating the shape of the dummy image is included to minimize the additional processing. This is corroborated by the sizes of the models:

  - retinanet.onnx: 149.433MB

  - retinanet_part1.onnx: 120.731MB

  - retinanet_part2.onnx: 28.840MB

from which we see that the sum of the two partial models is only 0.14MB bigger than the size of the whole model, indicating that the overlap is negligible and so is the additional processing for generating the shape of the dummy image.

The two parts are fed into infer_split.py for split inference.

- infer_split.py:

this script is used to run split inference. It is passed the two parts of the model. It runs the first part of the model and saves the results in numpy binary format NPZ. Then it proceeds to run inference using the second part of the model, which loads the NPZ files as input and produces the object detection results. A flag SAVE_FEATURES_IN_FILEs controls whether to write the FPN feature maps to the NPZ files, and it can be set to 0 to save storage, and in that case the feature maps out of the execution of part 1 are directly fed to part 2. This script also compares the performance between split inference and non-split inference in terms of normalized MSE.

usage: splitinfer.py [-h] [--mask] dataset_name model_part1_location model_part2_location Run split inference using ONNX models positional arguments: dataset_name Dataset name model_location Path to the unsplit ONNX Model model_part1_location Path to 1st part of the ONNX Model model_part2_location Path to 2nd part of the ONNX Model optional arguments: -h, --help show this help message and exit --mask Indicates if output of model is a Mask and needs to be converted

- calc_map.py:

This script is used to calculate the mean Average Precision (mAP) score for the predictions. It compares the predicted labels and their bounding boxes to the ground truth annotations that are provided by the dataset.

usage: calc_map.py [-h] [--ds DATASET_NAME] [--threshold THRESHOLD] video_name Calculate the mAP for the object detection prediction. positional arguments: video_name The name of the video sequence, e.g. Kimono. optional arguments: -h, --help show this help message and exit --ds DATASET_NAME Name of the dataset. Defaults to SFU-HW-Objects. --threshold THRESHOLD The threshold for the prediction confidence to consider the prediction.

- visualize.py:

The visualize script takes the ground truth annotations or the predictions and renders them on top of the video. This script is useful to inspect the prediction results.

usage: visualize.py [-h] [--sleep_time SLEEP_TIME] video_fn annotation_path Visualize Object Detection. positional arguments: video_fn Path to the video file annotation_path Path to the folder with annotations/predictions optional arguments: -h, --help show this help message and exit --sleep_time SLEEP_TIME Specifies the interval between the display of 2 consecutive frames

Annex C :
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023/11/16 | SA4#126 | S4-231933 |  |  |  | Initial draft of the TR: - Agreement of S4-231774: TR skeleton | 0.1.0 |
| 2024/01/31 | SA4#127 | S4-240389 |  |  |  | Agreed version of the TR at SA4#127 - Agreement of S4-240211 | 0.2.0 |
| 2024/08/23 | SA4#129-e | S4-241643 |  |  |  | Agreed version of the TR at SA4#129-e - Agreement of S4-241586 | 0.3.0 |
| 2024/11/20 | SA4#130 | S4-242112 |  |  |  | Agreed version of the TR at SA4#129-e - Agreement of S4-242154, S4-241951 | 0.4.0 |