3GPP TSG-SA4 Meeting #131	S4-250114

# Geneva, 17th - 21st February 2025

Source:	Apple Inc., Samsung Electronics

Title:	Frame packed stereoscopic content for SA4 specifications

Agenda item:	9.5

Document for:	Agreement

# 1 Introduction

Draft TS 26.265 gathers video operating points from various SA4 specifications. In the conclusion of TR 26.266 and at the onset of VOPS work, it was noted that frame-packed stereoscopic content is supported in various SA4 specifications, but this is not currently reflected in Draft TS 26.265 v0.5.0. Conversely, several SA4 normative specifications provide a varying degree of support for frame packed content. Proponent companies have advocated for frame packing HEVC at various occasions. The following provides an overview of support of frame packed stereoscopic content in SA4 specifications.

# 2 Overview of support for frame packed stereoscopic content

## 2.1 Support for frame packed stereoscopic in 3GPP TS 26.114 (MTSI)

In clause 5.2 (Codecs for MTSI clients in terminals) for video, specifically for HEVC, frame packing is disallowed given the yellow highlighted text in the following.

### 5.2.2	Video

MTSI clients in terminals offering video communication shall support:

-	H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 1.2;

-	H.265 (HEVC) [119] Main Profile, Main Tier, Level 3.1. The only exception to this requirement is for the MTSI client in constrained terminal offering video communication, in which case the MTSI client in constrained terminal should support H.265 (HEVC) Main Profile, Main Tier, Level 3.1.

In addition, they should support:

-	H.264 (AVC) [24] Constrained High Profile (CHP) Level 4.0;

-	H.265 (HEVC) [119] Main Profile, Main Tier, Level 4.0.

For backwards compatibility to previous releases, if H.264 (AVC) [24] Constrained High Profile Level 3.1 is supported, then H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 3.1 should also be offered.

H.264 (AVC) shall be used without requirements on output timing conformance (annex C of [24]). Each sequence parameter set of H.264 (AVC) shall contain the vui_parameters syntax structure including the num_reorder_frames syntax element set equal to 0.

H.265 (HEVC) Main Profile shall be used with general_progressive_source_flag equal to 1, general_interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, general_frame_only_constraint_flag equal to 1, and sps_max_num_reorder_pics[ i ] equal to 0 for all i in the range of 0 to sps_max_sub_layers_minus1, inclusive, without requirements on output timing conformance (annex C of [119]).

The same is applied for still images. However, the normative Annex Y for “Immersive Teleconferencing and Telepresence for Remote Terminals” (ITT4RT) specifies support for frame packing with frame_packing_arrangement_type equal to 4 (top-bottom packing arrangement) in clause Y.3 (Immersive 360-Degree Video Support). This may be a contradiction and should be checked if the “vanilla” “MTSI clients in terminals offering video communication” disallows this support and special case of “ITT4RT” allows it.

## 2.2 Support for frame packed stereoscopic in 3GPP TS 26.346

The MBMS specification provides support for frame packed stereoscopic 3D content for H.264/AVC only:

### 10.5.2	Stereoscopic 3D Video

If a MBMS client supports stereoscopic 3D video, it should support frame-packed stereoscopic 3D video with the following characteristics:

-	The bitstream conforms to H.264 (AVC) Constrained Baseline Profile Level 1.3 decoder [43], or conforms to H.264 (AVC) Progressive High Profile Level 3.1 decoder [43]. The Maximum VCL Bit Rate shall be constrained to be 14Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1000 and 1200, respectively.

-	Frame packing type is indicated by the frame packing arrangement SEI messages of H.264 (AVC) [43] as follows:

-	The syntax element frame_packing_arrangement_type has one of the defined values: 3 for Side-by-Side, 4 for Top-and-Bottom.

-	The syntax element quincunx_sampling_flag is equal to 0;

-	The syntax element content_interpretation_type is equal to 1;

-	The syntax elements spatial_flipping_flag is equal to 0;

-	The syntax element field_views_flag is equal to 0;

-	The syntax element current_frame_is_frame0_flag is equal to 0;

-	When an access unit contains a frame packing arrangement SEI message A and the access unit is neither an IDR access unit nor an access unit containing a recovery point SEI message, the following two constraints apply:

-	There shall be another access unit that precedes the access unit in both decoding order and output order and that contains a frame packing arrangement SEI message B.

-	The two frame packing arrangement SEI messages A and B shall have the same value for the syntax element frame_packing_arrangement_type.

If a MBMS client supports frame-packed stereoscopic 3D video, it shall support parsing of frame packing arrangement SEI messages as specified in H.264 (AVC) [43].

## 2.2 Frame packed stereoscopic content in TS 26.118

Various HEVC decoding configurations with frame_packing_arrangement_type equal to 4 are used in TS 26.118 (Virtual Reality (VR) profiles for streaming applications).

2.3 Misc. e.g. PSS (TS 26.234)

Support needs to be further analysed.

# 3 Conclusion and Proposal

To summarize, there is a varying support for frame packed content in SA4 specifications:

- For generic MTSI clients in TS 26.114, for HEVC specifically, frame packing is disallowed. However, for the special case of ITT4RT it is used normatively. It needs to be checked if this is contradicting.

- MBMS specification provides support for frame packed stereoscopic 3D content for H.264/AVC only.

- Various HEVC decoding configurations with frame packing are used in TS 26.118.

- Misc. specs such as PSS (TS 26.234) need to be analyzed and decided upon.

Given the relatively non-uniform support for frame packing, it is proposed to provide uniform frame packing support for HEVC content in draft TS 26.265. Additionally, draft TS 26.265 text in Table 4.4.2-1 stating “Typically restricted to no frame packing, but applications may use frame packing” should be revised as there is no typical restriction.