3GPP TSG-SA4 Meeting #131	S4-250121

# Geneva, 17th - 21st February 2025

Source:	Apple Inc.

Title:	On Picture Segmentation Information SEI Message in VSEI

Agenda item:	9.11

Document for:	Information

# 1 Abstract

This document provides information regarding the adoption of a new auxiliary image type (AUX_SEGMENT) and the corresponding "Picture Segmentation Information SEI message" into the Technologies under Consideration (TuC) of the VSEI specification by JVET. The proposal from JVET-AK0325, was reviewed and approved for inclusion in VSEI TuC, targeting a standardized approach for signaling segmentation information in video bitstreams. Given that 3GPP has applications that could benefit from this new mechanism, we believe that sharing this information with 3GPP SA4 is valuable.

# 2 Summary

The proposal from JVET-AK0325 [1] introduced the AUX_SEGMENT auxiliary image type, which, when signaled in a coded picture, indicates the presence of segmentation data. This segmentation data is further described by the newly defined Picture Segmentation Information SEI message, which provides necessary metadata for interpreting the segmentation values in the auxiliary image. This proposal was subsequently added to the VSEI TuC to ensure its consideration for future standardization [2][3].

This segmentation mechanism is particularly useful for applications that require structured segmentation of images for tasks such as object identification, region annotation, and content selection. In an example shown in Figure 1, six objects/segments were identified highlighted with a different value in the segmentation layer image. Each object gets an ID value assigned to it. For example, an ID equal to 5 could be associated with a clock.

Figure 1 Example of segmenting and mapping to values in the alpha frame to IDs.

In the above example, if a user wanted to select the Safari Browser window (object with ID equal to 4), the decoder could select the pixels in the texture image that corresponded to the samples in the segmentation plane that had the value associated with ID equal to 4. All other pixels would be ignored or zeroed out. If, instead, another object with another ID was needed, the decoder would select the corresponding texture pixels and ignore all others. Multiple objects could also be selected if desired. In an essence this can also be seen as a “dynamic” alpha mask with the user being able to control which object or objects should be opaque and which areas should be fully transparent. Further operations such as cropping could also benefit from this segmentation technique.

For further details please refer to [2] (and [1] if [2] is not yet published).

# References

- JVET-AK0325, D. Podborski, A. M. Tourapis (Apple), "New SEI message for Segmentation Information"

- JVET-AK2032, S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, Y.-K. Wang, "Technologies under consideration for future extensions of VSEI (version 7)"

- ISO/IEC 23002-7, Information technology — MPEG video technologies, Part 7: Versatile supplemental enhancement information messages for coded video bitstreams