3GPP TSG-SA4 Meeting #131	S4-250123

# Geneva, 17th - 21st February 2025

Source:	Apple Inc.

Title:	[VOPS] On web codecs

Agenda item:	9.5

Document for:	Agreement

# 1 Introduction

The WebCodecs API provides low-level access to media codecs for efficient video decoding within web applications. However, its support for layered video codecs may be limited when a layered video bitstream is carried in a single video track. This API is also used as basis for clause B.2 of draft TS 26.265. This enables comparison of the available capabilities of this API and what is needed by applications. One important issue identified is documented in the following.

# 2 Overview

The VideoDecoderConfig [1] structure contains a single description element, which typically holds codec-specific configuration data. Upon closer examination, this element corresponds to a video decoder configuration box, such as av1C for AV1 or hvcC for HEVC. While this approach works for conventional single-layer video codecs, it raises concerns for layered codecs that require multiple configuration records.

For example, for MV-HEVC in mp4, an additional lhvC configuration box is present alongside hvcC in VisualSampleEntry. The current WebCodecs API does not explicitly provide a mechanism to support multiple configuration records within VideoDecoderConfig. This limitation could prevent the lhvC configuration box from being correctly processed, leading to the loss of essential Sequence Parameter Sets and Picture Parameter Sets for the higher layers. Consequently, only the base layer may be successfully decoded, while additional layers could be discarded or improperly handled.

To better support layered video codecs, an extension to the WebCodecs API may be necessary, e.g., by allowing multiple description elements within the VideoDecoderConfig, or by mandating all the required NAL units to be inside a single description, which may go against the definitions in ISO/IEC 14496-15.

# 3 Proposal

- The gaps and issues are documented in clause B.2 (WebCodecs API) of draft TS 26.265.

- Identify way forward to address such gaps, and to liaise with organizations as W3C on such issues.

# References

- https://w3c.github.io/webcodecs/#video-decoder-config